| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080440 (4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-4080440 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 31.03.2013 | All | editorial changes. | CEDEX |
| Version 2.9.2 29.11.2013 | All | editorial changes. | CEDEX |
| Version 2.9.3 13.12.2013 | All | editorial changes. | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction |  | 4080440 |
| Test Case of Feature | The trackside generates and sends a radio message including packet 65(TSR) to a train. On-board checks and verifies and then accepts the message. |  | 1 |
| Applicable Mode/Level Combinations | L2: SB, SR, FS, OS, TR, PT, LS L3: SB, SR, FS, OS, TR, PT, LS |  |  |
| Target of Test | The target of this test is to check that The message including packet 65 is accepted and stored on-board. The TSR is correctly calculated (according to LRBG, distances and time stamps referring to on-board time) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 65 TSR is recorded. The received TSR corresponds to site data. One of the last eight LRBG:s is used as reference for the TSR. |
| Constraints | For PT mode the information “Recognition of Exit from TR mode” A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 | FS / OS / SR / SB / TR / PT/ LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received | I | RTM | Complete message is accepted and stored | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_PACKET=65) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | USE_FT4070201.1 | - | - | (Supervision of temporary speed restriction) The train supervises the temporary speed restrictions | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 | FS / OS / SR / SB / TR / PT / LS |
| Temporary Speed Restrictions | STORED | TSR has been received |
| ERTMS/ETCS level | 3/ 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction |  | 4080440 |
| Test Case of Feature | The trackside generates and sends a radio message including packet 65(TSR) to a train which rejects the TSR because new train data has been sent to the RBC and the acknowledgement of train data has not been received on-board. |  | 2 |
| Applicable Mode/Level Combinations | L2: SR, FS, OS, TR, PT, LS, SB L3: SR, FS, OS, TR, PT, LS, SB |  |  |
| Target of Test | The target of this test is to check that Packet 65 is rejected by on-board (when train data NOT is acknowledged by RBC) Comment: The train has sent out new train data to RBC but not received acknowledge from RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Packet 65 TSR is recorded. |
| Constraints | A cab needs to be active (SB Mode) For PT mode the information “Recognition of Exit from TR mode”. The acknowledgement of train data has not been received on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 | FS / OS / SR / SB / TR / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | Validated train data | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 2 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 3 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 (D_TSR=<D1>; V_TSR= <V1>)is received | I | RTM | TSR is rejected. | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 4 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_PACKET=65) is recorded | O | JRU | TSR is recorded. There is no changes on DMI. | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 5 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 6 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 7 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | USE_FT5110200.13 | - | - | Only in TR mode. This is for changing to PT mode. | L2 / L3 | SR / FS / OS / PT / LS / SB | - |
| 8 | L2 / L3 | SR / FS / OS / PT / LS / SB | USE_FT5110200.5 | - | - | Only in PT mode. This is for changing to FS mode. | L2 / L3 | SR / FS / OS / PT / LS / SB | - |
| 9 | L2 / L3 | SR / FS / OS / PT / LS / SB | USE_FT5040300.19 | - | - | Only in SB mode. This is for changing to FS mode. | L2 / L3 | SR / FS / OS / PT / LS / SB | - |
| 10 | L2 / L3 | SR / FS / OS LS | USE_FT4060300.21 OR USE_FT4060300.23 OR USE_FT4060300.59 | - | - | Only in SR, OS or LS mode. This is for changing to FS mode. | L2 / L3 | FS | - |
| 11 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | V_PERM is different from V1 (=V_TSR) when the train reaches the location given by D_TSR | L2 / L3 | FS | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |

| Step 5: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction |  | 4080440 |
| Test Case of Feature | The trackside generates and sends a radio message including packet 65(TSR) to a train. On-board checks and verifies and then accepts the message because an order to swich to L2/3 at a further location has been recibed |  | 3 |
| Applicable Mode/Level Combinations | L1: SB, SR, FS, OS, TR, PT, LS L0: SB, SR, FS, OS, TR, PT, LS NTC:SN |  |  |
| Target of Test | The target of this test is to check that - The message including packet 65 is accepted and stored on-board. - The TSR is correctly calculated (according to LRBG, distances and time stamps referring to on-board time) after the level transition |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 65 TSR is recorded. The received TSR corresponds to site data. One of the last eight LRBG:s is used as reference for the TSR. |
| Constraints | A transition level to L2/3 has been received For PT mode the information “Recognition of Exit from TR mode” A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / SB / TR / PT / LS /SN |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 /1 / 2 | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received | I | RTM | Complete message is accepted and stored | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_PACKET=65) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT5100100 | - | - | The train reach the transition level.Use the corresponding TC. | N/A | N/A | - |
| 4 | N/A | N/A | USE_FT4070201.1 | - | - | (Supervision of temporary speed restriction) The train supervises the temporary speed restrictions | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Message 24 (General radio message; RBC-> Train ) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| … | … | … | 21, 27, plus common optional packets |
| NID_PACKET | 8 | 65 | Packet 65 ( TSR) ; included in radio message 24 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| Optional packets | … | … | May be included |
| NID_PACKET | 8 | 11111111 | Packet 255 end information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / SB / TR / PT / LS /SN |
| Temporary Speed Restrictions | STORED | TSR has been received. |
| ERTMS/ETCS level | 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction |  | 4080440 |
| Test Case of Feature | The trackside generates and sends a Telegram including packet 65(TSR) to a train. The onboard ACCEPTS the message. |  | 4 |
| Applicable Mode/Level Combinations | L0: SB, UN L1: FS, OS, SR, SB, TR, LS L2: FS, OS, SR, SB, TR, PT, LS L3: FS, OS, SR, SB, TR, PT, LS |  |  |
| Target of Test | The target of this test is to check that The Telegram including packet 65 is accepted and stored on-board. The TSR is correctly calculated (according to LRBG, distances and time stamps referring to on-board time) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Telegram including packet 65 TSR is recorded. The received TSR corresponds to site data. |
| Constraints | A cab needs to be active (SB Mode) For PT mode the information “Recognition of Exit from TR mode” |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 | FS / OS / SR / UN / SB / TR / PT / LS |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | One Balise Group message containing packet 65 is received | I | BTM | Complete Telegram is ACCEPTED and stored | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 2 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=65) is recorded | O | JRU | Complete Telegram is recorded | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 3 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT5110200.13 | - | - | Only in TR mode. This is for changing to PT mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 4 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT5110200.5 | - | - | Only in L2/L3 and PT mode. This is for changing to FS mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 5 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT5110200.7 and USE_FT4060300.49 | - | - | Only in L1 and PT mode. This is for changing to SR mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 6 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT5040300.19 | - | - | Only in L2/L3 and SB mode. This is for changing to FS mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 7 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT5040300.16 and USE_FT4060300.49 | - | - | Only in L1 and SB mode. This is for changing to SR mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 8 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT5040300.27 and USE_FT4060300.58 | - | - | Only in L0 and SB mode. This is for changing to UN mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 9 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT4060300.21 OR USE_FT4060300.23 OR USE_FT4060300.59 | - | - | Only in L2/L3 and SR, OS or LS mode. This is for changing to FS mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 10 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT4060300.44 | - | - | Only in L1 and SR, OS or LS mode. This is for changing to FS mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 11 | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | USE_FT5100100.14 AND USE_FT4080425.12 AND USE_FT4060300.19 | - | - | Only in UN mode. This is for changing to FS mode. | L0 / L1 / L2 / L3 | UN / SR / FS / OS / TR / PT / LS / SB | - |
| 12 | L1 / L2 / L3 | FS | USE_FT4070201.1 | - | - | (Supervision of temporary speed restriction) The train supervises the temporary speed restrictions | L1 / L2 / L3 | FS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction |  | 4080440 |
| Test Case of Feature | Reception of a telegram including packet 65(TSR). The onboard ACCEPTS the message, because a Level 1 or a Level 2 transition order is stored onboard. |  | 5 |
| Applicable Mode/Level Combinations | LNTC: SB, TR, SN |  |  |
| Target of Test | The target of this test is to check that The Telegram including packet 65 is accepted and stored on-board. The TSR is correctly calculated (according to LRBG, distances and time stamps referring to on-board time) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Telegram including packet 65 TSR is recorded. The received TSR corresponds to site data. |
| Constraints | A cab needs to be active (SB Mode). Level 1 or a Level 2 transition order is stored onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7 / 13 | SB / TR / SN |
| Level Transition Order | STORED | L1 / L2 / L3 |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 65 is received | I | BTM | Complete Telegram is ACCEPTED and stored | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=65) is recorded | O | JRU | Complete Telegram is recorded | N/A | N/A | - |
| 3 | N/A | N/A | USE_FT5100100 | - | - | The TC depends on the level transition (L3:TC 20, L2: TC 21 or L1: TC 23. | N/A | N/A | - |
| 4 | N/A | N/A | USE_FT4060300 | - | - | The TC depends on the mode transition | N/A | N/A | - |
| 5 | N/A | N/A | USE_FT4070201 | - | - | (Supervision of temporary speed restriction) The train supervises the temporary speed restrictions | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/4/12 | FS/SR/OS/UN/LS |
| Level Transition Order | 2 / 3 / 4 | L1 / L2 / L3 |
| Temporary Speed Restrictions | STORED | TSR has been received |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction |  | 4080440 |
| Test Case of Feature | The onboard device receives a radio message including packet 65(Temporary Speed Restriction). The message is REJECTED |  | 6 |
| Applicable Mode/Level Combinations | L0: SB, UN L1: FS, OS, SR, SB, TR, PT, LS LNTC: SN |  |  |
| Target of Test | The target of this test is to check that - The message including packet 65 is accepted and stored on-board. - The TSR is correctly calculated (according to LRBG, distances and time stamps referring to on-board time) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 65 TSR is recorded. Check on the DMI that: The display does NOT change. The TSR is NOT stored onboard |
| Constraints | A cab needs to be active (SB Mode) for be sure is not rejected by 4.8.4.2 exceptions For PT mode the information “Recognition of Exit from TR mode” for be sure is not rejected by 4.8.4.2 exceptions Level transition order to L2/L3 is not stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | To L2/L3 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 (D_TSR=<D1>; V_TSR= <V1>)is received | I | RTM | The message is REJECTED. | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | - |
| 2 | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_PACKET=65) is recorded | O | JRU | - | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | - |
| 3 | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | USE_FT5110200.13 | - | - | Only in TR mode. This is for changing to PT mode. | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | - |
| 4 | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | USE_FT5110200.7 and USE_FT4060300.49 | - | - | Only in L1 and PT mode. This is for changing to SR mode. | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | - |
| 5 | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | USE_FT5040300.16 and USE_FT4060300.49 | - | - | Only in L1 and SB mode. This is for changing to SR mode. | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | - |
| 6 | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | USE_FT5040300.27 and USE_FT4060300.58 | - | - | Only in L0 and SB mode. This is for changing to UN mode. | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | - |
| 7 | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | USE_FT5100100.14 AND USE_FT4080425.12 AND USE_FT4060300.19 | - | - | Only in UN or SN mode. This is for changing to FS mode. | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | - |
| 8 | L1 | SR / FS / OS / LS | USE_FT4060300.44 | - | - | Only in L1 and SR, OS or LS mode. This is for changing to FS mode. | L0 / LNTC / L1 | UN / SN / SR / FS / OS / TR / PT / LS / SB | - |
| 9 | L1 | FS | USE_FT5100100.37 or USE_FT5100100.36 | - | - | For changing to L2 or L3 | L2 / L3 | FS | - |
| 10 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | V_PERM is different from V1 (=V_TSR) when the train reaches the location given by D_TSR | L2 / L3 | FS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction |  | 4080440 |
| Test Case of Feature | The trackside generates and sends a Telegram including packet 65(TSR) to a train to the on-board equipment. Comment : All information shall be given from Euroloop. Euroloop transmission equipment shall be available. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that The Telegram including packet 65 is accepted and stored on-board. The TSR is correctly calculated (according to LRBG, distances in in-fill area and next main signal) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Telegram including packet 65 TSR is recorded. The received TSR corresponds to site data(according to LRBG, distances in in-fill area and next main signal) |
| Constraints | Linking information is stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT4080411.1. | - | - | "Transmission of announcement of Euroloop via Balise Group (EOLM) to announce a Euroloop message | N/A | N/A | - |
| 2 | N/A | N/A | One Euroloop message containing packet 65 is received | I | LTM | Complete Telegram is accepted and stored,(after checking according to LRBG, distances in in-fill area and next main signal reference). | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7;NID_PACKET=65) is recorded | O | JRU | Complete Telegram is recorded | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | CSM. Normal Status If in FS mode: The permitted speed is updated. If in LS mode: The train speed increases until it overpasses the PS and then the supervision status changes to OvS. | N/A | N/A | - |
| 5 | N/A | N/A | USE_FT4070201.1 | - | - | (Supervision of temporary speed restriction) The train supervises the temporary speed restrictions | N/A | N/A | - |

| Step 2: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | - |
| NID_PACKET | 8 | 255 | 255 |
| NID_PACKET | 8 | 255 | 255 |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |