| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080433 (4.8.4: Accepted Information depending on the modes - Reversing Area Information) Total: 12 Test case(s) |  |
| REF: | Subset-076-5-2-4080433 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 31.03.2013 | All | Editorial changes in Test sequence table | CEDEX |
| Version 2.9.2 28.11.2013 | All | General review | CEDEX |
| Version 2.9.3 11.12.2013 | TC 13 | Correction of step 7 | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | Transmission and acceptance of reversing area information from BG or RBC |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS L2 : FS / LS / OS L3 : FS / LS / OS |  |  |
| Target of Test | The target of test is to check that: Reversing area information has been sent from BG or RBC to the on-board system and it is accepted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from BG or RBC containing reversing information has been RECORDED. Check on DMI that : The indication that reversing is permitted is DISPLAYED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 7 | FS / LS / OS |
| Radio communication session | ESTABLISHED | if L2 / L3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| MA Request Parameters | STORED | - |
| Mode Profile | STORED | IF OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | if L2 / L3 |
| RBC ID/Phone Number | STORED | if L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 138, packet 139 is received | I | BTM | If L1 | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If L1 | N/A | N/A | - |
| 4 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 138, packet 139 is received | I | RTM | If L2 / L3 | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If L2 / L3 | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving | I | INT | Train enters in reversing area | N/A | N/A | - |
| 7 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 | FS / LS / OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | If OS /LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | If L2 / L3 |
| RBC ID/Phone Number | STORED | If L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | Transmission of reversing area information from BG. The information is accepted because there is a transition level stored on-board. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN |  |  |
| Target of Test | The target of test is to check that: Reversing area information has been sent from BG to the on-board system and is accepted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from BG containing reversing information has been RECORDED. Check on DMI that : The indication that reversing is permitted is DISPLAYED (after changing to FS, LS or OS). |
| Constraints | A transition to L1: FS / LS / OS is stored on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | L1 |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC | UN/SN | Train is moving | I | INT | - | L0/LNTC | UN/SN | - |
| 2 | L0/LNTC | UN/SN | One Balise Group message containing packet 138, packet 139 is received | I | BTM | - | L0/LNTC | UN/SN | - |
| 3 | L0/LNTC | UN/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/LNTC | UN/SN | - |
| 4 | L0/LNTC | UN/SN | USE_FT5100100.15 | - | - | If L0: Transition to L1: FS / LS / OS | L0/LNTC | UN/SN | - |
| 5 | L0/LNTC | UN/SN | USE_FT5100100.23 | - | - | If LNTC: Transition to L1: FS / LS / OS | L1 | FS/LS/OS | - |
| 6 | L1 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L1 | FS/LS/OS | - |
| 7 | L1 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L1 | FS/LS/OS | - |
| 8 | L1 | FS/LS/OS | The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L1 | FS/LS/OS | - |
| 9 | L1 | FS/LS/OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L1 | FS/LS/OS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | IF OS / LS |
| ERTMS/ETCS level | 2 | L1 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | The onboard equipment receives reversing area information from RBC. The telegram is ACCEPTED, because a level 2/3 transition order is stored onboard. |  | 3 |
| Applicable Mode/Level Combinations | LNTC: UN / SB L0: SB / UN |  |  |
| Target of Test | The target of test is to check that: Reversing area information has been sent from RBC to the on-board system and it is accepted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-4.7.2 table - Output information Reversing allowed |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing reversing information has been RECORDED. Check on DMI that : The indication that reversing is permitted is DISPLAYED (after changing to FS, LS or OS). |
| Constraints | In SB mode, a cab needs to be active and valid Train Data are stored on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 / 3 | UN / SB / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Level 2/3 Announcement |
| ERTMS/ETCS level | 0 / 1 / 2 | L0/LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC | UN/SB/SN | Train is moving | I | INT | - | L0/LNTC | UN/SB/SN | - |
| 2 | L0/LNTC | UN/SB/SN | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 138, packet 139 is received | I | RTM | - | L0/LNTC | UN/SB/SN | - |
| 3 | L0/LNTC | UN/SB/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0/LNTC | UN/SB/SN | - |
| 4 | L0/LNTC | UN/SB/SN | USE _FT5100100.13,26 | - | - | If the current level is L0: (for changing to L2/L3) | L2/L3 | FS/LS/OS | - |
| 5 | L0/LNTC | UN/SB/SN | USE _FT5100100.21,20 | - | - | If the current level is LNTC: (for changing to L2/L3) | L2/L3 | FS/LS/OS | - |
| 6 | L2/L3 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L2/L3 | FS/LS/OS | - |
| 7 | L2/L4 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L2/L4 | FS/LS/OS | - |
| 8 | L2/L5 | FS/LS/OS | The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L2/L5 | FS/LS/OS | - |
| 9 | L2/L6 | FS/LS/OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L2/L6 | FS/LS/OS | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | If OS / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | The onboard equipment receives reversing information from a RBC. The telegram is ACCEPTED in the current mode. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SB / SR / PT |  |  |
| Target of Test | The target of test is to check that: Reversing information has been sent from a RBC to the on-board system. The information is accepted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing reversing information has been RECORDED. Check on DMI that : After changing to L2 / L3: SB, SR or PT the indication that reversing is permitted is DISPLAYED when the train is at standstill. |
| Constraints | If the testing mode is PT: the message 6 (Recognition of exit from TRIP mode) must have been received previously. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 / 6 / 2 / 8 | FS / LS / OS / SB / SR / PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | L2/L3 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS/LS/OS/SB/SR/PT | Train is moving | I | INT | - | L1 | FS/LS/OS/SB/SR/PT | - |
| 2 | L1 | FS/LS/OS/SB/SR/PT | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 138, packet 139 is received | I | RTM | - | L1 | FS/LS/OS/SB/SR/PT | - |
| 3 | L1 | FS/LS/OS/SB/SR/PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L1 | FS/LS/OS/SB/SR/PT | - |
| 4 | L1 | FS/LS/OS/SB/SR/PT | USE_5100100.37,36 | - | - | Transition to L2/L3 | L2/L3 | SB/SR/PT | - |
| 5 | L2/L3 | FS/LS/OS/SB/SR/PT | USE_FT5190600.1 [Cond. 31] USE_FT4060300.20,21,23,47,59,40,41,48,42,54, 46 [Cond. 31, 70, 10, 15, 40] USE_FT5190200.1,3,4,5 [Cond. 70, 72] USE_FT5190300.1,2,3 [Cond. 70, 72] USE_FT5090300.2,3 [Cond. 15] USE_FT5090200.1,3,4 [Cond. 40] | - | - | If L2/L3: SB,SR or PT: The on-board equipment changes to either FS, LS or OS mode | L2/L3 | SB/SR/PT | - |
| 6 | L2/L3 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L2/L3 | FS/LS/OS | - |
| 7 | L2/L3 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L2/L3 | FS/LS/OS | - |
| 8 | L2/L3 | FS/LS/OS | The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L2/L3 | FS/LS/OS | - |
| 9 | L2/L3 | FS/LS/OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L2/L3 | FS/LS/OS | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 | FS / LS / OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | If OS /LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | L2 / L3 |
| RBC ID/Phone Number | STORED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | Transmission of reversing area information via BG. The message is ACCEPTED because there is a level transition announcement to L1 stored on-board. |  | 5 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SB / SR / PT L3: FS / LS / OS / SB / SR / PT |  |  |
| Target of Test | The target of test is to check that : The on-board system receives the reversing area information from a BG and it is accepted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent via BG containing reversing information has been RECORDED. "Reversing permitted" is recorded after changing to L1. Check on DMI that: "Reversing permitted" is displayed after changing to L1. |
| Constraints | Before execution of test case the following preconditions have to be assured: A level transition announcement must have been received and accepted. Acknowledgement of train data must have been received and no emergency stop has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 / 6 / 2 / 8 | FS / LS / OS / SB / SR / PT |
| Level Transition Order | STORED | Transition to L1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS/LS/OS/SB/SR/PT | Train is moving | I | INT | - | L2/L3 | UN/SN | - |
| 2 | L2/L3 | FS/LS/OS/SB/SR/PT | One Balise Group message containing packet 138, packet 139 is received | I | BTM | - | L2/L3 | UN/SN | - |
| 3 | L2/L3 | FS/LS/OS/SB/SR/PT | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2/L3 | UN/SN | - |
| 4 | L2/L3 | FS/LS/OS/SB/SR/PT | USE_FT5100100.32,28 | - | - | Transition to L1 | L1 | FS/LS/OS | - |
| 5 | L1 | FS/LS/OS/SB/SR/PT | USE_FT5190600.1 [Cond. 31] USE_FT4060300.20,21,23,47,59,40,41,48,42,54, 46 [Cond. 31, 70, 10, 15, 40] USE_FT5190200.1,3,4,5 [Cond. 70, 72] USE_FT5190300.1,2,3 [Cond. 70, 72] USE_FT5090300.2,3 [Cond. 15] USE_FT5090200.1,3,4 [Cond. 40] | - | - | If L1: SB,SR or PT: The on-board equipment changes to either FS, LS or OS mode | L1 | FS/LS/OS | - |
| 6 | L1 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L1 | FS/LS/OS | - |
| 7 | L1 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L1 | FS/LS/OS | - |
| 8 | L1 | FS/LS/OS | The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L1 | FS/LS/OS | - |
| 9 | L1 | FS/LS/OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L1 | FS/LS/OS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 | FS / LS / OS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | Transmission and acceptance of reversing area from a BG or RTM. |  | 6 |
| Applicable Mode/Level Combinations | L1: SR / SB / PT L2: SR / SB / PT L3: SR / SB / PT |  |  |
| Target of Test | The target of test is to check that: Reversing area information has been sent from a BG or RTM to the on-board system and it is accepted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from BG or BTM containing reversing information has been RECORDED. The on-board equipment changes to mode FS, LS or OS Check on DMI that : The indication that reversing is permitted is DISPLAYED. |
| Constraints | In PT mode the information “Recognition of exit from TR mode” has been received In SB mode, a cab needs to be active and valid Train Data are stored on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 6 / 8 | SR / SB / PT |
| Radio communication session | ESTABISHED | If L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SR/SB/PT | Train is moving | I | INT | - | L1/L2/L3 | SR/SB/PT | - |
| 2 | L1/L2/L3 | SR/SB/PT | One Balise Group message (N_TOTAL=0) containing packet 138, packet 139 is received | I | BTM | If L1 | L1/L2/L3 | SR/SB/PT | - |
| 3 | L1/L2/L3 | SR/SB/PT | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If L1 | L1/L2/L3 | SR/SB/PT | - |
| 4 | L1/L2/L3 | SR/SB/PT | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 138, packet 139 is received | I | RTM | If L2 / L3 | L1/L2/L3 | SR/SB/PT | - |
| 5 | L1/L2/L3 | SR/SB/PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If L2 / L3 | L1/L2/L3 | SR/SB/PT | - |
| 6 | L1/L2/L3 | SR/SB/PT | USE_FT5190600.1 [Cond. 31] USE_FT4060300.20,21,23,47,59,40,41,48,42,54, 46 [Cond. 31, 70, 10, 15, 40] USE_FT5190200.1,3,4,5 [Cond. 70, 72] USE_FT5190300.1,2,3 [Cond. 70, 72] USE_FT5090300.2,3 [Cond. 15] USE_FT5090200.1,3,4 [Cond. 40] | - | - | If L1/L2/L3: SB,SR or PT: The on-board equipment changes to either FS, LS or OS mode | L1/L2/L3 | FS/LS/OS | - |
| 7 | L1/L2/L3 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L1/L2/L3 | FS/LS/OS | - |
| 8 | L1/L2/L3 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L1/L2/L3 | FS/LS/OS | - |
| 9 | L1/L2/L3 | FS/LS/OS | The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L1/L2/L3 | FS/LS/OS | - |
| 10 | L1/L2/L3 | FS/LS/OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L1/L2/L3 | FS/LS/OS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | Transmission and rejection of reversing area information from RBC because the Train Data are not valid |  | 7 |
| Applicable Mode/Level Combinations | L2 : SB L3 : SB |  |  |
| Target of Test | The target of test is to check that: Reversing area information has been sent from RBC to the on-board system and it is rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing reversing information has been RECORDED. Check on DMI that : The indication that reversing is permitted is NOT DISPLAYED. |
| Constraints | Train Data are not valid. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Data | NOT STORED | Not valid |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | SB | Train is at standstill (V_TRAIN=0) | I | INT | - | L2/L3 | SB | - |
| 2 | L2/L3 | SB | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 138, packet 139 is received | I | RTM | - | L2/L3 | SB | - |
| 3 | L2/L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2/L3 | SB | - |
| 4 | L2/L3 | SB | The driver enters and validates the "Fixed Train Data" | I | DMI | - | L2/L3 | SB | - |
| 5 | L2/L3 | SB | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | - | L2/L3 | SB | - |
| 6 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/L3 | SB | - |
| 7 | L2/L3 | SB | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2/L3 | SB | - |
| 8 | L2/L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2/L3 | SB | - |
| 9 | L2/L3 | SB | The driver selects "Start" | I | DMI | - | L2/L3 | SB | - |
| 10 | L2/L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2/L3 | SB | - |
| 11 | L2/L3 | SB | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | L2/L3 MA | L2/L3 | SB | - |
| 12 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/L3 | SB | - |
| 13 | L2/L3 | SB | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | - | L2/L3 | SB | - |
| 14 | L2/L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2/L3 | SB | - |
| 15 | L2/L3 | SB | USE_FT4060300.48 | - | - | Transition to FS | L2/L3 | FS | - |
| 16 | L2/L3 | FS | Train is moving | I | INT | Train enters in reversing area | L2/L3 | FS | - |
| 17 | L2/L3 | FS | Train is at standstill (V_TRAIN=0) | I | INT | - | L2/L3 | FS | - |
| 18 | L2/L3 | FS | NOT The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L2/L3 | FS | - |
| 19 | L2/L3 | FS | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L2/L3 | FS | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| Step 5: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 7: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| Step 11: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |

| Step 13: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Train Data | NOT STORED | Not valid |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | Transmission of reversing area information from BG. The information is rejected because there is not a transition level stored on-board. |  | 8 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN |  |  |
| Target of Test | The target of test is to check that: Reversing area information has been sent from BG to the on-board system and is rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from BG containing reversing information has been RECORDED. Check on DMI that : The indication that reversing is permitted is NOT DISPLAYED (after changing to FS, LS or OS). |
| Constraints | A transition to L1: FS / LS / OS is not stored on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Level Transition Order | NOT STORED | L1 |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC | UN/SN | Train is moving | I | INT | - | L0/LNTC | UN/SN | - |
| 2 | L0/LNTC | UN/SN | One Balise Group message (N_TOTAL=0) containing packet 138, packet 139 is received | I | BTM | - | L0/LNTC | UN/SN | - |
| 3 | L0/LNTC | UN/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/LNTC | UN/SN | - |
| 4 | L0/LNTC | UN/SN | USE_FT5100100.15 | - | - | If L0: Transition to L1: FS / LS / OS | L0/LNTC | UN/SN | - |
| 5 | L0/LNTC | UN/SN | USE_FT5100100.23 | - | - | If LNTC: Transition to L1: FS / LS / OS | L1 | FS/LS/OS | - |
| 6 | L1 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L1 | FS/LS/OS | - |
| 7 | L1 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L1 | FS/LS/OS | - |
| 8 | L1 | FS/LS/OS | NOT The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L1 | FS/LS/OS | - |
| 9 | L1 | FS/LS/OS | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L1 | FS/LS/OS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | IF OS / LS |
| ERTMS/ETCS level | 2 | L1 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | The onboard equipment receives reversing area information from RBC. The telegram is REJECTED, because a level 2/3 transition order is NOT stored onboard. |  | 9 |
| Applicable Mode/Level Combinations | LNTC: UN / SB L0: SB / UN |  |  |
| Target of Test | The target of test is to check that: Reversing area information has been sent from RBC to the on-board system and it is rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-4.7.2 table - Output information Reversing allowed |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing reversing information has been RECORDED. Check on DMI that : The indication that reversing is permitted is NOT DISPLAYED (after changing to FS, LS or OS). |
| Constraints | In SB mode, a cab needs to be active and valid Train Data are stored on-board. A transition to L2/3 is not stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 / 3 | UN / SB / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | Level 2/3 Announcement |
| ERTMS/ETCS level | 0 / 1 / 2 | L0/LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC | UN/SB/SN | Train is moving | I | INT | - | L0/LNTC | UN/SB/SN | - |
| 2 | L0/LNTC | UN/SB/SN | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 138, packet 139 is received | I | RTM | - | L0/LNTC | UN/SB/SN | - |
| 3 | L0/LNTC | UN/SB/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0/LNTC | UN/SB/SN | - |
| 4 | L0/LNTC | UN/SB/SN | USE _FT5100100.13,26 | - | - | If the current level is L0: (for changing to L2/L3) | L2/L3 | FS/LS/OS | - |
| 5 | L0/LNTC | UN/SB/SN | USE _FT5100100.21,20 | - | - | If the current level is LNTC: (for changing to L2/L3) | L2/L3 | FS/LS/OS | - |
| 6 | L2/L3 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L2/L3 | FS/LS/OS | - |
| 7 | L2/L3 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L2/L3 | FS/LS/OS | - |
| 8 | L2/L3 | FS/LS/OS | NOT The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L2/L3 | FS/LS/OS | - |
| 9 | L2/L3 | FS/LS/OS | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L2/L3 | FS/LS/OS | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | If OS / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | The onboard equipment receives reversing information from a RBC. The telegram is REJECTED because no transition order to L2/L3 is stored on board. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SB / SR / PT |  |  |
| Target of Test | The target of test is to check that: Reversing information has been sent from a RBC to the on-board system. The information is rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing reversing information has been RECORDED. Check on DMI that : After changing to L2 / L3: SB, SR or PT the indication that reversing is permitted is NOT DISPLAYED when the train is at standstill. |
| Constraints | A transition order to level L2/L3 is not stored on-board. If the testing mode is PT: the message 6 (Recognition of exit from TRIP mode) must have been received previously. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 / 6 / 2 / 8 | FS / LS / OS / SB / SR / PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | L2 / L3 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS/LS/OS/SB/SR/PT | Train is moving | I | INT | - | L1 | FS/LS/OS/SB/SR/PT | - |
| 2 | L1 | FS/LS/OS/SB/SR/PT | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 138, packet 139 is received | I | RTM | - | L1 | FS/LS/OS/SB/SR/PT | - |
| 3 | L1 | FS/LS/OS/SB/SR/PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L1 | FS/LS/OS/SB/SR/PT | - |
| 4 | L1 | FS/LS/OS/SB/SR/PT | USE_5100100.37,36 | - | - | Transition to L2/L3 | L2/L3 | SB/SR/PT | - |
| 5 | L2/L3 | FS/LS/OS/SB/SR/PT | USE_FT5190600.1 [Cond. 31] USE_FT4060300.20,21,23,47,59,40,41,48,42,54, 46 [Cond. 31, 70, 10, 15, 40] USE_FT5190200.1,3,4,5 [Cond. 70, 72] USE_FT5190300.1,2,3 [Cond. 70, 72] USE_FT5090300.2,3 [Cond. 15] USE_FT5090200.1,3,4 [Cond. 40] | - | - | If L2/L3: SB,SR or PT: The on-board equipment changes to either FS, LS or OS mode | L2/L3 | SB/SR/PT | - |
| 6 | L2/L3 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L2/L3 | FS/LS/OS | - |
| 7 | L2/L3 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L2/L3 | FS/LS/OS | - |
| 8 | L2/L3 | FS/LS/OS | NOT The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L2/L3 | FS/LS/OS | - |
| 9 | L2/L3 | FS/LS/OS | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L2/L3 | FS/LS/OS | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 | FS / LS / OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | If OS /LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | L2 / L3 |
| RBC ID/Phone Number | STORED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | Transmission of reversing area information via BG. The message is REJECTED because no transition order to L1 is stored on-board. |  | 11 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SB / SR / PT L3: FS / LS / OS / SB / SR / PT |  |  |
| Target of Test | The target of test is to check that : The on-board system receives the reversing area information from a BG and it is rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent via BG containing reversing information has been RECORDED. |
| Constraints | Before execution of test case the following preconditions have to be assured: No transition order to L1 is stored on-board. Acknowledgement of train data must have been received and no emergency stop has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 / 6 / 2 / 8 | FS / LS / OS / SB / SR / PT |
| Level Transition Order | NOT STORED | Transition to L1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS/LS/OS/SB/SR/PT | Train is moving | I | INT | - | L2/L3 | UN/SN | - |
| 2 | L2/L3 | FS/LS/OS/SB/SR/PT | One Balise Group message containing packet 138, packet 139 is received | I | BTM | - | L2/L3 | UN/SN | - |
| 3 | L2/L3 | FS/LS/OS/SB/SR/PT | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2/L3 | UN/SN | - |
| 4 | L2/L3 | FS/LS/OS/SB/SR/PT | USE_FT5100100.32,28 | - | - | Transition to L1 | L1 | FS/LS/OS | - |
| 5 | L1 | FS/LS/OS/SB/SR/PT | USE_FT5190600.1 [Cond. 31] USE_FT4060300.20,21,23,47,59,40,41,48,42,54, 46 [Cond. 31, 70, 10, 15, 40] USE_FT5190200.1,3,4,5 [Cond. 70, 72] USE_FT5190300.1,2,3 [Cond. 70, 72] USE_FT5090300.2,3 [Cond. 15] USE_FT5090200.1,3,4 [Cond. 40] | - | - | If L1: SB,SR or PT: The on-board equipment changes to either FS, LS or OS mode | L1 | FS/LS/OS | - |
| 6 | L1 | FS/LS/OS | Train is moving | I | INT | Train enters in reversing area | L1 | FS/LS/OS | - |
| 7 | L1 | FS/LS/OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L1 | FS/LS/OS | - |
| 8 | L1 | FS/LS/OS | NOT The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L1 | FS/LS/OS | - |
| 9 | L1 | FS/LS/OS | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L1 | FS/LS/OS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 | FS / LS / OS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Area Information |  | 4080433 |
| Test Case of Feature | Transmission and rejection of reversing area information from BG or RBC |  | 12 |
| Applicable Mode/Level Combinations | L1 : TR L2 : TR L3 : TR |  |  |
| Target of Test | The target of test is to check that: Reversing area information has been sent from BG or RBC to the on-board system and it is rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from BG or RBC containing reversing information has been RECORDED. Check on DMI that : The indication that reversing is permitted is NOT DISPLAYED. |
| Constraints | Reversing information is not stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | if L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | if L2 / L3 |
| RBC ID/Phone Number | STORED | if L2 / L3 |
| Reversing Area Information | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2 /L3 | TR | Train is moving | I | INT | - | L1/L2 /L3 | TR | - |
| 2 | L1/L2 /L3 | TR | One Balise Group message containing packet 138, packet 139 is received | I | BTM | If L1 | L1/L2 /L3 | TR | - |
| 3 | L1/L2 /L3 | TR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If L1 | L1/L2 /L3 | TR | - |
| 4 | L1/L2 /L3 | TR | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 138, packet 139 is received | I | RTM | If L2 / L3 | L1/L2 /L3 | TR | - |
| 5 | L1/L2 /L3 | TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If L2 / L3 | L1/L2 /L3 | TR | - |
| 6 | L1/L2 /L3 | TR | USE_FT5110200.13 | - | - | Transition from TR to PT | L1/L2 /L3 | PT | - |
| 7 | L1/L2 /L3 | PT | USE_FT4060300.20,21,23,47,59,42,54 [Cond. 31, 15] USE_FT5190600.1 [Cond. 31] USE_FT5190200.1-5 [Cond. 70] USE_FT5190300.1-3 [Cond. 70] USE_FT4060300.60-61 [Cond. 70] USE_FT5090300.2-3 [Cond. 15] | - | - | Transition from PT to FS, LS or OS | L1/L2 /L3 | FS/OS/LS | - |
| 8 | L1/L2 /L3 | FS/OS/LS | Train is moving | I | INT | Train enters in reversing area | L1/L2 /L3 | FS/OS/LS | - |
| 9 | L1/L2 /L3 | FS/OS/LS | Train is at standstill (V_TRAIN=0) | I | INT | - | L1/L2 /L3 | FS/OS/LS | - |
| 10 | L1/L2 /L3 | FS/OS/LS | NOT The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | L1/L2 /L3 | FS/OS/LS | - |
| 11 | L1/L2 /L3 | FS/OS/LS | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | L1/L2 /L3 | FS/OS/LS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 / 1 | FS / LS / OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | If L2 / L3 |
| RBC ID/Phone Number | STORED | If L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |