| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080446 (4.8.4: Accepted Information depending on the modes - Train running number) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-4080446 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 25.11.2013 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Train running number |  | 4080446 |
| Test Case of Feature | RBC modifies the actual train running number in different modes |  | 1 |
| Applicable Mode/Level Combinations | L2: SB/ FS/LS/SR/OS/TR/PT/RV/NL L3: SB/ FS/LS/SR/OS/TR/PT/RV/NL |  |  |
| Target of Test | The target of this test is to check that the train accepts a message from RBC which modifies the actual train running number |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.5.3 |  |
|  |  | Subset-026-3.18.4.5.5 |  |
|  |  | Subset-026-A.3.11 Train running number |  |
|  |  | Subset-026-4.8.3.1.1 Train Running number |  |
|  |  | Subset-026-4.8.4.2 Train Running number |  |
|  |  | Subset-026-4.8.4.2 [2] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | - The driver asks for the value of the Train Running Number on the DMI - The RBC sends the new Train Running Number by means of a RBC message (a general message with packet 140, including relevant variables NID_OPERATIONAL and Q_DIR). - Check the new NID_OPERATIONAL by means of the DMI |
| Constraints | - Q_DIR of packet 140 sent by RBC and stored in JRU has to be consistent with the LRBG variables Q_DIRLRBG, Q_DLRBG, Q_DIRTRAIN stored (e.g Q_DIRLRBG=nominal, Q_DLRBG=nominal ; Q_DIRTRAIN=nominal, and Q_DIR included in paket 140= nominal is one of the possible combinations valid for this test case). - A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 / 14 | SB / FS / OS / SR / TR / PT / NL / LS / RV |
| Position Report parameters | STORED | - |
| Train Position | KNOWN | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "Data view" | I | DMI | The driver acts on the DMI in order to see the current value of the Train Running Number. | N/A | N/A | - |
| 2 | N/A | N/A | The data from "Data View" window are displayed | O | DMI | The Train Running Number is displayed. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 140 is received | I | RTM | The range for the value of NID_OPERATIONAL will have no restriction, according to A.3.11 of the SRS. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24;NID_PACKET=140) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver presses "Data view" | I | DMI | The driver acts on the DMI in order to see the current value of the Train Running Number.The On-board Equipment accepted the new value of NID_OPERATIONAL | N/A | N/A | - |
| 6 | N/A | N/A | The data from "Data View" window are displayed | O | DMI | The Train Running Number is displayed. | N/A | N/A | - |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 140 | Train RUNNING NUMBER FROM RBC |
| Q_DIR | 2 | FINITE VALUE | Nominal or Reverse |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 / 14 | SB / FS / OS / SR / TR / PT / NL / LS / RV |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | The one sent by the RBC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Train data changed" is displayed | O | DMI | Check that the train running number is changed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Train running number |  | 4080446 |
| Test Case of Feature | RBC tries to modify the actual train running number in different modes and the train rejects the transmitted number |  | 2 |
| Applicable Mode/Level Combinations | L2: SH/PS/SL L3: SH/PS/SL L0: UN/SL L1: SH/PS/SL LNTC: SN/SL |  |  |
| Target of Test | The target of this test is to check that the train REJECTS a message from RBC which modifies the actual train running number to prove that the transmitted train running number has been rejected by On-board Equipment |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Train running number |  |
|  |  | Subset-026-4.8.4.2 Train Running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Method of Test: - The driver asks for the value of the Train Running Number on the DMI - The RBC sends the new Train Running Number by means of a RBC message (e.g. a general message with packet 140, including relevant variable NID_OPERATIONAL and Q_DIR). - Check the recorded MESSAGE in JRU - Check the there is no new Train Running Number on the DMI (except in SL mode) |
| Constraints | - Q_DIR of packet 140 sent by RBC and stored in JRU has to be consistent with the LRBG variables Q_DIRLRBG, Q_DLRBG, Q_DIRTRAIN stored (e.g Q_DIRLRBG=nominal, Q_DLRBG=nominal ; Q_DIRTRAIN=nominal, and Q_DIR included in paket 140= nominal is one of the possible combinations valid for this test case). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 5 / 12 / 13 / 15 | SH / UN / SL / LS / SN / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |
| Train Running Number | INVALID | Or UNKNOWN |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "Data view" | I | DMI | The driver acts on the DMI in order to see the current value of the Train Running Number. | N/A | N/A | - |
| 2 | N/A | N/A | The data from "Data View" window are displayed | O | DMI | The Train Running Number is displayed. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 140 is received | I | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | On-board Equipment rejects the train running number | - | - | The new NID_OPERATIONAL is not stored Onboard | N/A | N/A | - |
| 6 | N/A | N/A | The driver presses "Data view" | I | DMI | Driver requests the ETCS to see the train running number. This step cannot be tested in SL mode. | N/A | N/A | - |
| 7 | N/A | N/A | The data from "Data View" window are displayed | O | DMI | The train running number is displayed to the driver, which has not been modified since the message was rejected. This step cannot be tested in SL mode. | N/A | N/A | - |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 140 | Train RUNNING NUMBER FROM RBC |
| Q_DIR | 2 | 1 | nominal |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 5 / 12 / 13 / 15 | SH / UN / SL / LS / SN / PS |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |
| Train Running Number | INVALID | train running number remains unchanged after the transmission of the RBC MESSAGE, the same value of preceding conditions if INVALID, or UNKNOWN. |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The data from "Data View" window are displayed | - | DMI | Train data is DISPLAYED. Check that train running number remains unchanged after the transmission of the RBC MESSAGE |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |