| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080413 (4.8.4: Accepted Information depending on the modes - Exception [5]) Total: 8 Test case(s) |  |
| REF: | Subset-076-5-2-4080413 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 28.11.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.3 10.12.2013 | TCs 1, 2, 3, 4 | Editorial changes | CEDEX |
| Version 2.9.4 13.11.2013 | ALL | 2ND REVIEW | CEDEX |
| Version 2.9.5 15.11.2013 | ALL | 2ND REVIEW | CEDEX |
| Version 2.9.6 19.12.2013 | ALL | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Exception [5] |  | 4080413 |
| Test Case of Feature | The train receives an order via radio or balise to switch immediately from L2 to L0 at the current position. Since the on board equipment is in PT mode, the system does not switch to L0. |  | 1 |
| Applicable Mode/Level Combinations | L2 : PT |  |  |
| Target of Test | The target of this test is to check that: The system does not switch to level 0 when the order to switch immediately to Level 0 is received on board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [5] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0, D_LEVELTR = 0 or = 32767. New Level 0 is NOT RECORDED. |
| Constraints | The on-board must have received the information “Recognition of Exit from TR mode”. The train is moving backwards over a balise |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41: “Level Transition Order” (D_LEVELTR= 0 or = 32767; M_LEVELTR=0) is received | I | RTM | This radio message OR the balise message will be sent. An order to switch immediately to Level 0 is received from the RBC. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This radio message OR the balise message (below) will be recorded. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 41 is received | I | BTM | This balise message OR the radio message (above) will be sent. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This balise message OR the radio message (above) will be recorded. | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Level symbol "Level 0" is displayed | O | DMI | The On-board equipment does not switch to Level 0. | N/A | N/A | - |
| 6 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | or 32767 (Switch immediately) |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | or 32767 (Switch immediately) |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Exception [5] |  | 4080413 |
| Test Case of Feature | The train receives a conditional level transition order via BG to switch from L2 to L0 at the current position. Since the on board equipment is in PT mode, the system does not switch to L0. |  | 2 |
| Applicable Mode/Level Combinations | L2 : PT |  |  |
| Target of Test | The target of this test is to check that: The system does not switch to level 1 nor level 0 when a conditional level transition order to switch to Level 1 and Level 0 is received on board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [5] |  |
|  |  | Subset-026-7.4.2.11.2 Packet Number 46: Conditional Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 2 indication is DISPLAYED. Check on the JRU that: Packet Number 46: "Conditional Level Transition Order" is RECORDED with New Level 1 is NOT RECORDED. New Level 0 is NOT RECORDED. |
| Constraints | The on-board must have received the information “Recognition of Exit from TR mode”. The train is moving backwards over a balise. No level transitions can be stored onboard nor it is received in the same message (due to the requirement "4.8.3.1.1 Exception [11]") |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 46 ( M_LEVELTR=2; N_ITER=1; M_LEVELTR=0) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT the Level symbol "Level 1" is displayed | O | DMI | The On-board equipment does not switch to Level 1. | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT the Level symbol "Level 0" is displayed | O | DMI | The On-board equipment does not switch to Level 0. | N/A | N/A | - |
| 6 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 46 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | 2 | Transition to L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| N_ITER | 5 | 1 | - |
| M_LEVELTR(k) | 3 | 1 | Transition to L0 |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Exception [5] |  | 4080413 |
| Test Case of Feature | The train receives an order via radio or balise to switch immediately to NTC at the current position. Since the on board equipment is in PT mode, the system does not switch to NTC. |  | 3 |
| Applicable Mode/Level Combinations | L1,L2, L3 : PT |  |  |
| Target of Test | The target of this test is to check that: The system does not switch to level NTC when the order to switch immediately to Level NTC is received on board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [5] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level,1,2,or 3 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 1, D_LEVELTR = 0 or = 32767. New Level NTC is NOT RECORDED. |
| Constraints | The on-board must have received the information “Recognition of Exit from TR mode”. The train is moving backwards over a balise |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41: “Level Transition Order” (D_LEVELTR= 0 or = 32767; M_LEVELTR=1) is received | I | RTM | This radio message OR the balise message will be sent. An order to switch immediately to Level 1 is received from the RBC. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=1 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This radio message OR the balise message (below) will be recorded. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | This balise message OR the radio message (above) will be sent. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=1 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This balise message OR the radio message (above) will be recorded. | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Level symbol "Level NTC" is displayed | O | DMI | The On-board equipment does not switch to Level NTC. | N/A | N/A | - |
| 6 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit02=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 1 | Transition to level NTC |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 1 | Transition to level NTC |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Exception [5] |  | 4080413 |
| Test Case of Feature | The train receives an order via radio or balise to switch immediately from L3 to L2 at the current position. Since the on board equipment is in PT mode, the system does not switch to L2. |  | 4 |
| Applicable Mode/Level Combinations | L3 : PT |  |  |
| Target of Test | The target of this test is to check that: The system does not switch to level 2 when the order to switch immediately to Level 2 is received on board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [5] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 3 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3, D_LEVELTR = 0 or = 32767. New Level 2 is NOT RECORDED. |
| Constraints | The on-board must have received the information “Recognition of Exit from TR mode”. The train is moving backwards over a balise |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41: “Level Transition Order” (D_LEVELTR= 0 or = 32767; M_LEVELTR=3) is received | I | RTM | This radio message OR the balise message will be sent. An order to switch immediately to Level 2 is received from the RBC. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This radio message OR the balise message (below) will be recorded. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | This balise message OR the radio message (above) will be sent. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This balise message OR the radio message (above) will be recorded. | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Level symbol "Level 2" is displayed | O | DMI | The On-board equipment does not switch to Level 2. | N/A | N/A | - |
| 6 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 3 | Transition to level 2 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 3 | Transition to level 2 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Exception [5] |  | 4080413 |
| Test Case of Feature | The train receives an order via radio or balise to switch immediately from L2 to L1 at the current position. Since the on board equipment is in PT mode, the system does not switch to L1. |  | 5 |
| Applicable Mode/Level Combinations | L2 : PT |  |  |
| Target of Test | The target of this test is to check that: The system does not switch to level 1 when the order to switch immediately to Level 1 is received on board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [5] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 2, D_LEVELTR = 0 or = 32767. New Level 1 is NOT RECORDED. |
| Constraints | The on-board must have received the information “Recognition of Exit from TR mode”. The train is moving backwards over a balise |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41: “Level Transition Order” (D_LEVELTR= 0 or = 32767; M_LEVELTR=2) is received | I | RTM | This radio message OR the balise message will be sent. An order to switch immediately to Level 1 is received from the RBC. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=2 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This radio message OR the balise message (below) will be recorded. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | This balise message OR the radio message (above) will be sent. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=2 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This balise message OR the radio message (above) will be recorded. | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Level symbol "Level 1" is displayed | O | DMI | The On-board equipment does not switch to Level 1. | N/A | N/A | - |
| 6 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Exception [5] |  | 4080413 |
| Test Case of Feature | The train receives an order via radio or balise to switch immediately from L2 to L3 at the current position. Since the on board equipment is in PT mode, the system does not switch to L3. |  | 6 |
| Applicable Mode/Level Combinations | L2 : PT |  |  |
| Target of Test | The target of this test is to check that: The system does not switch to Level 3 when the order to switch immediately to Level 3 is received on board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [5] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 4, D_LEVELTR = 0 or = 32767. New Level 3 is NOT RECORDED. |
| Constraints | The on-board must have received the information “Recognition of Exit from TR mode”. The train is moving backwards over a balise |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41: “Level Transition Order” (D_LEVELTR= 0 or = 32767; M_LEVELTR=4) is received | I | RTM | This radio message OR the balise message will be sent. An order to switch immediately to Level 1 is received from the RBC. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=4 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This radio message OR the balise message (below) will be recorded. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | This balise message OR the radio message (above) will be sent. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=4 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This balise message OR the radio message (above) will be recorded. | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Level symbol "Level 3" is displayed | O | DMI | The On-board equipment does not switch to Level 3. | N/A | N/A | - |
| 6 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit05=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 4 | Transition to level 3 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 4 | Transition to level 3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Exception [5] |  | 4080413 |
| Test Case of Feature | The train receives an order via radio or balise to switch immediately from L3 to L0 at the current position. Since the on board equipment is in PT mode, the system does not switch to L0. |  | 7 |
| Applicable Mode/Level Combinations | L3 : PT |  |  |
| Target of Test | The target of this test is to check that: The system does not switch to level 0 when the order to switch immediately to Level 0 is received on board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [5] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0, D_LEVELTR = 0 or = 32767. New Level 0 is NOT RECORDED. |
| Constraints | The on-board must have received the information “Recognition of Exit from TR mode”. The train is moving backwards over a balise |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41: “Level Transition Order” (D_LEVELTR= 0 or = 32767; M_LEVELTR=0) is received | I | RTM | This radio message OR the balise message will be sent. An order to switch immediately to Level 1 is received from the RBC. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This radio message OR the balise message (below) will be recorded. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | This balise message OR the radio message (above) will be sent. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This balise message OR the radio message (above) will be recorded. | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Level symbol "Level 0" is displayed | O | DMI | The On-board equipment does not switch to Level 0. | N/A | N/A | - |
| 6 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Exception [5] |  | 4080413 |
| Test Case of Feature | The train receives an order via radio or balise to switch immediately from L3 to L1 at the current position. Since the on board equipment is in PT mode, the system does not switch to L1. |  | 8 |
| Applicable Mode/Level Combinations | L3 : PT |  |  |
| Target of Test | The target of this test is to check that: The system does not switch to level 1 when the order to switch immediately to Level 1 is received on board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [5] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 3 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 2, D_LEVELTR = 0 or = 32767. New Level 1 is NOT RECORDED. |
| Constraints | The on-board must have received the information “Recognition of Exit from TR mode”. The train is moving backwards over a balise |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41: “Level Transition Order” (D_LEVELTR= 0 or = 32767; M_LEVELTR=2) is received | I | RTM | This radio message OR the balise message will be sent. An order to switch immediately to Level 1 is received from the RBC. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=2 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This radio message OR the balise message (below) will be recorded. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | This balise message OR the radio message (above) will be sent. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=2 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This balise message OR the radio message (above) will be recorded. | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Level symbol "Level 1" is displayed | O | DMI | The On-board equipment does not switch to Level 1. | N/A | N/A | - |
| 6 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |