| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3140100 (3.14.1: Brake Command Handling) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-3140100 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 16.09.2013 | All | review | DLR |
| Version 2.9.2 04.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.14.1: Brake Command Handling |  | 3140100 |
| Test Case of Feature | Distance allowed for moving backwards in PT overpassed. |  | 1 |
| Applicable Mode/Level Combinations | L1: PT L2: PT L3: PT |  |  |
| Target of Test | The target of this test case is to check that : If the brake command was triggered due to an overpassed distance allowed for moving backwards in Post Trip mode, the brake command shall be released at standstill and after driver acknowledgement. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.7.4 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn for Post Trip distance exceeded |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • The service brake is RELEASED. Check on the DMI that: • the driver is requested for acknowledgement • after the driver acknowledges, the service brake symbol is switched off Check on the JRU that : • the driver acknowledgement is RECORDED • the brake release is RECORDED |
| Constraints | The brake command was triggered due to an overpassed distance allowed for moving backwards in Post Trip mode. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake commanded | O | TIU | Service brake is applied |
| The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | Service brake symbol is displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | The train reaches standstill | N/A | N/A | - |
| 2 | N/A | N/A | The Driver Request symbol "Acknowledgement" is displayed | O | DMI | Request for acknowledgement of service brake release is DISPLAYED. | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges service brake release request. | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0000) is recorded | O | JRU | Driver acknowledgement is RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | Service brake not commanded | O | TIU | The service brake is RELEASED | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Release of service brake is RECORDED | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The service brake indication is REMOVED | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | Service brake is RELEASED |
| The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | Service brake symbol is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.14.1: Brake Command Handling |  | 3140100 |
| Test Case of Feature | Distance allowed for moving backwards in PT overpassed. |  | 2 |
| Applicable Mode/Level Combinations | L1: PT L2: PT L3: PT |  |  |
| Target of Test | The target of this test case is to check that : If the brake command was triggered due to any further movement in the direction opposite to the train orientation while the distance allowed for moving backwards in Post Trip mode is still overpassed, the brake command shall be released at standstill and after driver acknowledgement. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.7.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TUI that: • The service brake is RELEASED. Check on the DMI that: • the driver is requested for acknowledgement • after the driver acknowledges, the service brake symbol is switched off Check on the JRU that : • the driver acknowledgement is RECORDED • the brake release is RECORDED |
| Constraints | The train moves backwards. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake commanded | O | TIU | Service brake is applied |
| The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | Service brake symbol is displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train movement exceeds the national/default value for the backwards movement. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The Service brake is APPLIED. | O | TIU | The brake command is triggered at Reverse Movement Protection after a distance specified by the national/default (default value = 200m) Current mode is UNCHANGED | N/A | N/A | - |
| 3 | N/A | N/A | The brake indication is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The brake intervention is RECORDED. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | The train reaches standstill | N/A | N/A | - |
| 6 | N/A | N/A | The Driver Request symbol "Acknowledgement" is displayed | O | DMI | Request for acknowledgement of service brake release is DISPLAYED. | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges service brake release request. | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0000) is recorded | O | JRU | Driver acknowledgement is RECORDED | N/A | N/A | - |
| 9 | N/A | N/A | Service brake not commanded | O | TIU | The service brake is RELEASED | N/A | N/A | - |
| 10 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Release of service brake is RECORDED | N/A | N/A | - |
| 11 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The service brake indication is REMOVED | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | Service brake is RELEASED |
| The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | Service brake symbol is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.14.1: Brake Command Handling |  | 3140100 |
| Test Case of Feature | Train movement during modifying/revalidating of train data. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SB, SN L1: FS, LS, OS, SR, SB L2: FS, LS, OS, SR, SB L3: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of this test is to check that: - During modifying/revalidating of train data a train movement is detected (the “while entering SR speed/distance limits” part of requirement 3.14.1.7.3 is tested in TC4) - The brakes are applied - The brake command shall be released at standstill and after driver acknowledgement |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.7.3 |  |
|  |  | Subset-026-3.18.3.3.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • Brake indication is DISPLAYED • Request to ACK service brake release is DISPLAYED (when train is at standstill) • Brake indication is REMOVED Check on the TIU that: • Brake is APPLIED. • Brake is RELEASED Check on the JRU that : • The Brake intervention is RECORDED. • Information displayed to the driver is RECORDED. • Driver’s acknowledgement is RECORDED • Release of Brake is RECORDED |
| Constraints | Before execution of test case the following preconditions have to be satisfied: • There is a complete set of Train Data stored on-board.• The Train status is after SoM • The Train movement happens while Train Data is modified/revalidated. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| On-board equipment detects a change of input information | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects ‘Train Data Entry’. | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0100) is recorded | O | JRU | Driver acknowledgement is RECORDED | N/A | N/A | - |
| 4 | N/A | N/A | The Train data window is displayed | O | DMI | A complete set of Train Data is displayed to the driver. | N/A | N/A | - |
| 5 | N/A | N/A | The driver is allowed to modify or simply revalidate the existing set of Train Data. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | Train starts moving while the driver is entering the new Train Data | N/A | N/A | - |
| 7 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | The Brake intervention is RECORDED. M_BRAKE_COMMAND_STATE=1 (Command) or =0 (Release) | N/A | N/A | - |
| 9 | N/A | N/A | The Main Window is displayed | O | DMI | The buttons of the Main Window including Train data are disabled until driver acknowledgment. | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | The Brake intervention is DISPLAYED. | N/A | N/A | - |
| 11 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | Train reaches standstill. | N/A | N/A | - |
| 13 | N/A | N/A | The Driver Request symbol "Acknowledgement" is displayed | O | DMI | Request for acknowledgement of service brake release is DISPLAYED. | N/A | N/A | - |
| 14 | N/A | N/A | The driver acknowledges service brake release request. | I | DMI | - | N/A | N/A | - |
| 15 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0000) is recorded | O | JRU | Driver acknowledgement is RECORDED | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | Service brake is RELEASED. | N/A | N/A | - |
| 17 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Brake release is RECORDED. | N/A | N/A | - |
| 18 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | Brake indication is REMOVED. | N/A | N/A | - |
| 19 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | The Main Window is displayed | O | DMI | In L1, L0 or L NTC, the DMI allows the driver to select again the option ‘Train Data Entry’ or the option ‘START’. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Train Data | STORED | New Train Data, if Train Data was modified. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.14.1: Brake Command Handling |  | 3140100 |
| Test Case of Feature | Train movement while entering SR speed/distance limits. |  | 4 |
| Applicable Mode/Level Combinations | L1: SR L2: SR L3: SR |  |  |
| Target of Test | The target of this test is to check that: - While entering SR speed/distance limits, a train movement is detected (the “while modifying/revalidating train data” part of requirement 3.14.1.7.3 is tested in TC4) - The brakes are applied - The brake command shall be released at standstill and after driver acknowledgement |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.8 |  |
|  |  | Subset-026-3.14.1.7.3 |  |
|  |  | Subset-026-4.4.11.1.5.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • Brake indication is DISPLAYED • Brake indication is REMOVED Check on the TIU that: • Brake is APPLIED. • Brake is RELEASED Check on the JRU that : • The Brake intervention is RECORDED. • Information displayed to the driver is RECORDED. • Driver’s acknowledgement is RECORDED • Release of Brake is RECORDED |
| Constraints | Before execution of test case the following preconditions have to be satisfied: • The Train movement happens while entering SR speed/distance limits. • The value for D_NVSTFF should be different from the default value (infinite) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | D_NVSTFF=FINITE VALUE |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Staff Responsible" is displayed and The speed and distance monitoring information is displayed | O | DMI | SR indication is DISPLAYED Target distance is AVAILABLE |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The SR speed / distance window is displayed | I | DMI | The driver is entering new SR speed/distance limits. The values are entered respecting the applicable unit, range and resolution. | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | Train starts moving while the driver is entering the new SR speed/distance limits | N/A | N/A | - |
| 3 | N/A | N/A | The "Special” Window is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | The Brake intervention is DISPLAYED. | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The Driver Request symbol "Acknowledgement of brake intervention" is displayed. | O | DMI | Request for acknowledgement of service brake release is DISPLAYED. | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges service brake release request. | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0000) is recorded | O | JRU | Driver acknowledgement is RECORDED | N/A | N/A | - |
| 9 | N/A | N/A | Service brake not commanded | O | TIU | The service brake is RELEASED | N/A | N/A | - |
| 10 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Release of service brake is RECORDED | N/A | N/A | - |
| 11 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The service brake indication is REMOVED | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |