| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3080500 (3.8.5: MA Update and Extension) Total: 14 Test case(s) |  |
| REF: | Subset-076-5-2-3080500 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 23.10.2013 | All | review | DLR |
| Version 2.9.2 06.12.2013 | All | review | DLR |
| Version 2.9.3 19.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has NOT reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from Balise Group BGa. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority from the Balise Group BGa and it extends the one already stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • The previous MA is extended. Is not displayed in LS. Check on the JRU that : • Balise Group BGa Message, with extended MA, is RECORDED. |
| Constraints | The second MA shall extend the one already stored on-board. The train receives the second MA before it reaches the Indication Limit of first MA. When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. This is the way to check on the DMI that the MA has been updated. The aim of this Test Case is not to test mode changes. The onboard equipment may have Linking information stored or not, but in case linking information is stored, Balise Group BGa shall be included. If test is run in LS, the permitted speed shall be lower than both the Limited Supervision mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | MA has been received previously. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target speed and target distance is displayed | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 5 (), packet 12 (), packet 21 (), packet 27 (), packet 255 is received | I | BTM | Other packets can be included. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The new Movement Authority is DISPLAYED. | O | DMI | When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. There is no speed and distance monitoring information in LS. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | Extended MA has been received. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has reached the SB intervention of previous Movement Authority. It receives an extended Movement Authority from Balise Group BGa. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority from the Balise Group BGa and it extends the one already stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • SB symbol is REMOVED. • The new Permitted Speed of MA is DISPLAYED. • There is no speed and distance monitoring information in LS. Check on the JRU that : • Balise Group BGa Message, with extended MA, is RECORDED. |
| Constraints | The second MA shall extend the one already stored on-board. The train receives the second MA after it reaches the SB intervention of first MA. The second MA is long enough, so the train has not reached the new Indication Limit. The aim of this Test Case is not to test mode changes. The onboard equipment may have Linking information stored or not, but in case linking information is stored, Balise Group BGa shall be included. If test is run in LS, the permitted speed shall be lower than both the Limited Supervision mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | MA has been received previously. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target speed and target distance is displayed An indication for braking is DISPLAYED. | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message () containing packet 5 (), packet 12 (), packet 21 (), packet 27 (), packet 255 () is received | I | BTM | If in FS mode, Packet 80: “Mode Profile” is not included. Other packets can be included. | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The new MA is displayed. | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 9 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The permitted speed of the new MA is displayed. The extended MA is long enough, so the train has not reached the new Indication Limit. There is no speed and distance monitoring information in LS. | N/A | N/A | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | Extended MA has been received. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has not reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from RBC. |  | 3 |
| Applicable Mode/Level Combinations | L2: FS, LS L3: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority from the Balise Group BGa and it extends the one already stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • There is no change on DMI: the previous MA is extended. There is no speed and distance monitoring information in LS. Check on the JRU that : • Message 3: “Movement Authority”, with extended MA, is RECORDED. |
| Constraints | The second MA shall extend the one already stored on-board. The train receives the second MA before it reaches the Indication Limit of first MA. When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. This is the way to check on the DMI that the MA has been updated. The aim of this Test Case is not to test mode changes. If test is run in LS, the permitted speed shall be lower than both the Limited Supervision mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | MA has been received previously. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target speed and target distance is displayed | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 5, packet 21 and packet 27 is received | I | RTM | If in FS mode, Packet 80: “Mode Profile” is not included. Other packets can be included. | N/A | N/A | - |
| 2 | N/A | N/A | The new Movement Authority is DISPLAYED. | O | DMI | When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 3 | N/A | N/A | Message 3: “Movement Authority” is RECORDED. | O | JRU | Extended MA is RECORDED | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Extended MA has been received. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from RBC. |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, LS L3: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority from RBC and it extends the one already stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • Indication for braking is REMOVED. • The new Permitted Speed of MA is DISPLAYED. • There is no speed and distance monitoring information in LS. Check on the JRU that : • Message 3: “Movement Authority”, with extended MA is RECORDED. |
| Constraints | The second MA shall extend the one already stored on-board. The train receives the second MA after it reaches the Indication Limit of first MA. The second MA is long enough, so the train has not reach the new Indication Limit. The aim of this Test Case is not to test mode changes. If test is run in LS, the permitted speed shall be lower than both the Limited Supervision mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | MA has been received previously. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target speed and target distance is displayed An indication for braking is DISPLAYED. | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Indication for braking is Displayed. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 5, packet 21, packet 27 is received | I | RTM | If in FS mode, Packet 80: “Mode Profile” is not included. Other packets can be included. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The new Movement Authority is DISPLAYED. | O | DMI | There is no change on DMI: the previous MA is extended. There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 7 | N/A | N/A | Indication for braking is REMOVED. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 9 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The Permitted Speed of new MA is DISPLAYED. | O | DMI | There is no speed and distance monitoring information in LS. | N/A | N/A | - |

| Step 4: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Extended MA has been received. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has not reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from infill Balise Group. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority from an infill Balise Group BGa when the indication limit has not been already reached, then • it extends the MA stored on-board by starting a new MA section at the infill location reference, i.e. the balise group at the next main signal |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 b) |  |
|  |  | Subset-026-3.8.5.1.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • The previous MA is extended. • There is no speed and distance monitoring information in LS. Check on the JRU that : • infill Balise Group BGa telegram, with extended MA, is RECORDED. |
| Constraints | The on-board equipment receives new Movement Authority from an infill Balise Group BGa and it extends the MA stored on-board by starting a new MA section at the infill location reference. The train receives the second MA before it reaches the Indication Limit of first MA. When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. This is the way to check on the DMI that the MA has been updated. If test is run in LS, the permitted speed shall be lower than both the Limited Supervision mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Main Balise Group BGb are included. |
| Movement Authority | STORED | MA has been received previously. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target speed and target distance is displayed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 136 (), packet 5 (), packet 12 (), packet 21 (), packet 27 (), packet 255 () is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The new Movement Authority is DISPLAYED. | O | DMI | The MA stored on-board is extended by starting a new MA section at the infill location reference, i.e. the balise group at the next main signal. When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. There is no speed and distance monitoring information in LS. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Main Balise Group BGb is included. |
| Movement Authority | STORED | Extended MA has been received. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | The MA stored on-board is extended by starting a new MA section at the infill location reference, i.e. the balise group at the next main signal |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from an infill Balise Group. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority from an infill Balise Group BGa when the train has reached the indication limit, then • it extends the MA stored on-board by starting a new MA section at the infill location reference, i.e. the balise group at the next main signal |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 b) |  |
|  |  | Subset-026-3.8.5.1.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • Indication Limit information is REMOVED. • The new Permitted Speed of MA is DISPLAYED. • There is no speed and distance monitoring information in LS. Check on the JRU that : • infill Balise Group BGa telegram, with extended MA, is RECORDED. |
| Constraints | The on-board equipment receives new Movement Authority from an infill Balise Group BGa and it extends the MA stored on-board by starting a new MA section at the infill location reference, i.e. the balise group at the next main signal. The train receives the second MA after it reaches the Indication Limit of first MA. If test is run in LS, the permitted speed shall be lower than both the Limited Supervision mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Main Balise Group BGb are included. |
| Movement Authority | STORED | MA has been received previously. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Indication Limit information is displayed. Target speed and target distance is displayed. | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 4 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 136 (), packet 5 (), packet 12 (), packet 21 (), packet 27 (), packet 255 () is received | I | BTM | - | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The new Movement Authority is DISPLAYED. | O | DMI | The MA stored on-board is extended by starting a new MA section at the infill location reference, i.e. the balise group at the next main signal There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / LS | 0 / 12 |
| Linking | STORED | Main Balise Group BGb is included. |
| Movement Authority | STORED | Extended MA has been received. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has NOT reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority with a OS mode profile from Balise Group BGa. |  | 7 |
| Applicable Mode/Level Combinations | L1: OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority and a new mode profile from the Balise Group BGa and it extends the one already stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • There is no change on DMI (only target distance is extended): the previous MA is extended. Check on the JRU that : • Balise Group BGa Message, with extended MA, is RECORDED. • New target distance of the new MA is RECORDED |
| Constraints | The second MA shall extend the one already stored on-board. The train receives the second MA before it reaches the Indication Limit of first MA. When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. This is the way to check on the DMI that the MA has been updated. The aim of this Test Case is not to test mode changes. The onboard equipment may have Linking information stored or not, but in case linking information is stored, Balise Group BGa shall be included. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | MA has been received previously. |
| Mode Profile | STORED | For OS mode |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The old Movement Authority is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 5 (), packet 12 (), packet 21 (), packet 27 (), packet 80 (), packet 255 is received | I | BTM | Packet 80: “Mode Profile” is included to continue in OS. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | New target distance is recorded in the JRU | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Since train is running in OS mode this information is available | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Request to show supervision limits (0001 1011) | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The previous MA is extended. (Only the target distance is extended). When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | Extended MA has been received. |
| Mode Profile | STORED | For OS mode |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from Balise Group BGa. |  | 8 |
| Applicable Mode/Level Combinations | L1: OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority and a Mode Profile from the Balise Group BGa and it extends the one already stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • Indication for braking is REMOVED. • The new Permitted Speed of MA is DISPLAYED. • The mode does not change. Check on the JRU that : • Balise Group BGa Message, with extended MA, is RECORDED. • New target distance, permitted and target speed (those that change) are RECORDED |
| Constraints | The second MA shall extend the one already stored on-board. The train receives the second MA after it reaches the Indication Limit of first MA. The second MA is long enough, so the train has not reached the new Indication Limit. The aim of this Test Case is not to test mode changes. The onboard equipment may have Linking information stored or not, but in case linking information is stored, Balise Group BGa shall be included. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | MA has been received previously. |
| Mode Profile | STORED | For OS mode |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The old Movement Authority is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Indication for braking is Displayed. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 5 (), packet 12 (), packet 21 (), packet 27 (), packet 80 (), packet 255 is received | I | BTM | - | N/A | N/A | - |
| 5 | N/A | N/A | Balise Group BGa Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | New permitted speed, target speed and target distance are recorded in the JRU | O | JRU | When changed | N/A | N/A | - |
| 7 | N/A | N/A | Indication for braking is REMOVED. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 9 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver requests the permitted speed, target speed and target distance | I | DMI | Since train is running in OS mode this information is available. The extended MA is long enough, so the train has not reached the new Indication Limit | N/A | N/A | - |
| 11 | N/A | N/A | Driver request is RECORDED | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The information about new target and permitted speed as well as target distance is shown to the driver | O | DMI | - | N/A | N/A | - |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | Extended MA has been received. |
| Mode Profile | STORED | For OS mode |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has NOT reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from RBC. |  | 9 |
| Applicable Mode/Level Combinations | L2: OS L3: OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority with mode profile from RBC and it extends the one already stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • There is no change on DMI: the previous MA is extended. (Only the target distance is updated) Check on the JRU that : • Message 3: “Movement Authority” and packet 80 “Mode Profile”, with extended MA, is RECORDED. • Message 146: “Acknowledgement” is RECORDED. (Only if in previous message M_ACK = 1). • New target distance of the new MA is RECORDED |
| Constraints | The second MA shall extend the one already stored on-board. The train receives the second MA before it reaches the Indication Limit of first MA. When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. This is the way to check on the DMI that the MA has been updated. The aim of this Test Case is not to test mode changes. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | MA has been received previously. |
| Mode Profile | STORED | For OS mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The old Movement Authority is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 5, packet 21, packet 27, packet 80 is received | I | RTM | Packet 80: “Mode Profile” is included to continue in OS. Other packets can be included. | N/A | N/A | - |
| 2 | N/A | N/A | Message 3: “Movement Authority” is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | New target distance is recorded in the JRU | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests indication of the permitted speed, target speed and target distance | I | DMI | Since train is running in OS mode this information is available | N/A | N/A | - |
| 5 | N/A | N/A | Driver request is RECORDED | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The information about new target and permitted speed as well as target distance is shown to the driver | O | DMI | The previous MA is extended. (Only the target distance is extended). When the train reaches the Indication Limit of the previous MA, the indication for braking is not displayed, since the MA has been extended. | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Extended MA has been received. |
| Mode Profile | STORED | For OS mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | The train has reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from RBC. |  | 10 |
| Applicable Mode/Level Combinations | L2: OS L3: OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment receives new Movement Authority with mode profile from RBC and it extends the one already stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • Indication for braking is REMOVED. • The new Permitted Speed of MA is DISPLAYED. Check on the JRU that : • Message 3: “Movement Authority” and packet number 80 “Mode Profile”, with extended MA is RECORDED. • Message 146: “Acknowledgement” is RECORDED. (Only if in previous message M_ACK = 1). • New target distance, permitted and target speed (if changed) are RECORDED |
| Constraints | The second MA shall extend the one already stored on-board. The train receives the second MA after it reaches the Indication Limit of first MA. The second MA is long enough, so the train has not reach the new Indication Limit. The aim of this Test Case is not to test mode changes. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | MA has been received previously. |
| Mode Profile | STORED | For OS mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The old Movement Authority is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Indication for braking is Displayed. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 5, packet 21, packet 27, packet 80 is received | I | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | Message 3: “Movement Authority” and packet number 80 “Mode Profile” is RECORDED. | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | New permitted speed, target speed and target distance are recorded in the JRU | O | JRU | When changed | N/A | N/A | - |
| 7 | N/A | N/A | Indication for braking is REMOVED. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 9 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver requests the permitted speed, target speed and target distance | I | DMI | Since train is running in OS mode this information is available. The extended MA is long enough, so the train has not reached the new Indication Limit | N/A | N/A | - |
| 11 | N/A | N/A | Driver request is RECORDED | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The information about new target and permitted speed as well as target distance is shown to the driver | O | DMI | - | N/A | N/A | - |

| Step 4: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Extended MA has been received. |
| Mode Profile | STORED | For OS mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new Movement Authority is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | Verify that Supervision data is reset/deleted/unchanged when MA with LOA is replaced by a new MA that defines a SvL. |  | 11 |
| Applicable Mode/Level Combinations | L1: FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to check that: • A new MA is received, which defines a SvL. • The old MA is replaced by the new one and • the track information beyond the new SvL will be deleted / reset as described in A3.4.1.2 b. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1.4 |  |
|  |  | Subset-026-A.3.4.1.3 table - Movement Authority |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • New speed and distance monitoring information is DISPLAYED (except in LS) Check on the JRU that : • BG message is RECORDED |
| Constraints | Initially the on-board is supervising an LOA, i.e. the initial MA defines a :LOA higher than 0. The onboard equipment may have Linking information stored or not, but in case linking information is stored, Balise Group BGa shall be included. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Movement Authority | STORED | MA with LOA higher than 0 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Speed and distance monitoring information is DISPLAYED | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | No indication for target monitoring | N/A | N/A | - |
| 2 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message () containing packet 12 (V_LOA), packet 255 () is received | I | BTM | The new MA has a an SvL with e.g. V_LOA = 0 with optional packets 21 and 27. The part from the new LoA to the EoA of the previous MA should be deleted. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Target monitoring information is displayed to the EoA location. See table end conditions (internal states) below for details. Since they are internal variables, not all of them are reflected in the DMI. There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | 0 | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Not yet applicable National Values | DELETED | beyond the new SvL |
| Linking | DELETED | beyond the new SvL |
| Movement Authority | DELETED | beyond the new SvL |
| Gradient Profile | DELETED | beyond the new SvL |
| International SSP | DELETED | beyond the new SvL |
| Axle load speed profile | DELETED | beyond the new SvL |
| List of Balises for SH area | DELETED | beyond the new SvL |
| Signalling related Speed Restriction | DELETED | beyond the new SvL |
| Route Suitability Data | DELETED | beyond the new SvL |
| Plain Text Information (location based) | DELETED | - |
| Fixed Text Information (location based) | DELETED | - |
| Mode Profile | DELETED | beyond the new SvL |
| RBC Transition Order | DELETED | beyond the new SvL |
| Radio Infill Area information | DELETED | beyond the new SvL |
| Track conditions excluding big metal masses | RESET TO INITIAL VALUE | - |
| Track condition big metal masses | RESET TO INITIAL VALUE | - |
| ERTMS/ETCS level | 2 | L1 |
| Reversing Area Information | DELETED | beyond the new SvL |
| Permitted Braking Distance Information | DELETED | beyond the new SvL |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| New speed and distance monitoring information is DISPLAYED | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | Verify that Supervision data is reset/deleted/unchanged when MA with LOA is replaced by a new MA that defines a SvL. |  | 12 |
| Applicable Mode/Level Combinations | L2: FS, LS, SR, OS L3: FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to check that: • A new MA is received, which defines a SvL. • The old MA is replaced by the new oneand • the track information beyond the new SvL will be deleted / reset as described in A3.4.1.2 b. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.1.4 |  |
|  |  | Subset-026-3.8.5.2.4 |  |
|  |  | Subset-026-A.3.4.1.3 table - Movement Authority |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • New speed and distance monitoring information is DISPLAYED (except in LS) Check on the JRU that : • RBC message is RECORDED |
| Constraints | Initially the on-board is supervising an LOA, i.e. the initial MA defines a :LOA greater than 0 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | MA with LOA higher than 0 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Speed and distance monitoring information is DISPLAYED | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | No indication for target monitoring There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 2 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (V_LOA=0) is received | I | RTM | The new MA has a an SvL with e.g. V_LOA = 0 with optional packets 21 and 27. The part from the new LoA to the EoA of the previous MA should be deleted. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Target monitoring information is displayed to the EoA location See table end conditions (internal states) below for details. Since they are internal variables, not all of them are reflected in the DMI There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | 0 | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | DELETED | beyond the new SvL |
| Movement Authority | DELETED | beyond the new SvL |
| Gradient Profile | DELETED | beyond the new SvL |
| International SSP | DELETED | beyond the new SvL |
| Axle load speed profile | DELETED | beyond the new SvL |
| List of Balises for SH area | DELETED | beyond the new SvL |
| Signalling related Speed Restriction | DELETED | beyond the new SvL |
| Route Suitability Data | DELETED | beyond the new SvL |
| Plain Text Information (location based) | DELETED | - |
| Fixed Text Information (location based) | DELETED | - |
| Mode Profile | DELETED | beyond the new SvL |
| RBC Transition Order | DELETED | beyond the new SvL |
| Radio Infill Area information | DELETED | beyond the new SvL |
| Track conditions excluding big metal masses | DELETED | - |
| Track condition big metal masses | DELETED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Reversing Area Information | DELETED | beyond the new SvL |
| Permitted Braking Distance Information | DELETED | beyond the new SvL |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| New speed and distance monitoring information is DISPLAYED | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | Verify that the reception of a new MA with an LOA is not considered an MA shortening |  | 13 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • A new MA with an LOA is received • The old MA is replaced by the new one and • Since this is NOT considered an MA shortening, the track information beyond the new LOA will NOT be deleted / reset as described in A3.4.1.2 b. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.2.4 |  |
|  |  | Subset-026-A.3.4.1.3 table - Movement Authority |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • New speed and distance monitoring information is DISPLAYED (except in LS) Check on the JRU that : • RBC message is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT AVAILABLE | - |
| Movement Authority | STORED | MA with LOA higher than 0 |
| Gradient Profile | STORED | D_GRADIENT must be much longer than the MA stored |
| International SSP | STORED | D_STATIC must be much longer than the MA stored |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Speed and distance monitoring information is DISPLAYED | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 2 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (V_LOA>0) is received | I | RTM | The new MA has a an SvL with e.g. V_LOA > 0. The part from the new LoA to the EoA of the previous MA should be deleted. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The target distance monitoring parameters shall not change. There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 is received | I | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The new MA has a Endsection which is longer then all previously received MA and reaches within the area of the GP and SSP stored within Staring conditions. The MA will only be accepted if the profiles have not been shortened | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | >0 | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 7: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Contains data of the new MA only |
| Gradient Profile | UNCHANGED | - |
| International SSP | UNCHANGED | - |
| Axle load speed profile | UNCHANGED | - |
| List of Balises for SH area | UNCHANGED | - |
| Signalling related Speed Restriction | UNCHANGED | - |
| Route Suitability Data | UNCHANGED | - |
| Plain Text Information (location based) | UNCHANGED | - |
| Fixed Text Information (location based) | UNCHANGED | - |
| Mode Profile | UNCHANGED | - |
| RBC Transition Order | UNCHANGED | - |
| Radio Infill Area information | UNCHANGED | - |
| Track conditions excluding big metal masses | UNCHANGED | - |
| Track condition big metal masses | UNCHANGED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Reversing Area Information | UNCHANGED | - |
| Permitted Braking Distance Information | UNCHANGED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| New speed and distance monitoring information is DISPLAYED | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.5: MA Update and Extension |  | 3080500 |
| Test Case of Feature | Verify that the reception of repositioning information is not considered an MA shortening |  | 14 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • Repositioning information is received • The old MA is update accordingly • Since this is NOT considered an MA shortening, the track information beyond the new LOA will NOT be deleted / reset as described in A3.4.1.2 b. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.5.2.2 |  |
|  |  | Subset-026-3.8.5.2.4 |  |
|  |  | Subset-026-3.4.4.4.2.1 |  |
|  |  | Subset-026-3.4.4.4.2.1 a) |  |
|  |  | Subset-026-3.4.4.4.2.1 b) |  |
|  |  | Subset-026-3.4.4.4.2.1 c) |  |
|  |  | Subset-026-A.3.4.1.3 table - Movement Authority |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The displayed target distance is updated according to the received repositioning information. (except in LS) Check on the JRU that : BG message is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | NID_BG=16383 Q_LINKORIENTATION=1 Linking information has announced a following balise group as unknown but containing repositioning information. |
| Movement Authority | STORED | MA with LOA higher than 0 |
| Gradient Profile | STORED | D_GRADIENT must be much longer than the MA stored |
| International SSP | STORED | D_STATIC must be much longer than the MA stored |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The target distance is DISPLAYED | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 2 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing) containing packet 5, packet 16 (Q_DIR=1), packet 255 is received | I | BTM | The train overpasses the BG in nominal direction | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The target distance monitoring parameters shall not change. There is no speed and distance monitoring information in LS. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing) containing packet 12 (Q_DIR=1), packet 255 is received | I | BTM | The train overpasses the BG in nominal direction | N/A | N/A | - |
| 8 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The new MA has a Endsection which is longer then all previously received MA and reaches within the area of the GP and SSP stored within Staring conditions. The MA will only be accepted if the profiles have not been shortened. | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 16 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| L_SECTION | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Not yet applicable National Values | UNCHANGED | - |
| Linking | UNCHANGED | - |
| Movement Authority | STORED | Contains data of the new MA only |
| Gradient Profile | UNCHANGED | - |
| International SSP | UNCHANGED | - |
| Axle load speed profile | UNCHANGED | - |
| List of Balises for SH area | UNCHANGED | - |
| Signalling related Speed Restriction | UNCHANGED | - |
| Route Suitability Data | UNCHANGED | - |
| Plain Text Information (location based) | UNCHANGED | - |
| Fixed Text Information (location based) | UNCHANGED | - |
| Mode Profile | UNCHANGED | - |
| RBC Transition Order | UNCHANGED | - |
| Radio Infill Area information | UNCHANGED | - |
| Track conditions excluding big metal masses | UNCHANGED | - |
| Track condition big metal masses | UNCHANGED | - |
| ERTMS/ETCS level | 2 | L1 |
| Reversing Area Information | UNCHANGED | - |
| Permitted Braking Distance Information | UNCHANGED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| New speed and distance monitoring information is DISPLAYED | O | DMI | There is no speed and distance monitoring information in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |