| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080443 (4.8.4: Accepted Information depending on the modes - Track Ahead Free up to level 2/3 transition location) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-4080443 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 20.11.2013 | All | Review | CEDEX |
| Version 2.9.3 17.12.2013 | All | Review | CEDEX |
| Version 2.9.4 23.12.2013 | All | Editorial | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free up to level 2/3 transition location |  | 4080443 |
| Test Case of Feature | The train receives a message including Packet 90 (Track Ahead Free up to level 2/3 transition location) via balise group. The message is ACCEPTED, because a level 2/3 transition is stored onboard. |  | 1 |
| Applicable Mode/Level Combinations | L0: SB, UN, TR LNTC: SB, SN, TR L1: SB, FS, LS, OS, SR, TR, PT |  |  |
| Target of Test | The target of this test is to check that: The message including packet 90 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Track Ahead Free up to level 2/3 transition location |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free up to level 2/3 transition location |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-3.8.2.7.1 |  |
|  |  | Subset-026-3.8.2.7.2 |  |
|  |  | Subset-026-3.8.2.8 e) |  |
|  |  | Subset-026-7.4.2.26.2 Packet Number 90: track ahead free up to level 2/3 transition location |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |
|  |  | Subset-026-7.4.3.4.2 Packet Number 9: Level 2/3 transition information |  |
|  |  | Subset-026-4.5.2.1 table - Request MA on reception of "track ahead free up to the level 2/3 transition location" (only level 0,1,NTC) |  |
|  |  | Subset-026-8.4.4.4.4 |  |
|  |  | Subset-026-8.4.4.4.4 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The use of the information is checked by means of the function "Request MA on reception of "track ahead free up to the level 2/3 transition location" (only level 0,1,NTC)". Check on the JRU that: - The packet 90 “Track Ahead free up to level 2/3 transition location ' is recorded. - The message to the RBC 132 "MA Request" is recorded. Check on the RTM that: - A message 132 "MA Request" is sent to the RBC |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Level Transition Order | 3 / 4 | level 2 / 3 transition order |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 90 is received | I | BTM | Complete message is accepted | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=90) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132; Q_MARQSTREASON=<Bit5=1>; NID_PACKET=9) is transmitted | O | RTM | Variable Q_MARQREASON must indicate that the delivery of the MA Request is due to the reception of the information "Track Ahead Free up to level 2/3 transition" | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 90 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Level 2/3 transition location balise group. |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 1xxxx | Bit5=1 to indicate the MA Request is due to the reception of information "Track Ahead Free up to level 2/3 transition" |
| NID_PACKET | 8 | 9 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_LTRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Level Transition Order | 3 / 4 | level 2 / 3 transition order |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free up to level 2/3 transition location |  | 4080443 |
| Test Case of Feature | The train receives a message including Packet 90 (Track Ahead Free up to level 2/3 transition location) via balise group. The message is REJECTED, because a level 2/3 transition is NOT stored onboard. |  | 2 |
| Applicable Mode/Level Combinations | L0: SB, UN, TR LNTC: SB, SN, TR L1: SB, FS, LS, OS, SR, TR, PT |  |  |
| Target of Test | The target of this test is to check that: The message including packet 90 is rejected on-board because of 4.8.3.1.1 Exception [9] |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Track Ahead Free up to level 2/3 transition location |  |
|  |  | Subset-026-4.8.3.1.1 Exception [9] |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free up to level 2/3 transition location |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.26.2 Packet Number 90: track ahead free up to level 2/3 transition location |  |
|  |  | Subset-026-3.8.2.7.1 |  |
|  |  | Subset-026-3.8.2.7.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The use of the information is checked by means of the function "Request MA on reception of "track ahead free up to the level 2/3 transition location" (only level 0,1,NTC)". Check on the JRU that: - The packet 90 “Track Ahead free up to level 2/3 transition location ' is recorded. - The message to the RBC 132 "MA Request" is NOT recorded. Check on the RTM that: - A message 132 "MA Request" is NOT sent to the RBC |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 90 is received | I | BTM | Complete message is rejected | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=90) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132; Q_MARQSTREASON=<Bit5=1>) is transmitted | O | RTM | Variable Q_MARQREASON must not indicate that the delivery of the MA Request is due to the reception of the information "Track Ahead Free up to level 2/3 transition" | N/A | N/A | - |
| 4 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 90 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Level 2/3 transition location balise group. |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free up to level 2/3 transition location |  | 4080443 |
| Test Case of Feature | The train receives a message including Packet 90 (Track Ahead Free up to level 2/3 transition location) via balise group. The message is REJECTED because the onboard equipment is already in L2/3. |  | 3 |
| Applicable Mode/Level Combinations | L2/L3: FS, OS, SR, SB, TR, PT, LS |  |  |
| Target of Test | The target of this test is to check that The message including packet 90 is rejected, because of the ERTMS/ETCS level stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Track Ahead Free up to level 2/3 transition location |  |
|  |  | Subset-026-7.4.2.26.2 Packet Number 90: track ahead free up to level 2/3 transition location |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The use of the information is checked by means of the function "Request MA on reception of "track ahead free up to the level 2/3 transition location" (only level 0,1,NTC)". Check on the JRU that: - The packet 90 “Track Ahead free up to level 2/3 transition location ' is recorded. - The message to the RBC 132 "MA Request" is NOT recorded. Check on the RTM that: - A message 132 "MA Request" is NOT sent to the RBC |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 | FS / OS / SR / SB / TR / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 90 is received | I | BTM | Complete message is rejected | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=90) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132; Q_MARQSTREASON=<Bit5=1>) is transmitted | O | RTM | Variable Q_MARQREASON must not indicate that the delivery of the MA Request is due to the reception of the information "Track Ahead Free up to level 2/3 transition" | N/A | N/A | - |
| 4 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 90 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Level 2/3 transition location balise group. |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 | FS / OS / SR / SB / TR / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free up to level 2/3 transition location |  | 4080443 |
| Test Case of Feature | The train receives a message including Packet 90 (Track Ahead Free up to level 2/3 transition location) via balise group. The message is REJECTED because the onboard equipment is not in the appropiate mode. |  | 4 |
| Applicable Mode/Level Combinations | L0: SH, PS, SL, NL LNTC: SH, PS, SL, NL L1: SH, PS, SL, NL, RV |  |  |
| Target of Test | The target of this test is to check that The message including packet 90 is rejected, because of the ERTMS/ETCS mode stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Track Ahead Free up to level 2/3 transition location |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free up to level 2/3 transition location |  |
|  |  | Subset-026-7.4.2.26.2 Packet Number 90: track ahead free up to level 2/3 transition location |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 90 (Track Ahead Free up to level 2/3 transition location) is recorded. Check on the RTM: The message 132 “MA request” is NOT sent including packet 9 “Level 2/3 transition information” |
| Constraints | A radio session is established. A level 2/3 transition order is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 11 / 14 / 15 | SH / SL / NL / RV / PS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | 3 / 4 | level 2 / 3 transition order |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 90 is received | I | BTM | Complete message is rejected | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=90) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132; Q_MARQSTREASON=<Bit5=1>) is transmitted | O | RTM | Variable Q_MARQREASON must not indicate that the delivery of the MA Request is due to the reception of the information "Track Ahead Free up to level 2/3 transition" | N/A | N/A | - |
| 4 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 90 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Level 2/3 transition location balise group. |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 11 / 14 / 15 | SH / SL / NL / RV / PS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | 3 / 4 | level 2 / 3 transition order |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |