| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3180400 (3.18.4 Additional Data) Total: 10 Test case(s) |  |
| REF: | Subset-076-5-2-3180400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, Answer to Review001 | MULTITEL |
| Version 2.9.2 10.02.7655 | TC6, TC7 | Modifications requested by issues 1-3, 6, 8 from Review001_Subset076-5-2-3180400_v291 | MULTITEL |
| Version 2.9.3 13.12.2013 | TC3-4, TC7 | Modifications requested by 4-5 and 7 from Review001_Subset076-5-2-3180400_v291 | MULTITEL |
| Version 2.9.4 20.12.2013 | TC7 | Add the requirement "7.4.2.36 Packet Number 140: Train running number from RBC" | MULTITEL |
| Version 2.9.5 09.01.2014 | TC6 | Modifications requested by issue 1 of Review001_Subset076-5-2-3180400_v294 | MULTITEL |
| Version 2.9.6 22.01.2014 | TC8 | Apply of issue 1 from Review001_Subset076-5-2-3180400_v295.doc | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | After SoM, the Driver manually changes level to L2/3 and Rbc Identity + Phone Number Status is Invalid. The on-board offers the possibility to the driver to select the RBC contact information by the same means as for Start of Mission. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB, NL L2: FS, LS, OS, SR, SB, NL L3: FS, LS, OS, SR, SB, NL |  |  |
| Target of Test | The target of this test is to check that, after a Start of Mission: On-board Equipment requests the driver to select the RBC contact information after a manual level change to L2/3 by the driver if Rbc Identity + Phone Number Status is Invalid |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.3.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU: DRIVER enters and validates level 2 / 3, The on-board requests the driver to select the RBC contact information. |
| Constraints | Train is at standstill Driver ID is valid RBC IDENTITY + PHONE NUMBER STATUS is INVALID |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | VALID | Entered after the desk is open |
| RBC ID/Phone Number | INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | DMI allows the driver to enter/re-validate the level | O | DMI | Level 2 / 3 indication is displayed because it is included in the level priority table | N/A | N/A | - |
| 2 | N/A | N/A | The driver confirms "Level" | I | DMI | DRIVER enters and validates level 2 / 3 | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Driver action is RECORDED. | N/A | N/A | - |
| 4 | N/A | N/A | The Level symbol "Level 2/3" is displayed | O | DMI | The new level manually selected by the driver is DISPLAYED | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | The new current level is RECORDED. | N/A | N/A | - |
| 6 | N/A | N/A | The “RBC contact window” is displayed with “Use short number”, “Enter RBC data” and “Radio network ID” enabled | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | INVALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | After SoM, the Driver manually changes level to L2/3 and No Mobile Terminal is registered to a Radio Network. The on-board offers the possibility to the driver to select the RBC contact information by the same means as for Start of Mission. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB, NL L2: FS, LS, OS, SR, SB, NL L3: FS, LS, OS, SR, SB, NL |  |  |
| Target of Test | The target of this test is to check that, after a Start of Mission: On-board Equipment requests the driver to select the RBC contact information after a manual level change to L2/3 by the driver if No Mobile Terminal is registered to a Radio Network |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.3.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU: DRIVER enters and validates level 2 / 3, The on-board requests the driver to select the RBC contact information. |
| Constraints | Train is at standstill Driver ID is valid No Mobile Terminal is registered to a Radio Network |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | VALID | Entered after the desk is open |
| Radio Network ID | INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | DMI allows the driver to enter/re-validate the level | O | DMI | Level 2 / 3 indication is displayed because it is included in the level priority table | N/A | N/A | - |
| 2 | N/A | N/A | The driver confirms "Level" | I | DMI | DRIVER enters and validates level 2 / 3 | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Driver action is RECORDED. | N/A | N/A | - |
| 4 | N/A | N/A | The Level symbol "Level 2/3" is displayed | O | DMI | The new level manually selected by the driver is DISPLAYED | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | The new current level is RECORDED. | N/A | N/A | - |
| 6 | N/A | N/A | The “RBC contact window” is displayed with “Use short number”, “Enter RBC data” and “Radio network ID” enabled | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | INVALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | Entry / modification and validation of a set of train data for trains without radio connection to RBC |  | 3 |
| Applicable Mode/Level Combinations | L0: NL LNTC: NL L1: NL |  |  |
| Target of Test | The target of this test case is to check that the driver is allowed to modify the train running number. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.5.2 |  |
|  |  | Subset-026-3.18.4.5.5 |  |
|  |  | Subset-026-A.3.11 Train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the driver can select ‘Train Data Entry’, . the driver is allowed to modify the train running number Check on the JRU that : The driver‘s selection action is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the Train Data is UNKNOWN or INVALID i.e each train data may be either unknown or stored in the On-Board equipment; in that case it has to be validated by the driver. The Desk is OPEN |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Train Data | UNKNOWN / INVALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No train data displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 | NL | The driver can select ‘Train running number’ on 'Main Window" | O | DMI | Starting conditions | L0/LNTC/L1 | NL | - |
| 2 | L0/LNTC/L1 | NL | The driver selects ‘Train running number’ | I | DMI | - | L0/LNTC/L1 | NL | - |
| 3 | L0/LNTC/L1 | NL | The “Train running number” is displayed | O | DMI | - | L0/LNTC/L1 | NL | - |
| 4 | L0/LNTC/L1 | NL | The driver enters the "Train Running Number" | I | DMI | The driver is allowed to modify the train running number | L0/LNTC/L1 | NL | - |
| 5 | L0/LNTC/L1 | NL | The driver confirms the "Train Running Number" | I | DMI | The driver validates the new train running number | L0/LNTC/L1 | NL | - |
| 6 | L0/LNTC/L1 | NL | ADDITIONAL DATA (NID_MESSAGE_JRU=24) is recorded | O | JRU | New data is RECORDED | L0/LNTC/L1 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train data entry is enabled | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | Entry / modification and validation of set of train data for trains with radio connection (it means session opened) to RBC |  | 4 |
| Applicable Mode/Level Combinations | L2: NL L3: NL |  |  |
| Target of Test | The target of this test case is to check that the driver is allowed to modify the train running number. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.5.2 |  |
|  |  | Subset-026-3.18.4.5.5 |  |
|  |  | Subset-026-A.3.11 Train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the driver can select ‘Train Data Entry’, . the driver is allowed to modified the train running number Check on the JRU that : The driver‘s selection action is recorded The modified Train Running Number is recorded upon validation by the driver. |
| Constraints | In this test case, the driver is expected to modify the train running number |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3/4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| THE TRAIN DATA REQUEST IS ALLOWED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | NL | The driver can select ‘Train running number’ on 'Main Window" | O | DMI | Starting conditions | L2/L3 | NL | - |
| 2 | L2/L3 | NL | The driver selects ‘Train running number’ | I | DMI | - | L2/L3 | NL | - |
| 3 | L2/L3 | NL | The “Train running number” is displayed | O | DMI | The current set of Train Data is displayed in the window. | L2/L3 | NL | - |
| 4 | L2/L3 | NL | The driver enters the "Train Running Number" | I | DMI | The driver is allowed to modify the train running number | L2/L3 | NL | - |
| 5 | L2/L3 | NL | The driver confirms the "Train Running Number" | I | DMI | The driver validates the modified train running number . | L2/L3 | NL | - |
| 6 | L2/L3 | NL | ADDITIONAL DATA (NID_MESSAGE_JRU=24) is recorded | O | JRU | Driver’s validation action is recorded on the JRU. The Train Running Number entered by the driver is recorded | L2/L3 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | session opened |
| Train Data | VALID | Train running number is new |
| ERTMS/ETCS level | 3/4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train data entry is enabled | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | It is possible to enter driver ID in a non-leading engine. |  | 5 |
| Applicable Mode/Level Combinations | L0: NL LNTC: NL L1: NL L2: NL L3: NL |  |  |
| Target of Test | The target of this test case is to check that it is possible to enter driver ID in a non-leading engine. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.1.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The on-board equipment allows the driver to modify the Driver Id. Check on the JRU that : The modified driver ID is recorded and is identical to the data entered by the driver. |
| Constraints | The train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | DRIVER_ID=< DRIVER_ID _0> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The "Driver ID" button is enabled | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver enters the "Driver ID" | I | DMI | Entered Driver ID: DRIVER_ID_1 (different from DRIVER_ID_0) | N/A | N/A | - |
| 4 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=<DRIVER_ID_1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | DRIVER_ID=< DRIVER_ID _1> CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train is at standstill | I | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | If the driver selects “Use of EIRENE short number” to contact the RBC and the communication session is successfully established, the ERTMS/ETCS on-board equipment shall store as valid RBC identity and telephone number, the RBC identity reported by EURORADIO and the EIRENE short number, respectively. |  | 6 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, PT, NL L3: FS, LS, OS, SR, SB, PT, NL |  |  |
| Target of Test | The target of this test case is to check that the ERTMS/ETCS on-board equipment stores as valid RBC identity and telephone number, the RBC identity reported by EURORADIO and the EIRENE short number, respectively. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.3.4 |  |
|  |  | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 d) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The RBC identity and telephone number are the RBC identity reported by EURORADIO and the EIRENE short number, respectively. Check on the RTM that: Message 136 “Train Position Report” including packet 5 “Train Running Number” is sent Check on the JRU that : Message 136 “Train Position Report” including packet 5 “Train Running Number” is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 11 / 12 | FS / OS / SR / SB / PT / NL / LS |
| Radio communication session | ESTABLISHED | At least one Mobile Terminal registered to a Radio Network |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | invalid / unknown | - |
| Train Running Number | VALID | The train running number has been provided by the driver |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects "Level" | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver selects "Level 2" or “Level 3” | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The RBC Contact Window is displayed | O | DMI | Because no valid RBC contact info is stored on-board | N/A | N/A | - |
| 6 | N/A | N/A | The driver selects "Use short number" | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | Main window with all buttons disabled. The hour glass symbol ST05 is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) is recorded | O | JRU | MASK=0000000000020000000000 | N/A | N/A | - |
| 9 | N/A | N/A | The on-board tries to set up a connection with the short phone number defined by EIRENE | O | RTM | According to EURORADIO specifications This attempt shall be repeated until successful or a defined number of times | N/A | N/A | - |
| 10 | N/A | N/A | Setting up of a safe connection is successful. | I | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | Message 155: “Initiation of a communication session“ is RECORDED. | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions are compatible. | N/A | N/A | - |
| 14 | N/A | N/A | Message 32: “RBC/RIU System Version Configuration Determination” is RECORDED. | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | Communication session established for on-board. | N/A | N/A | - |
| 16 | N/A | N/A | Message 159: “Session established” is RECORDED. | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0, packet 5 is transmitted | O | RTM | Packet 1 must be used instead of Packet 0 if position report is based on 2 BG | N/A | N/A | - |
| 18 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136; NID_PACKET=0; NID_PACKET=5) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | Main Window is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 20 | N/A | N/A | The driver selects "Level" | I | DMI | - | N/A | N/A | - |
| 21 | N/A | N/A | The driver selects "Level 2" or “Level 3” | I | DMI | - | N/A | N/A | - |
| 22 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 23 | N/A | N/A | Main window with all buttons disabled. The hour glass symbol ST05 is displayed | O | DMI | The main Windows is displayed instead of the RBC Contact Window. This proofs that the on-board has stored as valid RBC identity and telephone number, the RBC identity reported by EURORADIO and the EIRENE short number, respectively. | N/A | N/A | - |
| 24 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) is recorded | O | JRU | MASK=0000000000020000000000 | N/A | N/A | - |

| Step 11: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 13: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 15: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 17: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 5 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 11 / 12 | FS / OS / SR / SB / PT / NL / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Main window with all buttons disabled. The hour glass symbol ST05 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | It shall be possible to change the train running number while running from the RBC |  | 7 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / SB / NL L3: FS / LS / OS / SR / SB / NL |  |  |
| Target of Test | The target of this test case is to check that : The RBC can modify the Train Running number when the train is running Note: The test case can be used in SB mode but outside the scope of the Start of mission procedure |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.5.3 |  |
|  |  | Subset-026-3.18.4.5.4.1 |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.36 Packet Number 140: Train running number from RBC |  |
|  |  | Subset-026-4.7.2 table - Input information Train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Train Running is NID_OPERATIONAL_1 as received in packet 140 (step 1) Check on the JRU that : The radio message 3 with the included packets received from the RBC is recorded. Driver’s selection of ” Train Running Number” is recorded |
| Constraints | In this test case, the driver is expected to attempt to modify the Train Running Number. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_0> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Default Window is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 140 is received | I | RTM | Entered Train Running Number: NID_OPERATIONAL_1 (different from NID_OPERATIONAL_0) The on-board equipment has received the radio message 24 “General Message” with packet 140 “Train running number from RBC” | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 5 is transmitted | O | RTM | Check that the on-board does NOT sent a message 136 “position report” with packet 5. This is to test req. 3.18.4.5.4.1 | N/A | N/A | - |
| 4 | N/A | N/A | The driver selects “Main” | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The Main window is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver selects “Train Running Number” | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | The Train Running Number Window is displayed | O | DMI | The Train Running is now NID_OPERATIONAL_1 received in packet 140 (step 1) | N/A | N/A | - |
| 8 | N/A | N/A | The driver closes the " Train Running Number " window | I | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | The Main window is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver closes the "Main" window | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | The Default window is displayed | O | DMI | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 140 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | NID_OPERATIONAL_1 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 / 13 | FS / OS / SR / UN / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_1> |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Train Running Number Window is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | The on-board equipment, when the train is NOT at standstill, allows the driver to modify the train running number and the Driver ID (if allowed by national value). The driver modifies the train running number. |  | 8 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / NL L3: FS / LS / OS / SR / NL |  |  |
| Target of Test | The target of this test case is to check that : When the train is running, the driver is permitted to modify the train running number and the Driver ID (if allowed by national value). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.1.2 |  |
|  |  | Subset-026-3.18.4.5.3 |  |
|  |  | Subset-026-3.18.4.5.4 |  |
|  |  | Subset-026-3.18.4.5.5 |  |
|  |  | Subset-026-A.3.11 Train running number |  |
|  |  | Subset-026-4.7.2 table - Input information Train running number |  |
|  |  | Subset-026-7.4.3.4.1 Packet Number 5: Train running number |  |
|  |  | Subset-026-8.4.4.4.2 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Train Running Number is now NID_OPERATIONAL_1 entered by the driver Check on the JRU that : Message 136 including packet 5 is RECORDED |
| Constraints | The current Train Running Number has been provided by the driver. The system version number is not X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 / 13 | FS / OS / SR / UN / NL / LS / SN |
| National Values | STORED / NOT STORED | If STORED: M_NVDERUN=1 (Change of driver ID permitted while running) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_0> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train is moving. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The "Driver ID" button is enabled | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The "Train running number" button is enabled | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver presses “Train Running Number” | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver enters the "Train Running Number" [MAIN] | I | DMI | Entered Train Running Number: NID_OPERATIONAL_1 (different from NID_OPERATIONAL_0) | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 5 is transmitted | O | RTM | Message 136 “Train Position Report” including packet 5 “Train Running Number” is sent NID_OPERATIONAL in packet 5 is equal to NID_OPERATIONAL_1 entered by the driver in step 7. | N/A | N/A | - |
| 8 | N/A | N/A | Message 136 “Train Position Report” including packet 5 “Train Running Number” is RECORDED | O | JRU | - | N/A | N/A | - |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | NID_OPERATIONAL_1 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 / 13 | FS / OS / SR / UN / NL / LS / SN |
| National Values | STORED / NOT STORED | If STORED: M_NVDERUN=1 (Change of driver ID permitted while running) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_1> |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | If, following a change of National Values, the update of the adhesion factor is no more permitted to the driver, the adhesion factor previously modified by the driver to slippery rail shall immediately be reset to non slippery rail. |  | 9 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that : - a message including new national values is received (Q_NVDRIVER_ADHES = 0, update of the adhesion factor is no more permitted to the driver); - the driver hasn’t the possibility to modify the track adhesion factor. - the track adhesion factor is reset to non slippery rail - the driver is informed |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.6.2.1 |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : - the adhesion factor is reset from slippery to non slippery - the driver is not permitted to change the adhesion factor - the driver is informed Check on the JRU that - the national values are received |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| National Values | STORED | Q_NVDRIVER_ADHES = 1; The driver is allowed to modify the adhesion factor. |
| Adhesion factor | VALID | Adhesion factor = 100 % (Default value) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 and L3 |
| NOT RELEVANT | - | TIU | - |
| Default Window is Displayed | O | DMI | The Status symbol "Adhesion factor - slippery rail" is not displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects “Special” | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The Special window is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver selects “Adhesion” | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The <Adhesion> window is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver modifies "Adhesion Factor" | I | DMI | The driver selects Slippery rail | N/A | N/A | - |
| 7 | N/A | N/A | The driver confirms "Adhesion Factor" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | ADDITIONAL DATA (NID_MESSAGE_JRU=24; M_ADHESION = 0) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | ST_02 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The Special window is displayed | O | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | The driver closes the "Special" window | I | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | The Default window is displayed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 15 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 3 (Q_NVDRIVER_ADHES=0) is received. | I | BTM | Only for L1. Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 16 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; N_TOTAL=1; Q_NVDRIVER_ADHES=0) is recorded | O | JRU | Only for L1. Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) with Packet 3 (Q_NVDRIVER_ADHES = 0) is received. | I | RTM | Only for L2 /L3 Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 18 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24; NID_PACKET=3; Q_NVDRIVER_ADHES = 0) is recorded | O | JRU | Only for L2 /L3 Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 19 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is removed | O | DMI | ST_02 removed The onboard resets the adhesion factor from slippery to not slippery and the driver should be informed. | N/A | N/A | - |
| 20 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | The driver is not allowed to modify the adhesion factor. Use_FT9990200.43 | - | - | - | N/A | N/A | - |

| Step 15: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | 000 | - |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C will be the same than the one stored on-board |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | D_NVOVTRP will be different from the one currently being used |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | 0 | The driver is not allowed to modify Adhesion Factor |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 15: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| T_TRAIN | 1 | FINITE VALUE | On-board train time |
| NID_LRBG | 10+14 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C will be the same than the one stored on-board |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | D_NVOVTRP will be different from the one currently being used |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | 0 | The driver is not allowed to modify Adhesion Factor |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| National Values | STORED | Q_NVDRIVER_ADHES = 0; The driver is not allowed to modify the adhesion factor. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Default Window is displayed | O | DMI | The Status symbol "Adhesion factor - slippery rail" is not displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | Any trackside adhesion profile is not affected, when the driver update is reset due to reception of new national values with variable Q_NVDRIVER_ADHES set to 0. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that : - Any trackside adhesion profiles are not affected due to reset of the driver update of adhesion profile. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.6.2.1 |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - the adhesion factor is not reset to 100% - the driver is not permitted to change the adhesion factor Check on the JRU that - the national values are received |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| National Values | STORED | Q_NVDRIVER_ADHES = 1; The driver is allowed to modify the adhesion factor. |
| Adhesion factor | VALID | Adhesion factor = 100 % (Default value) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 and L3 |
| NOT RELEVANT | - | TIU | - |
| The Track Adhesion factor is Displayed | O | DMI | Default value = 100% |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is stopped (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The <Adhesion> window is displayed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver modifies "Adhesion Factor" | I | DMI | The driver selects Slippery rail | N/A | N/A | - |
| 4 | N/A | N/A | The driver confirms "Adhesion Factor" | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | ADDITIONAL DATA (NID_MESSAGE_JRU=24; M_ADHESION = 0) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | ST_02 | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 9 | N/A | N/A | The train accepts a new adhesion factor send by a balise group with an M_ADHESION = 0 (70%). -Use_FT4080402.1 for L2 or L2 -Use_FT4080402.3 for L1 | - | - | - | N/A | N/A | - |
| 10 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 3 (Q_NVDRIVER_ADHES=0) is received. | I | BTM | Only for L1. Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 11 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; N_TOTAL=1; Q_NVDRIVER_ADHES=0) is recorded | O | JRU | Only for L1. Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) with Packet 3 (Q_NVDRIVER_ADHES=0) is received. | I | RTM | Only for L2 /L3 Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24; NID_PACKET=3; Q_NVDRIVER_ADHES = 0) is recorded | O | JRU | Only for L2 /L3 Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 14 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The slippery rail icon ST_02 remains displayed. The onboard does NOT reset the adhesion factor from 70% to 100%. | N/A | N/A | - |
| 15 | N/A | N/A | The driver is not allowed to modify the adhesion factor. Use_FT9990200.43 | - | - | - | N/A | N/A | - |

| Step 10: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | 000 | - |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C will be the same than the one stored on-board |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | D_NVOVTRP will be different from the one currently being used |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | 0 | The driver is not allowed to modify Adhesion Factor |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 12: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| T_TRAIN | 1 | FINITE VALUE | On-board train time |
| NID_LRBG | 10+14 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C will be the same than the one stored on-board |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | D_NVOVTRP will be different from the one currently being used |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | 0 | The driver is not allowed to modify Adhesion Factor |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| National Values | STORED | Q_NVDRIVER_ADHES = 0; The driver is not allowed to modify the adhesion factor. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |