| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 6060302 (6.6.3.2 Packets received from balise, loop, RIU, RBC) Total: 24 Test case(s) |  |
| REF: | Subset-076-5-2-6060302 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.0.0 14.02.2014 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, Answer to Review001 | MULTITEL |
| Version 2.9.2 03.10.2013 | TC1, TC2, TC14, TC15, TC16, TC17, TC18, TC22 | Modifications requested by Review001_Subset-076-5-2-6060302_v291.doc | MULTITEL |
| Version 2.9.3 18.10.2013 | All | Answer to Review001_Subset-076-5-2-6060302_v292.doc | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 27 related to trackside system version number X = 1 |  | 1 |
| Applicable Mode/Level Combinations | L1: FS |  |  |
| Target of Test | The target of this test is to check that if the packet 27 is properly translated when the received NC_DIFF is different from 9 to 11. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 5 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 9 |  |
|  |  | Subset-026-6.6.3.2.3 [3] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives a packet 27 “International Static Speed Profile“, via a BG, containing 4 speed limitations related to 4 specific SSP category. Check on DMI that the Permitted Speed is updated according to the speed corresponding to the SSP category of the train. It involves that the NC_DIFF received from trackside is properly translated. |
| Constraints | The ERTMS/ETCS operational train category is TILT 4. There is no Speed Restriction < V_A. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Train Data | STORED | ERTMS/ETCS operational train category is TILT 4 NC_CDTRAIN (Cant Deficiency) = 7 (225 mm) NC_TRAIN = Passenger train in P |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Permitted Speed is DISPLAYED The Target Distance is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 27, packet 12 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | Balise Group BGa message, is RECORDED. | N/A | N/A | - |
| 3 | N/A | N/A | The Permitted Speed is updated to V_B | O | DMI | - | This Permitted Speed corresponds to “TILT 4” category | N/A | N/A | - |
| 4 | N/A | N/A | The Target Distance is updated to D_A | O | DMI | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | V_A | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | 4 | - |
| NC_DIFF(n) | 4 | 7 | TILT 4 (Q_DIFF = 0 & NC_CDDIFF = 7) |
| V_DIFF(n) | 7 | V_B | V_B < V_A |
| NC_DIFF(n) | 4 | 8 | TILT 7 (Q_DIFF = 0 & NC_CDDIFF = 10) |
| V_DIFF(n) | 7 | V_C | V_C < V_B < V_A |
| NC_DIFF(n) | 4 | 9 | TILT 6 (Q_DIFF = 1 & NC_DIFF = 0) |
| V_DIFF(n) | 7 | V_D | V_D < V_C < V_B < V_A |
| NC_DIFF(n) | 4 | 10 | PASS 1 (Q_DIFF = 1 & NC_DIFF = 1) |
| V_DIFF(n) | 7 | V_E | V_E < V_D < V_C < V_B < V_A |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| NC_DIFF(k,m) | 4 | FINITE VALUE | - |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Packet 12: Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | D_A |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 UNCHANGED | FS UNCHANGED |
| International SSP | STORED | V_B |
| ERTMS/ETCS level | 2 UNCHANGED | L1 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Permitted Speed is UPDATED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 27 related to trackside system version number X = 1 |  | 2 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that if the packet 27 is properly translated when the received NC_DIFF equal 9, 10 or 11. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 9 |  |
|  |  | Subset-026-6.6.3.2.3 [3] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives a packet 27 “International Static Speed Profile“, via a BG, containing 4 speed limitations related to 4 specific SSP category. Check on DMI that the Permitted Speed is updated according to the speed corresponding to the SSP category of the train. It involves that the NC_DIFF received from trackside is properly translated. |
| Constraints | The ERTMS/ETCS operational train category is TILT 6. There is no Speed Restriction < V_A. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 | FS/ OS/ SR |
| Train Data | STORED | ERTMS/ETCS operational train category is TILT 6 NC_CDTRAIN (Cant Deficiency) = 9 (275 mm) NC_TRAIN = Passenger train in P |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Permitted Speed is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 27 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Permitted Speed is updated to V_D | O | DMI | - | This Permitted Speed corresponds to “TILT 6” category | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | V_A | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | 4 | - |
| NC_DIFF(n) | 4 | 7 | TILT 4 (Q_DIFF = 0 & NC_CDDIFF = 7) |
| V_DIFF(n) | 7 | V_B | V_B < V_A |
| NC_DIFF(n) | 4 | 8 | TILT 7 (Q_DIFF = 0 & NC_CDDIFF = 10) |
| V_DIFF(n) | 7 | V_C | V_C < V_B < V_A |
| NC_DIFF(n) | 4 | 9 | TILT 6 (Q_DIFF = 1 & NC_DIFF = 0) |
| V_DIFF(n) | 7 | V_D | V_D < V_C < V_B < V_A |
| NC_DIFF(n) | 4 | 10 | PASS 1 (Q_DIFF = 1 & NC_DIFF = 1) |
| V_DIFF(n) | 7 | V_E | V_E < V_D < V_C < V_B < V_A |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| NC_DIFF(k,m) | 4 | FINITE VALUE | - |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 UNCHANGED | FS/ OS/ SR UNCHANGED |
| International SSP | STORED | V_D |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Permitted Speed is UPDATED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 39 related to trackside system version number X = 1 |  | 3 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that if a packet 39 is received with a value of M_TRACTION not included in Subset-026-6.6.3.2.3 [13], the ERTMS/ETCS on-board ignores the packet 39. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 10 |  |
|  |  | Subset-026-6.6.3.2.3 [13] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 39 “Track Condition Change of traction system“ containing a value for M_TRACTION not included in Subset-026-6.6.3.2.3 [13]. Check on DMI that the Change of traction system from packet 39 isn’t applied. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| Train Data | STORED | M_TRACTION >< Non interoperable value if train system version = X=1 |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 39 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | No Track Condition symbol <TC23-TC34> announcement is displayed | O | DMI | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 39 | Track Condition Change of traction system |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACTION | 15 | FINITE VALUE | - |
| M_TRACTION | 8 | 4 | Value not listed in “Subset-026-6.6.3.2.3 [13]” |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 UNCHANGED | FS/ OS/ SR/ LS UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 39 related to trackside system version number X = 1 |  | 4 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that if a packet 39 is received with a value of M_TRACTION included in Subset-026-6.6.3.2.3 [13]. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 10 |  |
|  |  | Subset-026-6.6.3.2.3 [13] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 39 “Track Condition Change of traction system“ containing a value for M_TRACTION included in Subset-026-6.6.3.2.3 [13]. Check on DMI that the Change of traction system from packet 39 is applied. Check on JRU that: the “Traction System voltage” is properly translated; the “Country identifier of the traction system” is properly translated. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| Train Data | STORED | M_TRACTION >< AC 25 kV 50 Hz if train system version = X=1 |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 39 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Track Condition symbol "Change of traction system: AC 25 kV 50 Hz" is displayed | O | DMI | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 39 | Track Condition Change of traction system |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACTION | 15 | FINITE VALUE | - |
| M_TRACTION | 8 | 31 | Value listed in “Subset-026-6.6.3.2.3 [13]” |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 UNCHANGED | FS/ OS/ SR/ LS UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 72 related to trackside system version number X = 1 |  | 5 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that if a packet 72 is received with a value of Q_TEXTCONFIRM <> 0, then Q_CONFTEXTDISPLAY and Q_TEXTREPORT (introduced in system version number X = 2) is set to 0. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.2.1.1 |  |
|  |  | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 27 |  |
|  |  | Subset-026-6.6.3.2.3 [6] |  |
|  |  | Subset-026-6.6.3.4.1.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Plain text information |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of plain text information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 72 “Packet for sending plain text messages“ containing a value of Q_TEXTCONFIRM <> 0, then Q_CONFTEXTDISPLAY and Q_TEXTREPORT (introduced in system version number X = 2) is set to 0. Check on JRU that a packet 72 is recorded Check on DMI that a “plain text message” is displayed and, after the driver acknowledges, is removed. Check on RTM that no driver acknowledgement report is sent. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 72 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The "plain text message" is displayed | O | DMI | - | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver acknowledges "plain text message" | I | DMI | - | - | N/A | N/A | - |
| 5 | N/A | N/A | The "plain text message" is removed | O | DMI | - | Q_CONFTEXTDISPLAY = 0 Driver acknowledgement always ends the text display, regardless of the end condition | N/A | N/A | - |
| 6 | N/A | N/A | No driver acknowledgement report is sent | O | RTM | - | Q_TEXTREPORT = 0 No driver acknowledgement report required | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Packet for sending plain text messages |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 2 | 10 m scale |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | - |
| D_TEXTDISPLAY | 15 | 0 | Start condition – now |
| M_MODETEXTDISPLAY | 4 | 15 | Start condition – not limited by mode |
| M_LEVELTEXTDISPLAY | 3 | 5 | Start condition – not limited by level |
| L_TEXTDISPLAY | 15 | 32766 | End condition – 327.660 km |
| T_TEXTDISPLAY | 10 | 1023 | End condition – Display of text not limited by time |
| M_MODETEXTDISPLAY | 4 | 15 | End condition – not limited by mode |
| M_LEVELTEXTDISPLAY | 3 | 5 | End condition – not limited by level |
| Q_TEXTCONFIRM | 2 | <> 0 | 01 - Continue display until confirmed 10 - Apply service brake if not confirmed when end conditions reached |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 UNCHANGED | FS/ OS/ SR/ LS UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 76 related to trackside system version number X = 1 |  | 6 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check the rejection of a packet 76 received from trackside system version number X = 1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 28 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 76 “Packet for sending fixed text messages“. Check on DMI that no "fixed text message" is displayed (the packet is rejected). |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 76 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | No "fixed text message" is displayed | O | DMI | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Packet for sending fixed text messages |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start eventcondition. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0 |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| Q_TEXT | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 UNCHANGED | FS/ OS/ SR/ LS UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 79 related to trackside system version number X = 1 |  | 7 |
| Applicable Mode/Level Combinations | L0: UN L1 / L2 / L3 : FS / LS / OS / SR / SB / TR / PT / NL |  |  |
| Target of Test | The target of this test case is to check that: the indication of geographical position is deleted if ordered by trackside. the function Determine Geographical position is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.6.1 |  |
|  |  | Subset-026-3.6.6.5 |  |
|  |  | Subset-026-3.6.6.9 |  |
|  |  | Subset-026-3.6.6.9 b) |  |
|  |  | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 29 |  |
|  |  | Subset-026-6.6.3.2.3 [14] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the indication of geographical position is displayed as long as an order to not report the geographical position is not received from trackside, the indication of geographical position is deleted if an order to not report the geographical position is received from trackside. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| ERTMS/ETCS level | 0/ 2/ 3/ 4 | L0/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 79 is received | I | BTM | 001 0000 | Version 1.0 The variable M_POSITION in the packet <> ‘1’048’575’ | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver presses "Geographical Position" | I | DMI | - | After the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | The driver request of The Geographical Position DISPLAY is recorded | N/A | N/A | - |
| 5 | N/A | N/A | The "Geographical Position" is displayed | O | DMI | - | Geographical position is DISPLAYED according to the M_POSITION and to the distance travelled from the geographical reference location. | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message (NID_BG=<BGb>) containing packet 79 is received | I | BTM | 001 0000 | Version 1.0 The variable M_POSITION in the packet = ‘1’048’575’ | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 8 | N/A | N/A | The "Geographical Position" is removed | O | DMI | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 79 | Geographical Position Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION | 20 | FINITE VALUE | <> ‘1’048’575’ |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION(k) | 20 | FINITE VALUE | Track kilometre reference value |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 79 | Geographical Position Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION | 20 | FINITE VALUE | = 1 048 575 |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION(k) | 20 | FINITE VALUE | Track kilometre reference value |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / NL / LS UNCHANGED |
| Geographical Position | NOT STORED | - |
| ERTMS/ETCS level | 0/ 2/ 3/ 4 UNCHANGED | L0/ L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 80 related to trackside system version number X = 1 |  | 8 |
| Applicable Mode/Level Combinations | L2 / L3 : FS |  |  |
| Target of Test | The target of this test is to check that, after receiving a packet 80 from trackside system version number X = 1: After receiving a OS request for a current location, a request for acknowledgement is DISPLAYED to the driver the beginning of the OS area shall be considered as both the EOA and SvL (instead of the EOA and SvL given by the MA) with no release speed, until the driver acknowledges when the driver acknowledges, the ERTMS/ETCS on-board equipment shall immediately switch to OS mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 6 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 30 |  |
|  |  | Subset-026-6.6.3.2.3 [7] |  |
|  |  | Subset-026-5.9.2.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Request for acknowledgement is DISPLAYED to the driver The mode initially DISPLAYED on the DMI changes to OS when the driver acknowledges the OS mode Check on the JRU that: The Driver Acknowledgement is RECORDED The transition to OS mode is RECORDED Check on the TIU that: Trains starts braking to OS area border as if it was both the EOA and SvL (instead of the EOA and SvL given by the MA), with no Release Speed This involves the proper translation of Mode profile with Q_MAMODE set to 1. |
| Constraints | To be in another mode than On Sight. SSP and gradient shall be at least available from the FRONT END of the train. The speed is lower than the OS mode permitted speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Gradient Profile | STORED | at least available from the FRONT END of the train |
| International SSP | STORED | at least available from the FRONT END of the train |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | FS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 80 (D_MAMODE=0; M_MAMODE=0) is received | I | RTM | 001 0000 | Version 1.0 | L2/ L3 | FS | - |
| 2 | L2/ L3 | FS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | - | L2/ L3 | FS | - |
| 3 | L2/ L3 | FS | The Target Distance is updated to D_A | O | DMI | - | Due to packet 15, Level 2/3 Movement Authority | L2/ L3 | FS | - |
| 4 | L2/ L3 | FS | Service brake commanded | O | TIU | - | Otherwise EB will be applied Trains starts braking to OS area border (D_MAMODE=0, current location) as if it was both the EOA and SvL (Q_MAMODE set to 1) | L2/ L3 | FS | - |
| 5 | L2/ L3 | FS | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | - | L2/ L3 | FS | - |
| 6 | L2/ L3 | FS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | Otherwise EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | L2/ L3 | FS | - |
| 7 | L2/ L3 | FS | The driver acknowledges "On Sight" | I | DMI | - | Driver acknowledges OS mode | L2/ L3 | OS | - |
| 8 | L2/ L3 | OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | Driver Acknowledgement of transition to OS mode is RECORDED | L2/ L3 | OS | - |
| 9 | L2/ L3 | OS | The Mode symbol "On Sight" is displayed | O | DMI | - | The On-board switches to OS mode. | L2/ L3 | OS | - |
| 10 | L2/ L3 | OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | - | Mode transition to OS is RECORDED | L2/ L3 | OS | - |
| 11 | L2/ L3 | OS | Service brake not commanded | O | TIU | - | Otherwise EB will be released Service brake is RELEASED | L2/ L3 | OS | - |
| 12 | L2/ L3 | OS | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | - | L2/ L3 | OS | - |
| 13 | L2/ L3 | OS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | SB / EB releasing is RECORDED | L2/ L3 | OS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | 0 | Set to zero if only one section in the MA. |
| L_ENDSECTION | 15 | D_A | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | The mode change is required for the current location |
| M_MAMODE | 2 | 0 | OS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| ERTMS/ETCS level | 3/ 4 UNCHANGED | L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 51 related to trackside system version number X = 1 |  | 9 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check the translation of a packet 51 received from trackside system version number X = 1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 17 |  |
|  |  | Subset-026-6.6.3.2.3 [4] |  |
|  |  | Subset-026-6.6.3.2.3 [5] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 51 “Axle load Speed Profile“. Check on DMI that: The Permitted Speed is updated according to the proper “Axle Load Speed Profile” category. The Permitted Speed is reinitialised immediately at initial states after receiving a packet 51 with Q_TRACKINIT = 1 |
| Constraints | The train’s “Axle load category” is D2 (M_AXLELOADCAT=7; 20 t < M_AXLELOAD ≤ 22.5 t). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 | FS/ OS/ SR |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 51 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Permitted Speed is updated to V_B | O | DMI | - | This Permitted Speed corresponds to “D2” category | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (NID_BG=<BGb>) containing packet 51 is received | I | BTM | 001 0000 | Version 1.0 Q_TRACKINIT = 1 | N/A | N/A | - |
| 5 | N/A | N/A | The Permitted Speed is updated to initial state. | O | DMI | - | D_TRACKINIT is set to 0 Empty Profile: Permitted Speed is resumed to the initial states of the related track description of the packet. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_AXLELOAD | 15 | FINITE VALUE | - |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | 3 | - |
| M_AXLELOAD(1) | 7 | 100000 | Axle load above 16t (HS17) |
| V_AXLELOAD(1) | 7 | V_A | Speed restriction to be applied if the axle load of the train > or = M_AXLELOAD(1) |
| M_AXLELOAD(2) | 7 | 101000 | Axle load above 20t (D2) |
| V_AXLELOAD(2) | 7 | V_B | V_B < V_A |
| M_AXLELOAD(3) | 7 | 101101 | Axle load above 22.5t (E4) |
| V_AXLELOAD(3) | 7 | V_C | V_C < V_B < V_A |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 UNCHANGED | FS/ OS/ SR UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 68 related to trackside system version number X = 1 |  | 10 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check the translation of a packet 68 received from trackside system version number X = 1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 24 |  |
|  |  | Subset-026-6.6.3.2.3 [8] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 68 “Track Condition“ containing M_TRACKCOND = 1 or 2. Check on DMI that: A “Non stopping area” (M_TRACKCOND=0) is displayed. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 68 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Track Condition symbol "Non stopping area" is displayed | O | DMI | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | No initial states to be resumed, profile to follow |
| D_TRACKCOND | 15 | 0 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 1 or 2 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 UNCHANGED | FS/ OS/ SR/ LS UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Condition symbol "Non stopping area" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 70 related to trackside system version number X = 1 |  | 11 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check if a packet 70 received from trackside system version number X = 1 and containing Q_TRACKINIT = 1 is not rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 25 |  |
|  |  | Subset-026-6.6.3.2.3 [11] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 70 “Route Suitability Data“ containing Q_TRACKINIT = 1. There is no packet 207 in the same message. Check on DMI that: The Permitted Speed is updated (Empty profile is resumed). |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 | FS/ OS/ SR |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Permitted Speed is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 70 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Permitted Speed is updated to initial state | O | DMI | - | D_TRACKINIT is set to 0 Empty Profile: Permitted Speed is resumed to the initial states of the related track description of the packet. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1 |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_AXLELOAD | 7 | FINITE VALUE | If Q_SUITABILITY = Max axle load. |
| M_TRACTION | 8 | FINITE VALUE | If Q_SUITABILITY = traction system |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_AXLELOAD(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_TRACTION(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = traction system |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 UNCHANGED | FS/ OS/ SR UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 70 related to trackside system version number X = 1 |  | 12 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check if a packet 70 received from trackside system version number X = 1 and containing Q_TRACKINIT = 0 is properly translated. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 25 |  |
|  |  | Subset-026-6.6.3.2.3 [11] |  |
|  |  | Subset-026-6.6.3.2.3 [12] |  |
|  |  | Subset-026-6.6.3.2.3 [13] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 70 “Route Suitability Data“ containing Q_TRACKINIT = 0, Q_SUITABILITY = 10 and M_TRACTION equal to one of the values that are listed in the translation table Subset-026-6.6.3.2.3 [13]. There is no packet 207 in the same message. Check on DMI that the Change of traction system from packet 70 is applied. Check on JRU that: the “Traction System voltage” is properly translated; the “Country identifier of the traction system” is properly translated. |
| Constraints | The traction system is different from “AC 15 kV 16.7 Hz”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 70 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Track Condition symbol "Change of traction system: AC 15 kV 16.7 Hz" is displayed | O | DMI | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only if Q_TRACKINIT = 0 |
| Q_SUITABILITY | 2 | 10 | Traction system |
| M_TRACTION | 8 | 11 | If Q_SUITABILITY = traction system Value listed in “Subset-026-6.6.3.2.3 [13]” |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_AXLELOAD(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_TRACTION(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = traction system |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 UNCHANGED | FS/ OS/ SR/ LS UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 70 related to trackside system version number X = 1 |  | 13 |
| Applicable Mode/Level Combinations | L1 / L2 / L3 : FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check if a packet 70 received from trackside system version number X = 1 and containing Q_TRACKINIT = 0 is properly translated. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 25 |  |
|  |  | Subset-026-6.6.3.2.3 [11] |  |
|  |  | Subset-026-6.6.3.2.3 [12] |  |
|  |  | Subset-026-6.6.3.2.3 [13] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives, via a BG, a packet 70 “Route Suitability Data“ containing Q_TRACKINIT = 0, Q_SUITABILITY <> 10 or M_TRACTION not equal to one of the values that are listed in the translation table Subset-026-6.6.3.2.3 [13]. There is no packet 207 in the same message. Check on DMI that no Change of traction system from packet 70 is applied. |
| Constraints | The traction system is different from “AC 15 kV 16.7 Hz”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 70 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | No Track Condition symbol <TC23-TC34> announcement is displayed | O | DMI | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only if Q_TRACKINIT = 0 |
| Q_SUITABILITY | 2 | 10 | Traction system |
| M_TRACTION | 8 | 9 | If Q_SUITABILITY = traction system Value not listed in “Subset-026-6.6.3.2.3 [13]” |
| N_ITER | 5 | 1 | - |
| D_SUITABILITY(k) | 15 | 0 | - |
| Q_SUITABILITY(k) | 2 | 00 or 01 | - |
| M_AXLELOAD(k) | 7 | FINITE VALUE | If Q_SUITABILITY= loading gauge |
| M_TRACTION(k) | 8 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 UNCHANGED | FS/ OS/ SR/ LS UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1/ L2/ L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 200 related to system version number X = 1 |  | 14 |
| Applicable Mode/Level Combinations | L0 : UN L1 / L2 / L3 : FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that if the packet 200 is properly translated |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 44 |  |
|  |  | Subset-026-6.6.3.2.3 [15] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives a packet 200 “Virtual Balise Cover marker“ (VBC), via a BGa. The variable NID_PACKET is set to 0 and both variables Q_DIR and L_PACKET are deleted. Check on DMI that the Text message in the BGb marked with a VBC marker is not displayed. |
| Constraints | A VBC (NID_VBCMK = VBCa) is stored onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS/ SR / UN / NL / LS |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | NID_VBCMK = VBCa |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L0/L1/ L2/ L3 | FS/OS/SR/UN/NL/LS | One Balise Group message (NID_BG=<BGa>) containing packet 200 (NID_VBCMK=VBCa), packet 72 (X_TEXT(L_TEXT)=‘ERROR: The packet 200 is wrongly translated‘) is received | I | BTM | 001 0001 | Version 1.1 | L0/L1/ L2/ L3 | FS/OS/SR/UN/NL/LS | - |
| 2 | L0/L1/ L2/ L3 | FS/OS/SR/UN/NL/LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | L0/L1/ L2/ L3 | FS/OS/SR/UN/NL/LS | - |
| 3 | L0/L1/ L2/ L3 | FS/OS/SR/UN/NL/LS | No "plain text message" is displayed | O | DMI | - | The text ‘ERROR: The packet 200 is wrongly translated‘ is not displayed. | L0/L1/ L2/ L3 | FS/OS/SR/UN/NL/LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | Version 1.1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 200 | Packet 200: Virtual Balise Cover marker |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_VBCMK | 6 | VBCa | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | - |
| D_TEXTDISPLAY | 15 | FINITE VALUE | - |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | - |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | - |
| L_TEXTDISPLAY | 15 | FINITE VALUE | - |
| T_TEXTDISPLAY | 10 | FINITE VALUE | - |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | - |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | - |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | ERROR: The packet 200 is wrongly translated | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 UNCHANGED | FS / OS/ SR / UN / NL / LS UNCHANGED |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 UNCHANGED | L0 / L1 / L2 / L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 203 related to system version number X = 1 |  | 15 |
| Applicable Mode/Level Combinations | L0 : UN L1 / L2 / L3 : LS / OS / SR / SH |  |  |
| Target of Test | The target of this test is to check that if the packet 203 is properly translated |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 45 |  |
|  |  | Subset-026-6.6.3.2.3 [16] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives a packet 203 “National Values for braking curves“ via a BGa. This packet must be appended to the packet 3 received in the same message, in order to form a single set of National Values, to which apply the distance to start of validity and the list of national area identifiers given in the packet 3. |
| Constraints | The national values are not known and the train OS X=2 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 3 / 4 / 12 | OS / SR / SH / UN / LS |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L0/L1/ L2/ L3 | OS / SR / SH / UN / LS | One Balise Group message (NID_BG=<BGa>) containing packet 3, packet 203 is received | I | BTM | 001 0001 | Version 1.1 | L0/L1/ L2/ L3 | OS / SR / SH / UN / LS | - |
| 2 | L0/L1/ L2/ L3 | OS / SR / SH / UN / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | L0/L1/ L2/ L3 | OS / SR / SH / UN / LS | - |
| 3 | L0/L1/ L2/ L3 | OS / SR / SH / UN / LS | The Permitted speed is updated | - | - | - | This step test the acceptation of the packet 3 to verify the maximum permitted speed in OS / SR / SH / UN / LS Use_FT9990200.3,5, 16 | L0/L1/ L2/ L3 | OS / SR / SH / UN / LS | - |
| 4 | L0/L1/ L2/ L3 | OS / SR / SH / UN / LS | This step test the acceptation of the packet 203 | - | - | - | Use_FT3130237.6,8 (e.g. guidance curve) to verify the application of the packets 203 | L0/L1/ L2/ L3 | OS / SR / SH / UN / LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | Version 1.1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | National Values |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of national area(s) to which the set applies |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSRBKTRG | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 203 | National Values for braking curves |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | - |
| A_NVP12 | 6 | FINITE VALUE | - |
| A_NVP23 | 6 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | - |
| A_NVP23(k) | 6 | FINITE VALUE | - |
| V_NVKVINT(k) | 7 | FINITE VALUE | - |
| M_NVKVINT(k) | 7 | FINITE VALUE | - |
| M_NVKVINT(k) | 7 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| L_NVKRINT | 5 | FINITE VALUE | - |
| M_NVKRINT | 5 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 UNCHANGED | FS / OS / SR / SH / UN / SL / TR / NL / LS / SN / RV / PS UNCHANGED |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 UNCHANGED | L0 / L1 / L2 / L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 206 related to system version number X = 1 |  | 16 |
| Applicable Mode/Level Combinations | L0 : UN L1 / L2 / L3 : FS / LS / SR / OS / NL / TR / SN |  |  |
| Target of Test | The target of this test is to check that if the packet 206 is properly translated |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 24 |  |
|  |  | Subset-026-6.6.3.2.3 [9] |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 46 |  |
|  |  | Subset-026-6.6.3.2.3 [17] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives a packet 206 “Track Condition“ and a packet 68 “Track Condition“ via a BGa. The variable NID_PACKET of packet 206 is set to 68 and the packet 68 is rejected. Check that the track condition inside packet 206 is applied. The non stopping area is displayed on the DMI. |
| Constraints | No constraints |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 11 / 12 / 13 | FS / OS/ SR / UN / TR / NL / LS / SN |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | One Balise Group message (NID_BG=<BGa>) containing packet 206 (Q_TRACKINIT=0; M_TRACKCONDBC=’0000’), packet 68 (Q_TRACKINIT=0; M_TRACKCOND<>0,1,2) is received | I | BTM | 001 0001 | Version 1.1 | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | - |
| 2 | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | - |
| 3 | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | The Track Condition symbol "Non stopping area" is displayed | O | DMI | - | - | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | Version 1.1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 206 | Packet 206: Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | 0 | - |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCONDBC | 4 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCONDBC(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | - |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | M_TRACKCOND <> 0,1,2 and M_TRACKCONDBC |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | 1,1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 UNCHANGED | FS / OS/ SR / UN / NL / LS UNCHANGED |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 UNCHANGED | L0 / L1 / L2 / L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 207 related to system version number X = 1 |  | 17 |
| Applicable Mode/Level Combinations | L0 : UN L1 / L2 / L3 : FS / LS / SR / OS / SN |  |  |
| Target of Test | The target of this test is to check that if the packet 207 is properly translated |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 25 |  |
|  |  | Subset-026-6.6.3.2.3 [11] |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 47 |  |
|  |  | Subset-026-6.6.3.2.3 [18] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives a packet 207 “Route Suitability Data“ and a packet 70 “Route Suitability Data“ via a LTM. The variable NID_PACKET of packet 207 is set to 70 and the packet 70 is REJECTED. Check that the route suitability inside packet 207 is applied. The non stopping area is displayed on the DMI. |
| Constraints | No constraints |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 / 13 | FS / OS / SR / LS / SN |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L1/ L2/ L3 | FS/OS/SR/LS/SN | One Balise Group message (NID_BG= BGa) containing packet 134 (NID_LOOP = LPa) is received | I | BTM | 001 0001 | Version 1.1 | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |
| 2 | L1/ L2/ L3 | FS/OS/SR/LS/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |
| 3 | L1/ L2/ L3 | FS/OS/SR/LS/SN | One Euroloop message (NID_LOOP = LPa) containing packet 207 (Q_SUITABILITY=00) and packet 70 is received | I | LTM | 001 0001 | Version 1.1 | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |
| 4 | L1/ L2/ L3 | FS/OS/SR/LS/SN | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | - | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |
| 5 | L1/ L2/ L3 | FS/OS/SR/LS/SN | The system status message "Route unsuitable - <comments>" is displayed | O | DMI | - | The route unsuitability of packet 207 is displayed on the DMI The route unsuitability of packet 70 is NOT displayed | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |
| 6 | L1/ L2/ L3 | FS/OS/SR/LS/SN | The driver selects "Override Route unsuitable" | I | DMI | - | - | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | Version 1.1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 134 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LOOP | 14 | LPa | - |
| D_LOOP | 15 | FINITE VALUE | - |
| L_LOOP | 15 | FINITE VALUE | - |
| Q_LOOPDIR | 1 | FINITE VALUE | - |
| Q_SSCODE | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | 1,1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 001 0001 | 1,1 |
| Q_MEDIA | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_LOOP | 14 | LPa | - |
| NID_PACKET | 8 | 207 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | - |
| D_SUITABILITY | 15 | FINITE VALUE | - |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | Insert a incompatible value |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY = Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0 |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY = loading gauge |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY = traction system |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0 |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | - |
| D_SUITABILITY | 15 | FINITE VALUE | - |
| Q_SUITABILITY | 2 | 1 | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | Insert a incompatible value |
| M_TRACTION | 8 | FINITE VALUE | If Q_SUITABILITY = traction system |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_AXLELOAD(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_TRACTION(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = traction system |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 / 13 UNCHANGED | FS / OS / SR / LS / SN UNCHANGED |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 239 related to system version number X = 1 |  | 18 |
| Applicable Mode/Level Combinations | L0 : UN L1 / L2 / L3 : FS / LS / SR / OS / NL / TR / SN |  |  |
| Target of Test | The target of this test is to check that if the packet 239 is properly translated |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 10 |  |
|  |  | Subset-026-6.6.3.2.3 [13] |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 48 |  |
|  |  | Subset-026-6.6.3.2.3 [19] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives a packet 239 “Track Condition Change of traction system “ and a packet 39 “Track Condition Change of traction system “ via a BGa. The variable NID_PACKET of packet 239 is set to 39. Check that the track condition Change of traction system inside packet 239 is applied and the track condition inside packet 39 is NOT applied. |
| Constraints | No constraints |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 11 / 12 / 13 | FS / OS/ SR / UN / TR / NL / LS / SN |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | One Balise Group message containing packet 239 (NID_CTRACTION incompatible) and a packet 39 (NID_CTRACTION incompatible) is received | I | BTM | 001 0001 | Version 1.1 | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | - |
| 2 | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | L1/ L2/ L3 | FS/OS/ SR/UN/TR/NL/LS/SN | - |
| 3 | L1/ L2/ L3 | FS/OS/SR/LS/SN | The Track Condition symbol <TC23¦TC25¦TC27¦TC29¦TC31¦TC33> is displayed | O | DMI | - | After the distance DTRa, the traction system voltage of packet 239 is displayed on the DMI | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |
| 4 | L1/ L2/ L3 | FS/OS/SR/LS/SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit66=1¦Bit68=1¦Bit70=1¦Bit72=1¦Bit74=1¦Bit76=1>) is recorded | O | JRU | - | - | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |
| 5 | L1/ L2/ L3 | FS/OS/SR/LS/SN | NOT The Track Condition symbol <TC23¦TC25¦TC27¦TC29¦TC31¦TC33> is displayed | O | DMI | - | After the distance DTRb, the traction system voltage of packet 39 is not displayed on the DMI | L1/ L2/ L3 | FS/OS/SR/LS/SN | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | Version 1.1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 239 | Packet 206: Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACTION | 15 | DTRa | - |
| M_VOLTAGE | 4 | FINITE VALUE | VALUE INCOMPATIBLE WITH THE TRAIN |
| NID_CTRACTION | 10 | FINITE VALUE | VALUE INCOMPATIBLE WITH THE TRAIN |
| NID_PACKET | 8 | 39 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACTION | 15 | DTRb | - |
| M_VOLTAGE | 4 | FINITE VALUE | VALUE INCOMPATIBLE WITH THE TRAIN |
| NID_CTRACTION | 10 | FINITE VALUE | VALUE INCOMPATIBLE WITH THE TRAIN |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | 1,1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 11 / 12 / 13 UNCHANGED | FS / OS/ SR / UN / TR / NL / LS / SN UNCHANGED |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 3 related to trackside system version number X = 1 and onboard system version number X = 1 |  | 19 |
| Applicable Mode/Level Combinations | L1 : LS |  |  |
| Target of Test | The target of this test is to check if a packet 3 received by an onboard system version number X = 1, from trackside system version number X = 1, the National Values Q_NVLOCACC, V_NVLIMSUPERV are set to their respective default value. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 2 |  |
|  |  | Subset-026-6.6.3.2.3 [1a] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train (system version number X=1) receives, via a BG, a packet 3 “National Values“. There is no packet 203 in the same message. The National Values Q_NVLOCACC, V_NVLIMSUPERV are set to their respective default value Check on DMI that, after reading packet 3: Service brake is APPLIED L_DOUBTUNDER and L_DOUBTOVER have reduced their values. |
| Constraints | The onboard system version number is X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| National Values | STORED | V_NVLIMSUPERV > 20 (=100km/h) The value stored is greater than the default value Q_NVLOCACC > 12 (default value) The other National Values for braking curves are the default ones |
| Linking | NOT STORED | - |
| Mode Profile | STORED | V_MAMODE=127 (Use the national speed value of the LS mode) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current train speed is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) is received | I | BTM | 001 0000 | - | N/A | N/A | - |
| 2 | N/A | N/A | GENERAL MESSAGE(NID_MESSAGE=1;L_DOUBTOVER= L_DOUBTOVER(1);L_DOUBTUNDER= L_DOUBTUNDER(1)) | O | JRU | - | L_DOUBTUNDER(1) and L_DOUBTOVER(1) after having recorded the Balise Group BGa. | N/A | N/A | - |
| 3 | N/A | N/A | Service brake not commanded | O | DMI | - | Service brake is not APPLIED. The permitted speed is equal or lower to the maximum Limited Supervision speed but upper to the default value of V_NVLIMSUPERV (100 km/h). | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (NID_BG=<BGb>) containing packet 3 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 6 | N/A | N/A | GENERAL MESSAGE(NID_MESSAGE=1;L_DOUBTOVER= L_DOUBTOVER(2);L_DOUBTUNDER= L_DOUBTUNDER(2)) | O | JRU | - | L_DOUBTUNDER(2) and L_DOUBTOVER(2) have reduced their values to take into account the reset of Q_NVLOCACC to the default value after having recorded the Balise Group BGb. | N/A | N/A | - |
| 7 | N/A | N/A | Cut off traction Service brake commanded | O | TIU | - | V_NVLIMSUPERV is set to the default value (100km/h) | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | - | N/A | N/A | - |
| 9 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | - | N/A | N/A | - |
| 10 | N/A | N/A | Service brake not commanded | O | TIU | - | Service brake is RELEASED. The speed is lower or equal to maximum Limited Supervision speed | N/A | N/A | - |
| 11 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | The service brake intervention indication is REMOVED. | N/A | N/A | - |
| 12 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | The release of service brake is RECORDED. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | National Values |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of national area(s) to which the set applies |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSRBKTRG | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 UNCHANGED | LS UNCHANGED |
| ERTMS/ETCS level | 2 UNCHANGED | L1 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 3 related to trackside system version number X = 1 and onboard system version number X = 2 |  | 20 |
| Applicable Mode/Level Combinations | L1 : LS |  |  |
| Target of Test | The target of this test is to check if a packet 3 received by an onboard system version number X = 2, from trackside system version number X = 1, the National Values Q_NVLOCACC, V_NVLIMSUPERV are not affected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 2 |  |
|  |  | Subset-026-6.6.3.2.3 [1b] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train (system version number X=2) receives, via a BG, a packet 3 “National Values“. There is no packet 203 in the same message. The National Values Q_NVLOCACC, V_NVLIMSUPERV are not affected. Check on DMI that, after reading packet 3: Service brake is NOT APPLIED L_DOUBTUNDER and L_DOUBTOVER didn’t have reduced their values. |
| Constraints | The onboard system version number is X = 2. There is no mismatch between the country or region identifier read from BG and the corresponding identifier(s) of the applicable set with which the National Value was received and stored |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| National Values | STORED | V_NVLIMSUPERV > 20 (=100km/h) The value stored is greater than the default value Q_NVLOCACC > 12 (default value) The other National Values for braking curves are the default ones |
| Linking | NOT STORED | - |
| Mode Profile | STORED | V_MAMODE=127 (Use the national speed value of the LS mode) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current train speed is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) is received | I | BTM | 001 0000 | - | N/A | N/A | - |
| 2 | N/A | N/A | GENERAL MESSAGE(NID_MESSAGE=1;L_DOUBTOVER= L_DOUBTOVER(1);L_DOUBTUNDER= L_DOUBTUNDER(1)) | O | JRU | - | L_DOUBTUNDER(1) and L_DOUBTOVER(1) after having recorded the Balise Group BGa. | N/A | N/A | - |
| 3 | N/A | N/A | Service brake not commanded | O | DMI | - | The permitted speed is equal or lower to the maximum Limited Supervision speed but upper to the default value of V_NVLIMSUPERV (100 km/h). | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (NID_BG=<BGb>) containing packet 3 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 6 | N/A | N/A | GENERAL MESSAGE(NID_MESSAGE=1;L_DOUBTOVER= L_DOUBTOVER(2);L_DOUBTUNDER= L_DOUBTUNDER(2)) | O | JRU | - | L_DOUBTUNDER(2) and L_DOUBTOVER(2) after having recorded the Balise Group BGb are similar to L_DOUBTUNDER(1) and L_DOUBTOVER(1) because the onboard still takes into account the stored value of Q_NVLOCACC (that is greater than the default value). | N/A | N/A | - |
| 7 | N/A | N/A | Service brake not commanded | O | TIU | - | V_NVLIMSUPERV is still greater than the default value (100km/h) | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | National Values |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of national area(s) to which the set applies |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSRBKTRG | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 UNCHANGED | LS UNCHANGED |
| ERTMS/ETCS level | 2 UNCHANGED | L1 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 3 related to trackside system version number X = 1 |  | 21 |
| Applicable Mode/Level Combinations | L1 : FS |  |  |
| Target of Test | The target of this test is to check if a packet 3 from trackside system version number X = 1, the National Values for braking curves are not affected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 2 |  |
|  |  | Subset-026-6.6.3.2.3 [2] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train (system version number X=2) receives, via a BG, a packet 3 “National Values“. There is no packet 203 in the same message. The National Values for braking curves are not affected. |
| Constraints | National values must be stored onboard. There is no mismatch between the country or region identifier read from BG and the corresponding identifier(s) of the applicable set with which the National Value was received and stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current train speed is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Guidance Curves are permitted to use. | O | DMI | - | Use_FT3130237.6,8 “Guidance Curve is permitted” | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 3 is received | I | BTM | 001 0000 | Version 1.0 | N/A | N/A | - |
| 3 | N/A | N/A | The Guidance Curves are permitted to use. | O | DMI | - | Use_FT3130237.6,8 “Guidance Curve is permitted” The National Values for braking curves haven’t been affected. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | National Values |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of national area(s) to which the set applies |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSRBKTRG | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 UNCHANGED | FS UNCHANGED |
| ERTMS/ETCS level | 2 UNCHANGED | L1 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Reception of packet 27 related to trackside system version number X = 1 |  | 22 |
| Applicable Mode/Level Combinations | L1: FS |  |  |
| Target of Test | The target of this test is to check that if the packet 27 is rejected because of an invalid consistency check. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.1.1 |  |
|  |  | Subset-026-6.6.3.1.2 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 5 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 9 |  |
|  |  | Subset-026-6.6.3.2.3 [3] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives a packet 27 “International Static Speed Profile“, via a BG, containing 2 speed limitations related to 2 specific SSP category. The balise announces M_VERSION = 1 but the packet is in version X=2 and should be rejected because of an invalid consistency. Check on DMI that the Permitted Speed is NOT updated according to the speed corresponding to the SSP category of the train. |
| Constraints | The ERTMS/ETCS operational train category is TILT 4. There is no Speed Restriction < V_A. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Train Data | STORED | ERTMS/ETCS operational train category is TILT 4 NC_CDTRAIN (Cant Deficiency) = 7 (225 mm) NC_TRAIN = Passenger train in P |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Permitted Speed is displayed The Target Distance is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 27, packet 12 is received | I | BTM | 001 0000 | Version 1.0 but packet 27 with version X=2 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | Balise Group BGa message, is RECORDED. | N/A | N/A | - |
| 3 | N/A | N/A | The Permitted Speed is NOT updated. | O | DMI | - | The Permitted Speed is NOT updated to V_B | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | Packet version X=2 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | V_A | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| Q_DIFF(1) | 2 | 1 or 2 | Variable not present in B2 |
| NC_DIFF(1) | 4 | 8 | If Q_DIFF(n) = 1 or 2 - TILT 7 |
| V_DIFF(1) | 7 | V_B | V_B < V_A |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0 |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2 |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Packet 12: Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | D_A |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 UNCHANGED | FS UNCHANGED |
| International SSP | STORED | V_A |
| ERTMS/ETCS level | 2 UNCHANGED | L1 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Intentionally deleted |  |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.2 Packets received from balise, loop, RIU, RBC |  | 6060302 |
| Test Case of Feature | The train is moving in SR mode. This TC tests the less restrictive conditions for Transition Condition Id [54] in older system version. |  | 24 |
| Applicable Mode/Level Combinations | L1: SR L2: SR L3: SR |  |  |
| Target of Test | The target of this test is to check that: - the train has received a list of expected balise groups related to SR mode that include the identity of the next balise group; - the train passes over a balise group sending a message which contains packet 137 “Stop if in Staff Responsible” (with the variable Q_SRSTOP set to 0) AND override is not active; - The mode changes to TR ; - The emergency brakes are applied. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.2.2.1 |  |
|  |  | Subset-026-6.6.2.2.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: - Packet 137: “Stop if in Staff Responsible” is RECORDED. The variable Q_SRSTOP is set to 0. - The transition to TRIP mode is RECORDED - Trip reason is RECORDED. - EB application is RECORDED. Check on the TIU that: - EB are APPLIED. Check on the DMI that: - The change to TR mode is DISPLAYED. - Trip reason is DISPLAYED - The EB application is DISPLAYED. |
| Constraints | The on-board operating with system version number X = 1. The variable Q_DIR in packet 137 ensures that the train direction coincides with the packet validity direction. The train trip is not inhibited (Procedure Override EOA). Override is not active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | The list includes BGa |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed The Target Distance is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | SR | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 137 (Q_SRSTOP=0) is received | I | BTM | 001 0000 | BGa telegram contains Packet 137: “Stop if in Staff Responsible” with variable Q_SRSTOP = 0. | L1 / L2 / L3 | SR | - |
| 2 | L1 / L2 / L3 | SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | L1 / L2 / L3 | SR | - |
| 3 | L1 / L2 / L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | - | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver.” Trip Reason: System Message “SR stop order”. | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | - | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit16=1>) is recorded | O | JRU | - | SR stop order | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | - | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | The Status symbol "Emergency Brake intervention" is displayed | O | DMI | - | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 10 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | - | L1 / L2 / L3 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 001 0000 | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | Bga | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 137 | - |
| Q_DIR | 2 | FINITE VALUE | = train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SRSTOP | 1 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 001 0000 | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | Bga | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| List of Balises in SR Authority + SR mode speed limit and distance | Stored | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |