| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5180400 (5.18.4: Passing a non stopping area) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5180400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 21.08.2013 | All | Answer to Review003_Subset-076-5-2-5180800_v291 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.4: Passing a non stopping area |  | 5180400 |
| Test Case of Feature | The on-board equipment has received track condition information indicating the location of two non stopping areas. The on-board equipment displays to the driver an announcement and an indication of both non stopping areas. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment displays to the driver an announcement and an indication of the non stopping areas according to the received track condition information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions Sound Horn, Non Stopping areas, Tunnel Stopping areas |  |
|  |  | Subset-026-4.7.2 table - Output information Track conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-5.18.4.2 |  |
|  |  | Subset-026-5.18.4.2 a) |  |
|  |  | Subset-026-5.18.4.2 b) |  |
|  |  | Subset-026-5.18.4.2 c) |  |
|  |  | Subset-026-5.18.4.3 |  |
|  |  | Subset-026-5.18.4.4 |  |
|  |  | Subset-026-5.18.4.4 a) |  |
|  |  | Subset-026-5.18.4.4 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that for both non stopping areas: The “Non stopping area announcement” is DISPLAYED when the max safe front end reaches the first SBI supervision limit (SBID) The “Non stopping area announcement” is REMOVED when the max safe front end reaches the non stopping area The “Non stopping area” is DISPLAYED when the max safe front end reaches the non stopping area. The “Non stopping area” is REMOVED when the min safe front end reaches the second SBI supervision limit (SBIG) Check on the JRU that the corresponding records are stored |
| Constraints | Track condition information indicating the location of two non-stopping areas has been received. The train speed is such that it allows the display of the announcement and indication of both non stopping areas. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND=FINITE VALUE Location of the 1st non stopping area L_TRACKCOND=FINITE VALUE Length of the 1st non stopping area M_TRACKCOND=0 1st Track condition: Non stopping area D_TRACKCOND(1)=FINITE VALUE Location of the 2nd non stopping area L_TRACKCOND(1)=FINITE VALUE Length of the 2nd non stopping area M_TRACKCOND(1)=0 2nd Track condition: Non stopping area |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Non-stopping information is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Track Condition symbol "Non stopping area announcement" is displayed | O | DMI | 1st non stopping area As son as the max safe front end reaches the first SBI supervision limit (SBID) “Non stopping area announcement” is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit53=0&Bit54=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Track Condition symbol "Non stopping announcement" is removed | O | DMI | D_LRBG > D_TRACKCOND The train reaches with its max safe front end the 1st non-stopping area ”Non stopping area announcement” is REMOVED | N/A | N/A | - |
| 4 | N/A | N/A | The Track Condition symbol "Non stopping area" is displayed | O | DMI | “Non stopping area” is DISPLAYED | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit53=1&Bit54=0>) is recorded | O | JRU | “Non stopping area” is DISPLAYED” is RECORDED. | N/A | N/A | - |
| 6 | N/A | N/A | The Track Condition symbol "Non stopping area" is removed | O | DMI | 1st non stopping area When the min safe front end reaches the second SBI supervision limit (SBIG), “Non stopping area” is REMOVED | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit53=0&Bit54=0>) is recorded | O | JRU | “ Non stopping information is REMOVED” is RECORDED | N/A | N/A | - |
| 8 | N/A | N/A | The Track Condition symbol "Non stopping area announcement" is displayed | O | DMI | 2nd non stopping area As son as the max safe front end reaches the first SBI supervision limit (SBID) “Non stopping area announcement” is DISPLAYED | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit53=0&Bit54=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The Track Condition symbol "Non stopping announcement" is removed | O | DMI | D_LRBG > D_TRACKCOND + D_TRACKCOND(1) The train reaches with its max safe front end the 2nd non-stopping area ”Non stopping area announcement” is REMOVED | N/A | N/A | - |
| 11 | N/A | N/A | The Track Condition symbol "Non stopping area" is displayed | O | DMI | “Non stopping area” is DISPLAYED | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit53=1&Bit54=0>) is recorded | O | JRU | “Non stopping area” is DISPLAYED” is RECORDED. | N/A | N/A | - |
| 13 | N/A | N/A | The Track Condition symbol "Non stopping area" is removed | O | DMI | 2nd non stopping area When the min safe front end reaches the second SBI supervision limit (SBIG), “Non stopping area” is REMOVED | N/A | N/A | - |
| 14 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit53=0&Bit54=0>) is recorded | O | JRU | “ Non stopping information is REMOVED” is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |