| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3150900 (3.15.9: Virtual Balise Cover) Total: 8 Test case(s) |  |
| REF: | Subset-076-5-2-3150900 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/New in Baseline 3 | INECO |
| Version 2.9.1 28.05.2013 | All | Second version/ UNISIG remarks from 20-05-2013 included; TC_Editor tool and Dictionary used. | INECO |
| Version 2.9.2 25.11.2013 | All | Third version/Internal modification;TC_Editorv27; Correction of Requirements blank space; Review closed | INECO |
| Version 2.9.3 04.12.2013 | All | Fourth version/ Added requirement | INECO |
| Version 2.9.4 20.12.2013 | TC8 | Fifth version / New created to cover exception 8.4.1.4.8 & 8.4.1.4.9 | INECO |
| Version 2.9.5 10.01.2014 | TC8 | Sixth version/ UNISIG remarks from 09-01-2014 included | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.9: Virtual Balise Cover |  | 3150900 |
| Test Case of Feature | The on-board equipment ignores any balise telegram that includes a VBC marker and a country/region identity that both match the VBC identity already stored on-board. |  | 1 |
| Applicable Mode/Level Combinations | L0: SH/ PS/ SL/ UN/ TR/ NL LNTC: SH/ PS/ SL/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: The ERTMS/ETCS on-board equipment reads a balise group message with telegrams that includes a VBC identity in the header of each balise from the group, packet 0: Virtual Balise Cover marker. Packet 254 is also received. The received VBC identity matches the one stored on board.(NID_VBCMK(1)=NID_VBCMK(2) and NID_C(1)=NID_C(2)) The on-board accepts the balise message but ignores all information contained in the telegrams. The message “Trackside malfunction” is not displayed to the driver. The function “Manage Virtual Balise Covers” is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.9.3 |  |
|  |  | Subset-026-3.15.9.3 a) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Virtual Balise Covers |  |
|  |  | Subset-026-7.4.2.0 Packet Number 0: Virtual Balise Cover marker |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: No message is displayed to the driver. Check on the JRU that : The balise telegram that includes a VBC identity is RECORDED. |
| Constraints | VBC´s and balises group applicable for same country/region. At least one VBC identity is stored on-board. New received VBC identity matches the stored VBC identity. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | At least one VBC Code stored on-board: VBC #1 set code= NID_VBCMK(1)+NID_C(1)+T_VBC(1) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1; NID_C=<country/region ID>) containing packet 0 (NID_VBCMK (2)) and packet 254 is received. | I | BTM | Packet Number 0: “Virtual Balise Cover marker” included in the header of each balise from the group NID_VBCMK (2)= VBC #1 set code | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The system status message "Trackside malfunction" is NOT displayed. | O | DMI | Information from packet 254 ignored by on-board equipment. Not available in PS, SL and NL modes | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Same value as stored country/region identity (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as stored VBC #1 set code |
| NID_PACKET | 8 | 254 | Default balise, loop and RIU information |
| Q_DIR | 2 | 01 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as stored VBC #1 set code |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14 /15 | FS/ OS/ SR/ SH/ SL/ UN/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Trackside malfunction" is NOT displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.9: Virtual Balise Cover |  | 3150900 |
| Test Case of Feature | The on-board equipment detects errors in the reading of the rest of the balise telegrams that includes a VBC identity. No reaction is applied. |  | 2 |
| Applicable Mode/Level Combinations | L0: SH/ PS/ SL/ UN/ TR/ NL LNTC: SH/ PS/ SL/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: The ERTMS/ETCS on-board equipment reads a balise message with telegrams that includes a VBC identity, packet 0: Virtual Balise Cover marker and packet 254. The received VBC identity matches the one stored onboard.(NID_VBCMK(2)= VBC #1 set code) The on-board equipment detects a data inconsistency in reading the balise group message (N_PIG >N_TOTAL ) and No command of service brake is applied. The function “Manage Virtual Balise Covers” is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.9.3 |  |
|  |  | Subset-026-3.15.9.3 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Virtual Balise Covers |  |
|  |  | Subset-026-4.8.2.3 |  |
|  |  | Subset-026-7.4.2.0 Packet Number 0: Virtual Balise Cover marker |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces TIU that: Service brake not commanded due to a data consistency error. Check on the JRU that : The balise group message that includes a VBC identity is RECORDED. |
| Constraints | VBC´s and balises group applicable for same country/region. At least one VBC identity stored on board. New received VBC identity matches the stored VBC identity. An error of data consistency is intentionally added. (N_PIG >N_TOTAL) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | At least one VBC Code stored on-board: VBC #1 set code = NID_VBCMK(1)+NID_C(1)+T_VBC(1) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1; NID_C=<country/region ID>) containing packet 0 (NID_VBCMK (2)) and packet 254 is received. | I | BTM | NID_VBCMK (2)= VBC #1 set code Error of data consistency: N_PIG >N_TOTAL | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Service brake not commanded. | O | TIU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 2 | I am the 3th |
| N_TOTAL | 3 | 1 | 2 balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Same value as stored country/region identity NID_C(1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as stored VBC #1 set code |
| NID_PACKET | 8 | 254 | Default balise, loop and RIU information |
| Q_DIR | 2 | 01 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | I am the 1st |
| N_TOTAL | 3 | 1 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as stored VBC #1 set code |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded. | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.9: Virtual Balise Cover |  | 3150900 |
| Test Case of Feature | If the ERTMS/ETCS on-board equipment receives from balise a new VBC with the same VBC identity as an already stored VBC, the new VBC shall replace the previous one, including its validity period. |  | 3 |
| Applicable Mode/Level Combinations | L0: SH/ PS/ SL/ UN/ TR/ NL LNTC: SH/ PS/ SL/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment receives a balise message that includes a VBC order (packet number 6: “Virtual Balise Cover order” ) to set a VBC in nominal direction and with a validity period of 3 day.(T_VBC1) The on-board equipment receives a new VBC order to set a VBC (packet number 6) with the same VBC identity as the already stored VBC order (NID_VBCMK (1) =NID_VBCMK (2); NID_C (1)) but with a validity period of 1 day. (T_VBC2) The on-board equipment replaces the previous order with the new VBC order, including its validity period. The validity period, T_VBC2, of the stored VBC elapsed. (Hereafter, all the information received from Balise group that includes a VBC marker with same VBC identity is accepted and but not ignored by the on-board equipment because of his elapsed validity period.) A balise message from a balise group set with a VBC marker and same VBC identity as the stored one is received onboard. The message from this balise group contains a data consistency error .(Variables in the balise group message have invalid values) The on-board equipment reacts and applies service break. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.9.4 |  |
|  |  | Subset-026-3.16.2.4.4 |  |
|  |  | Subset-026-3.16.2.4.4 c) |  |
|  |  | Subset-026-4.8.3.1.1 Virtual Balise Cover order |  |
|  |  | Subset-026-4.8.4.2 Virtual Balise Cover order |  |
|  |  | Subset-026-7.4.2.0 Packet Number 0: Virtual Balise Cover marker |  |
|  |  | Subset-026-7.4.2.2.1 Packet Number 6: Virtual Balise Cover order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces TIU that: Service break is commanded due to a data consistency error. Check on the JRU that : Two VBC orders, packet 6, with same VBC identity but different validity period are RECORDED. A balise group message with variables that have invalid values is RECORDED. |
| Constraints | The VBCs and balises group applicable for same area (Country/ region). (NID_C) VBC order (2) same VBC identity as VBC order (1) NID_VBCMK(1)=NID_VBCMK(2) VBC order(2) has a validity period shorter than VBC order (1) TVBC(1) > T_VBC(2) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message BGa containing packet 6 (Q_DIR (1) =1; Q_VBCO=1; NID_VBCMK (1); T_VBC (1) =3; NID_C) is received. | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Balise Group message (N_TOTAL=1) containing packet 6 (Q_DIR (2) =1; Q_VBCO=1; NID_VBCMK (2); T_VBC (2) =1; NID_C) is received. | I | BTM | The on-board equipment replaces the previous one including its validity period. Variables from Packet 6: NID_VBCMK(2)= NID_VBCMK(1) TVBC(1) > T_VBC(2) | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The train is at standstill.(for one day) | I | INT | - | N/A | N/A | - |
| 6 | N/A | N/A | The train is moving. | I | INT | The validity period of stored VBC code elapsed. | N/A | N/A | - |
| 7 | N/A | N/A | Balise Group message BGb containing packet 0 (NID_VBCMK (3)) and packet 254 is received. | I | BTM | The information received from this balise group is used by the on-board unit. Variables from packet 0: NID_VBCMK(3)= NID_VBCMK(2) (identity deleted) | N/A | N/A | - |
| 8 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Service brake commanded. | O | TIU | The error of data consistency is: N_PIG >N_TOTAL | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake” is displayed. | O | DMI | Brake command indication due to RAMS related supervision (service brake) Symbol ST01 | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The system status message "Balise read error" is displayed. | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C(1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 6 | Virtual Balise Cover order |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_VBCO | 1 | 1 | Set the Virtual Balise Cover |
| NID_VBCMK | 6 | FINITE VALUE | NID_VBCMK (1) |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| T_VBC | 8 | FINITE VALUE | Only if Q_VBCO = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 6 | Virtual Balise Cover order |
| Q_DIR | 2 | 00 | Q_DIR (2) ≠ Q_DIR (1) |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_VBCO | 1 | 1 | Set the Virtual Balise Cover |
| NID_VBCMK | 6 | FINITE VALUE | NID_VBCMK(2) = NID_VBCMK(1) |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| T_VBC | 8 | FINITE VALUE | Only if Q_VBCO = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 2 | I am the 3th |
| N_TOTAL | 3 | 1 | 2 balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C(1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as set NID_VBCMK(2) |
| NID_PACKET | 8 | 254 | Default balise, loop and RIU information |
| Q_DIR | 2 | 01 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | I am the 1st |
| N_TOTAL | 3 | 1 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as set NID_VBCMK(2) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | Set by trackside VBC #1 set code: NID_VBCMK(2)+NID_C(1)+T_VBC(1) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake is commanded | O | TIU | - |
| The Status symbol "Service Brake intervention" is displayed The system status message "Balise read error" is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.9: Virtual Balise Cover |  | 3150900 |
| Test Case of Feature | A VBC shall be retained on-board when the on-board equipment is switched off (i.e. enters No Power mode) and shall remain applicable when powered on again. |  | 4 |
| Applicable Mode/Level Combinations | L0: SH/ UN/ PS/ SL/ TR/ NL LNTC: SH/ PS/ SL/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: A VBC identity is stored on board. The ERTMS/ETCS on-board equipment is switched off. The mode changes to NP. The VBC data remains retained onboard. Status of the VBC data is UNCHANGED in NP mode. The on-board equipment is powered on again. The on-board equipment is in SB mode with a desk open and the possibility of the driver to set/remove a VBC is available. The VBC remains applicable. Status of the stored VBC data in SB mode is UNCHANGED. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.9.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: When the driver request to set/remove the VBC a list with VALID VBC data is DISPLAYED. |
| Constraints | The VBC information is stored onboard. The train is at standstill. Enabling conditions for the driver possibility to select “Remove VBC” are: (train is at standstill) AND (mode is SB) AND (at least one VBC is stored on-board) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | VBC #1 set code |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS / OS/ SR/ SH/ UN/ PS / SL/ SB/ TR/ PT/ NL/ / SN/ RV | The train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L0/ LNTC/ L1/ L2/ L3 | FS/ LS / OS/ SR/ SH/ UN/ PS / SL/ SB/ TR/ PT/ NL/ / SN/ RV | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS / OS/ SR/ SH/ UN/ PS / SL/ SB/ TR/ PT/ NL/ / SN/ RV | The on-board equipment changes to NP mode. | - | - | USE_FT4060300.1 The VBC data stored onboard remains UNCHANGED. | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | NP | Use_FT4060300.10 | - | - | The on-board equipment changes to SB mode. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | SB | Use_FT5040300.24 | - | - | Driver opens desk | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver presses “Settings”. | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 6 | L0/ LNTC/ L1/ L2/ L3 | SB | The data from “Settings “ window are displayed. | O | DMI | At standstill and SB mode, the on-board offers the possibility to the driver to select to set/remove a VBC inside the “Settings window”. More possibilities for driver selection are available. For more details see S1 state from SoM procedure. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 7 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver presses “Remove VBC”. | I | DMI | Enabling conditions for “Remove VBC” are fulfilled. One VBC code is stored onboard. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 8 | L0/ LNTC/ L1/ L2/ L3 | SB | “Remove VBC window” is displayed. | O | DMI | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | Unchanged |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The “Remove VBC” window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.9: Virtual Balise Cover |  | 3150900 |
| Test Case of Feature | A VBC shall be deleted by the on-board equipment when it is ordered by trackside. |  | 5 |
| Applicable Mode/Level Combinations | L0: SH/ UN/ PS/ SL/ TR/ NL LNTC: SH/ PS/ SL/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment receives from BGa a telegram that includes a VBC order (packet number 6: “Virtual Balise Cover order”) to remove one of the VBC stored on-board. The new received VBC identity to be removed matches with one stored on-board. (NID_VBCMK(3)=NID_VBCMK(1); NID_C and T_VBC(3)=T_VBC(1)) The on-board equipment deletes this VBC identity. Thereafter, all the information received from balises marked to be ignored (same identity as the removed one) together with the area in which the VBC is applicable is used by the on-board equipment. The on-board receives from BGb a balise message containing packet number 0 (the VBC identity matches the deleted one) and packet number 254. The on-board uses the information received with packet 254 by displaying on DMI the message “Trackside malfunction”. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.9.5 |  |
|  |  | Subset-026-3.15.9.5 a) |  |
|  |  | Subset-026-3.16.2.4.4 |  |
|  |  | Subset-026-3.16.2.4.4 c) |  |
|  |  | Subset-026-3.16.2.4.9 |  |
|  |  | Subset-026-4.8.3.1.1 Virtual Balise Cover order |  |
|  |  | Subset-026-4.8.4.2 Virtual Balise Cover order |  |
|  |  | Subset-026-7.4.2.0 Packet Number 0: Virtual Balise Cover marker |  |
|  |  | Subset-026-7.4.2.2.1 Packet Number 6: Virtual Balise Cover order |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Message “Trackside malfunction” is DISPLAYED. Check on the JRU that : Remove VBC order, packet 6, with same VBC identity is RECORDED. A balise group message with packet 0 and 254 is RECORDED. |
| Constraints | VBC´s and balises group applicable for same country/region. At least two VBCs are stored onboard: VBC (1)= NID_VBCMK(1)+NID_C+T_VBC(1) VBC (2)= NID_VBCMK(2)+NID_C+T_VBC(2) The VBC information sent with the order to remove a VBC shall match at least one stored onboard. NID_VBCMK(3)=NID_VBCMK(1) T_VBC is not relevant when Q_VBCO set to 0 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | At least two VBCs stored on-board VBC #1 set code = NID_VBCMK(1)+NID_C(1)+T_VBC(1) VBC #2 set code = NID_VBCMK (2) +NID_C (1) +T_VBC (2)… |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1; Q_DIR= 1; NID_C=< NID_C (1)>) containing packet 6 (Q_VBCO=0; NID_VBCMK (3); NID_C (1)) is received. | I | BTM | The on-board equipment deleted the VBC #1 set code from the list. VBC #3 set code = VBC #1 set code | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Balise Group message (N_TOTAL=1; Q_DIR= 1; NID_C=< NID_C (1)>) containing packet 0 (NID_VBCMK (1)) and packet 254 is received. | I | BTM | Identity previously deleted NID_VBCMK(1) | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The system status message "Trackside malfunction" is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 6 | Virtual Balise Cover order |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_VBCO | 1 | 0 | Remove the Virtual Balise Cover (1) |
| NID_VBCMK | 6 | FINITE VALUE | NID_VBCMK (1) |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| T_VBC | 8 | FINITE VALUE | Only if Q_VBCO = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as the removed VBC identity (1) NID_VBCMK (1) |
| NID_PACKET | 8 | 254 | Default balise, loop and RIU information |
| Q_DIR | 2 | 01 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as the removed VBC identity (1) NID_VBCMK (1) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | CHANGED : Deleted VBC identity (1) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Trackside malfunction" is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.9: Virtual Balise Cover |  | 3150900 |
| Test Case of Feature | A VBC shall be deleted by the on-board equipment when its validity period has elapsed. |  | 6 |
| Applicable Mode/Level Combinations | L0: SH/ UN/ TR/ NL LNTC: SH/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ TR/ PT/ NL/ RV L2 / L3: FS/ LS/ OS/ SR/ SH/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: The validity period of the VBC (1) identity stored on board elapsed. The on-board equipment deletes VBC (1). ((Only telegrams send by balises group marked to be ignored but have the same identity as the deleted one, together with the area in which the VBC is applicable are not ignored by the on-board equipment) The on-board receives from BG a balise message containing packet number 0 (identity VBC (1) no longer stored) and packet number 254. The on-board uses the information received with packet 254 by displaying on DMI the message “Trackside malfunction”. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.9.5 |  |
|  |  | Subset-026-3.15.9.5 b) |  |
|  |  | Subset-026-3.16.2.4.4 |  |
|  |  | Subset-026-3.16.2.4.4 c) |  |
|  |  | Subset-026-3.16.2.4.9 |  |
|  |  | Subset-026-4.8.3.1.1 Virtual Balise Cover order |  |
|  |  | Subset-026-4.8.4.2 Virtual Balise Cover order |  |
|  |  | Subset-026-7.4.2.0 Packet Number 0: Virtual Balise Cover marker |  |
|  |  | Subset-026-7.4.2.2.1 Packet Number 6: Virtual Balise Cover order |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Message “Trackside malfunction” is DISPLAYED. Check on the JRU that : A balise group message with packet 0 and 254 is RECORDED. |
| Constraints | Two different VBC are stored on-board. VBC #1 ≠ VBC #2 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 7/ 8 / 11/ 12/ 13/ 14 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | VBC #1 set code VBC #2 set code |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train is at standstill | I | INT | (V_TRAIN=0; TIME= 1 day) | N/A | N/A | - |
| 2 | N/A | N/A | Driver presses “Data view”. | I | DMI | After 1 day | N/A | N/A | - |
| 3 | N/A | N/A | The data from "Data View" window are displayed | O | DMI | Only “VBC #2 set code” is displayed. VBC #1 set code removed by on-board equipment because T_VBC(1) has elapsed | N/A | N/A | - |
| 4 | N/A | N/A | The driver closes the "Data View" window. | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The train is moving. | I | INT | - | N/A | N/A | - |
| 6 | N/A | N/A | Balise Group message (N_TOTAL=1; Q_DIR= 1; NID_C=< NID_C (1)>) containing packet 0 (NID_VBCMK (1)) and packet 254 is received. | I | BTM | NID_VBCMK(1) =VBC #1 set code The on-board equipment use the information received from this BG. | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The system status message "Trackside malfunction" is displayed. | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded. | O | JRU | - | N/A | N/A | - |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as the deleted VBC #1 set code NID_VBCMK(1) |
| NID_PACKET | 8 | 254 | Default balise, loop and RIU information |
| Q_DIR | 2 | 01 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as the deleted VBC #1 set code NID_VBCMK(1) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 7/ 8 / 11/ 12/ 13/ 14 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | CHANGED VBC #2 set code |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Trackside malfunction" is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.9: Virtual Balise Cover |  | 3150900 |
| Test Case of Feature | The VBC shall be deleted by the on-board equipment when a mismatch is detected between the country/region identity read from a balise group and the country/region identity of the VBC. Note: this means that the reception of a consistent balise group message is a necessary condition for deleting a VBC due to mismatching country/region identities. |  | 7 |
| Applicable Mode/Level Combinations | L0: SH/ UN/ TR/ NL LNTC: SH/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ TR/ PT/ NL/ RV L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment receives from BGa a consistent balise message that includes a VBC identity (packet 0) that contains a mismatch between the country/region identity read from this balise group and the country/region identity of the VBC stored onboard. Packet number 254 also included in the telegram. The on-board equipment deletes all VBC applicable for a different area than the last received one. The driver is informed trough DMI with the message “Trackside malfunction”. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.9.5 |  |
|  |  | Subset-026-3.15.9.5 d) |  |
|  |  | Subset-026-3.16.2.4.9 |  |
|  |  | Subset-026-7.4.2.0 Packet Number 0: Virtual Balise Cover marker |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Message “Trackside malfunction” is DISPLAYED. Check on the JRU that : A balise group message, with different country/region identity, containing packet 0 and 254 is RECORDED. |
| Constraints | This mismatch of country/region can be generated due to a change of train location (other country) and all EVC cabin are not active during the change of train location. At least one VBC is stored onboard (not more than 30 VBCs): VBC #1 set code = NID_VBCMK(1)+NID_C(1)+T_VBC(1) VBC #2 set code = NID_VBCMK(2)+NID_C(1)+T_VBC(1) The VBC information received from the trackside mismatch the one stored onboard. NID_C(2)≠ NID_C(1) Comment: If more VBCs are stored on-board, all VBC identities corresponding to NID_C (1) are deleted from the list by the on-board equipment after reading first balise with a VBC identity applicable for another area (country/region NID_C (2)). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | VBC #1 set code (NID_C(1)) VBC #2 set code (NID_C(1)) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=2; Q_DIR= 1; NID_C=< NID_C (2)>) containing packet 0 and packet 254 is received. | I | BTM | A mismatch between the country/region identity read from this balise group and the country/region identity of the VBC stored onboard. NID_C(2)≠ NID_C(1) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The system status message "Trackside malfunction" is displayed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Another country/ region NID_C (2) |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 254 | Default balise, loop and RIU information |
| Q_DIR | 2 | 01 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Another country/ region NID_C (2) |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Trackside malfunction" is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.9: Virtual Balise Cover |  | 3150900 |
| Test Case of Feature | A message transmitted by a balise group contains several packets 0. A message transmitted by a balise group contains several packets 6. The on-board use all received information in a correct way. VBCs shall be deleted by the on-board equipment when it is ordered by trackside. |  | 8 |
| Applicable Mode/Level Combinations | L0: SH/ UN/ PS/ SL/ TR/ NL LNTC: SH/ PS/ SL/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment receives from BGa a telegram that includes several VBC orders (two packets number 6: “Virtual Balise Cover order”) to set and remove VBCs. The identities (pairs of variables NID_VBCMK and NID_C) of the corresponding VBC transmitted in the same message are different. The on-board equipment sets one VBC identities that does not match with the ones stored on-board and removes one from the already stored ones. (NID_VBCMK(3) new and stored on-board; NID_VBCMK(4)=NID_VBCMK(2) and it is removed) Thereafter, all the information received from balises marked to be ignored (same identity as the removed one) together with the area in which the VBC is applicable is will be used by the on-board equipment. The on-board receives from BGb a balise message containing two packets number 0 (the VBC identity matches the deleted one) and packet number 254. The on-board uses the information received with packet 254 by displaying on DMI the message “Trackside malfunction”. The on-board accepts duplicated packets 0 (8.4.1.4.8) and 6 (8.4.1.4.9). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.9.5 |  |
|  |  | Subset-026-3.15.9.5 a) |  |
|  |  | Subset-026-3.16.2.4.9 |  |
|  |  | Subset-026-4.8.3.1.1 Virtual Balise Cover order |  |
|  |  | Subset-026-4.8.4.2 Virtual Balise Cover order |  |
|  |  | Subset-026-7.4.2.0 Packet Number 0: Virtual Balise Cover marker |  |
|  |  | Subset-026-7.4.2.2.1 Packet Number 6: Virtual Balise Cover order |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Message “Trackside malfunction” is DISPLAYED. Check on the JRU that : Set and Remove VBC order, two packets 6, are RECORDED. A balise group message with two packets 0 and 254 is RECORDED. |
| Constraints | VBC´s and balises group applicable for same country/region. At least two VBCs are stored on-board: VBC (1)= NID_VBCMK(1)+NID_C+T_VBC(1) VBC (2)= NID_VBCMK(2)+NID_C+T_VBC(2) The VBC information sent with the order to remove a VBC shall match at least one stored on-board. NID_VBCMK(4)=NID_VBCMK(2) T_VBC is not relevant when Q_VBCO set to 0 No linking information is used |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | At least two VBCs stored on-board VBC #1 set code = NID_VBCMK(1)+NID_C(1)+T_VBC(1) VBC #2 set code = NID_VBCMK (2) +NID_C (1) +T_VBC (2)… |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1; Q_DIR= 1; NID_C=< NID_C (1)>) containing packet 6 with (Q_VBCO=1; NID_VBCMK (3); NID_C (2)) and packet 6 with (Q_VBCO=0; NID_VBCMK (4); NID_C (1)) is received. | I | BTM | The on-board equipment : Deletes the VBC #2 set code from the list because VBC # 4 set code = VBC #2 set code And Sets the new VBC # 3 set code for a different country/region. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Balise Group message (N_TOTAL=1; Q_DIR= 1; NID_C=< NID_C (1)>) containing two packet 0 (NID_VBCMK (2)) and packet 254 is received. | I | BTM | Identity previously deleted NID_VBCMK(2) | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The system status message "Trackside malfunction" is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 6 | Virtual Balise Cover order |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_VBCO | 1 | 1 | Set the Virtual Balise Cover (3) |
| NID_VBCMK | 6 | FINITE VALUE | NID_VBCMK (3) |
| NID_C | 10 | FINITE VALUE | NID_C (2) |
| T_VBC | 8 | FINITE VALUE | Set to Several days |
| NID_PACKET | 8 | 6 | Virtual Balise Cover order |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_VBCO | 1 | 0 | Remove the Virtual Balise Cover (2) |
| NID_VBCMK | 6 | FINITE VALUE | NID_VBCMK(4) = NID_VBCMK(2) |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| T_VBC | 8 | FINITE VALUE | Only if Q_VBCO = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | I am the 1st |
| N_TOTAL | 3 | 1 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as the removed VBC identity (2) NID_VBCMK (2) |
| NID_PACKET | 8 | 254 | Default balise, loop and RIU information |
| Q_DIR | 2 | 01 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | I am the 2nd |
| N_TOTAL | 3 | 1 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C (1) |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | Same value as the removed VBC identity (2) NID_VBCMK (2) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 3 / 4/ 5/ 7/ 8 / 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | CHANGED : Deleted VBC #2 set code Stored: VBC #3 set code = NID_VBCMK(3)+NID_C(2)+T_VBC(2) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Trackside malfunction" is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |