| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080428 (4.8.4: Accepted Information depending on the modes - Position Report Parameters) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-4080428 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 30.08.2013 | All | General review | CEDEX |
| Version 2.9.2 26.11.2013 | TC3 | Review | CEDEX |
| Version 2.9.3 29.11.2013 | All | Update of "USE_FT" steps | CEDEX |
| Version 2.9.4 10.12.2013 | TCs 3, 4 | Editorial change (RTM tables mismatch) | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Position Report Parameters |  | 4080428 |
| Test Case of Feature | The train receives a message including Packet 58 (Position report parameters) that is transmitted from RBC. The message is ACCEPTED. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN, NL, SB LNTC: SN, NL, SB L1: FS, LS, OS, SR, SB, NL, RV L2: FS, LS, OS, SR, SB, PT, NL, RV L3: FS, LS, OS, SR, SB, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that The message including packet 58 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Position Report Parameters |  |
|  |  | Subset-026-4.8.4.2 Position Report Parameters |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position Report Parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 58 (Position report parameters ) is recorded. |
| Constraints | A cab needs to be active (SB Mode). In PT mode, the reception of packet 58 has to follow the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SB / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Position Report parameters | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 is received | I | RTM | Complete message is accepted | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded, Message 24 and packet 58 sents by RBC | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.15-21: the train informs the trackside according to parameters transmitted from trackside | - | - | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | FINITE VALUE | - |
| D_CYCLOC | 15 | FINITE VALUE | - |
| M_LOC | 3 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LOC(k) | 15 | FINITE VALUE | - |
| Q_LGTLOC(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SB / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Position Report parameters | STORED | Position report parameters have been received. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Position Report Parameters |  | 4080428 |
| Test Case of Feature | The train receives a message including Packet 58 (Position report parameters) that is transmitted from RBC. The message is REJECTED. |  | 2 |
| Applicable Mode/Level Combinations | L0: SH / PS / SL / TR LNTC: SL / TR / PS / SH L1: SH / PS / SL / TR L2: SH / PS / SL / TR L3: SH / PS / SL / TR |  |  |
| Target of Test | The target of this test is to check that The message including packet 58 is rejected, not stored on-board but recorded in the JRU |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Position Report Parameters |  |
|  |  | Subset-026-4.8.4.2 Position Report Parameters |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position Report Parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 58 (Position report parameters ) is recorded. Check that: No Position Report Parameters packet is stored onboard |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 7 / 15 | SH / SL / TR / PS |
| Radio communication session | ESTABLISHED | - |
| Position Report parameters | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 is received | I | RTM | Complete message is rejected. | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded, Message 24 and packet 58 sents by RBC | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | Use_FT4060300.62 to change to SH mode (Transition [23]) | - | - | If in PS mode | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | Use_FT4060300.18 to change to SB mode (Transition [19]) | - | - | If in SH mode | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | Use_FT4060300.7 to change to SB mode (Transition [2]) | - | - | If in SL mode | L0 / LNTC / L1 / L2 / L3 | SH / PS / SL / TR | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | SB / TR | Not SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | - | L0 / LNTC / L1 / L2 / L3 | SB / TR | - |
| 7 | L0 / LNTC / L1 / L2 / L3 | SB / TR | Not MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=136) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB / TR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | FINITE VALUE | - |
| D_CYCLOC | 15 | FINITE VALUE | - |
| M_LOC | 3 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LOC(k) | 15 | FINITE VALUE | - |
| Q_LGTLOC(k) | 1 | FINITE VALUE | - |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7 | SB / TR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Position Report Parameters |  | 4080428 |
| Test Case of Feature | The train is at standstill in SL mode and the remote control signal of SL is lost. The on-board changes to SB and a communication session is opened. The desk is still closed and the train receives a message including Packet 58 (Position report parameters) that is transmitted from the RBC. The message is REJECTED. |  | 3 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that The message including packet 58 is rejected when it is received in SB and the desk is closed |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.3.1.1 Position Report Parameters |  |
|  |  | Subset-026-4.8.4.2 Position Report Parameters |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position Report Parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 58 (Position report parameters ) is recorded. Check on the RTM that: No message 136 is sent by the on-board after receiving packet 58 |
| Constraints | The train is at standstill. The previous mode was SL and a communication session was open when the Sleeping signal was lost and the on-board changed to SB mode. The desk is closed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | It was established when the Sleeping signal was lost (the previous mode was SL) and the mode changed to SB mode. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The cab is not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24; M_LOC = 000; and T_CYCLOC = 00001010) containing packet 58 is received | I | RTM | The complete message is rejected. With M_LOC = 000 (Now), and T_CYCLOC = 10 s. | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 4 | N/A | N/A | Not SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | Check that no message 136 is sent after the reception of Packet 58, nor every 10 seconds. | N/A | N/A | - |
| 5 | N/A | N/A | Not MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | FINITE VALUE | - |
| D_CYCLOC | 15 | FINITE VALUE | - |
| M_LOC | 3 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LOC(k) | 15 | FINITE VALUE | - |
| Q_LGTLOC(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Position Report parameters | STORED | M_LOC = 000 (Now), and T_CYCLOC = 10 s |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The cab is not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |