| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080444 (4.8.4: Accepted Information depending on the modes - Track Condition big metal masses) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-4080444 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 27.11.2013 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Condition big metal masses |  | 4080444 |
| Test Case of Feature | Acceptance of message including packet 67 (Track Condition Big Metal Masses) that is transmitted via Balise Group. |  | 1 |
| Applicable Mode/Level Combinations | L0: SH, UN, SL, SB, TR, NL, PS L1/L2/L3: FS, OS, SR, SH, SL, SB, TR, PT, NL, LS, PS LNTC: SH, SL, SB, TR, NL, SN, PS |  |  |
| Target of Test | The target of this test is to check that a message including packet 67 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-3.12.1.2.1.2 |  |
|  |  | Subset-026-4.4.17.1.2 |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Condition Big Metal Masses |  |
|  |  | Subset-026-4.8.3.1.1 Track Condition big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track condition big metal masses |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.19 Packet Number 67: Track Conditions Big Metal Masses |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check the acceptance of the information, this test case use the ETCS functionality "4.5.2.1 table - Manage Track Condition Big Metal Masses". Check on the JRU that: - Message including packet 67 (Track condition Big Metal masses) is recorded. - Brakes application are NOT recorded. Check on the TIU: - Brakes are NOT applied. Check on the DMI: - There are NO modifications on the information displayed. |
| Constraints | In order to see that the information is used on-board, it is assumed that the default on-board reaction to the BTM integrity alarm is the application of the Emergency Brake. This assumption is based on the experience but it is not clearly described in the specifications. In case the ERTMS/ETCS on-board equipment provides another reaction, the TC will have to be modified accordingly. For PT mode the information "Recognition of exit from TR mode" shall be stored |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / PS |
| Track condition big metal masses | NOT STORED | - |
| Train Data | STORED | To fulfill condition [4] for SB in 4.8.4.2 Track condition big metal masses |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 67 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=67) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN < V_PERMITTED) | I | INT | The train enters into the big metal masses area. | N/A | N/A | - |
| 4 | N/A | N/A | BTM alarm due to big metal masses is received | I | INT | The BTM triggers an internal alarm due to big metal masses. | N/A | N/A | - |
| 5 | N/A | N/A | NOT Emergency brake commanded | O | TIU | There is no on-board reaction to the BTM integrity alarm. The emergency brake reaction here prescribed is just an assumption based on experience, but not mandatory in the specifications. | N/A | N/A | - |
| 6 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving (V_TRAIN < V_PERMITTED) | I | INT | The train leaves the big metal masses area. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 67 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / PS |
| Track condition big metal masses | NOT STORED | - |
| Train Data | STORED | To fulfill condition [4] for SB in 4.8.4.2 Track condition big metal masses |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Condition big metal masses |  | 4080444 |
| Test Case of Feature | Acceptance of message including packet 67 (Track Condition Big Metal Masses) that is transmitted via in-fill Balise Group. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that an in-fill message including packet 67 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-3.12.1.2.1.2 |  |
|  |  | Subset-026-4.4.17.1.2 |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Condition Big Metal Masses |  |
|  |  | Subset-026-4.8.3.1.1 Track Condition big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track condition big metal masses |  |
|  |  | Subset-026-7.4.2.19 Packet Number 67: Track Conditions Big Metal Masses |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check the acceptance of the information, this test case use the ETCS functionality "4.5.2.1 table - Manage Track Condition Big Metal Masses". Check on the JRU that: - Message including packets 136 and 67 (Track condition Big Metal masses) is recorded. - Brakes application are NOT recorded. Check on the TIU: - Brakes are NOT applied. Check on the DMI: - There are NO modifications on the information displayed. |
| Constraints | In order to see that the information is used on-board, it is assumed that the default on-board reaction to the BTM integrity alarm is the application of the Emergency Brake. This assumption is based on the experience but it is not clearly described in the specifications. In case the ERTMS/ETCS on-board equipment provides another reaction, the TC will have to be modified accordingly. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Including the Main Balise group |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Track condition big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 136 and packet 67 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=136; NID_PACKET=67) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN < V_PERMITTED) | I | INT | The location of the main BG is passed without Reading any BG message" | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving (V_TRAIN < V_PERMITTED) | I | INT | The train enters into the big metal masses area. | N/A | N/A | - |
| 5 | N/A | N/A | BTM alarm due to big metal masses is received | I | INT | The BTM triggers an internal alarm due to big metal masses. | N/A | N/A | - |
| 6 | N/A | N/A | NOT Emergency brake commanded | O | TIU | There is no on-board reaction to the BTM integrity alarm. The emergency brake reaction here prescribed is just an assumption based on experience, but not mandatory in the specifications. | N/A | N/A | - |
| 7 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving (V_TRAIN < V_PERMITTED) | I | INT | The train leaves the big metal masses area. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 67 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Condition big metal masses |  | 4080444 |
| Test Case of Feature | Supervision at the entry and the exit of the track condition big metal masses area. |  | 3 |
| Applicable Mode/Level Combinations | L0: SH, UN, SL, SB, TR, NL, PS L1/L2/L3: FS, OS, SR, SH, SL, SB, TR, PT, NL, LS, PS LNTC: SH, SL, SB, TR, NL, SN, PS |  |  |
| Target of Test | The target of this test is to check that the on-board equipment supervises the entry in the Big Metal masses area with the maximum safe antenna position and the exit with the minimum safe antenna position. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-3.12.1.2.1.2 |  |
|  |  | Subset-026-4.4.17.1.2 |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Condition Big Metal Masses |  |
|  |  | Subset-026-4.8.3.1.1 Track Condition big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track condition big metal masses |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.19 Packet Number 67: Track Conditions Big Metal Masses |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check the acceptance of the information, this test case uses the ETCS functionality "4.5.2.1 table - Manage Track Condition Big Metal Masses". Besides, in order to check the limits at the entry and the exit of the big metal masses area, the train will be requested to stop at places where the estimated antenna position is outside the area, but within the additional boundaries provided by the confidence interval. Check on the JRU that: - Message including packet 67 (Track condition Big Metal masses) is recorded. - Brakes application are NOT recorded. Check on the TIU: - Brakes are NOT applied. Check on the DMI: - There are NO modifications on the information displayed. |
| Constraints | In order to see that the information is used on-board, it is assumed that the default on-board reaction to the BTM integrity alarm is the application of the Emergency Brake. This assumption is based on the experience but it is not clearly described in the specifications. In case the ERTMS/ETCS on-board equipment provides another reaction, the TC will have to be modified accordingly. In order to ease the test case implementation, it is required for the on-board equipment a "big" confidence interval. For PT mode the information "Recognition of exit from TR mode" shall be stored |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / PS |
| Track condition big metal masses | NOT STORED | - |
| Train Data | STORED | To fulfill condition [4] for SB in 4.8.4.2 Track condition big metal masses |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 67 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=67) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | The current location of the train is such that the train antenna position is outside the begining of the big metal masses area but at a distance less than the current confidence interval on-board. | N/A | N/A | - |
| 4 | N/A | N/A | BTM alarm due to big metal masses is received | I | INT | The BTM triggers an internal alarm due to big metal masses. | N/A | N/A | - |
| 5 | N/A | N/A | NOT Emergency brake commanded | O | TIU | There is no on-board reaction to the BTM integrity alarm. The emergency brake reaction here prescribed is just an assumption based on experience, but not mandatory in the specifications. | N/A | N/A | - |
| 6 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving (V_TRAIN < V_PERMITTED) | I | INT | The train approaches the end of the big metal masses area. | N/A | N/A | - |
| 9 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | The current location of the train is such that the train antenna position is outside the end of the big metal masses area but at a distance less than the current confidence interval on-board. | N/A | N/A | - |
| 10 | N/A | N/A | BTM alarm due to big metal masses is received | I | INT | The BTM triggers an internal alarm due to big metal masses. | N/A | N/A | - |
| 11 | N/A | N/A | NOT Emergency brake commanded | O | TIU | There is no on-board reaction to the BTM integrity alarm. The emergency brake reaction here prescribed is just an assumption based on experience, but not mandatory in the specifications. | N/A | N/A | - |
| 12 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | Train is moving (V_TRAIN < V_PERMITTED) | I | INT | The train leaves the big metal masses area. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 67 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / PS |
| Track condition big metal masses | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |