| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080442 (4.8.4: Accepted Information depending on the modes - Track Ahead Free Request) Total: 9 Test case(s) |  |
| REF: | Subset-076-5-2-4080442 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 25.10.2013 | All | REVIEW N/A | CEDEX |
| Version 2.9.3 06.12.2013 | All | REVIEW N/A | CEDEX |
| Version 2.9.4 12.12.2013 | All | Editorial changes | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Acceptance of track ahead Free Request information. The train is under the supervision of an RBC. It receives a message 34 ‘Track Ahead Free Request’ from the supervising RBC. The part of the line where the EUROCAB equipment has to display the ‘Track ahead Free Request’ to the driver is ahead the position of the train. The driver acknowledges when the ‘Track Ahead Free Request’ is displayed to him. The On-Board equipment sends a message 149 ‘Track Ahead Free Granted’ to the supervising RBC, indicating that he confirms that the track ahead the train is free. Track ahead free request is removed after the driver has acknowledged. |  | 1 |
| Applicable Mode/Level Combinations | L2: LS, OS SR, SB L3: LS, OS SR, SB |  |  |
| Target of Test | To check that the Track ' Ahead Free Request' is presented to the driver at the indicated position. To check that the train sends a message ‘Track Ahead Free Granted’ on the Driver’s acknowledgement of a ‘Track Ahead Free Request’. To check that the Track ahead free request is removed after the driver has acknowledged.. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-3.15.5.2 |  |
|  |  | Subset-026-3.15.5.2 a) |  |
|  |  | Subset-026-3.15.5.3 |  |
|  |  | Subset-026-3.15.5.4 |  |
|  |  | Subset-026-3.15.5.2 |  |
|  |  | Subset-026-4.4.11.1.9 |  |
|  |  | Subset-026-4.4.12.1.5 |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.6.9 Message 149: Track Ahead Free Granted |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the Track ' Ahead Free Request' is presented to the driver at the indicated position. Check on the JRU To check that the train sends a message ‘Track Ahead Free Granted’ on the Driver’s acknowledgement of a ‘Track Ahead Free Request’. Check on the JRU and DMI that Track ahead free request is removed after the driver has acknowledged.. |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | When the train reaches the D_TAFDISPLAY location | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;DMI_SYMB_STATUS=<Bit82=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver confirms "Track ahead is free" | I | DMI | Before the train reaches the D_TAFDISPLAY + L_TAFDISPLAY location | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Track Ahead Free Granted" (NID_MESSAGE=149) is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=149) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is removed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;DMI_SYMB_STATUS=<Bit82=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 7: Radio Message 149 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 149 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Acceptance of track ahead Free Request information .The train is running under the supervision of an RBC. It receives a message 34 ‘Track Ahead Free Request’ from the supervising RBC. The part of the line where the EUROCAB equipment has to display the ‘Track ahead Free Request’ to the driver is ahead the position of the train and inside the Movement Authority. The driver does not acknowledge when the ‘Track Ahead Free Request’ is displayed to him. The On-Board equipment does not send any message 149 ‘Track Ahead Free Granted’ to the supervising RBC, indicating that he does not confirm that the track ahead the train is free. Track ahead free request is removed after the train has left the indicated area There are no restrictive consequence by the onboard system if the driver does not acknowledge. |  | 2 |
| Applicable Mode/Level Combinations | L2: LS, OS, SR L3: LS, OS, SR |  |  |
| Target of Test | To check that the Track ' Ahead Free Request' is presented to the driver at the indicated position. To check that the train does not send a message ‘Track Ahead Free Granted’ if the Driver does not acknowledge a ‘Track Ahead Free Request’. To check that the Track ahead free request is removed after the train has left the indicated area. To check that there are no restrictive consequence by the onboard system if the driver does not acknowledge. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-3.15.5.2 |  |
|  |  | Subset-026-3.15.5.2 a) |  |
|  |  | Subset-026-3.15.5.2 b) |  |
|  |  | Subset-026-3.15.5.3 |  |
|  |  | Subset-026-3.15.5.5 |  |
|  |  | Subset-026-4.4.11.1.9 |  |
|  |  | Subset-026-4.4.12.1.5 |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the Track ' Ahead Free Request' is presented to the driver at the indicated position. Check on the JRU that the train does not send a message ‘Track Ahead Free Granted’ if the Driver does not acknowledge a ‘Track Ahead Free Request’. Check on the JRU and DMI that the Track ahead free request is removed after the train has left the indicated area. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 12 | OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | When the train reaches the D_TAFDISPLAY location | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;DMI_SYMB_STATUS=<Bit82=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver confirms "Track ahead is free" | I | DMI | There shall be no restrictive consequence by the onboard system if the driver does not acknowledge. | N/A | N/A | - |
| 6 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is removed | O | DMI | When the train reaches the D_TAFDISPLAY + L_TAFDISPLAY location | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;DMI_SYMB_STATUS=<Bit82=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 12 | OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Replace of track ahead Free Request information. The train is under the supervision of an RBC. It receives a message 34 ‘Track Ahead Free Request’ from the supervising RBC. A new message 34 ‘Track Ahead Free Request’ from the supervising RBC. The part of the line where the EUROCAB equipment has to display the ‘Track ahead Free Request’ to the driver is ahead the position of the train. The driver acknowledges when the ‘Track Ahead Free Request’ is displayed to him (from the last message). The On-Board equipment sends a message 149 ‘Track Ahead Free Granted’ to the supervising RBC, indicating that he confirms that the track ahead the train is free. Track ahead free request is removed after the driver has acknowledged. |  | 3 |
| Applicable Mode/Level Combinations | L2: LS, OS SR, SB L3: LS, OS SR, SB |  |  |
| Target of Test | To check that the Track ' Ahead Free Request' is presented to the driver at the indicated position of the second message. To check that the train sends a message ‘Track Ahead Free Granted’ on the Driver’s acknowledgement of a ‘Track Ahead Free Request’. To check that the Track ahead free request is removed after the driver has acknowledged.. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-3.15.5.2 |  |
|  |  | Subset-026-3.15.5.2 a) |  |
|  |  | Subset-026-3.15.5.3 |  |
|  |  | Subset-026-3.15.5.4 |  |
|  |  | Subset-026-3.15.5.6 |  |
|  |  | Subset-026-3.15.5.2 |  |
|  |  | Subset-026-4.4.11.1.9 |  |
|  |  | Subset-026-4.4.12.1.5 |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |
|  |  | Subset-026-8.6.9 Message 149: Track Ahead Free Granted |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the Track ' Ahead Free Request' is presented to the driver at the indicated position of the second message. Check on the JRU To check that the train sends a message ‘Track Ahead Free Granted’ on the Driver’s acknowledgement of a ‘Track Ahead Free Request’. Check on the JRU and DMI that Track ahead free request is removed after the driver has acknowledged.. |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | WITH D_TAFDISPLAY=D1 L_TAFDISPLAY=100 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | WITH D_TAFDISPLAY=D2>D1 +100 L_TAFDISPLAY | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is NOT displayed | O | DMI | When the train reaches the D_TAFDISPLAY= D1 location | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving | I | INT | Arriving D=D2 | N/A | N/A | - |
| 7 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | When the train reaches the D_TAFDISPLAY= D2 location | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;DMI_SYMB_STATUS=<Bit82=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver confirms "Track ahead is free" | I | DMI | Before the train reaches the D_TAFDISPLAY + L_TAFDISPLAY location | N/A | N/A | - |
| 10 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Track Ahead Free Granted" (NID_MESSAGE=149) is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=149) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is removed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;DMI_SYMB_STATUS=<Bit82=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | D1 | - |
| D_TAFDISPLAY | 15 | 100 | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 3: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | D2 | - |
| D_TAFDISPLAY | 15 | 100 | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 11: Radio Message 149 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 149 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Acceptance of track ahead Free Request information.The train is running under the supervision of an RBC. It receives a message 34 ‘Track Ahead Free Request’ from the supervising RBC. The part of the line where the EUROCAB equipment has to display the ‘Track ahead Free Request’ to the driver is behind the position of the train. The On-Board equipment does not ever display the ‘Track Ahead Free Request’ to the driver and the On-Board equipment does not send any message 149 ‘Track Ahead Free Granted’ to the supervising RBC, indicating that he does not confirm that the track ahead the train is free. |  | 4 |
| Applicable Mode/Level Combinations | L2: LS, OS SR, SB L3: LS, OS SR, SB |  |  |
| Target of Test | To check that the train does not display any ‘Track Ahead Free Request’ to the driver when the train is beyond the displaying area. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-3.15.5.2 |  |
|  |  | Subset-026-3.15.5.2 a) |  |
|  |  | Subset-026-3.15.5.2 b) |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the train does not display any ‘Track Ahead Free Request’ to the driver when the train is beyond the displaying area. |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | After it has passed the D_TAFDISPLAY + L_TAFDISPLAY location. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Acceptance of track ahead Free Request information .The train is in the Post Trip mode under the supervision of an RBC. It receives a message 6 ‘Recognition of exit from Trip mode’ and then a message 34 ‘Track Ahead Free Request’ from the supervising RBC. The message 34 has a more recent time stamp that the message 6. The position of the train is included in the part of the line where the EUROCAB equipment has to display the ‘Track ahead Free Request’ to the driver. The driver acknowledges when the ‘Track Ahead Free Request’ is displayed to him. The On-Board equipment sends a message 149 ‘Track Ahead Free Granted’ to the supervising RBC, indicating that he confirms that the track ahead the train is free. |  | 5 |
| Applicable Mode/Level Combinations | L2 / L3: PT |  |  |
| Target of Test | To check that the Track ' Ahead Free Request' is presented after receiving the message To check that the train sends a message ‘Track Ahead Free Granted’ on the Driver’s acknowledgement of a ‘Track Ahead Free Request’, when the message 6 is older than the message 34. To check that the Track ahead free request is removed after the driver has acknowledged. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-3.15.5.2 |  |
|  |  | Subset-026-3.15.5.2 a) |  |
|  |  | Subset-026-3.15.5.3 |  |
|  |  | Subset-026-3.15.5.4 |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |
|  |  | Subset-026-8.6.9 Message 149: Track Ahead Free Granted |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the Track ' Ahead Free Request' is presented after receiving the message Check on the JRU that the train sends a message ‘Track Ahead Free Granted’ on the Driver’s acknowledgement of a ‘Track Ahead Free Request’, when the message 6 is older than the message 34. Check on the JRU and DMI that the Track ahead free request is removed after the driver has acknowledged. |
| Constraints | The train has received a message 6 ‘Recognition of exit from Trip mode |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | The value of the variable T_TRAIN indicates a more recent time than the value of the variable T_TRAIN included in the message 6. The train minimum safe front end position is between the D_TAFDISPLAY location and the D_TAFDISPLAY + L_TAFDISPLAY | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;DMI_SYMB_STATUS=<Bit82=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver confirms "Track ahead is free" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Track Ahead Free Granted" (NID_MESSAGE=149) is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=149) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is removed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;DMI_SYMB_STATUS=<Bit82=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 7: Radio Message 149 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 149 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Acceptance of track ahead Free Request information. The train is in the Post Trip mode under the supervision of an RBC. It receives a message 6 ‘Recognition of exit from Trip mode’ and then a message 34 ‘Track Ahead Free Request’ from the supervising RBC. The message 34 has a more recent time stamp that the message 6. The position of the train is not included in the part of the line where the EUROCAB equipment has to display the ‘Track ahead Free Request’ to the driver. The ‘Track Ahead Free Request’ is not displayed to the driver. The driver does not acknowledge and the On-Board equipment does not send any message 149 ‘Track Ahead Free Granted’ to the supervising RBC. |  | 6 |
| Applicable Mode/Level Combinations | L2 / L3: PT |  |  |
| Target of Test | To check that the On-Board equipment does not display any ‘Track Ahead Free Request’ to the Driver when the train minimum front end position is outside the interval defined by the D_TAFDISPLAY location and the D_TAFDISPLAY + L_TAFDISPLAY one |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-3.15.5.2 |  |
|  |  | Subset-026-3.15.5.2 a) |  |
|  |  | Subset-026-3.15.5.2 b) |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the On-Board equipment does not display any ‘Track Ahead Free Request’ to the Driver when the train minimum front end position is outside the interval defined by the D_TAFDISPLAY location and the D_TAFDISPLAY + L_TAFDISPLAY one |
| Constraints | The train has received a message 6 ‘Recognition of exit from Trip mode’ older than the message 34 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | The value of the variable T_TRAIN indicates a more recent time than the value of the variable T_TRAIN included in the message 6. The train minimum safe front end position is outside the interval defined by the D_TAFDISPLAY location and the D_TAFDISPLAY + L_TAFDISPLAY one | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | The ‘Track Ahead Free Request’ is not displayed to the driver | N/A | N/A | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Rejection of track ahead Free Request information. The train is in the Post Trip mode under the supervision of an RBC. It receives a message 34 ‘Track Ahead Free Request’ from the supervising RBC before a message 6 ‘Recognition of exit from Trip mode’ . The ‘Track Ahead Free Request’ is not displayed to the driver when the minimum safe front end position is within the interval defined by the [D_TAFDISPLAY, D_TAFDISPLAY + L_TAFDISPLAY]. The driver does not acknowledge and the On-Board equipment does not send any message 149 ‘Track Ahead Free Granted’ to the supervising RBC. |  | 7 |
| Applicable Mode/Level Combinations | L2 / L3: PT |  |  |
| Target of Test | To check that the On-Board equipment does not display any ‘Track Ahead Free Request’ to the Driver when the message 6 is received after the message 34. This Test negative condition of Subset-026-4.8.4.2 [1] |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the On-Board equipment does not display any ‘Track Ahead Free Request’ to the Driver when the message 6 is received after the message 34. |
| Constraints | The train has received a message 34 ‘Track Ahead Free Request’ before message 6 ‘Recognition of exit from Trip mode’ |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | The variables D_TAFDISPLAY and L_TAFDISPLAY included in message 34 are used to define the interval [D_TAFDISPLAY, D_TAFDISPLAY + L_TAFDISPLAY] where ‘Track Ahead Free Request’ should have to be displayed | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Recognition of exit from TRIP mode" (NID_MESSAGE=6) is received | I | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | Whatever is the position of the train regarding D_TAFDISPLAY, no ‘Track Ahead Free Request’ is displayed to the driver. | N/A | N/A | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 3: Radio Message 6 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 6 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Rejection of track ahead Free Request information. The train is under the supervision of an RBC. It receives a message 34 ‘Track Ahead Free Request’ which is rejected due to the current mode. The ‘Track Ahead Free Request’ is not displayed to the driver when the minimum safe front end position is within the interval defined by the [D_TAFDISPLAY, D_TAFDISPLAY + L_TAFDISPLAY]. The driver does not acknowledge and the On-Board equipment does not send any message 149 ‘Track Ahead Free Granted’ to the supervising RBC. |  | 8 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2 : FS, TR L3 : FS, TR |  |  |
| Target of Test | To check that the On-Board equipment does not display any ‘Track Ahead Free Request’ to the Driver when the message 34 is received and after changing to a mode where the TAF would be managed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the On-Board equipment does not display any ‘Track Ahead Free Request’ to the Driver when the message 34 is received. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 7 | FS / TR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | The variables D_TAFDISPLAY and L_TAFDISPLAY included in message 34 are used to define the interval [D_TAFDISPLAY, D_TAFDISPLAY + L_TAFDISPLAY] where ‘Track Ahead Free Request’ should have to be displayed | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | - |
| 2 | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | - |
| 3 | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15, packet 21, packet 27, packet 80 is received | I | RTM | Only in FS mode. This is for changing to either OS or LS mode. | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | - |
| 4 | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=3) is recorded | O | JRU | Only in FS mode. This is for changing to either OS or LS mode. | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | - |
| 5 | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | USE_FT5110200.13 | - | - | Only in TR mode. This is for changing to PT mode. | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | - |
| 6 | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | USE_FT4060300.53 | - | - | Only in UN or SN mode. This is for changing to L2/3 and SR mode. | L0 / LNTC / L2 / L3 | UN / SN / FS / TR | - |
| 7 | L2 / L3 | OS / LS / SR / PT | NOT The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | Whatever is the position of the train regarding in the interval [D_TAFDISPLAY, D_TAFDISPLAY + L_TAFDISPLAY], no ‘Track Ahead Free Request’ is displayed to the driver. | L2 / L3 | OS / LS / SR / PT | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 8 / 12 | OS / SR / PT / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Ahead Free Request |  | 4080442 |
| Test Case of Feature | Rejection of track ahead Free Request information. The train is under the supervision of an RBC. It receives a message 34 ‘Track Ahead Free Request’ which is rejected due to the current level. The ‘Track Ahead Free Request’ is not displayed to the driver when the minimum safe front end position is within the interval defined by the [D_TAFDISPLAY, D_TAFDISPLAY + L_TAFDISPLAY]. The driver does not acknowledge and the On-Board equipment does not send any message 149 ‘Track Ahead Free Granted’ to the supervising RBC. |  | 9 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1 : SB, LS, SR, OS |  |  |
| Target of Test | To check that the On-Board equipment does not display any ‘Track Ahead Free Request’ to the Driver when the message 34 is received |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.5.1 |  |
|  |  | Subset-026-4.7.2 table - Input information Track Ahead Free |  |
|  |  | Subset-026-4.7.2 table - Output information Track Ahead Free request |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Ahead Free Request |  |
|  |  | Subset-026-8.7.14 Message 34: Track Ahead Free Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and DMI that the On-Board equipment does not display any ‘Track Ahead Free Request’ to the Driver when the message 34 is received. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | The variables D_TAFDISPLAY and L_TAFDISPLAY included in message 34 are used to define the interval [D_TAFDISPLAY, D_TAFDISPLAY + L_TAFDISPLAY] where ‘Track Ahead Free Request’ should have to be displayed | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=34) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | Whatever is the position of the train regarding in the interval [D_TAFDISPLAY, D_TAFDISPLAY + L_TAFDISPLAY], no ‘Track Ahead Free Request’ is displayed to the driver. | N/A | N/A | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |