| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080435 (4.8.4: Accepted Information depending on the modes - Revocation of Emergency Stop (Conditional or Unconditional)) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-4080435 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.2 29.11.2013 | ALL | Editorial changes | CEDEX |
| Version 2.9.3 11.12.2013 | TCs 1, 2 and 4 | Editorial changes and general review | CEDEX |
| Version 2.9.4 20.12.2013 | All | Editorial changes | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Revocation of Emergency Stop (Conditional or Unconditional) |  | 4080435 |
| Test Case of Feature | Acceptance of Revocation of Emergency Stop information. The train received the Message 15 ‘Conditional Emergency Stop’ from the supervising RBC. The train responded to the RBC by sending it a message 147 ‘Acknowledgement of Emergency Stop’. The train receives a message 18 ‘Revocation of Emergency Stop’ from the supervising RBC, including a request for acknowledgement. The Train sends a message 146 ‘Acknowledgement’ to the RBC. The current EOA is not modified. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS , LS, OS |  |  |
| Target of Test | To check that the train acknowledges the revocation of a Conditional Emergency Stop message when it is requested. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.10.3.2 |  |
|  |  | Subset-026-4.8.3.1.1 Revocation of Emergency Stop (Conditional or Unconditional) |  |
|  |  | Subset-026-4.8.4.2 Revocation of Emergency Stop (Conditional or Unconditional) |  |
|  |  | Subset-026-8.6.8 Message 147: Acknowledgement of Emergency Stop |  |
|  |  | Subset-026-8.7.8 Message 18: Revocation of Emergency Stop |  |
|  |  | Subset-026-8.6.7 Message 146: Acknowledgement |  |
|  |  | Subset-026-8.7.6 Message 15: Conditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check that the received from and sent to the RBC messages are recorded in the JRU and the Target speed and Target distance are displayed on the DMI properly. To check that the train acknowledges the revocation of a Conditional Emergency Stop message when it is requested. And to check that a subsequent MA is accepted by the onboard when sent by the RBC |
| Constraints | None |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS/OS/LS | USE_FT9990400.17,18,19 | - | - | Use the correct TC ‘Shortening of MA after acceptance of conditional emergency stop‘. The train has received the Message 15 ‘Conditional Emergency Stop’ from the supervising RBC. The train has responded to the RBC by sending it a message 147 ‘Acknowledgement of Emergency Stop’ | L2 / L3 | FS/OS/LS | - |
| 2 | L2 / L3 | FS/OS/LS | SA-DATA.Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18) is received | I | RTM | - | L2 / L3 | FS/OS/LS | - |
| 3 | L2 / L3 | FS/OS/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | FS/OS/LS | - |
| 4 | L2 / L3 | FS/OS/LS | SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) is transmitted | O | RTM | In The message 18 M_ACK variable should be the value ‘Acknowledgement required’ (M_ACK=1) | L2 / L3 | FS/OS/LS | - |
| 5 | L2 / L3 | FS/OS/LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | FS/OS/LS | - |
| 6 | L2 / L3 | FS/OS/LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | The length of the MA will be "L1". | L2 / L3 | FS/OS/LS | - |
| 7 | L2 / L3 | FS/OS/LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=3) is recorded | O | JRU | - | L2 / L3 | FS/OS/LS | - |
| 8 | L2 / L3 | FS/OS/LS | The Mode symbol "Full Supervision" is displayed | O | DMI | Only for OS or LS mode. The onboard changes to FS mode | L2 / L3 | FS | - |
| 9 | L2 / L3 | FS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | Only for OS or LS mode. FS mode is recorded | L2 / L3 | FS | - |
| 10 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | A new TD due to the MA is displayed | L2 / L3 | FS | - |
| 11 | L2 / L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<L1>) is recorded | O | JRU | D_TARGET changes to the new value (L1) given in the last received MA | L2 / L3 | FS | - |

| Step 2: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| Step 4: Radio Message 146 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 146 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| Step 6: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Revocation of Emergency Stop (Conditional or Unconditional) |  | 4080435 |
| Test Case of Feature | Rejection of Revocation of Emergency Stop information The train receives a message 18 ‘Revocation of Emergency Stop’ from the supervising RBC and a level transition to level 2 at a further location, as well as a Conditional Emergency Stop are stored onboard. The message 18 is REJECTED according to the level mode combination. After switching to level 2, a new L2 MA is received but the target speed and distances displayed on the DMI are not updated, since the emergency stop message has not been revoked. |  | 2 |
| Applicable Mode/Level Combinations | LNTC: SN L0: UN L1: FS, LS, OS |  |  |
| Target of Test | To check that: - the on-board has a level transition to level 2 at a further location, as well as an Conditional Emergency Stop stored onboard, and it receives Message 18. - the train does acknowledge the revocation of a Conditional Emergency Stop message before changing to Level 2, although it is rejected but it is a consistent message. - the new L2 MA received after changing to Level 2 is rejected since the emergency message has not been revoked - the train stops before reaching the new stop location given by the conditional emergency stop message. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.10.1.4 |  |
|  |  | Subset-026-3.10.2.4 |  |
|  |  | Subset-026-3.16.3.5.1 |  |
|  |  | Subset-026-4.8.3.1.1 Revocation of Emergency Stop (Conditional or Unconditional) |  |
|  |  | Subset-026-4.8.4.2 Revocation of Emergency Stop (Conditional or Unconditional) |  |
|  |  | Subset-026-8.7.8 Message 18: Revocation of Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check that: On RTM: Message 18 is received before changing to Level 2, with M_ACK = 1. Message 146 is sent before changing to Level 2. Message 147 is sent after changing to Level 2 Message 3 is received after changing to Level 2. On DMI: The new target speed and target distance are displayed after changing to Level 2 When the message 3 is received, the target speed and the distances displayed are not updated On JRU: the message 18 “revocation of emergency stop” is recorded the message 146 “Acknowledgement” is recorded the message 147 “Acknowledgement of Emergency Stop” is recorded the message 3 “Movement Authority” is recorded |
| Constraints | A level 2/3 transition order is stored onboard. A Conditional Emergency Stop (Message 15) is stored on-board. The current EoA is beyond the stop location given by the conditional emergency stop |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 4 / 13 / 12 | FS / OS / UN / SN / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Level Transition order to level 2/3 is stored onboard |
| Conditional Emergency Stop | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only if in OS mode. | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 2 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | SA-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | including a stop location that the train has not yet passed and that it is before the EOA or the LOA included in the Movement Authority for Level 2 stored on-board. | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 3 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 4 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | The speed and distance monitoring information is displayed | O | DMI | No changes are displayed. The message is stored in the Transition Buffer before it is accepted. | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 5 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | SA-DATA.Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18) is received | I | RTM | The message is REJECTED M_ACK = 1 for testing that the message is consistent and it has been rejected due to the level-mode combination | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 6 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 7 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) is transmitted | O | RTM | Indicating that message 18 is consistent | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 8 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 9 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | One Balise Group message (N_TOTAL=1) containing packet 41 (D_LEVELTR=0, M_LEVELTR=3) is received | I | BTM | - | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 10 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / UN / SN / LS | - |
| 11 | L0 / LNTC / L1 | FS / OS / UN / SN / LS | The Level symbol "Level 2" is displayed | O | DMI | - | L2 | FS / OS /LS | - |
| 12 | L2 | FS / OS /LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3) is recorded | O | JRU | - | L2 | FS / OS /LS | - |
| 13 | L2 | FS / OS /LS | The system status message "Emergency stop" is displayed | O | DMI | - | L2 | FS / OS /LS | - |
| 14 | L2 | FS / OS /LS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit17=1>) is recorded | O | JRU | - | L2 | FS / OS /LS | - |
| 15 | L2 | FS / OS /LS | The speed and distance monitoring information is displayed | O | DMI | If in FS or OS mode: The target speed and distance are displayed according to the Stop Location defined in the Message 15 The Level 2 MA previously received and used now must have the EoA or LoA beyond the stop location given in Message 15. If in LS mode: The new EoA after accepting the message 15 will be closed enough in order to make the supervision status change from NoS to OvS, WaS or IntS. | L2 | FS / OS /LS | - |
| 16 | L2 | FS / OS /LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L2 | FS / OS /LS | - |
| 17 | L2 | FS / OS /LS | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted | O | RTM | Indicating that the train takes into account the Conditional Emergency Stop | L2 | FS / OS /LS | - |
| 18 | L2 | FS / OS /LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 | FS / OS /LS | - |
| 19 | L2 | FS / OS /LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) is received | I | RTM | The message is REJECTED | L2 | FS / OS /LS | - |
| 20 | L2 | FS / OS /LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 | FS / OS /LS | - |
| 21 | L2 | FS / OS /LS | The speed and distance monitoring information is displayed | O | DMI | There are no changes on the DMI (so the corresponding message is not recorded on the JRU) | L2 | FS / OS /LS | - |

| Step 2: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 5: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| Step 7: Radio Message 146 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 146 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 9: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 17: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | FINITE VALUE | - |

| Step 19: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Shortened to the Stop Location defined in the message 15 |
| Gradient Profile | STORED | Shortened to the Stop Location defined in the message 15 The information beyond the stop location shall be deleted |
| International SSP | STORED | Shortened to the Stop Location defined in the message 15 The information beyond the stop location shall be deleted |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Revocation of Emergency Stop (Conditional or Unconditional) |  | 4080435 |
| Test Case of Feature | The train receives a message 18 “Revocation of Emergency Stop” from RBC. But two ore more Conditional Emergency Stop messages (15) with different identifiers are stored onboard. |  | 3 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | To check that the train acknowledges the revocation of a Conditional Emergency Stop message when it is requested. After acknowledgement, the Reception of an “Emergency Stop” message is still displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.10.3.2 |  |
|  |  | Subset-026-3.10.3.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Accepted Emergency Stop(s) |  |
|  |  | Subset-026-4.8.3.1.1 Revocation of Emergency Stop (Conditional or Unconditional) |  |
|  |  | Subset-026-4.8.4.2 Revocation of Emergency Stop (Conditional or Unconditional) |  |
|  |  | Subset-026-8.6.8 Message 147: Acknowledgement of Emergency Stop |  |
|  |  | Subset-026-8.7.8 Message 18: Revocation of Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI: The Target speed and Target distance is displayed on the DMI properly. After acknowledgement, the Reception of an “Emergency Stop” message is still displayed Check on JRU: All in and out going messages are recorded. |
| Constraints | None |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Conditional Emergency Stop | STORED | Two ore more Conditional Emergency Stop messages with different identifiers are stored onboard |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Target Distance according to D_EMERGENCYSTOP (2) is DISPLAYED D_EMERGENCYSTOP (1) > D_EMERGENCYSTOP (2) | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18; M_ACK=1) is received | I | RTM | the revocation of an emergency message shall have no effect on the management of other emergency messages possibly received. See req Subset-026-3.10.3.3. With NID_EM = NID_EM (1) The first “conditional emergency stop” message will be revoked. | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | - | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=3) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Not the Mode symbol "Full Supervision" is displayed | O | DMI | Only for OS or LS mode. The onboard does not change to FS mode | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only for FS mode. Target Distance according to D_EMERGENCYSTOP (2) is DISPLAYED Because D_EMERGENCYSTOP (1) > D_EMERGENCYSTOP (2) the D_EMERGENCYSTOP (2) defines the new EOA. The Emergency Stop 2 is not influenced by the revocation of Emergency Stop 1. The EoA/LoA sent in the previous message 3 is beyond D_EMERGENCYSTOP (2), but it is not taken as Target Distance. | N/A | N/A | - |

| Step 2: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| Step 4: Radio Message 146 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 146 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| Step 6: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | stopping location of emergency message is the new EoA |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Conditional Emergency Stop | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Revocation of Emergency Stop (Conditional or Unconditional) |  | 4080435 |
| Test Case of Feature | Acceptance of Revocation of Emergency Stop information. The train receives a message 18 ‘Revocation of Emergency Stop’ from the supervising RBC, including a request for acknowledging. The message is accepted. When receiving a new MA, it is accepted and the new EoA is updated. |  | 4 |
| Applicable Mode/Level Combinations | L2 / L3: PT |  |  |
| Target of Test | To check that the train accepts the revocation of a Conditional Emergency Stop message by terms of a reception of the acceptance of a new MA. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.10.3.2 |  |
|  |  | Subset-026-4.8.3.1.1 Revocation of Emergency Stop (Conditional or Unconditional) |  |
|  |  | Subset-026-4.8.4.2 Revocation of Emergency Stop (Conditional or Unconditional) |  |
|  |  | Subset-026-8.6.7 Message 146: Acknowledgement |  |
|  |  | Subset-026-8.7.8 Message 18: Revocation of Emergency Stop |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check that a subsequent MA is accepted by the onboard when sent by the RBC |
| Constraints | "Recognition of Exit from TR mode" has been previously received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Unconditional Emergency Stop | STORED | One of them: Unconditional or conditional Emergency Stop |
| Conditional Emergency Stop | STORED | One of them: Unconditional or conditional Emergency Stop |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | PT | SA-DATA.Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18; M_ACK=1) is received | I | RTM | - | L2 / L3 | PT | - |
| 2 | L2 / L3 | PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=18) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 3 | L2 / L3 | PT | SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) is transmitted | O | RTM | - | L2 / L3 | PT | - |
| 4 | L2 / L3 | PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=146) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 5 | L2 / L3 | PT | The system status message "Emergency stop" is removed | O | DMI | - | L2 / L3 | PT | - |
| 6 | L2 / L3 | PT | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit17=0>) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 7 | L2 / L3 | PT | The driver selects "Start" | I | DMI | To request RBC for a New MA for L2/L3 | L2 / L3 | PT | - |
| 8 | L2 / L3 | PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 9 | L2 / L3 | PT | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | - | L2 / L3 | PT | - |
| 10 | L2 / L3 | PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 11 | L2 / L3 | PT | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21 (Q_DIR=1), packet 27 (Q_DIR=1) is received | I | RTM | - | L2 / L3 | PT | - |
| 12 | L2 / L3 | PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=3) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 13 | L2 / L3 | PT | The Mode symbol "Full Supervision" is displayed | O | DMI | The onboard changes to FS mode | L2 / L3 | FS | - |
| 14 | L2 / L3 | FS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | FS mode is recorded | L2 / L3 | FS | - |
| 15 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | MA is displayed on DMI | L2 / L3 | FS | - |

| Step 1: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| Step 3: Radio Message 146 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 146 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| Step 9: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |

| Step 11: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |