| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3180300 (3.18.3: Train Data) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-3180300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 16.10.2013 | All | review | DLR |
| Version 2.9.2 04.12.2013 | All | review | DLR |
| Version 2.9.3 19.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.3: Train Data |  | 3180300 |
| Test Case of Feature | The Train Data shall be sent again once the safe radio connection has been re-established. |  | 1 |
| Applicable Mode/Level Combinations | L2: SB, FS, LS, SR, OS, SN L3: SB, FS, LS, SR, OS, SN |  |  |
| Target of Test | 3.18.3.4.2 In case the safe radio connection is lost before the acknowledgement is received, the Train Data shall be sent again once the safe radio connection has been re-established within the on-going communication session. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.3.4.2 |  |
|  |  | Subset-026-3.5.4.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that the radio connection is lost. Check that the radio connection is established again. Check that the radio message "validated Train Data" is sent again. Check that the radio message ""Acknowledgement of Train Data" is received. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver is allowed either to validate or to modify the train data | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver validates the train data | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0101) is recorded | O | JRU | The drivers action is recorded | N/A | N/A | - |
| 4 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | The complete set of Train Data is recorded on the JRU. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | The safe connection is lost. | N/A | N/A | - |
| 8 | N/A | N/A | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | NOT MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 and 4 (M_ERROR= 5) is transmitted | O | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 transmitted | O | RTM | - | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | Reference to received train data message in step 14 | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 5: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 4 / 6 / 12 / 13 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 12: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_PACKET | 8 | 4 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | FINITE VALUE | error type identifier. |

| Step 14: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 4 / 6 / 12 / 13 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 16: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.3: Train Data |  | 3180300 |
| Test Case of Feature | Train length increased |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, OS L2: FS, OS L3: FS, OS |  |  |
| Target of Test | The target of test is to check that: • The Train Data regarding the train length has been increased so the track description, if any stored on-board, shall be considered as unknown in rear of the former min safe rear end of the train. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.3.8 |  |
|  |  | Subset-026-4.4.9.1.4 |  |
|  |  | Subset-026-4.4.12.1.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Entry in FS/OS |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • after the train length increase is effective by Train Data modification an indication of “ENTRY IN FS/OS MODE” is displayed until SSP and gradient are known for the whole length of the train. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Gradient Profile | STORED | - |
| International SSP | STORED | If Packet 27 has been received; Q_FRONT = 0/1 |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver modifies the "Length" and validates the "Flexible Train Data" | I | DMI | The new Train Data set has an increased train length | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONs=21) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | The complete set of Train Data is recorded on the JRU. | N/A | N/A | - |
| 4 | N/A | N/A | The system status message "Entering FS" or "Entering OS" is displayed | O | DMI | Depending of the mode (FS or OS). No change of supervised speed due to the added length and regarding the change of value of the profile is shown on the DMI. | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | "Entering FS" or "Entering OS" System Status Massage is recorded | N/A | N/A | - |
| 6 | N/A | N/A | The system status message "Entering FS" or "Entering OS" is removed | O | DMI | DMI As soon as the train covers the distance equal to the added length | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | The indication removal is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status messages "Entering FS" and "Entering OS" are not displayed | O | DMI | ENTRY OF FS/OS MODE is not shown on the DMI |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.3: Train Data |  | 3180300 |
| Test Case of Feature | At standstill, the driver revalidates the Train Data |  | 3 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SB, SN L1: FS, LS, OS, SR, SB L2: FS, LS, OS, SR, SB L3: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of test is to check that at standstill, it is possible for the driver to revalidate the Train Data that require driver validation according to the specific train implementation |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.3.2.3 |  |
|  |  | Subset-026-3.18.3.3 |  |
|  |  | Subset-026-A.3.11 Train Data: Cant deficiency train category |  |
|  |  | Subset-026-A.3.11 Train Data: Other international train category |  |
|  |  | Subset-026-A.3.11 Train Data: Train length |  |
|  |  | Subset-026-A.3.11 Train Data: Cant deficiency train category |  |
|  |  | Subset-026-A.3.11 Train Data: Brake percentage |  |
|  |  | Subset-026-A.3.11 Train Data: Maximum train speed |  |
|  |  | Subset-026-A.3.11 Train Data: Loading gauge |  |
|  |  | Subset-026-A.3.11 Train Data: Axle load category |  |
|  |  | Subset-026-A.3.11 Train Data: Train fitted with airtight system |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - the driver validates the train data Check on the JRU that: - the driver action on the DMI is recorded - the train data validated by the driver are recorded with their initial value |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN=0 | I | INT | Train is at standstill |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver enters/modifies the "Train Category" and validates the "Flexible Train Data" | I | DMI | The Driver selects among PASS 1-3, TILT 1-7, FP 1-4 and FG 1-4. | N/A | N/A | - |
| 3 | N/A | N/A | The driver modifies the "Length" and validates the "Flexible Train Data" | I | DMI | The Driver tries to set a value out of [0-4095] and can not validate the "Flexible Train Data" | N/A | N/A | - |
| 4 | N/A | N/A | The echo text "++++" in red is displayed | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver modifies the "Length" and validates the "Flexible Train Data" | I | DMI | The Driver set a value in [0-4095] | N/A | N/A | - |
| 6 | N/A | N/A | The echo text "++++" in red is removed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver modifies the "Brake Percentage" and validates the "Flexible Train Data" | I | DMI | The Driver tries to set a value out of [10-250%] and can not validate the "Flexible Train Data" | N/A | N/A | - |
| 8 | N/A | N/A | The echo text "++++" in red is displayed | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver modifies the "Brake Percentage" and validates the "Flexible Train Data" | I | DMI | The Driver set a value in [10-250%] | N/A | N/A | - |
| 10 | N/A | N/A | The echo text "++++" in red is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | The driver modifies the "Maximum Speed" and validates the "Flexible Train Data" | I | DMI | The Driver tries to set a value out of [0-600] and can not validate the "Flexible Train Data" The resolution of the speed is 5km/h. | N/A | N/A | - |
| 12 | N/A | N/A | The echo text "++++" in red is displayed | O | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | The driver modifies the "Maximum Speed" and validates the "Flexible Train Data" | I | DMI | The Driver set a value in [0-600]. The resolution of the speed is 5km/h. | N/A | N/A | - |
| 14 | N/A | N/A | The echo text "++++" in red is removed | O | DMI | - | N/A | N/A | - |
| 15 | N/A | N/A | The driver enters/modifies the "Loading Gauge" and validates the "Flexible Train Data" | I | DMI | The Driver selects among G1, GA, GB GC and Out of GC | N/A | N/A | - |
| 16 | N/A | N/A | The driver enters/modifies the "Axle Load Category" and validates the "Flexible Train Data" | I | DMI | The Driver selects among A, HS17, B1, B2, C2, C3, C4, D2, D3, D4, D4XL, E4, E5 | N/A | N/A | - |
| 17 | N/A | N/A | The driver enters/modifies the "Airtight" and validates the "Flexible Train Data" | I | DMI | The Driver selects among Yes or No | N/A | N/A | - |
| 18 | N/A | N/A | The driver validates the "Flexible Train Data" | I | DMI | It is also allowed that the driver validates the "Fixed Train Data" instead of the "Flexible Train Data" | N/A | N/A | - |
| 19 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONs=21) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | Check that all the train data validated are recorded with their initial value | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.3: Train Data |  | 3180300 |
| Test Case of Feature | The on-board equipment sends the Train Data to the RBC following any entry/modification of Train Data when a communication session is already established |  | 4 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SB, SN L1: FS, LS, OS, SR, SB L2: FS, LS, OS, SR, SB L3: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of test is to check that following any entry/modification of Train Data when a communication session is already established, the ERTMS/ETCS on-board equipment of the leading engine sends the following set of Train Data to the RBC: a) Train category(ies). b) Train length. c) Maximum train speed. d) Loading gauge. e) Axle load category. f) Traction system(s) accepted by the engine. g) Train fitted with airtight system. h) List of National Systems available on-board i) Axle number |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.3.4 |  |
|  |  | Subset-026-3.18.3.4 a) |  |
|  |  | Subset-026-3.18.3.4 b) |  |
|  |  | Subset-026-3.18.3.4 c) |  |
|  |  | Subset-026-3.18.3.4 d) |  |
|  |  | Subset-026-3.18.3.4 e) |  |
|  |  | Subset-026-3.18.3.4 f) |  |
|  |  | Subset-026-3.18.3.4 g) |  |
|  |  | Subset-026-3.18.3.4 h) |  |
|  |  | Subset-026-3.18.3.4 i) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that the train data are sent to the RBC by the onboard equipment Check on the JRU that the train data are sent to the RBC by the onboard equipment |
| Constraints | The train data entry configuration is set to Flexible train data entry. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN=0 | I | INT | Train is at standstill |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver enters/modifies and validates the "Flexible Train Data" | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONs=21) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0, packet 11 is transmitted | O | RTM | Packet 1 must be used if position report is based on 2 BG Packet 11 contains the following data: a) Train category(ies) b) Train length c) Maximum train speed d) Loading gauge e) Axle load category f) Traction system(s) accepted by the engine g) Train fitted with airtight system h) List of National Systems available on-board i) Axle number | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129; NID_PACKET=0; NID_PACKET=11) is recorded | O | JRU | Packet 1 must be used if position report is based on 2 BG Packet 11 contains the following data: a) Train category(ies) b) Train length c) Maximum train speed d) Loading gauge e) Axle load category f) Traction system(s) accepted by the engine g) Train fitted with airtight system h) List of National Systems available on-board i) Axle number | N/A | N/A | - |

| Step 5: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |