| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3090300 (3.9.3: Infill by radio) Total: 18 Test case(s) |  |
| REF: | Subset-076-5-2-3090300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 28.10.2013 | All | review | DLR |
| Version 2.9.2 03.12.2013 | All | review | DLR |
| Version 2.9.3 06.12.2013 | All | review | DLR |
| Version 2.9.4 08.01.2014 | All | review | DLR |
| Version 2.9.5 14.01.2014 | All | review | DLR |
| Version 2.9.6 22.01.2014 | TC2 TC20 TC22 | Initial condition “Radio Infill Area information” added “Preceded by” and “Followed by” changed to “Use_” “Preceded by” changed to “Use_” | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 10.03.2015 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Starting event is received from the balise. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 133 from a balise the on-board unit establish a radio connection: a) The on-board shall request the set-up of a safe connection with the RIU. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the RIU. c) When the on-board receives the system version it shall consider the communication session established and it shall send a session established report, including its telephone numbers, to the RIU. d) The on-board unit shall inform the radio infill unit as soon as the location identified (position report) in the order is passed |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 a) |  |
|  |  | Subset-026-3.5.3.15 |  |
|  |  | Subset-026-3.9.3.4 |  |
|  |  | Subset-026-3.9.3.9 |  |
|  |  | Subset-026-3.9.3.10 |  |
|  |  | Subset-026-3.9.3.10 a) |  |
|  |  | Subset-026-3.9.3.11 |  |
|  |  | Subset-026-3.9.3.11 a) |  |
|  |  | Subset-026-3.9.3.11 b) |  |
|  |  | Subset-026-3.9.3.11 c) |  |
|  |  | Subset-026-3.9.3.11 d) |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.6.11 Message 153: Radio infill request |  |
|  |  | Subset-026-3.17.3.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 155: Initiation of a communication session is RECORDED • Message 32: RBC/RIU System Version is RECORDED • Message 159: Session established is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | Refers only to the communication session to the “accepting” RIU |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Refers only to the link to the “accepting” RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT4080429.3 "Transmission of radio infill area information for establishing a communication session with RIU" | - | - | Connection request to the RTM Mandatory parameter: NID_C, NID_RIU, application type Optional parameter: radio subscriber number (NID_RADIO), mobile network ID, quality of service. Received NID_RADIO is FFFF FFFF FFFF FFFF (stored onboard), so the on-board part of the RTM has to use the short number according to the EIRENE requirements. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Confirmation of the vital connection from RTM | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The trackside ETCS system version is accepted since it is compatible with at least one of the ETCS versions stored on-board. At this point, the on- board equipment considers the communication session with the RBC as established. | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | Communication session established for trackside. Only for information – not relevant for on-board equipment. | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as the location received during preceding TC FT4080429.3 is passed | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |

| Step 4: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 6: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 001 000X OR 010 0000 | Version 1.X or 2.0 (compatible with at least one of the ETCS versions stored onVersion of the ERTMS/ETCS systemboard) |

| Step 8: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 10: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | FINITE VALUE | Start: end of inStart: end of inStart: end of infill |
| NID_PACKET | 8 | 0 | Position report Can also be Packet 1 |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| M_LEVEL | 3 | 2 | L1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | Refers for the communication session to the “accepting” RIU |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Refers for the link to the “accepting” RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Versions are not compatible |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 133 from a balise the on-board unit establish a radio connection: a) The on-board shall request the set-up of a safe connection with the RIU. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the RIU. c) When the on-board receives the system version it shall consider the communication session established and it shall send a message about incompatible versions to trackside. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 a) |  |
|  |  | Subset-026-3.9.3.4 |  |
|  |  | Subset-026-3.9.3.9 |  |
|  |  | Subset-026-8.6.12 Message 154: No compatible version supported |  |
|  |  | Subset-026-8.6.11 Message 153: Radio infill request |  |
|  |  | Subset-026-3.5.3.15 |  |
|  |  | Subset-026-3.17.3.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 155: Initiation of a communication session is RECORDED • Message 32: RBC/RIU System Version is RECORDED • Message 154: No compatible version is RECORDED Termination of the radio link shall follow |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | Refers only to the communication session to the “accepting” RIU NID_RADIO ≠ “FFFF FFFF FFFF FFFF” |
| Radio Infill Area information | STORED | A Packet Number 133 has been received from a balise |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Refers only to the link to the “accepting” RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Connection request to the RTM Mandatory parameter: NID_C, NID_RIU, application type Optional parameter: radio subscriber number (NID_RADIO), mobile network ID, quality of service. Received NID_RADIO is NOT FFFF FFFF FFFF FFFF (use short number stored onboard). | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Confirmation of the vital connection from RTM | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | ETCS system version is not compatible with any of the onboard ETCS versions. Communication session established for on-board | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "No compatible version supported" (NID_MESSAGE=154) is transmitted | O | RTM | To inform the RIU that the received ETCS system version is not compatible with any of the onboard ETCS versions. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The system status message "Trackside not compatible" is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 5: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 000 XXXX or >010 0000 | ETCS system version is not compatible with any of the onboard ETCS versions. |

| Step 7: Radio Message 154 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 154 | No compatible version supported |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| M_ACK | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | Refers for the communication session to the “accepting” RIU |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Refers for the link to the “accepting” RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | On-board unit inform Radio infill unit when detects that main balise group was missed. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment stops to send message 153 to infill although it does not pass over the main balise indicated in the order by the radio infill. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.9.3.10 |  |
|  |  | Subset-026-3.9.3.10 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the radio infill when it detects that the main BG is missed. Check on the JRU that : • The information sent to the radio infill unit is recorded. |
| Constraints | There is some radio infill information stored on-board, There is already a communication session established with the radio infill unit. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | The missing main balise group BGa is included |
| Radio Infill Area information | STORED | Packet 133: Radio infill area information |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT TELEGRAM FROM BALISE (NID_MESSAGE_JRU=12, NID_ERRORBG=<BGa>) is recorded | O | JRU | The train passes over the last possible location of the main balise group with the MinAnt, and it detects nothing. | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT3040400.1,2,3: "Linking reaction" | - | - | Choise of Use_FT depends on linking reaction | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as it detects that the main BG has been missed. Message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | 1 | The train is exiting the radio infill area. |
| NID_PACKET | 8 | 0 | Position report. Can also be Packet 1. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| M_LEVEL | 3 | 2 | L1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Starting event is received from the balise, a communication session with the indicated RIU exists |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: a) After receiving Packet Number 133 from a balise the on-board unit detects the existing communication session to the requested ETCS-ID. b) The on-board does not establish a communication session, because it has already been established with the same RBC/ RIU |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.9.3.5.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 153: Radio infill request is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT4080429.3 "Transmission of radio infill area information for establishing a communication session with RIU" | - | - | Received RIU-ID is the same as the one of the current connection | N/A | N/A | - |
| 2 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | Nothing will happen because, the Communication Session is still established with the current RIU | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted | I | RTM | Nothing will happen because, the Communication Session is still established with the current RIU | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) containing packet X is transmitted | O | RTM | Nothing will happen because, the Communication Session is still established with the current RIU | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Starting event is received from the balise and a the on-board equipment is connected with another radio infill unit |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 133 from a balise the on-board unit establish a radio connection using the given RIU number (phone number or short number given from trackside): a) The on-board shall disconnect the existing connection. b) The on-board shall request the set-up of a safe connection with the RIU. c) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the RIU. d) When the on-board receives the system version it shall consider the communication session established and it shall send a session established report, including its telephone numbers, to the RIU. e) The on-board unit shall inform the radio infill unit as soon as the location identified (position report) in the order is passed |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 a) |  |
|  |  | Subset-026-3.9.3.4 |  |
|  |  | Subset-026-3.9.3.5 |  |
|  |  | Subset-026-3.9.3.5.1 |  |
|  |  | Subset-026-3.9.3.5.1 a) |  |
|  |  | Subset-026-3.9.3.5.1 b) |  |
|  |  | Subset-026-3.9.3.10 |  |
|  |  | Subset-026-3.9.3.10 a) |  |
|  |  | Subset-026-3.9.3.11 |  |
|  |  | Subset-026-3.9.3.11 a) |  |
|  |  | Subset-026-3.9.3.11 b) |  |
|  |  | Subset-026-3.9.3.11 c) |  |
|  |  | Subset-026-3.9.3.11 d) |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.6.11 Message 153: Radio infill request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Packet 133: Radio infill area information is RECORDED • Message 155: Initiation of a communication session is RECORDED • Message 32: RBC/RIU System Version is RECORDED • Message 159: Session established is RECORDED |
| Constraints | Only one session at a time can be handled by OBU. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED ESTABLISHED | Communication session with the “accepting” RIU_2 NOT ESTABLISHED A communication session with a RIU_1 (other than the "acceptiong" RIU_2) is ESTABLISHED. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Refers only to a connection to RIU_1 (another RIU than the “accepting” RIU_2) |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 133 is received | I | BTM | Received RIU-ID is NOT the same as the one of the current connection | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The train tries to release the safe connection | N/A | N/A | - |
| 8 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Connection request to the RTM Mandatory parameter: NID_C, NID_RIU, application type and network address Optional parameter: mobile network ID, quality of service. | N/A | N/A | - |
| 9 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Confirmation of the vital connection from RTM If connection establishment was not successful, the connection request (step 2) shall be repeated until successful. | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32; M_VERSION=010 0000) is received | I | RTM | System versions are compatible Communication session established for on-board | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | - | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 133 | Radio infill area information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | RIU_2 | Other than the RIU currently connected with the onboard |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session. Sent to RIU_1 |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session. Received from RIU_1 |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 10: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session. Sent to RIU_2 |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 12: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | ETCS system version. Version 2.0 compatible with the onboard ETCS version. |

| Step 14: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |

| Step 16: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_BG | 14 | FINITE VALUE | - |
| Q_INFILL | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report. Can also be Packet 1. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| M_LEVEL | 3 | 2 | L1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED ESTABLISHED | Communication session to another destination than the “accepting” RIU is NOT ESTABLISHED Communication session with the “accepting” RIU is ESTABLISHED |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Refers only for the link to the “accepting” RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | The on-board equipment does not include radio. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: After receiving Packet Number 133 from a balise the on-board unit shall ignore this packet (because the on-board is not equipped with radio). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.3 |  |
|  |  | Subset-026-3.9.3.3 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Packet 133: Radio infill area information is RECORDED • No other action related to the missing radio unit is triggered by the on-board. |
| Constraints | There is no radio available on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | The on-board equipment does not include radio. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | The on-board equipment does not include radio. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 133 (Q_RIU=1) and packet 255 is received | I | BTM | There is no radio available on-board | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT Service brake commanded | O | TIU | There is no reaction at all | N/A | N/A | - |
| 4 | N/A | N/A | NOT Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The mode symbol “System Failure” is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The mode symbol “Trip” is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 133 | Radio infill area information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | The on-board equipment does not include radio. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | There is no radio available on-board |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | The train passes the main signal balise group which contains an order to terminate the communication session with the RIU (packet 133). The on-board equipment sends to the RIU the radio message 153 (Radio infill request) indicating that it exits the radio infill area. At the required location, the on-board equipment sends a radio message 156 “Termination of a communication session” to the RIU. On reception of radio message 39 “Acknowledgement of termination of a communication session” from the RIU, it considers the communication session terminated and releases the safe connection. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that : • The on-board sends a radio message 153 (Radio infill request) indicating that it exits the radio infill area when the main signal balise group is passed. • At the required location, the on-board equipment sends a radio message 156 “Termination of a communication session” to the RIU according to the packet 133 (Radio infill area information) requiring the termination of the communication session with the RIU received from a balise group. • No further message is sent by the on-board once the radio message 156 has been sent. • On reception of radio message 39 “Acknowledgement of termination of a communication session” from the RIU, the on-board considers the communication session terminated and releases the safe connection. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.10 |  |
|  |  | Subset-026-3.9.3.10 b) |  |
|  |  | Subset-026-3.9.3.11 |  |
|  |  | Subset-026-3.9.3.11 a) |  |
|  |  | Subset-026-3.9.3.11 b) |  |
|  |  | Subset-026-3.9.3.11 c) |  |
|  |  | Subset-026-3.9.3.11 d) |  |
|  |  | Subset-026-3.9.3.16 |  |
|  |  | Subset-026-8.6.11 Message 153: Radio infill request |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM that: • The radio message 153 (Radio infill request) including a position report packet is sent to the RIU • The radio message 156 (Termination of a communication session) is sent to the RIU. • A radio message 39 (Acknowledgement of termination of a communication session) is received from the RIU. Check on the JRU that: • The main signal balise group message with packet 133 is recorded • The radio message 153 (Radio infill request) including a position report packet sent by the on-board is recorded. • The radio message 156 (Termination of a communication session) sent by the on-board is recorded. • The radio message 39 (Acknowledgement of termination of a communication session) received by the on-board is recorded. |
| Constraints | There is some radio infill information stored on-board, There is already a communication session established with the radio infill unit. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BG1> ) containing packet 133 (Q_RIU=0) and packet 255 is received | I | BTM | The train passes over a main signal balise group BG1 containing an order to terminate the communication session with the RIU (packet 133): Q_RIU = 0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | The on-board equipment sends a radio message 153 (Radio infill request) including a position report packet to the RIU | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | D_LRBG = D_INFILL The location where to disconnect to RIU is reached | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The train tries to release the safe connection. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG1 | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 133 | Radio infill area information |
| Q_DIR | 2 | 1 | nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | Current RIU id |
| NID_RADIO | 64 | FINITE VALUE | Current RIU phone number |
| D_INFILL | 15 | FINITE VALUE | Location where to disconnect the RIU |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | BG1 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. Second balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG1 | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_BG | 14 | BG1 | Id of the main signal balise group BG 1 |
| Q_INFILL | 1 | 1 | Train exits the radio infill area |
| NID_PACKET | 8 | 0 | Position report. Can also be Packet 1. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| M_LEVEL | 3 | 2 | L1 |

| Step 5: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | TERMINATED |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| DENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Check for plausibility and correctness of variables according to the variable definitions. A radio infill message is accepted on-board when the message is correct and complete. |  | 8 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that a radio infill message is accepted when: • The message is complete and correct according to variables values. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.14 |  |
|  |  | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-8.4.1.3.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • The radio infill message is RECORDED and accepted.The telegrams with the variables are fully complete and plausible. |
| Constraints | The announcement of radio infill area information (Packet 133: Radio infill area information) has already been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | The main balise group BGb to which the radio infill refers is included. |
| Movement Authority | STORED | L1 MA |
| Radio Infill Area information | STORED | Packet 133: Radio infill area information |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 is received | I | RTM | The train receives a radio infill message. Message and variables values are accepted. i.e. checking of plausibility and correctness. If M_ACK = 1 (acknowledgement required), Use_FT3160300.9 (Radio message acknowledgement for link to Radio infill unit). | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | Complete message and variables are RECORDED. | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | speed and distance according to the new MA is displayed. This proofs acceptance of infill MA. Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill location reference |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | The main balise group BGb to which the radio infill refers is included. |
| Movement Authority | STORED | Received L1 MA |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | - | DMI | According to received L1 MA Restricted in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Check for plausibility of a radio infill message according to variables definitions. One variable of the message has an invalid value. |  | 9 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that when a variable of the message has an invalid value: • Message is rejected. • Radio message consistency error is reported to the track |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.14 |  |
|  |  | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-3.16.1.1 a) |  |
|  |  | Subset-026-3.16.3.1.1.1 |  |
|  |  | Subset-026-3.16.3.1.1.2 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • Error message is DISPLAYED. Check on the JRU that : • Radio infill message is RECORDED. • Error message is RECORDED. It has been considered that the variable with invalid value is Q_DIR (included in any Packet which contains this variable). Any other variable of the message which can contain invalid values could be chosen. |
| Constraints | The announcement of radio infill area information (Packet 133: Radio infill area information) has already been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | The main balise group BGb to which the radio infill refers is included. |
| Movement Authority | STORED | L1 MA |
| Radio Infill Area information | STORED | Packet 133: Radio infill area information |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 is received | I | RTM | The train receives a radio infill message. One of the variables in the radio infill message has an invalid value. Q_DIR=3 (Spare), for example. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | Radio infill message is RECORDED. | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Speed and distance are NOT updated according to the new MA. This proofs rejection of infill MA. Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 4 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Since speed and distance information has not changed, no message 20 is recorded in the JRU | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 4 is transmitted | O | RTM | Radio message consistency error is reported to the track | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | Value different from the actual message length. |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill location reference. |
| Q_DIR | 2 | 3 | Invalid value |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 4 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 3 | Radio message consistency error |
| NID_PACKET | 8 | 0 | Position report. Can also be Packet 1. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 12 | FS / LS |
| M_LEVEL | 3 | 2 | L1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | The main balise group BGb to which the radio infill refers is included. |
| Movement Authority | STORED | Same as initial L1 MA |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | - | DMI | NOT According to received L1 MA, but according to initial MA Restricted in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Check that a new Radio infill information is ignored, when the referred main signal balise group was already passed. Linking information is stored on-board. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The train passes over a balise group, after that: - a radio infill message is received with reference to balise group received before - the radio infill information is ignored. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.15 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • current mode is displayed Check on the JRU that : • The balise group message is accepted and recorded • The radio infill information is rejected and recorded |
| Constraints | Radio infill equipment is present on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | Linking information |
| Radio Infill Area information | STORED | The main signal BG is BGa |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 12 is received | I | BTM | The main signal balise group is passed at the announced location. The MA enables the train to run beyond the end of the TC. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 (NID_BG=<BGa>) and packet 12 (N_ITER=0; L_ENDSECTION=0) is received | I | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Emergency brake commanded | O | TIU | The infill information is ignored. | N/A | N/A | - |
| 6 | N/A | N/A | NOT The mode symbol “Trip” is displayed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group 1 ≤ N ≤ 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. Second balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group 1 ≤ N ≤ 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill location reference |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | "BGa" | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | 0 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Check that a new Radio infill information is ignored, when the referred main signal balise group is missed. Linking information is stored on-board. |  | 11 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The train detects that a main balise group is missed, after that: - a radio infill message is received with reference to that missed balise group - the radio infill information is ignored. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.10 b) |  |
|  |  | Subset-026-3.9.3.15 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • current mode is displayed Check on the JRU that : • The radio infill message is recorded. • Check that packet 136 is recorded and ignored by on-board. |
| Constraints | The linking information of missed balise group was set to “no reaction” |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | Linking information includes the main signal balise group BGa with linking reaction set to no reaction |
| Radio Infill Area information | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Radio infill request" (NID_MESSAGE=153; Q_INFILL=1; NID_BG=<BGa>) containing packet 0 is transmitted @ MinAnt | O | RTM | The train has reached the end of the expectation window of the main signal balise group BGa without finding it. Linking reaction is “no reaction”. Packet 1 instead of 0 must be used if the position report is based on two BG. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded @ MinAnt | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37; NID_BG=<BGa>) containing packet 136 and packet 12 (N_ITER=0; L_ENDSECTION=0) is received | I | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Emergency brake commanded | O | TIU | The infill information is ignored. | N/A | N/A | - |
| 6 | N/A | N/A | NOT The mode symbol “Trip” is displayed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_INFILL | 1 | 1 | End of Infill Area |
| NID_PACKET | 8 | 0 | Position report. Can also be Packet 1. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | FS / LS |
| M_LEVEL | 3 | 2 | L1 |

| Step 3: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | 0 | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill location reference |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | "BGa" | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | 0 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Monitoring of radio message from RIU to on-board equipment when the messages arrives according to time stamp rules. |  | 12 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • Any Message (according to SRS 8.4.4) is transmitted from RIU. • The message is respecting time stamps according to previous sending time and train time (NOT in advance to real train time ) • The message will be accepted by the on-board (T_TRAIN time and message time may have wrap around or initial set to unknown without affecting the system) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.4 |  |
|  |  | Subset-026-3.9.3.11 |  |
|  |  | Subset-026-3.9.3.11 d) |  |
|  |  | Subset-026-3.9.3.14 |  |
|  |  | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-3.16.1.1 b) |  |
|  |  | Subset-026-3.16.3.3.2 |  |
|  |  | Subset-026-3.16.3.3.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • The new message is recorded. • Sending time from RBC is according to Train time (in advance to train time or initial set to unknown ) • Message time and T_time shall be accepted • The Message is accepted. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 is received | I | RTM | Another (or more) message is received with a new packet. • Received T_TRAIN is greater than T_TRAIN on last message sent to RIU (at last step of FT3090300.4) • Received T_TRAIN is not in advance of the real on-board time (i.e. T_TRAIN is not greater than on-board time) . Check that: • Message is accepted and stored by the on-board. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | Message is recorded. | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | speed and distance according to the new MA is displayed. This proofs acceptance of infill MA. Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 136 | Infill location reference |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | - | DMI | According to received L1 MA Restricted in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | Monitoring of radio message from RIU to the on-board equipment when the 2nd message is received with older time stamp in reference with the on-board and previous sent message time stamp. |  | 13 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • 1st Message is received with correct time. • 2nd Message is older (in time) than the previous received. • The message is NOT accepted by on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.4 |  |
|  |  | Subset-026-3.9.3.11 |  |
|  |  | Subset-026-3.9.3.11 d) |  |
|  |  | Subset-026-3.9.3.14 |  |
|  |  | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-3.16.1.1 b) |  |
|  |  | Subset-026-3.16.3.3.2 |  |
|  |  | Subset-026-3.16.3.3.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Errors are displayed Check on the JRU that: • The received messages are recorded. • Older messages (in sequence / time ) are rejected by on-board |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 is received | I | RTM | One (or more) message is received : • First message is correct according to on-board time Message is complete. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | speed and distance according to the new MA is displayed. This proofs acceptance of infill MA. Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 is received | I | RTM | Immediately after the first message a second message is received with older time stamp. The message is rejected by the on-board. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Speed and distance are NOT updated according to the new MA. This proofs rejection of infill MA. Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 8 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Since speed and distance information has not changed, no message 20 is recorded in the JRU | N/A | N/A | - |

| Step 1: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 136 | Infill location reference |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 5: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 136 | Infill location reference |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | On-board unit inform Radio infill unit when detects and reads main balise group. |  | 14 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment stops send message 153 to infill information when passes over the main balise indicated in the order by the radio infill. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.10 |  |
|  |  | Subset-026-3.9.3.10 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the radio infill when it reads the main BG. Check on the JRU that : • The information sent to the radio infill unit is recorded. |
| Constraints | There is some radio infill information stored on-board, There is already a communication session established with the radio infill unit. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Radio Infill Area information | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message is received | I | BTM | The train passes over the main balise group. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The Balise group is recorded | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as it reads the main BG. Message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio infill reques - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | FINITE VALUE | Start: end of inStart: end of inStart: end of infill |
| NID_PACKET | 8 | 0 | Position report. Can also be Packet 1. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| M_LEVEL | 3 | 2 | L1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RADIO COMMUNICATION for infill ESTABLISHED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | The on-board receives an order to establish a communication session for infill via balise and is able to handle two communication session at a time. |  | 15 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The on-board has a connection to an RIU (RIU 1) for infill • The on-board receives an order to establish a communication session to another RIU (RIU 2) • The on-board terminates the communication session to RIU1 and as soon as a new communication session can be handled the on-board establishes a new communication session to RIU2. • Since OBU is able to handle two sessions at a time, the communication with RIU2 is initiated immediately after sending message 156 to RIU1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.5 |  |
|  |  | Subset-026-3.9.3.5.2 |  |
|  |  | Subset-026-3.9.3.5.2 a) |  |
|  |  | Subset-026-3.9.3.5.2 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | • The on-board terminates the communication until it receives a communication session order by trackside • The on-board establishes a communication session |
| Constraints | • A communication session is already established |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With RIU1 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With RIU1 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT4080429.3 "Transmission of radio infill area information for establishing a communication session with RIU" | - | - | NID_RIU is different with the NID_RIU of the already established Radio infill connection | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) containing is transmitted | O | RTM | The location where to disconnect to RIU is reached | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Before termination of communication with RIU1 is completed, a connection request to RIU2 is sent | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The train tries to release the safe connection. | N/A | N/A | - |
| 8 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Confirmation of the vital connection from RTM If connection establishment was not successful, the connection request (step 2) shall be repeated until successful. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32; M_VERSION=010 0000) is received | I | RTM | System versions are compatible Communication session established for on-board | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 14 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 16 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |

| Step 9: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session. Sent to RIU_2 |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 11: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | ETCS system version. Version 2.0 compatible with the onboard ETCS version. |

| Step 13: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 15: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | FINITE VALUE | Start: end of infill |
| NID_PACKET | 8 | 0 | Position Report Can also be Packet 1 |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 12 | FS / LS |
| M_LEVEL | 3 | 2 | L1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With RIU2 |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With RIU2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | The on-board receives an order to establish a communication session for infill via radio and the OBU is able to handle only one communication session at a time. |  | 16 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The on-board has a connection to an RIU (RIU 1) for infill • The on-board receives an order to establish a communication session to another RIU (RIU 2) via radio • The order is ignored |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.6 |  |
|  |  | Subset-026-3.9.3.6 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | • The on-board receives an order to establish a communication session to another RIU (RIU 2) via radio • The order is ignored |
| Constraints | • A communication session is already established • Only one session at a time can be handled by OBU. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With RIU1 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Connection to RIU1 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 143 is received | I | RTM | The OBU RECEIVES a Message 24. Within the Message is a Packet Number 143: Session Management with neighbouring Radio Infill Unit. Within the packet is an order to connect to RIU 2 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The connection order is ignored, because the OBU can handle one connection at a time only. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 143 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RIU | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | “RIU2“ | - |
| NID_RADIO | 64 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With RIU1 |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Connection to RIU1 is still established |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | The on-board receives an order to establish a communication session for infill via radio and the OBU is able to handle two communication sessions at a time. |  | 17 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The on-board has a connection to an RIU (RIU 1) for infill • The on-board receives an order to establish a communication session to another RIU (RIU 2) via radio • The existing communication session shall be maintained and the new one shall be established. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.6 |  |
|  |  | Subset-026-3.9.3.6 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | • The on-board receives an order to establish a communication session to another RIU (RIU 2) via radio • The existing communication session shall be maintained and the new one shall be established. |
| Constraints | • The OBU is able to handle two communication sessions at a time. • One communication session is already established |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With RIU1 |
| ERTMS/ETCS level | 0 / 1 / 2 / 12 | FS / OS / SR / LS |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With RIU1 |
| Service brake NOT commanded and Emergency brake NOT commanded | O | TIU | SB and EB are RELEASED |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT3050300.14 "The Session management with neighbouring RIU shall be accepted by on-board equipment" | - | - | The OBU RECEIVES a Message 24: within the Message is a Packet Number 143: Radio infill area information with NID_RIU = "RIU 2". | N/A | N/A | - |
| 2 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The connection with RIU 1 is maintained. | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DISCONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With both RIU1 and RIU2 |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With both RIU1 and RIU2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.9.3: Infill by radio |  | 3090300 |
| Test Case of Feature | The on-board receives an order to establish a communication session for infill via radio and the OBU is able to handle two communication sessions at a time. The OBU has already two open communication sessions at a time and the OBU should maintain the connection to the actual infill area and terminate that one for the left infill area. |  | 18 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The on-board has two connections to a RIU (RIU1 and RIU2) for infill • The on-board receives an order to establish a communication session to another RIU (RIU3) via radio infill • The existing communication session to RIU2 (RIU2 is not the RIU for the current infill area) shall be terminated • The connection to RIU3 shall be initialized when the connection to RIU2 is terminated |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.9.3.6 |  |
|  |  | Subset-026-3.9.3.6 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | • The on-board receives an order to establish a communication session to another RIU (RIU3) via radio infill • The existing communication session to RIU2 RIUis terminated • The connection to RIU3 is initialized after the connection to RIU2 has been terminated |
| Constraints | • The OBU is able to handle two communication sessions at a time. • Two communication sessions are already established (with RIU1 and RIU2) • RIU1 is the RIU for the current infill area and the position of the main Balise group of RIU1 has not been passed |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With both RIU1 and RIU2 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With both RIU1 and RIU2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 143 is received | I | RTM | Within the packet is an order to connect to RIU3 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The on-board terminates the connection to RIU2 and maintains the connection to RIU1 | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The train tries to release the safe connection. | N/A | N/A | - |
| 8 | N/A | N/A | Use_FT3050300.28 "A communication session initiated by the on-board unit is established successfully" | - | - | The train opens a new communication session to RIU3 | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 143 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RIU | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | “RIU3“ | - |
| NID_RADIO | 64 | FINITE VALUE | - |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With both RIU1 and RIU3. Session with RIU2 is TERMINATED |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With both RIU2 and RIU3. Connection with RIU1 is not set-up. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |