| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5050400 (5.5.4: Degraded Situation) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5050400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation of the FT | DLR |
| Version 2.9.0 31.03.2013 | All | First version/Review for Baseline 3 | INECO |
| Version 2.9.1 01.08.2013 | All | Second version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.2 01.10.2013 | All | Third version/Review UNISIG from 21.08.2013 included; FT renumbered | INECO |
| Version 2.9.3 03.01.2014 | All | Fourth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.5.4: Degraded Situation |  | 5050400 |
| Test Case of Feature | The train is in level 2 or 3 and no communication session is open. End of mission procedure is triggered. No communication session needs to be established to report end of mission. |  | 1 |
| Applicable Mode/Level Combinations | L2: SH / SL / SB L3: SH / SL / SB |  |  |
| Target of Test | The target of this test case is to check that : The actual level is 2 or 3, no communication session is open EoM procedure is triggered The on-board equipment does not initiate a communication session to report to RBC the end of mission. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.5.4.1.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Transmissions of message 155 and 150 are NOT recorded. Check on the RTM that : No communication session is established after mode change The trackside is not informed about the End of Mission. |
| Constraints | No communication session exists. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 6 | SH / SL / SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) containing packet 0/1 is transmitted. | O | RTM | The on-board equipment does not establish a radio communication session. | N/A | N/A | - |
| 2 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0/1 is transmitted. | O | RTM | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 6 | SH / SL / SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.5.4: Degraded Situation |  | 5050400 |
| Test Case of Feature | The on-board equipment has entered in a mode considered as an End of Mission. It sends to the RBC a radio message 150 “End of Mission”. As it does not receive an order to terminate the communication session, it repeats the end of mission message a defined number of times. After unanswered end of mission report, the on-board equipment sends a radio message 156 “Termination of a communication session” to the RBC. It receives Message 39 (Acknowledgement of termination of a communication session), and it considers the communication session terminated and releases the safe connection. |  | 2 |
| Applicable Mode/Level Combinations | L2: SH / SL / SB L3: SH / SL / SB |  |  |
| Target of Test | The target of this test is to check that : The on-board sends radio message 150 “End of Mission” to RBC (first request) No answer is received from RBC The on-board repeats 3 times each 15s the sending of the radio message 150 “End of Mission” when no answer is received from the RBC. The termination of the communication session is initiated by the on-board equipment when no answer to the End of mission message is received. After waiting within the fixed waiting time (60s), the on-board sends a radio message 156 “Termination of a communication session” to the RBC. No further message is sent by the on-board once the radio message 156 has been transmitted. Message 39 is received, and the on-board equipment considers the communication session terminated and releases the safe connection. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.5.2 |  |
|  |  | Subset-026-3.5.5.2 a) |  |
|  |  | Subset-026-3.5.5.2 c) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-5.5.3.1.3 |  |
|  |  | Subset-026-5.5.3.1.3 a) |  |
|  |  | Subset-026-5.6.2.2 A100 |  |
|  |  | Subset-026-A.3.1 - Repetition of radio messages (i.e. excluding the first sending) |  |
|  |  | Subset-026-A.3.1 - Waiting time before radio message repetition |  |
|  |  | Subset-026-A.3.1 - Additional delay time to disconnection on supervision of safe radio connection |  |
|  |  | Subset-026-5.5.4.1.1 |  |
|  |  | Subset-026-5.5.4.1.1.1 |  |
|  |  | Subset-026-8.6.10 Message 150: End of Mission |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Four radio message 150 (End of Mission) are transmitted to the RBC. A radio message 156 (Termination of a communication session) is transmitted to the RBC. A radio message 39 (Acknowledgement of termination of a communication session) is received from the RBC Check on the JRU that: The four radio messages 150 (End of Mission) sent by the on-board are recorded. The radio message 156 (Termination of a communication session) sent by the on-board is recorded. The radio message 39 (Acknowledgement of termination of a communication session) is recorded. |
| Constraints | Communication session between on-board and trackside is ESTABLISHED. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 6 | SH / SL / SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 is transmitted | O | RTM | No answer is received from the RBC and a time out occurs (15 s): The first repetition of end of mission message is transmitted. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 is transmitted | O | RTM | No answer is received from the RBC and a time out occurs (15s): The second repetition of end of mission message is transmitted | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 is transmitted | O | RTM | No answer is received from the RBC and a time out occurs (15s): The third and last repetition of end of mission message is transmitted | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | No answer is received from the RBC and a time out occurs (60s) | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | The on-board equipment considers the communication session terminated. | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-DISCONNECT. Request is transmitted | O | RTM | The on-board release the safe connection | N/A | N/A | - |

| Step 1: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | End of Mission |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | 0 | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 3: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | End of Mission |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | 0 | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 5: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | End of Mission |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | 0 | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 7: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | End of Mission |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | 0 | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 9: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session (with Accepting RBC1) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 11: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 + t’; t’ > 0 |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 6 | SH / SL / SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |