| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5100300 (5.10.3: Specific Additional Requirements) Total: 16 Test case(s) |  |
| REF: | Subset-076-5-2-5100300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 01.08.2013 | All | Second version/Internal modification; imported to TC_Editorv27b; Correction of Requirements blank space | INECO |
| Version 2.9.2 03.10.2013 | All | Third version/UNISIG remarks to v 2.9.0 from 06/09 included. | INECO |
| Version 2.9.3 15.11.2013 | All | Fourth version/ Internal modification; Added new requirements | INECO |
| Version 2.9.4 02.12.2013 | All | Fifth version/ Updated after bilateral meeting with UNISIG on 29.11.2013; Changes to TC1, TC that applies to” Based on requirements field” after the study Requirements coverage_v12 was done. | INECO |
| Version 2.9.5 13.12.2013 | All | Sixth version/ UNISIG remarks from 27/11/2013included | INECO |
| Version 2.9.6 03.01.2014 | All | Eighth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | A position report is sent to RBC after a level transition from L1 to L2/L3. Radio connection established. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / TR / NL |  |  |
| Target of Test | The target of test is to check that: When the ERTMS/ETCS on-board equipment has switched to the new level, it shall report the new on-board level, including a position report The function "Determine ERTMS/ETCS level" is active. The function "Report train position when change of level due to trackside order" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 g) |  |
|  |  | Subset-026-5.10.3.1.5 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS level |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when change of level due to trackside order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Packet 0 or 1: Position Report is RECORDED including the current level (M_LEVEL). |
| Constraints | The transition to the current level is fulfilled. The train reports its position by means of Packet 0 or Packet 1.Radio connection established in L1 area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 11 / 12 | FS / OS / SR / TR / NL / LS |
| Radio communication session | ESTABLISHED | with relevant RBC before the level transition |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is DISPLAYED. | O | DMI | DMI Object: LE03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS / OS / SR / TR / NL | The on-board equipment performs a level transition from L1 to L2/L3 | - | - | USE_FT5100100.37 : Order to switch immediately and execution of level transition L1 / L2 Or USE_FT5100100.36 :Order to switch immediately and execution of level transition L1 / L3 | L2 / L3 | FS / LS / OS / SR / TR / NL | - |
| 2 | L2 / L3 | FS / LS / OS / SR / TR / NL | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3/4) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR / TR / NL | - |
| 3 | L2 / L3 | FS / LS / OS / SR / TR / NL | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | L2 / L3 | FS / LS / OS / SR / TR / NL | - |
| 4 | L2 / L3 | FS / LS / OS / SR / TR / NL | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR / TR / NL | - |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 11 / 12 | FS / OS / SR / TR / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/Level 3" is displayed. | O | DMI | DMI object: LE04 / LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | A position report is sent to RBC after a level transition from L0 to L2/L3. Radio connection established. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN / NL |  |  |
| Target of Test | The target of test is to check that: When the ERTMS/ETCS on-board equipment has switched to the new level, it shall report the new on-board level, including a position report The function "Determine ERTMS/ETCS level" is active. The function "Report train position when change of level due to trackside order" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 g) |  |
|  |  | Subset-026-5.10.3.2.5 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-4.7.2 table - Output information Current ERTMS/ETCS Level |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS level |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when change of level due to trackside order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Indication of the new level (Symbol LE04 or LE05) is DISPLAYED. Check on the JRU that: Packet 0 is RECORDED including the current level (M_LEVEL). |
| Constraints | The transition to the current level is fulfilled. The train reports its position by means of Packet 0. Radio connection established in L0 area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 11 | UN / NL |
| Radio communication session | ESTABLISHED | with relevant RBC before the level transition |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is DISPLAYED. | O | DMI | DMI Object: LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | N/A | The on-board equipment performs a level transition from L0 to L2/L3 | - | - | USE_FT5100100.13 (for transitions L0/L2): Order to switch immediately and execution of level transition L0 / L2 OR USE_FT5100100.26 (for transitions L0/L3): Order to switch immediately and execution of level transition L0 / L3 | L2 / L3 | N/A | - |
| 2 | L2 / L3 | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3/4) is recorded | O | JRU | - | L2 / L3 | N/A | - |
| 3 | L2 / L3 | N/A | The Level symbol "Level 2/Level 3" is displayed. | O | DMI | DMI object: LE04 / LE05 | L2 / L3 | N/A | - |
| 4 | L2 / L3 | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04/05=1>) is recorded | O | JRU | - | L2 / L3 | N/A | - |
| 5 | L2 / L3 | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | L2 / L3 | N/A | - |
| 6 | L2 / L3 | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | N/A | - |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 11 / 12 | FS / OS / SR / TR / NL / LS |
| Radio communication session | ESTABLISHED | - |
| Position Report parameters | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/Level 3" is displayed. | O | DMI | DMI object: LE04 / LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | The train receives a conditional level transition and the current level is contained. If the level is contained in the priority list of the conditional level transition order, the on-board shall not change the level. |  | 3 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / LNTC: FS / LS / OS / SR / UN / SL / SB / TR / NL / SN / RV |  |  |
| Target of Test | The target of this test is to check that: The conditional level transition containing a priority list is received. The on-board checks if the current level is contained in the priority list. The on-board equipment does not change the level, because the current level is contained in list. The on-board stores the table of ERTMS/ETCS levels supported by trackside. The function "Determine ERTMS/ETCS level" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.14.1 |  |
|  |  | Subset-026-5.10.3.14.2 |  |
|  |  | Subset-026-5.10.3.14.4 |  |
|  |  | Subset-026-7.4.2.11.2 Packet Number 46: Conditional Level Transition Order |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS level |  |
|  |  | Subset-026-4.8.3.1.1 Conditional Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The level indication does not change. Check on the JRU that : The message is recorded. |
| Constraints | A list of ETCS levels is available on-board. On-board is fitted for all levels. Packet 46 rejected if a level transition order is received in the same message, or if a previous level transition order has announced a level transition still to be executed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | Packet 41 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level X is DISPLAYED. | O | DMI | This step not apply in SL mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | One Balise Group message (N_TOTAL=1) containing packet 46 is received | I | BTM | - | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 2 | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 3 | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=X) is recorded | O | JRU | Triggering event: Every 5 seconds The current level is contained in the priority list of the Conditional level transition order so no level transition is performed Check that M_LEVEL from JRU record header is the same | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 4 | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | The Level symbol "Level X" is displayed. | O | DMI | The level indication does NOT change | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 5 | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 46 | Conditional level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | Same value as the current level and included in the priority list |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(1) | 3 | 4 | - |
| NID_NTC(1) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| N_ITER(1) | 5 | FINITE VALUE | - |
| M_LEVELTR(2) | 3 | 3 | - |
| NID_NTC(2) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| N_ITER(2) | 5 | FINITE VALUE | - |
| M_LEVELTR(3) | 3 | 2 | - |
| NID_NTC(3) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| N_ITER(3) | 5 | FINITE VALUE | - |
| M_LEVELTR(4) | 3 | 1 | - |
| NID_NTC(4) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| N_ITER(4) | 5 | FINITE VALUE | - |
| M_LEVELTR(5) | 3 | 0 | - |
| NID_NTC(5) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level X is DISPLAYED. | O | DMI | Same level This step not apply in SL mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | The train receives a conditional level transition and the current level is not contained in the list. If the current level is not contained in the priority list of the conditional level transition order, the on-board shall evaluate the conditional level transition order in the same way as an immediate level transition order. |  | 4 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / LNTC: FS / LS / OS / SR / UN / SL / SB / TR / NL / RV |  |  |
| Target of Test | The target of this test is to check that: The conditional level transition containing a priority list is received. The on-board equipment evaluates the conditional level transition in the same way as an immediate normal level transition order. The on-board stores the table of ERTMS/ETCS levels supported by trackside. The function "Determine ERTMS/ETCS level" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.14.1 |  |
|  |  | Subset-026-5.10.3.14.3 |  |
|  |  | Subset-026-5.10.3.14.4 |  |
|  |  | Subset-026-7.4.2.11.2 Packet Number 46: Conditional Level Transition Order |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS level |  |
|  |  | Subset-026-4.8.3.1.1 Conditional Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The message 46 is recorded |
| Constraints | A list of ETCS levels is available on-board. On-board is fitted for all levels. Packet 46 rejected if a level transition order is received in the same message, or if a previous level transition order has announced a level transition still to be executed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | Packet 41 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level X is DISPLAYED. | O | DMI | This step not apply in SL mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | One Balise Group message (N_TOTAL=1) containing packet 46 is received | I | BTM | The current level X not contained in the priority list of the conditional level transition order | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 2 | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 3 | LX | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=Y) is recorded | O | JRU | Triggering event: When the level changes. Immediate level transition is performed The on-board equipment selects the level with the highest priority that it allows to use : LY | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 4 | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | The Level symbol "Acknowledge Level Y announcement" is displayed. | O | DMI | Optional step. If the selected level needs the driver ack. | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 5 | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | The driver confirms "Level". | I | DMI | Optional step. If the selected level needs the driver ack. | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 6 | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. If the selected level needs the driver ack. | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 7 | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | The Level symbol "Level Y" is displayed. | O | DMI | The level indication changes | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |
| 8 | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<BitXX=1>) is recorded | O | JRU | - | LY | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 46 | Conditional level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | = LX |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | (k) ≠ LX (Not contained in the list) |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | LY |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level Y is DISPLAYED. | O | DMI | Different level This step does not apply in SL mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | Transition from Level 1 to level 2/3 area .When the ERTMS/ETCS communication session is open in rear of the transition border, Train Data shall be sent to the RBC (which acknowledges the data). |  | 5 |
| Applicable Mode/Level Combinations | L1: FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that: A level transition from Level 1 to Level 2/3 area at a further transition is stored The on-board equipment establishes a communication session with the responsible RBC in rear of the border location. Train Data sent to RBC (which acknowledges the data) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-5.10.3.1.3 |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The balise group message including packet 42 (Session Management) is STORED The establishment of the communication session is STORED The message 129 (Validated Train Data) is STORED (not in SL/NL) The message 8 (Acknowledgement of Train Data) is STORED (not in SL/NL) |
| Constraints | In SL or NL modes no Train Data shall be sent. The Radio Session shall be established in SL mode.(Q_SLEEPSESION=1) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| Level Transition Order | STORED | From L1 to L2/L3 at a further location |
| Train Data | VALID | Validated train data |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | Train location: in rear of the border of level transition location | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 42 is received . | I | BTM | Packet 42: Q_RBC = 1 Q_SLEEPSESSION=1 The packet 42 is received on board via balise group in rear of the border location | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | A Radio Communication Session ordered by the trackside is established | - | - | USE_ FT3050300.21,22 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | The RBC acknowledges train data. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | FINITE VALUE | two or more balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | FINITE VALUE | Unlinked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | 10 | Both Directions |
| L_PACKET | 13 | FINITE VALUE | Related to NID_PACKET |
| Q_RBC | 1 | 1 | Order to connect |
| NID_RBC | 24 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| Q_SLEEPSESSION | 1 | 1 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | FINITE VALUE | >=001 two or more balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | FINITE VALUE | Unlinked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 2 | L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 7: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | - |
| Train Data | VALID | Validated train data |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | Transition from Level NTC to level 2/3 area .When the ERTMS/ETCS communication session is open in rear of the boarder location, Train Data shall be sent to the RBC (which acknowledges the data). |  | 6 |
| Applicable Mode/Level Combinations | LNTC: SN / SL / NL |  |  |
| Target of Test | The target of this test is to check that: Transition from Level NTC to level 2/3 area. Train location: in rear of the boarder location. The on board system opens a communication session with the RBC in rear of the border location. Train Data sent to RBC ( which acknowledges the data) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-5.10.3.7.2 |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The balise group message including packet 42 (Session Management ordering the establishment of the radio communication session) is STORED The establishment of the Radio communication session is STORED The message 129 (Validated Train Data) is STORED The message 8 (Acknowledgement of Train Data) is STORED Check on the RTM that : The On-board system establishes a communication session when the packet 42 is received on board. Train data is sent to the RBC when the communication session has been established The message 8 “Acknowledgement of train data” is received On Board |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 / 13 | SL / NL / SN |
| Radio communication session | NOT ESTABLISHED | - |
| Level Transition Order | STORED | From LNTC to L2/L3 at a further location |
| Train Data | VALID | Validated train data |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | Train location: in rear of the border of level transition location | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 42 is received . | I | BTM | Packet 42: Q_RBC = 1 Q_SLEEPSESSION=1 The packet 42 is received on board via balise group in rear of the border location | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | A Radio Communication Session ordered by the trackside is established | - | - | USE_ FT3050300.21,22 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | This message is not sent in NL or SL mode | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | The RBC acknowledges train data. This message is not sent in NL or SL mode | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | FINITE VALUE | two or more balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | FINITE VALUE | Unlinked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | 10 | Both Directions |
| L_PACKET | 13 | FINITE VALUE | Related to NID_PACKET |
| Q_RBC | 1 | 1 | Order to connect |
| NID_RBC | 24 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | FINITE VALUE | >=001 two or more balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | FINITE VALUE | Unlinked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 1 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. Values to be assigned according to 7.3.1.3 |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 7: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 / 13 | SL / NL / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | - |
| Train Data | VALID | Validated train data |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | Transition from Level 0 ( Unfitted) to level 2/3 area. Train approaches from L0 area to level 2/3 area, receives the order to contact RBC in rear of the border location, opens the session, and sends train data. |  | 7 |
| Applicable Mode/Level Combinations | L0: UN/ SL/ NL |  |  |
| Target of Test | The target of this test is to check that: The On-board Equipment receives the order to contact RBC. Opens the communication session . Sends train data (except when it is in SL or NL mode). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-5.10.3.2.2 |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU: The balise group message including packet 42 (Session Management) is STORED The establishment of the communication session is STORED The message 129 (Validated Train Data) is STORED The message 8 (Acknowledgement of Train Data) is STORED Check in the RTM: The On board system establishes a communication session when the packet 42 is received on board. Train data is sent to the RBC when the communication session has been established The message 8 “Acknowledgement of train data” is received On Board |
| Constraints | In SL or NL modes no Train Data shall be sent. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 11 | UN / SL / NL |
| Radio communication session | NOT ESTABLISHED | - |
| Level Transition Order | STORED | From L0 to L2/L3 at a further location |
| Train Data | VALID | Validated train data |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | Train location: in rear of the border of level transition location | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 42 is received . | I | BTM | Packet 42: Q_RBC = 1 Q_SLEEPSESSION=1 The packet 42 is received on board via balise group in rear of the border location | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | A Radio Communication Session ordered by the trackside is established | - | - | USE_ FT3050300.21,22 : “Establishing a communication session initiated by On-board via RBC ID and telephone number" | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | This message is not sent in NL or SL mode | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | The RBC acknowledges train data. This message is not sent in NL or SL mode | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | FINITE VALUE | two or more balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | FINITE VALUE | Unlinked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | 10 | Both Directions |
| L_PACKET | 13 | FINITE VALUE | Related to NID_PACKET |
| Q_RBC | 1 | 1 | Order to connect |
| NID_RBC | 24 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | FINITE VALUE | >=001 two or more balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | FINITE VALUE | Unlinked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 0 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 7: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 / 13 | SL / NL / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | - |
| Train Data | VALID | Validated train data |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | Leading engines are requested to terminate session after announcement of the complete exit from level 2/3 area to level 1 area with a position report to RBC and are requested to terminate session; NL engines are only requested to terminate the session. |  | 8 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / NL |  |  |
| Target of Test | The target of the test is to check that a leading engine leaving L2/3 area after the transition to L1 is requested: To send a position report in order to report to RBC the complete exit (the train has passed the level transition border with its min safe rear end, i.e. when the whole train has left the area,) from level 2/3 area To terminate the communication session. The target of test is also to check that a non leading engine is required only to terminate the session. The function " Report train position when train rear passes a level transition border (from level 2/3 to 0, NTC, 1)" is active . |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.3.3 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 f) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train rear passes a level transition border (from level 2/3 to 0, NTC, 1) |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: The relevant messages exchanged between On-board Equipment and RBC |
| Constraints | The border balise group telegrams is received, it becomes LRBG. Also to check that a non leading engine is required only to terminate the session without sending a position report. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS / OS / SR / NL / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Position Report parameters | STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | On-board Equipment checks the train position and detects that the train entered completely in L1 area A balise group is located at the border, which becomes LRBG D_LRBG = L_TRAININT Note: this step is not carried out when the mode is NL | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | Note: this step is not carried out when the mode is NL Note: this step is not carried out for non leading engines | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Note: this step is not carried out when the mode is NL Note: this step is not carried out for non leading engines | N/A | N/A | - |
| 4 | N/A | N/A | The RBC orders to terminate the Communication Session. | - | - | USE_FT3050500.5: Termination of communication session ordered by RBC Steps concerning also the non leading engine. | N/A | N/A | - |

| Step 2: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | The balise group placed on the border of transition areas |
| D_LRBG | 15 | FINITE VALUE | = L_TRAININT |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | 1 | Train integrity confirmed by integrity monitoring devices |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 2 | L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS / OS / SR / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| Movement Authority | STORED | - |
| Position Report parameters | STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | Leading engines announce the complete exit from level 2/3 area to LEVEL 0 area with a position report to RBC and are requested to terminate session ; NL engines are only requested to terminate the session. |  | 9 |
| Applicable Mode/Level Combinations | L0 : UN / NL |  |  |
| Target of Test | The target of the test is to check that a leading engine leaving L2/3 area after the transition to L0 is requested: To send a position report in order to report to RBC the complete exit (the train has passed the level transition border with its min safe rear end, i.e. when the whole train has left the area,) from level 2/3 area To terminate the communication session. The target of test is also to check that a non leading engine is required only to terminate the session. The function Report train position when train rear passes a level transition border (from level 2/3 to 0, NTC, 1) is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.6.2 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 f) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train rear passes a level transition border (from level 2/3 to 0, NTC, 1) |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU: The relevant messages exchanged between On-board Equipment and RBC. |
| Constraints | The border balise group telegrams are received, it becomes LRBG. The non leading engines are required only to terminate the communication session without sending a position report. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 11 | UN / NL |
| Radio communication session | ESTABLISHED | - |
| Position Report parameters | STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | On-board Equipment checks the train position and detects that train entered completely in L0 area A balise group is located at the border, which becomes LRBG D_LRBG = L_TRAININT Note: this step is not carried out when the mode is NL | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | Note: this step is not carried out when the mode is NL Note: this step is not carried out for non leading engines | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Note: this step is not carried out when the mode is NL Note: this step is not carried out for non leading engines | N/A | N/A | - |
| 4 | N/A | N/A | General Message 24 including packet 42 “Session management " sent by RBC General Message is RECORDED On-board Equipment answers with a message 156 “Termination of a communication session” Message 156 is RECORDED RBC acknowledges termination of communication session with message 39 message 39 is RECORDED | - | - | Steps concerning also the NL engine USE_ FT3050500.5: Termination of communication session ordered by RBC | N/A | N/A | - |

| Step 2: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | The balise group placed on the border of transition areas |
| D_LRBG | 15 | FINITE VALUE | = L_TRAININT |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | 1 | Train integrity confirmed by integrity monitoring devices |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 0 | L0 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 11 | UN / NL |
| Radio communication session | NOT ESTABLISHED | - |
| Position Report parameters | STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | Leading engines announce the complete exit (with its min safe rear end ) from level 2 / 3 to LEVEL NTC area with a position report to RBC and are requested to terminate session ; NL engines are only requested to terminate the session. |  | 10 |
| Applicable Mode/Level Combinations | LNTC : SN / NL |  |  |
| Target of Test | The target of the test is to check that a leading engine leaving L2/3 area to LNTC area is requested: To send a position report in order to report to RBC the complete exit (the train has passed the level transition border with its min safe rear end, i.e. when the whole train has left the level 2/3 area,) from level 2/3 area To terminate the communication session The target of test is also to check that a non leading engine is required only to terminate the session. The function Report train position when train rear passes a level transition border (from level 2/3 to 0, NTC, 1) is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.10.3 |  |
|  |  | Subset-026-3.6.5.1.2 |  |
|  |  | Subset-026-3.6.5.1.2 f) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train rear passes a level transition border (from level 2/3 to 0, NTC, 1) |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU: The relevant messages exchanged between On-board Equipment and RBC |
| Constraints | The border balise group telegrams are received, it becomes LRBG. Also to check that a non leading engine is required only to terminate the session without sending a position report. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 / 13 | NL / SN |
| Radio communication session | ESTABLISHED | - |
| Position Report parameters | STORED | Position report is required when the min safe rear end of train has passed the border |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | On-board Equipment checks the train position and detects that train entered completely in LNTC area. A balise group is located at the border, which becomes LRBG D_LRBG = L_TRAININT Note: this step is not carried out when the mode is NL | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | Note: this step is not carried out when the mode is NL Note: this step is not carried out for non leading engines | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Note: this step is not carried out when the mode is NL Note: this step is not carried out for non leading engines | N/A | N/A | - |
| 4 | N/A | N/A | General Message 24 including packet 42 “Session management " sent by RBC General Message is RECORDED On-board Equipment answers with a message 156 “Termination of a communication session” Message 156 is RECORDED RBC acknowledges termination of communication session with message 39 message 39 is RECORDED | - | - | Steps concerning also the NL engine USE_ FT3050500.5: Termination of communication session ordered by RBC | N/A | N/A | - |

| Step 2: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | The balise group placed on the border of transition areas |
| D_LRBG | 15 | FINITE VALUE | = L_TRAININT |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | 1 | Train integrity confirmed by integrity monitoring devices |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 1 | LNTC |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 / 13 | NL / SN |
| Radio communication session | NOT ESTABLISHED | - |
| Position Report parameters | STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | The train has executed the Level Transition from L1 to Level 2 and no L2/L3 MA has been received. The train is supervised with the Level 1 MA that covers the first section of the Level 2 area. |  | 11 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The train is supervised with the Level 1 MA after the execution of the level transition if there is no L2/L3 MA stored on board. The function "Determine ERTMS/ETCS level" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The train is supervised by the same MA after the execution of the Level Transition. Check on the TIU that: When the level transition is performed, brakes are not APPLIED (The train is supervised according to the L1 MA after the Level Transition). |
| Constraints | No L2 Movement authority and track description shall be sent to the on-board equipment when entering in the new area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | Valid for L1 and covers at least the first section of the new level area |
| Gradient Profile | STORED | Valid for L1 and covers at least the first section of the new level area |
| International SSP | STORED | Valid for L1 and covers at least the first section of the new level area |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed The speed and distance monitoring information is displayed. | O | DMI | (on driver request in LS/OS mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS/OS/ LS | The on-board changes the level | - | - | USE_FT5100100.37 | L2 | FS/OS/ LS | - |
| 2 | L2 | FS/OS/ LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3) is recorded | O | JRU | Triggering event: When the level changes | L2 | FS/OS/ LS | - |
| 3 | L2 | FS/OS/ LS | Service brake not commanded | O | TIU | - | L2 | FS/OS/ LS | - |
| 4 | L2 | FS/OS/ LS | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Step considered only in OS / LS | L2 | FS/OS/ LS | - |
| 5 | L2 | FS/OS/ LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Step considered only in OS / LS | L2 | FS/OS/ LS | - |
| 6 | L2 | FS/OS/ LS | The speed and distance monitoring information is displayed. | O | DMI | - | L2 | FS/OS/ LS | - |
| 7 | L2 | FS/OS/ LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded. | O | JRU | According to L1 MA | L2 | FS/OS/ LS | - |
| 8 | L2 | FS/OS/ LS | The train is moving. | I | INT | Train is moving with L1 MA until the EoA of the section or until a L2/L3 MA is received. | L2 | FS/OS/ LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | No L2 MA stored. Same L1 MA from the starting conditions |
| Gradient Profile | STORED | Not updated. Same information from the starting conditions |
| International SSP | STORED | Not updated. Same information from the starting conditions |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 2" is displayed The speed and distance monitoring information is displayed. | O | DMI | DMI Object: LE04 |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | The train has executed the Level Transition from L1 to Level 3 and no L2/L3 MA has been received, the train is supervised with the Level 1 MA that covers the first section of the Level 3 area. |  | 12 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The train is supervised with the Level 1 MA after the execution of the level transition if there is no L2/L3 MA stored on board The function "Determine ERTMS/ETCS level" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The train is supervised by the same MA after the execution of the Level Transition Check on the TIU that: When the level transition is performed, brakes are not APPLIED (The train is supervised according to the L1 MA after the Level Transition). |
| Constraints | No L3 Movement authority and track description shall be sent to the on-board equipment when entering in the new area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | Valid for L1 and covers at least the first section of the new level area |
| Gradient Profile | STORED | Valid for L1 and covers at least the first section of the new level area |
| International SSP | STORED | Valid for L1 and covers at least the first section of the new level area |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed The speed and distance monitoring information is displayed. | O | DMI | (on driver request in LS/OS mode) |
| NOT RELEVANT | - | BTM | - |
| Train is moving | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS/OS/ LS | The on-board changes the level | - | - | USE_FT5100100.36 | L3 | FS/OS/ LS | - |
| 2 | L3 | FS/OS/ LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=4) is recorded | O | JRU | Triggering event: When the level changes | L3 | FS/OS/ LS | - |
| 3 | L3 | FS/OS/ LS | Service brake not commanded | O | TIU | - | L3 | FS/OS/ LS | - |
| 4 | L3 | FS/OS/ LS | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Step considered only in OS / LS | L3 | FS/OS/ LS | - |
| 5 | L3 | FS/OS/ LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Step considered only in OS / LS | L3 | FS/OS/ LS | - |
| 6 | L3 | FS/OS/ LS | The speed and distance monitoring information is displayed. | O | DMI | - | L3 | FS/OS/ LS | - |
| 7 | L3 | FS/OS/ LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded. | O | JRU | According to L1 MA | L2 | FS/OS/ LS | - |
| 8 | L3 | FS/OS/ LS | The train is moving. | I | INT | Train is moving with L1 MA until the EoA of the section or until a L2/L3 MA is received. | L3 | FS/OS/ LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | No L3 MA information stored. Same L1 MA information from Starting conditions |
| Gradient Profile | STORED | Not updated. Same information from the starting conditions |
| International SSP | STORED | Not updated. Same information from the starting conditions |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 3" is displayed The speed and distance monitoring information is displayed. | O | DMI | DMI Object: LE05 |
| NOT RELEVANT | - | BTM | - |
| Train is moving | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | If no Level 1 MA and track description has been received when entering the new area, the train shall still be supervised according to the level 2 MA previously received from the RBC. |  | 13 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The train executes the Level Transition to Level 1 and no L1 MA and track description has been received. The train is supervised with the Level 2 MA that covers the first section of the Level 1 area. The function "Determine ERTMS/ETCS level" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.3.2 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The train is supervised by the same MA after the execution of the Level Transition. Check on the TIU that: When the level transition is performed, brakes are not APPLIED (The train is supervised according to the L2 MA after the Level Transition). |
| Constraints | No Level 1 MA and track description shall be sent to the on-board when entering in the new area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | Received from RBC and valid for L2 ; covers at least the first section of the new level area |
| Gradient Profile | STORED | Valid for L2 and covers at least the first section of the new level area |
| International SSP | STORED | Valid for L2 and covers at least the first section of the new level area |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" is displayed | O | DMI | DMI Object: symbol LE04 |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 | FS/OS/ LS | Immediate level transition from L2 to L1is performed | - | - | USE_FT5100100.32 | L1 | FS/OS/ LS | - |
| 2 | L1 | FS/OS/ LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=2) is recorded | O | JRU | - | L1 | FS/OS/ LS | - |
| 3 | L1 | FS/OS/ LS | Service brake not commanded | O | TIU | - | L1 | FS/OS/ LS | - |
| 4 | L1 | FS/OS/ LS | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Step considered only in OS / LS | L1 | FS/OS/ LS | - |
| 5 | L1 | FS/OS/ LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0001 1011) is recorded | O | JRU | Step considered only in OS / LS | L1 | FS/OS/ LS | - |
| 6 | L1 | FS/OS/ LS | The speed and distance monitoring information is displayed. | O | DMI | - | L1 | FS/OS/ LS | - |
| 7 | L1 | FS/OS/ LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded. | O | JRU | According to L2 MA stored on-board | L1 | FS/OS/ LS | - |
| 8 | L1 | FS/OS/ LS | The train is moving. | I | INT | Train is moving with L2 MA until the EoA of the section or until a L1 MA is received. | L1 | FS/OS/ LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | No L1 MA information stored. Same L2 MA information from Starting conditions |
| Gradient Profile | STORED | Not updated. Same description from Starting conditions |
| International SSP | STORED | Not updated. Same description from Starting conditions |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | According to L2 MA |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | If no Level 1 MA and track description has been received when entering the new area, the train shall still be supervised according to the level 3 MA previously received from the RBC. |  | 14 |
| Applicable Mode/Level Combinations | L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The train executes the Level Transition to Level 1 and no L1 MA and track description has been received, the train is supervised with the Level 3 MA that covers the first section of the Level 1 area No service brake is applied. The function "Determine ERTMS/ETCS level" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.3.2 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The train is supervised by the same MA after the execution of the Level Transition Check on the TIU that: When the level transition is performed, brakes are not APPLIED (The train is supervised according to the L3 MA after the Level Transition). |
| Constraints | No Level 1 MA and track description shall be sent to the on-board when entering in the new area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | Received from RBC and valid for L3 ; covers at least the first section of the new level area |
| Gradient Profile | STORED | Valid for L3 and covers at least the first section of the new level area |
| International SSP | STORED | Valid for L3 and covers at least the first section of the new level area |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 3" is displayed | O | DMI | DMI Object: Symbol LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L3 | FS/OS/ LS | Immediate level transition from L3 to L1 is performed | - | - | USE_FT5100100.28 | L1 | FS/OS/ LS | - |
| 2 | L1 | FS/OS/ LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=2) is recorded | O | JRU | - | L1 | FS/OS/ LS | - |
| 3 | L1 | FS/OS/ LS | Service brake not commanded | O | TIU | - | L1 | FS/OS/ LS | - |
| 4 | L1 | FS/OS/ LS | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Step considered only in OS / LS | L1 | FS/OS/ LS | - |
| 5 | L1 | FS/OS/ LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1011) is recorded | O | JRU | Step considered only in OS / LS | L1 | FS/OS/ LS | - |
| 6 | L1 | FS/OS/ LS | The speed and distance monitoring information is displayed. | O | DMI | - | L1 | FS/OS/ LS | - |
| 7 | L1 | FS/OS/ LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded. | O | JRU | According to L3 MA stored on-board | L1 | FS/OS/ LS | - |
| 8 | L1 | FS/OS/ LS | The train is moving. | I | INT | Train is moving with L3 MA until the EoA of the section or until a L1 MA is received. | L1 | FS/OS/ LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | No L1 MA information stored. Same L3 MA information from Starting conditions |
| Gradient Profile | STORED | Not updated. Same description from Starting conditions |
| International SSP | STORED | Not updated. Same description from Starting conditions |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | According to L3 MA |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | When the level transition location is passed with the estimated front end a position report shall be sent to the RBC. In case the ERTMS/ETCS on-board equipment is interfaced to the National System through an STM, please refer to SUBSET-035 for the STM state transition order. |  | 15 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of test is to check that: When the level transition location is passed with the estimated front end a position report shall be sent to the RBC after a level transition from LNTC to L2/L3 The function "Report train position when change of level due to trackside order" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.7.5 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 g) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when change of level due to trackside order |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Indication of current level (Symbol LE04 or LE05) is DISPLAYED. Check on the JRU that: Packet 0 is RECORDED including the current level (M_LEVEL). |
| Constraints | The transition to the current level is fulfilled. The train reports its position by means of Packet 0. Radio connection established before the transition. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Radio communication session | ESTABLISHED | with relevant RBC before the level transition |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SN | Order to switch immediately and execution of level transition LNTC / L2 OR Order to switch immediately and execution of level transition LNTC / L3 | - | - | USE_ FT5100100.21 (for transitions LNTC/L2) Or USE_FT5100100.20 (for transitions LNTC/L3) | LNTC | SN | - |
| 2 | LNTC | SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3/4) is recorded | O | JRU | Triggering event: When the level changes | L2 / L3 | FS / LS / OS / SR / TR / NL | - |
| 3 | L2/L3 | FS / LS / OS / SR / TR / NL | The Level symbol "Acknowledge Level 2/ Level 3" is displayed. | O | DMI | DMI Object: LE13/LE15 This step applies only if a previous driver ack was not performed before the level transition. | L2/L3 | FS / LS / OS / SR / TR / NL | - |
| 4 | L2/L3 | FS / LS / OS / SR / TR / NL | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit56=1>) is recorded | O | JRU | This step applies only if a previous driver ack was not performed before the level transition. | L2/L3 | FS / LS / OS / SR / TR / NL | - |
| 5 | L2/L3 | FS / LS / OS / SR / TR / NL | The driver confirms the "Level" | I | DMI | This step applies only if a previous driver ack was not performed before the level transition. | L2/L3 | FS / LS / OS / SR / TR / NL | - |
| 6 | L2/L3 | FS / LS / OS / SR / TR / NL | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | This step applies only if a previous driver ack was not performed before the level transition. | L2/L3 | FS / LS / OS / SR / TR / NL | - |
| 7 | L2/L3 | FS / LS / OS / SR / TR / NL | Train is moving. | I | INT | Train location: the estimated front end of the train passes the level transition location | L2/L3 | FS / LS / OS / SR / TR / NL | - |
| 8 | L2/L3 | FS / LS / OS / SR / TR / NL | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | L2/L3 | FS / LS / OS / SR / TR / NL | - |
| 9 | L2/L3 | FS / LS / OS / SR / TR / NL | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/L3 | FS / LS / OS / SR / TR / NL | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 11 / 12 | FS / OS / SR / TR / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is DISPLAYED. | O | DMI | DMI Object: LE04 / LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3: Specific Additional Requirements |  | 5100300 |
| Test Case of Feature | The ERTMS/ETCS communication session is open but Train Data shall not be sent to the RBC (which acknowledges the data) when the on-board equipment is in SL or NL mode. |  | 16 |
| Applicable Mode/Level Combinations | L1: SL / NL |  |  |
| Target of Test | The target of this test is to check that: Reception of a Transition order from Level 1 to Level 2/3 area. The on-board equipment establishes a communication session with the responsible RBC in rear of the border location. Train Data are not sent to RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-5.10.3.1.3 |  |
|  |  | Subset-026-5.10.3.2.2 |  |
|  |  | Subset-026-5.10.3.7.2 |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The balise group message including packet 42 (Session Management) is STORED The establishment of the communication session is STORED Check on the RTM interface that: The message 129 (Validated Train Data) is not transmitted. |
| Constraints | In SL or NL modes no Train Data shall be sent. The Radio Session shall be established in SL mode.(Q_SLEEPSESION=1) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 | SL / NL |
| Radio communication session | NOT ESTABLISHED | - |
| Level Transition Order | STORED | - |
| Train Data | VALID | Validated train data |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | Train location: in rear of the border of level transition location | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 42 is received. | I | BTM | Packet 42: Q_RBC = 1 Q_SLEEPSESSION=1 The packet 42 is received on board via balise group in rear of the border location | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Radio Communication Session is ESTABLISHED | - | - | USE_ FT3050300.21, 22 | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | two or more balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | FINITE VALUE | Unlinked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | 10 | Both Directions |
| L_PACKET | 13 | FINITE VALUE | Related to NID_PACKET |
| Q_RBC | 1 | 1 | Order to connect |
| NID_RBC | 24 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| Q_SLEEPSESSION | 1 | 1 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | Second |
| N_TOTAL | 3 | 1 | two or more balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | FINITE VALUE | Unlinked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 | SL / NL |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | - |
| Train Data | VALID | same valid train data |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |