| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080453 (4.8.4: Accepted Information depending on the modes - Data to be used by applications outside ERTMS/ETCS) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-4080453 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 31.03.2013 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 16.10.2013 | All | Second version/Internal modification;TC_Editorv28; Correction of Requirements blank space; Additional correction made by the author (See review sheet) | INECO |
| Version 2.9.2 02.12.2013 | All | Third version/ Requirements related with table 4.8.3 and 4.8.4 removed; requirement 3.20.1.2 added. | INECO |
| Version 2.9.3 02.01.2014 | All | Fourth version/ Internal version: Cross-reference checked; Old reference removed, Requirement 4.5.2.1 table – Provide Juridical Data is added | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Data to be used by applications outside ERTMS/ETCS |  | 4080453 |
| Test Case of Feature | The on-board equipment receives a radio message from the RBC containing Data to be used by systems outside ETCS. The message is recorded in the JRU. |  | 1 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN L1 / L2 / L3: FS / LS / OS / SR / SH / PS / SL/ SB / TR / PT / NL/ RV |  |  |
| Target of Test | The target of this test is to check that : Data from RBC to be used by systems outside ETCS are ignored by the on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.20.1.2 |  |
|  |  | Subset-026-4.5.2.1 table - Provide Juridical Data |  |
|  |  | Subset-026-7.4.2.11 Packet Number 44: Data used by applications outside the ERTMS/ETCS system |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Radio message is received. Check on the JRU that: The received radio message containing Data to be used by a system outside ETCS is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio message (NID_MESSAGE=24; M_ACK=1) containing packet 44 (NID_XUSER=102)is received. | I | RTM | These data are ignored by the on-board | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 44 | Data used by applications outside the ERTMS/ETCS system |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_XUSER | 9 | 102 | Identifier of the target system |
| NID_NTC | 8 | FINITE VALUE | Only if NID_XUSER= 102 |
| Other data, depending on NID_XUSER | - | - | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Data to be used by applications outside ERTMS/ETCS |  | 4080453 |
| Test Case of Feature | The on-board equipment receives a balise message containing Data to be used by a system outside ETCS. The message is recorded in the JRU. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1 / L2 / L3: FS / LS / OS / SR / SH / PS / SL/ SB / TR / PT / NL/ RV |  |  |
| Target of Test | The target of this test is to check that : Data from Eurobalise to be used by a system outside ETCS are ignored by the on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.20.1.2 |  |
|  |  | Subset-026-4.5.2.1 table - Provide Juridical Data |  |
|  |  | Subset-026-7.4.2.11 Packet Number 44: Data used by applications outside the ERTMS/ETCS system |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM that: BG message is received. Check on JRU that: The received balise message containing Data to be used by a system outside ETCS is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 44 (NID_XUSER=102)is received. | I | BTM | These data are ignored by the on-board | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | 1 | 2 balises in the group |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 44 | Data used by applications outside the ERTMS/ETCS system |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_XUSER | 9 | 102 | Identifier of the target system |
| NID_NTC | 8 | FINITE VALUE | Only if NID_XUSER= 102 |
| Other data, depending on NID_XUSER | - | - | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | 1 | 2 balises in the group |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |