| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5100200 (5.10.2:Table of priority of trackside supported levels) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-5100200 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 03.01.2014 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3; TC Editor tool used. | INECO |
| Version 2.9.1 19.11.2013 | All | Second version/ UNISIG Review from 02.10.2013 included. | INECO |
| Version 2.9.2 10.12.2013 | All | Third version/ UNISIG Review from 03.01.2014 included. | INECO |
| Version 2.9.3 03.01.2014 | All | Fourth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.2:Table of priority of trackside supported levels |  | 5100200 |
| Test Case of Feature | The train is fitted for level 0, level NTC, level 1 and level 2. The train receives an announcement of a level transition and the supported levels are level 3, level 2, and level 1and level 0. The on-board equipment selects the level with the highest priority that it “Available to use” which is level 2. The driver is informed of the selected level. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN/ SL/ TR/ NL LNTC: SN/ SL/ TR/ NL L1 : FS / LS / OS / SR / SL / TR / NL |  |  |
| Target of Test | The target of this test is to check that: The announcement of level transition containing several supported levels is received. The on-board equipment selects the level with the highest priority that it Available to use.(the level is configured on-board) The selected level is displayed to the driver |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.2.4 |  |
|  |  | Subset-026-5.10.2.4.1 |  |
|  |  | Subset-026-5.10.2.4.1 a) |  |
|  |  | Subset-026-5.10.2.4.1 c) |  |
|  |  | Subset-026-5.10.2.6 |  |
|  |  | Subset-026-5.10.2.8 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol “announcement of level 2” is displayed to the driver. Check on the JRU that : The Level Transition order is recorded ( packet 41 ) |
| Constraints | Level 2 is configured on-board and at least one Mobile Terminal is in working condition (registered or not to a network). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 11 / 12 / 13 | FS / OS / SR / UN / SL / TR / NL / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |
| Table of priority of trackside supported levels | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41 is received. | I | RTM | A table of priority of all supported levels in the next area is received. D_LEVELTR <> 0 and <> 32767 M_LEVELTR(1) = 4 M_LEVELTR(2) = 3 M_LEVELTR(3) = 2 M_LEVELTR(4) = 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Level 2 announcement" is displayed. | O | DMI | The on-board equipment selects the level with the highest priority that it available for use : L2 In SL mode there is no announcement displayed DMI Object: LE12 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <> 32767 |
| M_LEVELTR | 3 | 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(1) | 3 | 3 | - |
| NID_NTC(1) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(1) | 15 | FINITE VALUE | - |
| N_ITER(1) | 5 | FINITE VALUE | - |
| M_LEVELTR(2) | 3 | 2 | - |
| NID_NTC(2) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(2) | 15 | FINITE VALUE | - |
| N_ITER(2) | 5 | FINITE VALUE | - |
| M_LEVELTR(3) | 3 | 0 | - |
| NID_NTC(3) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(3) | 15 | FINITE VALUE | - |
| N_ITER(3) | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 11 / 12 / 13 | FS / OS / SR / UN / SL / TR / NL / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | D_LEVELTR=FINITE VALUE <>0 and <>32767 M_LEVELTR=3 Transition to level 2 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2 announcement" is displayed | O | DMI | DMI Object: LE12 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.2:Table of priority of trackside supported levels |  | 5100200 |
| Test Case of Feature | The train is fitted for level 0, level 1 and level 2. The train receives level transition information (order to switch immediately) and the priority list levels are level 3, level 2, level 1 and level 0. The on-board equipment selects the level with the highest priority that it is available to use: level 2 and switch to it. The driver is informed of the selected level. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN / SL/ TR / NL L1 : FS / LS / OS / SR / SL / TR / NL |  |  |
| Target of Test | The target of this test is to check that: The level transition order containing several supported level is received. The on-board equipment selects the level with the highest priority that it is available to use and switched to that level The selected level is displayed to the driver |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.2.4 |  |
|  |  | Subset-026-5.10.2.4.1 |  |
|  |  | Subset-026-5.10.2.4.1 a) |  |
|  |  | Subset-026-5.10.2.4.1 c) |  |
|  |  | Subset-026-5.10.2.5 |  |
|  |  | Subset-026-5.10.2.6 |  |
|  |  | Subset-026-5.10.2.8 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Level symbol "Level 2" is displayed to the driver. Check on the JRU that : The Level Transition order is recorded ( packet 41 ) The immediate Level Transition to Level 2 is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 7 / 11 / 12 | FS / OS / SR / UN / SL / TR / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 2 | L0 / L1 |
| Table of priority of trackside supported levels | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 | FS / LS/ OS / SR / UN/ SL / TR / NL | Balise Group message containing packet 41 is received. | I | BTM | A table of priority of all supported levels in the next area is received. D_LEVELTR = 0 or = 32767 (order to switch immediately) M_LEVELTR(1) = 4 M_LEVELTR(2) = 3 M_LEVELTR(3) = 2 M_LEVELTR(3) = 0 | L0 / L1 | FS / LS/ OS / SR / UN/ SL / TR / NL | - |
| 2 | L0 / L1 | FS / LS/ OS / SR / UN/ SL / TR / NL | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 | FS / LS/ OS / SR / UN/ SL / TR / NL | - |
| 3 | L0 / L1 | FS / LS/ OS / SR / UN/ SL / TR / NL | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3) is recorded | O | JRU | The on-board equipment selects the level with the highest priority that it available for use : L2 | L2 | FS / LS/ OS / SR / UN/ SL / TR / NL | - |
| 4 | L2 | FS / LS/ OS / SR / UN/ SL / TR / NL | The Level symbol "Level 2” is displayed. | O | DMI | DMI Object: Symbol LE04 In SL mode there is no mode symbol displayed | L2 | FS / LS/ OS / SR / UN/ SL / TR / NL | - |
| 5 | L2 | FS / LS/ OS / SR / UN/ SL / TR / NL | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | L2 | FS / LS/ OS / SR / UN/ SL / TR / NL | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | = 0 or = 32767 |
| M_LEVELTR | 3 | 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(1) | 3 | 3 | - |
| NID_NTC(1) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(1) | 15 | FINITE VALUE | - |
| N_ITER(1) | 5 | FINITE VALUE | - |
| M_LEVELTR(2) | 3 | 2 | - |
| NID_NTC(2) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(2) | 15 | FINITE VALUE | - |
| N_ITER(2) | 5 | FINITE VALUE | - |
| M_LEVELTR(3) | 3 | 0 | - |
| NID_NTC(3) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(3) | 15 | FINITE VALUE | - |
| N_ITER(3) | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 7 / 11 / 12 / 15 | FS / OS / SR / SH / UN / SL / TR / NL / LS / PS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 | L2 |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2 " is displayed. | O | DMI | DMI Object: LE04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.2:Table of priority of trackside supported levels |  | 5100200 |
| Test Case of Feature | The train is fitted for level 0 and level 1. The train receives a level transition order and the supported levels are level 3 and level 2. The on-board equipment is not fitted for any of the announced levels, it selects the level with the lowest priority: the level 2. The driver is informed of the selected level. |  | 3 |
| Applicable Mode/Level Combinations | L0 / L1 : FS / LS / OS / SR / UN / SL / SB / TR / PT / NL /RV |  |  |
| Target of Test | The target of this test is to check that: The level transition order containing several supported level is received. The on-board equipment selects the level with the lowest priority. The selected level is displayed to the driver |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.2.4 |  |
|  |  | Subset-026-5.10.2.4.1 |  |
|  |  | Subset-026-5.10.2.4.1 c) |  |
|  |  | Subset-026-5.10.2.5 |  |
|  |  | Subset-026-5.10.2.6 |  |
|  |  | Subset-026-5.10.2.7 |  |
|  |  | Subset-026-5.10.2.8 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The message “level transition announcement” is displayed to the driver. Check on the JRU that : The Level Transition order is recorded ( packet 41 ) |
| Constraints | Train is fitted for Level 0 and Level 1 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 14/ 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / RV / PS |
| ERTMS/ETCS level | 0 / 2 | L0 / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | Balise Group message containing packet 41 is received. | I | BTM | A table of priority of all supported levels in the next area is received: Priority 1: L3 Priority 2: L2 Packet 41: D_LEVELTR = 0 or = 32767 (order to switch immediately) M_LEVELTR(1) = 4 M_LEVELTR(2) = 3 | L0 / L1 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | - |
| 2 | L0 / L1 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | - |
| 3 | L0 / L1 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3) is recorded | O | JRU | The on-board equipment selects the level with the lowest priority, L2 even if is not available for use and makes the transition. (degraded situation) | L2 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | - |
| 4 | L2 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | The Level symbol "Level 2" is displayed | O | DMI | DMI Object: Symbol LE04 Not shown in SL mode | L2 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | - |
| 5 | L2 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | L2 | FS / LS/ OS / SR / SH / UN/ PS / SL / TR / NL | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | = 0 or = 32767 |
| M_LEVELTR | 3 | 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(1) | 3 | 3 | - |
| NID_NTC(1) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(1) | 15 | FINITE VALUE | - |
| N_ITER(1) | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14/ 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / RV / PS |
| Level Transition Order | NOT STORED | To level 2 |
| ERTMS/ETCS level | 3 | L2 |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2 announcement" is displayed | O | DMI | DMI Object: LE12 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.2:Table of priority of trackside supported levels |  | 5100200 |
| Test Case of Feature | The train is fitted for level 0, level NTC and level 1. The train receives level transition information (order to switch immediately) and the supported levels are level 0 and level NTC. The on-board equipment selects the level with the highest priority that it is available to use: level NTC. The driver is informed of the selected level. |  | 4 |
| Applicable Mode/Level Combinations | L0 / L1 / LNTC FS / LS / OS / SR / SH / UN / PS / SL / SB / TR / PT / NL / SE / SN / RV |  |  |
| Target of Test | The target of this test is to check that: The level transition order containing several supported level is received. The on-board equipment selects the level with the highest priority that it is available to use. The selected level is displayed to the driver |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.2.4 |  |
|  |  | Subset-026-5.10.2.4.1 |  |
|  |  | Subset-026-5.10.2.4.1 b) |  |
|  |  | Subset-026-5.10.2.4.1 c) |  |
|  |  | Subset-026-5.10.2.6 |  |
|  |  | Subset-026-5.10.2.8 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The message “announcement of level NTC” is displayed to the driver. Check on the JRU that : The Level Transition order is recorded ( packet 41 ) |
| Constraints | Level NTC is “available for use” if the concerned National System is available on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 7 / 11 / 12 / 13 | FS / OS / SR / UN / SL / TR / NL / LS /SN |
| ERTMS/ETCS level | 0/ 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 41 is received. | I | BTM | A table of priority of all supported levels in the next area is received: Priority 1: LNTC Priority 2: L0 Packet 41: D_LEVELTR <> 0 and <> 32767 (order to switch at further location) M_LEVELTR(1) = 1 M_LEVELTR(2) = 0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Level NTC announcement" is displayed | O | DMI | The on-board equipment selects the level with the highest priority that it is available to use : LNTC DMI Object: Symbol LE08 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <> 32767 |
| M_LEVELTR | 3 | 1 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(1) | 3 | 0 | - |
| NID_NTC(1) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(1) | 15 | FINITE VALUE | - |
| N_ITER(1) | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 14/ 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / RV / PS |
| Level Transition Order | STORED | To level NTC |
| ERTMS/ETCS level | 0/ 1 / 2 | L0 / LNTC / L1 |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: LE08 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |