| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4042000 (4.4.20: PASSIVE SHUNTING) Total: 5 Test case(s) |  |
| REF: | Subset-076-5-2-4042000 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 15.08.2013 | All | review | DLR |
| Version 2.9.2 22.11.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.20: PASSIVE SHUNTING |  | 4042000 |
| Test Case of Feature | As the engine is coupled to a leading engine, its ERTMS/ETCS on-board equipment shall not perform any train movement supervision. |  | 1 |
| Applicable Mode/Level Combinations | L0: SH, PS LNTC: SH, PS L1: SH, PS L2: SH, PS L3: SH, PS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment engine shall not perform any train movement supervision, when it is coupled to a leading engine. The special function “Continue Shunting on desk closure” shall allow one and only one transition from Shunting mode to Passive Shunting mode. For the negative test of req. 4.4.20.1.5 see TC 5. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.20.1.3 |  |
|  |  | Subset-026-4.4.20.1.4 |  |
|  |  | Subset-026-4.4.20.1.5 |  |
|  |  | Subset-026-4.4.20.1.6 |  |
|  |  | Subset-026-4.4.20.1.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : - Indication of SH mode is DISPLAYED. - The driver enables the function “Continue Shunting on desk closure” - passive shunting input signal is received from TIU - The driver closes the active desk - The train switches to PS mode - The train starts moving - The train receives a danger for shunting information and nothings happen - The train stops - The driver opens the desk - The special function “Continue Shunting on desk closure” shall be inactive once the Shunting mode is left. - When closing the desk with the function inactive the mode will change to SB |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | If in L2/L3. If not terminated because of End of Mission. |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Shunting" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SH | Train is at standstill (V_TRAIN=0) | I | INT | - | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SH | The Mode symbol "Shunting" is displayed | O | DMI | Indication of SH mode | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SH | Passive shunting permitted | I | TIU | “passive shunting input signal” is received from the train interface | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SH | The driver selects "MAINTAIN SHUNTING" | I | DMI | The driver selects "Continue Shunting on desk closure" | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The driver selects "Continue Shunting on desk closure" is recorded | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | SH | Cab not active | I | TIU | The driver closes the desk | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 7 | L0 / LNTC / L1 / L2 / L3 | SH | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | Closing of desk is recorded | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 8 | L0 / LNTC / L1 / L2 / L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=15) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 9 | L0 / LNTC / L1 / L2 / L3 | PS | Train is moving (V_TRAIN=40) | I | INT | - | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 10 | L0 / LNTC / L1 / L2 / L3 | PS | One Balise Group message (N_TOTAL=0) containing packet 132 (Q_ASPECT=0) is received | I | BTM | Packet 132 (danger for shunting), Q_ASPECT = 0 (Stop if in SH mode) | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 11 | L0 / LNTC / L1 / L2 / L3 | PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 12 | L0 / LNTC / L1 / L2 / L3 | PS | Emergency brake not commanded | O | TIU | - | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 13 | L0 / LNTC / L1 / L2 / L3 | PS | Train is at standstill (V_TRAIN=0) | I | INT | The driver stops the train | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 14 | L0 / LNTC / L1 / L2 / L3 | PS | Cab active | I | TIU | - | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 15 | L0 / LNTC / L1 / L2 / L3 | SH | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | Opening of desk is recorded | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 16 | L0 / LNTC / L1 / L2 / L3 | SH | The driver selects "Main" | I | DMI | - | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 17 | L0 / LNTC / L1 / L2 / L3 | SH | The Maintain Shunting button in the Main window is enabled | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 18 | L0 / LNTC / L1 / L2 / L3 | SH | Cab not active | I | TIU | The driver closes the desk | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 19 | L0 / LNTC / L1 / L2 / L3 | SH | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | Closing of desk is recorded | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 20 | L0 / LNTC / L1 / L2 / L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB | - |

| Step 10: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 132 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_ASPECT | 1 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT AFFECTED | - | RTM | - |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT AFFECTED | - | BTM | - |
| NOT AFFECTED | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.20: PASSIVE SHUNTING |  | 4042000 |
| Test Case of Feature | When receiving a communication session establishment order, the ERTMS/ETCS on-board in Passive Shunting mode shall not establish the communication session |  | 2 |
| Applicable Mode/Level Combinations | L0: PS LNTC: PS L1: PS L2: PS L3: PS |  |  |
| Target of Test | The target of this test is to check that: When receiving a communication session establishment order, the ERTMS/ETCS on-board in Passive Shunting mode shall not establish the communication session, but shall store the RBC ID/phone number information. The function “Manage “Stop Shunting on desk opening” information” is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.20.1.9 |  |
|  |  | Subset-026-4.4.20.1.12 |  |
|  |  | Subset-026-4.8.4.2 Session Management |  |
|  |  | Subset-026-4.10.1.3 line Radio Network ID |  |
|  |  | Subset-026-4.10.1.3 line RBC ID/Phone Number |  |
|  |  | Subset-026-7.4.2.31 Packet Number 135: Stop Shunting on desk opening |  |
|  |  | Subset-026-4.5.2.1 table - Manage "Stop Shunting on desk opening" information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: - The train is in PS mode, nothing is displayed - The train receives a session management packet - The train shall store the RBC ID/phone number information but shall not open a communication session - The train stops - The driver activates the cab - The OBU switches to SB mode (because Packet Number 135: Stop Shunting on desk opening is received) - The driver revalidates the driver ID - The driver revalidates the Train Data - The driver selects manual L2 when the last level was non equal to L2 - The train tries to connect the RBC which was announced by BTM until the train was in PS mode. |
| Constraints | The OBU is in PS mode |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| Level Transition Order | STORED | Only if L2/3: D_LEVELTR=0; M_LEVELTR=2 (Level 1) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | PS | Train is moving (V_TRAIN=40) | I | INT | - | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | PS | One Balise Group message (N_TOTAL=0) containing packet 42, packet 135 is received | I | BTM | NID_RBC, NID_RADIO are different from any stored or previously received ones | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | PS | NOT SA-CONNECT.Request is transmitted | O | RTM | The OBU shall not open a communication session | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | PS | Train is at standstill (V_TRAIN=0) | I | INT | - | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | PS | Cab active | I | TIU | The driver opens the desk | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 7 | L0 / LNTC / L1 / L2 / L3 | PS | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | PS | - |
| 8 | L0 / LNTC / L1 / L2 / L3 | PS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | Mode change to SB mode Only if L2/3: Level changes to L1 | L0 / LNTC / L1 | SB | - |
| 9 | L0 / LNTC / L1 | SB | The driver confirms “Level” | I | DMI | Only if position is not valid | L0 / LNTC / L1 | SB | - |
| 10 | L0 / LNTC / L1 | SB | The driver confirms the "Driver ID" | I | DMI | - | L0 / LNTC / L1 | SB | - |
| 11 | L0 / LNTC / L1 | SB | Any message (NID_MESSAGE_JRU=ALL; DRIVER_ID=<entered ID>) is recorded | O | JRU | DRIVER_ID is recorded | L0 / LNTC / L1 | SB | - |
| 12 | L0 / LNTC / L1 | SB | The driver selects "Level 2" and confirms "RBC Data" | I | DMI | Check that the RBC ID and Phone Number are thosewhich were announced by the Balise group in Step 2 | L0 / LNTC / L1 | SB | - |
| 13 | L2 | SB | Cab not active | I | TIU | The driver closes the desk | L2 | SB | - |
| 14 | L2 | SB | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | L2 | SB | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 135 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT AFFECTED | - | RTM | - |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT AFFECTED | - | BTM | - |
| NOT AFFECTED | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.20: PASSIVE SHUNTING |  | 4042000 |
| Test Case of Feature | When in Passive Shunting mode, the ERTMS/ETCS on-board shall not manage RBC-RBC hand-over |  | 3 |
| Applicable Mode/Level Combinations | L2: PS L3: PS |  |  |
| Target of Test | The target of this test is to check that: When in Passive Shunting mode, the ERTMS/ETCS on-board shall not manage RBC-RBC hand-over, except for storing the RBC ID/phone number information given at the RBC/RBC border. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.20.1.9 |  |
|  |  | Subset-026-4.4.20.1.13 |  |
|  |  | Subset-026-4.8.4.2 RBC transition order |  |
|  |  | Subset-026-4.8.4.2 [8] |  |
|  |  | Subset-026-4.10.1.3 line Radio Network ID |  |
|  |  | Subset-026-4.10.1.3 line RBC ID/Phone Number |  |
|  |  | Subset-026-4.10.1.3 line RBC Transition Order |  |
|  |  | Subset-026-7.4.2.31 Packet Number 135: Stop Shunting on desk opening |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: - The train is in PS mode, nothing is displayed - The train receives a RBC transition order - The train shall store the RBC ID/phone number information but shall not open a communication session - The train stops well after the RBC/RBC border - The driver activates the cab (because Packet Number 135: Stop Shunting on desk opening is received) - The OBU switches to SB mode - The driver revalidates the driver ID - The train tries to connect the RBC which was announced by BTM when the train was in PS mode. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| Level Transition Order | NOT STORED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | - |
| RBC ID/Phone Number | VALID | RBCx |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | PS | Train is moving (V_TRAIN=40) | I | INT | - | L2 / L3 | PS | - |
| 2 | L2 / L3 | PS | One Balise Group message (N_TOTAL=0) containing packet 131 (D_RBCTR=0), packet 135 is received | I | BTM | NID_RBC, NID_RADIO are different to the previously received one RBCx | L2 / L3 | PS | - |
| 3 | L2 / L3 | PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | PS | - |
| 4 | L2 / L3 | PS | NOT SA-CONNECT.Request is transmitted | O | RTM | The OBU shall not open a communication session | L2 / L3 | PS | - |
| 5 | L2 / L3 | PS | Train is at standstill (V_TRAIN=0) | I | INT | - | L2 / L3 | PS | - |
| 6 | L2 / L3 | PS | Cab active | I | TIU | The driver opens the desk | L2 / L3 | PS | - |
| 7 | L2 / L3 | PS | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | L2 / L3 | PS | - |
| 8 | L2 / L3 | PS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | Mode change to SB mode | L2 / L3 | SB | - |
| 9 | L2 / L3 | SB | The driver confirms the "Driver ID" | I | DMI | - | L2 / L3 | SB | - |
| 10 | L2 / L3 | SB | Any message (NID_MESSAGE_JRU=ALL; DRIVER_ID=<entered ID>) is recorded | O | JRU | DRIVER_ID is recorded | L2 / L3 | SB | - |
| 11 | L2 / L3 | SB | Use_FT3050300.28: "A communication session initiated by the on-board unit is established successfully" | - | - | Check that the connection is set up to RBCy with data from packet 131, step 2. Check NID_RBC and NID_RADIO | L2 / L3 | SB | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Acceptin. RBC identity. |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Accepting. RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 135 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT AFFECTED | - | BTM | - |
| NOT AFFECTED | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.20: PASSIVE SHUNTING |  | 4042000 |
| Test Case of Feature | In case of balise group message consistency error (refer to 3.16.2.4.4 and 3.16.2.5.1), the ERTMS/ETCS onboard equipment shall not command the service brake |  | 4 |
| Applicable Mode/Level Combinations | L0: SH, PS LNTC: SH, PS L1: SH, PS L2: SH, PS L3: SH, PS |  |  |
| Target of Test | The target of this test case is to check that In case of balise group message consistency error (refer to 3.16.2.4.4 and 3.16.2.5.1), the ERTMS/ETCS onboard equipment shall not command the service brake. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.2.4.4 |  |
|  |  | Subset-026-3.16.2.5.1 |  |
|  |  | Subset-026-4.4.20.1.15 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test has to be done with balises market as linked and unlinked. Check that: - The train is in PS mode - The train overpasses BG which has one Balise missed - SB is not applied - The train overpasses BG which telegram cannot be decoded - SB is not applied - The train overpasses BG which has invalid variables within message - SB is not applied - The train overpasses BG which message counter do not match - SB is not applied |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | If L2/L3. If not terminated because of previous End of Mission. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | One balise within the BG is missed | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (N_TOTAL=0) is received | I | BTM | A balise is detected, but no telegram is decoded (e.g. wrong CRC). | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message (N_TOTAL=0) is received | I | BTM | Telegram of the BG has invalid variables (E.g. M_DUP = 12) | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | One Balise Group message (N_TOTAL=0) is received | I | BTM | Telegram has a message counter which do not matches (E.g. M_MCOUNT=254) | N/A | N/A | - |
| 14 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1), One balise within the BG is missed |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1). Telegram of the BG cannot be decoded |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | 12 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 13: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | 254 | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group. Telegram has a messager counter which do not matches (E.g. M_MCOUNT=254) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | If L2/L3 |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT AFFECTED | - | BTM | - |
| NOT AFFECTED | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.20: PASSIVE SHUNTING |  | 4042000 |
| Test Case of Feature | It shall only be possible to enter in Passive Shunting mode from the Shunting mode; while in Shunting mode, the driver shall have the possibility to enable the function “Continue Shunting on desk closure”. |  | 5 |
| Applicable Mode/Level Combinations | L0: UN, PS, SL, SB, TR, SF, IS, NP, NL LNTC: PS, SL, SB, TR, SF, IS, NP, NL, SN L1: FS, LS, OS, SR, PS, SL, SB, TR, PT, SF, IS, NP, NL, RV L2: FS, LS, OS, SR, PS, SL, SB, TR, PT, SF, IS, NP, NL, RV L3: FS, LS, OS, SR, PS, SL, SB, TR, PT, SF, IS, NP, NL, RV |  |  |
| Target of Test | It is not possible to enable the function "Continue Shunting on desk closure" when not in SH mode. To fully cover req. 4.4.20.1.5 this is the negative test. The function availability is tested in TC 1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.20.1.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | It is not possible to enable the function “Continue Shunting on desk closure”. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 9 / 10 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / UN / SL / SB / TR / PT / SF / IS / NL / LS / NS / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | Passive shunting permitted | I | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The Maintain Shunting button in the Main window is disabled | O | DMI | It is not possible to select "Continue Shunting on desk closure" since the train is not in SH mode. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 9 / 10 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / UN / SL / SB / TR / PT / SF / IS / NL / LS / NS / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT AFFECTED | - | RTM | - |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT AFFECTED | - | BTM | - |
| NOT AFFECTED | - | INT | - |