| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4040400 (4.4.4: NO POWER) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-4040400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 16.10.2013 | All | Review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.4: NO POWER |  | 4040400 |
| Test Case of Feature | Permanent application of emergency brakes |  | 1 |
| Applicable Mode/Level Combinations | L0: NP LNTC: NP L1: NP L2: NP L3: NP |  |  |
| Target of Test | The target of this test case is to check that: - the emergency brake command is transmitted from TIU to train - the emergency brake command is recorded in JRU - no emergency brake revocation is transmitted from TIU to train as long as in NP mode - no emergency brake revocation is recorded in JRU as long as in NP mode |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX / INECO / DLR / MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.4.1.2 |  |
|  |  | Subset-026-4.5.2.1 table - Command Emergency Brake |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that - the “apply emergency brake”-command is transmitted from TIU to train equipment (emergency brake) immediately after changing to NP mode - no “apply emergency brake”-revocation is transmitted from TIU to train equipment (emergency brake) as long as in NP mode Check on the JRU that - the emergency brake command is recorded immediately after changing to NP mode - no emergency brake revocation is recorded as long as in NP mode |
| Constraints | Mode changes to and from NP are carried out, but not tested in the present test case. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L0 / LNTC / L1 / L2 / L3>" is not displayed and The Mode symbol "Mode <NP>" is not displayed | O | DMI | There is no output information in NP mode. MASK=0000000000000000000000; Mode and level NOT DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / UN / PS / SL / SB / TR / PT / SF / NL / SN / RV | Use_FT4060300.1 ( The ETCS on-board system is not powered (Transition [29])) | - | - | Transition to NP | L0 / LNTC / L1 / L2 / L3 | NP | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | NP | Emergency brake commanded | O | TIU | The emergency brake command is immediately after change to NP transmitted to train equipment (emergency brake). | L0 / LNTC / L1 / L2 / L3 | NP | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | NP | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | NO Indication on DMI in NP mode | L0 / LNTC / L1 / L2 / L3 | NP | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | NP | Emergency brake commanded | O | TIU | NO revocation of emergency brake is transmitted to train equipment (emergency brake) as long as in NP mode. | L0 / LNTC / L1 / L2 / L3 | NP | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| NO Indication on DMI in NP mode | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |