| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080420 (4.8.4: Accepted Information depending on the modes - Linking) Total: 10 Test case(s) |  |
| REF: | Subset-076-5-2-4080420 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.3.3 29.02.2012 | All | Final delivery after clean-up process according with Subset 026 v2.3.0 and Subset 108 v 1.2.0 | INECO/DLR/CEDEX |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 02.08.2013 | All | General revision | CEDEX |
| Version 2.9.2 29.11.2013 | All | Update of "Use_FT" steps | CEDEX |
| Version 2.9.3 10.12.2013 | TC 11 | Editorial error | CEDEX |
| Version 2.9.4 08.01.2014 | TC 4, 15, 16, 18 | General corrections | CEDEX |
| Version 2.9.5 21.01.2014 | 11 | delete preceded step modify constrain | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | The trackside generates and sends a message including packet 5 (Linking Information) via RBC to a train. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS, OS, LS L3: FS, OS, LS |  |  |
| Target of Test | The target of this test is to check that The message including packet 5 is accepted and stored on-board. The Linking is correct |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.6 |  |
|  |  | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-3.6.3.2.6 a) |  |
|  |  | Subset-026-3.6.3.2.6 b) |  |
|  |  | Subset-026-3.6.3.2.6 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 5 Linking Information is recorded. The Linking information is correct according to site data |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received | I | RTM | Complete message is accepted | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3160200.68 | - | - | The train uses the linking information: the system reacts according to the linking information when the next linked BG is not found. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | The trackside generates and sends a message including packet 5 (Linking) via RBC to a train which rejects the message because train data has been sent to the RBC and the acknowledgement of train data has not been received. |  | 2 |
| Applicable Mode/Level Combinations | L2: FS, OS, SR, SB, LS L3: FS, OS, SR, SB, LS |  |  |
| Target of Test | The target of this test is to check that The message including packet 5 is rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 5 Linking Information is recorded. No transition to TRIP mode is apply when the train arrives at the place of the first balise group of the linking information with reaction TRIP Check on the DMI that: No transition to TRIP mode is DISPLAYED. |
| Constraints | A cab needs to be active (SB Mode) The first balise in the received linking information should have TRIP reaction. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| ERTMS/ETCS level | 03/04 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS / OS / SR / SB / LS | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | Validated train data | L2/L3 | FS / OS / SR / SB / LS | - |
| 2 | L2/L3 | FS / OS / SR / SB / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) containing packet 0, packet 11 is recorded | O | JRU | Packet 1 must be used instead of Packet 0, if position report is based on 2 BG | L2/L3 | FS / OS / SR / SB / LS | - |
| 3 | L2 / L3 | FS / OS / SR / SB / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received | I | RTM | Linking information is rejected. | L2 / L3 | FS / OS / SR / SB / LS | - |
| 4 | L2 / L3 | FS / OS / SR / SB / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Linking information is recorded | L2 / L3 | FS / OS / SR / SB / LS | - |
| 5 | L2/L3 | FS / OS / SR / SB / LS | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2/L3 | FS / OS / SR / SB / LS | - |
| 6 | L2/L3 | FS / OS / SR / SB / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2/L3 | FS / OS / SR / SB / LS | - |
| 7 | L2 / L3 | FS / OS / SR / SB / LS | USE_FT4060300 for changing to FS, LS or OS mode | - | - | Only if in SB or SR mode | L2 / L3 | FS / OS / SR / SB / LS | - |
| 8 | L2 / L3 | FS / OS / LS | Not the Mode symbol "Trip" is displayed | O | DMI | The location of the announced (and rejected) linked BG is reached. Either no BG shall be placed in this location or, if BG is placed, the NID_BG shall not match with the information included in P5 | L2 / L3 | FS / OS / LS | - |
| 9 | L2 / L3 | FS / OS / LS | Not GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |
| NID_PACKET | 8 | 0 | Packet 1 must be used instead of Packet 0, if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 5: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 03/04 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | A telegram including packet 5 Linking Information is transmitted via balise group and is received by the onboard equipment. No linking data were received previously. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, OS, SR, SB, LS |  |  |
| Target of Test | The target of this test is to check that The Balise telegram, including packet 5, is accepted and stored on the JRU on-board. The Linking is correct. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.6 |  |
|  |  | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-3.6.3.2.6 a) |  |
|  |  | Subset-026-3.6.3.2.6 b) |  |
|  |  | Subset-026-3.6.3.2.6 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Balise telegram including packet 5 Linking Information is recorded. The Linking information is correct according to site data |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / SB / LS | One Balise Group message containing packet 5 is received | I | BTM | Complete message is accepted | L1 | FS / OS / SR / SB / LS | - |
| 2 | L1 | FS / OS / SR / SB / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Complete message is recorded | L1 | FS / OS / SR / SB / LS | - |
| 3 | L1 | FS / OS / SR / SB / LS | Use_FT4060300 for changing to FS, LS or OS mode | - | - | Only if in SB or SR mode | L1 | FS / OS / SR / SB / LS | - |
| 4 | L1 | FS / OS / LS | Use_FT3160200.68 | - | - | The train uses the linking information: the system reacts according to the linking information when the next linked BG is not found. | L1 | FS / OS / LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | The onboard equipment receives telegram including packet 5 Linking Information via balise group. The telegram is REJECTED. |  | 4 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L2: FS, OS, SR, SB, LS L3: FS, OS, SR, SB, LS |  |  |
| Target of Test | The target of this test is to check that The message including packet 5 is rejected and stored on-board . The first balise in the linking information should have TRIP reaction. Since packet 5 is REJECTED, no trip reaction is applied after changing to L1 first and passing over the location of the BG announced in the packet 5. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 5 Linking Information is recorded. No transition to TRIP mode is applied when the train arrives at the place of the first balise group of the linking information with reaction TRIP Check on the DMI that: No transition to TRIP mode is DISPLAYED |
| Constraints | A cab needs to be active (SB Mode) The first balise in the linking information should have TRIP reaction |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | One Balise Group message containing packet 5 is received | I | BTM | Complete message is rejected | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | - |
| 2 | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Complete message is recorded | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | - |
| 3 | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=2) is received | I | BTM | - | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | - |
| 4 | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | - |
| 5 | L0 / LNTC / L2 / L3 | FS / OS / SR / SB / LS | The Level symbol "Level 1" is displayed | O | DMI | - | L1 | FS / OS / SR / SB / LS | - |
| 6 | L1 | FS / OS / SR / SB / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | FS / OS / SR / SB / LS | - |
| 7 | L1 | FS / OS / SR / SB / LS | USE_FT4060300 for changing to FS, LS or OS mode | - | - | Only if in SB or SR mode | L1 | FS / OS / SR / SB / LS | - |
| 8 | L1 | FS / OS / LS | Not the Mode symbol "Trip" is displayed | O | DMI | The location of the announced (and rejected) linked BG is reached. Either no BG shall be placed in this location or, if BG is placed, the NID_BG shall not match with the information included in P5 | L1 | FS / OS / LS | - |
| 9 | L1 | FS / OS / LS | Not GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 | FS / OS / LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 2 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | The train passes over Euroloop and the On-Board equipment receives a message including linking data (Packet 5) and it accepts it. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | To check that the message including the packet 5 is decoded and stored on-board, and it accepts the message. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.6 |  |
|  |  | Subset-026-3.9.1.3 |  |
|  |  | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-4.8.4.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that the Euroloop Message including a packet 5 ‘Linking Information’ is recorded. |
| Constraints | The Onboard Equipment receives the Announcement of an Euroloop infill telegram |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| EOLM information | STORED | Q_SSCODE HAS BEEN RECEIVED |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message containing packet 5 is received | I | LTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | The complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3160200.68 | - | - | The train uses the linking information: the system reacts according to the linking information when the next linked BG is not found. | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| EOLM information | STORED | Q_SSCODE HAS BEEN RECIVED |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | The trackside generates and sends a message including packet 5 (Linking Information) via RBC to a train: |  | 6 |
| Applicable Mode/Level Combinations | L2: SR, SB, PT L3: SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that The message including packet 5 is accepted and stored on-board. Check the Linking is correct after changing to FS, OS or LS. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.6 |  |
|  |  | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 5 Linking Information is recorded. The train transit to FS The Linking information is correct according to site data |
| Constraints | A cab needs to be active (SB Mode) Recognition of Exit from TR mode (PT mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 6 / 8 | SR / SB / PT |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SR / SB / PT | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received | I | RTM | Complete message is accepted | L2 / L3 | SR / SB / PT | - |
| 2 | L2 / L3 | SR / SB / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded | L2 / L3 | SR / SB / PT | - |
| 3 | L2 / L3 | SR / SB / PT | Use_FT4060300 for changing to FS, LS or OS mode | - | - | - | L2 / L3 | SR / SB / PT | - |
| 4 | L2 / L3 | FS / OS / LS | Use_FT3160200.68 | - | - | The train uses the linking information: the system reacts according to the linking information when the next linked BG is not found. | L2 / L3 | FS / OS / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | The trackside generates and sends a message including packet 5 (Linking Information) via RBC to a train. The linking is accepted because a level transition has been received. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS, OS, SB, SR, LS L0: UN, SB LNTC: SN |  |  |
| Target of Test | The target of this test is to check that The message including packet 5 is accepted and stored on-board because a level transition has been received. The Linking is correct and shall be used by the on-board equipment after changing to L2 / L3. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.6 |  |
|  |  | Subset-026-5.10.1.7 |  |
|  |  | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 5 Linking Information is recorded. The Linking information is correct according to site data |
| Constraints | A cab needs to be active (SB Mode) An order to switch to L2/3 at a further location has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | To L2/L3 |
| ERTMS/ETCS level | 0 /1/ 2 | LNTC /L0 /L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC /L0 /L1 | FS / OS / SR / UN / SB / LS / SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received | I | RTM | Complete message is accepted | LNTC /L0 /L1 | FS / OS / SR / UN / SB / LS / SN | - |
| 2 | LNTC /L0 /L1 | FS / OS / SR / UN / SB / LS / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded | LNTC /L0 /L1 | FS / OS / SR / UN / SB / LS / SN | - |
| 3 | LNTC /L0 /L1 | FS / OS / SR / UN / SB / LS / SN | The Level symbol "Level 2" is displayed | O | DMI | Only if the Next Level is L2, when the train reaches the previously announced location. | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | - |
| 4 | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | Only if the Next Level is L2 | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | - |
| 5 | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | The Level symbol "Level 3" is displayed | O | DMI | Only if the Next Level is L3, when the train reaches the previously announced location. | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | - |
| 6 | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit05=1>) is recorded | O | JRU | Only if the Next Level is L3 | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | - |
| 7 | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | USE_FT4060300 for changing to FS, LS or OS mode | - | - | Only if the current mode is different from FS, OS and LS. | L2 / L3 | FS / OS / SR / UN / SB / LS / SN | - |
| 8 | L2 / L3 | FS / OS / LS | Use_FT3160200.68 | - | - | The train uses the linking information: the system reacts according to the linking information when the next linked BG is not found. | L2 / L3 | FS / OS / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3/ 4 | L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | A telegram including packet 5 Linking Information is transmitted via balise group and is received by the onboard equipment. No linking data were received previously. The linking information is accepted because a L1 LEVEL transition has been recieved. |  | 8 |
| Applicable Mode/Level Combinations | L2,L3: FS, OS, SR, SB, LS L0: UN, SB LNTC: SN |  |  |
| Target of Test | The target of this test is to check that The Balise telegram, including packet 5, is accepted and stored on the JRU on-board because there is a L1 level transition stored onboard The Linking is correct and shall be used by the on-board equipment after changing to L1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.6 |  |
|  |  | Subset-026-5.10.1.7 |  |
|  |  | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Balise telegram including packet 5 Linking Information is recorded. The The Linking information is correct according to site data |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 | FS / OS / SR / UN / SB / LS |
| Linking | NOT STORED | - |
| Level Transition Order | STORED | To L1 |
| ERTMS/ETCS level | 0 /1 /3 /4 | L0 / LNTC /L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC /L2 /L3 | FS / OS / SR / UN / SB / LS | One Balise Group message containing packet 5 is received | I | BTM | Complete message is accepted | L0 / LNTC /L2 /L3 | FS / OS / SR / UN / SB / LS | - |
| 2 | L0 / LNTC /L2 /L3 | FS / OS / SR / UN / SB / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Complete message is recorded | L0 / LNTC /L2 /L3 | FS / OS / SR / UN / SB / LS | - |
| 3 | L0 / LNTC /L2 /L3 | FS / OS / SR / UN / SB / LS | The Level symbol "Level 1" is displayed | O | DMI | The train reaches the previously announced location. | L1 | FS / OS / SR / UN / SB / LS | - |
| 4 | L1 | FS / OS / SR / UN / SB / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | FS / OS / SR / UN / SB / LS | - |
| 5 | L1 | FS / OS / SR / UN / SB / LS | USE_FT4060300 for changing to FS, LS or OS mode | - | - | Only if the current mode is different from FS, OS and LS. | L1 | FS / OS / LS | - |
| 6 | L1 | FS / OS / LS | Use_FT3160200.68 | - | - | The train uses the linking information: the system reacts according to the linking information when the next linked BG is not found. | L1 | FS / OS / LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | The onboard equipment receives telegram including packet 5 Linking Information via radio. The message is REJECTED. The first balise in the linking information should have TRIP reaction |  | 9 |
| Applicable Mode/Level Combinations | L2: TR L3: TR |  |  |
| Target of Test | The target of this test is to check that The message including packet 5 is rejected No linking information is stored onboard |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 5 Linking Information is recorded. No transition to TRIP mode is applied when the train arrives at the place of the first balise group of the linking information with reaction TRIP Check on the DMI that: NO linking information is stored. No transition to TRIP mode is DISPLAYED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | TR | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received | I | RTM | - | L2 / L3 | TR | - |
| 2 | L2 / L3 | TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 3 | L2 / L3 | TR | Use_5110200.13 for changing to PT mode (Transition [7]) | - | - | - | L2 / L3 | TR | - |
| 4 | L2 / L3 | PT | USE_FT4060300 for changing to FS, LS or OS mode | - | - | - | L2 / L3 | PT | - |
| 5 | L2 / L3 | FS / OS / LS | Train is moving | I | INT | - | L2 / L3 | FS / OS / LS | - |
| 6 | L2 / L3 | FS / OS / LS | Not the Mode symbol "Trip" is displayed | O | DMI | Transition to TR mode is not performed when it is reached the location where linked BG is expected to be found Either no BG shall be placed in this location or, if BG is placed, the NID_BG shall not match with the information included in P5 | L2 / L3 | FS / OS / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Linking |  | 4080420 |
| Test Case of Feature | A telegram including packet 5 Linking Information is transmitted via radio and is received by the onboard equipment The linking information is REJECTED . |  | 10 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: FS, OS, SR, SB, LS |  |  |
| Target of Test | The target of this test is to check that The radio message including packet 5 is rejected. The first balise in the linking information should have TRIP reaction. Since packet 5 is REJECTED, no trip reaction is applied after changing to L2/3 first and passing over the location of the BG announced in the packet 5. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.6 |  |
|  |  | Subset-026-4.8.3.1.1 Linking |  |
|  |  | Subset-026-4.8.4.2 Linking |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Balise telegram including packet 5 Linking Information is recorded. A level 2/3 transition is recorded. No transition to TRIP mode is apply when the train arrives at the place of the first balise group of the linking information with reaction TRIP Check on the DMI that: The L2/L3 is displayed No transition to TRIP mode is DISPLAYED |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS / OS / SR / SB / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received | I | RTM | - | L0 / LNTC / L1 | FS / OS / SR / SB / LS | - |
| 2 | L0 / LNTC / L1 | FS / OS / SR / SB / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / SB / LS | - |
| 3 | L0 / LNTC / L1 | FS / OS / SR / SB / LS | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=<3¦4>) is received | I | BTM | M_LEVELTR will be either 3 or 4 (L2 or L3) | L0 / LNTC / L1 | FS / OS / SR / SB / LS | - |
| 4 | L0 / LNTC / L1 | FS / OS / SR / SB / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / SB / LS | - |
| 5 | L0 / LNTC / L1 | FS / OS / SR / SB / LS | The Level symbol "Level 2" is displayed | O | DMI | Only if the Next Level is L2. | L2 / L3 | FS / OS / LS | - |
| 6 | L2 / L3 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | Only if the Next Level is L2 | L2 / L3 | FS / OS / LS | - |
| 7 | L0 / L1 | FS / OS / SR / SB / LS | The Level symbol "Level 3" is displayed | O | DMI | Only if the Next Level is L3. | L2 / L3 | FS / OS / LS | - |
| 8 | L2 / L3 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit05=1>) is recorded | O | JRU | Only if the Next Level is L3 | L2 / L3 | FS / OS / LS | - |
| 9 | L2 / L3 | FS / OS / LS | USE_FT4060300 for changing to FS, LS or OS mode | - | - | Only if the current mode is different from FS, OS and LS. | L2 / L3 | FS / OS / LS | - |
| 10 | L2 / L3 | FS / OS / LS | Train is moving | I | INT | - | L2 / L3 | FS / OS / LS | - |
| 11 | L2 / L3 | FS / OS / LS | Not the Mode symbol "Trip" is displayed | O | DMI | Transition to TR mode is not performed when it is reached the location where linked BG is expected to be found Either no BG shall be placed in this location or, if BG is placed, the NID_BG shall not match with the information included in P5 | L2 / L3 | FS / OS / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 3 /4 | L2 /L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |