| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080406 (4.8.4: Accepted Information depending on the modes - Trackside constituent System Version) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-4080406 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 27.11.2013 | All | Review | CEDEX |
| Version 2.9.3 09.01.2014 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Trackside constituent System Version |  | 4080406 |
| Test Case of Feature | The on-board equipment shall not consider the reception of unknown messages as a message data consistency error. The unknown message is ignored in case of unknown radio infill message from a RIU operating with a system version only differing by Y with regards to the highest system version number X supported by on-board. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, SR, OS L2: FS, LS, SR, OS L3: FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment, operating (as the highest system version regarding number X) system version X.Z, ignores unknown radio messages from a RIU operating a system version X.Y (where X is the same than on-board and Y is different from Z). There are no observable reactions on none of the interfaces in relation to the data that was ignored by the on-board equipment. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-3.17.3.11 |  |
|  |  | Subset-026-3.17.3.11 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The radio infill message was recorded (since there is no inconsistency problem with the data message). The on-board equipment did NOT command the emergency brake since the message was only ignored. Check on the TIU that : Emergency brake is NOT applied. Check on the DMI that : There is NO indication showing an ETCS version incompatibility between the RIU and the on-board equipment. Check on each relevant interface that there are no reaction related to the new information received from the RIU, i.e. the on-board equipment ignored information that is not known by the older on-board versions. |
| Constraints | The Radio Infill information only differs by Y with regards to the highest system version number X supported by the on-board. New information (e.g. a new pre-defined value for a variable or a new definition of the validity range of a variable) unknown by on-board ETCS versions is available in the radio infill message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | The radio session is established with a Radio Infill Unit. |
| Level Transition Order | STORED | Only in L2/L3. The order to connect to the Radio infill unit is only accepted if L1 transition is stored on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| RBC/RIU System Version | STORED | Version X.Y |
| Operated System Version | STORED | Version X.Z (where Z<> Y) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | Both | RTM | The connection is set up with a RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Unknown Euroradio Message is received | I | RTM | The onboard equipment receives a radio infill message where the NID_MESSAGE is different to all the possible values for NID_MESSAGE for on-board operated system version X.Z. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | Radio message is recorded since there is no data inconsistency. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The system status message "Trackside not compatible" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | The radio session is established with a Radio Infill Unit. |
| Level Transition Order | STORED | Only in L2/L3. The order to connect to the Radio infill unit is only accepted if L1 transition is stored on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| RBC/RIU System Version | STORED | Version X.Y |
| Operated System Version | STORED | Version X.Z (where Z <> Y) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | Both | RTM | The connection is set up with a RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Trackside constituent System Version |  | 4080406 |
| Test Case of Feature | The on-board equipment shall not consider the reception of unknown packets as a message data consistency error. The unknown packet is ignored if being attached to a radio infill message from a RIU operating with a system version only differing by Y with regards to the highest system version number X supported by on-board. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, SR, OS L2: FS, LS, SR, OS L3: FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment, operating (as the highest system version regarding number X) system version X.Z, ignores unknown radio packets from a RIU operating a system version X.Y (where X is the same than on-board and Y is different from Z). There are no observable reactions on none of the interfaces in relation to the data that was ignored by the on-board equipment. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-3.17.3.11 |  |
|  |  | Subset-026-3.17.3.11 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The radio infill message was recorded (since there is no inconsistency problem with the data message). The on-board equipment did NOT command the emergency brake since the packet was only ignored. Check on the TIU that : Emergency brake is NOT applied. Check on the DMI that : There is NO indication showing an ETCS version incompatibility between the RIU and the on-board equipment. Check on each relevant interface that there are no reaction related to the new information received from the RIU, i.e. the on-board equipment ignored information that is not known by the older on-board versions. |
| Constraints | The Radio Infill information only differs by Y with regards to the highest system version number X supported by the on-board. New information (e.g. a new pre-defined value for a variable or a new definition of the validity range of a variable) unknown by on-board ETCS versions is available in the radio infill message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | The radio session is established with a Radio Infill Unit. |
| Level Transition Order | STORED | Only in L2/L3. The order to connect to the Radio infill unit is only accepted if L1 transition is stored on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| RBC/RIU System Version | STORED | Version X.Y |
| Operated System Version | STORED | Version X.Z (where Z <> Y) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | Both | RTM | The connection is set up with a RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing Unknown packet is received | I | RTM | The onboard equipment receives a radio infill message 24, containing a NID_PACKET that is different to all the possible values for NID_PACKET for on-board operated system version X.Z. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | Radio message is recorded since there is no data inconsistency. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The system status message "Trackside not compatible" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | FINITE VALUE | NID_PACKET that is different to all the possible values for NID_PACKET for on-board operated system version X.Z. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | The radio session is established with a Radio Infill Unit. |
| Level Transition Order | STORED | Only in L2/L3. The order to connect to the Radio infill unit is only accepted if L1 transition is stored on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| RBC/RIU System Version | STORED | Version X.Y |
| Operated System Version | STORED | Version X.Z (where Z <> Y) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | Both | RTM | The connection is set up with a RIU |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Trackside constituent System Version |  | 4080406 |
| Test Case of Feature | The on-board equipment shall not consider the reception of unknown packets from a RBC as a message data consistency error. The unknown packet is ignored if being attached to a radio message from a RBC operating with a system version only differing by Y with regards to the highest system version number X supported by on-board. |  | 3 |
| Applicable Mode/Level Combinations | L0: SH / PS / UN / SL / NL LNTC: SH / PS / SN / SL / NL L1: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / NL / RV L2: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / NL / RV L3: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment, operating (as the highest) system version X.Z, ignores unknown radio messages from a RBC operating a system version X.Y (where X is the same than on-board and Y is different from Z). There are no observable reactions on none of the interfaces in relation to the data that was ignored by the on-board equipment. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-3.17.3.11 |  |
|  |  | Subset-026-3.17.3.11 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The radio message was recorded (since there is no inconsistency problem with the data message). The on-board equipment did NOT command the emergency brake since the packet was only ignored. Check on the TIU that : Emergency brake is NOT applied. Check on the DMI that : There is NO indication showing an ETCS version incompatibility between the RBC and the on-board equipment. Check on each relevant interface that there are no reaction related to the new information received from the RBC, i.e. the on-board equipment ignored information that is not known by the older on-board versions. |
| Constraints | The RBC information only differs by Y with regards to the highest system version number X supported by the on-board. New information (e.g. a new pre-defined value for a variable or a new definition of the validity range of a variable) unknown by on-board ETCS version is available in the radio message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| RBC/RIU System Version | STORED | Version X.Y |
| Operated System Version | STORED | Version X.Z (where Z <> Y) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONECTION SET UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing Unknown packet is received | I | RTM | The onboard equipment receives a radio message 24, containing a NID_PACKET that is different to all the possible values for NID_PACKET for on-board operated system version X.Z. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Radio message is recorded since there is no data inconsistency. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The system status message "Trackside not compatible" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | FINITE VALUE | NID_PACKET that is different to all the possible values for NID_PACKET for on-board operated system version X.Z. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| RBC/RIU System Version | STORED | Version X.Y |
| Operated System Version | STORED | Version X.Z (where Z <> Y) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONECTION SET UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Trackside constituent System Version |  | 4080406 |
| Test Case of Feature | The on-board equipment shall not consider the reception of unknown messages from a RBC as a message data consistency error. The unknown message is ignored if being sent by a RBC operating with a system version only differing by Y with regards to the highest system version number X supported by on-board. |  | 4 |
| Applicable Mode/Level Combinations | L0: SH / PS / UN / SL / NL LNTC: SH / PS / SN / SL / NL L1: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / NL / RV L2: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / NL / RV L3: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment, operating (as the highest) system version X.Z, ignores unknown radio packets from a RBC operating a system version X.Y (where X is the same than on-board and Y is different from Z). There are no observable reactions on none of the interfaces in relation to the data that was ignored by the on-board equipment. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-3.17.3.11 |  |
|  |  | Subset-026-3.17.3.11 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The radio message was recorded (since there is no inconsistency problem with the data message). The on-board equipment did NOT command the emergency brake since the message was only ignored. Check on the TIU that : Emergency brake is NOT applied. Check on the DMI that : There is NO indication showing an ETCS version incompatibility between the RBC and the on-board equipment. Check on each relevant interface that there are no reaction related to the new information received from the RBC, i.e. the on-board equipment ignored information that is not known by the older on-board versions. |
| Constraints | The RBC information only differs by Y with regards to the highest system version number X supported by the on-board. New information (e.g. a new pre-defined value for a variable or a new definition of the validity range of a variable) unknown by on-board ETCS versions is available in the radio message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| RBC/RIU System Version | STORED | Version X.Y |
| Operated System Version | STORED | Version X.Z (where Z <> Y) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONECTION SET UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Unknown Euroradio Message is received | I | RTM | The onboard equipment receives a radio message where the NID_MESSAGE is different to all the possible values for NID_MESSAGE for on-board operated system version X.Z. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Radio message is recorded since there is no data inconsistency. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The system status message "Trackside not compatible" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| RBC/RIU System Version | STORED | Version X.Y |
| Operated System Version | STORED | Version X.Z (where Z <> Y) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONECTION SET UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |