| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080454 (4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas) Total: 13 Test case(s) |  |
| REF: | Subset-076-5-2-4080454 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.1 06.06.2016 | All | First version/review | CEDEX |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | The on-board equipment receives from the RBC a radio message 24 (general message) containing a packet 68 (track conditions). The message is accepted and stored on-board. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB L3: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of this test is to check that : The radio message 24 with the packet 68 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: The radio message 24 with the packet 68 is recorded. |
| Constraints | A cab is active (in SB). Valid Train Data are stored on-board (in SB) Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has been already received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | The management of the track condition allows to check that the track condition information is accepted. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | The on-board equipment receives from the RBC a radio message 24 (general message) containing a packet 68 (track conditions). As “Recognition of Exit from TR mode” has already been received, the message is accepted and stored on-board. |  | 2 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test is to check that : The radio message 24 with the packet 68 is accepted and stored on-board when a “Recognition of Exit from TR mode” has already been received. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.12.1.6 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: The radio message 24 with the packet 68 is recorded. |
| Constraints | A cab is active. A “Recognition of Exit from TR mode” has been received with a more recent time stamp. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | The management of the track condition allows to check that the track condition information is accepted. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | MAINTAINED | - |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | The train is not in a level transition area to L2 / L3. The on-board equipment receives from the RBC a radio message 24 (general message) containing a packet 68 (track conditions). The message is recorded but it is rejected. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check that : The radio message 24 with the packets 68 is rejected when no announcement of transition to L2 / L3 has been received. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.12.1.6 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: no indication about new track conditions information is displayed Check on JRU that: The radio message 24 with the packet 68. |
| Constraints | A cab is active (in SB). Valid Train Data are stored on-board (in SB) No announcement of transition to L2 / L3 has been received. In PT A “Recognition of Exit from TR mode” has been received with a more recent time stamp. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | To L2/3 |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / SB / PT / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 2 | L1 | FS / OS / SR / SB / PT / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | The track condition information are rejected as no announcement of transition to L2 / L3 has been received. | L1 | FS / OS / SR / SB / PT / LS | - |
| 3 | L1 | FS / OS / SR / SB / PT / LS | USE_FT5100100.37 if in L1 | - | - | This is for changing to L2 | L1 | FS / OS / SR / SB / PT / LS | - |
| 4 | L2 | FS / OS / SR / SB / PT / LS | USE_5040300.19 | - | - | Only if in SB mode. This is for changing to FS | L2 | FS / OS / SR / SB / PT / LS | - |
| 5 | L2 | FS / OS / SR / SB / PT / LS | USE_4060300.21 | - | - | Only if in SR mode. This is for changing to FS | L2 | FS / OS / SR / SB / PT / LS | - |
| 6 | L2 | FS / OS / SR / SB / PT / LS | NOT The Track Condition symbol "Non stopping area" is displayed | O | DMI | If track condition is "Non stopping area" | L2 | FS / OS / SR / SB / PT / LS | - |
| 7 | L2 | FS / OS / SR / SB / PT / LS | NOT The Track Condition symbol "Sound horn" is displayed | O | DMI | If track condition is "Sound horn" | L2 | FS / OS / SR / SB / PT / LS | - |
| 8 | L2 | FS / OS / SR / SB / PT / LS | NOT The Track Condition symbol "Tunnel stopping area" is displayed | O | DMI | If track condition is "Tunnel stopping area” | L2 | FS / OS / SR / SB / PT / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 8 / 12 | FS / OS / PT / LS |
| Radio communication session | MAINTAINED | - |
| Track conditions excluding big metal masses | UNKNOWN | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | The on-board equipment receives from the RBC a radio message 24 (general message) containing a packet 68 (track conditions. As the train data has been sent to the supervising RBC and the acknowledgement of train data has not been received yet from it, the message is recorded but it is rejected. |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, LS, SR, OS, SB, PT L3: FS, LS, SR, OS, SB, PT |  |  |
| Target of Test | The target of this test is to check that : The radio message 24 with the packet 68 is rejected when no acknowledgment of train data has been received from supervising RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.12.1.6 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: no indication about new track conditions information is displayed Check on JRU that: The radio message 24 with the packet 68 is recorded. |
| Constraints | A cab is active. In PT a “Recognition of Exit from TR mode” has been received with a more recent time stamp. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / SR / OS / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / SR / SB / PT / LS | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | Validated train data | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 2 | L2 / L3 | FS / OS / SR / SB / PT / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 3 | L2 / L3 | FS / OS / SR / SB / PT / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 4 | L2 / L3 | FS / OS / SR / SB / PT / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | The track condition information are rejected as no acknowledgement of train data has been received. | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 5 | L2 / L3 | FS / OS / SR / SB / PT / LS | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 6 | L2 / L3 | FS / OS / SR / SB / PT / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 7 | L2 / L3 | FS / OS / SR / SB / PT / LS | USE_5040300.19 | - | - | Only if in SB mode. This is for changing to FS | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 8 | L2 / L3 | FS / OS / SR / SB / PT / LS | USE_4060300.21 | - | - | Only if in SR mode. This is for changing to FS | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 9 | L2 / L3 | FS / OS / PT / LS | NOT The Track Condition symbol "Non stopping area" is displayed | O | DMI | If track condition is "Non stopping area" | L2 / L3 | FS / OS / PT / LS | - |
| 10 | L2 / L3 | FS / OS / PT / LS | NOT The Track Condition symbol "Sound horn" is displayed | O | DMI | If track condition is "Sound horn" | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 11 | L2 / L3 | FS / OS / PT / LS | NOT The Track Condition symbol "Tunnel stopping area" is displayed | O | DMI | If track condition is "Tunnel stopping area” | L2 / L3 | FS / OS / SR / SB / PT / LS | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 5: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 2 / 8 / 12 | FS / OS / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Track conditions excluding big metal masses | UNKNOWN | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | The on-board equipment receives from the RBC a radio message 24 (general message) containing a packet 68 (track conditions). The message is REJECTED. |  | 5 |
| Applicable Mode/Level Combinations | L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that : The radio message 24 with the packet 68 is rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: NO indication about new track conditions information is stored or displayed Check on JRU that: The radio message 24 with the packets 68 is recorded. |
| Constraints | A cab is active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | ESTABLISHED | - |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SH | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | The packet is REJECTED. | L2 / L3 | SH | - |
| 2 | L2 / L3 | SH | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 3 | L2 / L3 | SH | USE_FT4060300.32 | - | - | This is for changing to TR mode. | L2 / L3 | SH | - |
| 4 | L2 / L3 | TR | NOT The Track Condition symbol "Non stopping area" is displayed | O | DMI | If track condition is "Non stopping area" | L2 / L3 | TR | - |
| 5 | L2 / L3 | TR | NOT The Track Condition symbol "Sound horn" is displayed | O | DMI | If track condition is "Sound horn" | L2 / L3 | TR | - |
| 6 | L2 / L3 | TR | NOT The Track Condition symbol "Tunnel stopping area" is displayed | O | DMI | If track condition is "Tunnel stopping area” | L2 / L3 | TR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | MAINTAINED | - |
| Track conditions excluding big metal masses | UNKNOWN | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | Acceptance of message 24 (General message) including packets 68 (Track Condition) |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that the message 24 including packet 68 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.9.1.3 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message 24 (General message) including packets 68 (Track Condition) is recorded. |
| Constraints | A cab is active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | L2/3 announcement |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | The management of the track condition allows to check that the track condition information is accepted. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track conditions |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | - |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | Valid for values due to sound horn, non stopping areas or tunnel stopping areas: 0000, 0001, 0010. |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | Valid for values due to sound horn, non stopping areas or tunnel stopping areas: 0000, 0001, 0010. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | MAINTAINED | - |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | Acceptance of message including packets 68 (Track Condition) that is transmitted via Balise Group. Level 1 |  | 7 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that a message including packets 68 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message including packets 68 (Track Condition) is recorded. |
| Constraints | A cab is active. “Recognition of Exit from TR mode” with a more recent time stamp if in PT mode |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / SB / PT / LS | One Balise Group message containing packet 68 is received | I | BTM | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 2 | L1 | FS / OS / SR / SB / PT / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=68) is recorded | O | JRU | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 3 | L1 | FS / OS / SR / SB / PT / LS | USE_5040300.16 and USE_4060300.49 | - | - | Only if in SB mode. This is for changing to SR | L1 | FS / OS / SR / SB / PT / LS | - |
| 4 | L1 | FS / OS / SR / PT / LS | USE_4060300.44 | - | - | Only if in SR mode. This is for changing to FS | L1 | FS / OS / SR / PT / LS | - |
| 5 | L1 | FS / OS / PT / LS | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | The management of the track condition allows to check that the track condition information is accepted. | L1 | FS / OS / PT / LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 8 / 12 | FS / OS / PT / LS |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | Rejection of message including packets 68 (Track Condition) that is transmitted via Balise Group. Level transition announcement to L1 has not been received. |  | 8 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SN L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that a message including packets 68 is recorded but rejected in case of no announcement of level transition to level 1 has been received by the onboard equipment. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message including packets 68 (Track Condition) is recorded. Check on the DMI that: No new information is displayed on DMI. |
| Constraints | A cab is active. “Recognition of Exit from TR mode” with a more recent time stamp if in PT mode No announcement of transition to L1 has been received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 12 / 13 | FS / OS / SR / UN / SB / PT / LS / SN |
| Level Transition Order | NOT STORED | To L1 |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | One Balise Group message containing packet 68 is received | I | BTM | - | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | - |
| 2 | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=68) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | - |
| 3 | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | USE_FT5100100.15 if in L0 or USE_FT5100100.23 if in LNTC or USE_FT5100100.32 if in L2 or USE_FT5100100.28 if in L3 | - | - | This is for changing to L1 | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | - |
| 4 | L1 | FS / OS / SR / SB / PT / LS | USE_5040300.16 and USE_4060300.49 | - | - | Only if in SB mode. This is for changing to SR | L1 | FS / OS / SR / SB / PT / LS | - |
| 5 | L1 | FS / OS / SR / PT / LS | USE_4060300.44 | - | - | Only if in SR mode. This is for changing to FS | L1 | FS / OS / SR / PT / LS | - |
| 6 | L1 | FS / OS / PT / LS | NOT The Track Condition symbol "Non stopping area" is displayed | O | DMI | If track condition is "Non stopping area" | L1 | FS / OS / PT / LS | - |
| 7 | L1 | FS / OS / PT / LS | NOT The Track Condition symbol "Sound horn" is displayed | O | DMI | If track condition is "Sound horn" | L1 | FS / OS / PT / LS | - |
| 8 | L1 | FS / OS / PT / LS | NOT The Track Condition symbol "Tunnel stopping area" is displayed | O | DMI | If track condition is "Tunnel stopping area” | L1 | FS / OS / PT / LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 8 / 12 | FS / OS / PT / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | Acceptance of message including packet 68 (Track Condition) that is transmitted via infill Balise Group. |  | 9 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that a message including packet 68 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.9.1.3 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message including packet 68 (Track Condition) is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Including the main balise group |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 136, packet 68 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=136, 68) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | After the train passes over the location of the main BG without reading it and without reacting (due to the actual linking information). The management of the track condition allows to check that the track condition information is accepted. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | Acceptance of Euroloop message including packet 68 (Track Condition) that is transmitted via Euroloop. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that a Euroloop message including packet 68 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.9.1.3 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message 24 (General message) including packets 68 (Track Condition) is recorded. Note: features FT5180400.1, FT5180900.1-2 and FT5180800.1-5 allow to check that track condition information is accepted. Check on the JRU that: Message 24 (General message) including packets 68 (Track Condition) is recorded. Note: features FT5180400.1, FT5180900.1-2 and FT5180800.1-5 allow to check that track condition information is accepted. |
| Constraints | The on-board has read a balise telegram including packet 134 (Announcement of a loop), transmitted via balise group. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | The next main BG is included in the actual linking information with "No Reaction" in case it is lost. |
| EOLM information | STORED | - |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message containing packet 136, packet 68 is received | I | LTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7; NID_PACKET=136, 68) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | After the train passes over the location of the main BG without reading it and without reacting (due to the actual linking information). The management of the track condition allows to check that the track condition information is accepted. | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | Acceptance of message including packet 68 (Track Condition) that is transmitted via Balise Group. Level transition announcement to L1 has previously been received. |  | 11 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SN L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that a message including packets 68 is recorded and accepted in case an announcement of level transition to level 1 has been received by the onboard equipment. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message including packet 68 (Track Condition) is recorded. Check on the DMI that: New information is displayed on DMI. |
| Constraints | A cab is active. “Recognition of Exit from TR mode” with a more recent time stamp if in PT mode An announcement of transition to L1 has previously been received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 12 / 13 | FS / OS / SR / UN / SB / PT / LS / SN |
| Level Transition Order | STORED | L1 announcement |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | One Balise Group message containing packet 68 is received | I | BTM | - | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | - |
| 2 | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=68) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | - |
| 3 | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | The Level symbol "Level 1" is displayed | O | DMI | When the train reaces the stored location or when an immediately level transition order is received. | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | - |
| 4 | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / OS / SR / SB / PT / LS | - |
| 5 | L1 | FS / OS / SR / SB / PT / LS | USE_5040300.16 and USE_4060300.49 | - | - | Only if in SB mode. This is for changing to SR | L1 | FS / OS / SR / SB / PT / LS | - |
| 6 | L1 | FS / OS / SR / PT / LS | USE_4060300.44 | - | - | Only if in SR mode. This is for changing to FS | L1 | FS / OS / SR / PT / LS | - |
| 7 | L1 | FS / OS / PT / LS | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | The management of the track condition allows to check that the track condition information is accepted. | L1 | FS / OS / PT / LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 8 / 12 | FS / OS / PT / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | The on-board equipment receives from the RBC a radio message 24 (general message) containing a packet 68. The message is accepted and stored on-board. |  | 12 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB L3: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of this test is to check that : The radio message 24 with the packets 68 is accepted and stored on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: The radio message 24 with the packets 68 is recorded. |
| Constraints | A cab is active (in SB). Valid Train Data are stored on-board (in SB) Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has been already received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | The management of the track condition allows to check that the track condition information is accepted. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Track Conditions sound horn, non stopping areas, tunnel stopping areas |  | 4080454 |
| Test Case of Feature | The on-board equipment receives from the RBC a radio message 24 (general message) containing a packet 68 (track conditions) and it accepts it because there is a level transition order to L2/3 stored on-board. |  | 13 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that : The radio message 24 with the packet 68 is accepted when an announcement of transition to L2 / L3 has been received. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Track Conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: An indication about new track conditions information is displayed after changing to L2/3 Check on JRU that: The radio message 24 with the packet 68 is recorded. |
| Constraints | A cab is active (in SB). Valid Train Data are stored on-board (in SB) An announcement of transition to L2 / L3 has been received. In PT A “Recognition of Exit from TR mode” has been received with a more recent time stamp. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 12 / 13 | FS / OS / SR / UN / SB / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | To L2/3 |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / PT / LS / SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / PT / LS / SN | - |
| 2 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / PT / LS / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / PT / LS / SN | - |
| 3 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / PT / LS / SN | USE_FT5100100.14 if in L0 or USE_FT5100100.21 if in LNTC or USE_FT5100100.37 if in L1 | - | - | This is for changing to L2 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / PT / LS / SN | - |
| 4 | L2 / L3 | FS / OS / SR / SB / PT / LS | USE_5040300.19 | - | - | Only if in SB mode. This is for changing to FS | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 5 | L2 / L3 | FS / OS / SR / SB / PT / LS | USE_4060300.21 | - | - | Only if in SR mode. This is for changing to FS | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 6 | L2 / L3 | FS / OS / PT / LS | Use_FT5180400.1 (non stopping areas) or Use_FT5180900.1,2 (sound horn) or Use_FT5180800.1-5 (tunnel stopping areas) | - | - | The management of the track condition allows to check that the track condition information is accepted. | L2 / L3 | FS / OS / PT / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 8 / 12 | FS / OS / PT / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |