| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080452 (4.8.4: Accepted Information depending on the modes - Initiation of session) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-4080452 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | CEDEX |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 29.11.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.3 12.12.2013 | All | Editorial changes | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Initiation of session |  | 4080452 |
| Test Case of Feature | Transmission Initiation of Session from RBC in a Message 38 (Initiation of a communication session). The information is rejected because the on-board equipment is not in the appropriate mode. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2: SH, PS L3: SH, PS |  |  |
| Target of Test | The target of this test is to check that: When the on-board receives the message “Initiation of a communication session“, it is rejected |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 Initiation of session |  |
|  |  | Subset-026-4.8.3.1.1 Initiation of Session |  |
|  |  | Subset-026-8.7.16 Message 38: Initiation of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The message 38 is recorded No message 159 is recorded Check on the RTM that: The message 38 is received on-board The message 159 is not sent by the on-board equipment |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 13 / 15 | SH / UN / SN / PS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Indication is received | I | RTM | Parameter: Called ETCS-ID, Calling ETCS-ID, application type The on-board equipment has to compare the called ETCS-ID with ist own ETCS-ID. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Response is transmitted | O | RTM | A safe connection is now set-up | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=38) | I | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=38) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10_NID_MESSAGE=159) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 38 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 38 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | always set to unknown |
| M_ACK | 1 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | always set to unknown |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 13 / 15 | SH / UN / SN / PS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Initiation of session |  | 4080452 |
| Test Case of Feature | Transmission Initiation of Session from RBC in a Message 38 (Initiation of a communication session). The information is rejected because the on-board equipment is not in the appropriate level. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / SL / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test is to check that: When the on-board receives the message “Initiation of a communication session“, it is rejected |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 Initiation of session |  |
|  |  | Subset-026-4.8.3.1.1 Initiation of Session |  |
|  |  | Subset-026-8.7.16 Message 38: Initiation of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The message 38 is recorded No message 159 is recorded Check on the RTM that: The message 38 is received on-board The message 159 is not sent by the on-board equipment |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SL / SB / TR / PT / LS / NL / RV |
| Radio communication session | NOT STABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Indication is received | I | RTM | The on-board equipment has to compare the called ETCS-ID with ist own ETCS-ID. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Response is transmitted | O | RTM | A safe connection is now set-up | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=38) | I | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=38) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10_NID_MESSAGE=159) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 38 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 38 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | always set to unknown |
| M_ACK | 1 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | always set to unknown |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SL / SB / TR / PT / LS / NL / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |