| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080437 (4.8.4: Accepted Information depending on the modes - Session Management) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-4080437 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 02.09.2013 | All | editoral changes in Test of Squence table | CEDEX |
| Version 2.9.2 11.11.2013 | All | review | CEDEX |
| Version 2.9.3 11.12.2013 | All | Editorial changes | CEDEX |
| Version 2.9.4 20.12.2013 | All | Editorial changes | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Session Management |  | 4080437 |
| Test Case of Feature | Acceptance of Session Management information. The on-board equipment receives an order to establish the radio communication session from a balise group. The previous stored RBC ID/phone number is different from the new received one. The onboard does not establish the communication session, but it stores the new RBC ID/phone number. |  | 1 |
| Applicable Mode/Level Combinations | L0 : SH, PS LNTC :SH, PS L1:SH, PS L2: SH, PS L3: SH, PS |  |  |
| Target of Test | The target of this test is to check that: The new RBC ID/ phone number is recorded in the JRU. The onboard equipment does not set up the safe connection and radio message 155 is not sent to the RBC. This Test case tests condition [3] of Subset-026-4.8.4.2 |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 a) |  |
|  |  | Subset-026-3.5.5.2 |  |
|  |  | Subset-026-3.5.5.2 a) |  |
|  |  | Subset-026-3.5.5.2 c) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-3.5.5.6 |  |
|  |  | Subset-026-4.8.3.1.1 Session Management |  |
|  |  | Subset-026-4.8.4.2 Session Management |  |
|  |  | Subset-026-4.8.4.2 [3] |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM that: The balise message with packet 42 (Session Management) is received by the onboard. The radio message 155 is not sent by the onboard equipment to the RBC Check on the JRU that: The balise message with packet 42 received by the on-board is recorded The new received RBC ID/phone number included in packet 42 is recorded . |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Radio Network ID | STORED | Only for level L2 / L3 |
| RBC ID/Phone Number | STORED | Only for level L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | SH / PS | One Balise Group message containing packet 42 (Q_RBC=1) is received | I | BTM | - | L0 / LNTC / L1 | SH / PS | - |
| 2 | L0 / LNTC / L1 | SH / PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=42) is recorded | O | JRU | - | L0 / LNTC / L1 | SH / PS | - |
| 3 | L0 / LNTC / L1 | SH / PS | Not SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L0 / LNTC / L1 | SH / PS | - |
| 4 | L0 / LNTC / L1 | SH / PS | Cab active | I | TIU | Only in PS mode | L0 / LNTC / L1 | SH / PS | - |
| 5 | L0 / LNTC / L1 | SH / PS | The Mode symbol "Shunting" is displayed | O | DMI | Only in PS mode MO01 is the Shunting symbol | L0 / LNTC / L1 | SH | - |
| 6 | L0 / LNTC / L1 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | Only in PS mode Bit16=1 means the Shunting symbol (MO01) is displayed | L0 / LNTC / L1 | SH | - |
| 7 | L0 / LNTC / L1 | SH | USE_FT4060300.18 to change to SB mode | - | - | - | L0 / LNTC / L1 | SB | - |
| 8 | L0 / LNTC / L1 | SB | USE_FT5040300.8 to validate the Driver Id and USE_FT5040300.8 to go to S10 and USE_FT5040500.TC3 to change to L2 / L3 and USE_FT5040300.39 to see and revalidate the RBC ID and Phone Number and USE_FT5040300.22 to establish a radio session | - | - | - | L2 / L3 | SB | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Session Management |  | 4080437 |
| Test Case of Feature | Rejection of Session Management information. The on-board equipment receives an order to establish the radio communication session from a balise group. The onboard does not establish the communication session, since it is in level 1 an PT mode. |  | 2 |
| Applicable Mode/Level Combinations | L1:PT |  |  |
| Target of Test | The target of this test is to check that: The onboard equipment does not set up the safe connection and radio message 155 is not sent to the RBC. This Test case tests condition [1] of Subset-026-4.8.4.2 |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 a) |  |
|  |  | Subset-026-3.5.5.2 |  |
|  |  | Subset-026-3.5.5.2 a) |  |
|  |  | Subset-026-3.5.5.2 c) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-3.5.5.6 |  |
|  |  | Subset-026-4.8.3.1.1 Session Management |  |
|  |  | Subset-026-4.8.4.2 Session Management |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM that: The balise message with packet 42 (Session Management) is received by the onboard. The radio message 155 is not sent by the onboard equipment to the RBC Check on the JRU that: The balise message with packet 42 received by the on-board is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 42 (Q_RBC=1) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=42) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Not SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) containing packet X is transmitted | O | RTM | This step tests that Session Management information is rejected | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |