| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3060600 (3.6.6: Geographical position reporting) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-3060600 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 27.11.2013 | TC1, TC2, TC3, TC4 | Application of the accepted comments of Review003_Subset-076-5-2-3060600_v291.doc | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.6: Geographical position reporting |  | 3060600 |
| Test Case of Feature | No more indication of geographical position if ordered by trackside. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN / SB / NL L1 / L2 / L3: FS / LS / OS / SR / SB / TR / PT / NL |  |  |
| Target of Test | The target of this test case is to check that: the indication of geographical position is deleted if ordered by trackside. The function Determine Geographical position is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.6.1 |  |
|  |  | Subset-026-3.6.6.2 |  |
|  |  | Subset-026-3.6.6.5 |  |
|  |  | Subset-026-3.6.6.9 |  |
|  |  | Subset-026-3.6.6.9 b) |  |
|  |  | Subset-026-4.5.2.1 table - Determine Geographical Position |  |
|  |  | Subset-026-4.7.2 table - Output information Geographical position |  |
|  |  | Subset-026-7.4.2.25 Packet Number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the indication of geographical position is displayed as long as an order to not report the geographical position is not received from trackside. The indication of geographical position is deleted if an order to not report the geographical position is received from trackside. |
| Constraints | The distance travelled from the geographical reference location shall be taken into account when calculating the geographical position. The train speed is slow enough to allows the JRU to record two positions distant of 1 meter. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / SB / TR / PT / NL / LS |
| Geographical Position | STORED | M_POSITION : STORED - <> 16777215 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Receiving of geographical position information (packet 79) | - | - | Use_FT4080415.3 : Transmission of geographical position information via balise group (M_POSITION <> ‘16777215’) | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=00011110b) is recorded | O | JRU | The driver request of The Geographical Position DISPLAY is recorded in the JRU | N/A | N/A | - |
| 4 | N/A | N/A | The "Geographical Position" is displayed | O | DMI | Geographical position is displayed according to the M_POSITION | N/A | N/A | - |
| 5 | N/A | N/A | Any message (NID_MESSAGE_JRU=ALL; TRAIN_POSITION=d1) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The "Geographical Position" is displayed | O | DMI | Geographical position is displayed according to the M_POSITION + 1m | N/A | N/A | - |
| 7 | N/A | N/A | Any message (NID_MESSAGE_JRU=ALL; TRAIN_POSITION=d2) is recorded | O | JRU | d2=d1+1m | N/A | N/A | - |
| 8 | N/A | N/A | Receiving of geographical position information (packet 79) | - | - | Use_FT4080415.3 : Transmission of geographical position information via balise group The variable M_POSITION in the packet must have the value ‘16777215’ | N/A | N/A | - |
| 9 | N/A | N/A | The "Geographical Position" is removed | O | DMI | Geographical position is not displayed any longer | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| Geographical Position | STORED | M_POSITION : STORED - Value of track kilometre reference must be set to special value "16777215" (No more reporting after this reference location) D_POSOFF : STORED - Value of kilometre offset must be set to 0 m in order to make sure that the reference location is passed. |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Geographical position is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.6: Geographical position reporting |  | 3060600 |
| Test Case of Feature | indication of geographical position is changed after new given information by trackside. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN L1 / L2 / L3: FS / LS / OS / SR / SB / TR / PT / NL |  |  |
| Target of Test | The target of this test case is to check that: the indication of geographical position is displayed and after receiving of new information the indication is changed. The function Determine Geographical position is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.6.1 |  |
|  |  | Subset-026-3.6.6.3 |  |
|  |  | Subset-026-3.6.6.5 |  |
|  |  | Subset-026-3.6.6.9 a) |  |
|  |  | Subset-026-3.6.6.9.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Geographical position |  |
|  |  | Subset-026-7.4.2.25 Packet number 79: Geographical Position Information |  |
|  |  | Subset-026-4.5.2.1 table - Determine Geographical Position |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the indication of geographical position is displayed . If a new order is received, the new information will be used. |
| Constraints | First geographical information over BG, second (different from first) geographical information over radio |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | only Level 2/3 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Receiving of geographical position information (packet 79) | - | - | Use_FT4080415.3 : Transmission of geographical position information via balise group (M_POSITION <> ‘16777215’) | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=00011110b) is recorded | O | JRU | The driver request of The Geographical Position DISPLAY is recorded in the JRU | N/A | N/A | - |
| 4 | N/A | N/A | The "Geographical Position" is displayed | O | DMI | Geographical position is displayed according to the M_POSITION | N/A | N/A | - |
| 5 | N/A | N/A | Receiving of new geographical position information (packet 79) | - | - | Level2/3: Use_FT4080415.1 : Transmission of geographical position information from RBC (M_POSITION <> ‘16777215’) Level 1: Use_FT4080415.3 : Transmission of geographical position information via balise group (M_POSITION <> ‘16777215’) Only the value of the variable M_POSITION in the new packet 79 is different from the first M_POSITION transmitted over BG | N/A | N/A | - |
| 6 | N/A | N/A | The "Geographical Position" is displayed | O | DMI | Changed according to the new M_POSITION value | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | only Level 2/3 |
| Geographical Position | STORED | M_POSITION : STORED - The value from packet 79 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only Level 2 / 3 |
| NOT RELEVANT | - | TIU | - |
| Geographical position is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.6: Geographical position reporting |  | 3060600 |
| Test Case of Feature | Indication of geographical position, the onboard shall use the data related to the most recently reported balise group |  | 3 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / TR / PT / NL L3 FS / LS / OS / SR / TR / PT / NL |  |  |
| Target of Test | The target of this test case is to check that: the geographical position is indicated to the driver. the on-board equipment uses the data related to the most recently reported balise group The function Determine Geographical position is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.6.1 |  |
|  |  | Subset-026-3.6.6.4.1 |  |
|  |  | Subset-026-3.6.6.4.2 |  |
|  |  | Subset-026-3.6.6.5 |  |
|  |  | Subset-026-4.7.2 table - Output information Geographical position |  |
|  |  | Subset-026-7.4.2.25 Packet number 79: Geographical Position Information |  |
|  |  | Subset-026-4.5.2.1 table - Determine Geographical Position |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the geographical position is displayed, the indication is based on latest received Geographical Position Information. The onboard equipment shall use the data related to the most recently reported balise group |
| Constraints | The distance travelled from the geographical reference location shall be taken into account when calculating the geographical position. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 8 / 11 / 12 | FS / OS / SR / TR / PT / NL / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train receives any message from a balise group (A) which should be ACCEPTED. | I | BTM | This BG (A) is part of the last reported balise groups memorised onboard. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The train receives any message from a balise group (B) which should be ACCEPTED. | I | BTM | This BG (B) is part of the last reported balise groups memorised onboard. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The train receives any message from a balise group (C) which should be ACCEPTED. | I | BTM | This BG (C) is part of the last reported balise groups memorised onboard. | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The train receives a geographical position information which should be ACCEPTED. (BG A and B are included as a reference) | - | - | Use_FT4080415.1 : Transmission of geographical position information from RBC Notice that more the BG IDs from BG A and BG B needs to be included in the geographical position information. | N/A | N/A | - |
| 8 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 9 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=00011110b) is recorded | O | JRU | The driver request of The Geographical Position DISPLAY is recorded in the JRU | N/A | N/A | - |
| 10 | N/A | N/A | Geographical position is DISPLAYED according to the data related to the most recently reported balise group B. | O | DMI | M_POSITION value of the BG (B) according to received geographical position information. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media : Balise (0) |
| N_PIG | 3 | 0 | First balise in group |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG (A) |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media : Balise (0) |
| N_PIG | 3 | 1 | second balise in group |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG (A) |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media : Balise (0) |
| N_PIG | 3 | 0 | First balise in group |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG (B) |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media : Balise (0) |
| N_PIG | 3 | 1 | second balise in group |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG (B) |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media : Balise (0) |
| N_PIG | 3 | 0 | First balise in group |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG (C) |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media : Balise (0) |
| N_PIG | 3 | 1 | second balise in group |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG (C) |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 8 / 11 / 12 | FS / OS / SR / TR / PT / NL / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Geographical position is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.6: Geographical position reporting |  | 3060600 |
| Test Case of Feature | Indication of geographical position is displayed after information given by trackside containing one balise group not yet passed at the time of reception of the information. The on-board equipment displays the geographical information after passing the BG, and possibly a given offset. |  | 4 |
| Applicable Mode/Level Combinations | L0 : UN, SB, NL L1 : FS, LS, OS, SR, SB, TR, PT,NL L2 : FS, LS, OS, SR, SB, TR, PT,NL L3 : FS, LS, OS, SR, SB, TR, PT,NL |  |  |
| Target of Test | The target of this test case is to check that: the indication of geographical position corresponds to data related to a balise group not yet passed at the time of reception of the information. The function Determine Geographical position is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.6.1 |  |
|  |  | Subset-026-3.6.6.5 |  |
|  |  | Subset-026-4.5.2.1 table - Determine Geographical Position |  |
|  |  | Subset-026-4.7.2 table - Output information Geographical position |  |
|  |  | Subset-026-7.4.2.25 Packet number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that The indication of geographical position, containing one balise group not yet passed at the time of reception of the information, is displayed when the train actually passes over that BG (and possibly a given offset). |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 1 / 2/ 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train passes over the BG that is the geographical position reference balise group previously given in Packet 79. | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | Possibly, after the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=00011110b) is recorded | O | JRU | The driver request of The Geographical Position DISPLAY is recorded in the JRU | N/A | N/A | - |
| 5 | N/A | N/A | Geographical position is DISPLAYED according to the M_POSITION related to the geographical position reference BG in Packet 79. | O | DMI | The Bg is the one just passed. | N/A | N/A | - |
| 6 | N/A | N/A | The driver requests to hide the Geographical Position | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=00011111b) is recorded | O | JRU | The driver request of The Geographical Position DISPLAY is recorded in the JRU | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Except 254 |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | N of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Except 254 |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| ERTMS/ETCS level | 1 / 2/ 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.6: Geographical position reporting |  | 3060600 |
| Test Case of Feature | Indication of geographical position based on latest received Geographical Position Information. The calculated geographical position becomes negative |  | 5 |
| Applicable Mode/Level Combinations | L0 : UN, SB, NL L1 : FS, LS, OS, SR, SB, TR, PT,NL L2 : FS, LS, OS, SR, SB, TR, PT,NL L3 : FS, LS, OS, SR, SB, TR, PT,NL |  |  |
| Target of Test | The target of this test case is to check that: the geographical position is indicated to the driver. the on-board equipment uses the latest received Geographical Position Information for indication. The on-board equipment stops calculating the position when the calculated geographical position becomes negative. The function Determine Geographical position is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.6.1 |  |
|  |  | Subset-026-3.6.6.5 |  |
|  |  | Subset-026-3.6.6.9 |  |
|  |  | Subset-026-3.6.6.9 c) |  |
|  |  | Subset-026-4.5.2.1 table - Determine Geographical Position |  |
|  |  | Subset-026-4.7.2 table - Output information Geographical position |  |
|  |  | Subset-026-7.4.2.25 Packet number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the geographical position is displayed, the indication is based on latest received Geographical Position Information. the indication of geographical position is displayed until the calculated geographical position becomes negative. the indication of geographical position is deleted if the calculated geographical position becomes negative.. |
| Constraints | The distance travelled from the geographical reference location shall be taken into account when calculating the geographical position. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| Geographical Position | STORED | M_POSITION is based on already received packet 79. Q_MPOSITION : based on already received packet 79. Track kilometre will have to decrease when the geographical position starts to be calculated. |
| ERTMS/ETCS level | 1 / 2/ 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=00011110b) is recorded | O | JRU | The driver request of The Geographical Position DISPLAY is recorded in the JRU | N/A | N/A | - |
| 3 | N/A | N/A | Geographical position is DISPLAYED according to the value of M_POSITION. The track kilometre value decreases when the train moves. | O | DMI | Route kilometre (variable M_POSITION) is based on just received packet 79. | N/A | N/A | - |
| 4 | N/A | N/A | Geographical position is NOT DISPLAYED any longer. | O | DMI | The train moves the same distance as the value given by M_POSITION. So the train reaches the track kilometre 0. Then, the calculated geographical position becomes negative. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| ERTMS/ETCS level | 1 / 2/ 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Geographical position is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.6: Geographical position reporting |  | 3060600 |
| Test Case of Feature | Indication of geographical position based on the latest received Geographical Position Information. The on-board equipment shall continue calculating the position from a track kilometre reference until no more geographical position information is available |  | 6 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, OS, SR L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test case is to check that: the geographical position is indicated to the driver. the on-board equipment uses the latest received Geographical Position Information for indication. The on-board equipment stops calculating the position when there is a mode change to SN. The function Determine Geographical position is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.6.1 |  |
|  |  | Subset-026-3.6.6.5 |  |
|  |  | Subset-026-3.6.6.9 |  |
|  |  | Subset-026-3.6.6.9 d) |  |
|  |  | Subset-026-4.5.2.1 table - Determine Geographical Position |  |
|  |  | Subset-026-4.7.2 table - Output information Geographical position |  |
|  |  | Subset-026-7.4.2.25 Packet number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the geographical position is displayed, the indication is based on latest received Geographical Position Information. the indication of geographical position is displayed until the calculated geographical position becomes negative. the geographical position stops to be calculated when there is a mode change to SN |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / 12 |
| Geographical Position | STORED | M_POSTION is based on already received packet 79. |
| ERTMS/ETCS level | 1 / 2/ 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=00011110b) is recorded | O | JRU | The driver request of The Geographical Position DISPLAY is recorded in the JRU | N/A | N/A | - |
| 3 | N/A | N/A | Geographical position is DISPLAYED according to the value of M_POSITION | O | DMI | Route kilometre (variable M_POSITION) is based on just received packet 79. | N/A | N/A | - |
| 4 | N/A | N/A | The on-board device receives an order to switch to Level NTC. | - | - | Use_FT5100100.16 “Order to switch immediately to Level NTC at level Border (Level Transition L0/LNTC)“ OR Use_FT5100100.34 “Order to switch immediately to Level NTC at level Border (Level Transition L1/LNTC)“ OR Use_FT5100100.30 “Order to switch immediately to Level NTC at level Border (Level Transition L2/LNTC)“ OR Use_FT5100100.25 “Order to switch immediately to Level NTC at level Border (Level Transition L3/LNTC)“ | N/A | N/A | - |
| 5 | N/A | N/A | The on-board switches to SN mode. | - | - | Use_ FT4060300.6 | N/A | N/A | - |
| 6 | N/A | N/A | Geographical position is NOT DISPLAYED any longer. | O | DMI | The on-board stops calculating the geographical position. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Geographical Position | NOT STORED | DELETED |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Geographical position is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.6: Geographical position reporting |  | 3060600 |
| Test Case of Feature | Geographical position information has been given for a single balise group and no linking information is available. The on-board equipment shall ignore the geographical position information related to these single balise groups. |  | 7 |
| Applicable Mode/Level Combinations | L0: UN, SB, NL L1, L2, L3: FS, LS, OS, SR, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that : In cases where single balise groups are used as a reference for geographical position information and where no linking information is available, the on-board equipment shall ignore the geographical position information related to these single balise groups. The function Determine Geographical position is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.6.1 |  |
|  |  | Subset-026-3.6.6.5 |  |
|  |  | Subset-026-3.6.6.7 |  |
|  |  | Subset-026-4.5.2.1 table - Determine Geographical Position |  |
|  |  | Subset-026-4.7.2 table - Output information Geographical position |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The geographical position information related to the single balise group is not displayed. |
| Constraints | Geographical position information has been given for a single balise group and no linking information is available. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| Linking | UNKNOWN | - |
| Geographical Position | STORED | M_POSITION : Track kilometre of the single balise group. D_POSOFF : 0 - No offset |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver is not allowed to request the indication of geographical position | I | DMI | After the train front end has passed over the location specified by the D_POSOFF value | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / UN / SB / TR / PT / NL / LS |
| Linking | UNKNOWN | - |
| Geographical Position | STORED | D_POSOFF : 0 - No offset M_POSITION : Track kilometre of the single balise group. |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Geographical position is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |