| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080438 (4.8.4: Accepted Information depending on the modes - SR Authorisation) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-4080438 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 31.03.2013 | All | editorial changes | CEDEX |
| Version 2.9.2 11.11.2013 | ALL | REVIEW | CEDEX |
| Version 2.9.3 11.12.2013 | ALL | Editorial changes | CEDEX |
| Version 2.9.4 23.12.2013 | TC3 and TC4 | Editorial changes | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - SR Authorisation |  | 4080438 |
| Test Case of Feature | Transmission and acceptance of an SR Authorisation and a list of balises for SR authorisation from RBC (independently of an empty or not empty list) |  | 1 |
| Applicable Mode/Level Combinations | L2 : SR, SB, PT L3 : SR, SB, PT |  |  |
| Target of Test | The target of this test is to check: Message 2 with packet 63 containing a list of balises or an empty list has been RECEIVED from RBC and RECORDED by the on-board system. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 SR Authorisation + (optional) List of Balises in SR mode |  |
|  |  | Subset-026-4.8.4.2 SR Authorisation+ (optional) list of Balises in SR mode |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.16 Packet Number 63: List of Balises in SR Authority |  |
|  |  | Subset-026-8.7.1 Message 2: SR Authorisation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing a list of balises (or empty list) has been RECEIVED and RECORDED. Check that: The list of balises in SR mode is accepted |
| Constraints | In PT mode the information “Recognition of Exit of TR mode” has been received previously A cab needs to be active and valid Train Data are stored on-board (if in SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 2 / 8 | SR / SB / PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2) containing packet 63 is received | I | RTM | Received message is accepted. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=2;NID_PACKET=63) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | USE_FT5040300.17 | - | - | The train changes to SR mode if it is not in this mode already (Transition [8]) | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | The driver requests to see the Target distance in SR mode | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Driver action ‘Request to show supervision limits’ is recorded | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target distance is displayed after driver request, and it matches the value of D_SR. | N/A | N/A | - |
| 7 | N/A | N/A | USE_FT4041100.3,4 | - | - | (Supervision of list of balises during staff responsible movement) | N/A | N/A | - |

| Step 1: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 63 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | SR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - SR Authorisation |  | 4080438 |
| Test Case of Feature | Transmission of list of balises for SR authorisation from RBC (independent of an empty or not empty list) The information is rejected because the train data has been sent to the RBC and no acknowledgement of train data has been received yet |  | 2 |
| Applicable Mode/Level Combinations | L2 : SR, SB, PT L3 : SR, SB, PT |  |  |
| Target of Test | The target of this test is to check: Message 2 with packet 63 containing a list of balises or an empty list has been RECEIVED from RBC and RECORDED by the on-board system. Message is REJECTED because the acknowledgement of train data has not been received yet |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.3.1.1 SR Authorisation + (optional) List of Balises in SR mode |  |
|  |  | Subset-026-4.8.4.2 SR Authorisation+ (optional) list of Balises in SR mode |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-8.7.1 Message 2: SR Authorisation |  |
|  |  | Subset-026-7.4.2.16 Packet Number 63: List of Balises in SR Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing a list of balises (or empty list) has been RECEIVED and RECORDED. |
| Constraints | In PT mode the information “Recognition of Exit of TR mode” has been received previously A cab needs to be active and valid Train Data are stored on-board (if in SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 6 / 8 | SR / SB / PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SR / SB / PT | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | Validated train data | L2 / L3 | SR / SB / PT | - |
| 2 | L2 / L3 | SR / SB / PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | SR / SB / PT | - |
| 3 | L2 / L3 | SR / SB / PT | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2) containing packet 63 is received | I | RTM | The message is rejected | L2 / L3 | SR / SB / PT | - |
| 4 | L2 / L3 | SR / SB / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=2;NID_PACKET=63) is recorded | O | JRU | - | L2 / L3 | SR / SB / PT | - |
| 5 | L2 / L3 | SR / SB / PT | The Mode symbol "Mode <SB / PT>" is displayed | O | DMI | This step will be checked only if the starting mode was different from SR mode. This way the rejection of Message 2 is tested. | L2 / L3 | SR / SB / PT | - |
| 6 | L2 / L3 | SR / SB / PT | The driver requests the speed and distance monitoring information to be displayed | I | DMI | This step will be checked only if the starting mode was SR mode | L2 / L3 | SR / SB / PT | - |
| 7 | L2 / L3 | SR / SB / PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | This step will be checked only if the starting mode was SR mode | L2 / L3 | SR / SB / PT | - |
| 8 | L2 / L3 | SR / SB / PT | The speed and distance monitoring information is displayed | O | DMI | The Target distance is displayed after the driver request, and it matches the supervising value used before testing this Test Case. The value of D_SR received in the first step is not supervised.This step will be checked only if the starting mode was SR mode. | L2 / L3 | SR / SB / PT | - |
| 9 | L2 / L3 | SR / SB / PT | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | SR / SB / PT | - |
| 10 | L2 / L3 | SR / SB / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | SR / SB / PT | - |
| 11 | L2 / L3 | SR / SB / PT | USE_FT4060300.56 | - | - | (Transition [37]) if the current mode is different from SR mode, | L2 / L3 | SR / SB / PT | - |
| 12 | L2 / L3 | SR | One Balise Group message is received | I | BTM | The train reads a BG that was not in the ‘List of Balises in SR Authority’ received before. If the list was sent empty, any BG can be received. | L2 / L3 | SR | - |
| 13 | L2 / L3 | SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 14 | L2 / L3 | SR | The Mode symbol "Staff Responsible" is displayed | O | DMI | The train does not changed to TR mode. This way the rejection of Packet 63 is tested. | L2 / L3 | SR | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 3: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 63 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | - |

| Step 9: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| Step 12: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - SR Authorisation |  | 4080438 |
| Test Case of Feature | The onboard receives a message 2 (SR authorisation ) with packet 63 (List of Balises in SR Authority) from RBC. The packet is REJECTED because the on-board operating mode is not the correct one for accepting this information (independently of an empty or not empty list) |  | 3 |
| Applicable Mode/Level Combinations | L2 : FS, LS, OS, TR L3 : FS, LS, OS, TR |  |  |
| Target of Test | The target of this test is to check: Message 2 (SR authorisation) with packet 63 (List of Balises in SR Authority) has been RECEIVED from RBC and RECORDED by the on-board system. The SR authorisation message and List of Balises in SR Authority packet is stored onboard. After changing to SR mode, the rejection of the List of Balises in SR is checked |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 SR Authorisation+ (optional) list of Balises in SR mode |  |
|  |  | Subset-026-7.4.2.16 Packet Number 63: List of Balises in SR Authority |  |
|  |  | Subset-026-8.7.1 Message 2: SR Authorisation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : A message sent from RBC containing a list of balises (or empty list) has been RECEIVED and RECORDED. Check that: After changing to SR mode, the train does not supervise the List of Balises in SR Authority (independently of an empty or not empty list). |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 12 | FS / OS / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / LS / TR | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2) containing packet 63 is received | I | RTM | The message is REJECTED. | L2 / L3 | FS / OS / LS / TR | - |
| 2 | L2 / L3 | FS / OS / LS / TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=2;NID_PACKET=63) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS / TR | - |
| 3 | L2 / L3 | FS / OS / LS / TR | USE_FT5110200.13 | - | - | Only if in TR mode | L2 / L3 | FS / OS / LS / PT | - |
| 4 | L2 / L3 | FS / OS / LS / PT | Use_ FT4060300.56 | - | - | The on-board changes to SR mode | L2 / L3 | SR | - |
| 5 | L2 / L3 | SR | USE_FT4041100.5 | - | - | This is to test the List of balises in SR mode was rejected | L2 / L3 | SR | - |

| Step 1: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 63 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - SR Authorisation |  | 4080438 |
| Test Case of Feature | The onboard receives a message 2 (SR authorisation ) with packet 63 (List of Balises in SR Authority) from RBC when it is not in L2/L3. The telegram is REJECTED (independently of an empty or not empty list). |  | 4 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN L1 : SR, SB |  |  |
| Target of Test | The target of this test is to check: Message 2 (SR authorisation) with packet 63 (List of Balises in SR Authority) has been RECEIVED from RBC and RECORDED by the on-board system. The SR authorisation message and List of Balises in SR Authority packet is stored onboard. After changing to SR mode and L2/L3, the rejection of the List of Balises in SR is checked |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 SR Authorisation+ (optional) list of Balises in SR mode |  |
|  |  | Subset-026-7.4.2.16 Packet Number 63: List of Balises in SR Authority |  |
|  |  | Subset-026-8.7.1 Message 2: SR Authorisation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing a list of balises (or empty list) has been RECEIVED and RECORDED. Check that: After changing to SR mode and L2/L3, the train does not supervise the List of Balises in SR Authority (independently of an empty or not empty list). |
| Constraints | A cab needs to be active and valid Train Data are stored on-board (if in SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 6 / 13 | SR / UN / SB / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2) containing packet 63 is received | I | RTM | The message is REJECTED. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=2;NID_PACKET=63) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT5100100 or Use_FT5100300 | - | - | The train changes to L2/L3 (for the rest of level transitions) | N/A | N/A | - |
| 4 | N/A | N/A | The Mode symbol "Mode <SB / PT>" is displayed | O | DMI | If the starting mode was different from SR mode at the beginning of the Test Case. This step will be checked only if the starting mode was different from SR mode. This way the rejection of Message 2 is tested. | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | The driver requests to see the Target distance in SR mode. This step will be checked only if the starting mode was SR mode | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | This step will be checked only if the starting mode was SR mode | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target distance is displayed after the driver request, and it matches the supervising value used before testing this Test Case. The value of D_SR received in the first step is not supervised.This step will be checked only if the starting mode was SR mode. | N/A | N/A | - |
| 8 | N/A | N/A | Use_FT4060300.56 or Use_FT5080300 | - | - | Only if the testing mode is different from SR mode. (Transition [37]) or (Transition [45]) | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message is received | I | BTM | The train reads a BG that was not in the ‘List of Balises in SR Authority’ received before. If the list was sent empty, any BG can be received. | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The Mode symbol "Staff Responsible" is displayed | O | DMI | The train does not changed to TR mode. This way the rejection of Packet 63 is tested. | N/A | N/A | - |

| Step 1: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 63 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |