| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080434 (4.8.4: Accepted Information depending on the modes - Reversing Supervision Information) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-4080434 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 31.03.2013 | All | editorail changes7 change in Starting cond on Interfaces | CEDEX |
| Version 2.9.2 29.11.2013 | All | REview | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Supervision Information |  | 4080434 |
| Test Case of Feature | The onboard equipment receives reversing supervision information from RBC. The telegram is ACCEPTED, because a level 2/3 transition order is stored onboard. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN, SB L1: FS, LS, OS, SR, SB LNTC: SB, SN |  |  |
| Target of Test | The target of test is to check that: Reversing supervision information has been sent from the RBC to the on-board system, and it is accepted. The message shall only be accepted if a Level 2/3 announcement is stored onboard. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that A message sent from the RBC containing reversing information has been RECORDED. The on-board equipment changes to L2/3 and then it changes to RV mode. Then the acceptance of the reversing supervision area is actually checked. |
| Constraints | In SB mode, a cab needs to be active and valid Train Data are stored on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/4/6/12/13 | FS /OS /SR /UN /SB/ LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | L2 MA (packet 15). This is necessary if the starting mode is different from FS / OS / LS |
| Gradient Profile | STORED | This is necessary if the starting mode is different from FS / OS / LS |
| International SSP | STORED | This is necessary if the starting mode is different from FS / OS / LS |
| Level Transition Order | STORED | Level 2/3 announcement message |
| ERTMS/ETCS level | 0/ 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | UN / SB / SN / FS / LS / OS / SR / SB | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 138 (Q_DIR=1) and packet 139 (Q_DIR=1) is received | I | RTM | The message is accepted. | L0 / LNTC / L1 | UN / SB / SN / FS / LS / OS / SR / SB | - |
| 2 | L0 / LNTC / L1 | UN / SB / SN / FS / LS / OS / SR / SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / LNTC / L1 | UN / SB / SN / FS / LS / OS / SR / SB | - |
| 3 | L0 / LNTC / L1 | UN / SB / SN / FS / LS / OS / SR / SB | USE_FT5100100 | - | - | Use the correspondig TC to change to L2 or L3 | L2 / L3 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | The train is at standstill. | - | - | The train has to be in the reversing area defined in packet 138. | L2 / L3 | FS / OS / LS | - |
| 5 | L2 / L3 | FS / OS / LS | USE_FT4060300.22 | - | - | to change to RV mode | L2 / L3 | RV | - |
| 6 | L2 / L3 | RV | The speed and distance monitoring information is displayed | O | DMI | The displayed Target distance matches the value given at the beginning of this TC. | L2 / L3 | RV | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| NID_PACKET | 8 | 138 | Packet Number 138 : Reversing Area Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | Reversing supervision information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | value not equal to the previous stored value but greater than the previous |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Reversing Area Information | STORED | - |
| Reversing Supervision Information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Supervision Information |  | 4080434 |
| Test Case of Feature | On-board receives a BG message or a radio message from the RBC including Packet 139 ( Reversing supervision information) via balise group.The Reversing Supervision Information is ACCEPTED. |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, SR, SB, RV L2 : FS, LS, OS, SR, SB, PT, RV L3 : FS, LS, OS, SR, SB, PT, RV |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment receives Packet 139 ( Reversing supervision information) via balise group or radio. The on-board equipment changes to RV mode, only if the current mode is different from RV mode, and the target distance for reversing is displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM, if the level is L1, that : The On board system receives a BG message including Packet 139 ( Reversing supervision information) Check on the RTM, if the level is L2/L3, that : The On board system receives a general message including Packet 139 ( Reversing supervision information) Check on the JRU that : The BG or radio message including Packet 139 ( Reversing supervision information) is RECORDED Then the on-board equipment changes to RV mode, only if the current mode is different from RV mode. Then the acceptance of the reversing supervision area is actually checked: Check on the DMI that : The new Target Distance defined by the value of D_REVERSE received in Packet 139 is displayed |
| Constraints | The on-board equipment must have Packets 139 and 138 stored (or used, if the testing mode is RV mode) on-board before testing this TC. In SB mode, a cab needs to be active and valid Train Data are stored on-board In PT mode, the on-board equipment must have received the information “Recognition of Exit from TR mode” with a more recent time stamp |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 / 14 | FS /OS /SR /SB/ PT/ LS / RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Supervision Information | STORED | D_REVERSE: This value is refered to in the steps of the TC as D_REVERSE_ini |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the onboard is in RV mode. D_REVERSE_ini is the value of D_REVERSE, which is being supervised at the beginning of this TC, when this starts in RV mode | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN < permitted) | I | INT | If the current level is L1, choose the following two steps. If the current level is L2/L3, choose the steps 5 and 6. D_REVERSE_ini is the value of D_REVERSE, which is stored on-board at the beginning of this TC, when this does not start in RV mode | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 139 (Q_DIR=1), packet 138 is received | I | BTM | The value of D_REVERSE in packet 139 has to be different from the value D_REVERSE_ini. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 139 (Q_DIR=1), packet 138 is received | I | RTM | The value of D_REVERSE in packet 139 has to be different from the value D_REVERSE_ini. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving (V_TRAIN < permitted) | I | INT | The on-board equipment changes to either FS, LS or OS mode (only if the current mode is different from RV mode) | N/A | N/A | - |
| 8 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | The train has to be in the reversing area defined in the previously stored packet 138. | N/A | N/A | - |
| 9 | N/A | N/A | USE_FT4060300.22 | - | - | to change to RV mode (only if the current mode is different from RV mode) | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target Distance is displayed and it is different to the value D_REVERSE_ini. The target distance matches the value of D_REVERSE that was received in the step 3 or 5. | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | if L2 / L3 |
| RBC ID/Phone Number | STORED | if L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Supervision Information |  | 4080434 |
| Test Case of Feature | On-board receives a BG message including Packets 138 (Reversing area information) and 139 ( Reversing supervision information. The message is ACCEPTED, because an announcement of level transition to Level 1 has been received. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN / SB LNTC : SB / SN L2: FS / LS / OS / SR / SB L3: FS / LS / OS / SR / SB |  |  |
| Target of Test | The target of this test case is to check that : On-board equipment receives a BG message including Packets 138 (Reversing area information) and 139 ( Reversing supervision information). The on-board equipment changes to L1 and then to RV mode, and it supervises the distance for reversing. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM that : The On board system receives a general telegram including Packets 138 (Reversing area information) and 139 ( Reversing supervision information) Check on the JRU that : The general telegram including Packets 138 (Reversing area information) and 139 ( Reversing supervision information) is RECORDED The on-board equipment changes to L1 and then it changes to RV mode. Then the acceptance of both the reversing supervision area and the reversing area information are actually checked. |
| Constraints | In SB mode, a cab needs to be active and valid Train Data are stored on-board Level transition order is stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 12 / 13 | FS/ OS/ SR/ UN/ SB/ PT/ LS/ SN |
| Movement Authority | STORED | L1 MA (packet 12). This is necessary if the starting mode is different from FS / OS / LS |
| Gradient Profile | STORED | This is necessary if the starting mode is different from FS / OS / LS |
| International SSP | STORED | This is necessary if the starting mode is different from FS / OS / LS |
| Level Transition Order | STORED | Level Transition to Level 1 |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | UN / SB / SN / FS / LS / OS / SR / SB | One Balise Group message containing packet 138 (Q_DIR=1) and packet 139 (Q_DIR=1) , is received | I | BTM | The message is ACCEPTED. | L0 / LNTC / L2 / L3 | UN / SB / SN / FS / LS / OS / SR / SB | - |
| 2 | L0 / LNTC / L2 / L3 | UN / SB / SN / FS / LS / OS / SR / SB | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SB / SN / FS / LS / OS / SR / SB | - |
| 3 | L0 / LNTC / L2 / L3 | UN / SB / SN / FS / LS / OS / SR / SB | USE_FT5100100 | - | - | Use the correspondig TC to change to L1 | L1 | FS / OS / LS | - |
| 4 | L1 | FS / OS / LS | The train is at standstill. | - | - | The train has to be in the reversing area defined in packet 138. | L1 | FS / OS / LS | - |
| 5 | L1 | FS / OS / LS | USE_FT4060300.22 | - | - | to change to RV mode | L1 | RV | - |
| 6 | L1 | RV | The speed and distance monitoring information is displayed | O | DMI | The displayed Target distance matches the value given at the beginning of this TC. | L1 | RV | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Reversing Supervision Information |  | 4080434 |
| Test Case of Feature | Transmission of reversing information via in-fill Balise Group and reception by the on-board equipment. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: the reversing information transmitted via in-fill Balise Group is accepted by the on board the on-board equipment changes to RV mode and it supervises the distance for reversing |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The message received from the Balise Group containing the reversing information is RECORDED Then the on-board equipment changes to RV mode. Then the acceptance of the the reversing supervision area is actually checked |
| Constraints | The on-board system shall be equipped with the in-fill transmission media. Linking information including the Main balise group is recorded. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Including the main balise group |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 138 (Q_DIR=1)and packet 139 (Q_DIR=1), is received | I | BTM | The telegram is an announcement of the reversing area, and it indicates: its start and end and permitted distance to run and maximum speed, after switching to RV mode. Packet 136 included in BG telegram defines location reference of data contained in packets 138 and 139. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | The train has to be in the reversing area defined in packet 138. | N/A | N/A | - |
| 4 | N/A | N/A | USE_FT4060300.22 | - | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Linking | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |