| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3120100 (3.12.1: Track Conditions) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-3120100 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | CEDEX |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 02.08.2013 | All | Editorial changes/review 1 changes | CEDEX |
| Version 2.9.2 18.11.2013 | All | Requirements correction | CEDEX |
| Version 2.9.3 11.05.2013 | 2 | duplicated packet 255 deleted | CEDEX |
| Version 2.9.4 19.12.2013 | All | Review | CEDEX |
| Version 2.9.5 08.01.2014 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.1: Track Conditions |  | 3120100 |
| Test Case of Feature | The on-board equipment receives from the RBC a radio message 24 (general message) containing a packet 68 (track conditions) to resume track conditions to initial state (initial state shall be used by the on-board equipment) The on-board delete the previous stored track condition and continue run. The message is accepted and stored on-board The on-board equipment NOT displays anymore a profile indicating the location of previous received track conditions |  | 1 |
| Applicable Mode/Level Combinations | L2 : FS, LS, OS, NL, PT, TR L3 : FS, LS, OS, NL, PT, TR |  |  |
| Target of Test | The target of this test is to check that : The radio message 24 with the packet 68 is accepted and stored on-board. The on-board equipment NOT displays ANYMORE the previous received track condition information The on-board equipment changes to initial the state of track conditions |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-3.6.4.6 |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |
|  |  | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.12.1.6 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The state of track conditions profile is changed to initial state according to the received track condition information. Check on TIU that: No track conditions is changed according to the previous received profile and the EVC (or Driver) Action |
| Constraints | Previous Track condition information indicating a location where it have to be applied has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 7/ 8/ 11/ 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Condition symbol is displayed. | O | DMI | Profile indicating where the previous track condition should be applied is DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Track Condition symbol is displayed. | O | DMI | When the max safe front end of the train reaches a location in rear of the beginning of the track condition announced. | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (with No track condition) is received. | I | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded. | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT The Track Condition symbol announcement is displayed. | O | DMI | The train reaches the start location of the previous track condition section with its max safe front end ( = D_TRACKCOND ) | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Track Condition symbol is displayed. | O | DMI | - | N/A | N/A | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track conditions |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | NO MATTER | - |
| Q_TRACKINIT | 1 | 1 | Empty profile, initial state to be resumed |
| D_TRACKINIT | 15 | 0 | 0 m |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | The same as the previous track condition |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 7/ 8/ 11/ 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 3/ 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT The Track Condition symbol is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.1: Track Conditions |  | 3120100 |
| Test Case of Feature | The on-board equipment receives from the balises a telegram containing a packet 68 (track conditions) to resume the track condition to initial state (the initial state shall be used by the on-board equipment) The on-board delete all track condition and continue run. The message is accepted and stored on-board The on-board equipment NOT displays anymore a profile indicating the location of previous received track conditions |  | 2 |
| Applicable Mode/Level Combinations | L1 :FS, LS, OS, NL, PT, TR |  |  |
| Target of Test | The target of this test is to check that : The balises telegram witt the packet 68 is accepted and stored on-board. The on-board equipment NOT displays ANYMORE the previous received track condition information The on-board equipment changes to initial the state of track conditions |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-3.6.4.6 |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.6 |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Power control |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |
|  |  | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-4.8.3.1.1 Track Conditions excluding big metal masses |  |
|  |  | Subset-026-4.8.4.2 Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The state of track conditions profile is changed to initial state according to the received track condition information. Check on TIU that: No track conditions is changed according to the previous received profile and the EVC (or Driver) Action |
| Constraints | Previous Track condition information indicating a location where it have to be applied has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 7/ 8/ 11/ 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Condition symbol is displayed. | O | DMI | Profile indicating where the previous track condition should be applied is DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Track Condition symbol is displayed. | O | DMI | When the max safe front end of the train reaches a location in rear of the beginning of the track condition announced. | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 68 (with No track condition) is received. | I | BTM | - | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT The Track Condition symbol announcement is displayed. | O | DMI | The train reaches the start location of the previous track condition section with its max safe front end ( = D_TRACKCOND ) | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Track Condition symbol is displayed. | O | DMI | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink (direction of transmission (1= track to train)) |
| M_VERSION | 7 | 001 0000 | Class 1 |
| Q_MEDIA | 1 | 0 | Type of media (0 = Balise) |
| N_PIG | 3 | 0 | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked = 1 |
| NID_PACKET | 8 | 68 | Track conditions |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | Empty profile, initial state to be resumed |
| D_TRACKINIT | 15 | 0 | 0 M |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | The same as the previous track condition |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 7/ 8/ 11/ 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |