| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4050200 (4.5.2 Active Functions Table – Empty case = function shall be inactive) Total: 28 Test case(s) |  |
| REF: | Subset-076-5-2-4050200 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 01.08.2013 | All | Second version/Internal modification; TC_Editorv27; Correction of Requirements blank space. Removed "Empty Case of Requirements" | INECO |
| Version 2.9.2 07.11.2013 | All | Third version/ UNISIG remarks from review sheet (October 2013) added. | INECO |
| Version 2.9.3 25.11.2013 | All | Fourth version/ Feature closed with a bilateral meeting on 10.01.2014; TC2, 9, 30 and 33 deleted. | INECO |
| Version 2.9.4 17.12.2013 | All | Fifth version/ Feature renumbered. | INECO |
| Version 2.9.5 10.01.2014 | All | Sixth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | No detection of loss of safe radio connection (T_NVCONTACT) and no initiation of the installed reaction. |  | 1 |
| Applicable Mode/Level Combinations | L2: SR / PT/ NL / RV L3: SR / PT/ NL / RV |  |  |
| Target of Test | The target of this test is to check that: When the difference between the time stamp of the latest consistent received message and the current on-board time is greater than the T_NVCONTACT parameter (national value) The on-board shall apply no reaction, as define in the M_NVCONTACT parameter. The function “Check Safe Radio Connection “ is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Check safe radio connection (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI interface that: The system status message "Communication error" is not displayed Check on the TIU interface that: service break not commanded Check on RTM interface that: no safe radio message received in due time (T_NVCONTACT) Check on the JRU that : General message from RBC is recorded. |
| Constraints | Packet 3: National Values has been previously received with T_NVCONTACT= FINITE VALUE and M_NVCONTACT = 01 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 8 / 11/ 14 | SR / PT/ NL / RV |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | Variable T_NVCONTACT= finite value Variable M_NVCONTACT= 01: System reaction if T_NVCONTACT elapses–service brake |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | With period lower than T_NVCONTACT. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | no safe radio message received in due time (T_NVCONTACT) | N/A | N/A | - |
| 4 | N/A | N/A | NOT MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The system status message "Communication error" is not displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 8 / 11/ 14 | SR / PT/ NL / RV |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | Variable T_NVCONTACT: Maximum time since creation in the RBC of last received telegram (Default value ‘1111 1111=(’ , Finite Values ‘0s’ to ‘254s’) Variable M_NVCONTACT: System reaction if T_NVCONTACT elapses (M_NVCONTACT=01- Service Brake) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | When the train reaches standstill, the on-board equipment does not send a position report. |  | 2 |
| Applicable Mode/Level Combinations | L0 : UN/ SB/ TR/ NL LNTC : SL/ SB/ TR/ NL L1 : SL/ SB/ TR/ PT/ NL L2: SL/ SB /TR/ PT/ NL L3: SL/ SB /TR/ PT/ NL |  |  |
| Target of Test | The target of this test is to check that: The train reaches standstill for any reason No message 136 is send to RBC by the on-board equipment. The function “Report train position when train reaches standstill” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Report train position when train reaches standstill |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message 136: “Train position report” is NOT RECORDED. |
| Constraints | Train at standstill. For SB mode, a cab is active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4/ 5/ 6/ 7/ 8/ 11 | UN/ SL/ SB /TR/ PT/ NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC /L1 / L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving. | I | INT | V_TRAIN≠0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill. | I | INT | (V_TRAIN=0) | N/A | N/A | - |
| 2 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3/ 4/ 5/ 6/ 7/ 8/ 11 | SH/ UN/ SL/ SB /TR/ PT/ NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC /L1 / L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train at standstill. | I | INT | V_TRAIN=0 |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | When the mode changes from SH to PS, the on-board equipment does not send a position report. |  | 3 |
| Applicable Mode/Level Combinations | L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that: The driver selects “Maintain Shunting” A mode change from SH to PS is performed by the on-board equipment No message 136 is send to RBC by the on-board equipment due to mode change The function “Report train position when mode changes to… ” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Report train position when mode changes to… |  |
|  |  | Subset-026-4.6.2 row 3 column 4 |  |
|  |  | Subset-026-4.6.3 Condition Id [26] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU interface that: Cab not active Check on the RTM that: No Message 136: “Train position report” is transmitted. Check on the JRU that: Driver action is recorded The Cab status not active is recorded A mode transition to PS is recorded. |
| Constraints | Desks are closed. “Continue shunting on desk closure” function is active. The “passive shunting “ input signal is received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Cab active Passive shunting permitted. | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SH | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 2 | L2 / L3 | SH | PASSIVE SHUNTING INPUT (NID_MESSAGE_JRU=31) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 3 | L2 / L3 | SH | The driver selects "MAINTAIN SHUNTING" | I | DMI | “Continue shunting on desk closure” function is active. | L2 / L3 | SH | - |
| 4 | L2 / L3 | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 1111) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 5 | L2 / L3 | SH | Cab not active. | I | TIU | - | L2 / L3 | SH | - |
| 6 | L2 / L3 | SH | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded | O | JRU | The ERTMS/ETCS on-board equipment switches immediately to Passive Shunting mode. | L2 / L3 | SH | - |
| 7 | L2 / L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=15) is recorded | O | JRU | - | L2 / L3 | PS | - |
| 8 | L2 / L3 | PS | NOT SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | L2 / L3 | PS | - |
| 9 | L2 / L3 | PS | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | PS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not send a position report when loss of train integrity is detected. |  | 4 |
| Applicable Mode/Level Combinations | L3: TR/ SL / NL |  |  |
| Target of Test | The target of this test is to check that: The train is at standstill. The loss of “Train Integrity” is detected by the on-board equipment. The on-board equipment does not send a position report, message 136 with packet 0 (Q_LENGHT=3) The function “Report train position when loss of train integrity is detected” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Report train position when loss of train integrity is detected |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM interface that: NO Message 136: “Train position report” with packet 0 (Q_LENGHT=3) is transmitted. |
| Constraints | Train integrity information confirmed by Integrity monitoring device. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 7 / 11 | SL / TR/ NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Train integrity confirmed | I | TIU | Train integrity information confirmed by Integrity monitoring device |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train integrity lost. | I | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message “Train Position Report” (NID_MESSAGE=136) containing packet 0 (Q_LENGHT=3) is transmitted. | O | RTM | Trackside is not informed that Train integrity is lost. | N/A | N/A | - |
| 3 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 7 / 11 | SL / TR / NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Train Integrity lost | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | V_TRAIN=0 |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not send a position report when train front/rear passes a RBC/RBC border. |  | 5 |
| Applicable Mode/Level Combinations | L2: SL / NL L3: SL / NL |  |  |
| Target of Test | The target of this test is to check that: The train passes the RBC/RBC boarder with its max safe front end The on-board equipment does not send a position report to the Handing Over RBC The train passes the RBC/RBC boarder with its min safe rear end. The on-board equipment does not send a position report to the Handing Over RBC The function “Report train position when train front/rear passes a RBC/RBC border” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Report train position when train front/rear passes a RBC/RBC border |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM interface that: After passing a RBC/RBC border with its max safe front end, the on-board equipment does not send a position report (message 136 with packet 0 or 1 ) to the Handing Over RBC. After passing a RBC/RBC border with its min safe rear end, the on-board equipment does not send a position report (message 136 with packet 0 or 1 ) to the Handing Over RBC |
| Constraints | The on-board equipment can handle two communication sessions. Two communication sessions are opened. The order to switch to another RBC at a given location received and stored onboard. The communication session with the Accepting RBC is already established. Only in Level 2/3. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 | SL / NL |
| Radio communication session | Established | With both RBCs |
| RBC Transition Order | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train is moving. | I | INT | The train passes a RBC/RBC border with its max safe front end. | N/A | N/A | - |
| 2 | N/A | N/A | NOT SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The train is moving. | I | INT | The train passes a RBC/RBC border with its min safe rear end. | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 or 1 is transmitted. | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 | SL / NL |
| Radio communication session | ESTABLISHED | With both RBCs |
| RBC Transition Order | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not send a position report when the whole train has left the level 2/3 area. |  | 6 |
| Applicable Mode/Level Combinations | L0: SL/ NL LNTC: SL/ NL L1: SL/ NL |  |  |
| Target of Test | The target of this test is to check that: The train passes the level transition boarder with its min safe rear end. The on-board equipment does not send a position report to RBC The function “Report train position when train rear passes a level transition border ( from level 2/3 to 0, NTC, 1)” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Report train position when train rear passes a level transition border (from level 2/3 to 0, NTC, 1) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM interface that: After passing the level transition border with its min safe rear end, the on-board equipment does not send a position report (message 136 with packet 0 or 1 ) to RBC. |
| Constraints | The whole train has left the level 2/3 area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 | SL / NL |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0/ 1 / 2 | L0 / LNTC/ L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train is moving. | I | INT | The train passes the level transition border with its min safe rear end.(level transition from L2/L3 to L0/LNTC/L1) | N/A | N/A | - |
| 2 | N/A | N/A | NOT SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 or 1 is transmitted. | O | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 | SL / NL |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0/ 1 / 2 | L0 / LNTC/ L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not send a position report when the estimated front end of the train passed the level transition border. |  | 7 |
| Applicable Mode/Level Combinations | L0: SL LNTC: SL L1 : SL |  |  |
| Target of Test | The target of this test is to check that: The train passes the level transition border with its estimate front end The on-board equipment does not send a position report to inform RBC about the level transition The function “Report train position when change of level due to trackside order” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Report train position when change of level due to trackside order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM interface that: When the level transition location is passed with the estimate front end of the train, the on-board equipment does not send a position report (message 136 with packet 0 or 1 ) to RBC. |
| Constraints | Level transition order received from trackside and stored on-board.(Transition from L0 /LNTC/ L1 to L2/3) A communication session is established with an order received via balise group in rear of the border location. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | L2/ L3. At a further location (D_LEVELTR≠32767). |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC/ L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Not active in SL |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1 | SL | The train is moving.(D_LEVELTR distance) | I | INT | The train passes the location given in the announcement (D_LEVETR) with the estimate front end. | L0/ LNTC/ L1 | SL | - |
| 2 | L0/ LNTC/ L1 | SL | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3/4) is recorded | O | JRU | When level changes | L2 / L3 | SL | - |
| 3 | L2 / L3 | SL | NOT SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 or 1 is transmitted. | O | RTM | - | L2 / L3 | SL | - |
| 4 | L2 / L3 | SL | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | SL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Not active in SL |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not request MA cyclically when MA timer elapsing (T_TIMEOUTRQST). |  | 8 |
| Applicable Mode/Level Combinations | L2 : SR / PT L3: SR / PT |  |  |
| Target of Test | The target of this test is to check that: The time before any section timer expires or the LOA speed timer expires (T_TIMEOUTRQST). The on-board equipment does not send a MA Request, message 132. The function “Request MA Cyclically” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Request MA Cyclically respect to approach of target indication point (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM interface that: When the T´+ T_LOA -T_TIMEOUTRQST elapses the on-board equipment does not send cyclically message 132 : MA Request to RBC. When the T´+ T_LOA -T_TIMEOUTRQST + TCYCRQST elapses the on-board equipment does not send cyclically message 132 : MA Request to RBC. Check on the JRU interface that: A MA for level 2/3 is recorded. |
| Constraints | No Gradient and SSP stored on-board. Information from Packet 57: MA Request Parameters already stored on-board with variables values as T_TIMEOUTRQST≠1023, T_MAR=255 and T_CYCRQST≠255 For PT mode only if following the reception of the information “Recognition of Exit TR mode” with a more recent time stamp. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 8 | SR / PT |
| Radio communication session | ESTABLISHED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| MA Request Parameters | STORED | T_TIMEOUTRQST≠1023 T_MAR=255 and T_CYCRQST≠255 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA .Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) is containing packet 15 is received | I | RTM | T´= message reception time T_LOA≠1023, V_LOA≠0 No optional packets 21 and 27 included in the message. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA. No Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | T´+ T_LOA -T_TIMEOUTRQST has elapsed | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-DATA. No Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | T´+ T_LOA -T_TIMEOUTRQST+ T_CYCRQST has elapsed | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | ≠0 |
| T_LOA | 10 | FINITE VALUE | ≠1023 |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 8 | SR / PT |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Message 3 with packet 15 only |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| MA Request Parameters | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not request MA cyclically respect to approach of target indication point (T_MAR) |  | 9 |
| Applicable Mode/Level Combinations | L2 : SR L3: SR |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment does not send a request for a new MA (radio message 132) according to value of the variable T_MAR (time before reaching the pre-indication location for EOA/LOA) previously received. The function “Request MA Cyclically” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Request MA Cyclically respect to approach of target indication point (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM interface that: When the T´+ T_MAR elapses the on-board equipment does not send cyclically message 132 : MA Request to RBC. |
| Constraints | No Gradient and SSP stored on-board. Information from Packet 57: MA Request Parameters already stored on-board with variables values as T_TIMEOUTRQST=1023, T_MAR≠255 and T_CYCRQST≠255 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| MA Request Parameters | STORED | T_TIMEOUTRQST=1023 T_MAR≠255 and T_CYCRQST≠255 |
| ERTMS/ETCS level | 3 / 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA .Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) is containing packet 15 is received | I | RTM | No optional packets 21 and 27 included in the message. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | The train approaches the target indication point with T_MAR. | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-DATA. No Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | The train reaches the target indication point with T_MAR. | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA. No Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | T_MAR + T_CYCRQST has elapsed | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| MA Request Parameters | STORED | - |
| ERTMS/ETCS level | 3/4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The Unconditional Emergency Stop is not managed by the on-board equipment. The train receives the Message 16 ‘Unconditional Emergency Stop’ from the supervising RBC, and there is an order to switch to L2/3 at a further location stored on-board, together with an MA for Level 2/3. The Message 16 is stored on-board but not used due to no transition to level 2/3 performed. The train is not tripped. |  | 10 |
| Applicable Mode/Level Combinations | L0 : UN |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives from RBC information about an “Unconditional Emergency stop”, message 16. The information is accepted and stored by the on-board equipment but not managed until the level transition to 2/3 is effective. The on-board equipment receives from Balise Group an order to switch to L0 (level with the highest priority), telegram including packet 41:Level transition order . The train is not tripped The on-board equipment doesn´t send message 147, with Q_EMERGENCYSTOP set to 3. The function “Manage Unconditional Emergency Stop” is INACTIVE. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage Unconditional Emergency Stop |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI interface that: The Level symbol "Level <L2 / L3> announcement" is removed Check on the JRU interface that: Reception of an “Unconditional Emergency Stop” is recorded. Balise group message 41:” Level transition order” to level 0 is recorded. No trip order is recorded. |
| Constraints | An order to switch to L2/L3 at a further location is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | For level 2 or 3 |
| Gradient Profile | STORED | For level 2 or 3 |
| International SSP | STORED | For level 2 or 3 |
| Level Transition Order | STORED | Order to switch to L2/L3 |
| ERTMS/ETCS level | 0 | L0 |
| Table of priority of trackside supported levels | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L2 / L3> announcement" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=16 ) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Balise Group message containing packet 41 (M_LEVELTR=0; D_LEVELTR=32767) is received. | I | BTM | The balise group is placed at level border. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=0; M_MODE=4) is recorded | O | JRU | Check the values of variables M_MODE<> 7 and M_LEVEL<> 3/4 The on-board equipment does not switch to Level 2/3, continues in level 0 and it is not tripped. | N/A | N/A | - |
| 6 | N/A | N/A | The Level symbol "Level <L2 / L3> announcement" is removed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=0¦Bit14=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147; Q_EMERGENCYSTOP=3) containing packet 0 or 1 is transmitted | O | RTM | Level transition to Level 2/3 not effective. Conditional emergency stop information is rejected. | N/A | N/A | - |

| Step 1: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | Unconditional Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | FINITE VALUE | X Balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| ERTMS/ETCS level | 0 | L0 |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The Conditional Emergency Stop is not managed by the on-board equipment. The on-board equipment receives a Conditional Emergency message. A level 2/3 transition announcement has been previously received. The message is stored but not used because the level transition execution to Level 2/3 is not effective. |  | 11 |
| Applicable Mode/Level Combinations | L0 : UN |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives from RBC information about a “Conditional Emergency stop”, message 15. The information is accepted and stored by the on-board equipment but not managed until the level transition to 2/3 is effective. The on-board equipment receives from Balise Group an order to switch immediately to L0 (level with the highest priority), telegram including packet 41:Level transition order . The on-board equipment doesn´t send message 147, with Q_EMERGENCYSTOP set to 3. The function Manage Conditional Emergency Stop is INACTIVE |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage Conditional Emergency Stop |  |
|  |  | Subset-026-8.7.6 Message 15: Conditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of message 15 is recorded Reception of telegram with packet 41 is recorded. |
| Constraints | An order to switch to L2/3 at a further location is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | For level 2 or 3 |
| Gradient Profile | STORED | For level 2 or 3 |
| International SSP | STORED | For level 2 or 3 |
| Level Transition Order | STORED | Order to switch to L2/L3 |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L2 / L3> announcement" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=15) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Balise Group message containing packet 41 (M_LEVELTR=0; D_LEVELTR=32767) is received. | I | BTM | The balise group is placed at level border. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=0; M_MODE=4) is recorded | O | JRU | The on-board equipment does not switch to Level 2/3, continues in level 0. | N/A | N/A | - |
| 6 | N/A | N/A | The Level symbol "Level <L2 / L3> announcement" is removed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=0¦Bit14=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147; Q_EMERGENCYSTOP=3) containing packet 0 or 1 is transmitted | O | RTM | Level transition to Level 2/3 not effective. Conditional emergency stop information is rejected. | N/A | N/A | - |

| Step 1: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | Conditional Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| ERTMS/ETCS level | 0 | L0 |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not determine the Most Restrictive Speed Profile, based on SSP.(V_STATIC does not apply even if it is the MRSP) |  | 12 |
| Applicable Mode/Level Combinations | L1 : SR L2 / L3 : SR |  |  |
| Target of Test | The target of this test case is to check that: Train is moving with V_TRAIN= National Value Mode Speed Limit (V_NVSTFF). The on-board equipment receives a balise group message including packet 27.(V_STATIC is the Most Restrictive Speed) The driver request to see the permitted speed. The displayed permitted speed is equal to National Value Mode Speed Limit (V_NVSTFF) The function “Determine most restrictive Speed Profile , based on: SSP” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: When the driver request to see the permitted speed, V_STATIC is not displayed . Check on the JRU interface that: Reception of balise message containing packet 27 is recorded. |
| Constraints | V_STATIC < National Value Mode Speed Limit (V_NVSTFF) The speed restriction set by the international train category must be the most restrictive one |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | V_NVSTFF |
| International SSP | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TRAIN = V_NVSTFF |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 27 is received | I | BTM | V_STATIC < V_NVSTFF | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | ≠ V_STATIC | N/A | N/A | - |
| 6 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | < V_NVSTFF |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | - |
| International SSP | STORED | V_STATIC not used |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_TRAIN = V_NVSTFF |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not determine the Most Restrictive Speed Profile, based on ASP.(V_AXLELOAD does not apply even if it is the MRSP) |  | 13 |
| Applicable Mode/Level Combinations | L1 : SR L2/ L3 : SR |  |  |
| Target of Test | The target of this test case is to check that: Train is moving with V_TRAIN= National Value Mode Speed Limit (V_NVSTFF ) The on-board equipment receives a balise group message including packet 51.(Q_TRACKINIT=0; V_AXLELOAD is the Most Restrictive Speed) The train reaches D_AXLELOAD where V_AXLELOAD applies. The driver request to see the permitted speed. The displayed permitted speed is equal to National Value Mode Speed Limit (V_NVSTFF) The function “Determine most restrictive Speed Profile , based on: ASP” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - ASP |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: Axle Load Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: When the driver request to see the permitted speed, V_AXLELOAD is not displayed . Check on the JRU interface that: Reception of balise message containing packet 51 is recorded. |
| Constraints | V_AXLELOAD < National Value Mode Speed Limit (V_NVSTFF) The speed restriction set by the Axle Load must be the most restrictive one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | V_NVSTFF |
| Axle load speed profile | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TRAIN = V_NVSTFF |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 51 (Q_TRACINIT=0) is received | I | BTM | V_AXLELOAD < V_NVSTFF | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | The train reaches the distance D_AXLELOAD | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | ≠ V_AXLELOAD | N/A | N/A | - |
| 7 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | No record because of no change on speed and distance monitoring information | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE |  |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | - |
| D_AXLELOAD | 15 | FINITE VALUE | - |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | The train axle load category is greater than the received axle load profile. |
| V_AXLELOAD(n) | 7 | FINITE VALUE | < V_NVSTFF |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | - |
| Axle load speed profile | STORED | V_AXLELOAD not used |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_NVSTFF |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not determine the Most Restrictive Speed Profile, based on Signalling related speed restriction when evaluated as a speed limit. (V_MAIN does not apply even if it is the MRSP) |  | 14 |
| Applicable Mode/Level Combinations | L2 / L3 : SR |  |  |
| Target of Test | The target of this test case is to check that: Train is moving with V_TRAIN= National Value Mode Speed Limit (V_NVSTFF). The on-board equipment receives a balise group message including packet 12. (V_MAIN< V_TRAIN). Packet 21 and 27 not included. The driver request to see the permitted speed. The displayed permitted speed is equal to National Value Mode Speed Limit (V_NVSTFF) The function “Determine most restrictive Speed Profile , based on: Signalling related speed restriction when evaluated as a speed limit” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Signalling related speed restriction when evaluated as a speed limit |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: When the driver request to see the permitted speed, V_MAIN is not displayed . |
| Constraints | The speed restriction set by the signal must be the most restrictive one. V_MAIN < National Value Mode Speed Limit (V_NVSTFF) Gradient and SSP information not stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | V_NVSTFF |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | STORED | (M_LEVETR= 2) at further location |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving. | I | INT | CSM: V_TRAIN = V_NVSTFF |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 12 (Q_DIR=1) is received | I | BTM | V_MAIN< V_TRAIN Packet 21 and 27 not included. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | ≠ V_MAIN | N/A | N/A | - |
| 6 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | No record because of no change on speed and distance monitoring information | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | < V_NVSTFF |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | STORED | (M_LEVETR= 2) at further location |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_NVSTFF |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not determine the Most Restrictive Speed Profile, based on Mode related speed restriction. (V_MAMODE does not apply even if it is the MRSP) |  | 15 |
| Applicable Mode/Level Combinations | L1: FS L2: FS L3: FS |  |  |
| Target of Test | The target of test is to check that: Train is moving with V_MAIN The on-boar equipment receives a balise group message containing packet 12 and packet 80.(V_MAMODE the MRSP) The permitted speed is not updated to V_MAMODE. Train reach the distance D_MAMODE. The permitted speed is updated to V_MAMODE. The function “Determine Most Restrictive Speed Profile , based on Mode related speed restriction” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Mode related speed restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Permitted speed is not updated to V_MAMODE before the distance D_MAMODE is reached. The driver requests to see permitted speed (only in OS and SH modes). Speed and distances monitoring information is updated. Check on the JRU that: Mode change is recorded. Application of Service Brake is RECORDED. Release of Service Brake is RECORDED. |
| Constraints | The train has previously stored the national and default values There are no temporary speed restrictions. In L2/3, Packet 12+80 accepted because SSP and gradients are already available on-board and do covers the full length of the MA. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| National Values | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | covers the full length of the MA |
| International SSP | STORED | covers the full length of the MA |
| Level Transition Order | STORED | Only in L2/L3 (M_LEVETR=2) |
| Mode Profile | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_MAIN |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | FS | One Balise Group message (N_TOTAL=1) containing packet 12 and packet 80 is received | I | BTM | V_MAMODE< V_MAIN | L1/L2/L3 | FS | - |
| 2 | L1/L2/L3 | FS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1/L2/L3 | FS | - |
| 3 | L1/L2/L3 | FS | The speed and distance monitoring information is displayed | O | DMI | ≠ V_MAMODE | L1/L2/L3 | FS | - |
| 4 | L1/L2/L3 | FS | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERMITTED ≠ V_MAMODE) is recorded | O | JRU | No record because of no change on speed and distance monitoring information | L1/L2/L3 | FS | - |
| 5 | L1/L2/L3 | FS | Train is moving | I | INT | Train in rear of distance D_MAMODE. | L1/L2/L3 | FS | - |
| 6 | L1/L2/L3 | FS | Transition to OS/SH/LS is performed | - | - | USE_FT4060300.39 Transition Cond ID [50] for SH or USE_FT4060300.54 Transition Cond ID [15] for OS Or USE_FT5190300.3 Transition Cond ID [70] for LS | L1/L2/L3 | OS/SH/LS | - |
| 7 | L1/L2/L3 | OS/SH/LS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only if in OS or SH mode | L1/L2/L3 | OS/SH/LS | - |
| 8 | L1/L2/L3 | OS/SH/LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only if in OS or SH mode | L1/L2/L3 | OS/SH/LS | - |
| 9 | L1/L2/L3 | OS/SH/LS | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN > V_MAMODE Status request: Intervention status | L1/L2/L3 | OS/SH/LS | - |
| 10 | L1/L2/L3 | OS/SH/LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TRAIN > V_MAMAODE; M_SDMTYPE= 0; M_SDMSUPSTAT= 4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | L1/L2/L3 | OS/SH/LS | - |
| 11 | L1/L2/L3 | OS/SH/LS | Service brake commanded | O | TIU | - | L1/L2/L3 | OS/SH/LS | - |
| 12 | L1/L2/L3 | OS/SH/LS | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | Indication of warning due to over-speeding (V_MAMODE < Train Speed < V_MAIN) | L1/L2/L3 | OS/SH/LS | - |
| 13 | L1/L2/L3 | OS/SH/LS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1/L2/L3 | OS/SH/LS | - |
| 14 | L1/L2/L3 | OS/SH/LS | Train is moving | I | INT | V_TRAIN=<V_MAMODE> | L1/L2/L3 | OS/SH/LS | - |
| 15 | L1/L2/L3 | OS/SH/LS | Service brake not commanded | O | TIU | - | L1/L2/L3 | OS/SH/LS | - |
| 16 | L1/L2/L3 | OS/SH/LS | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The speed is lower or equal to the minimum of speed permitted by the SSP. | L1/L2/L3 | OS/SH/LS | - |
| 17 | L1/L2/L3 | OS/SH/LS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | L1/L2/L3 | OS/SH/LS | - |
| 18 | L1/L2/L3 | OS/SH/LS | The speed and distance monitoring information is displayed | O | DMI | Train Speed =< V_MAMODE (Permitted Speed) Status request: Normal status | L1/L2/L3 | OS/SH/LS | - |
| 19 | L1/L2/L3 | OS/SH/LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TRAIN < V_MAMAODE>; M_SDMTYPE= 0; M_SDMSUPSTAT= 0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | L1/L2/L3 | OS/SH/LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | < V_MAIN |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1/ 3/ 12 | OS/ SH/ LS |
| National Values | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | covers the full length of the MA |
| International SSP | STORED | covers the full length of the MA |
| Level Transition Order | STORED | Only in L2/L3 (M_LEVETR=2) |
| Mode Profile | STORED | V_MAMODE |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_MAMODE. Normal Status Only if in OS or SH mode V_PERMITTED = V_MAMODE |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not continue Shunting on desk closure (Enabling transition to Passive Shunting mode) |  | 16 |
| Applicable Mode/Level Combinations | L0 : UN L1 : FS / LS / OS / SR / SL / SB / TR / PT / NL / RV L2 / L3 : FS / LS / OS / SR / SL / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: The driver enables the function “continue Shunting on desk closure”. Transition to PS mode is not recorded. The function “continue Shunting on desk closure” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Continue Shunting on desk closure (Enabling transition to Passive Shunting mode) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU interface that: The on-board equipment does not transit to Passive Shunting |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / RV |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2 / L3 | FS / LS / OS / SR / UN / SL / SB / TR / PT / NL / RV | Passive shunting permitted | I | TIU | - | L0 / L1 / L2 / L3 | FS / LS / OS / SR / UN / SL / SB / TR / PT / NL / RV | - |
| 2 | L0 / L1 / L2 / L3 | FS / LS / OS / SR / UN / SL / SB / TR / PT / NL / RV | PASSIVE SHUNTING INPUT (NID_MESSAGE_JRU=31) is recorded | O | JRU | - | L0 / L1 / L2 / L3 | FS / LS / OS / SR / UN / SL / SB / TR / PT / NL / RV | - |
| 3 | L0 / L1 / L2 / L3 | FS / LS / OS / SR / UN / SL / SB / TR / PT / NL / RV | Cab not active | I | TIU | Driver closes the desk. | L0 / L1 / L2 / L3 | SB | - |
| 4 | L0 / L1 / L2 / L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | L0 / L1 / L2 / L3 | SB | - |
| 5 | L0 / L1 / L2 / L3 | SB | The on-board equipment switches to SB mode. | - | - | USE_FT4060300.8,9 (Desk is closed (Transition [28])) | L0 / L1 / L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not determine the Most Restrictive Speed Profile, based on LX speed restriction. (V_LX does not apply even if it is the MRSP) |  | 17 |
| Applicable Mode/Level Combinations | L0: UN L1: SR |  |  |
| Target of Test | The target of test is to check that: Train is moving with V_TRAIN= National Value Mode Speed Limit (V_NVSTFF/ V_NVUNFIT/). The on-board equipment receives a balise group message containing packet 88.(where Q_LXSTATUS = 1 ; V_LX< National Value Mode Speed Limit ) The train reaches the LX start location. ( D_LX starts) The driver request to see the permitted speed. The displayed permitted speed is equal to National Value Mode Speed Limit (V_NVSTFF; V_NVUNFIT) The function “Determine Most Restrictive Speed Profile , based on: LX speed ” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - LX speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: When the driver request to see the permitted speed, V_LX is not displayed . The permitted speed displayed on driver request is equal to V_NVSTFF/V_NVUNFIT Check on the JRU that: Reception of balise message containing packet 88 is recorded. |
| Constraints | In L0, an order to switch to Level 1 at a further location already stored on-board The speed restriction set by the non-protected level crossing must be the most restrictive one. V_LX < National Value Mode Speed Limit (V_NVSTFF/ V_NVUNFIT) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| National Values | STORED | V_NVSTFF / V_NVUNFIT |
| Level Transition Order | STORED | Only for L0 ( M_LEVELTR=2) |
| ERTMS/ETCS level | 0 / 2 | L0 / L1 |
| Level Crossing information | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_NVSTFF / V_NVUNFIT |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 88 (Q_LXSTATUS = 1, V_LX) is received | I | BTM | V_LX< V_TRAIN Q_LXSTATUS set to 1 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | The train reaches the distance D_LX where the non protected LX starts | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_NVSTFF / V_NVUNFIT | N/A | N/A | - |
| 7 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | No record because of no change on speed and distance monitoring information | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 88 | Level Crossing Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | < V_TRAIN |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| National Values | STORED | V_NVSTFF / V_NVUNFIT |
| Level Transition Order | STORED | Only in L0 (M_LEVETR=2) |
| ERTMS/ETCS level | 0 / 2 | L0 / L1 |
| Level Crossing information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_NVSTFF / V_NVUNFIT |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not determine the Most Restrictive Speed Profile, based on Speed Restriction to ensure a given permitted braking distance. |  | 18 |
| Applicable Mode/Level Combinations | L1: SR L2 / L3: SR |  |  |
| Target of Test | The target of test is to check that: Train is moving with V_TRAIN= National Value Mode Speed Limit (V_NVSTFF) The on-board equipment receives a balise group message containing packet 52. The driver request to see the permitted speed. The displayed permitted speed is equal to National Value Mode Speed Limit (V_NVSTFF) The function “Determine MRSP based on Speed Restriction to ensure a given permitted braking distance” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Speed restriction to ensure a given permitted braking distance |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: The speed and distance information is not updated Check on the JRU interface that: Driver action is recorded. Speed and distance info is recorded |
| Constraints | In level 2 / 3 an order to switch to L1 at a further location has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | - |
| Level Transition Order | STORED | Only for L2/L3 (M_LEVELTR=2 at further location) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Permitted Braking Distance Information | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1 ) containing packet 52 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TARGET= <V_MRSP> is the speed restriction due to National values stored | N/A | N/A | - |
| 6 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET= <V_MRSP>) is recorded | O | JRU | There is no update of permitted speed so, no change to be recorded. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | - |
| Level Transition Order | STORED | Level Transition Order to L1. (Only for L2 / L3) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Permitted Braking Distance Information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_NVSTFF |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not supervise Train Movements against Backwards Distance Monitoring. (D_NVPOTRP is exceed and brakes shall not be triggered) |  | 19 |
| Applicable Mode/Level Combinations | L0: UN / SH L1: FS / LS / OS / SR/ SH L2: FS / LS / OS / SR/ SH |  |  |
| Target of Test | The target of test is to check that: The train is at standstill. The on-board detects a backwards movement exceeding the national value for the allowed backwards distance. ( D_NVPOTRP). Service brake command is not triggered. The function “Supervise train movements against Backwards Distance Monitoring” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Backwards Distance Monitoring |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU interface that: Service brake not commanded when D_NVPOTRP exceeded. |
| Constraints | Train is at standstill. The train is moving backwards a distance D_NVROLL > D_NVPOTRP. Packet 3 with D_NVPOTRP already stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 / 2 / 3 / 4 / 12 | FS / OS / SR/ / SH/ UN/ LS |
| National Values | STORED | D_NVROLL > D_NVPOTRP |
| ERTMS/ETCS level | 0 / 2 / 3 | L0 / L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving. | I | INT | The train is moving backwards. D_NVPOTRP distance exceeded | N/A | N/A | - |
| 3 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 / 2 / 3 / 4 / 12 | FS / OS / SR/ / SH/ UN/ LS |
| National Values | STORED | D_NVPOTRP |
| ERTMS/ETCS level | 0 / 2 / 3 | L0 / L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not supervise Train Movements against Roll Away Protection. (D_NVROLL is exceed and brakes shall not be triggered) |  | 20 |
| Applicable Mode/Level Combinations | L0 / LNTC/ L1 / L2 / L3: NL |  |  |
| Target of Test | The target of test is to check that: The train is at standstill. The on-board detects a roll away movement exceeding the national value for the allowed roll away distance. ( D_NVROLL ). Service brake command is not triggered. The function “Supervise train movements against Roll Away Protection (RAP)” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Roll Away Protection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU interface that: Service brake not commanded when D_NVROLL exceeded. |
| Constraints | Train at standstill. The train is moving in a direction that conflicts with the direction controller. Packet 3 with D_NVROLL already stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| National Values | STORED | D_NVROLL |
| ERTMS/ETCS level | 0 / 1 / 2/ 3 / 4 | L0 / LNTC/ L1/ L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | The train is moving in a direction that conflicts with the direction controller. D_NVROLL for roll away exceeded | N/A | N/A | - |
| 2 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| National Values | STORED | D_NVROLL |
| ERTMS/ETCS level | 0 / 1 / 2/ 3 / 4 | L0 / LNTC/ L1/ L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not supervise Train Movements against Reverse Movement Protection. (D_NVROLL is exceed and brakes shall not be triggered) |  | 21 |
| Applicable Mode/Level Combinations | L0 : UN |  |  |
| Target of Test | The target of test is to check that: The on-board equipment receives a balise group message containing packet 12. (Packet 21 and 27 not included) The train comes to standstill. The on-board detects a reverse movement exceeding the national value for the allowed reverse distance. ( D_NVROLL ). Service brake command is not triggered. The function “Supervise train movements against Reverse Movement Protection (RMP)” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Reverse Movement Protection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU interface that: Service brake not commanded when D_NVROLL exceeded. |
| Constraints | At standstill, the permitted movement of train shall be the one of the currently stored MA. An order to switch to Level 1 at a further location already stored on-board. Packet 3 with D_NVROLL already stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| National Values | STORED | D_NVROLL |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | STORED | M_LEVELTR = 2 |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_NVUNFIT |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1) containing packet 12 (Q_DIR=1) is received | I | BTM | Packet 21 and 27 not included. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is at standstill | I | INT | Train comes to standstill for any reason | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | Train is moving in the opposite direction than the permitted one. D_NVROLL for reverse movements exceeded | N/A | N/A | - |
| 5 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | < V_NVUNFIT |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| National Values | STORED | D_NVROLL |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not supervise Train Movements against the information ” List of expected balises for shunting”. The train shall not be tripped when passing a balise group not contained in the list. |  | 22 |
| Applicable Mode/Level Combinations | L1 : FS/ LS / OS L2 : FS / LS / OS L3 : FS / LS / OS |  |  |
| Target of Test | The target of test is to check that: The on-boar equipment receives a balise group message containing packet 12, 21, 27 and 49 (containing a list of expected balises for shunting). The on-board equipment receives a balise group telegram from a balise not included in the list. The train is not tripped. The function “Supervise train movements against the information List of balises for SH area ” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Supervise “danger for shunting” information and list of expected balises for shunting |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: The mode does not changed to TR |
| Constraints | An order to switch to Level 1 at a further location already stored on-board. SSP and Gradients information not stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | STORED | Only for L2/L3 |
| List of Balises for SH area | NOT STORED | - |
| Mode Profile | STORED | Only for LS / OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 12 (Q_DIR=1), packet 21, 27, 80 and packet 49 (N_ITER=3) is received | I | BTM | Packet 21 and 27 covers the full length of the received MA | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 3 is received | I | BTM | BG not contained in packet 49 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Mode symbol "Trip" is displayed. | O | DMI | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | < V_MAIN |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 49 | List of Balises for SH area |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | 3 | - |
| Q_NEWCOUNTRY(k) | 1 | 0 | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | 1 | - |
| Q_NEWCOUNTRY(k) | 1 | 0 | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | 2 | - |
| Q_NEWCOUNTRY(k) | 1 | 0 | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | 3 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | 4 | Not included in the list of Balises for SH area |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 3 | National Values |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | 4 | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Only for L2/3 |
| List of Balises for SH area | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Mode <FS / LS / OS>" is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board equipment does not supervise Train Movements against the information ” Signalling related speed restriction when evaluated as a trip order”. (V_MAIN set to 0 and the train is not tripped) |  | 23 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN |  |  |
| Target of Test | The target of test is to check that: The on-boar equipment receives a balise group message containing packet 12 (V_MAIN =0) The train is not tripped. The function “Signalling related speed restriction when evaluated as a trip order” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Supervise Signalling related speed restriction when evaluated as a trip order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: The mode does not changed to TR |
| Constraints | An order to switch to Level 1 at a further location already stored on-board. The Movement authority information is received before reaching the announced Level transition order. SSP and Gradients information not stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| National Values | STORED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 12 (Q_DIR=1) is received | I | BTM | V_MAIN=0 Packet 21 and 27 not included. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT. The Mode symbol "Trip" is displayed. | O | DMI | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Movement authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | 0 | Trip order |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| National Values | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Unfitted/ SN" is displayed. | O | DMI | DMI Object: MO17/ MO19 |
| NOT RELEVANT | - | BTM | - |
| Train is moving. | I | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not manage Track Conditions excluding Sound Horn, Non Stopping Areas, Tunnel Stopping Areas and Big Metal Masses information. (Q_TRACKINIT set to 0 and no Track Condition Symbol is displayed) |  | 24 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN L1: SR L2 / L3 : SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives a balise group message including packet 68 “Track Condition”. The train reaches with its max safe front end the area where the profile type Track Condition applies. Track Condition Symbol excluding Sound Horn, Non Stopping Areas and Tunnel Stopping Areas is not displayed to the driver. The function “Manage Track Conditions excluding Sound Horn, Non Stopping Areas, Tunnel Stopping Areas and Big Metal Masses” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: Track Condition Symbol excluding Sound Horn, Non Stopping Areas and Tunnel Stopping Areas is not displayed to the driver. Check on the JRU interface that: The on-board equipment receives a balise group message including packet 68 “Track Condition”. |
| Constraints | In L0/ L2/ L3, an order to switch to Level 1 at a further location already stored on-board. The M_TRACKCOND value is different to 0,1 and 2. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 | SR / UN / SN |
| Level Transition Order | STORED | Only for L0 / LNTC/ L2 / L3 (M_LEVELTR= 2) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1) containing packet 68 (Q_TRACKINIT=0) is received | I | BTM | Q_TRACKINT set to 0 M_TRACKCOND ≠ 0,1 and 2 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_TRACKCOND | N/A | N/A | - |
| 4 | N/A | N/A | NOT Track Condition Symbol excluding Sound Horn, Non Stopping Areas and Tunnel Stopping Areas is displayed. | O | DMI | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 15 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | - |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | M_TRACKCOND ≠ 0,1,2 |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND (k) | 15 | FINITE VALUE | - |
| L_TRACKCOND (k) | 15 | FINITE VALUE | - |
| M_TRACKCOND (k) | 4 | FINITE VALUE | M_TRACKCOND ≠ 0,1 2 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 | SR / UN / SN |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT Track Condition Symbol is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | Inside Track condition length. |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not manage Track Sound Horn, Non Stopping Areas, Tunnel Stopping Areas information. |  | 25 |
| Applicable Mode/Level Combinations | L0 : UN L1 : SR L2 / L3 : SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives a radio message including packet 68 “Track Condition”. Track Conditions Sound Horn, Non Stopping Areas or Tunnel Stopping Areas are not displayed The function “Manage Track Conditions Sound Horn, Non Stopping Areas, Tunnel Stopping Areas” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage Track Conditions Sound Horn, Non Stopping areas, Tunnel Stopping areas |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: Track Conditions Sound Horn, Non Stopping Areas or Tunnel Stopping Areas are not displayed Check on the JRU interface that: The on-board equipment receives a radio message including packet 68 “Track Condition”. |
| Constraints | In L0/ L1, an order to switch to Level 2/3 at a further location already stored on-board. The M_TRACKCOND value is equal to 0,1 or 2. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | For L0 / L1 |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (Q_DIR=1; D_TRACKCOND) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_TRACKCOND | N/A | N/A | - |
| 4 | N/A | N/A | NOT Track Condition symbol "TCXX" is displayed. | O | DMI | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 15 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | - |
| D_TRACKCOND | 15 | FINITE VALUE | - |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | M_TRACKCOND = 0,1,2 |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND (k) | 15 | FINITE VALUE | - |
| L_TRACKCOND (k) | 15 | FINITE VALUE | - |
| M_TRACKCOND (k) | 4 | FINITE VALUE | M_TRACKCOND = 0,1 2 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| Level Transition Order | STORED | Only for L0 / L1 |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT Track Condition Symbol is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not manage Route Suitability information. |  | 26 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN L1: SR L2 / L3 : SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives a radio message including packet 70 “Route Suitability Data”. The system status message "Route unsuitable – X" is not displayed The function “Manage Route Suitability” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage Route Suitability |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: The system status message "Route unsuitable – X" is not displayed Check on the JRU interface that: The on-board equipment receives a radio message including packet 70 “Route Suitability Data”. |
| Constraints | In L0 / L1, an order to switch to Level 2/3 at a further location already stored on-board. At least one unsuitability exists between the train and the trackside. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 | SR / UN / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | Only for L0 / L1 |
| Route Suitability Data | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (Q_DIR=1; D_SUITABILITY) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_SUITABILITY | N/A | N/A | - |
| 4 | N/A | N/A | NOT the system status message "Route unsuitable – X" is displayed. | O | DMI | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | Route Suitability |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 15 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | - |
| D_SUITABILITY | 15 | FINITE VALUE | Only if Q_TRACKINIT=0, D_SUITABILITY and the following variables follows |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY=loading gauge |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY=Max axle load |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY= traction system and M_VOLTAGE≠ 0 |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY (k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY (k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE (k) | 8 | FINITE VALUE | If Q_SUITABILITY=loading gauge |
| M_AXLELOADCAT (k) | 7 | FINITE VALUE | If Q_SUITABILITY=Max axle load |
| M_VOLTAGE (k) | 4 | FINITE VALUE | If Q_SUITABILITY = traction system |
| NID_CTRACTION (k) | 10 | FINITE VALUE | If Q_SUITABILITY= traction system and M_VOLTAGE≠ 0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 | SR / UN / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | Only for L0 / L1 |
| Route Suitability Data | STORED | - |
| ERTMS/ETCS level | 0 / 1/ 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT The system status message "Route unsuitable" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not manage RBC/RBC Handover |  | 27 |
| Applicable Mode/Level Combinations | L2 / L3 : SB / PT |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives a Radio message including packet 131 “RBC transition order”. The on-board equipment do not transmits message 155 to the Accepting RBC. The function “Manage RBC/RBC handover” is inactive |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage RBC/RBC Handover |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM interface that: No transmission of message 155 to the Accepting RBC Check on the JRU interface that: Message including packet 131 “RBC transition order” is recorded. |
| Constraints | In PT, Recognition of Exit from TR mode was received recently. Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| RBC Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 131 is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) containing packet X is transmitted | O | RTM | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | RBC transition order with null distance to execution |
| NID_C | 10 | FINITE VALUE | “Accepting” RBC identity |
| NID_RBC | 14 | FINITE VALUE | “Accepting” RBC identity |
| NID_RADIO | 64 | FINITE VALUE | “Accepting” RBC radio subscriber number |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| RBC Transition Order | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.5.2 Active Functions Table – Empty case = function shall be inactive |  | 4050200 |
| Test Case of Feature | The on-board does not manage “Inhibition of revocable TSRs from balises in L2/L3. |  | 28 |
| Applicable Mode/Level Combinations | L2 / L3 : SR |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS/ETCS on-board equipment receives an order from RBC to reject revocable TSRs from balises in L2/L3.(Packet 64) The ETRMS/ETCS on-board equipment receives from a Balise Group a telegram containing a TSR. (Packet 65) The train reaches TSR area and speed restriction due to TSR is applied. The function “Inhibition of revocable TSRs” is inactive. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Inhibition of revocable TSRs from balises(only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: The permitted speed is equal to V_TSR. Check on the JRU interface that: The on-board equipment receives a radio message including packet 64 The on-board equipment receives a balise group message including packet 65. |
| Constraints | V_TSR is lower than permitted speed in UN /SR The inhibition is stored and is effective immediately and applies only for revocable TSRs received from balises thereafter. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | - |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 64(Q_DIR=1) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 65(Q_DIR=1; D_TSR=0) is received | I | BTM | NID_TSR = 0-126 (revocable TSR) | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Train is moving. | I | INT | The train drives inside TSR area with it maximum safe front end. | N/A | N/A | - |
| 6 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Speed value defined as V_TSR in stored message is used by the on-board equipment. CSM: V_TSR | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 64 | Inhibition of revocable Temporary Speed Restriction from balises in L2/L3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 000 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | 0-126 (revocable TSR) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | Most restrictive speed profile |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 001 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | - |
| Inhibition of revocable TSRs from balises in L2/3 | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_TSR |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |