| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5190600 (5.19.6: Exit of Limited Supervision mode) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5190600 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 05.10.2012 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 09.10.2013 | TC1 | Update with the current guidelines | MULTITEL |
| Version 2.9.2 12.11.2013 | TC1 | Response to Review002_Subset-076-5-2-5190600_v291 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.6: Exit of Limited Supervision mode |  | 5190600 |
| Test Case of Feature | The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA of the MA). The system switches to Full Supervision Mode immediately |  | 1 |
| Applicable Mode/Level Combinations | L2: LS L3: LS |  |  |
| Target of Test | The target of this test case is to check that on-board switches to FS mode when the train reaches the end of the LS area . |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.6.2 row 5 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [31] |  |
|  |  | Subset-026-5.19.6.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the mode initially displayed changes to FS. Check on the JRU that : the new current mode FS is recorded. |
| Constraints | The Limited Supervision area ends before the EOA of the current MA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | After he end of the Limited Supervision area, no specific mode is required by a Mode Profile. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LS mode and level is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | LS | The Mode symbol "Full Supervision" is displayed | O | DMI | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. | L2 / L3 | FS | - |
| 2 | L2 / L3 | FS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | The mode change to FS is stored. | L2 / L3 | FS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| FS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |