| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4070201 (4.7.2: DMI versus Mode Table - Output) Total: 41 Test case(s) |  |
| REF: | Subset-076-5-2-4070201 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.0.0 14.02.2014 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, TCs renumbered | MULTITEL |
| Version 2.9.2 04.10.2013 | TC11, TC33 | Modifications requested by issues 52 to 53 of Review003_Subset-076-5-2-4070201v291 | MULTITEL |
| Version 2.9.3 27.11.2013 | All | Modifications requested by Review003_Subset-076-5-2-4070201v291 | MULTITEL |
| Version 2.9.4 09.01.2014 | TC3 | Modification requested by issue 1 of Review001_Subset-076-5-2-4070201_v293 | MULTITEL |
| Version 2.9.5 22.01.2014 | TC22 | Apply of issue 1 from Review001_Subset-076-5-2-4070201_v294 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Train receives TSR message where speed < maximum train speed and any other previously stored static speed restriction |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, OS, SR L2: FS, OS, SR L3: FS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: The train speed stored on-board as TSR will be shown to driver as permitted speed if train achieves TSR area |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.4.1 |  |
|  |  | Subset-026-3.11.1.2 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The correct permitted speed (as received as TSR) has to be shown on the DMI when train achieves TSR area |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Temporary Speed Restrictions | STORED | NID_TSR=A / D_TSR = D_TSR(A) / L_TSR = L_TSR(A) / Q_FRONT=0 / /V_TSR = V_TSR(A) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only if in OS/SR | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=27) is recorded | O | JRU | Only if in OS/SR | N/A | N/A | - |
| 3 | N/A | N/A | The train runs an optional distance greater than zero but doesn't drive in section defined in TSR. The PS will decrease until it reaches the value of the TSR. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | Permitted speed recorded on JRU | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The train drives in TSR area. The maximum safe front end of the train reaches the beginning of the TSR area. Speed value defined as TSR in stored message will be shown to driver on DMI | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | Permitted speed recorded on JRU | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 UNCHANGED | FS / OS / SR |
| Temporary Speed Restrictions | STORED UNCHANGED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The permitted speed value is DISPLAYED | O | DMI | The speed value that is displayed on the DMI is a permitted speed equal to the stored Temporary Speed Restriction |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Indication of Level 2 is displayed to the driver as long as the current level is 2 |  | 2 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test is to check that: There is a permanent indication of Level 2 to the driver while the system is in level 2 Note : The indication of level L2 is displayed on the DMI as soon as the level transition from levels L0, LNTC, L1 or L3 to L2 occurs. These transitions are covered by at least one of the test cases of transition features from levels L0, LSTL, L1 or L3 to level L2. The indication of level L2 is deleted from the DMI as soon as the level transition to levels L0, LNTC, L1 or L3 occurs These transitions are covered at least by one test cases of level transition features from L2 to L0, LNTC, L1 or L3. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Current ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a permanent indication of Level 2 to the driver. |
| Constraints | If in SB or TR mode, the desk is open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SH / SB / TR / PT / NL / LS / RV |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| L2 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Permanent Indication of Level 2 to the driver | O | DMI | The level indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 6 / 7 / 8 / 11 / 12 / 14 UNCHANGED | FS / OS / SR / SH / SB / TR / PT / NL / LS / RV |
| ERTMS/ETCS level | 3 UNCHANGED | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Indication of Level 2 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | On-board supervision of on sight speed defined by trackside. |  | 3 |
| Applicable Mode/Level Combinations | L1: OS L2: OS L3: OS |  |  |
| Target of Test | The target of this test is to check that: The ERTMS/ETCS on-board equipment supervises train movements against the lowest of the maximum train speed and the maximum speed for on sight area given by trackside as mode related speed restriction. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.11.1.2 |  |
|  |  | Subset-026-3.11.7.1 |  |
|  |  | Subset-026-3.11.7.1.1 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train speed is DISPLAYED. Permitted Speed is DISPLAYED at Driver’s request |
| Constraints | The train has previously received Packet Number 80: “Mode profile“ including variable V_MAMODE (Required mode related speed). No other speed restriction (e.g. TSR) is lower than the mode related speed restriction. V_MAMODE shall be different than "default value" to be relevant. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Mode Profile | STORED | 0 ≤ V_MAMODE ≤ 120 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train speed is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Driver requests display of Permitted speed at Driver’s Request | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Driver request is RECORDED | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Permitted speed is DISPLAYED on the DMI on Driver’s Request | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The ERTMS/ETCS on-board equipment supervises train speed versus a mode related speed restriction given by trackside (V_MAMODE). | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver accelerates and the Warning Indication is DISPLAYED | O | DMI | “Warning Indication due to over-speeding in a ceiling speed monitoring section.”, | N/A | N/A | - |
| 6 | N/A | N/A | The driver decelerates and the Warning Indication is REMOVED | O | DMI | “Warning Indication due to over-speeding in a ceiling speed monitoring section.”, | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 UNCHANGED | OS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train speed is DISPLAYED. Permitted Speed is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Display of the LSSMA when the generic LS function marker is not stored on-board |  | 4 |
| Applicable Mode/Level Combinations | L1: LS L2: LS L3: LS |  |  |
| Target of Test | The target of this test case is to check that, if the generic LS function marker is not stored on-board: - the toggle on or off order are ignored and - the LSSMA is displayed when the target speed at the EOA/LOA is lower than the Limited Supervision mode speed limit AND the LSSMA is lower than the maximum train speed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.4 |  |
|  |  | Subset-026-4.4.19.1.4 b) |  |
|  |  | Subset-026-4.4.19.1.4.7 |  |
|  |  | Subset-026-4.4.19.1.4.7 a) |  |
|  |  | Subset-026-4.4.19.1.4.7 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: the order to toggle on the LSSMA display is ignored the on-board receive a new MA with V_LOA < V_NVLIMSUPERV and V_LOA < V_MAXTRAIN the LSSMA number display is recorded the order to toggle off the LSSMA display is ignored |
| Constraints | There is no MRSP element lower than V_LOA between the minimum safe front end of the train and the EOA/LOA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if level is L2/L3 |
| NOT RELEVANT | - | TIU | - |
| LSSMA is NOT displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | LS | The LSSMA number is NOT displayed | O | DMI | - | L1/L2/L3 | LS | - |
| 2 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 180 (Q_LSSMA= 1, T_LSSMA=0) is received. | I | BTM | Optional step, only if Level is L1 | L1 | LS | - |
| 3 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Optional step, only if Level is L1 | L1 | LS | - |
| 4 | L2/L3 | LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1, T_LSSMA=0) is received | I | RTM | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 5 | L2/L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 6 | L1/L2/L3 | LS | Not The LSSMA number is displayed | O | DMI | - | L1/L2/L3 | LS | - |
| 7 | L1/L2/L3 | LS | Not LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1/L2/L3 | LS | - |
| 8 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 12, packet 21, packet 27, packet 80 is received. | I | BTM | Optional step, only if Level is L1 V_LOA < all speed related values (V_...) received in these packets V_LOA < V_NVLIMSUPERV and V_LOA < V_MAXTRAIN | L1 | LS | - |
| 9 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Optional step, only if Level is L1 | L1 | LS | - |
| 10 | L2/L3 | LS | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (Q_DIR=1), packet 21, packet 27, packet 80 (D_MAMODE=0) is received. | I | RTM | Optional step, only if Level is L2 or L3 V_LOA < all speed related values (V_...) received in these packets V_LOA < V_NVLIMSUPERV and V_LOA < V_MAXTRAIN | L2/L3 | LS | - |
| 11 | L2/L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 12 | L1/L2/L3 | LS | The LSSMA number is displayed | O | DMI | LSSMA = V_LOA | L1/L2/L3 | LS | - |
| 13 | L1/L2/L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1/L2/L3 | LS | - |
| 14 | L1/L2/L3 | LS | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1/L2/L3 | LS | - |
| 15 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 180 (Q_LSSMA= 0) is received. | I | BTM | Optional step, only if Level is L1 | L1 | LS | - |
| 16 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Optional step, only if Level is L1 | L1 | LS | - |
| 17 | L2/L3 | LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=0) is received | I | RTM | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 18 | L2/L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 19 | L1/L2/L3 | LS | The LSSMA number is displayed | O | DMI | The toggle off order has been ignored | L1/L2/L3 | LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | 0 | immediate toggle on |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | 0 | Only if Q_LSSMA = 1 |

| Step 8: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | > V_LOA |
| V_LOA | 7 | FINITE VALUE | V_LOA < V_NVLIMSUPERV and V_LOA < V_MAXTRAIN |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | > V_LOA |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | > V_LOA |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | V_MAMODE > V_LOA |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | if k>0, V_MAMODE > V_LOA |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | > V_LOA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | > V_LOA |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | > V_LOA |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | > V_LOA |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | V_LOA < V_NVLIMSUPERV and V_LOA < V_MAXTRAIN |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | > V_LOA |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | > V_LOA |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | > V_LOA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | > V_LOA |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | > V_LOA |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | > V_LOA |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | V_MAMODE > V_LOA |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | if k>0, V_MAMODE > V_LOA |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 15: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 0 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 15: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 0 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | New MA received |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | New MP received |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if level is L2/L3 |
| NOT RELEVANT | - | TIU | - |
| LSSMA is NOT displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | There is a correct cab signalling displayed to the driver when the current mode is SR |  | 5 |
| Applicable Mode/Level Combinations | L1: SR L2: SR L3: SR |  |  |
| Target of Test | The target of this test is to check that: There is a correct cab signalling to the driver in Level 1/2/3 (Permitted speed, Target speed and Target distance are DISPLAYED upon driver request with Train Speed) in SR mode |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.11.1.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Target Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Target distance |  |
|  |  | Subset-026-4.7.2 table - Output information Train Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a correct cab signalling to the driver while the system is in Level 1/2/3 and SR mode when the driver request it. Check on JRU that: The driver requests to show and to hide supervision limits are recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED / NOT STORED | If STORED: V_NVSTFF=<V_NVSTFF_1> If NOT STORED: V_NVSTFF=40km/h |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Cab Signalling is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | There is no indication to the driver of Permitted speed Target distance Target speed until the driver requests it | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=27) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | On-Board Equipment displays permitted speed (National value received <V_NVSTFF_1> or Default value 40km/h), target distance and target speed Distance monitoring shall be displayed when train will be close enough to the target distance, otherwise distance monitoring shall not be displayed | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests the speed and distance monitoring information to be removed | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=25) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is removed | O | DMI | On-Board Equipment stops to display permitted speed, target distance and target speed | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 UNCHANGED | SR |
| National Values | UNCHANGED | - |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Permitted speed V_NVSTFF, target distance and target speed no more DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Visualisation of the data |  | 6 |
| Applicable Mode/Level Combinations | L0 : SH, UN, SB, TR, NL LNTC: SH, SB, TR, NL, SN L1 : FS, LS, OS, SR, SH, SB, TR, PT, NL, RV L2 : FS, LS, OS, SR, SH, SB, TR, PT, NL, RV L3 : FS, LS, OS, SR, SH, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that the on-board Equipment, allows the driver to view the current Driver ID, Train running number, RBC contact information and Virtual Balise Covers. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Driver id |  |
|  |  | Subset-026-4.7.2 table - Output information Train running number |  |
|  |  | Subset-026-4.7.2 table - Output information RBC contact information - Radio network-id |  |
|  |  | Subset-026-4.7.2 table - Output information RBC contact information - RBC-id |  |
|  |  | Subset-026-4.7.2 table - Output information RBC contact information - RBC phone number |  |
|  |  | Subset-026-4.7.2 table - Output information Virtual Balise Covers |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver is allowed to view the existing Driver ID, Train running number, RBC contact information and Virtual Balise Covers. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS/ SR / SH / UN / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |
| Train Running Number | VALID | - |
| Virtual Balise Covers | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The on-board equipment authorises the driver to ask for the data view | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "Data view" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The data from "Data View" window are displayed | O | DMI | The current Driver ID, Train running number, RBC contact information and Virtual Balise Covers is displayed in the window | N/A | N/A | - |
| 3 | N/A | N/A | The driver closes the "Data View" window | I | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 UNCHANGED | FS/ OS/ SR / SH / UN / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID UNCHANGED | - |
| Radio Network ID | VALID UNCHANGED | - |
| RBC ID/Phone Number | VALID UNCHANGED | - |
| Train Running Number | VALID UNCHANGED | - |
| Virtual Balise Covers | VALID UNCHANGED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Limited Supervision mode is permanently displayed to the driver when the on-board equipment is in Limited Supervision mode |  | 7 |
| Applicable Mode/Level Combinations | L1: LS L2: LS L3: LS |  |  |
| Target of Test | The target of this test is to check that: There is a permanent indication of the Limited Supervision mode while the system is in Limited Supervision mode |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a permanent indication of the LS mode to the driver while the system is in LS mode. |
| Constraints | The indication of LS mode is displayed on the DMI as soon as one of the mode transition to LS occurs. These transitions are covered by at least one of the relevant test cases of mode transition features. The indication of LS mode is deleted from the DMI as soon as one of the transition to another mode occurs These transitions are covered at least by one of the relevant test cases of mode transition features. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Permanent Indication of LS mode to the driver | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 UNCHANGED | LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Indication of LS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Reversing area information is available on-board and the train is stopped beyond the reversing permitted area. Reversing is not allowed. |  | 8 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: Information about reversing is not available to the driver if the train is stopped beyond the reversing permitted area. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.13.1.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Reversing allowed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The reversing permitted symbol (ST06) is NOT DISPLAYED. |
| Constraints | The reversing permitted symbol (ST06) is displayed to the driver when the following condition are fulfilled: the train is at standstill with the front end of the train inside the reversing area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | D_STARTREVERSE = FINITE_VALUE / L_REVERSEAREA = FINITE VALUE |
| Reversing Supervision Information | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is stopped beyond the end of the reversing area (D_STARTREVERSE+ L_REVERSEAREA) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | Information about a permitted reversing movement is NOT DISPLAYED | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Information about a permitted reversing movement is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Reversing area information is available on-board and the train is stopped inside the reversing area. |  | 9 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: The driver is informed that reversing is possible if the train is stopped inside the reversing area. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.13.1.3 |  |
|  |  | Subset-026-3.15.4.4 |  |
|  |  | Subset-026-3.15.4.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Reversing allowed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The reversing permitted symbol (ST06) is DISPLAYED. |
| Constraints | The reversing permitted symbol (ST06) is displayed to the driver when the following condition are fulfilled: the train is at standstill with the front end of the train inside the reversing area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | D_STARTREVERSE = FINITE_VALUE / L_REVERSEAREA = FINITE VALUE |
| Reversing Supervision Information | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is stopped inside the permitted reversing area | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver is informed that reversing is possible | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The indication to the driver that reversing is possible is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode indication is DISPLAYED. Train speed (v=0) is DISPLAYED. Information about the reversing area is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | O | INT | V_TRAIN=0 / The train is at standstill |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Reversing area information is available on-board and the train is stopped before reaching the reversing area. Reversing is not allowed. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: Selection of reversing is not available to the driver if the train is stopped before reaching the reversing area. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.13.1.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Reversing allowed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The reversing permitted symbol (ST06) is NOT DISPLAYED |
| Constraints | The reversing permitted symbol (ST06) is displayed to the driver when the following condition are fulfilled: the train is located inside the reversing area and the train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | D_STARTREVERSE = FINITE_VALUE / L_REVERSEAREA = FINITE VALUE |
| Reversing Supervision Information | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is stopped before reaching the Distance to start of reversing permitted area (D_STARTREVERSE) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The reversing permitted symbol (ST06) is NOT DISPLAYED | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The reversing permitted symbol (ST06) is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | When the on-board is in Reversing mode a permanent indication of Reversing is displayed. |  | 11 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: The RV mode indication is DISPLAYED. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: RV mode indication is DISPLAYED. |
| Constraints | How the train enters in RV mode does not matter. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1/ L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| RV mode is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | RV mode is DISPLAYED. | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 UNCHANGED | RV |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1/ L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| RV mode is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | When the on-board is in NTC National mode a permanent indication of NTC National is displayed. |  | 12 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: The SN mode indication is DISPLAYED. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SN mode indication is DISPLAYED. |
| Constraints | How the train enters in SN mode does not matter. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SN mode is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SN mode is DISPLAYED. | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 UNCHANGED | SN |
| ERTMS/ETCS level | 1 UNCHANGED | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SN mode is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Indication of Non Leading mode to the driver |  | 13 |
| Applicable Mode/Level Combinations | L0: NL L1: NL L2: NL L3: NL LNTC: NL |  |  |
| Target of Test | The target of this test case is to check that: the NL mode and the train speed are permanently displayed to the driver as long as on-board equipment is in NL mode. Comment: The transitions to and from the NL mode are covered by the test cases of the relevant mode transition features. The present test case aims at the permanent indication of the mode on the DMI display. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.4.15.1.6 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |
|  |  | Subset-026-4.7.2 table - Output information Train Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that NL mode is permanently indicated to the driver as long as in NL mode |
| Constraints | Mode changes to and from NL are not tested in the present feature. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3/ |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NL mode is permanently displayed to the driver | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 UNCHANGED | NL |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 UNCHANGED | L0/ LNTC/ L1/ L2/ L3/ |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Isolation mode is displayed as long as the current ETCS mode is isolation |  | 14 |
| Applicable Mode/Level Combinations | L0: IS L1: IS L2: IS L3: IS LNTC: IS |  |  |
| Target of Test | The target of this test case is to check that: The Isolation mode is displayed to the driver as long as the current ETCS mode is isolation. Note: The indication of the isolated mode is not specified (no impact on interoperability, company specific solution must be accepted). The mode indication is changed as soon as the mode transition to any different mode occurs. These transitions have to be done by a special operating procedure (see Subset-026-4.4.3.1.3), which are not covered by any test case. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.4.3.1.2 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: There is an indication of the isolation mode to the driver (by any means, e.g. the isolation device). |
| Constraints | Mode changes to and from IS are not tested in the present feature. The isolation mode has to be indicated by any means. (tbd. ( For the test has to be defined a indication device, because there are no requirements to the way, how the isolation status has to be shown to the driver.) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 10 | IS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Implementation dependent | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The mode IS is indicated. | O | DMI | The indication is done by any means (e.g. by the isolation device) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 10 UNCHANGED | IS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 UNCHANGED | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Implementation dependent | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | An indication of System Failure (SF) is displayed to the driver as long as the current mode is SF. |  | 15 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / LNTC: SF |  |  |
| Target of Test | The target of this test case is to check that: a clear indication of the System Failure (SF) is shown to the driver while in this mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a permanent indication of SF mode to the driver while in this mode. |
| Constraints | The desk must be open. Mode changes to and from SF are not tested in this feature. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 9 | System Failure (SF) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode (SF) is DISPLAYED. | O | DMI | The on-board equipment has just changed to SF. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The System Failure mode (SF) is permanently indicated to the driver. | O | DMI | Indication of mode SF to the driver while the system is in mode SF. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 9 UNCHANGED | System Failure (SF) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / L1 / L2 / L3 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode (SF) is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | An indication of Post Trip mode (PT) is displayed to the driver as long as the current mode is PT. |  | 16 |
| Applicable Mode/Level Combinations | L1 / L2 / L3: PT |  |  |
| Target of Test | The target of this test case is to check that: - the train Speed is displayed in PT mode - a clear indication of the Post Trip mode (PT) is shown to the driver while in this mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |
|  |  | Subset-026-4.7.2 table - Output information Train Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: There is a permanent indication of PT mode to the driver while in this mode. |
| Constraints | Mode changes to and from PT are not tested in this feature. The desk must be open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | Post Trip (PT) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode (PT) is DISPLAYED. | O | DMI | The on-board equipment has just changed to PT. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment has just changed to PT. The Post Trip mode (PT) is permanently DISPLAYED to the driver. | O | DMI | Indication of mode PT to the driver while the system is in mode PT. | N/A | N/A | - |
| 2 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 UNCHANGED | Post Trip (PT) |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode (PT) is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Indication of Trip mode and the reason to the driver. |  | 17 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / LNTC: TR |  |  |
| Target of Test | The target of this test case is to check that: a clear indication of Trip mode is shown to the driver while in this mode. the reason of the train trip (Trip Reason) is send from on-board equipment to the driver when in Trip mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.4.13.1.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Trip reason |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : there is a permanent indication of TR mode to the driver. the reason of the train trip (Trip Reason) displayed. |
| Constraints | Mode changes to and from TR are not tested in this feature. The desk must be open. . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | Trip (TR) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode (TR) is DISPLAYED, reason (Trip Reason) is DISPLAYED. | O | DMI | The on-board equipment has changed to TR. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Trip mode (TR) is permanently DISPLAYED | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |
| 2 | N/A | N/A | The reason (Trip Reason) is DISPLAYED | O | DMI | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 UNCHANGED | Trip (TR) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode (TR) is DISPLAYED, reason (Trip Reason) is DISPLAYED. | O | DMI | The on-board equipment has changed to TR. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | The on-board equipment is in Stand By mode. Then it has to be checked that the SB mode is permanently displayed to the driver. |  | 18 |
| Applicable Mode/Level Combinations | L0 : SB L1 : SB L2 : SB L3 : SB LNTC: SB |  |  |
| Target of Test | The target of this test case is to check that : The SB mode is indicated to the driver (DMI), Note : The indication of SB mode is displayed on the DMI as soon as a mode transition to SB occurs. These transitions are covered by at least one of the test cases of the relevant mode transition features. The indication of SB mode is deleted from the DMI as soon as a transition from SB to another mode occurs These transitions are covered at least by one of the test cases of the relevant mode transition features. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : SB mode is permanently displayed to the driver |
| Constraints | The desk must be open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 / 1 | L0 / L1 / L2 / L3 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SB mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SB mode is permanently displayed to the driver when the on-board equipment is in SB mode. | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 / 1 UNCHANGED | L0 / L1 / L2 / L3 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SB mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | The on-board equipment is in unfitted mode. Then it has to be checked that the UN mode is permanently displayed to the driver. |  | 19 |
| Applicable Mode/Level Combinations | L0 : UN |  |  |
| Target of Test | The target of this test case is to check that : The UN mode and the train speed are indicated to the driver. Note : The indication of UN mode is displayed on the DMI as soon as a mode transition to UN occurs. These transitions are covered by at least one of the test cases of the relevant mode transition features. The indication of UN mode is deleted from the DMI as soon as a transition from UN to another mode occurs These transitions are covered at least by one of the test cases of the relevant mode transition features. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.4.10.1.4 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |
|  |  | Subset-026-4.7.2 table - Output information Train Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : UN mode is permanently displayed to the driver as long as the on-board equipment is in UN mode. |
| Constraints | The desk must be open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| UN mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | UN mode is permanently DISPLAYED to the driver when the driver is in UN mode. | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |
| 2 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 UNCHANGED | UN |
| ERTMS/ETCS level | 0 UNCHANGED | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| UN mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | When the onboard is in Shunting mode a permanent indication of Shunting is displayed. |  | 20 |
| Applicable Mode/Level Combinations | L0: SH LNTC: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that: The SH mode indication is DISPLAYED. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SH mode indication is DISPLAYED. |
| Constraints | The desk must be open. How the train enters in SH mode does not matter. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1/ L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SH mode is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SH mode is DISPLAYED to the driver. | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 UNCHANGED | SH |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 UNCHANGED | L0 / L1/ L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SH mode is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | An indication of SR is displayed to the driver as long as the current mode is SR. |  | 21 |
| Applicable Mode/Level Combinations | L1 / L2 / L3: SR |  |  |
| Target of Test | The target of this test case is to check that: - a clear indication of the Staff Responsible mode (SR) is shown to the driver while in this mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a permanent indication of SR mode to the driver while in this mode. |
| Constraints | The desk must be open. Mode changes to and from SR are not tested in this feature. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | Staff Responsible (SR) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode (SR) is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Permanent Indication of mode SR to the driver | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 UNCHANGED | SR |
| ERTMS/ETCS level | 2/ 3/ 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Once in On Sight mode, if SSP and gradient are not known for the whole length of the train, an indication “ENTRY IN ON SIGHT” shall be clearly displayed to the driver until SSP and gradient are known for the whole length of the train. |  | 22 |
| Applicable Mode/Level Combinations | L1: OS L2: OS L3: OS |  |  |
| Target of Test | The target of this test case is to check that : the information “entry in On Sight” is displayed to the driver as soon as the estimated front end of the train passes the level transition border location if and only if the SSP & gradient profiles are NOT known for the whole length of the train. the indication “entry in OS” is deleted from the DMI as soon as the train has fully entered the ETCS area covered by the SSP and gradient profiles stored on-board. Note: The indication “entry in On Sight mode” appears on the DMI immediately after a mode transition to OS has occurred. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-4.4.12.1.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Entry in FS/OS |  |
|  |  | Subset-026-3.6.4.6 |  |
|  |  | Subset-026-3.6.4.2.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the information “entry in On Sight” is displayed to the driver as soon as the estimated front end of the train passes the level transition border location if and only if the SSP & gradient profiles are NOT known for the whole length of the train. the indication “entry in On Sight” is deleted from the DMI as soon as the whole train is inside the ETCS area covered by the SSP and gradient profiles stored on-board. Check on the JRU that: indication and deletion of “entry in On Sight” information is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | Packet 80 “Mode Profile” (with variable M_MAMODE = “00” On Sight) is included |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| OS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The system switched to OS mode The message “entry in On Sight” is displayed to the driver. | O | DMI | SSP & gradient profiles are NOT known for the whole length of the train | N/A | N/A | - |
| 2 | N/A | N/A | Indication of “Entry in OS mode” information is RECORDED | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The indication “entry in On Sight” is deleted as soon as the whole train is fully inside the area covered by the track description data . | O | DMI | The train moves a certain distance. | N/A | N/A | - |
| 4 | N/A | N/A | Deletion of “Entry in OS mode” is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 UNCHANGED | OS |
| Movement Authority | STORED UNCHANGED | Packet 80 “Mode Profile” (with variable M_MAMODE = “00” On Sight) is included |
| Gradient Profile | STORED UNCHANGED | - |
| International SSP | STORED UNCHANGED | - |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Indication of “entry in On Sight” mode is DELETED OS Mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | On Sight mode is permanently displayed to the driver when the on-board equipment is in On Sight mode |  | 23 |
| Applicable Mode/Level Combinations | L1: OS L2: OS L3: OS |  |  |
| Target of Test | The target of this test is to check that: There is a permanent indication of the On Sight mode while the system is in On Sight mode |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a permanent indication of the OS mode to the driver while the system is in OS mode. |
| Constraints | The desk must be open. The indication of OS mode is displayed on the DMI as soon as one of the mode transition to OS occurs. These transitions are covered by at least one of the relevant test cases of mode transition features. The indication of OS mode is deleted from the DMI as soon as one of the transition to another mode occurs These transitions are covered at least by one of the relevant test cases of mode transition features. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| OS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Permanent Indication of OS mode to the driver | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 UNCHANGED | OS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Indication of OS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Once in Full Supervision mode, if SSP and gradient are not known for the whole length of the train, an indication “ENTRY IN FULL SUPERVISION” shall be clearly displayed to the driver until SSP and gradient are known for the whole length of the train. |  | 24 |
| Applicable Mode/Level Combinations | L1 : FS L2 : FS L3 : FS |  |  |
| Target of Test | The target of this test case is to check that : the information “Entry in FS” is displayed to the driver as soon as the estimated front end of the train passes the level transition border location if and only if the SSP & gradient profiles are NOT known for the whole length of the train. the indication “Entry in FS” is deleted from the DMI as soon as the train has fully entered the ETCS area covered by the SSP and gradient profiles stored on-board. Note: The indication “Entry in FS mode” appears on the DMI immediately after a mode transition to FS has occurred. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-4.4.9.1.4 |  |
|  |  | Subset-026-4.7.2 table - Output information Entry in FS/OS |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the information “Entry in FS” is displayed to the driver as soon as the estimated front end of the train passes the level transition border location if and only if the SSP & gradient profiles are NOT known for the whole length of the train. the indication “Entry in FS” is deleted from the DMI as soon as the whole train is inside the ETCS area covered by the SSP and gradient profiles stored on-board. Check on the JRU that: indication and deletion of “Entry in FS mode” information is recorded. |
| Constraints | The Gradient & SSP profiles are NOT known for the whole length of the train when the on-board equipment switches to FS mode. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| FS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | FS | L1 / L2 / L3 | "Entry in FS mode" is DISPLAYED to the driver | O | DMI | - | FS | L1 / L2 / L3 | - |
| 2 | FS | L1 / L2 / L3 | Indication of “Entry in FS mode” information is RECORDED | O | JRU | - | FS | L1 / L2 / L3 | - |
| 3 | FS | L1 / L2 / L3 | The indication “Entry in FS” is REMOVED as soon as the whole train is fully inside the area covered by the track description data . | O | DMI | Movement of the train: SSP + gradient are then known for the whole length of the train. | FS | L1 / L2 / L3 | - |
| 4 | FS | L1 / L2 / L3 | Deletion of “Entry in FS mode” information is RECORDED | O | JRU | - | FS | L1 / L2 / L3 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 UNCHANGED | FS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The message “Entry in FS mode” is NOT DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Full Supervision mode is permanently displayed to the driver when the on-board equipment is in Full Supervision mode. |  | 25 |
| Applicable Mode/Level Combinations | L1 : FS L2 : FS L3 : FS |  |  |
| Target of Test | The target of this test case is to check on the DMI that : The FS mode is indicated to the driver Note : The indication of FS mode is displayed on the DMI as soon as one of the mode transition to FS occurs. These transitions are covered by at least one of the relevant test cases of mode transition features. The indication of FS mode is deleted from the DMI as soon as one of the transition to another mode occurs. These transitions are covered at least by one of the relevant test cases of mode transition features. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : FS mode is permanently displayed to the driver as long as the on-board equipment is in FS mode. |
| Constraints | The desk must be open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| FS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | FS mode is permanently displayed to the driver when the on-board equipment is in FS mode. | O | DMI | The mode indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 UNCHANGED | FS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| FS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Indication of level NTC is displayed to the driver while the current level is NTC |  | 26 |
| Applicable Mode/Level Combinations | LNTC : SH / SB / TR / NL / SN |  |  |
| Target of Test | There is a permanent indication of level NTC to the driver while the system is in level NTC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Current ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Select a reasonable number of test cases to prove the feature in NTC level. Check on the DMI that LNTC is displayed |
| Constraints | If in SB or TR mode, the desk is open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 6 / 7 / 11 / 13 | SH / SB / TR / NL / SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LNTC indication is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Level NTC is DISPLAYED | O | DMI | The level indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 6 / 7 / 11 / 13 UNCHANGED | SH / SB / TR / NL / SN |
| ERTMS/ETCS level | 1 UNCHANGED | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LNTC is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Indication of level 3 to the driver is always displayed |  | 27 |
| Applicable Mode/Level Combinations | L3 : FS/ LS /OS/ SR/SH/ SB/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is: - to check that the On-board Equipment shows without interruption to the driver the indication of level 3 always active as output on the DMI screen when the stored variable M_LEVEL is = 4, and it is VALID. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Current ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Select a reasonably reduced set of the features of the feature list relevant for the respective Level and check that the level indication is continuously displayed on the screen of DMI. |
| Constraints | If in SB or TR mode, the desk is open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/6/7/8/11/12/14 | FS/ OS/ SR/SH/ SB/ TR/ PT/ NL/ LS/ RV |
| ERTMS/ETCS level | 4 | L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL INFORMATION DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Level 3 is DISPLAYED | O | DMI | The level indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/6/7/8/11/12/14 UNCHANGED | FS/ OS/ SR/SH/ SB/ TR/ PT/ NL/ LS/ RV |
| ERTMS/ETCS level | 4 UNCHANGED | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 3 information DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | There is a correct cab signalling displayed to the driver when the current mode is RV |  | 28 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: There is a correct cab signalling to the driver in Level 1/2/3 (Permitted speed,Train speed and Target distance always are DISPLAYED) in RV mode |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.18.1.5 |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Train Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Target distance |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a correct cab signalling to the driver while the system is in Level 1/2/3 and RV mode |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Cab Signalling is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | In RV mode and Level 1/2/3 there is a permanent and correct cab signalling displayed which is: Train speed is displayed always Permitted speed is displayed always Target distance is displayed always | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 UNCHANGED | RV |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Cab Signalling is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 29

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Visualisation of the Train Data |  | 29 |
| Applicable Mode/Level Combinations | L0 : UN, SB, TR LNTC: SB, TR, SN L1 : FS, LS, OS, SR, SB, TR, PT, RV L2 : FS, LS, OS, SR, SB, TR, PT, RV L3 : FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that the Onboard Equipment, allows the driver to view the current set of Train Data. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Train Data (refer to 3.18.3.2) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver is allowed to view the existing set of Train Data. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS/ SR / UN / SB / TR / PT / LS / SN / RV |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The on-board equipment authorises the driver to ask for the data view | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "Data view" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The data from "Data View" window are displayed | O | DMI | The current set of Train Data is displayed in the window | N/A | N/A | - |
| 3 | N/A | N/A | The driver closes the "Data View" window | I | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 UNCHANGED | FS/ OS/ SR / UN / SB / TR / PT / LS / SN / RV |
| Train Data | VALID UNCHANGED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The on-board equipment authorises the driver to ask for the data | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 30

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | There is a correct cab signalling displayed to the driver when the current mode is OS |  | 30 |
| Applicable Mode/Level Combinations | L1: OS L2: OS L3: OS |  |  |
| Target of Test | The target of this test is to check that: On Driver's request, there is a correct cab signalling to the driver in Level 1/2/3 (Permitted speed, Target speed, Release speed and Target distance are DISPLAYED upon driver request if and only if a valid MA is stored on-board) in OS mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.12.1.4 |  |
|  |  | Subset-026-4.7.2 table - Output information Train Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Target Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Target distance |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a correct cab signalling to the driver while the system is in Level 1/2/3 and OS mode when the driver request it. Check on JRU that: The driver requests to show and to hide supervision limits are recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | Packet 80 “Mode Profile” (with variable M_MAMODE = “00” On Sight) is included |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Cab Signalling is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Train Speed is displayed. There is no indication to the driver, until request it, of Permitted speed, Target speed, Target distance, Release speed. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests to see the supervision limits in OS mode | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1011) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The correct cab signalling driver request in OS mode is displayed: Train Speed, Permitted speed, Target speed, Target distance, Release speed | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests to hide supervision limits | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1001) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Train Speed is displayed. There is no indication to the driver of Permitted speed, Target speed, Target distance, Release speed | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 UNCHANGED | OS |
| Movement Authority | STORED | Packet 80 “Mode Profile” (with variable M_MAMODE = “00” On Sight) is included |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Cab Signalling is NOT DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 31

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | There is a correct cab signalling displayed to the driver when the current mode is FS |  | 31 |
| Applicable Mode/Level Combinations | L1: FS L2: FS L3: FS |  |  |
| Target of Test | The target of this test is to check that: There is a permanent correct cab signalling to the driver in Level 1/2/3 (Permitted speed, Target speed, Release speed and Target distance are DISPLAYED if and only if a valid MA is stored on-board) in FS mode |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.9.1.6 |  |
|  |  | Subset-026-4.7.2 table - Output information MRSP |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Target Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Target distance |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a correct cab signalling to the driver while the system is in Level 1/2/3 and FS mode |
| Constraints | There must be a speed restriction in front of the train (because the Permitted Speed has to be different from the Target Speed to be distinguished on the DMI). The release speed (V_RELEASEDP or V_RELEASEOL) must be different from 0. The train must be close enough to the end of the MA to allow the display of the target speed and target distance (the must not be in Ceiling Speed Monitoring). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Cab Signalling is DISPLAYED | O | DMI | Permitted speed, target speed, release speed and target distance |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | In FS mode and Level 1/2/3 there is a permanent and correct cab signalling displayed which is: Permitted speed is displayed always Target speed is displayed always Target distance is displayed always Release speed is displayed if the conditions to be displayed are fulfilled | O | DMI | The speed and distance indications may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 UNCHANGED | FS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Cab signalling is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 32

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Indication of Level 1 is displayed to the driver while the current Level is 1 |  | 32 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test is to check that: There is a permanent indication of Level 1 to the driver while the system is in level 1 Note : The indication of level L1 is displayed on the DMI as soon as the level transition from levels L0, LNTC, L2 or L3 to L1 occurs. These transitions are covered by at least one of the test cases of transition features from levels L0, LNTC, L2 or L3 to level L1. The indication of level L1 is deleted from the DMI as soon as the level transition to levels L0, LNTC, L2 or L3 occurs These transitions are covered at least by one test cases of level transition features from L1 to L0, LNTC, L2 or L3. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Current ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is an indication of Level 1 to the driver. |
| Constraints | If in SB or TR mode, the desk is open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SH / SB / TR / PT / NL / LS / RV |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| L1 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Permanent Indication of Level 1 to the driver | O | DMI | The level indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 6 / 7 / 8 / 11 / 12 / 14 UNCHANGED | FS / OS / SR / SH / SB / TR / PT / NL / LS / RV |
| ERTMS/ETCS level | 2 UNCHANGED | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| L1 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 33

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | The on-board equipment has received information about change of traction system. The on-board equipment displays a message to the driver about the change request of traction system. |  | 33 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment displays a message to the driver about the change request of traction system. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Power control |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: A message indicating a change request of traction system is displayed |
| Constraints | Information about change of traction power has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11/ 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | Track condition, change of traction system / D_TRACTION = FINITE VALUE / M_VOLTAGE = FINITE VALUE |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Information of currently used traction power system is AVAILABLE | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Message Change to traction system <M_VOLTAGE> is DISPLAYED | O | DMI | No indication of distance is required | N/A | N/A | - |
| 2 | N/A | N/A | The display of the Message Change Information is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 UNCHANGED | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED UNCHANGED | Track condition, change of traction system / D_TRACTION = FINITE VALUE / M_VOLTAGE = FINITE VALUE |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Condition symbol "M_VOLTAGE" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 34

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | There is no cab signalling (only the current train speed is displayed) to the driver as long as the current level is Level 0 |  | 34 |
| Applicable Mode/Level Combinations | L0: SH / UN / SB / TR / NL |  |  |
| Target of Test | The target of this test is to check that: There is neither Permitted speed (except in Shunting if conditions are fulfilled), nor Target speed, nor Target distance (i.e. no cab signalling) displayed on the DMI as long as the system is in level 0 |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Permitted Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Target Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Target distance |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is NO cab signalling (Permitted speed (except in Shunting if conditions are fulfilled), Target speed and Target distance) displayed to the driver while the system is in Level 0 |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 6 / 7 / 11 | SH / UN / SB / TR / NL |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No cab signalling is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | No cab signalling is displayed to the driver while the system is in Level 0 | O | DMI | No permitted speed (except in SH if conditions are fulfilled), no target speed, no target distance are displayed to the driver. Only the current train speed is displayed. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 6 / 7 / 11 UNCHANGED | SH / UN / SB / TR / NL |
| ERTMS/ETCS level | 0 UNCHANGED | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No cab signalling is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 35

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | An indication of Level 0 is displayed to the driver as long as the current ETCS level is 0 |  | 35 |
| Applicable Mode/Level Combinations | L0: SH / UN / SB / TR / NL |  |  |
| Target of Test | The target of this test is to check that: There is a permanent indication of Level 0 to the driver as long as the system is in level 0 Note : The indication of level L0 is displayed on the DMI as soon as the level transition from levels L1, LNTC, L2 or L3 to L0 occurs. These transitions are covered by at least one of the test cases of transition features from levels L1, LNTC, L2 or L3 to level L0. The indication of level L0 is deleted from the DMI as soon as the level transition to levels L1, LNTC, L2 or L3 occurs. These transitions are covered at least by one test cases of level transition features from L0 to L1, LNTC, L2 or L3. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Current ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is a permanent indication of Level 0 to the driver. |
| Constraints | If in SB or TR mode, the desk is open. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 6 / 7 / 11 | SH / UN / SB / TR / NL |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| L0 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Permanent Indication of Level 0 to the driver | O | DMI | The level indication may not be permanent if the driver uses the menus. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 6 / 7 / 11 UNCHANGED | SH / UN / SB / TR / NL |
| ERTMS/ETCS level | 0 UNCHANGED | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Indication of Level 0 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 36

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Intentionally deleted |  |

## Test case 37

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | The on-board equipment is in SH mode and indicates the permitted speed on the DMI after a driver request. |  | 37 |
| Applicable Mode/Level Combinations | L0, LNTC, L1, L2, L3: SH |  |  |
| Target of Test | The target of this test is to check that : The Train speed is displayed in SH mode The permitted speed in SH is displayed on the DMI after a driver request. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.10 |  |
|  |  | Subset-026-4.7.2 table - Output information Train Speed |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The permitted speed is displayed after a driver request to show supervision limits and hidden after a driver request to hide supervision limits. Check on JRU that: The driver requests to show and to hide supervision limits are recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| National Values | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The permitted speed (V_NVSHUNT) is NOT DISPLAYED | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests to show supervision limits | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver action ‘Request to show supervision limits’ is RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The permitted speed (V_NVSHUNT) is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver requests to hide supervision limits | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver action ‘Request to hide supervision limits’ is RECORDED | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The permitted speed (V_NVSHUNT) is NOT DISPLAYED | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 UNCHANGED | SH |
| National Values | STORED UNCHANGED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The permitted speed (V_NVSHUNT) is NOT DISPLAYED any more. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 38

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | The LSSMA display toggle on order shall toggle off the LSSMA display if it is already toggle on and the delay timer of the new order is not equal to zero. |  | 38 |
| Applicable Mode/Level Combinations | L1: LS L2: LS L3: LS |  |  |
| Target of Test | The target of this test case is to check that: if the LSSMA display if it is toggle on, it will be toggled off after the reception of a toggle on order for which T_LSSMA > 0. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.4.5 |  |
|  |  | Subset-026-4.4.19.1.4.5 a) |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |
|  |  | Subset-026-4.4.19.1.4.3 |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 180 (T_LSSMA > 0) is recorded The Limited Supervision symbol "Limited Supervision frame" is removed After T_LSSMA, the Limited Supervision symbol "Limited Supervision frame" is displayed |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if level is L2/L3 |
| NOT RELEVANT | - | TIU | - |
| LSSMA is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | LS | The LSSMA number is displayed | O | DMI | - | L1/L2/L3 | LS | - |
| 2 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 180 (Q_LSSMA= 1, T_LSSMA>0) is received. | I | BTM | Optional step, only if Level is L1 | L1 | LS | - |
| 3 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Optional step, only if Level is L1 | L1 | LS | - |
| 4 | L2/L3 | LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1, T_LSSMA>0) is received | I | RTM | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 5 | L2/L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 6 | L1/L2/L3 | LS | The LSSMA number is removed | O | DMI | - | L1/L2/L3 | LS | - |
| 7 | L1/L2/L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=0>) is recorded | O | JRU | - | L1/L2/L3 | LS | - |
| 8 | L1/L2/L3 | LS | The LSSMA number is displayed | O | DMI | The on-board pass over the first balise of the BG containing packet 180 since T_LSSMA. | L1/L2/L3 | LS | - |
| 9 | L1/L2/L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1/L2/L3 | LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Value >0 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Value >0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if level is L2/L3 |
| NOT RELEVANT | - | TIU | - |
| LSSMA is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 39

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | On reception of an order to toggle (on or off) the LSSMA display, a toggle on order which has not been executed yet shall be deleted by the on-board equipment. |  | 39 |
| Applicable Mode/Level Combinations | L1: LS L2: LS L3: LS |  |  |
| Target of Test | The target of this test case is to check that: after the reception of an order to toggle (on or off) the LSSMA display, a toggle on order which has not been executed yet (because the on-board delay timer has not reached the delay time-out value) will be deleted by the on-board equipment. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.4.4 |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 180 (T_LSSMA > 0) is recorded Reception of a second packet 180 before T_LSSMA has been reached is recorded At T_LSSMA, the Limited Supervision symbol "Limited Supervision frame" is NOT displayed |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if level is L2/L3 |
| NOT RELEVANT | - | TIU | - |
| LSSMA is NOT displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | LS | Not The LSSMA number is displayed | O | DMI | - | L1/L2/L3 | LS | - |
| 2 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 180 (Q_LSSMA= 1, T_LSSMA>0) is received. | I | BTM | Optional step, only if Level is L1 | L1 | LS | - |
| 3 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Optional step, only if Level is L1 | L1 | LS | - |
| 4 | L2/L3 | LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1, T_LSSMA>0) is received | I | RTM | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 5 | L2/L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 6 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 180 is received. | I | BTM | Optional step, only if Level is L1 Q_LSSMA=0 or T_LSSMA_2 > T_LSSMA | L1 | LS | - |
| 7 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Optional step, only if Level is L1 | L1 | LS | - |
| 8 | L2/L3 | LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 is received | I | RTM | Optional step, only if Level is L2 or L3 Q_LSSMA=0 or T_LSSMA_2 > T_LSSMA | L2/L3 | LS | - |
| 9 | L2/L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 10 | L1/L2/L3 | LS | Not The LSSMA number is displayed | O | DMI | The delay timer started by the reception of packet 180 has reached T_LSSMA. | L1/L2/L3 | LS | - |
| 11 | L1/L2/L3 | LS | Not LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1/L2/L3 | LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Value >0 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Value >0 |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | FINITE VALUE | Q_LSSMA=0 or T_LSSMA_2 > T_LSSMA (step2) |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | Q_LSSMA=0 or T_LSSMA_2 > T_LSSMA (step4) |
| T_LSSMA | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if level is L2/L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 40

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment delete the stored generic LS function marker as soon as a Limited Supervision mode profile is received without packet 181 it in the same balise group message. |  | 40 |
| Applicable Mode/Level Combinations | L1: LS |  |  |
| Target of Test | The target of this test case is to check that: The stored generic LS function marker is deleted by the ERTMS/ETCS on-board equipment as soon as a Limited Supervision mode profile is received without packet 181 it in the same balise group message. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.4.8 |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 80 without packet 180 in BG is recorded The LSSMA number is removed Reception of packet 180 (T_LSSMA = 0) is recorded The LSSMA number is still not displayed |
| Constraints | The target speed at the EOA/LOA is NOT lower than the Limited Supervision mode speed limit, AND/OR the LSSMA is NOT lower than the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LSSMA is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | LS | The LSSMA number is displayed | O | DMI | - | L1 | LS | - |
| 2 | L1 | LS | One Balise Group message containing packet 12, packet 21, packet 27, packet 80 is received. | I | BTM | No packet 181 is present in the BG | L1 | LS | - |
| 3 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | LS | - |
| 4 | L1 | LS | The LSSMA number is removed | O | DMI | - | L1 | LS | - |
| 5 | L1 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=0>) is recorded | O | JRU | - | L1 | LS | - |
| 6 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 180 (Q_LSSMA= 1, T_LSSMA=0) is received. | I | BTM | - | L1 | LS | - |
| 7 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | LS | - |
| 8 | L1 | LS | Not The LSSMA number is displayed | O | DMI | - | L1 | LS | - |
| 9 | L1 | LS | Not DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1 | LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | 0 | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LSSMA is NOT displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 41

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Output |  | 4070201 |
| Test Case of Feature | Display of the LSSMA when the lowest speed is a MRSP element between the minimum safe front end of the train and the EOA/LOA |  | 41 |
| Applicable Mode/Level Combinations | L1: LS L2: LS L3: LS |  |  |
| Target of Test | The target of this test case is to check that the LSSMA is the lowest speed between: - the MRSP element between the minimum safe front end of the train and the EOA/LOA - the target speed at the EOA/LOA |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.4 |  |
|  |  | Subset-026-4.4.19.1.4 a) |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Received packets containing a new mode profile with a V_MAMODE (MRSP speed) < V_LOA, the Generic LS function marker and the order to toggle on the LSSMA display are recorded. The LSSMA display is recorded The updated LSSMA is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if level is L2/L3 |
| NOT RELEVANT | - | TIU | - |
| The LSSMA number is NOT displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | LS | NOT The LSSMA number is displayed | O | DMI | - | L1/L2/L3 | LS | - |
| 2 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 12, packet 80, packet 180 (Q_LSSMA= 1, T_LSSMA=0) and packet 181 is received. | I | BTM | Optional step, only if Level is L1 V_MAMODE < V_LOA | L1 | LS | - |
| 3 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Optional step, only if Level is L1 | L1 | LS | - |
| 4 | L2/L3 | LS | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27, packet 80, packet 180 (Q_LSSMA= 1, T_LSSMA=0) and packet 181 is received. | I | RTM | Optional step, only if Level is L2 or L3 V_MAMODE < V_LOA | L2/L3 | LS | - |
| 5 | L2/L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Optional step, only if Level is L2 or L3 | L2/L3 | LS | - |
| 6 | L1/L2/L3 | LS | The LSSMA number is displayed | O | DMI | LSSMA = V_MAMODE (lowest MRSP between the min safe front end of the train and the EOA/LOA) | L1/L2/L3 | LS | - |
| 7 | L1/L2/L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1/L2/L3 | LS | - |
| 8 | L1/L2/L3 | LS | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1/L2/L3 | LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | > V_MAMODE |
| V_LOA | 7 | FINITE VALUE | > V_MAMODE |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | > V_MAMODE |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | > V_MAMODE |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | < V_LOA |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | 0 | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 181 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | New MA received |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | New MP received |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if level is L2/L3 |
| NOT RELEVANT | - | TIU | - |
| LSSMA is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |