| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3080300 (Structure of a Movement Authority (MA)) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-3080300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 11.11.2013 | All | First version/review | CEDEX |
| Version 2.9.1 20.11.2013 | All | Review | CEDEX |
| Version 2.9.2 11.12.2013 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Structure of a Movement Authority (MA) |  | 3080300 |
| Test Case of Feature | In case the main signal is at danger in level 1, the on-board shall supervise the given distance as the distance to EOA. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of test is: Check that OBU supervises MA distance associated to the main signal at danger in L1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.3.8 |  |
|  |  | Subset-026-5.8.4.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | In order to check the MA distance supervision by the OBU, it is firstly necessary to avoid the entry in TR associated to the main signal at danger. To do that, the MA shall be announced by an in-fill BG. The train will approach to the main signal and shall stop beforeoverpassing the EoA. The driver shall select an Override EoA before overpassing the EoA and shall move the train. The Trip inhibition shall be deactivated once the stored EoA is overpassed with the minimum safe antenna position. Check on the JRU: - The in-fill MA is stored. - The new Target Distance is recorded (only in FS mode). - The Driver action "Select Override EoA" is recorded. - The DMI Symbol "Override EoA is active" display is recorded. - The DMI Symbol "Override EoA is active" removal is recorded at a distance corresponding to the minimum safe antenna overpassing the EoA. Check on the DMI: - The update of the MA when reading the in-fill information (only in FS mode). - The removal of the DMI symbol "Override EoA is active" at a distance corresponding to the minimum safe antenna overpassing the EoA. |
| Constraints | The MA will be given with a Release Speed different from zero, in order to let the train get closer to the main BG. In order to ensure the deactivation of the Override EoA is produced because of the overpassing of the former EoA, this EoA shall be placed at a distance lower than the D_NVOVTRP value. The movement from the Driver selection of Override EoA to overpassing the EoA will take less time than T_NVOVTRP. No message from the main BG will be sent. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Signalling related Speed Restriction | STORED | - |
| Mode Profile | STORED | Only in LS mode |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS, LS | Train is moving | I | INT | - | L1 | FS, LS | - |
| 2 | L1 | FS, LS | One Balise Group message (N_TOTAL=1) containing packet 136, packet 12, packet 21 and packet 27 is received | I | BTM | The in-fill MA indicates the next signal is at danger. | L1 | FS, LS | - |
| 3 | L1 | FS, LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS, LS | - |
| 4 | L1 | FS, LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode. The new EoA is displayed. | L1 | FS, LS | - |
| 5 | L1 | FS, LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Only if in FS mode. The variable D_TARGET corresponds with the new EoA. | L1 | FS, LS | - |
| 6 | L1 | FS, LS | Train is at standstill | I | INT | The train stops close to the EoA. The main BG will not be read. | L1 | FS, LS | - |
| 7 | L1 | FS, LS | The driver selects "Override EoA" | I | DMI | - | L1 | SR | - |
| 8 | L1 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | M_DRIVERACTIONS = 0000 1110 | L1 | SR | - |
| 9 | L1 | SR | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | L1 | SR | - |
| 10 | L1 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L1 | SR | - |
| 11 | L1 | SR | Train is moving | I | INT | The speed of the train shall be lower than V_NVSUPOVTRP | L1 | SR | - |
| 12 | L1 | SR | Train is moving | I | INT | The minimum safe antenna position of the train overpasses the EoA. | L1 | SR | - |
| 13 | L1 | SR | The Mode symbol "Override EOA is active" is removed | O | DMI | - | L1 | SR | - |
| 14 | L1 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=0>) is recorded | O | JRU | - | L1 | SR | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Corresponds to the main BG. |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | 0 | Signal at danger. |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | This value has to be lower than D_NVOVTRP. |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | 1 | The danger point shall be included together with a release speed > 0 in order to let the train come closer to the BG at the main signal. |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | The value shall be > 0. |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |