| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5180200 (5.18.2: Passing a powerless section with pantograph to be lowered) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5180200 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 21.08.2013 | All | Answer to Review003_Subset-076-5-2-5180200_v291 | MULTITEL |
| Version 2.9.3 27.11.2013 | All | Answer to Review001_Subset-076-5-2-5180200_v292 | MULTITEL |
| Version 2.9.4 13.12.2013 | TC1 | Add the requirement “4.5.2.1 table - Determine train position referenced to LRBG” | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.2: Passing a powerless section with pantograph to be lowered |  | 5180200 |
| Test Case of Feature | The on-board equipment has received track condition information indicating the location of a powerless section where lowering of the pantograph is required. The on-board equipment displays to the driver a profile indicating the location where lowering of the pantograph is required and where raising of the pantograph is permitted. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check that : -The on-board equipment displays to the driver a state of pantograph profile according to the received track condition information -The on-board equipment updates the state of pantograph profile according to train location. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.12.1.2.1.3 |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.5.2.1 table - Determine train position referenced to LRBG |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Pantograph control |  |
|  |  | Subset-026-5.18.2.2 |  |
|  |  | Subset-026-5.18.2.2.1 |  |
|  |  | Subset-026-5.18.2.2.2 |  |
|  |  | Subset-026-5.18.2.2.2 a) |  |
|  |  | Subset-026-5.18.2.2.2 b) |  |
|  |  | Subset-026-5.18.2.3 |  |
|  |  | Subset-026-5.18.2.3 a) |  |
|  |  | Subset-026-5.18.2.3 b) |  |
|  |  | Subset-026-5.18.2.5 |  |
|  |  | Subset-026-5.18.2.5 a) |  |
|  |  | Subset-026-5.18.2.5 b) |  |
|  |  | Subset-026-5.18.2.5.1 |  |
|  |  | Subset-026-5.18.2.5.1 a) |  |
|  |  | Subset-026-5.18.2.5.1 b) |  |
|  |  | Subset-026-5.18.2.6 |  |
|  |  | Subset-026-A.3.1 - Time between minimum safe rear end of the train leaving a track condition area and on-board deleting the applicable indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI/JRU that: The state of pantograph profile is displayed according to the received track condition information. |
| Constraints | The pantograph status is raised before the TC begins. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND=FINITE VALUE, Location of the powerless section L_TRACKCOND=FINITE VALUE, Length of the powerless section M_TRACKCOND=3, Powerless section – lower pantograph |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Lower pantograph announcement” is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) -automatically (symbol TC02) -manually (the driver is requested to act) (symbol TC03) | O | DMI | When the max safe front end of the train reaches a location in rear of the beginning of the powerless section, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit45=1¦Bit46=1>) is recorded | O | JRU | TC02 (automatic execution) or TC03 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | “Lower pantograph announcement” (symbol TC02 or symbol TC03) is REMOVED. | O | DMI | As soon as the train reaches the start location of the powerless section with its max safe front end ( = D_TRACKCOND ) | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit45=0&Bit46=0>) is recorded | O | JRU | TC02 (automatic execution) TC03 (driver requested to act) | N/A | N/A | - |
| 5 | N/A | N/A | “Lowered pantograph” (symbol TC01) is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit44=1>) is recorded | O | JRU | TC01 | N/A | N/A | - |
| 7 | N/A | N/A | ”Lowered Pantograph” (symbol TC01) is REMOVED. | O | DMI | As soon as the train reaches the end location of the powerless section with its min safe front end. ( = D_TRACKCOND + L_ TRACKCOND ) | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit44=0>) is recorded | O | JRU | TC01 | N/A | N/A | - |
| 9 | N/A | N/A | “Raise Pantograph” is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) -Automatically (TC04) -Manually (the driver is requested to act) (TC05) | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit47=1¦Bit48=1>) is recorded | O | JRU | TC04 (automatic execution) or TC05 (driver requested to act) | N/A | N/A | - |
| 11 | N/A | N/A | “Raise pantograph” (symbol TC04 or symbol TC05) is REMOVED | O | DMI | The “Raise pantograph” information shall remain displayed for a fixed time (see Appendix A3.1, 5s) after the train reached the min safe rear end of the end location of powerless section. | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit47=0&Bit48=0>) is recorded | O | JRU | TC04 (automatic execution) TC05 (driver requested to act) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |