| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5180600 (5.18.6 Passing an “air tightness” area) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5180600 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 02.12.2013 | All | First version/review | MULTITEL |
| Version 2.9.1 13.01.2014 | All | Apply of issues from Review1.0_Subset-076-5-2-5180600 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.6 Passing an “air tightness” area |  | 5180600 |
| Test Case of Feature | The on-board equipment has received track condition information indicating the location of an air tightness section where is "Closing air conditioning intake" required. The on-board equipment displays to the driver a profile indicating the location where "Closing air conditioning intake" is required and where "Opening air conditioning intake" is permitted. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check that: - The on-board equipment displays to the driver the “Close air conditioning intake announcement” information according to the received track condition information - The on-board equipment updates the “Close air conditioning intake announcement” information according to train location. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.12.1.2.1.3 |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Air tightness control |  |
|  |  | Subset-026-5.18.6.2 |  |
|  |  | Subset-026-5.18.6.2.1 |  |
|  |  | Subset-026-5.18.6.2.2 |  |
|  |  | Subset-026-5.18.6.2.2 a) |  |
|  |  | Subset-026-5.18.6.2.2 b) |  |
|  |  | Subset-026-5.18.6.3 |  |
|  |  | Subset-026-5.18.6.3 a) |  |
|  |  | Subset-026-5.18.6.3 b) |  |
|  |  | Subset-026-5.18.6.4 |  |
|  |  | Subset-026-5.18.6.4 a) |  |
|  |  | Subset-026-5.18.6.4 b) |  |
|  |  | Subset-026-5.18.6.4.1 |  |
|  |  | Subset-026-5.18.6.4.1 a) |  |
|  |  | Subset-026-5.18.6.4.1 b) |  |
|  |  | Subset-026-5.18.6.5 |  |
|  |  | Subset-026-A.3.1 - Time between minimum safe rear end of the train leaving a track condition area and on-board deleting the applicable indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: - “Close air conditioning intake announcement” information is displayed to the driver - “Air conditioning intake closed” information is updated according to train location - “Open air conditioning intake” information is displayed to the driver Check on JRU that: The symbols for: “Close air conditioning intake announcement”, “Air conditioning intake closed”, “Open air conditioning intake” are recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND=FINITE VALUE, Location of the Air tightness section L_TRACKCOND=FINITE VALUE, Length of the Air tightness section M_TRACKCOND=0101 Air tightness |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Close air conditioning intake announcement” is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) automatically (symbol TC19) or manually (the driver is requested to act) (symbol TC21) | O | DMI | When the max safe front end of the train reaches a location in rear of the beginning of the “air tightness” area, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=1¦Bit64=1>) is recorded | O | JRU | TC19 (automatic execution) or TC21 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | "Air conditioning intake closed" replaces “Close air conditioning intake announcement” (symbol TC19 or TC21 is replaced by symbol TC19). | O | DMI | As soon as the train reaches the start location of the air tightness section with its max safe front end ( = D_TRACKCOND ) If the related functionality is executed (application dependent) manually; otherwise, the symbols are identical | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=1&Bit64=0>) is recorded | O | JRU | If the related functionality is executed (application dependent) manually; otherwise, the symbols are identical | N/A | N/A | - |
| 5 | N/A | N/A | "Air conditioning intake closed" is REMOVED | O | DMI | As soon as the train reaches the end location of the air tightness section with its min safe rear end. | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | “Open air conditioning intake” is DISPLAYED It is DISPLAYED if the related functionality is executed (application dependent) automatically (symbol TC20) or manually (the driver is requested to act) (symbol TC22) | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit63=1¦Bit65=1) is recorded | O | JRU | TC20 (automatic execution) or TC22 (driver requested to act) | N/A | N/A | - |
| 9 | N/A | N/A | “Open air conditioning intake” is REMOVED | O | DMI | The “Open Air Conditioning intake” information shall remain displayed for a fixed time (5s, see Appendix A3.1) after the minimum safe rear end of the train has passed the end of the “Air tightness area”. | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit63=0&Bit65=0) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |