| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3110500 (3.11.5: Temporary Speed Restrictions) Total: 12 Test case(s) |  |
| REF: | Subset-076-5-2-3110500 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO/CEDEX/MULTITEL |
| Version 2.9.1 24.06.2013 | All | Second version/ Format of Template updated: Dictionary steps used; UNISIG first review comments included. | INECO |
| Version 2.9.2 01.08.2013 | All | Third version/Internal modification;TC_Editorv27; Correction of Requirements blank space; Renumbering | INECO |
| Version 2.9.3 17.10.2013 | All | Fourth version/ UNISIG comments from 28.07.2013 included/ Overspeed issue included. | INECO |
| Version 2.9.4 09.12.2013 | All | Fifth version/ UNISIG comments from 05.12.2013 included | INECO |
| Version 2.9.5 26.12.2013 | All | Sixth version/ Internal version: cross-reference checked; old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | Train receives two different Temporary Speed Restrictions with overlap. Permitted speed for both TSR-s < maximum train speed and any other stored static speed restriction. |  | 1 |
| Applicable Mode/Level Combinations | L0 : UN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment receives from balises group two temporary speed restrictions that overlaps. The most restrictive TSR will be shown to driver as permitted speed if train is in overlap The function “Determine Most Restrictive Speed Profile based on TSR” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.4 |  |
|  |  | Subset-026-4.5.2.1 table - TSR |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The correct permitted speed (received as TSR) has to be shown on the DMI according to combined TSR. (If in FS, OS or SR mode) For LS mode the DMI aspect coherent with Overspeed Status (train speed a bit over the most restrictive TSR speed) is displayed. The speed and distance monitoring information is displayed (Intervention status) Status symbol “ST01” is displayed and removed Check on the JRU that : Exceeding of speed limit is recorded. Service brake commanded state (commanded and release) is recorded. Check on the TIU that: Service brake commanded and released. |
| Constraints | Two TSR´s with overlap area are received by on-board equipment. V_TSR (B) < V_TSR (A) V_TRAIN shall be increased when both TSR´s area ends, to check that the delay of the train is taken into account. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | Only in L2/L3 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TRAIN |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1) containing packet 65 is received . | I | BTM | Temporary Speed Restriction (1) NID_TSR = A D_TSR (A) L_TSR (A) Q_FRONT =0 V_TSR (A) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Balise Group message (N_TOTAL=1) containing packet 65 is received . | I | BTM | Temporary Speed Restriction (2) NID_TSR = B D_TSR (B) > D_TSR (A) D_TSR (B) +L_TSR (B) < D_TSR (A) + L_TSR (A) Q_FRONT= 1 V_TSR (B) < V_TSR (A) | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Train is moving. | I | INT | The train runs in rear of the first TSR area. | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Maximum train speed or any lower static speed restriction will be DISPLAYED to driver on DMI as maximum permitted speed For OS/SR mode only on driver request. | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20;; M_SDMSUPSTAT=0) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving. | I | INT | The train enters the area defined in the first TSR. | N/A | N/A | - |
| 9 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Optional step. Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 10 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR Permitted Speed is updated with the value of V_TSR(A) V_TRAIN < V_TSR (A) CSM - Normal Status- V_TSR(A) If in LS mode train speed will be a bit over the V_TSR(A). The DMI aspect is Overspeed Status. | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A Km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: V_PERM (Permitted speed displayed to the driver) value must be V_TSR(A) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |
| 13 | N/A | N/A | Train is moving. | I | INT | The train drives into TSR overlap area where both TSR´s are valid. | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR , Permitted Speed is updated with the value of V_TSR(B) ( V_TRAIN < V_TSR(B) CSM - Normal Status- V_TSR(B) If in LS mode train speed is displayed. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0 ) is recorded | O | JRU | V_PERM (Permitted speed displayed to the driver) value must be V_TSR(B) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 16 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_ TRAIN >V_ TSR(B) Status requested: Intervention status | N/A | N/A | - |
| 17 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= B km/h ; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 18 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 19 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 21 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 22 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_TSR(B) Status requested: Normal status | N/A | N/A | - |
| 23 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 24 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 25 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 26 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 27 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 28 | N/A | N/A | Train is moving. | I | INT | The front of train passes end of TSR overlap area. | N/A | N/A | - |
| 29 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode speed value defined in first TSR(A) will be DISPLAYED to driver on DMI as maximum permitted speed CSM – Normal Status- V_TSR(A) If in LS mode train speed will be a bit over the V_TSR(A). The DMI aspect is Overspeed Status. | N/A | N/A | - |
| 30 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: V_PERM (Permitted speed displayed to the driver) value must be V_TSR(A) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) | N/A | N/A | - |
| 31 | N/A | N/A | Train is moving. | I | INT | The Min safe rear end passes the point defined as end of TSR area. | N/A | N/A | - |
| 32 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode maximum train speed or any lower static speed restriction will be displayed to driver on DMI as maximum permitted speed CSM – Normal Status- actual Permitted speed If in LS the DMI aspect changes to Normal Status. | N/A | N/A | - |
| 33 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | 01 | Nominal |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR(A) | 8 | FINITE VALUE | - |
| D_TSR(A) | 15 | FINITE VALUE | - |
| L_TSR(A) | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| V_TSR(A) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | 01 | Nominal |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR(B | 8 | FINITE VALUE | - |
| D_TSR(B) | 15 | FINITE VALUE | - |
| L_TSR(B) | 15 | FINITE VALUE | < L_TSR(A) |
| Q_FRONT | 1 | 1 | - |
| V_TSR(B) | 7 | FINITE VALUE | < V_TSR (A) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | Only in L2/L3 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Supervision Status: Normal Status If in FS, OS or SR mode the speed value that is displayed on the DMI is a permitted speed equal to the maximum train speed or any lower static speed restriction (not the TSR). |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | Actual Permitted speed |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | An update for current TSR is transmitted via balise group and is received by the on-board ERTMS/ETCS System, & older TSR is revocable. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The ERTMS/ETCS on-board equipment accepts the new received TSR ( V_TSR(B)) The on-board equipment replaces the previous revocable TSR (A) with TSR (B) The permitted speed is updated to V_TSR(B) The function “Determine Most Restrictive Speed Profile based on TSR” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.9 |  |
|  |  | Subset-026-4.4.10.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - TSR |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Permitted Speed is UPDATED (If in FS, OS or SR mode). The speed and distance monitoring information is displayed (Intervention status) Status symbol “ST01” is displayed and removed Check on the JRU that: Reception of a Balise telegram including Packet Number 65: “Temporary Speed Restriction” is RECORDED. Exceeding of speed limit is recorded. Service brake commanded state (commanded and release) is recorded. Check on the TIU that: Service brake commanded and released. |
| Constraints | The train has previously received a TSR (A). The on-board equipment receives a new Temporary Speed Restriction (TSR B) with the same identity as an already received TSR (A). New TSR will be calculated such that no brake intervention is needed due to change of Permitted Speed, this is: new V_TSR > old V_TSR. The new Temporary Speed Restriction shall replace the previous one because TSR (A) is revocable. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. (The onboard equipment could not have Linking information). |
| Temporary Speed Restrictions | STORED | First TSR: Packet 65: Temporary Speed Restriction (1) NID_TSR (A) ≠ 255 D_TSR (A) L_TSR (A) Q_FRONT = 0 V_TSR (A) |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | Only in L2/L3 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TRAIN |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1) containing packet 65 is received. | I | BTM | NID_TSR(B) = NID_TSR(A) D_TSR (B) L_TSR (B) Q_FRONT = 0 V_TSR (B) > V_TSR (A) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Optional Step: Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional Step: Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_TSR (B) CSM - Normal Status If in FS, OS or SR mode Permitted Speed is updated with the value of V_TSR(B) CSM - Normal Status- V_TSR(B) | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in FS/OS/SR: V_PERM (Permitted speed value) must be V_TSR(B) | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED+10>) | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Intervention status, so V_TSR (B) < V_TRAIN | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 10 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TSR (B) > V_TRAIN CSM - Normal Status | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 17 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 19 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | 01 | Nominal |
| L_PACKET | 13 | 71 | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR(B) | 8 | FINITE VALUE | Same identity as the stored one NID_TSR(A) |
| D_TSR(B) | 15 | 0 | TSR valid since now |
| L_TSR(B) | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| V_TSR(B) | 7 | FINITE VALUE | > V_TSR (A) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. (The onboard equipment could not have Linking information). |
| Temporary Speed Restrictions | STORED | Second TSR: Packet 65: Temporary Speed Restriction (1) NID_TSR = B D_TSR (B) L_TSR (B) Q_FRONT =0 V_TSR (B) |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | Only in L2/L3 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Supervision Status: Normal Status If in FS, OS or SR mode V_PERMITTED = V_TSR (B) |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | Supervision of Temporary Speed Restriction by the on-board equipment. An update for current TSR is transmitted via balise group and is received by the on-board ERTMS/ETCS System, & older TSR is non-revocable. The new TSR is considered as an additional TSR. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The ERTMS/ETCS on-board equipment accepts the new received TSR (V_TSR (B)) inside of TSR (1). The on-board equipment keeps older TSR (A) because it is marked as non-revocable and considers the received TSR (B) as additional TSR. Inside of overlap area, the permitted speed is not updated to V_TSR (B). When the full train length has left the area of the first TSR, the permitted speed is updated to V_TSR (B). The function “Determine Most Restrictive Speed Profile based on TSR” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.9 |  |
|  |  | Subset-026-4.5.2.1 table - TSR |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed Restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Permitted Speed is NOT UPDATED to V_TSR (B) when inside the overlap area of Both TSR´s (If in FS, OS or SR mode).The speed and distance monitoring information is displayed (Intervention status) Status symbol “ST01” is displayed and removed Permitted Speed is UPDATED to V_TSR (B) when the full length of the train inside L_TSR (B) (If in FS, OS or SR mode). For LS mode the DMI aspect coherent with Overspeed Status (train speed a bit over the V_TSR(B) speed) is displayed. Check on the JRU that: Balise telegram including Packet Number 65: “Temporary Speed Restriction” is RECORDED. Exceeding of speed limit is recorded. Service brake commanded state (commanded and release) is recorded. Check on the TIU that: Service brake commanded and released. |
| Constraints | The train has previously received a TSR and is using it (TSR (A)). The on-board equipment receives a new Temporary Speed Restriction (TSR (B)) with the same identity as an already received TSR (A). Both temporary speed restrictions should overlap each other. New TSR will be calculated such that no brake intervention is needed due to change of Permitted Speed, this is: new V_TSR > old V_TSR. The new Temporary Speed Restriction shall replace the previous one, except when the Temporary Speed Restriction is identified as non-revocable. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. (The onboard equipment could not have Linking information). |
| Temporary Speed Restrictions | STORED | Packet 65: Temporary Speed Restriction (1) NID_TSR (A) = 255 (Non-Revocable) D_TSR (A) L_TSR (A) Q_FRONT=0 V_TSR (A) |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | Only in L2/L3 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TSR(A) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1) containing packet 65 is received | I | BTM | BGa must be located before L_TSR of the old TSR expires. TSR (2) with the same identity as an already received TSR (1) and V_TSR (B) > V_TSR (A) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | Train location : Inside overlap area of both TSRs. | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional Step: Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional Step: Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Check that the first Temporary Speed Restriction is taken into account for the train speed supervision. V_TSR(A) > V_TRAIN CSM - Normal Status If in FS, OS or SR mode Permitted Speed is the value of V_TSR(A) and NOT V_TSR (B) CSM - Normal Status- V_TSR(A) | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in FS/OS/SR: V_PERM (Permitted speed value must be V_TSR(A)) | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TSR(A) < V_TRAIN Status requested: Intervention status | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h ;M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 10 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_TSR(A) Status requested: Normal status | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 17 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 19 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | Train is moving | I | INT | Train location: Full train length has left TSR (A) area. | N/A | N/A | - |
| 21 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Check that the second Temporary Speed Restriction is taken into account for the train speed supervision. If in FS, OS or SR mode Permitted Speed is the value of V_TSR(B) CSM - Normal Status- V_TSR(B) If in LS mode train speed will be a bit over the V_TSR(B). The DMI aspect is Overspeed Status. | N/A | N/A | - |
| 22 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_TSR(B) Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 8 | 01 | Nominal |
| L_PACKET | 2 | 71 | - |
| Q_SCALE | 13 | FINITE VALUE | - |
| NID_TSR | 2 | 255 | Same as previously received one |
| D_TSR | 8 | 0 | TSR valid since now |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 15 | 0 | - |
| V_TSR | 1 | FINITE VALUE | Greater than previously received one |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 /8 /12 | FS / OS / SR / UN / TR /PT / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. (The onboard equipment could not have Linking information). |
| Temporary Speed Restrictions | STORED | There are two TSRs stored: Packet 65: Temporary Speed Restriction (1) NID_TSR (1) (NON-Revocable) D_TSR (A) L_TSR (A) Q_FRONT =0 V_TSR (A) Packet 65: Temporary Speed Restriction (2) NID_TSR (2) (NON-Revocable) D_TSR (B) L_TSR (B) Q_FRONT =0 V_TSR (B) |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | Only in L2 / L3 |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode V_PERMITTED = V_TSR (B) Supervision Status: Normal Status If in LS mode Supervision Status: Overspeed Status. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | The Temporary Speed Restrictions shall only be revoked on request from the trackside. Reception and acceptance of Temporary Speed Restriction Revocation (packet 66) by on-board if Level 2 or Level 3 is active. |  | 4 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that: On-board equipment receives and accepts the packet 66 (transmitted in General Radio Message 24) and On-board equipment revokes the stored temporary speed restriction correctly without delay for the train length (the stored Temporary Speed Restriction is revocable). The function “Delete Revoked TSR” is active on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.5 |  |
|  |  | Subset-026-3.11.5.8 |  |
|  |  | Subset-026-4.5.2.1 table - Delete revoked TSR |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The permitted speed changes to less restrictive value after revocation of TSR (for the location of TSR) (If in FS, OS or SR mode). For LS mode the DMI aspect coherent with Normal Status (train speed greater than revoked V_TSR) is displayed. Check on the JRU that: Radio Message 24 with packet 66 is recorded |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable. The revocation of the TSR shall take place immediately without considering the delay of train length. The “ACK of Train data” has been previously received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | First TSR: Packet 65: Temporary Speed Restriction (1) NID_TSR = A ≠ 1111 1111 FINITE VALUE revocable and send by RBC, that is value 127...253 V_TSR Q_FRONT=0 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TSR |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 66 is received | I | RTM | Reception of TSR Revocation order NID_TSR = NID_TSR (A) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | The train is located in the area of TSR, given in packet 65. | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is less restrictive immediately after the revocation of the TSR, i.e., the PS is higher than the previously received value of V_TSR. If in LS mode train speed will be a bit over the V_TSR. The DMI aspect is Normal Status. | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <other than V_TSR>; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 66 | Temporary Speed Restriction Revocation |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked (Identity of stored TSR) NID_TSR = NID_TSR (A) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR actual permitted train speed but not V_TSR. If in LS mode V_TRAIN > V_TSR. Status Normal. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | The Temporary Speed Restrictions shall only be revoked on request from the trackside. Reception and acceptance of Temporary Speed Restriction Revocation (packet 66) by on-board. |  | 5 |
| Applicable Mode/Level Combinations | L0: UN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment receives and accepts the packet 66 (transmitted by balise) and The on-board equipment revokes immediately the stored temporary speed restriction correctly without delay for the train length The function “Delete Revoked TSR” is active on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.5 |  |
|  |  | Subset-026-3.11.5.8 |  |
|  |  | Subset-026-4.5.2.1 table - Delete revoked TSR |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The permitted speed changes to less restrictive value after revocation of TSR (for the location of TSR) (If in FS, OS or SR mode). For LS mode the DMI aspect coherent with Normal Status (train speed greater than revoked V_TSR) is displayed. Check on the JRU that: Balise telegram with packet 66 is recorded. |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Temporary Speed Restrictions | STORED | First TSR: Packet 65: Temporary Speed Restriction (1) NID_TSR = A revocable and send by non-RBC that are values 0....126 Q_FRONT=0 V_TSR |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TSR |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1) containing packet 66 (Temporary Speed Restriction Revocation) is received. | I | BTM | NID_TSR = NID_TSR (A) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Reception of TSR Revocation order is attested | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | The train is located in the area of TSR, given in packet 65. | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is less restrictive immediately after the revocation of the TSR, i.e., the PS is higher than the previously received value of V_TSR. If in LS mode train speed will be a bit over the V_TSR. The DMI aspect is Normal Status. | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= < other than V_TSR>; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 000 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 66 | Temporary Speed Restriction Revocation |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked. = NID_TSR (A) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Temporary Speed Restrictions | NOT STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR actual permitted train speed but not V_TSR. If in LS mode V_TRAIN > V_TSR. Status Normal. |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | A new Temporary Speed Restriction shall not replace a previously received Temporary Speed Restriction with another identity. |  | 6 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment receives a radio message 24 from the RBC containing a temporary speed restriction (packet 65) with a different identifier which partially overlaps the previous one. Both temporary speed restrictions are taken into account by the on-board equipment for the train speed supervision. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.4 |  |
|  |  | Subset-026-3.11.5.7 |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed restriction |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The displayed permitted speed indicates that both temporary speed restrictions are taken into account. (If in FS, OS or SR mode). The speed and distance monitoring information is displayed (Intervention status) Status symbol “ST01” is displayed and removed For LS mode the DMI aspect coherent with Overspeed Status (train speed greater than the speed define by TSR(B)) is displayed.. Check on the JRU that: The radio message 24 “general message” with packet 65 “temporary speed restriction” is recorded. Exceeding of speed limit is recorded. Service brake commanded state (commanded and release) is recorded. Check on the TIU that: Service brake commanded and released. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 /12 | FS/ OS/ SR/ SB/ TR/ PT/ LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | First TSR: Packet 65: Temporary Speed Restriction (1) NID_TSR = A ≠ 255 FINITE VALUE, revocable D_TSR (A) L_TSR (A) Q_FRONT =0 V_TSR (A) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TSR(A) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received | I | RTM | NID_TSR (B) ≠ NID_TSR (A) D_TSR(B) < D_TSR (A) + L_TSR(A) V_TSR (B) > V_TSR (A) This TSR partially overlaps the first one, and only the most restrictive speed is taken into account in the overlapped area. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train location: Inside overlap area | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Check that the first Temporary Speed Restriction is taken into account for the train speed supervision No modification on permitted speed value occurred | N/A | N/A | - |
| 7 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | V_TSR(A) is still taken into account and speed and distance monitoring information has not changed. | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN > V_TSR(A) Status requested: Intervention status | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 10 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_TSR(A) Status requested: Normal status | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 17 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 19 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | Train is moving | I | INT | Train location: Full train length has left TSR (A) area. | N/A | N/A | - |
| 21 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Check that the second Temporary Speed Restriction is taken into account for the train speed supervision. If in LS mode train speed will be a bit over the V_TSR(B). The DMI aspect is Overspeed Status. | N/A | N/A | - |
| 22 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20, V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_TSR(B) Only if in LS mode :M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction (Second TSR) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | NID_TSR(B) ≠ NID_TSR (A) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | V_TSR (B) > V_TSR (A) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 /12 | FS/ OS/ SR/ SB/ TR/ PT |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | Second TSR: Packet 65: Temporary Speed Restriction (2) NID_TSR = B ≠ 255 FINITE VALUE revocable D_TSR (B) L_TSR (B) V_TSR (B) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR actual permitted train speed V_TSR(B). If in LS mode V_TRAIN > V_TSR (B). Overspeed status. |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | An update for current TSR is transmitted via RBC and is received by the on-board ERTMS/ETCS System, & older TSR is revocable. |  | 7 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment receives a radio message 24 from the RBC containing a temporary speed restriction (packet 65) with the same identifier. Only the last temporary speed restriction is taken into account by the on-board equipment for the train speed supervision. The previous revocable temporary speed restriction is replaced by the last received one. The function “Determine Most Restrictive Speed Profile based on TSR” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.7 |  |
|  |  | Subset-026-3.11.5.9 |  |
|  |  | Subset-026-4.5.2.1 table - TSR |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed restriction |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The displayed permitted speed indicates that only the last temporary speed restriction is taken into account. (If in FS, OS or SR mode). The speed and distance monitoring information is displayed (Intervention status) Status symbol “ST01” is displayed and removed The DMI aspect coherent with Normal Status is displayed. Check on the JRU that: The radio message 24 “general message” with packet 65 “temporary speed restriction” is recorded. Exceeding of speed limit is recorded. Service brake commanded state (commanded and release) is recorded. Check on the TIU that: Service brake commanded and released. |
| Constraints | The train has previously received a TSR (A). The on-board equipment receives a new Temporary Speed Restriction (TSR B) with the same identity as an already received TSR (A). New TSR will be calculated such that no brake intervention is needed due to change of Permitted Speed, this is: new V_TSR > old V_TSR. The new Temporary Speed Restriction shall replace the previous one because TSR (A) is revocable. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS/ OS/ SR / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | First TSR: Packet 65: Temporary Speed Restriction (1) NID_TSR = A ≠ 1111 1111 FINITE VALUE revocable and send by RBC, that is value 127...253 D_TSR (A) L_TSR (A) V_TSR (A) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TSR(A) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received | I | RTM | NID_TSR ( B) = NID_TSR (A) V_TSR (B) > V_TSR (A) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | CSM - Normal Status- V_TSR(B) If in FS, OS or SR Permitted Speed is updated with the value of V_TSR(B) V_TRAIN < V_TSR (B) | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving. | I | INT | Train location: Inside TSR area | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Intervention status, so V_TSR(B) < V_TRAIN | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 10 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_TSR(B) Status requested: Normal status If in FS ,OS or SR mode: Indication of permitted speed as V_TSR(B) If in LS mode V_TRAIN is displayed. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 17 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 19 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | 127-253 = NID_TSR(A) Temporary Speed Restriction from RBC (revocable) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | Temporary Speed Restriction NID_TSR = B ≠ 1111 1111 FINITE VALUE revocable and send by RBC, that is value 127...253 D_TSR (B) L_TSR (B) V_TSR (B) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_TSR (B). Normal status. If in FS, OS or SR actual permitted train speed V_TSR(B). In LS permitted speed is not displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | An update for current TSR is transmitted via RBC and is received by the on-board ERTMS/ETCS System, & older TSR is non-revocable. The new TSR is considered as an additional TSR. |  | 8 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment receives a radio message 24 from the RBC containing a temporary speed restriction (packet 65) with the same identifier. Both temporary speed restrictions are taken into account by the on-board equipment for the train speed supervision. The previous non-revocable temporary speed restriction is not replaced by the last received one. The function “Determine Most Restrictive Speed Profile based on TSR” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.7 |  |
|  |  | Subset-026-3.11.5.9 |  |
|  |  | Subset-026-4.5.2.1 table - TSR |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed restriction |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The displayed permitted speed indicates that both temporary speed restrictions are taken into account. (If in FS, OS or SR mode). The speed and distance monitoring information is displayed (Intervention status) Status symbol “ST01” is displayed and removed For LS mode the DMI aspect coherent with Overspeed Status (train speed greater than speed define by TSR(B)) is displayed. Check on the JRU that: The radio message 24 “general message” with packet 65 “temporary speed restriction” is recorded. Exceeding of speed limit is recorded. Service brake commanded state (commanded and released) is recorded. Check on the TIU that: Service brake commanded and released. |
| Constraints | The train has previously received a non-revocable TSR and is using it (TSR (A)). The on-board equipment receives a new Temporary Speed Restriction (TSR (B)) with the same identity as an already received TSR (A). New TSR will be calculated such that no brake intervention is needed due to change of Permitted Speed, this is: new V_TSR > old V_TSR. Both temporary speed restrictions should overlap each other. The new Temporary Speed Restriction shall replace the previous one, except when the Temporary Speed Restriction is identified as non-revocable. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | TSR: Packet 65: Temporary Speed Restriction (1) NID_TSR = A =255 (Non-Revocable) D_TSR (A) L_TSR (A) V_TSR (A) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TSR(A) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received | I | RTM | TSR (2) with the same identity as an already received TSR (1); V_TSR (B) > V_TSR (A) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | Train location : Inside overlap area of both TSR´s. | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | CSM - Normal Status- V_TSR(A) If in FS; OS or SR Permitted Speed is the value of V_TSR(A) and NOT V_TSR (B) If in LS mode; V_TRAIN < V_TSR (A) | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Intervention status, so V_TSR(A) < V_TRAIN | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 9 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_TSR(A) Status requested: Normal status | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 15 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 16 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 18 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | Train is moving | I | INT | Train location: Full train length has left TSR (A) area. | N/A | N/A | - |
| 20 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR Permitted Speed is the value of V_TSR(B) CSM - Normal Status- V_TSR(B) If in LS mode train speed will be a bit over the V_TSR(B). The DMI aspect is Overspeed Status. | N/A | N/A | - |
| 21 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_TSR(B) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | 255 | Non-revocable speed restriction |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | > V_TSR (A) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | Second TSR: Packet 65: Temporary Speed Restriction (2) NID_TSR (B)= 255 D_TSR (B) L_TSR (B) Q_FRONT =0 V_TSR (B) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR actual permitted train speed V_TSR(B). If in LS mode V_TRAIN > V_TSR (B). Overspeed status. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | In case the train has changed its orientation any Temporary Speed Restriction shall be deleted (operational requirement: will be executed due to the mode change). |  | 9 |
| Applicable Mode/Level Combinations | L0: UN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that: The train is stopped inside the TSR area The train changes its orientation and perform a mode transition to SB (End of Mission Procedure) in Cab (A) and to SB (SoM procedure) in Cab (B) Due to mode change to SB, TSR information is deleted. Train stars moving again. The permitted speed shown to the driver shall be different from TSR speed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.10 |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.10.1.3 line Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The correct permitted speed (different from TSR) is shown on the DMI (If in FS, OS or SR mode). For LS mode the DMI aspect coherent with Normal Status (train speed greater than speed define by TSR) is displayed. Check on the JRU that: Speed and distance monitoring information is recorded. |
| Constraints | Train at standstill inside of the TSR area. The situation is the following: The driver closes the desk A and leaves the cab A of the leading engine of the train, to go to cab B and open desk B of this same engine. Desk A and desk B are connected to the same ERTMS/ETCS on-board equipment.(For more details see section 5.12.2 “The driver uses the same engine (a mission is ongoing)”) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Temporary Speed Restrictions | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ L1/ L2/ L3 | FS / LS / OS / SR / UN | The driver closes the desk on cab A. The on-board equipment switches to SB mode. | - | - | USE_FT4060300.8,9 (Desk is closed (Transition [28])) Due to mode change to SB, TSR information is deleted. | L0/ L1/ L2/ L3 | SB | - |
| 2 | L0/ L1/ L2/ L3 | SB | Cab B active | I | TIU | The driver opens the desk (cab B). Train orientation is change. | L0/ L1/ L2/ L3 | SB | - |
| 3 | L0/ L1/ L2/ L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | L0/ L1/ L2/ L3 | SB | - |
| 4 | L0/ L1/ L2/ L3 | SB | SoM Procedure and on-board equipment reach the state FS/LS/OS/SR/UN mode | - | - | USE_FT5040300 (SoM Procedure) | L0/ L1/ L2/ L3 | SB | - |
| 5 | L0/ L1/ L2/ L3 | SB | A mode transition from SB to go to the applicable modes is performed. | - | - | USE_FT4060300.49 from L1/SB to L1/SR. or USE_FT4060300.58 from L0/SB to L0/UN or USE_FT4080425.7 from L2/SB to L2/ FS, OS or LS | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | - |
| 6 | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | Train is moving (V_TRAIN=40) | I | INT | The train moves again inside the TSR area. | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | - |
| 7 | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS/ SR. | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | - |
| 8 | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS/ SR | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | - |
| 9 | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | The speed and distance monitoring information is displayed | O | DMI | Check that the Temporary Speed Restriction is not taken into account for the train speed supervision. If in FS, OS or SR Permitted Speed is NOT V_TSR CSM - Normal Status If in LS mode train speed will be a bit over the V_TSR(B). The DMI aspect is Normal Status. | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | - |
| 10 | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= < another than V_TSR>) is recorded | O | JRU | The speed value is a permitted speed equal to the maximum train speed or any lower static speed restriction (not the TSR). | L0/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Temporary Speed Restrictions | NOT STORED | DELETED |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR actual permitted train speed not V_TSR. If in LS mode V_TRAIN > V_TSR (B). Normal status. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | Inhibition of revocable Temporary Speed Restriction from balises in L2/L3. When ERTMS/ETCS on-board equipment has accepted an order to reject revocable TSRs from balises, this inhibition shall be stored and shall be effective immediately, but only for revocable TSRs received from balises thereafter. |  | 10 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The ERTMS/ETCS on-board equipment receives an order from RBC to reject revocable TSRs from balises in L2/L3.(Packet 64) The ETRMS/ETCS on-board equipment receives from a Balise Group a telegram containing a TSR. (Packet 65) The train reaches TRS area and no speed restriction due to TSR is applied. Train permitted speed remains constant. The function “Inhibition of revocable TSRs from balises” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.13 |  |
|  |  | Subset-026-4.5.2.1 table - Inhibition of revocable TSRs from balises(only level 2/3) |  |
|  |  | Subset-026-4.8.3.1.1 Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-4.8.4.2 Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-7.4.2.16.1 Packet Number 64: Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : When entering in TSR area, the permitted speed shown on DMI is different from the TSR speed (only in FS or OS mode).Permitted Speed is NOT UPDATED. The driver asks for the permitted speed to be DISPLAYED (only and OS mode) For LS mode the DMI aspect coherent with Normal Status (train speed greater than speed define by TSR) is displayed. Check on the JRU that : RBC General Message 24 including Packet 64 is received and stored correctly on-board A Balise telegram with Packet 65 is received and stored correctly by the on-board equipment |
| Constraints | The inhibition is stored and is effective immediately and applies only for revocable TSRs received from balises thereafter. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Permitted speed active only in FS mode, and available in OS mode. |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 64 is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Balise Group message containing packet 65 is received | I | BTM | Packet 65 with NID_TSR = 0-126 (revocable TSR) | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Train is moving. | I | INT | The train drives inside TSR area with it maximum safe front end. | N/A | N/A | - |
| 6 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step: Only applicable for OS mode | N/A | N/A | - |
| 7 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step: Only applicable for OS mode | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Speed value defined as V_TSR in stored message will not be used by the on-board equipment. If in FS or OS mode Permitted speed does not change to the TSR speed. If in LS mode train speed will be a bit over the V_TSR. The DMI aspect is Normal Status. | N/A | N/A | - |
| 9 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <other than V_TSR>) is recorded | O | JRU | Speed and distance monitoring information does not change. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 64 | Inhibition of revocable Temporary Speed Restriction from balises in L2/L3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 000 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | 0-126 (revocable TSR) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in LS mode: V_TRAIN > V_TSR; Normal Status. If in FS or OS mode, actual permitted speed but not the TSR speed |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | Deletion of the inhibition of revocable Temporary Speed Restriction from balises in L2/L3 due to termination of the communication session with the RBC that order the inhibition. |  | 11 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: For any reason the communication session established with the RBC that ordered the inhibition is terminated The train receives a balise telegram (Packet 65- revocable TSR) The inhibition of revocable TSR from balises is not valid any more due to the termination of the communication session with the RBC that order the inhibition, so When the train reaches TRS area, train permitted speed changes to V_TSR. The function “Inhibition of revocable TSRs from balises” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.14 |  |
|  |  | Subset-026-3.11.5.14 a) |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : When entering TRS area the Permitted Speed is UPDATED (If in FS/ OS mode). The permitted speed shown on DMI is the TSR speed (only in FS mode) The permitted speed shown on DMI on driver request is the TSR speed (only in OS mode)The speed and distance monitoring information is displayed (Intervention status) Status symbol “ST01” is displayed and removed Check on the JRU that : The termination of a communication session is RECORDED. The Balise telegram with Packet 65 is received and stored correctly by the on-board equipment New permitted speed, V_TSR is RECORDED. Exceeding of speed limit is recorded. Service brake commanded state (commanded and release) is recorded. Check on the TIU that: Service brake commanded and released. |
| Constraints | The Temporary Speed Restriction is taken into account for the train speed supervision |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Permitted speed active only in FS mode, and available in OS mode. |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TRAIN |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The communication session with RBC is Terminated.(for any reason) | - | - | USE_FT3050500.5 (Termination of radio communication session ordered by RBC) | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 65 is received. | I | BTM | Packet 65 with NID_TSR = 0-126 (revocable TSR), D_TSR=0 V_TSR > actual train speed | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | The train drives inside TSR area with its maximum safe front end. Speed value defined as V_TSR in stored message will become the permitted speed. | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | CSM - Normal Status- V_TSR If in FS or OS mode: The Permitted Speed is updated with the value of V_TSR. If in LS mode: V_TRAIN < V_TSR | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <TSR speed km/h>>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Intervention status, so V_TSR < V_TRAIN | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <TSR speed km/h>; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 11 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 12 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 14 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Normal status V_TRAIN < V_TSR | N/A | N/A | - |
| 16 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20, V_PERM= <same value as V_TSR>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 17 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 18 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 20 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | = 0-126 (revocable TSR) |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | > actual train speed |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | NOT ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | Revocable and received from balises |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in LS mode: V_TRAIN < V_TSR. Normal Status If in FS or OS mode the speed value that is displayed on the DMI is a permitted speed equal to the TSR speed. |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.5: Temporary Speed Restrictions |  | 3110500 |
| Test Case of Feature | Deletion of the inhibition of revocable Temporary Speed Restriction from balises in L2/L3 due to RBC/RBC handover when the train front end crosses the RBC/RBC border. |  | 12 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The train is running in L2/L3 and receives from the Handing Over RBC an order to reject revocable TSRs from balises (Packet 64) Message 24 with packet 64 is stored and being effective immediately When the train front end crosses the RBC/RBC border, the inhibition of revocable TSRs from balises is deleted The on-board receives from balise group BG a telegram with Packet 65- revocable TSR Balise telegram with packet 65 is stored and accepted by the on-board equipment When the train reaches TSR area, train permitted speed changes to TSR speed |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.5.14 |  |
|  |  | Subset-026-3.11.5.14 b) |  |
|  |  | Subset-026-7.4.2.16.1 Packet Number 64: Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-4.7.2 table - Output information Permitted Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : When entering TRS area the Permitted Speed is UPDATED. (only in FS/ OS mode) The permitted speed shown on DMI is the TSR speed (only in FS mode) The driver asks for the permitted speed to be DISPLAYED (only in OS mode)Check on the JRU that : RBC General Message 24 including Packet 64 is received and stored correctly on-board RBC/RBC handover is RECORDED. A Balise telegram with Packet 65 is received and stored correctly by the on-board equipment New permitted speed is RECORDED. Service brake commanded state (commanded and release) is recorded. Check on the TIU that: Service brake commanded and released. |
| Constraints | The on-board can handle two communication sessions simultaneously. Radio communication is established with both RBCs. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | with Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Permitted speed active only in FS mode, and available in OS mode. |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 64 is received | I | RTM | Inhibition order received from Handing Over RBC. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | An RBC/RBC Handover is performed. | - | - | USE_FT3150100.1 (RBC / RBC Handover function) The Inhibition Order is Deleted. | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message containing packet 65 is received | I | BTM | Packet 65 with NID_TSR = 0-126 (revocable TSR) | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving. | I | INT | The train drives inside TSR area with its maximum safe front end. Speed value defined as V_TSR in stored message will become the permitted speed. | N/A | N/A | - |
| 7 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | CSM - Normal Status If in FS or OS mode The Permitted Speed is updated with V_TSR. If in LS mode: V_TRAIN < V_TSR | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <same as V_TSR>>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Intervention status, so V_TSR < V_TRAIN | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <same as V_TSR>; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 13 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_TSR CSM: Normal Status | N/A | N/A | - |
| 18 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <same as V_TSR km/h>>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 19 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 20 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 22 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 64 | Inhibition of revocable Temporary Speed Restriction from balises in L2/L3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | = 0-126 (revocable TSR) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | < actual train speed |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Accepting RBC |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in LS mode, V_TRAIN < V_TSR. Normal Status If in FS or OS mode the speed value that is displayed on the DMI is a permitted speed equal to the TSR speed. |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |