| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080407 (4.8.4: Accepted Information depending on the modes - Co-operative Shortening of MA) Total: 5 Test case(s) |  |
| REF: | Subset-076-5-2-4080407 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 04.11.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.3 12.12.2013 | All | Editorial changes | CEDEX |
| Version 2.9.4 18.12.2013 | All | Review | CEDEX |
| Version 2.9.5 20.12.2013 | All | Review | CEDEX |
| Version 2.9.6 08.01.2014 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Co-operative Shortening of MA |  | 4080407 |
| Test Case of Feature | A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). The train receives the Message 9 – Request to shorten MA from the supervising RBC, including an MA shorter than the MA available on-board the train. The train checks that it is not possible to cope with the new MA included in the received message 9. The train responds to the RBC by sending it a message 138 – Request to shorten MA is rejected in order to inform the RBC that the train does not accept the new MA. The target speed and distances displayed to the DMI are not changed. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | To check that the train send a message 138 to the RBC when it does not accept a request to shorten MA from an RBC and that it does not take into account the new MA included in the request to shorten MA |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.6.1 |  |
|  |  | Subset-026-3.8.6.1 b) b) |  |
|  |  | Subset-026-3.8.6.1 c) |  |
|  |  | Subset-026-4.8.3.1.1 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |
|  |  | Subset-026-8.6.6 Message 138: Request to Shorten MA is rejected |  |
|  |  | Subset-026-8.7.5 Message 9: Request to Shorten MA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check the target speed and distance displayed on the DMI and the recorded messages in the JRU, both sent to and received from the RBC. |
| Constraints | Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has been received already. SSP and gradient given together with the MA cover the full length of the MA. No emergency stop(s) have been accepted and are not yet revoked or deleted onboard A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Request to Shorten MA" (NID_MESSAGE=9)is received | I | RTM | including an MA shorter than the MA available on-board the train | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The train rejects the new MA. to check in the JRU that as the message is rejected the PS, TD and TS is not modified | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if in FS or OS mode. The DMI displayed target speed and distance are not modified according to the target speed and distance included in the message 9. Target speed and target distance are displayed in FS mode and ,on request by driver, in OS mode. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Request to shorten MA is rejected" (NID_MESSAGE=138) is transmitted | O | RTM | This message tests that the Co-operative shortening of MA information has been accepted in terms of its subsequent acceptance/rejection evaluation | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 9 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 9 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 5: Radio Message 138 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 138 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time stamp contained in the request |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Co-operative Shortening of MA |  | 4080407 |
| Test Case of Feature | Rejection of Co-operative Shortening of MA . A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). The train has sent its Train Data to the supervising RBC and has not yet received the related acknowledgement from the RBC. The train receives the Message 9 – Request to shorten MA from the RBC, including an MA shorter than the MA available on-board the train. Because the train has not yet received the Train Data acknowledgement from the RBC, the train does not take into account the message 9: the On-Board equipment sends to the RBC neither a Message 137 ‘Request to shorten MA is granted’ nor a Message 138 ‘Request to shorten MA is rejected’. The target speed and distances are displayed to the DMI are not changed. |  | 2 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | To check that a train rejects a co-operative request to shorten MA by sending neither a message 137 nor a message 138 to the RBC when the train has not yet received the acknowledgement of its Train Data by the RBC. This Test case tests condition [3] of Subset-026-4.8.3.1.1 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.6.1 |  |
|  |  | Subset-026-8.7.5 Message 9: Request to Shorten MA |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |
|  |  | Subset-026-4.8.3.1.1 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train sends its Train Data to the RBC and receives the message 9 before the acknowledgement of the Train data (message 8) is received from the RBC. To check the target speed and distance displayed on the DMI and the recorded messages in the JRU, both sent to and received from the RBC. |
| Constraints | SSP and gradient given together with the MA cover the full length of the MA. No emergency stop(s) have been accepted and are not yet revoked or deleted onboard A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | STORED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / LS | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | Validated train data. Packet 1 must be used instead of Packet 0, if position report is based on 2 BGs | L2 / L3 | FS / OS / LS | - |
| 2 | L2 / L3 | FS / OS / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 3 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "Request to Shorten MA" (NID_MESSAGE=9) is received | I | RTM | including an MA shorter than the MA available on-board the train | L2 / L3 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 5 | L2 / L3 | FS / OS / LS | NOT SA-DATA.Request with Euroradio Message "Request to shorten MA is rejected" (NID_MESSAGE=138) is transmitted | O | RTM | This tests that the Co-operative shortening of MA information has been rejected | L2 / L3 | FS / OS / LS | - |
| 6 | L2 / L3 | FS / OS / LS | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 7 | L2 / L3 | FS / OS / LS | NOT SA-DATA.Request "Request to shorten MA is granted" (NID_MESSAGE=137) is transmitted | O | RTM | This tests that the Co-operative shortening of MA information has been rejected | L2 / L3 | FS / OS / LS | - |
| 8 | L2 / L3 | FS / OS / LS | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 9 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | FS / OS / LS | - |
| 10 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 11 | L2 / L3 | FS / OS / LS | USE_FT4060300.59 (for changing to FS mode) | - | - | Only if in LS mode. | L2 / L3 | FS / OS | - |
| 12 | L2 / L3 | FS / OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. The driver asks for the TS and TD. | L2 / L3 | FS / OS | - |
| 13 | L2 / L3 | FS / OS | The speed and distance monitoring information is displayed | O | DMI | The DMI displayed target speed and distance are not modified according to the target speed and distance included in the message 9. Target speed and target distance are displayed in FS mode and ,on request by driver, in OS mode. No JRU message is recorded after this step, because no change is displayed on the DMI | L2 / L3 | FS / OS | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 3: Radio Message 9 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 9 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 9: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Co-operative Shortening of MA |  | 4080407 |
| Test Case of Feature | Rejection of Co-operative Shortening of MA information. The train receives the Message 9 – Request to shorten MA from the RBC. Neither a Message 137 ‘Request to shorten MA is granted’ nor a Message 138 ‘Request to shorten MA is rejected’ is sent. |  | 3 |
| Applicable Mode/Level Combinations | L0: SH, UN, PS, SL, SB, TR, NL, L1: FS, LS, OS, SR, SH, PS, SL, SB, TR,PT, NL, RV L2: SR, SH, PS, SL, SB, TR,PT, NL, RV L3: SR, SH, PS, SL, SB, TR,PT, NL, RV LNTC: SH, PS, SL, SB, TR, NL, SN |  |  |
| Target of Test | To check that a train rejects a co-operative request to shorten MA by sending neither a message 137 nor a message 138 to the RBC when the train is in the applicable mode/level combinations. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-8.7.5 Message 9: Request to Shorten MA |  |
|  |  | Subset-026-4.8.3.1.1 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check that a train rejects a co-operative request to shorten MA by sending neither a message 137 nor a message 138 to the RBC To check the target speed and distance displayed on the DMI |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR/ SH / UN / SL / SB / TR / PT / NL / LS / NS / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Request to Shorten MA" (NID_MESSAGE=9) is received | I | RTM | including an MA shorter than the MA available on-board the train | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Request to shorten MA is rejected" (NID_MESSAGE=138) is transmitted | O | RTM | This tests that the Co-operative shortening of MA information has been rejected | N/A | N/A | - |
| 4 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA.Request "Request to shorten MA is granted" (NID_MESSAGE=137) is transmitted | O | RTM | This tests that the Co-operative shortening of MA information has been rejected | N/A | N/A | - |
| 6 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI displayed target speed and distance are not modified according to the target speed and distance included in the message 9. Target speed and target distance are displayed in FS mode and ,on request by driver, in OS mode. No JRU message is recorded after this step, because no change is displayed on the DMI | N/A | N/A | - |

| Step 1: Radio Message 9 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 9 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR/ SH / UN / SL / SB / TR / PT / NL / LS / NS / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Co-operative Shortening of MA |  | 4080407 |
| Test Case of Feature | Rejection of Co-operative Shortening of MA. A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). SSP and gradient given together with the MA cover the full length of the MA.The train receives the Message 9 – Request to shorten MA from the RBC, including an MA shorter than the MA available on-board the train. The SSP and gradient given together with the MA do not cover the full length of the MA, so the train does not take into account the message 9: the On-Board equipment sends to the RBC neither a Message 137 ‘Request to shorten MA is granted’ nor a Message 138 ‘Request to shorten MA is rejected’. The target speed and distances are displayed to the DMI are not changed. |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | To check that a train rejects a co-operative request to shorten MA by sending neither a message 137 nor a message 138 to the RBC when the train receives the Message 9 – Request to shorten MA from the RBC including an MA shorter than the MA available on-board and a SSP and gradient which do not cover the full length of the given MA. This Test case tests condition [4] of Subset-026-4.8.3.1.1 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.6.1 |  |
|  |  | Subset-026-8.7.5 Message 9: Request to Shorten MA |  |
|  |  | Subset-026-4.8.2.1 d) |  |
|  |  | Subset-026-4.8.3.1.1 Exception [4] |  |
|  |  | Subset-026-4.8.3.1.1 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check the target speed and distance displayed on the DMI and the recorded messages in the JRU, both sent to and received from the RBC. |
| Constraints | A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). The train has sent its Train Data to the RBC and has received the related acknowledgement from the RBC. In message 9 SSP and gradient given together with the MA do not cover the full length of the MA. No emergency stop(s) have been accepted and are not yet revoked or deleted onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Only if in FS or OS mode. The Target Distance is D0 and the Target Speed is V0. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Request to Shorten MA" (NID_MESSAGE=9) is received | I | RTM | including an MA shorter than the MA available on-board the train. SSP and gradient given together with the MA do not cover the full length of the MA | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Request to shorten MA is rejected" (NID_MESSAGE=138) is transmitted | O | RTM | This tests that the Co-operative shortening of MA information has been rejected | N/A | N/A | - |
| 8 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | NOT SA-DATA.Request "Request to shorten MA is granted" (NID_MESSAGE=137) is transmitted | O | RTM | This tests that the Co-operative shortening of MA information has been rejected | N/A | N/A | - |
| 10 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if in FS or OS mode. The DMI displayed target speed and distance are not modified according to the target speed and distance included in the message 9. The Target Distance is D0 and the Target Speed is V0. | N/A | N/A | - |

| Step 5: Radio Message 9 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 9 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 /12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Co-operative Shortening of MA |  | 4080407 |
| Test Case of Feature | Rejection of Co-operative Shortening of MA . A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). The train stores an emergency stop(s) information that was accepted but not yet revoked or deleted onboard.The train receives the Message 9 – Request to shorten MA from the RBC, including an MA shorter than the MA available on-board. The train does not take into account the message 9: the On-Board equipment sends to the RBC neither a Message 137 ‘Request to shorten MA is granted’ nor a Message 138 ‘Request to shorten MA is rejected’. The target speed and distances are displayed to the DMI are not changed. |  | 5 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | To check that a train rejects a co-operative request to shorten MA by sending neither a message 137 nor a message 138 to the RBC when the train stores an emergency stop(s) information that was accepted but not yet revoked or deleted onboard. This Test case tests condition [5] of Subset-026-4.8.3.1.1 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.6.1 |  |
|  |  | Subset-026-8.7.5 Message 9: Request to Shorten MA |  |
|  |  | Subset-026-4.8.3.1.1 Exception [5] |  |
|  |  | Subset-026-4.8.3.1.1 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 Co-operative Shortening of MA + (optional) Mode Profile + (optional) List of Balises for SH area |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | To check the target speed and distance displayed on the DMI and the recorded messages in the JRU, both sent to and received from the RBC. |
| Constraints | An emergency stop(s) has been accepted and is not yet revoked or deleted onboard SSP and gradient given together with the MA cover the full length of the MA. The train has sent its Train Data to the RBC and has received the related acknowledgement from the RBC. A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). The train has sent its Train Data to the RBC and has not yet received the related acknowledgement from the RBC. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Conditional Emergency Stop | STORED | not yet revoked or deleted onboard |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Only if in FS or OS mode. The Target Distance is D0 and the Target Speed is V0. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Request to Shorten MA" (NID_MESSAGE=9) is received | I | RTM | including an MA shorter than the MA available on-board the train | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Request to shorten MA is rejected" (NID_MESSAGE=138) is transmitted | O | RTM | This tests that the Co-operative shortening of MA information has been rejected | N/A | N/A | - |
| 8 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | NOT SA-DATA.Request "Request to shorten MA is granted" (NID_MESSAGE=137) is transmitted | O | RTM | This tests that the Co-operative shortening of MA information has been rejected | N/A | N/A | - |
| 10 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if in FS or OS mode. The DMI displayed target speed and distance are not modified according to the target speed and distance included in the message 9. The Target Distance is D0 and the Target Speed is V0. | N/A | N/A | - |

| Step 5: Radio Message 9 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 9 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 /12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Conditional Emergency Stop | STORED | not yet revoked or deleted onboard |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |