| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 6060304 (6.6.3.4 Messages transmitted to RBC/RIU) Total: 11 Test case(s) |  |
| REF: | Subset-076-5-2-6060304 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/New in Baseline 3 | INECO |
| Version 2.9.1 28.05.2013 | All | Second version/ UNISIG review comments from 13-05-2013 included; TC11 created new. | INECO |
| Version 2.9.2 15.07.2013 | TC11 | Third version/ line as 5,6,26,30A,35 and 36 from UNISIG review sheet(13-05-2013) answered; Step 10&11 from TC11 changed. | INECO |
| Version 2.9.3 01.08.2013 | All | Fourth version/Internal modification;TC_Editorv27; Correction of Requirements blank space. | INECO |
| Version 2.9.4 07.01.2014 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission of the message 132: “MA Request” by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated ” T[4]” & data is unchanged “U[1]” |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ OS |  |  |
| Target of Test | The target of this test is to check that: A situation which leads to shorten the current MA and track description to the current position of the on-board equipment or the new stop location. The on-board equipment informs the RBC/RIU by sending a radio message 132 “MA request” that is translated for the RBC system version X=1 as: the variable Q_MARQSTREASON is being replaced with Q_TRACKDEL. The transmitted value to X=1 RBC: Q_MARSQREASON = x1xxx into Q_TRACKDEL set to 1. The position report packet should be sent with M_MODE unchanged for FS or OS mode. The RBC/RIU understands correctly the received message and sends a valid Movement Authority. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 1 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 10 |  |
|  |  | Subset-026-6.6.3.4.5 [4] |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-8.6.3 Message 132: MA Request |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Data from message 132 and packet 0 are translated correctly by the on-board and the trackside send the requested information.. Check on the JRU that: The Message 132 :MA Request is RECORDED. The Message 3: Movement Authority is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | No linked balise group message is received | I | BTM | - | The balise group missed was linked with service brake reaction | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded. | O | TIU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Service Brake " is displayed. | O | DMI | - | Brake command indication due to RAMS related supervision (service brake) Symbol ST01 | N/A | N/A | - |
| 4 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | - | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is removed | O | DMI | - | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted to a system version X=1 RBC. | O | RTM | 001 0000 | Q_TRACKDEL = 1 For Message132, data is translated as: Q_MARQSTREASON is replaced with Q_TRACKDEL And value x1xxx is set to 1 (track description deleted) | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21 and packet 27 is received. | I | RTM | 001 0000 | The onboard should take into account exception (3) requirement 6.6.3.2.3 | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 8: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| Q_TRACKDEL | 1 | 1 | Track description deleted |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 | 0 / 1 |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 10: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NC_DIFF(n) | 4 | FINITE VALUE | - |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| NC_DIFF(k,m) | 4 | FINITE VALUE | - |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 | FS/ OS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Movement Authority | STORED | CHANGED |
| Gradient Profile | STORED | CHANGED |
| International SSP | STORED | CHANGED |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission of the message 132: “MA Request” by the on-board equipment to an RBC certified to the system version number X = 1.Reception of the message 3:” Movement Authority” from the X=1 RBC. Tested action: Data is translated: T[4] & data is unchanged U[1] |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment requests a MA to the RBC/RIU by sending message 132 that is translated for the RBC system version X=1 as: the variable Q_MARQTREASON is being replaced with Q_TRACKDEL. The transmitted value to X=1 RBC: Q_MARSQREASON ≠ x1xxx into Q_TRACKDEL set to 0. The position report packet should be sent with M_MODE unchanged for FS or OS mode. The RBC/RIU understands the received message and sends a valid Movement Authority. Message 3 with packet 15 is received correctly and no translation is necessary. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.3.2 Table - row 2 |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 2 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 10 |  |
|  |  | Subset-026-6.6.3.4.5 [1] |  |
|  |  | Subset-026-6.6.3.4.5 [4] |  |
|  |  | Subset-026-7.4.3.2 Packet Number 1: Position Report based on two balise groups |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-8.6.3 Message 132: MA Request |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Data from message 132 is translated correctly by the on-board and the trackside send the requested information Data received from trackside, message 3 with packets 15 is used correctly by the on-board equipment. Check on the JRU that: The Message 132 :MA Request is RECORDED. The Message 3: Movement Authority with packets15 is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 | FS/ OS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 1 is transmitted to a system version X=1 RBC. | O | RTM | 001 0000 | Q_TRACKDEL=0 For Message132, the on-board effectues the translation to system version X=1 as: Q_MARQSTREASON is replaced with Q_TRACKDEL And Value diferent from x1xxx is set to 0 (no track description deleted) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) | I | RTM | 001 0000 | No data translation is necesary.Data from message and packet remains unchanged. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | - | MA and track description for level 2/3 is used. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | - | N/A | N/A | - |

| Step 1: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| Q_TRACKDEL | 1 | 0 | No Track description deleted |
| NID_PACKET | 8 | 1 | Position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PRVLRBG | - | - | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 | FS/ OS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Movement Authority | STORED | New MA |
| Gradient Profile | STORED | UNCHANGED |
| International SSP | STORED | UNCHANGED |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission of the message 136: “Train Position report ” with packet 4: ”Error Reporting ” by the on-board equipment to an RBC/RIU certified to the system version number X = 1. Tested action: data is Unchanged “U[2]”. |  | 3 |
| Applicable Mode/Level Combinations | L2/ L3: SL/ NL |  |  |
| Target of Test | The target of this test is to check that : Packet 4: “Error Reporting” is properly translated and the error reported by the on-board equipment to the X=1 RBC/RIU is understood as a Fatal Error. Due to a Safety Critical Fault in a sleeping/non leading engine the train is not stopped, if possible, and the on-board equipment informs RBC/RIU about the error by sending the message 136 with packet 4. The on-board equipment sets the variable M_ERROR, from packet 4, from 6(X=2) to 7(X=1). The RBC if informed about the Fatal Error (Equipment in SL or NL mode). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 11 |  |
|  |  | Subset-026-6.6.3.4.5 [2] |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.4 Packet Number 4: Error reporting |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Data from packet 4 (M_ERROR) sent by the on-board equipment is understood by the RBC as a Fatal Error. Check on the JRU that: The radio message 136 (Train Position Report) with the packet 4 (Error Reporting) is RECORDED. Additional check: M_ERROR = 7 |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5/ 11 | SL/ NL |
| Radio communication session | ESTABLISHED | System version X=1 |
| ERTMS/ETCS level | 3/ 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | A safety critical fault occur | I | INT | - | - | N/A | N/A | - |
| 2 | N/A | N/A | SAFETY CRITICAL FAULT IN MODE SL, NL OR PS (NID_MESSAGE_JRU=27) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 4 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | Safety Critical fault For packet 4, data is translated as: M_ERROR value set from 6 to 7 The RBC understands the error as a Fatal Error (Equipment in SL or NL mode) | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 5 / 11 | SL / NL |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |
| NID_PACKET | 8 | 4 | Error Reporting |
| L_PACKET | 13 | 29 | - |
| M_ERROR | 8 | 7 | Safety critical fault transmitted as Fatal Error Value 6 set to 7 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5/ 11 | SL/ NL |
| Radio communication session | ESTABLISHED | UNCHANGED |
| ERTMS/ETCS level | 3/ 4 | L2 /L3 |
| Radio Network ID | STORED | UNCHANGED |
| RBC ID/Phone Number | STORED | UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | No transmission of packet 4: Error Reporting by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: data is unchanged “U[2]”. |  | 4 |
| Applicable Mode/Level Combinations | L2/ L3: FS/ LS /OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment deletes packet 4: “Error Reporting” from the message 136 when the error is due to a double linking error.(M_ERROR = 7) (X=2) The on-board equipment does not inform RBC/RIU about the error of not finding two balise groups included in the linking information. Message 136 is sent without packet 4. Reason: Value of M_ERROR = 7 not understood by the RBC certified with system version X=1. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 11 |  |
|  |  | Subset-026-6.6.3.4.5 [2] |  |
|  |  | Subset-026-3.16.2.7.1.1 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Packet 4 (M_ERROR=7) is not sent by the on-board equipment with message 136. Check on the JRU that: The radio message 136 (Train Position Report) without packet 4 (Error Reporting) is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. Linking information stored on-board. The reference locations of the two consecutive BG (marked as linked and included in the linking information) are not found in the expectation windows. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS /LS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Linking | STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT One Balise Group message is received | I | BTM | - | Train passes the expectation window of the first announced BG A. The balise group is not detected. | N/A | N/A | - |
| 2 | N/A | N/A | NOT One Balise Group message is received | I | BTM | - | Train passes the expectation window of the second announced BG B. The balise group is not detected. | N/A | N/A | - |
| 3 | N/A | N/A | The system status message "Balise read error" is displayed | O | DMI | - | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | Service brake commanded. | O | TIU | - | - | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Service Brake " is displayed. | O | DMI | - | Brake command indication due to RAMS related supervision (service brake) Symbol ST01 | N/A | N/A | - |
| 7 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 9 | N/A | N/A | Train reaches standstill. Train is at standstill. | I | INT | - | V_MAIN = 0 | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) to the RBC X=1. | O | RTM | 001 0000 | Packet 4 is deleted because M_ERROR = 7 (X=2). The RBC not informed about the double linking error. | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded. | O | TIU | - | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake intervention" is removed. | O | DMI | - | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 15 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=0) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 10: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 | 0 / 1 |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS /LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 | L1 /L2 /L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | No transmission of packet 4: Error Reporting by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: data is unchanged “U[2]”. |  | 5 |
| Applicable Mode/Level Combinations | L1 : FS/ OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment deletes packet 4: “Error Reporting” from the message 136 when the identity of the error is due to a double repositioning error.(M_ERROR = 8) (X=2) The on-board equipment sends a position report to the RBC without sending the error of double repositioning error. Message 136 is send without packet 4. Reason: Value of M_ERROR = 8 not understood by the RBC certified with system version X=1. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 11 |  |
|  |  | Subset-026-6.6.3.4.5 [2] |  |
|  |  | Subset-026-3.16.2.7.2.2 |  |
|  |  | Subset-026-7.4.2.5 Packet Number 16: Repositioning Information |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Packet 4 (M_ERROR=8) is not sent by the on-board equipment with message 136. Check on the JRU that: The radio message 136 (Train Position Report) without packet 4 (Error Reporting) is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. Linking information with repositioning information (BG1 with NID_BG=16383) stored on-board. A second BG is found within the window of BG1 with the same criteria. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 | FS/ OS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Linking | STORED | BG1 marked as NID_BG=16383 BG2 not included in the linking information |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group 1 message (N_TOTAL=1; NID_C=<country/region>) containing packet 16 is received | I | BTM | 001 0000 | The Balise group is passed before D_LINK | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The distance monitoring information is displayed | O | DMI | - | TDM and MA length are updated. | N/A | N/A | - |
| 4 | N/A | N/A | DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | Balise Group 2 message (N_TOTAL=1; NID_C=<country/region>) containing packet 16 is received | I | BTM | 001 0000 | The on-board antenna hasn’t left the expectation window of the BG1. BG2 found satisfied the same criteria as BG1 | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | - | N/A | N/A | - |
| 8 | N/A | N/A | Service brake commanded. | O | TIU | - | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Service Brake " is displayed. | O | DMI | - | Symbol ST01 | N/A | N/A | - |
| 10 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 11 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 12 | N/A | N/A | Train reaches standstill. Train is at standstill. | I | INT | - | V_MAIN = 0 | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) to the RBC X=1. | O | RTM | 001 0000 | Packet 4 is deleted because M_ERROR = 8 (X=2). The RBC not informed about the double repositioning error. | N/A | N/A | - |
| 14 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 15 | N/A | N/A | The new speed and distance monitoring information is displayed. | O | DMI | - | MA and Track description are shortened to the current train position. Driver is informed. | N/A | N/A | - |
| 16 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | - | N/A | N/A | - |
| 17 | N/A | N/A | Service brake not commanded. | O | TIU | - | - | N/A | N/A | - |
| 18 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed. | O | DMI | - | - | N/A | N/A | - |
| 19 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 20 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=0) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 16 | Repositioning information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| L_SECTION | 15 | FINITE VALUE | New length of movement authority |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 16 | Repositioning information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| L_SECTION | 15 | FINITE VALUE | New length of movement authority |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 13: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 | 0 / 1 |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS /LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission and translation of packet 11: “Validated Train data” with message 129 by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated T[3] |  | 6 |
| Applicable Mode/Level Combinations | L0 : UN/ SB/ TR LNTC: SN/ SB/ TR L1: FS/ LS/ OS/ SR/ SB/ TR/ PT L2: FS/ LS/ OS/ SR/ SB/ TR/ PT L3: FS/ LS/ OS/ SR/ SB/ TR/ PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment translates correctly packet 11: “Validated Train Data” to the X=1 RBC. Following any entry/ modification of TRAIN DATA when a communication session is already established or following the successful establishment of a communication session, the on-board equipment sends a valid set of Train Data to the RBC. Message 129 with packet 11. Translation applies to variables as: NID_OPERATIONAL[3a], NC_TRAIN [3b], M_LOADINGGAUGE [3c], M_AXLELOAD[3d] and N_ITER [3e] The RBC (X=1) understand correctly and acknowledges the reception of this set of Train Data with message 8: “Acknowledgment of Train Data”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.3.2 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 7 |  |
|  |  | Subset-026-6.6.3.4.5 [3] |  |
|  |  | Subset-026-6.6.3.4.5 [3a] |  |
|  |  | Subset-026-6.6.3.4.5 [3b] |  |
|  |  | Subset-026-6.6.3.4.5 [3c] |  |
|  |  | Subset-026-6.6.3.4.5 [3d] |  |
|  |  | Subset-026-6.6.3.4.5 [3e] |  |
|  |  | Subset-026-3.18.3.4 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The transmitted radio message 129 with packet 11 is RECORDED. The acknowledgment of the RBC, message 8 is RECORDED |
| Constraints | Radio communication session is established Following any entry/modification of Train Data or following the successful establishment of a communication session A Cab is active (for SB mode) The train belongs to a “Cant Deficiency “ train category. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7/ 8/ 12/ 13 | FS/ OS/ SR/ UN/ SB/ TR/ PT/ LS/ SN |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | The Stored Variables values for t0 (X=2) are: NC_CDTRAIN = 5 M_LOADINGGAUGE ≠0 M_AXLELOAD= D2/D3/D4/D4xL N_ITER ≠0 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL = NID_OPERATIONAL1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | For Message 129 no translation is necessary. For Packet 11 the translation is: NID_OPERATIONAL=NID_OPERTAIONAL1 NC_CDTRAIN translated to NC_TRAIN and set to xxx xxxx x1xx xxxx M_LOADINGGAUGE set to 0 M_AXLELOAD= 0101101 N_ITER set to 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | The RBC understands and acknowledges this set of Train Data. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | - | N/A | N/A | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | One of the applicable mode |
| M_LEVEL | 3 | FINITE VALUE | One of the applicable level |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | NID_OPERATIONAL = NID_OPERATIONAL1 |
| NC_TRAIN | 15 | FINITE VALUE | International Train category |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | 0 | - |
| M_AXLELOAD | 7 | 0101101 | 22,5 t |
| M_AIRTIGHT | 2 | FINITE VALUE | Airtight system presence |
| N_ITER | 5 | 0 | - |
| M_TRACTION (k) | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available |

| Step 3: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message : t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7/ 8/ 12/ 13 | FS/ OS/ SR/ UN/ SB/ TR/ PT/ LS/ SN |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Train Data | STORED | UNCHANGED |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |
| Train Running Number | STORED | UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission and translation of packet 11: “Validated Train data” with message 129 by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated T[3] |  | 7 |
| Applicable Mode/Level Combinations | L0 : UN/ SB/ TR LNTC: SN/ SB/ TR L1: FS/ LS/ OS/ SR/ SB/ TR/ PT L2: FS/ LS/ OS/ SR/ SB/ TR/ PT L3: FS/ LS/ OS/ SR/ SB/ TR/ PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment translates correctly packet 11: “Validated Train Data” to the X=1 RBC. Following any entry/ modification of TRAIN DATA when a communication session is already established or following the successful establishment of a communication session, the on-board equipment sends a valid set of Train Data to the RBC. Message 129 with packet 11. Translation applies to variables as: NID_OPERATIONAL[3a], NC_TRAIN [3b], M_LOADINGGAUGE [3c], M_AXLELOAD[3d] and N_ITER [3e] The RBC (X=1) understand correctly and acknowledges the reception of this set of Train Data with message 8: “Acknowledgment of Train Data”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.3.2 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 7 |  |
|  |  | Subset-026-6.6.3.4.5 [3] |  |
|  |  | Subset-026-6.6.3.4.5 [3a] |  |
|  |  | Subset-026-6.6.3.4.5 [3b] |  |
|  |  | Subset-026-6.6.3.4.5 [3c] |  |
|  |  | Subset-026-6.6.3.4.5 [3d] |  |
|  |  | Subset-026-6.6.3.4.5 [3e] |  |
|  |  | Subset-026-3.18.3.4 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The transmitted radio message 129 with packet 11 is RECORDED. The acknowledgment of the RBC, message 8 is RECORDED |
| Constraints | Radio communication session is established Following any entry/modification of Train Data or following the successful establishment of a communication session A Cab is active (for SB mode) The train belongs to “other international “ train category. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7/ 8/ 12/ 13 | FS/ OS/ SR/ UN/ SB/ TR/ PT/ LS/ SN |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | The Stored Variables values for t0 (X=2) are: NC_TRAIN = xxx xxxx xxxx xx1x M_LOADINGGAUGE ≠0 M_AXLELOAD= HS17 N_ITER ≠0 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL = NID_OPERATIONAL1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | For Message 129 no translation is needed. For Packet 11 the translation is: NID_OPERATIONAL = NID_OPERATIONAL1 NC_TRAIN set to xxx x1xx xxxx xxxx M_LOADINGGAUGE set to 0 M_AXLELOAD= 0100010 N_ITER set to 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | The RBC understands and acknowledges this set of Train Data. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | - | N/A | N/A | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | One of the applicable mode |
| M_LEVEL | 3 | FINITE VALUE | One of the applicable level |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | NID_OPERATIONAL = NID_OPERATIONAL1 |
| NC_TRAIN | 15 | FINITE VALUE | Other International Train category |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | 0 | - |
| M_AXLELOAD | 7 | 0100010 | 17 t |
| M_AIRTIGHT | 2 | FINITE VALUE | Airtight system presence |
| N_ITER | 5 | 0 | - |
| M_TRACTION (k) | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available |

| Step 3: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7/ 8/ 12/ 13 | FS/ OS/ SR/ UN/ SB/ TR/ PT/ LS/ SN |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | The transmission of Data Train (message 129 containing packet 11) by the on-board equipment to the RBC certified with X=1 instead of sending the Train running number (message 136 containing Packet 5) |  | 8 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SB/ NL |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment sends the Train Running number to the X=1 RBC using message 129 with packet 11 instead of message 136 with packet 5. Clause 3.18.4.5.4 shall be replaced with: Only if valid Train Data is available: following any entry/modification of the train running number when a communication session is already established, the ERTMS/ETCS on-board equipment shall send the Train Data to the RBC. The RBC (X=1) understand correctly and acknowledges the reception of this set of Train Data with message 8: “Acknowledgment of Train Data”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.2 |  |
|  |  | Subset-026-6.6.3.3.2 Table - row 4 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The transmitted radio message 129 with packet 11 is RECORDED. The acknowledgment of the RBC, message 8 is RECORDED. |
| Constraints | Radio communication session is established. Entry/modification of the Train Running Number by driver Only for SB mode: Train at standstill and A Cab is active . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | VALID |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL1 = VALID |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver enters/modifies the "Train Running Number" [MAIN]. (NID_OPERATIONAL2) | I | DMI | - | NID_OPERATIONAL1 ≠ NID_OPERATIONAL2 Only for SB mode: Train at standstill with a cab active. | N/A | N/A | - |
| 2 | N/A | N/A | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded | O | JRU | - | NID_OPERATIONAL2 | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | The RBC (X=1) is informed about the new train running number with packet 11 where NID_OPERATIONAL2 variable is included. Translation of packet 11 tested in FT6060304.6, 7. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | Check that message 136 with packet 5 is not recorded. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | The RBC receives the information correctly and acknowledges this set of Train Data. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 3: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | One of the applicable mode |
| M_LEVEL | 3 | FINITE VALUE | One of the applicable level |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | = NID_OPERATIONAL2 |
| NC_TRAIN | 15 | FINITE VALUE | International Train category |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | 0 | - |
| M_AXLELOAD | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | Airtight system presence |
| N_ITER | 5 | 0 | - |
| M_TRACTION (k) | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available |

| Step 5: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | CHANGED ( VALID |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | The transmission of Train Data (message 129 containing packet 11) by the on-board equipment to the RBC certified with X=1 instead of sending the Train running number (message 136 containing Packet 5) |  | 9 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SB/ NL |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment sends the Train Running number to the X=1 RBC using message 129 with packet 11 instead of message 136 with packet 5. Clause 3.18.4.5.4 shall be replaced with: Only if valid Train Data is available: following the successful establishment of a communication session when valid train running number is already available, the ERTMS/ETCS on-board equipment shall send the Train Data to the RBC. The RBC (X=1) understand correctly and acknowledges the reception of this set of Train Data with message 8: “Acknowledgment of Train Data”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.1.1 |  |
|  |  | Subset-026-6.6.3.4.2 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 3 |  |
|  |  | Subset-026-6.6.3.3.2 Table - row 4 |  |
|  |  | Subset-026-6.6.3.3.2 Table - row 16 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 20 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 24 |  |
|  |  | Subset-026-6.6.3.4.5 [6] |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The establishment of a radio communication session initiated by the on-board equipment is recorded. The session established report is sended by the onboard to the trackside including its telephone numbers. The transmitted radio message 129 with packet 11 is RECORDED. The acknowledgment of the RBC, message 8 is RECORDED. |
| Constraints | Radio communication session not yet established. A valid Train running number is available. Only for SB mode: Train at standstill and A Cab is active . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | NOT ESTABLISHED | - |
| Train Data | STORED | VALID |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | NID_OPERATIONAL = NID_OPERATIONAL1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | Type of action according to table 6.6.3.4.5 :[U] | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Configuration Determination" (NID_MESSAGE=32) is received | I | RTM | 001 0000 | Message 32: “RBC/RIU System Version” Type of action according to table 6.6.3.3.2:[U] | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | Versions are compatible Communication session established for on-board. | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 3 is transmitted. | O | RTM | 001 0000 | Type of action according to table 6.6.3.4.5:[U][6] | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | Communication session established for trackside. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | The RBC (X=1) is informed about the new train running number with packet 11 where NID_OPERATIONAL = NID_OPERATIONAL1 variable is included. Translation of packet 11 tested in FT6060304.6 or 7. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | Check that message 136 with packet 5 is not recorded. | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | The RBC receives the information correctly and acknowledges this set of Train Data. | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 3: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 5: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | 82 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RBC |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | Version of the ERTMS/ETCS language. |

| Step 7: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | 74 | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_PACKET | 8 | 3 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_RADIO(k) | 64 | FINITE VALUE | - |

| Step 9: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | One of the applicable mode |
| M_LEVEL | 3 | FINITE VALUE | One of the applicable level |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | NID_OPERATIONAL = NID_OPERATIONAL1 |
| NC_TRAIN | 15 | FINITE VALUE | International Train category |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | 0 | - |
| M_AXLELOAD | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | Airtight system presence |
| N_ITER | 5 | 0 | - |
| M_TRACTION (k) | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available |

| Step 11: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | ESTABLISHED | CHANGED |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Exception: If the train running number is received from the RBC (X=1), the Train Data is not sent back to the RBC by the on-board equipment. |  | 10 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that : - The on-board equipment receives from the RBC a new Train Running Number. - Message 24 with packet 140 is sent by the X=1 RBC and NID_OPERATIONAL is translated according to T [10]. - The On-board equipment does not send back to the RBC the set of new Train Data. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.3 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 41 |  |
|  |  | Subset-026-6.6.3.2.3 [10] |  |
|  |  | Subset-026-7.4.2.36 Packet Number 140: Train running number from RBC |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : - The radio message 24 with packet 140 is RECORDED. - The recorded NID_OPERATIONAL has been translated (T[10]: FFFF1234 to 1234FFFF) - Message 129 with packet 11 is NOT RECORDED. |
| Constraints | Radio communication session is established |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | Train data is available |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL1 = VALID |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 140 (NID_OPERATIONAL2=FFFF1234) is received. | I | RTM | 001 0000 | NID_OPERATIONAL1 ≠ NID_OPERATIONAL2 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | NID_OPERATIONAL2 is recorded following the translation T[10]: 1234FFFF | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted. | O | RTM | - | The on-board does not send back to the RBC the Train Data. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| M_ACK | 1 | 0 | No ACK required |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| NID_PACKET | 8 | 140 | Train Running Number from RBC |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FFFF1234 | NID_OPERATIONAL2 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | CHANGED (NID_OPERATIONAL2) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission of the message 132: “MA Request” by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated ” T[4]” & data is unchanged “U[1]” when EVC in LS mode. |  | 11 |
| Applicable Mode/Level Combinations | L2 / L3: LS |  |  |
| Target of Test | The target of this test is to check that when in LS mode: A situation which leads to shorten the current MA and track description to the current position of the on-board equipment or the new stop location. The on-board equipment informs the RBC/RIU by sending a radio message 132 “MA request” that is translated for the RBC system version X=1 as: the variable Q_MARQSTREASON is being replaced with Q_TRACKDEL. The transmitted value to X=1 RBC: Q_MARSQREASON = x1xxx into Q_TRACKDEL set to 1. The position report packet is sent with M_MODE=12. The RBC/RIU understands correctly the received message and sends a valid Movement Authority. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 1 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 10 |  |
|  |  | Subset-026-6.6.3.4.5 [1] |  |
|  |  | Subset-026-6.6.3.4.5 [4] |  |
|  |  | Subset-026-7.4.2.36 Packet Number 140: Train running number from RBC |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode profile |  |
|  |  | Subset-026-8.6.3 Message 132: MA Request |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Data from message 132 and packet 0 are translated correctly by the on-board and the trackside send the requested information. Check on the JRU that: The Message 132 :MA Request with packet 0 is RECORDED. The Message 3: Movement Authority is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | No linked balise group message is received | I | BTM | - | The balise group missed was linked with service brake reaction | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded. | O | TIU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Service Brake " is displayed. | O | DMI | - | Symbol ST01 | N/A | N/A | - |
| 4 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted to a system version X=1 RBC. | O | RTM | 001 0000 | Q_TRACKDEL = 1 For Message132, data is translated as: Q_MARQSTREASON is replaced with Q_TRACKDEL And value x1xxx is set to 1 (track description deleted) When the applicable mode is LS, M_MAMODE=12 then, no translation is effected. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 and packet 80 is received | I | RTM | 001 0000 | Translation of Packets 21, 27 and 80 out of scope of test case target. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 7: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| Q_TRACKDEL | 1 | 1 | Track description deleted |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | If the applicable mode is LS, M_MAMODE=12 then, no translation is effected |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 9: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NC_DIFF(n) | 4 | FINITE VALUE | - |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| NC_DIFF(k,m) | 4 | FINITE VALUE | - |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 12 | If the applicable mode is LS, M_MAMODE=12 then, no translation is effected |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 1 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Movement Authority | STORED | UPDATED |
| Gradient Profile | STORED | UPDATED |
| International SSP | STORED | UPDATED |
| Mode Profile | STORED | UPDATED |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |