| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3120300 (3.12.3: Text Transmission) Total: 20 Test case(s) |  |
| REF: | Subset-076-5-2-3120300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, Answer to Review001, Test case 6 deleted (the requirements are tested in other TCs), 6 TCs added: 6, 8, 13, 14, 15, 19 (before renumbering: 27, 26, 25, 23, 22, 21), TCs renumbered | MULTITEL |
| Version 2.9.2 16.03.4917 | All | Modification requested by Review001_Subset-076-5-2-3120300_v291 | MULTITEL |
| Version 2.9.3 29.11.2013 | TC3, TC5, TC11, TC17, TC18 | Modification requested by Review001_Subset-076-5-2-3120300_v292 | MULTITEL |
| Version 2.9.4 23.01.2014 | new TC21 | Apply of issue 1 from Review001_Subset-076-5-2-3120300_v293 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a distance (location) condition. The end condition is limited to a distance (location) condition. |  | 1 |
| Applicable Mode/Level Combinations | L0 : UN, SB, TR LNTC : SB, TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment starts to display the plain text message at the required location when the start condition for display is limited to a distance condition. The on-board equipment stops to display the plain text message at the required location when the end condition for display is limited to a distance condition. The on-board equipment displays the correct plain text message. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.6 |  |
|  |  | Subset-026-3.12.3.4.3.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Plain Text Information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |
|  |  | Subset-026-3.12.3.4.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The plain text message is displayed when the location to start the display is reached. The plain text message is removed when the location to stop the display is reached. The correct plain text message is displayed. |
| Constraints | Information about plain text message has been received. Level and Mode starting conditions for displaying the plain text message are fulfilled. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED | Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY<>32767 Distance (location of LRBG+D_TEXTDISPLAY) from where the text shall be displayed, but in front of the current train front end M_MODETEXTDISPLAY=1-14 The value should have the corresponding value for the tc starting mode. M_LEVELTEXTDISPLAY=0-4 The value should have the corresponding value for the tc starting level L_TEXTDISPLAY=<>32767 Length on which the text shall be displayed T_TEXTDISPLAY=1023 End event: the display of the text is not limited by the time M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=0 No confirmation required X_TEXT(L_TEXT) STORED Plain text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "plain text message" is displayed | O | DMI | The end conditions will be evaluated. D ≥ D_TEXTDISPLAY (third condition is fulfilled) The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The "plain text message" is removed | O | DMI | The length of displaying the text message refers to the start condition (location) D_TEXTDISPLAY D ≥ D_TEXTDISPLAY + L_TEXTDISPLAY The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED | N/A | N/A | - |
| 4 | N/A | N/A | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded | O | JRU | The deletion of the message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a location condition. The end condition is limited to a time condition. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC: SB, TR L1, L2, L3:FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment starts to display the plain text message at the required location when the start condition for display is limited to a distance condition. The on-board equipment stops to display the plain text message when the specified time has elapsed when the end condition for display is limited to a time condition. The on-board equipment displays the correct plain text message. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.4 |  |
|  |  | Subset-026-4.7.2 table - Output information Plain Text Information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The plain text message is displayed when the location to start the display is reached. The plain text message is removed when the specified time has elapsed. The correct plain text message is displayed. |
| Constraints | Information about plain text message has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED | Q_TEXTDISPLAY=0 or 1 Display message as soon as / until all or one events are fulfilled Does not matter because there is only one event D_TEXTDISPLAY≠ 32767 Distance (location of LRBG+D_TEXTDISPLAY) from where the text shall be displayed, but in front of the current train front end M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY=32767 End event: the display of the text is not distance limited T_TEXTDISPLAY≠ 1023 Duration for which a text shall be displayed M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=0 No confirmation required X_TEXT(L_TEXT) STORED Plain text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "plain text message" is displayed | O | DMI | The end conditions will be evaluated. D ≥ D_TEXTDISPLAY T = T0 The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | The "plain text message" is removed | O | DMI | T ≥ T0 + T_TEXTDISPLAY The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED | N/A | N/A | - |
| 4 | N/A | N/A | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded | O | JRU | The deletion of the message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified mode. The end condition is limited to a condition on exit from a specified mode. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC: SB, TR L1, L2, L3:FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment starts to display the plain text message when it enters in the specified mode when the start condition for display is limited to a mode condition. The on-board equipment stops to display the plain text message when it exits from a specified mode when the end condition for display is limited to a mode condition. The on-board equipment displays the correct plain text message. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.4 |  |
|  |  | Subset-026-4.7.2 table - Output information Plain Text Information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The plain text message is displayed when the on-board equipment enters in the specified mode. The plain text message is removed when the on-board equipment exits from the specified mode. The correct plain text message is displayed. |
| Constraints | Information about plain text message has been received. The specified modes in start condition and end condition should be identical. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED | Q_TEXTDISPLAY=0 or 1 Display message as soon as / until all or one events are fulfilled Does not matter because there is only one event D_TEXTDISPLAY=32767 Start event: the display of the text is not distance limited M_MODETEXTDISPLAY≠ 15 Start event: the text is displayed when entering in the specified mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY=32767 End event: the display of the text is not distance limited T_TEXTDISPLAY=1023 End event: the display of the text is not time limited M_MODETEXTDISPLAY≠ 15 End event: the text is removed when exiting from the specified mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=0 No confirmation required X_TEXT(L_TEXT) STORED Plain text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 *) M_MODE ≠ M_MODETEXTDISPLAY |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The Mode changes | - | - | Use one of the test cases of the features related to mode changes with for the new mode: M_MODE = M_MODETEXTDISPLAY (start condition) Use_FT4060300.3,8,9,11,16,17,19-31,35-38,40-44,46-50,52-56,58 OR Use_FT4060300.59,60,64,67 OR Use_FT5110200.12,13 OR Use_FT5190200.6 OR Use_FT5190300.2-4 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 2 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain text message" is displayed | O | DMI | The end conditions are evaluated. The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 3 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | The display of the message is RECORDED | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 4 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The Mode changes | - | - | Use one of the test cases of the features related to mode changes with for the new mode: M_MODE = M_MODETEXTDISPLAY (end condition) Use_FT4060300.3,8,9,11,16,17,19-31,35-38,40-44,46-50,52-56,58 OR Use_FT4060300.59,60,64,67 OR Use_FT5110200.12,13 OR Use_FT5190200.6 OR Use_FT5190300.2-4 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 5 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain text message" is removed | O | DMI | The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 6 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded | O | JRU | The deletion of the message is RECORDED | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV M_MODE ≠ M_MODETEXTDISPLAY (start event) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified level. The end condition is limited to a condition on exit from a specified level. |  | 4 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC : SB, TR L1, L2, L3:FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment starts to display the plain text message when it enters in the specified level when the start condition for display is limited to a level condition. The on-board equipment stops to display the plain text message when it exits from a specified level when the end condition for display is limited to a level condition. The on-board equipment displays the correct plain text message. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.4 |  |
|  |  | Subset-026-3.12.3.4.5 |  |
|  |  | Subset-026-4.7.2 table - Output information Plain Text Information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The plain text message is displayed when the on-board equipment enters in the specified level. The plain text message is removed when the on-board equipment exits from the specified level. The correct plain text message is displayed. |
| Constraints | Information about plain text message has been received. The specified levels in start condition and end condition should be identical. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED | Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY=32767 Start event: the display of the text is not distance limited M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY≠ 5 Start event: the text is displayed when entering in the specified level L_TEXTDISPLAY=32767 End event: the display of the text is not distance limited T_TEXTDISPLAY=1023 End event: the display of the text is not time limited M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY≠ 5 End event: the text is removed when exiting from the specified level Q_TEXTCONFIRM=0 No confirmation required X_TEXT(L_TEXT) STORED Plain text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 *) M_LEVEL ≠ M_LEVELTEXTDISPLAY |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The level changes immediately. | - | - | Use_FT5100100.7-13,15-17,19-21,23-32,34,36-37 For new level: M_LEVEL = M_LEVELTEXTDISPLAY (start condition) | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 2 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The "plain text message" is displayed | O | DMI | The end conditions are evaluated. The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 3 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 4 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The level changes immediately. | - | - | Use_FT5100100.7-13,15-17,19-21,23-32,34,36-37 For new level: M_LEVEL ≠ M_LEVELTEXTDISPLAY (end condition) | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 5 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The "plain text message" is removed | O | DMI | - | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 6 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded | O | JRU | The deletion of the message is RECORDED | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 7 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The level changes | - | - | Use_FT5100100.7-13,15-17,19-21,23-32,34,36-37 For new level: M_LEVEL = M_LEVELTEXTDISPLAY (start condition) | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 8 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The "plain text message" is NOT displayed | O | DMI | The plain text message ‘X_TEXT(L_TEXT)’ is NOT displayed. The text message was already displayed and the end condition was already fulfilled, the start location shall not be re-evaluated. | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 *) M_LEVEL ≠ M_LEVELTEXTDISPLAY |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. All of the Start / End conditions shall be fulfilled to display the message |  | 5 |
| Applicable Mode/Level Combinations | L1: FS |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment displays the message only when all of the Start / End conditions shall be fulfilled to display the message The on-board equipment displays the correct plain text message. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.3.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Plain Text Information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The on-board equipment displays the message only when all of the Start / End conditions shall be fulfilled to display the message The on-board equipment displays the correct plain text message. This means the message is displayed between D0+D_TEXTDISPLAY and D0+D_TEXTDISPLAY+L_TEXTDISPLAY where D0 is the location reference of the BG transmitting the text information or of the LRBG given in the message transmitting the text information. |
| Constraints | Information about plain text message has been received. Although the test can be done in any combination of modes and levels the test will start in L1 FS and end in L0 UN |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | Enough to perform the test |
| Plain Text Information (location based) | STORED | Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY different from 0 and 32767 Distance (location of LRBG+D_TEXTDISPLAY) from where the text shall be displayed, but in front of the current train front end M_MODETEXTDISPLAY=1 Display the text for On sight M_LEVELTEXTDISPLAY=2 Display the text for L1 L_TEXTDISPLAY=1000 Display the text for 1000 m T_TEXTDISPLAY=1023 The display of the text is not limited by the time M_MODETEXTDISPLAY=1 Remove the message when leaving OS M_LEVELTEXTDISPLAY=2 Remove the message when leaving L1 Q_TEXTCONFIRM=0 No confirmation required X_TEXT(L_TEXT) STORED Plain text message to display |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS | One Balise Group message containing packet 80 (D_MAMODE=0;M_MAMODE=0) is received | - | - | Balise message including mode profile for OS (D=0) is RECEIVED Use_FT4060300.46 On Sight is requested for the current location and the driver acknowledges within the driver acknowledgement time after the change to OS mode. Condition Id [40] | L1 | OS | - |
| 2 | L1 | OS | The "plain text message" is displayed | O | DMI | All start conditions are fulfilled The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED D1=D0+D_TEXTDISPLAY (D0 is the location reference of the BG transmitting the text information or of the LRBG given in the message transmitting the text information) | L1 | OS | - |
| 3 | L1 | OS | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | The display of the message is RECORDED | L1 | OS | - |
| 4 | L1 | OS | One Balise Group message containing packet 41 (D_LEVELTR=0¦ D_LEVELTR=32767,M_LEVELTR=0) is received | - | - | Balise message including transition to L0-UN (D=0) is RECEIVED This balise group should be located in the display area of the text message Use_FT4060300.3 ERTMS/ ETCS Level switches to level 0. Transition Id [21] | L0 | UN | - |
| 5 | L0 | UN | The "plain text message" is removed | O | DMI | When the L_TEXTDISPLAY is covered The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED D2=D0+D_TEXTDISPLAY+L_TEXTDISPLAY (D0 is the location reference of the BG transmitting the text information or of the LRBG given in the message transmitting the text information) | L0 | UN | - |
| 6 | L0 | UN | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded | O | JRU | The deletion of the message is RECORDED | L0 | UN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | Texts displayed on DMI, fixed text messages and the data entry dialogs are displayed in language selected by the driver |  | 6 |
| Applicable Mode/Level Combinations | L0 : UN, SB, TR LNTC : SB, TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The fixed texts are displayed in language selected by the driver. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Input information Selection of language |  |
|  |  | Subset-026-3.12.3.3.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that the fixed text message visible on the DMI are permanently displayed in the language chosen by the driver (i.e. DMI data is influenced by the chosen language). |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Fixed Text Information (location based) | STORED | Q_TEXTDISPLAY=0 Display message as soon as / until one of the events is fulfilled D_TEXTDISPLAY< 32767 Distance (location of LRBG+D_TEXTDISPLAY) from where the text shall be displayed, but in front of the current train front end M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY< 32767 Length on which the text shall be displayed T_TEXTDISPLAY=1023 The display of the text is not limited by the time M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=0 No confirmation required Q_TEXT STORED Fixed text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Language used to display information to the driver | STORED | <LANGUAGE_A> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| All labels, all FIXED text messages and ALL data entry dialogs are displayed in <LANGUAGE_A> A fixed text message is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Settings" [MAIN] | I | DMI | Language=<LANGUAGE_A> | N/A | N/A | - |
| 2 | N/A | N/A | The driver modifies "Language" | I | DMI | Selection of <LANGUAGE_B> | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=29) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The "fixed text message" is displayed | O | DMI | The FIXED text messages is displayed in <LANGUAGE_B> | N/A | N/A | - |
| 5 | N/A | N/A | The driver modifies "Language" | I | DMI | Selection of <LANGUAGE_A> | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=29) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The "fixed text message" is displayed | O | DMI | The FIXED text messages is displayed in <LANGUAGE_A> | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. Only one of the Start / End conditions shall be fulfilled to display the message |  | 7 |
| Applicable Mode/Level Combinations | L1 :FS |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment displays the message only when only one of the Start / End conditions shall be fulfilled to display the message The on-board equipment displays the correct plain text message. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.3.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Plain Text Information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The on-board equipment displays the message only when only one of the Start / End conditions shall be fulfilled to display the message The on-board equipment displays the correct plain text message. |
| Constraints | Information about plain text message has been received. Althoug the test can be done in any combination of modes and levels the test will start in L1 FS |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | Enough to perform the test |
| Plain Text Information (location based) | STORED | Q_TEXTDISPLAY=0 Display message as soon as / until one of the events is fulfilled D_TEXTDISPLAY=500 Distance (location of LRBG+D_TEXTDISPLAY) from where the text shall be displayed, but in front of the current train front end M_MODETEXTDISPLAY=1 Display the text for On sight M_LEVELTEXTDISPLAY=3 Display the text for L2 L_TEXTDISPLAY=1000 Display the text for 1000 m T_TEXTDISPLAY=1023 The display of the text is not limited by the time M_MODETEXTDISPLAY=0 Remove the message when leaving FS M_LEVELTEXTDISPLAY=2 Remove the message when leaving L1 Q_TEXTCONFIRM=0 No confirmation required X_TEXT(L_TEXT) STORED Plain text message to display |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "plain text message" is displayed | O | DMI | It is not necessary to switch to OS to display the message When distance condition is fulfilled The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | The "plain text message" is removed | O | DMI | It is not necessary to leave FS to remove the message When the L_TEXTDISPLAY is covered The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED | N/A | N/A | - |
| 4 | N/A | N/A | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded | O | JRU | The deletion of the message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 UNCHANGED | FS |
| ERTMS/ETCS level | 2 UNCHANGED | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about plain text messages. The plain text message is not displayed if the end condition is immediately fulfilled. |  | 8 |
| Applicable Mode/Level Combinations | L0 : UN, SB, TR LNTC : SB, TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The plain text message is not displayed if the end condition is immediately fulfilled. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.6 |  |
|  |  | Subset-026-4.7.2 table - Output information Plain Text Information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |
|  |  | Subset-026-3.12.3.4.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: No plain text message is displayed. |
| Constraints | Information about plain text message has been received. Level and Mode starting conditions for displaying the plain text message are fulfilled. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED | Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY<>32767 Distance (location of LRBG+D_TEXTDISPLAY) from where the text shall be displayed, but in front of the current train front end M_MODETEXTDISPLAY=1-14 The value should have the corresponding value for the tc starting mode. M_LEVELTEXTDISPLAY=0-4 The value should have the corresponding value for the tc starting level L_TEXTDISPLAY=0 Length on which the text shall be displayed T_TEXTDISPLAY=1023 End event: the display of the text is not limited by the time M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=0 No confirmation required X_TEXT(L_TEXT) STORED Plain text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | No "plain text message" is displayed | O | DMI | As soon as D ≥ D_TEXTDISPLAY (third start condition is fulfilled), the end conditions (L_TEXTDISPLAY=0) will be evaluated and the plain text message ‘X_TEXT(L_TEXT)’ is NOT DISPLAYED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed text messages. The fixed text message is displayed according to the start / end conditions. The start condition is limited to a location condition. The end condition is limited to a location condition. |  | 9 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3 : FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment starts to display the fixed text message at the required location when the start condition for display is limited to a distance condition. The on-board equipment stops to display the fixed text message at the required location when the end condition for display is limited to a distance condition. The on-board equipment displays the fixed text message in the language selected by the driver. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.3.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Fixed text information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The fixed text message is displayed when the location to start the display is reached. The fixed text message is removed when the location to stop the display is reached. The fixed text message is displayed according to the language selected by the driver. |
| Constraints | Information about fixed text message has been received. Table containing the fixed messages in the language selected by the driver (English) is STORED onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Fixed Text Information (location based) | STORED | Q_TEXTDISPLAY=0 Display message as soon as / until one of the events is fulfilled D_TEXTDISPLAY< 32767 Distance (location of LRBG+D_TEXTDISPLAY) from where the text shall be displayed, but in front of the current train front end M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY< 32767 Length on which the text shall be displayed T_TEXTDISPLAY=1023 The display of the text is not limited by the time M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=0 No confirmation required Q_TEXT STORED Fixed text message to display |
| ERTMS/ETCS level | 0 / 1 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "fixed text message" is displayed | O | DMI | D ≥ D_TEXTDISPLAY The fixed text message ‘Q_TEXT’ is DISPLAYED in English | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | The "fixed text message" is removed | O | DMI | D ≥ D_TEXTDISPLAY + L_TEXTDISPLAY The fixed text message ‘Q_TEXT’ is REMOVED | N/A | N/A | - |
| 4 | N/A | N/A | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded | O | JRU | The deletion of the message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed text messages. The fixed text message is displayed according to the start / end conditions. The start condition is limited to a location condition. The end condition is limited to a time condition. |  | 10 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3: FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment starts to display the fixed text message at the required location when the start condition for display is limited to a distance condition. The on-board equipment stops to display the fixed text message when the specified time has elapsed when the end condition for display is limited to a time condition. The on-board equipment displays the fixed text message in the language selected by the driver. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.3.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Fixed text information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The fixed text message is displayed when the location to start the display is reached. The fixed text message is removed when the specified time has elapsed. The fixed text message is displayed according to the language selected by the driver. |
| Constraints | Information about fixed text message has been received. Table containing the fixed messages in the language selected by the driver (English) is STORED onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Fixed Text Information (location based) | STORED | Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY< 32767 Distance (location of LRBG+D_TEXTDISPLAY) from where the text shall be displayed, but in front of the current train front end M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY=32767 End event: the display of the text is not distance limited T_TEXTDISPLAY ><1023 Duration for which a text shall be displayed M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=0 No confirmation required Q_TEXT STORED Fixed text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "fixed text message" is displayed | O | DMI | D ≥ D_TEXTDISPLAY T = T0 The fixed text message ‘Q_TEXT’ is DISPLAYED in the language selected by the driver. | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | The "fixed text message" is removed | O | DMI | T ≥ T0 + T_TEXTDISPLAY The fixed text message ‘Q_TEXT’ is REMOVED | N/A | N/A | - |
| 4 | N/A | N/A | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded | O | JRU | The deletion of the message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed text messages. The fixed text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified mode. The end condition is limited to a condition on exit from a specified mode. |  | 11 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3: FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment starts to display the fixed text message when it enters in the specified mode when the start condition for display is limited to a mode condition. The on-board equipment stops to display the fixed text message when it exits from a specified mode when the end condition for display is limited to a mode condition. The on-board equipment displays the fixed text message in the language selected by the driver. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.3.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Fixed text information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The fixed text message is displayed when the on-board equipment enters in the specified mode. The fixed text message is removed when the on-board equipment exits from the specified mode. The fixed text message is displayed according to the language selected by the driver. |
| Constraints | Information about fixed text message has been received. The specified modes in start condition and end condition should be identical. Table containing the fixed messages in the language selected by the driver (English) is STORED onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / LS / OS / SR / UN / SB / TR / PT / RV M_MODE ≠ M_MODETEXTDISPLAY_1 (start condition of text display) |
| Fixed Text Information (not location based) | STORED | Q_TEXTDISPLAY=0 Display message as soon as / until one of the events is fulfilled D_TEXTDISPLAY=32767 Start event: the display of the text is not distance limited M_MODETEXTDISPLAY M_MODETEXTDISPLAY_1><15 Start event: the text is displayed when entering in the specified mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY=32767 End event: the display of the text is not distance limited T_TEXTDISPLAY=1023 End event: the display of the text is not time limited M_MODETEXTDISPLAY M_MODETEXTDISPLAY_2><15 End event: the text is removed when exiting from the specified mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=0 No confirmation required Q_TEXT STORED Fixed text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The mode changes. | - | - | Use one of the test cases of the features related to mode changes with: For the old mode : M_MODE ≠ M_MODETEXTDISPLAY (start condition) For the new mode : M_MODE = M_MODETEXTDISPLAY_1 (start condition) Use_FT4060300.3,8,9,11,16,17,19-31,35-38,40-44,46-50,52-56,58 OR Use_FT4060300.59,60,64,67 OR Use_FT5110200.12,13 OR Use_FT5190200.6 OR Use_FT5190300.2-4 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 2 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "fixed text message" is displayed | O | DMI | The fixed text message ‘Q_TEXT’ is DISPLAYED in the language selected by the driver | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 3 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | The display of the message is RECORDED | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 4 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The mode changes. | - | - | Use one of the test cases of the features related to mode changes with for the new mode: M_MODE = M_MODETEXTDISPLAY_2 (end condition) Use_FT4060300.3,8,9,11,16,17,19-31,35-38,40-44,46-50,52-56,58 OR Use_FT4060300.59,60,64,67 OR Use_FT5110200.12,13 OR Use_FT5190200.6 OR Use_FT5190300.2-4 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 5 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "fixed text message" is removed | O | DMI | The fixed text message ‘Q_TEXT’ is REMOVED | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 6 | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded | O | JRU | The deletion of the message is RECORDED | L0/LNTC/L1/L2/L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV M_MODE ≠ M_MODETEXTDISPLAY_2 (end condition of text display) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed text messages. The fixed text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified level. The end condition is limited to a condition on exit from a specified level. |  | 12 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3: FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment starts to display the fixed text message at when it enters in the specified level when the start condition for display is limited to a level condition. The on-board equipment stops to display the fixed text message when it exits from a specified level when the end condition for display is limited to a level condition. The on-board equipment displays the fixed text message in the language selected by the driver. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.3.1 |  |
|  |  | Subset-026-4.7.2 table - Output information Fixed text information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The fixed text message is displayed when the on-board equipment enters in the specified level. The fixed text message is removed when the on-board equipment exits from the specified level. The fixed text message is displayed according to the language selected by the driver. |
| Constraints | Information about fixed text message has been received. The specified levels in start condition and end condition should be identical. Table containing the fixed messages in the language selected by the driver (English) is STORED onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Fixed Text Information (not location based) | STORED | Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY=32767 Start event: the display of the text is not distance limited M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY M_LEVELTEXTDISPLAY_1 < 5 Start event: the text is displayed when entering in the specified level L_TEXTDISPLAY=32767 End event: the display of the text is not distance limited T_TEXTDISPLAY=1023 End event: the display of the text is not time limited M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY M_LEVELTEXTDISPLAY_2 < 5 End event: the text is removed when exiting from the specified level Q_TEXTCONFIRM=0 No confirmation required Q_TEXT STORED Fixed text message to display |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 M_LEVEL ≠ M_LEVELTEXTDISPLAY_1 (start event of text display) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The level changes. | - | - | Use_FT5100100.7-13,15-17,19-21,23-32,34,36-37 For new level: M_LEVEL = M_LEVELTEXTDISPLAY_1 (start condition) | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 2 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The "fixed text message" is displayed | O | DMI | The fixed text message ‘Q_TEXT’ is DISPLAYED in the language selected by the driver. | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 3 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | The display of the message is RECORDED | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 4 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The level changes. | - | - | Use_FT5100100.7-13,15-17,19-21,23-32,34,36-37 For new level: M_LEVEL ≠ M_LEVELTEXTDISPLAY_2 (end condition) | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 5 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | The "fixed text message" is removed | O | DMI | The fixed text message ‘Q_TEXT’ is REMOVED | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |
| 6 | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded | O | JRU | The deletion of the message is RECORDED | L0/LNTC/L1/L2/L3 | FS/OS/SR/UN/SB/TR/PT/NL/LS/RV | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 M_LEVEL ≠ M_LEVELTEXTDISPLAY_2(end event of text display) | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed text messages. The text message requires EB application when end conditions are reached and the message has not been confirmed by the driver. The text message is displayed according to the display conditions. As the message is not acknowledged when the end conditions are reached, the EB is applied. As soon as the driver confirms the message, the EB is released and the message is removed. |  | 13 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : the EB is applied when end conditions are reached for message requiring EB application when end conditions are reached and the message has not been confirmed by the driver. The message is displayed until it is confirmed by the driver. The EB is released and the message is removed as soon as the message is confirmed by the driver. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.7 |  |
|  |  | Subset-026-3.12.3.4.7.1 |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |
|  |  | Subset-026-4.7.2 table - Output information Fixed text information |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of fixed text information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The text message is displayed according to the display conditions. The text message is removed when it is confirmed by the driver. Check on TIU that: The EB is applied when end conditions to display message are reached. The EB is released when the driver confirms the message. Check on JRU that: Emergency brake application is recorded Emergency brake revocation is recorded The display of the message is recorded The cancellation of the message is recorded |
| Constraints | Information about fixed text message has been received. The text message requires EB application when end conditions are reached and the message has not been confirmed by the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Fixed Text Information (location based) | STORED | Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY<> 32767 Distance in dependency from the LRBG from where the text shall be displayed M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY<>32767 Length on which the text shall be displayed T_TEXTDISPLAY=1023 End event: the display of the text is not time limited M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=11 Confirmation required: command application of the emergency brake when display end condition is fulfilled, unless the text has already been acknowledged by the driver |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "fixed text message" is displayed | O | DMI | If train front end at location of LRBG+ D_TEXTDISPLAY D ≥ location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | Emergency brake commanded | O | TIU | As soon as end conditions are reached. D ≥ location of LRBG + D_TEXTDISPLAY + L_TEXTDISPLAY The EB is APPLIED | N/A | N/A | - |
| 4 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | As soon as end conditions are reached. The EB application is RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | As soon as end conditions are reached. The EB application is DISPLAYED | N/A | N/A | - |
| 6 | N/A | N/A | The driver acknowledges | I | DMI | The driver confirms the message Before standstill | N/A | N/A | - |
| 7 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The confirmation of the message is RECORDED | N/A | N/A | - |
| 8 | N/A | N/A | Emergency brake not commanded | O | TIU | As soon as the driver confirms the message The EB is RELEASED | N/A | N/A | - |
| 9 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | As soon as the driver confirms the message The EB revocation is RECORDED | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | As soon as the driver confirms the message The message is REMOVED The EB application is NOT DISPLAYED | N/A | N/A | - |
| 11 | N/A | N/A | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded | O | JRU | The removal of the text message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed or plain text messages. The text message requires driver acknowledgement to be removed. The text message is displayed according to the display conditions. As the message is acknowledged, the message is removed. |  | 14 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : the message is removed as soon as the message has been confirmed by the driver. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |
|  |  | Subset-026-3.12.3.4.3.2 |  |
|  |  | Subset-026-3.12.3.4.3.2 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The text message is displayed according to the display conditions. The text message is removed when it is confirmed by the driver. Check on JRU that: The display of the message is recorded The cancellation of the message is recorded |
| Constraints | Information about fixed or plain text message has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED/NOT STORED | If STORED, Fixed Text Information is not STORED and: Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY<> 32767 Distance in dependency from the LRBG from where the text shall be displayed M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY<>32767 Length on which the text shall be displayed T_TEXTDISPLAY=1023 End event: the display of the text is not time limited M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=10 Confirmation required: command application of the service brake when display end condition is fulfilled, unless the text has already been acknowledged by the driver Q_CONFTEXTDISPLAY=0, Driver acknowledgement always ends the text display, regardless of the end condition |
| Fixed Text Information (location based) | STORED/NOT STORED | If STORED, Plain Text Information is not STORED and see Plain Text Information for variables’ description |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| SB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "plain or fixed text message" is displayed | O | DMI | If train front end at location of LRBG+ D_TEXTDISPLAY D ≥ location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "plain or fixed text message" | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The confirmation of the message is RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | The "plain or fixed text message" is removed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is NOT displayed | O | DMI | The SB application is NOT DISPLAYED | N/A | N/A | - |
| 7 | N/A | N/A | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded | O | JRU | The removal of the text message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed or plain text messages. The text message requests driver acknowledgement and report of driver acknowledgement. As soon as the driver acknowledges the message, the report of driver acknowledgment, including the text message identifier, is sent to the RBC referenced in the request.. |  | 15 |
| Applicable Mode/Level Combinations | L2, L3 : FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : As soon as the driver acknowledges the message, the report of driver acknowledgment, including the text message identifier, is sent to the RBC referenced in the request. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.5.2 |  |
|  |  | Subset-026-8.6.16 Message 158: Text Message Acknowledged by Driver |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM that: The report of driver acknowledgement, including including the text message identifier, is sent to the RBC referenced in the request. Check on JRU that: The message sent to the RBC is RECORDED |
| Constraints | The on-board equipment has received information about fixed or plain text messages. The text message requests driver acknowledgement and report of driver acknowledgement. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED/NOT STORED | If STORED, Fixed Text Information is not STORED and: Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY<> 32767 Distance in dependency from the LRBG from where the text shall be displayed M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY<>32767 Length on which the text shall be displayed T_TEXTDISPLAY=1023 End event: the display of the text is not limited by time M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=10 Confirmation required: command application of the service brake when display end condition is fulfilled, unless the text has already been acknowledged by the driver Q_CONFTEXTDISPLAY=0 Driver acknowledgement always ends the text display, regardless of the end condition Q_TEXTREPORT=1 Driver acknowledgement report required NID_TEXTMESSAGE<>255 Text message identifier. Identity of a text message from trackside to be used in a report of driver acknowledgement to the RBC NID_C<>1023 Identity number of the country or region. Code used to identify the country or region in which the balise group, the RBC or the RIU is situated. These need not necessarily follow administrative or political boundaries. NID_RBC<>16382 RBC ETCS identity number. This variable provides the identity of the RBC belonging to NID_C. |
| Fixed Text Information (location based) | STORED/NOT STORED | If STORED, Plain Text Information is not STORED and see Plain Text Information for variables’ description |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "plain or fixed text message" is displayed | O | DMI | As soon as train front end reaches location of LRBG+ D_TEXTDISPLAY D ≥ location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "plain or fixed text message" | I | DMI | The driver confirms the text message | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The confirmation of the message is RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Text message acknowledged by driver" (NID_MESSAGE=158) containing packet 0 is transmitted | O | RTM | As soon as the message is confirmed by the driver The report of driver acknowledgement is SENT to the RBC specified by NID_RBC in starting conditions Packet 1 must be used if position report is based on 2 BG | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | The report of driver acknowledgement is RECORDED | N/A | N/A | - |

| Step 5: Radio Message 158 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 158 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_TEXTMESSAGE | 8 | <>255 | Same as in starting conditions |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR/ UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed text messages. The text message requires SB application when end conditions are reached and the message has not been confirmed by the driver. The text message is displayed according to the display conditions. As the message is not acknowledged when the end conditions are reached, the SB is applied. As soon as the driver confirms the message, the SB is released and the message is removed. |  | 16 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : the SB is applied when end conditions are reached for message requiring SB application when end conditions are reached and the message has not been confirmed by the driver. The message is displayed until it is confirmed by the driver. The SB is released and the message is removed as soon as the message is confirmed by the driver. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.7.1 |  |
|  |  | Subset-026-3.12.3.4.7 |  |
|  |  | Subset-026-3.14.1.7.5 |  |
|  |  | Subset-026-3.14.1.8 |  |
|  |  | Subset-026-4.7.2 table - Output information Fixed text information |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of fixed text information |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The text message is displayed according to the display conditions. The text message is removed when it is confirmed by the driver. Check on TIU that: The SB is applied when end conditions to display message are reached. The SB is released when the driver confirms the message. Check on JRU that: Service brake application is recorded Service brake revocation is recorded The display of the message is recorded The cancellation of the message is recorded |
| Constraints | Information about fixed text message has been received. The text message requires SB application when end conditions are reached and the message has not been confirmed by the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Fixed Text Information (location based) | STORED | Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY<> 32767 Distance in dependency from the LRBG from where the text shall be displayed M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY<>32767 Length on which the text shall be displayed T_TEXTDISPLAY=1023 End event: the display of the text is not time limited M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=10 Confirmation required: command application of the service brake when display end condition is fulfilled, unless the text has already been acknowledged by the driver |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| SB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "fixed text message" is displayed | O | DMI | If train front end at location of LRBG+ D_TEXTDISPLAY D ≥ location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | Service brake commanded | O | TIU | As soon as end conditions are reached. D ≥ location of LRBG + D_TEXTDISPLAY + L_TEXTDISPLAY The SB is APPLIED | N/A | N/A | - |
| 4 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | As soon as end conditions are reached. The SB application is RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | As soon as end conditions are reached. The SB application is DISPLAYED | N/A | N/A | - |
| 6 | N/A | N/A | The driver acknowledges | I | DMI | The driver confirms the message Before standstill | N/A | N/A | - |
| 7 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The confirmation of the message is RECORDED | N/A | N/A | - |
| 8 | N/A | N/A | Service brake not commanded | O | TIU | As soon as the driver confirms the message The SB is RELEASED | N/A | N/A | - |
| 9 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | As soon as the driver confirms the message The SB revocation is RECORDED | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | As soon as the driver confirms the message The message is REMOVED The SB application is NOT DISPLAYED | N/A | N/A | - |
| 11 | N/A | N/A | The "fixed text message" is removed | O | DMI | As soon as the message is confirmed by the driver The text message is REMOVED | N/A | N/A | - |
| 12 | N/A | N/A | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded | O | JRU | The removal of the text message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| SB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about a fixed or plain text message. The text message requires to be confirmed by the driver. The text message is displayed according to the display conditions. A newer message of the same class which requires no driver confirmation is received and displayed but it is hidden behind the previous one. Another message to be confirmed is received and displayed. The third message is hidden until the first one is confirmed. |  | 17 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : Messages to be confirmed by the driver are not hidden behind newer messages of the same class which require no driver confirmation. If there is more than one text message for which a driver’s acknowledgement is required, they shall be managed according to a FIFO principle. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.1.10 |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The first message to be confirmed by the driver is displayed according to the display conditions. The first message is not hidden behind newer messages of the same class which require no driver confirmation. The first message is not hidden behind newer messages which require driver confirmation. |
| Constraints | Information about fixed or plain text message has been received. The message requires a confirmation by the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Radio communication session | ESTABLISHED | Only if the test is done in L2 / L3 and the messages are received by radio |
| Plain Text Information (location based) | STORED/NOT STORED | If STORED, Fixed Text Information is not STORED and see Fixed Text Information for variables’ description |
| Fixed Text Information (location based) | STORED/NOT STORED | If STORED, Plain Text Information is not STORED and: The end condition is only the driver acknowledgement All the other end conditions shall be not restrictive. Q_TEXTCLASS =00 or 01 Q_TEXTCONFIRM=01 Confirmation required |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if the test is done in L2 / L3 and the messages are received by radio |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is displayed | O | DMI | When start conditions are fulfilled: The first text message is DISPLAYED This message should not be confirmed yet. | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the first message is RECORDED | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | One Balise Group message containing packet 72 (Q_TEXTCONFIRM=0) is received | I | BTM | If fixed text message is used, packet 76 is sent instead of packet 72. A newer message of the same class as the previous one which requires no driver confirmation is received and its start conditions to be displayed are fulfilled Q_TEXTCLASS = 00 or 01 (Same as in start conditions) Q_TEXTCONFIRM = 00 (No confirmation required) End condition shall be not restrictive to produce the displaying of this text message after the other two have been confirmed. | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=72; Q_TEXTCONFIRM=0) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The on-board device changes into Level 1/2/3 | - | - | Only if in LNTC Execution of level transition from LNTC to L1/L2/L3 at announced position Use_FT5100100.17 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 6 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is displayed | O | DMI | The second message is hidden behind the first one. | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 7 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | One Balise Group message containing packet 72 (Q_TEXTCONFIRM=1) is received | I | BTM | If fixed text message is used, packet 76 is sent instead of packet 72. Another message to be confirmed is received and its start conditions to be displayed are fulfilled Q_TEXTCLASS = 00 or 01 (Same as in start conditions) Q_TEXTCONFIRM = 01 (Confirmation Required) All the other end conditions shall be not restrictive. | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 8 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=72; Q_TEXTCONFIRM=1) is recorded | O | JRU | - | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 9 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is displayed | O | DMI | The third message is hidden behind the first one | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 10 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The driver acknowledges "plain or fixed text message" | I | DMI | The driver confirms the first message | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 11 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The confirmation of the first message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 12 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is removed | O | DMI | The first message is REMOVED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 13 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded | O | JRU | The removal of the first message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 14 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is displayed | O | DMI | As soon as the driver confirms the first message The third message appears. | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 15 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the third message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 16 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The driver acknowledges "plain or fixed text message" | I | DMI | The driver confirms the third message | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 17 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The confirmation of the third message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 18 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is removed | O | DMI | The third message is REMOVED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 19 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded | O | JRU | The removal of the third message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 20 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is displayed | O | DMI | The second message appears | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 21 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the second message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 22 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is removed | O | DMI | When the end conditions of the second message are fulfilled The second message is removed If the other messages have not been confirmed before the end conditions of the second message this one may not be presented. | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 23 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded | O | JRU | The removal of the second message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | If fixed text message is used, packet 76 is sent instead of packet 72 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 0 / 1 | Same as in start conditions |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | 0 | No confirmation required |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | If fixed text message is used, packet 76 is sent instead of packet 72 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 0 / 1 | Same as in start conditions |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | 1 | Confirmation required |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Radio communication session | ESTABLISHED | Only if the test is done in L2 / L3 and the messages are received by radio |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if the test is done in L2 / L3 and the messages are received by radio |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed or plain text messages. The text message regards auxiliary information and requires to be confirmed by the driver. The text message is displayed according to the display conditions. A newer message regarding important information which requires no driver confirmation is received and is not displayed although it regards important information |  | 18 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : Messages containing auxiliary information to be confirmed by the driver are not hidden behind newer messages containing important information that do not require driver confirmation. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |
|  |  | Subset-026-3.12.3.1.10 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The first message to be confirmed by the driver is displayed according to the display conditions. The first message is not hidden behind newer messages of higher class but that require no driver confirmation. |
| Constraints | Information about fixed or plain text message has been received. The message requires a confirmation by the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Radio communication session | ESTABLISHED | Only if the test is done in L2 / L3 and the messages are received by radio |
| Plain Text Information (location based) | STORED/NOT STORED | If STORED, Fixed Text Information is not STORED and see Fixed Text Information for variables’ description |
| Fixed Text Information (location based) | STORED/NOT STORED | If STORED, Plain Text Information is not STORED and: The end condition is only the driver acknowledgement All the other end conditions shall be not restrictive. Q_TEXTCLASS 00 Auxiliary information Q_TEXTCONFIRM 01 Confirmation required |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if the test is done in L2 / L3 and the messages are received by radio |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is displayed | O | DMI | When start conditions are fulfilled: The first text message is DISPLAYED This message should not be confirmed yet. | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the first message is RECORDED | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | One Balise Group message containing packet 72 (Q_TEXTCLASS=1; Q_TEXTCONFIRM=0) is received | I | BTM | If fixed text message is used, packet 76 is sent instead of packet 72. A newer message of a higher class than the previous one which requires no driver confirmation is received and its start conditions to be displayed are fulfilled Q_TEXTCLASS = 01 (Important information) Q_TEXTCONFIRM = 00 (No confirmation required) End condition should not exceed the conditions of message one | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=72; Q_TEXTCLASS=1; Q_TEXTCONFIRM=0) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The on-board device changes into Level 1/2/3 | - | - | Only if in LNTC Execution of level transition from LNTC to L1/L2/L3 at announced position Use_FT5100100.17 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 6 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is displayed | O | DMI | The second message is hidden behind the first one. | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 7 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The driver acknowledges "plain or fixed text message" | I | DMI | The driver confirms the first message | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 8 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The confirmation of the first message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 9 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | The "plain or fixed text message" is removed | O | DMI | The first message is REMOVED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |
| 10 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded | O | JRU | The removal of the first message is RECORDED | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / NL / LS / RV | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | If fixed text message is used, packet 76 is sent instead of packet 72 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 1 | Important Information |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | 0 | No confirmation required |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Radio communication session | ESTABLISHED | Only if the test is done in L2 / L3 and the messages are received by radio |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if the test is done in L2 / L3 and the messages are received by radio |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed or plain text messages. The text message requires SB application when end conditions are reached and the message has not been confirmed by the driver. The text message requires driver acknowledgement and one or more events to be removed. The text message is displayed according to the display conditions. As the message is acknowledged and the end conditions are reached, the message is removed. |  | 19 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3 :FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : the message is removed as soon as both the end conditions are reached and the message has been confirmed by the driver. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.4.7.2 |  |
|  |  | Subset-026-3.12.3.4.3.2 |  |
|  |  | Subset-026-3.12.3.4.3.2 b) |  |
|  |  | Subset-026-3.12.3.4.7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The text message is displayed according to the display conditions. The text message is removed when the end conditions are reached and it is confirmed by the driver. Check on JRU that: The display of the message is recorded The cancellation of the message is recorded |
| Constraints | Information about fixed or plain text message has been received. The text message requires SB application when end conditions are reached and the message has not been confirmed by the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED/NOT STORED | If STORED, Fixed Text Information is not STORED and: Q_TEXTDISPLAY=1 Display message as soon as / until all events are fulfilled D_TEXTDISPLAY<> 32767 Distance in dependency from the LRBG from where the text shall be displayed M_MODETEXTDISPLAY=15 Start event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 Start event: the display of the text is not limited by the level L_TEXTDISPLAY<>32767 Length on which the text shall be displayed T_TEXTDISPLAY=1023 End event: the display of the text is not time limited M_MODETEXTDISPLAY=15 End event: the display of the text is not limited by the mode M_LEVELTEXTDISPLAY=5 End event: the display of the text is not limited by the level Q_TEXTCONFIRM=10 Confirmation required: command application of the service brake when display end condition is fulfilled, unless the text has already been acknowledged by the driver |
| Fixed Text Information (location based) | STORED/NOT STORED | If STORED, Plain Text Information is not STORED and see Plain Text Information for variables’ description |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| SB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "plain or fixed text message" is displayed | O | DMI | If train front end at location of LRBG+ D_TEXTDISPLAY D ≥ location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "plain or fixed text message" | I | DMI | Before reaching end conditions, the driver confirms the message | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The confirmation of the message is RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | The "plain or fixed text message" is removed | O | DMI | As soon as the end conditions are reached D ≥ location of LRBG + D_TEXTDISPLAY + L_TEXTDISPLAY The message is REMOVED | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is NOT displayed | O | DMI | The SB application is NOT DISPLAYED | N/A | N/A | - |
| 7 | N/A | N/A | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded | O | JRU | The removal of the text message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.3: Text Transmission |  | 3120300 |
| Test Case of Feature | The on-board equipment has received information about fixed or plain text messages: The first message to be displayed is an auxiliary information. The second message be displayed is an important information. The third message to be displayed is an auxiliary information. The first message is displayed. When the second message has to be displayed, it hides the first one. When the third message has to be displayed, it is hidden by the second one. When the second message is removed according to its end condition for display, the third message reappears. The appearance of the auxiliary information message is not the same than the appearance of the important information message. |  | 20 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR LNTC:SB,TR L1, L2, L3 : FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that : Messages are displayed or hidden according to their class. Hidden messages reappear once other messages are deleted. The appearance of a message depends on the class. The function Manage Text Display to the driver is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.3.1.10 |  |
|  |  | Subset-026-4.5.2.1 table - Manage Text Display to the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The first message is displayed until the second one is displayed. The second message is displayed until its end conditions for display are reached. The third message appears when the second one is removed. The appearance of the first and third messages is not the same than the appearance of the second message. |
| Constraints | Information about fixed or plain text message has been received: The first message to be displayed is an auxiliary information. The second message be displayed is an important information. The third message to be displayed is an auxiliary information. The end conditions of the second message occur before the end conditions of the other messages. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| Plain Text Information (location based) | STORED/NOT STORED | If STORED, Fixed Text Information is not STORED and: There have to be more messages with auxiliary information than there is display space. First message to be displayed Q_TEXTCLASS=00 Auxiliary information Second message to be displayed, Q_TEXTCLASS=01 Important information Third message to be displayed , Q_TEXTCLASS=00 Auxiliary information |
| Fixed Text Information (location based) | STORED/NOT STORED | If STORED, Plain Text Information is not STORED and see Plain Text Information for variables’ description |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The "plain or fixed text message" is displayed | O | DMI | The N text message is DISPLAYED with the appearance of auxiliary information The number (N) of messages with auxiliary information will depend on the DMI configuration | N/A | N/A | - |
| 2 | N/A | N/A | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the N message is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | The "plain or fixed text message" is displayed | O | DMI | A text message is DISPLAYED with the appearance of important information. The message is an important information and its start conditions to be displayed are fulfilled The first message is hidden. | N/A | N/A | - |
| 4 | N/A | N/A | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the text message with the appearance of important information is RECORDED. | N/A | N/A | - |
| 5 | N/A | N/A | The "plain or fixed text message" is NOT displayed | O | DMI | A new message is hidden behind the second one. The new message is an auxiliary information and its start conditions to be displayed are fulfilled | N/A | N/A | - |
| 6 | N/A | N/A | The "plain or fixed text message" is removed | O | DMI | When the end conditions of the text message with the appearance of important information are reached, the message is REMOVED n | N/A | N/A | - |
| 7 | N/A | N/A | The "plain or fixed text message" is displayed | O | DMI | …and the last message is DISPLAYED with the appearance of auxiliary information | N/A | N/A | - |
| 8 | N/A | N/A | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded | O | JRU | The display of the last message is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 14 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |