| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080419 (4.8.4: Accepted Information depending on the modes - Level Transition Order) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-4080419 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 05.10.2012 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 05.02.2013 | All | Second version/review for Baseline 3 | CEDEX |
| Version 2.9.2 31.03.2013 | All | Third version/review for Baseline 3 | CEDEX |
| Version 2.9.3 30.06.2013 | All | changes In starting and end conditions and Template standardised with a dedicated tool. | CEDEX |
| Version 2.9.4 19.08.2013 | All | Editorial changes | CEDEX |
| Version 2.9.5 18.12.2013 | TC7 | Minor change in the Sequence of Steps | CEDEX |
| Version 2.9.6 21.01.2014 | ALL | Change followed step to use | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Level Transition Order |  | 4080419 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to L2 at the current position via infill BG. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: The system receives an order to switch to level 2 by means of D_LEVELTR = 0 via infill BG, and it accepts this order. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3 and D_LEVELTR = 0. The indication to the driver is recorded |
| Constraints | There must be linking information stored on-board with linking reaction for the following main BG set to “No reaction”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 12 | FS / LS |
| Linking | STORED | The linking reaction for the next main BG must be “No reaction” |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | NOT STORED | - |
| Mode Profile | STORED | It will be necessary in case the starting mode is LS mode. |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS /LS | One Balise Group message (N_TOTAL=1) containing packet 136, packet 41 (D_LEVELTR=0; M_LEVELTR=3) is received | I | BTM | The BG message is an infill message. | L1 | FS /LS | - |
| 2 | L1 | FS /LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS /LS | - |
| 3 | L1 | FS /LS | The Level symbol "Level 2" is displayed | O | DMI | The train reaches the location of the main BG announced in the linking information, but no BG message is read. The On-board equipment switches to Level 2. | L2 | FS /LS | - |
| 4 | L2 | FS /LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | New Level 2 is RECORDED. | L2 | FS /LS | - |
| 5 | L2 | FS /LS | USE_FT5100300.1 | - | - | A position report is sent to RBC after a level transition from L1 to L2/3. (Only if a radio connection is established) | L2 | FS /LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 3 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | - |
| Mode Profile | STORED | It will be necessary in case the starting mode is LS mode. |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 | L2 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Level Transition Order |  | 4080419 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to L3 at the current position via infill BG. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: The system receives an order to switch to level 3 by means of D_LEVELTR = 0 via infill BG, and it accepts this order. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 3 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 4 and D_LEVELTR = 0. The indication to the driver is recorded |
| Constraints | There must be linking information stored on-board with linking reaction for the following main BG set to “No reaction”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 12 | FS / LS |
| Linking | STORED | The linking reaction for the next main BG must be “No reaction” |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | NOT STORED | - |
| Mode Profile | STORED | It will be necessary in case the starting mode is LS mode. |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS /LS | One Balise Group message (N_TOTAL=1) containing packet 136, packet 41 (D_LEVELTR=0; M_LEVELTR=4) is received | I | BTM | The BG message is an infill message. | L1 | FS /LS | - |
| 2 | L1 | FS /LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS /LS | - |
| 3 | L1 | FS /LS | The Level symbol "Level 3" is displayed | O | DMI | The train reaches the location of the main BG announced in the linking information, but no BG message is read. The On-board equipment switches to Level 3. | L3 | FS /LS | - |
| 4 | L3 | FS /LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) | O | JRU | New Level 3 is RECORDED. | L3 | FS /LS | - |
| 5 | L3 | FS /LS | USE_FT5100300.1 | - | - | A position report is sent to RBC after a level transition from L1 to L2/3. (Only if a radio connection is established) | L3 | FS /LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | - |
| Mode Profile | STORED | It will be necessary in case the starting mode is LS mode. |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 4 | L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Level Transition Order |  | 4080419 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to LNTC via infill BG. |  | 3 |
| Applicable Mode/Level Combinations | L1 : FS, LS |  |  |
| Target of Test | The target of this test is to check that: The system receives an order to switch to level NTC by means of D_LEVELTR = 0, as infill information, i.e., after Packet 136, , and it accepts this order. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level NTC indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 1 and D_LEVELTR = 0. The indication to the driver is recorded |
| Constraints | There must be linking information stored on-board with linking reaction for the following main BG set to “No reaction”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 12 | FS / LS |
| Linking | STORED | The linking reaction for the next main BG must be “No reaction” |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | NOT STORED | - |
| Mode Profile | STORED | It will be necessary in case the starting mode is LS mode. |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS /LS | One Balise Group message (N_TOTAL=1) containing packet 136, packet 41 (D_LEVELTR=0; M_LEVELTR=1) is received | I | BTM | The BG message is an infill message. | L1 | FS /LS | - |
| 2 | L1 | FS /LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS /LS | - |
| 3 | L1 | FS /LS | The Level symbol "Level NTC" is displayed | O | DMI | The train reaches the location of the main BG announced in the linking information, but no BG message is read. The On-board equipment switches to Level NTC. | LNTC | SN | - |
| 4 | LNTC | SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) | O | JRU | New Level NTC is RECORDED. | LNTC | SN | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 1 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Linking | NOT STORED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Level Transition Order | NOT STORED | - |
| Mode Profile | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 1 | LNTC |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |