| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5050300 (5.5.3: End of Mission Procedure) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-5050300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 05.10.2012 | All | First version/review | INECO |
| Version 2.9.1 22.04.2013 | All | Second version / Update version including UNISIG review comments from 27/02/13 and RINA comments | INECO |
| Version 2.9.2 27.05.2013 | All | Third version/ TCs template updated and UNISIG review comments from 16/05/13 included | INECO |
| Version 2.9.3 01.08.2013 | All | Fourth version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.4 03.01.2014 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.5.3: End of Mission Procedure |  | 5050300 |
| Test Case of Feature | A communication session with RIU exists. Immediately after mode changed to SB mode and the end of mission procedure is triggered, the on-board equipment terminates this communication session with the RIU. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS/ LS/ OS/ SR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that : A communication session with RIU exists. The on-board equipment immediately switches to SB mode as soon as the driver closes the desk. The on-board equipment terminates the communication session without reporting the end of mission to RIU. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.5.3.1.2 |  |
|  |  | Subset-026-5.5.3.1.2 a) |  |
|  |  | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-4.6.3 Condition Id [28] |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to SB mode when the desk is closed. Check on the JRU that: Closing of desk is RECORDED The new current mode SB is RECORDED. The trackside is not informed about the End of Mission. Message 150 Not RECORDED. Message 156 is RECORDED. Message 39 is RECORDED. Check on the RTM that: The disconnection of the radio link |
| Constraints | The transition “Desk is closed (Transition [28]) “is only realisable at standstill. No message 150 “End of Mission “shall be sent. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 14 / 12 | FS / OS / SR / PT / NL / RV / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / NL / RV / PT/ LS | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L1 | FS / OS / SR / NL / RV / PT/ LS | - |
| 2 | L1 | FS / OS / SR / NL / RV / PT/ LS | Cab not active | I | TIU | Driver closed the Desk; The mode change to SB (internal variable M_MODE) | L1 | SB | - |
| 3 | L1 | SB | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | L1 | SB | - |
| 4 | L1 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L1 | SB | - |
| 5 | L1 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | Symbol MO13 | L1 | SB | - |
| 6 | L1 | SB | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Message 150 “End of Mission “is not sent. | L1 | SB | - |
| 7 | L1 | SB | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | Message 150 “End of Mission “is not recorded. | L1 | SB | - |
| 8 | L1 | SB | SA-DATA. Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) | I | RTM | - | L1 | SB | - |
| 9 | L1 | SB | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | L1 | SB | - |
| 10 | L1 | SB | SA-DISCONNECT. Request is transmitted | O | RTM | - | L1 | SB | - |

| Step 6: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 8: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | Always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.5.3: End of Mission Procedure |  | 5050300 |
| Test Case of Feature | The train is at standstill. A communication session with RIU exists. The end of mission procedure was triggered by driver selection of SH. The on-board equipment terminates this communication session with the RIU. |  | 2 |
| Applicable Mode/Level Combinations | L1 : SH |  |  |
| Target of Test | The target of this test case is to check that : A communication session with RIU exists. The on-board equipment terminates the communication session without reporting the end of mission to RIU. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.5.3.1.2 |  |
|  |  | Subset-026-5.5.3.1.2 a) |  |
|  |  | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-5.6.2.2 A100 |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Indication of SH mode is DISPLAYED Check on the JRU that: The trackside is not informed about the End of Mission. Message 150 is Not RECORDED. Check on the RTM that : The disconnection of the radio link |
| Constraints | End of Mission procedure already Triggered. Mode transition from FS / LS / OS / SR / PT to SH already performed in FT5060200. 5 A communication session with the RIU exists. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP. | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Shunting" is displayed | O | DMI | Symbol MO01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | SH | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Message 150 “End of Mission “is not sent. | L1 | SH | - |
| 2 | L1 | SH | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | Message 150 “End of Mission “is not recorded. | L1 | SH | - |
| 3 | L1 | SH | SA-DATA. Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) | I | RTM | - | L1 | SH | - |
| 4 | L1 | SH | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | L1 | SH | - |
| 5 | L1 | SH | SA-DISCONNECT. Request is transmitted | O | RTM | - | L1 | SH | - |

| Step 1: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 3: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | Always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | with RIU |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.5.3: End of Mission Procedure |  | 5050300 |
| Test Case of Feature | Inform track-side about End of Mission procedure. A communication session with RBC exists. |  | 3 |
| Applicable Mode/Level Combinations | L0: SH / SB LNTC: SH / SB L1: SH / SB L 2: SH / SB L 3: SH / SB |  |  |
| Target of Test | The target of this test case is to check that : If the End of Mission procedure was triggered and a communication session with an RBC exist, than : The end of mission shall be reported to the RBC by means of the message “End of Mission”. The RBC shall request to terminate the communication session. The ERTMS/ETCS on-board equipment shall terminate the communication session. End of procedure |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.5.3.1.3 |  |
|  |  | Subset-026-5.5.3.1.3 a) |  |
|  |  | Subset-026-5.5.3.1.3 c) |  |
|  |  | Subset-026-8.6.10 Message 150: End of Mission |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: Message 150 ‘End of Mission’ is RECORDED. Check on the RTM that: The disconnection of the radio link is performed |
| Constraints | Communication Session between on-board and track-side is ESTABLISHED |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 6 | SH / SB |
| Radio communication session | ESTABLISHED | With RBC |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 or packet 1 is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The RBC requests to terminate the communication session. | - | - | USE_FT3050500.5: “Termination of communication session ordered by RBC” OR USE_FT5050400.2: “Termination of communication session to RBC due to unanswered "end of mission" report after unsuccessful repetitions”. | N/A | N/A | - |

| Step 1: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | End of Mission |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | ONLY IF PACKET 1 IS NOT USED |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 1 | ONLY IF PACKET 0 IS NOT USED |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PRVLRBG | 24 | FINITE VALUE | Used as reference for all directional information in the packet: a move from PRVLRBG towards the LRBG defines the "nominal" direction. |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | Train orientation according to reference direction. |
| Q_DLRBG | 2 | FINITE VALUE | Train front position according to reference direction. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | Actual running direction according to reference direction. |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 6 | SH / SB |
| Radio communication session | NOT ESTABLISHED | with RBC |
| ERTMS/ETCS level | 0/ 1/ 2/ 3 / 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |