| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3100200 (3.10.2 Emergency Stop) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-3100200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, Answer to Review001 Target distance replaced with permitted speed in starting conditions on interface | MULTITEL |
| Version 2.9.2 06.11.2013 | All | Answer to Review005_Subset076-5-2-3100200_v290 | MULTITEL |
| Version 2.9.3 28.11.2013 | All | Answer to Review001_Subset076-5-2-3100200_v292 | MULTITEL |
| Version 2.9.4 12.10.2013 | TC1 | Answer to issue 2 of Review002_Subset-076-5-2-3100200_v292 | MULTITEL |
| Version 2.9.5 09.01.2014 | TC1 | Answer to issue 1 of Review001_Subset-076-5-2-3100200_v294 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.10.2 Emergency Stop |  | 3100200 |
| Test Case of Feature | The train receives a Movement Authority which is REJECTED, because a valid emergency stop message is stored on-board. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS, OS L3: FS, OS |  |  |
| Target of Test | To check that the new MA is rejected before the emergency message has been revoked. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.10.2.4 |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Received from and sent to the RBC messages are recorded Check on the DMI that: The speed and distance monitoring information is not updated according to received MA. |
| Constraints | The emergency stop location defines the actual EOA and SvL |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Requested only if in OS |
| Conditional Emergency Stop | STORED | VALID |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Permitted Speed is DISPLAYED Indication of Emergency stop message received is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 is received | I | RTM | The message is REJECTED | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is not updated | O | DMI | Nothing changes on DMI | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Has to be unique |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time(T_TRAIN time and message time ) |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED UNCHANGED | - |
| Linking | STORED UNCHANGED | - |
| Movement Authority | STORED UNCHANGED | - |
| Gradient Profile | STORED UNCHANGED | - |
| International SSP | STORED UNCHANGED | - |
| Mode Profile | STORED UNCHANGED | Requested only if in OS |
| Conditional Emergency Stop | STORED UNCHANGED | - |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Permitted Speed is DISPLAYED Indication of Emergency stop message received is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |