| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5040300 (5.4.3: Table of requirements for “Start of Mission” procedure) Total: 56 Test case(s) |  |
| REF: | Subset-076-5-2-5040300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, TCs renumbered | MULTITEL |
| Version 2.9.2 03.10.2013 | TC1, TC2, TC3, TC4, TC5, TC6, TC7, TC8, TC9, TC22, TC23, TC25, TC36, TC39, TC40 | Modifications requested by Review001_Subset076-5-2-5040300_v291 | MULTITEL |
| Version 2.9.3 24.10.2013 | All | Modifications requested by Review002_Subset076-5-2-5040300_v291 and issues 61 and 62 from Review003_Subset076-5-2-5040300_v291 | MULTITEL |
| Version 2.9.4 04.12.2013 | All | Modifications requested by issue 63 from Review004_Subset-076-5-2-5040300_v293 | MULTITEL |
| Version 2.9.5 19.12.2013 | All | Correction of misprints in requirements | MULTITEL |
| Version 2.9.6 09.01.2014 | TC4 | Modifications requested by issue 1 from Review001_Subset-076-5-2-5040300_v295 | MULTITEL |
| Version 2.9.7 20.01.2014 | TC11, TC12, TC20, TC31, TC32, TC33 | Modifications requested by Review001_Subset-076-5-2-5040300_v296 | MULTITEL |
| Version 2.9.8 27.01.2014 | TC15-17, 27 | Removal of notes related to "followed by" step in “END CONDITIONS (INTERNAL STATES)” table | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. |  | 1 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB, the on-board equipment : Captures the Train Data and requests the driver to re-validate/enter a complete set of Train Data. Requests the driver to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. Note: The data input order and the way the on-board equipment displays data that needs to be input is out of the scope of this test case (this is a matter of implementation). As soon as a complete set of train data is available on-board, the on-board equipment will authorise the driver to validate the data. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S12 |  |
|  |  | Subset-026-5.4.3.2 D12 |  |
|  |  | Subset-026-5.4.3.2 D10 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : If train data available on-board (e.g. from another source or already stored on-board), these train data are initially displayed to the driver (starting condition within the scope of the Start Of Mission procedure). The driver is not allowed to validate the train data UNTIL the driver has entered a complete set of valid Train Data. When the driver has input a valid set of Train Data, the driver is allowed to validate the train data. The on-board requests the driver to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. Check on the JRU that : Driver‘s validation action is recorded. Each data of the set of Train Data is recorded (Data Entry Completed) and is identical to what the driver has entered. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The train data shall be acquired by the on-board system of a leading engine. The set of Train Data will not be entered by means of external devices. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | INVALID / UNKNOWN | - |
| ERTMS/ETCS level | 0 / 1 / 2 VALID | L0 / LNTC / L1 A valid ERTMS/ETCS Level has been entered/confirmed by the driver |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The Train Data window is displayed to the driver. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | If the status of the Train data is "invalid"/"unknown", the driver is requested to re-validate/enter the Train data. | - | - | Use_FT5040300.47 (unknown Train data) Use_FT5040300.2 (invalid Train data validated by driver) Use_FT5040300.3 (invalid Train data modified by driver before validation) | N/A | N/A | - |
| 2 | N/A | N/A | If the status of the Train running number is "invalid"/"unknown", the driver is requested to re-validate/enter the Train running number. | - | - | Use_FT5040300.45 (invalid Train running number) OR Use_FT5040300.46 (unknown Train running number) | N/A | N/A | - |
| 3 | N/A | N/A | The DMI allows the driver to select the option ‘START’. | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The DMI allows the driver to select the option ‘START’. | O | DMI | The driver is allowed to start a mission since all necessary data is available on-board. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  | 2 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB, the on-board equipment : Captures the Train Data and requests the driver to validate an existing complete set of Train Data. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.3.2 |  |
|  |  | Subset-026-3.18.3.2 a) |  |
|  |  | Subset-026-3.18.3.2 b) |  |
|  |  | Subset-026-3.18.3.2 c) |  |
|  |  | Subset-026-3.18.3.2 d) |  |
|  |  | Subset-026-3.18.3.2 e) |  |
|  |  | Subset-026-3.18.3.2 f) |  |
|  |  | Subset-026-3.18.3.2 g) |  |
|  |  | Subset-026-3.18.3.2 h) |  |
|  |  | Subset-026-3.18.3.2 i) |  |
|  |  | Subset-026-3.18.3.2 k) |  |
|  |  | Subset-026-3.18.3.2.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Train Data (refer to 3.18.3.2) |  |
|  |  | Subset-026-5.4.3.2 S12 |  |
|  |  | Subset-026-5.4.3.3 - Following S12: driver has (re-) validated train data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : If train data available on-board (e.g. from another source or already stored on-board), these train data are initially displayed to the driver (starting condition within the scope of the Start Of Mission procedure). A complete set of Train Data is initially displayed. The driver is allowed to validate the train data at any time (the driver is not required to enter any Train Data since he is allowed to validate the new set of train data). Check on the JRU that : Driver‘s validation action is recorded. The validated set of Train Data is recorded (Data Entry Completed). It can be checked that the set of Train Data is identical to the initial set. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure in S12 after the driver has selected Train Data (in S10). The train data shall be acquired by the on-board system of a leading engine. The set of Train Data will not be entered by means of external devices. The driver does NOT modify the existing set of Train Data before validating it |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | INVALID | There is a complete set of Train Data stored on-board |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1/ L2 /L3 A valid ERTMS/ETCS Level has been entered/confirmed by the driver |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The Train Data window is displayed to the driver. | O | DMI | The driver is allowed to validate the train data. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. | O | DMI | The driver might be requested to validate the “Train category(ies)”, “Train length”, “Traction / brake parameters”, “Maximum train speed”, “Loading gauge”, “Axle load category”, “Train fitted with airtight system”, but not the “Traction system(s) accepted by the engine”, “List of National Systems available on-board” or “Axle number”. | N/A | N/A | - |
| 2 | N/A | N/A | The driver validates the train data. | I | DMI | The on-board equipment allows the driver to simply validate the existing set of Train Data. | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Triggered by : The driver validation. | N/A | N/A | - |
| 4 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | Triggered by : Data Entry completed | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) The on-board equipment allows the driver to modify train data before the validation |  | 3 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB, the on-board equipment : Captures the Train Data and requests the driver to validate an existing complete set of Train Data. Allows the driver to modify train data before the validation |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.3.2 |  |
|  |  | Subset-026-3.18.3.2 a) |  |
|  |  | Subset-026-3.18.3.2 b) |  |
|  |  | Subset-026-3.18.3.2 c) |  |
|  |  | Subset-026-3.18.3.2 d) |  |
|  |  | Subset-026-3.18.3.2 e) |  |
|  |  | Subset-026-3.18.3.2 f) |  |
|  |  | Subset-026-3.18.3.2 g) |  |
|  |  | Subset-026-3.18.3.2 h) |  |
|  |  | Subset-026-3.18.3.2 i) |  |
|  |  | Subset-026-3.18.3.2 k) |  |
|  |  | Subset-026-3.18.3.2.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Train Data (refer to 3.18.3.2) |  |
|  |  | Subset-026-5.4.3.2 S12 |  |
|  |  | Subset-026-5.4.3.3 - Following S12: driver has (re-) validated train data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : If train data available on-board (e.g. from another source or already stored on-board), these train data are initially displayed to the driver (starting condition within the scope of the Start Of Mission procedure). A complete set of Train Data is initially displayed. The driver is allowed to validate the train data at any time (the driver is not required to enter any Train Data since he is allowed to validate the new set of train data). The driver modifies at least one of the existing Train Data before validating the complete set. Check on the JRU that : Driver‘s validation action is recorded. The validated set of Train Data is recorded (Data Entry Completed). It can be checked that the set of Train Data is identical to the initial set but for the train data modified by the driver. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure in S12 after the driver has selected Train Data (in S10). The train data shall be acquired by the on-board system of a leading engine. The set of Train Data will not be entered by means of external devices. The driver modifies at least one of the existing Train Data before validating the complete set |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | INVALID | There is a complete set of Train Data stored on-board |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1/ L2 /L3 A valid ERTMS/ETCS Level has been entered/confirmed by the driver |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The Train Data window is displayed to the driver. | O | DMI | The driver is allowed to validate the train data. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. | O | DMI | The driver might be requested to validate the “Train category(ies)”, “Train length”, “Traction / brake parameters”, “Maximum train speed”, “Loading gauge”, “Axle load category”, “Train fitted with airtight system”, but not the “Traction system(s) accepted by the engine”, “List of National Systems available on-board” or “Axle number”. | N/A | N/A | - |
| 2 | N/A | N/A | The driver modifies at least one of the train data. | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver validates the train data. | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Triggered by : The driver validation. | N/A | N/A | - |
| 5 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | Triggered by : Data Entry completed | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. Radio communication session is established. |  | 4 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB, the on-board equipment : Captures the Train Data and requests the driver to re-validate/enter a complete set of Train Data. Requests the driver to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. Sends the Train Data to the RBC once the Train Data and the Train running number are valid. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.3.4 |  |
|  |  | Subset-026-3.18.3.4 a) |  |
|  |  | Subset-026-3.18.3.4 b) |  |
|  |  | Subset-026-3.18.3.4 c) |  |
|  |  | Subset-026-3.18.3.4 d) |  |
|  |  | Subset-026-3.18.3.4 e) |  |
|  |  | Subset-026-3.18.3.4 f) |  |
|  |  | Subset-026-3.18.3.4 g) |  |
|  |  | Subset-026-3.18.3.4 h) |  |
|  |  | Subset-026-3.18.3.4 i) |  |
|  |  | Subset-026-5.4.3.2 S12 |  |
|  |  | Subset-026-5.4.3.2 D12 |  |
|  |  | Subset-026-5.4.3.2 D10 |  |
|  |  | Subset-026-5.4.3.2 D11 |  |
|  |  | Subset-026-5.4.3.2 S11 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated train data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of train data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : If train data available on-board (e.g. from another source or already stored on-board), these train data are initially displayed to the driver (starting condition within the scope of the Start Of Mission procedure). The driver is not allowed to validate the train data UNTIL the driver has entered a complete set of valid Train Data. When the driver has input a valid set of Train Data, the driver is allowed to validate the train data. The on-board requests the driver to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. Check on the JRU that : Driver‘s validation action is recorded. Each data of the set of Train Data is recorded (Data Entry Completed) and is identical to what the driver has entered. The Message 129 with packet 11 validated train data is sent to the RBC. The Message 8 is received from the RBC. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The train data shall be acquired by the on-board system of a leading engine. The set of Train Data will not be entered by means of external devices. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Data | INVALID / UNKNOWN | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| Desk open | I | TIU | - |
| The Train Data windwo is displayed to the driver. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | If the status of the Train data is "invalid"/"unknown", the driver is requested to re-validate/enter the Train data. | - | - | Use_FT5040300.47 (unknown Train data) Use_FT5040300.2 (invalid Train data validated by driver) Use_FT5040300.3 (invalid Train data modified by driver before validation) | N/A | N/A | - |
| 2 | N/A | N/A | If the status of the Train running number is "invalid"/"unknown", the driver is requested to re-validate/enter the Train running number. | - | - | Use_FT5040300.45 (invalid Train running number) OR Use_FT5040300.46 (unknown Train running number) | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The DMI allows the driver to select ‘START’. | O | DMI | - | N/A | N/A | - |

| Step 3: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | - |
| NID_CTRACTION(k) | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If N_ITER different from 0 |

| Step 5: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 10+14 | FINITE VALUE | Not: 16777215 (unknown) |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | Unchanged |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The DMI allows the driver to select the option ‘START’. | O | DMI | The driver is allowed to start a mission since all necessary data is available on-board. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | During a start of mission, the driver shall have the opportunity to set a new VBC with the same VBC identity as an already stored VBC. The new VBC shall replace the previous one, including its validity period. |  | 5 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that during a start of mission, the on-board equipment allows the driver to set a new VBC with the same VBC identity as an already stored VBC. The new VBC shall replace the previous one, including its validity period. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.9.2 |  |
|  |  | Subset-026-3.15.9.4 |  |
|  |  | Subset-026-4.7.2 table - Input information Virtual Balise Cover |  |
|  |  | Subset-026-5.4.3.2 S1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is allowed to set a new VBC. The driver enter a VBC with the same VBC identity (NID_VBCMK and NID_C) as an already stored VBC AND a different validity period (T_VBC). Check on the JRU that: The virtual balise cover set by the driver is recorded and is identical to the data entered by the driver, including its validity period. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. There is, at least, one VBC stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The driver is allowed to set/remove a Virtual Balise Cover. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment offers the possibility to set/remove a Virtual Balise Cover. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver chooses to set/remove a Virtual Balise Cover | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver sets and validate a new VBC with the same VBC identity as an already stored VBC and a different validity period | I | DMI | The VBC has the same NID_VBCMK and NID_C as an already stored and a different T_VBC ERA_ERTMS_015560 v330: When converting the entered ‘VBC set code’ into binary format, the 6 less significant bits shall form the variable NID_VBCMK, the next 10 bits shall form the variable NID_C and the remaining bits shall form the variable T_VBC | N/A | N/A | - |
| 4 | N/A | N/A | The new VBC is RECORDED with the correct validity period. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  | 6 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that the On-Board equipment offers the driver to select either SH or NL, ‘Train Data Entry’ and that the driver selects ‘Train Data Entry’ before going further in the Start of Mission procedure. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Input information Train Data (refer to 3.18.3.2) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S10 c) |  |
|  |  | Subset-026-5.4.3.2 S12 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the driver can select either SH or NL (“Non-Leading” button is enabled if the “non leading” input signal is received) or ‘Train Data Entry’. the driver is not allowed to go further in the Start of Mission procedure before having selected one of these options thre driver selects ‘Train Data Entry’. Check on the JRU that : The driver‘s selection action is recorded. |
| Constraints | The test case is done for a driver input of the data, but it should be valid also if the source of data is a different one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | In L2/L3, must be established to enable the “Shunting” button |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1 / L2 / L 3 |
| Driver ID | VALID | A valid Driver ID is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No train data are displayed to the driver | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver can select either SH or NL or ‘Train Data Entry’ | O | DMI | Starting conditions The “Non-Leading” button is enabled if the “non leading” input signal is received. | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects ‘Train Data Entry’ | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | A Train data window is is displayed on the DMI. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver’s selection is recorded on the JRU. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0/1/2/3/4 UNCHANGED | L0 / LNTC / L1 / L2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| A Train data window is is displayed on the DMI. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  | 7 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that when the status of the Driver ID is UNKNOWN within the scope of the Start of Mission procedure in SB, the on-board equipment: requests the driver to enter a Driver ID value. Prevents the driver from going on UNTIL the driver has entered a valid driver ID and validated it. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Input information Driver id |  |
|  |  | Subset-026-5.4.3.2 S1 |  |
|  |  | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.3 - Following S1: driver has entered driver ID |  |
|  |  | Subset-026-5.4.3.3 - Following D2: stored position is "invalid" or "unknown" |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is no Driver ID displayed since status of Driver ID is UNKNOWN (starting condition within the scope of the Start Of Mission procedure) The driver is not allowed to go further in the Start of Mission procedure UNTIL the driver has entered a valid Driver ID. When the driver has input a valid Driver ID, it is possible for him to validate it. Check on the JRU that : The driver ID is recorded and is identical to the data entered by the driver. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the Driver ID is UNKNOWN, i.e. there is no data stored on-board. The Driver ID will not be entered by means of external devices. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | UNKNOWN | There is no driver ID stored on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| There is NO initial Driver ID displayed. | O | DMI | The driver is not allowed to validate the Driver ID. The driver is allowed to set/remove a Virtual Balise Cover. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed | O | DMI | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. | N/A | N/A | - |
| 2 | N/A | N/A | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver enters and validates a correct Driver ID | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a “GENERAL MESSAGE”. | O | JRU | A “GENERAL MESSAGE” is recorded every 5s by the on-board equipment. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 INVALID / CHANGED | L0 / LNTC / L1 / L2 / L3 This change occurs only if all the following conditions were met before the driver validates the Driver ID: The status of ERTMS/ETCS level was VALID and The status of Train Position was INVALID or UNKNOWN. |
| Driver ID | VALID | - |
| RBC ID/Phone Number | INVALID / CHANGED | This change occurs only if all the following conditions were met before the driver validates the Driver ID: The status of ERTMS/ETCS level was VALID and The status of Train Position was INVALID or UNKNOWN. |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  | 8 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that when the status of the Driver ID is INVALID within the scope of the Start of Mission procedure in SB, the on-board equipment : Displays an existing Driver ID (since there is a data stored on-board). Allows the driver to simply revalidate an existing Driver ID (no need for the driver to input any new data). Prevents the driver from going on UNTIL the driver has validated the Driver Id. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Input information Driver id |  |
|  |  | Subset-026-5.4.3.2 S1 |  |
|  |  | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.3 - Following S1: driver has re-validated/re-entered driver ID |  |
|  |  | Subset-026-5.4.3.3 - Following D2: stored position is "invalid" or "unknown" |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is an initial Driver ID displayed (starting condition within the scope of the Start Of Mission procedure), (because status of Driver ID is INVALID ( there is a Driver ID stored on-board) and. The driver is required to validate the Driver Id for going on Check on the JRU that : The driver ID is recorded and is identical to the data initially displayed in the displayed Driver ID . |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the Driver ID is INVALID ( there is Driver ID stored on-board. It does not matter whether this data was previously entered by the driver or from an external device. The driver does NOT modify the existing Driver ID before validating it (modification + validation : see test case 3). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| There is a Driver ID displayed. | O | DMI | It is possible for the driver to validate the Driver Id. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. | O | DMI | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. | N/A | N/A | - |
| 2 | N/A | N/A | The driver validates the Driver Id. | I | DMI | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. | N/A | N/A | - |
| 3 | N/A | N/A | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a “GENERAL MESSAGE”. | O | JRU | A “GENERAL MESSAGE” is recorded every 5s by the on-board equipment. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 INVALID / CHANGED | L0 / LNTC / L1 / L2 / L3 This change occurs only if all the following conditions were met before the driver validates the Driver ID: The status of ERTMS/ETCS level was VALID and The status of Train Position was INVALID or UNKNOWN. |
| Driver ID | VALID | - |
| RBC ID/Phone Number | INVALID / CHANGED | This change occurs only if all the following conditions were met before the driver validates the Driver ID: The status of ERTMS/ETCS level was VALID and The status of Train Position was INVALID or UNKNOWN. |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment allows the driver to modify an existing Driver ID before validating it within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is INVALID. |  | 9 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that when the status of the Driver ID is INVALID within the scope of the Start of Mission procedure in SB, the on-board equipment : Displays an existing Driver ID (since there is a data stored on-board). Allows the driver to modify the existing Driver ID with 16 alphanumeric characters. Allows the driver to validate the modified Driver ID Prevents the driver from going on UNTIL the driver has validated the Driver Id. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.1.4 |  |
|  |  | Subset-026-A.3.11 Driver ID |  |
|  |  | Subset-026-4.7.2 table - Input information Driver id |  |
|  |  | Subset-026-5.4.3.2 S1 |  |
|  |  | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.3 - Following S1: driver has re-validated/re-entered driver ID |  |
|  |  | Subset-026-5.4.3.3 - Following D2: stored position is "invalid" or "unknown" |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is an initial Driver ID displayed (starting condition within the scope of the Start Of Mission procedure), (because status of Driver ID is INVALID ( there is a Driver ID stored on-board). The driver is allowed to modify an existing Driver ID before validating it The driver is allowed to validate a modified Driver The driver is required to validate the Driver Id in order to go on. Check on the JRU that : The driver ID is recorded and is identical to the data initially displayed. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the Driver ID is INVALID ( there is Driver ID stored on-board. It does not matter whether this data was previously entered by the driver or from an external device. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| There is a Driver ID displayed. | O | DMI | It is possible for the driver to validate the Driver Id. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | There is an initial Driver ID displayed. | O | DMI | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. | N/A | N/A | - |
| 2 | N/A | N/A | The driver modifies the Driver ID (e.g. a new valid data is input). | I | DMI | The on-board equipment allows the driver to modify an existing Driver ID. The new Driver ID must be "1234567890azbycx". | N/A | N/A | - |
| 3 | N/A | N/A | The modified Driver ID is displayed on the DMI | O | DMI | The on-board equipment has clearly authorised the driver to enter a new Driver ID. | N/A | N/A | - |
| 4 | N/A | N/A | The driver validates the modified Driver ID. | I | DMI | The on-board equipment authorises the driver to validate a modified Driver ID. | N/A | N/A | - |
| 5 | N/A | N/A | The new Driver ID entered by the driver is recorded on the JRU with the DRIVER_ID variable in a “GENERAL MESSAGE”. | O | JRU | A “GENERAL MESSAGE” is recorded every 5s by the on-board equipment. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 INVALID / CHANGED | L0 / LNTC / L1 / L2 / L3 This change occurs only if all the following conditions were met before the driver validates the Driver ID: The status of ERTMS/ETCS level was VALID and The status of Train Position was INVALID or UNKNOWN |
| Driver ID | VALID | The driver has modified an existing driver ID and then validated the modified data. |
| RBC ID/Phone Number | INVALID / CHANGED | This change occurs only if all the following conditions were met before the driver validates the Driver ID: The status of ERTMS/ETCS level was VALID and The status of Train Position was INVALID or UNKNOWN |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  | 10 |
| Applicable Mode/Level Combinations | L0: SB LNTC : SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that, within the scope of the Start of Mission procedure in SB mode, when the status of the ERTMS/ETCS Level data is INVALID, the driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Input information ERTMS/ETCS level |  |
|  |  | Subset-026-5.4.3.2 S2 |  |
|  |  | Subset-026-5.4.3.2 D2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Driver‘s action is recorded. The ERTMS/ETCS Level data selected by the driver is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | UNKNOWN / INVALID / VALID | - |
| ERTMS/ETCS level | INVALID | - |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | UNKNOWN / INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| A list of all available ERTMS/ETCS Level data is displayed to the driver. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | N/A | The driver selects "Level <L0/LNTC/L1/L2/L3>" | I | DMI | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. | L0 / LNTC / L1 / L2 / L3 | N/A | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | N/A | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1 / L2 / L3 |
| Train Running Number | INVALID | Unchanged |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| The ERTMS/ETCS Level selected by the driver is displayed. | O | DMI | The On-Board equipment requests the driver either : -L2/L3: to enter the RBC ID & RBC Phone Number OR -L0/LNTC/L1: to select “Train Data Entry“. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment allows the driver to validate an existing ERTMS/ETCS Level data within the scope of the Start of Mission procedure in SB mode when the status of the ERTMS/ETCS Level data is INVALID. |  | 11 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that when the status of the ERTMS/ETCS Level data is INVALID within the scope of the Start of Mission procedure in SB, the on-board equipment : Displays the existing ERTMS/ETCS Level data (since there is a data stored on-board). Allows the driver to simply revalidate the existing ERTMS/ETCS Level (no need for the driver to input any new data). Prevents the driver from going on UNTIL the driver validates the ERTMS/ETCS level. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Input information ERTMS/ETCS level |  |
|  |  | Subset-026-5.4.3.2 S2 |  |
|  |  | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.3 - Following S2: driver has re-validated/re-entered level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is an initial ERTMS/ETCS Level initially displayed (starting condition). The On-Board equipment requires the driver to validate the ERTM/ETCS level (since the status of ERTMS/ETCS Level data is INVALID). ( The driver is not required by the on-board equipment to enter any ERTMS/ETCS Level. The driver cannot going on UNTIL he validates the ERTM/ETCS level. Check on the JRU that : Driver‘s validation action is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the ERTMS/ETCS Level data is INVALID ( there is a ERTMS/ETCS Level data stored on-board and proposed to the driver for validation). The driver does NOT modify the existing ERTMS/ETCS Level before validating it. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | UNKNOWN / INVALID /VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 INVALID | L0 / LNTC / L1 / L2 / L3 There is a ERTMS/ETCS Level data stored on-board, which needs to be revalidated. |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | UNKNOWN / INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| There is an initial ERTMS/ETCS Level data displayed. | O | DMI | The On-board equipment requires the driver to validate the ERTMS/ETCS level. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver validates the ERMS/ETCS level. | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The initial ERTMS/ETCS Level is still displayed on the DMI. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Driver’s validation action is recorded on the JRU. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1 / L2 / L3 The driver has revalidated an existing ERTMS/ETCS Level data. |
| Train Running Number | INVALID | Unchanged |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The ERTMS/ETCS Level initially displayed is still displayed. | O | DMI | The On-Board equipment requests the driver either : -L2/L3: to enter the RBC ID & RBC Phone Number OR -L0/LNTC/L1: to select “Train Data Entry“. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment allows the driver to modify an existing ERTMS/ETCS Level data before validating it within the scope of the Start of Mission procedure in SB mode when the status of the ERTMS/ETCS Level data is INVALID. |  | 12 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that when the status of the ERTMS/ETCS Level data is INVALID within the scope of the Start of Mission procedure in SB, the on-board equipment : Displays the ERTMS/ETCS Level window to the driver with an existing ERTMS/ETCS Level data inside (since there is a data stored on-board). Allows the driver to modify the existing ERTMS/ETCS Level data. Allows the driver to validate the modified ERTMS/ETCS Level data. Prevents the driver from going on UNTIL the driver validates the ERTME/ETCS level. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Input information ERTMS/ETCS level |  |
|  |  | Subset-026-5.4.3.2 S2 |  |
|  |  | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.3 - Following S2: driver has re-validated/re-entered level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is an initial ERTMS/ETCS Level initially displayed (starting condition). The On-Board equipment requests the driver to validate the ERTMS/ETCS level (since the status of ERTMS/ETCS Level data is INVALID). The driver is allowed to modify the existing ERTMS/ETCS Level data before validation The driver is allowed to validate the modified ERTMS/ETCS Level data . The driver is required to validate one ERTMS/ETCS level before going to in the procedure. The new ERTMS/ETCS Level data entered by the driver is displayed. Check on the JRU that : Driver‘s validation action is recorded. The modified ERTMS/ETCS Level data is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the ERTMS/ETCS Level data is INVALID ( there is a ERTMS/ETCS Level data stored on-board and proposed to the driver for validation). The driver does modify the existing ERTMS/ETCS Level before validating it. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | UNKNOWN / INVALID /VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 INVALID | L0 / LNTC / L1 / L2 / L3 There is a ERTMS/ETCS Level data stored on-board, which needs to be revalidated. |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | UNKNOWN / INVALID /VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open. Remote control is not detected | I/O | TIU | The on-board equipment does not detect any remote control (no sleeping input signal). |
| There is an initial ERTMS/ETCS Level data displayed. | O | DMI | The On-board equipment allows the driver to validate the ERTMS/ETCS level. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | A ETCS level is displayed to the driver | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver modifies the ERTMS/ETCS Level data (e.g. a new valid data is input). | I | DMI | The on-board equipment allows the driver to modify an existing ERTMS/ETCS Level data. | N/A | N/A | - |
| 3 | N/A | N/A | The new ERTMS/ETCS Level data is displayed on the DMI The On-Board equipment still allows the driver to validate it. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver validates the ERTMS/ETCS level. | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | Driver’s validation action is recorded on the JRU. | O | JRU | Triggered by : The driver acts on the on-board DMI. | N/A | N/A | - |
| 6 | N/A | N/A | The ERTMS/ETCS Level data re-entered by the driver is recorded on the JRU | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1 / L2 / L3 The driver has re-entered an ERTMS/ETCS Level data. |
| Train Running Number | INVALID | Unchanged |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Remote control is not detected | I | TIU | Unchanged |
| The new ERTMS/ETCS Level data entered by the driver is displayed. | O | DMI | The On-Board equipment requests the driver either : -L2/L3: to enter the RBC ID & RBC Phone Number OR -L0/LNTC/L1: to select “Train Data Entry“. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | If at least one Mobile Terminal is registered to a Radio Network and if the status of the RBC-ID/phone number is “invalid" or "valid", on-board equipment offers the driver the possibility to order the on-board equipment to use the last stored RBC-ID/phone number |  | 13 |
| Applicable Mode/Level Combinations | L2/L3: SB |  |  |
| Target of Test | The target of this test is to check that, within the procedure S3 of start of mission in SB mode, if at least one Mobile Terminal is registered to a Radio Network and if the status of the RBC-ID/phone number is “invalid" or "valid", the on-board equipment offers the driver the possibility to order the on-board equipment to use the last stored RBC-ID/phone number |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.2 |  |
|  |  | Subset-026-5.4.3.2 S3 |  |
|  |  | Subset-026-5.4.3.2 S3 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Radio data window is displayed with the possibility to the driver to select ‘Contact last RBC’, ‘Use short number’, ‘Enter RBC data’ and ‘Radio network ID’. The driver presses the ‘Contact last RBC’ button. Check on the JRU that : The “RBC CONTACT INFORMATION ENTERED BY THE DRIVER” message is RECORDED with the variable Q_RBCENTRY=0 (Contact last known RBC). |
| Constraints | At least one Mobile Terminal registered to a Radio Network RBC ID/Phone Number STATUS is INVALID |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3/4 VALID | L 2/ L 3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | INVALID / VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level, Mode and Radio data window are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment displays the Radio data window and offers the possibility to the driver to select: - ‘Contact last RBC’, - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses the “Contact last RBC” button. | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=0) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level and Mode are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to use the EIRENE short number (trackside call routing function) |  | 14 |
| Applicable Mode/Level Combinations | L2/L3: SB |  |  |
| Target of Test | The target of this test is to check that, within the procedure S3 of start of mission in SB mode, if at least one Mobile Terminal is registered to a Radio Network, the on-board equipment offers the driver the possibility to use the EIRENE short number (trackside call routing function). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.2 |  |
|  |  | Subset-026-5.4.3.2 S3 |  |
|  |  | Subset-026-5.4.3.2 S3 d) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Radio data window is displayed with the possibility to the driver to select ‘Use short number’, ‘Enter RBC data’ and ‘Radio network ID’. The driver presses the ‘Use short number’ button. Check on the JRU that : The “RBC CONTACT INFORMATION ENTERED BY THE DRIVER” message is RECORDED with the variable Q_RBCENTRY=1 (Use short number). |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3/4 VALID | L 2/ L 3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | INVALID / UNKNOWN | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level, Mode and Radio data window are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment displays the Radio data window and offers the possibility to the driver to select: - ‘Contact last RBC’, - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses the “Use short number” button. | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | INVALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level and Mode are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The driver selects ”Start” within the scope of the Start of Mission procedure in SB mode and in ERTMS Level NTC. |  | 15 |
| Applicable Mode/Level Combinations | LNTC: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB mode and in Level NTC, the on-board equipment : Allows the driver to request “Start”, Displaysan acknowledgement request for running under supervision of the selected NTC mode as soon as the driver has requested “Start”,Requests (waits for) the driver to acknowledge the acknowledgement request for running under supervision of the selected NTC mode (subset-026-5.4.4.1 E20). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S22 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-5.4.3.2 S20 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to request “Start” (starting condition within the scope of the Start Of Mission procedure) at any time. Once the driver has selected ”Start”, an acknowledgement request for running under supervision of the selected NTC mode is displayed on the DMI. Check on the JRU that : The selection by the driver of “Start” is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 1 VALID | LNTC ERTMS/ETCS Level NTC has been entered or confirmed by the driver |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The On-Board equipment allows the driver to select “Start“. | O | DMI | ERTMS Level NTC and SB mode are displayed on the DMI. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects ”Start”. | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | An acknowledgement request for running under supervision of the selected NTC mode is displayed on the DMI | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in SN mode | N/A | N/A | - |
| 3 | N/A | N/A | Driver’s selection of “Start” is recorded on the JRU. | O | JRU | Triggered by : The driver acts on the on-board DMI. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 1 UNCHANGED | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| An acknowledgement request for running under supervision of the selected NTC mode is displayed on the DMI. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The driver selects ”Start” within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  | 16 |
| Applicable Mode/Level Combinations | L1: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB mode and in Level 1, the on-board equipment : Allows the driver to request “Start”, Displays an acknowledgement request for running in SR mode on the DMI as soon as the driver has selected “Start”, Requests (waits for) the driver to acknowledge the acknowledgement request for running in SR mode (subset-026-5.4.4.1 E22). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S24 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-5.4.3.2 S20 c) |  |
|  |  | Subset-026-5.4.3.3 - Following E10, E12, E30, E31, E32, on-board deletes stored position data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to select “Start” (starting condition within the scope of the Start Of Mission procedure) at any time. Once the driver has selected ”Start”, an acknowledgement request for running in SR mode is displayed on the DMI. The driver acknowledges. Check on the JRU that : The selection by the driver of “Start” is recorded. The acknowledgement of the driver is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | INVALID | - |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 2 VALID | L1 - ERTMS/ETCS Level 1 has been entered or confirmed by the driver |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The On-Board equipment allows the driver to select “Start“. | O | DMI | ERTMS Level 1 and SB mode are displayed on the DMI. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | SB | The driver selects ”Start”. | I | DMI | - | L1 | SB | - |
| 2 | L1 | SB | Driver’s selection of “Start” is recorded on the JRU | O | JRU | Triggered by: The driver acts on the on-board DMI. | L1 | SB | - |
| 3 | L1 | SB | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode | L1 | SB | - |
| 4 | L1 | SB | The acknowledgement request for running in Staff Responsible mode is recorded | O | JRU | - | L1 | SB | - |
| 5 | L1 | SB | The driver acknowledges “Staff Responsible” | I | DMI | Symbol MO10 | L1 | SB | - |
| 6 | L1 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded | O | JRU | - | L1 | SB | - |
| 7 | L1 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - | L1 | SR | - |
| 8 | L1 | SR | The Mode symbol “Staff Responsible” is displayed | O | DMI | - | L1 | SR | - |
| 9 | L1 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - | L1 | SR | - |
| 10 | L1 | SR | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded | O | JRU | - | L1 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Train Position | UNKNOWN | - |
| ERTMS/ETCS level | 2 UNCHANGED | L1 |
| Driver ID | VALID | Unchanged |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 2 or 3: Driver presses start, an MA request is sent and the On-board receives an SR authorisation. |  | 17 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB mode and in Level 2 or 3, the on-board equipment: Allows the driver to requests “Start” Sends the radio message 132 “MA request” to the RBC as soon as the driver selects ”Start” and the RBC responds, Displays on the DMI an acknowledgement request for running in SR mode when the radio message 2 “SR authorisation” is received on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-5.4.3.2 S20 d) |  |
|  |  | Subset-026-5.4.3.2 S21 |  |
|  |  | Subset-026-5.4.3.2 S24 |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to select ”Start”(starting condition within the scope of the Start Of Mission procedure) at any time. An acknowledgement request for running in SR mode is displayed on the DMI as soon as the radio message 2 “SR authorisation” is received from the RBC. Check on the JRU that : The selection by the driver of ”Start” is recorded. Recording related to the MA request to the RBC. The radio message 2 “SR authorisation” received from the RBC is recorded. Check on the RTM that : After the driver has selected ”Start”, the on-board equipment requests the RBC for a MA. The radio message 2 “SR authorisation” is received from the RBC. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | A radio communication session is established. |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 ERTMS/ETCS Level 2 or 3 has been entered or confirmed by the driver |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| Desk open | I | TIU | - |
| The On-Board equipment allows the driver to select “Start“. | O | DMI | ERTMS Level 2 or 3 and SB mode are displayed on the DMI. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | T = t’ The driver selects ”Start”. | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Driver’s selection of ”Start” is recorded on the JRU. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | Packet 1 must be used if position report is based on 2 BG | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded | O | JRU | Packet 1 must be used if position report is based on 2 BG | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2) is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=2) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode It shall also be checked that there is no more request to the RBC for a MA. | N/A | N/A | - |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| Radio communication session | UNCHANGED | - |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| An acknowledgement request for running in Staff Responsible mode is displayed on the DMI. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 2 or 3, the driver selects ”Start”. The on-board equipment then requests the RBC for a Movement Authority and a Movement Authority for OS/LS/SH is received from the RBC while the front end of the train is already inside the OS/LS/SH area. |  | 18 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB mode and in Level 2 or 3, the on-board equipment : Allows the driver to select “Start”, Sends the radio message 132 “MA request” to the RBC as soon as the driver selects “Start” and the RBC responds (covered in FT5040300.20 Old(268.1)). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S20 d) |  |
|  |  | Subset-026-5.4.3.2 S21 |  |
|  |  | Subset-026-5.4.3.2 S25 |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to select ”Start” (starting condition within the scope of the Start Of Mission procedure) at any time. The ack to OS/LS/SH mode is displayed on the DMI. Check on the JRU that : The selection by the driver of ”Start” is recorded. Recording related to the MA request to the RBC. The radio message 3 “Movement Authority” with at least the packets 15, 21, 27 and 80 received from the RBC is recorded. Check on the RTM that : After the driver has selected ”Start”, the on-board equipment requests the RBC for a MA. The radio message 3 “Movement Authority” with at least the packets 15, 21, 27 and 80 is received from the RBC. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The mode profile shall define a OS/LS/SH profile such as the front end of the train is already inside the OS/LS/SH area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | A radio communication session is established. |
| Train Data | VALID | A valid Train Running Number is available on-board |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 ERTMS/ETCS Level 2 or 3 has been entered or confirmed by the driver |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| Desk open | I | TIU | - |
| The On-Board equipment allows the driver to select “Start“. | O | DMI | ERTMS Level 2 or 3 and SB mode are displayed on the DMI. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | T = t’ The driver selects ”Start”. | I | DMI | - | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | Driver’s selection of ”Start”is recorded on the JRU. | O | JRU | - | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | Packet 1 must be used if position report is based on 2 BG | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded | O | JRU | Packet 1 must be used if position report is based on 2 BG | L2 / L3 | SB | - |
| 5 | L2 / L3 | SB | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27, packet 80 (<M_MAMODE=0¦1¦2>) is received | I | RTM | The mode profile (packet 80) shall define a OS/LS/SH profile such as the front end of the train is already inside the OS/LS/SH area. | L2 / L3 | SB | - |
| 6 | L2 / L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3; NID_PACKET=15; NID_PACKET=21; NID_PACKET=27; NID_PACKET=80;<M_MAMODE=0¦1¦2>) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 7 | L2 / L3 | SB | An acknowledgement request for OS/SH/LS mode is displayed to the driver and the driver acknowledges. Mode is changed to OS/SH/LS. | - | - | Use_FT4060300.42: OS Condition Id [15] The on-board equipment immediately switches to OS mode because the front end of the train is inside the OS area. OR Use_FT4060300.45: SH Condition Id [50] The on-board equipment immediately switches to SH mode because the front end of the train is inside the SH area. OR Use_FT4060300.60: LS Condition Id [70] The on-board equipment immediately switches to LS mode because the front end of the train is inside the LS area. | L2 / L3 | OS / SH / LS | - |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 0 / 1 / 2 | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 3 / 12 | OS / SH / LS |
| Radio communication session | UNCHANGED | - |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| OS/LS/SH mode is displayed on the DMI. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 2 or 3: The driver selects ”Start”. The on-board equipment requests the RBC for a Movement Authority and a Movement Authority for FS mode is received from the RBC together with a Train running number. |  | 19 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB mode and in Level 2 or 3, the on-board equipment: Allows the driver to select “Start”. Sends the radio message 132 “MA request” to the RBC as soon as the driver selects “Start” and the RBC responds. Movement Authority for FS mode is received from the RBC together with a Train running number |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S20 d) |  |
|  |  | Subset-026-5.4.3.2 S21 |  |
|  |  | Subset-026-3.18.4.5.1 |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to select “Start” (starting condition within the scope of the Start Of Mission procedure) at any time. The FS mode is displayed on the DMI (see the end conditions of FT4060300.48 “Train Data is valid and MA and track description is available and mode profile requires no specific mode (transition [10])”). Check on the JRU that : The selection by the driver of the “Start of Mission” procedure is recorded. Recording related to the MA request to the RBC. The radio message 3 “Movement Authority” with at least the packets 15, 21, 27 and 140 but without the packet 80 received from the RBC is recorded. Check on the RTM that : After the driver has selected ”Start”, the on-board equipment requests the RBC for a MA. The radio message 3 “Movement Authority” with at least the packets 15, 21, 27 and 140 but without the packet 80 is received from the RBC. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. There is no mode profile packet 80 included in the radio message 3 “Movement Authority”. The radio message 3 received from the RBC is complete and valid, i.e. all packets necessary for the on-board equipment to switch to FS are available on-board and correct (packets 15, 21 and 27). Packet 140 is included in the message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | A radio communication session is established. |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 ERTMS/ETCS Level 2 or 3 has been entered or confirmed by the driver |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| Desk open | I | TIU | - |
| The On-Board equipment allows the driver to select “Start“. | O | DMI | ERTMS Level 2 or 3 and SB mode are displayed on the DMI. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | SB | T = t’ The driver selects ”Start”. | I | DMI | - | N/A | SB | - |
| 2 | N/A | SB | Driver’s selection of ”Start” is recorded on the JRU. | O | JRU | - | N/A | SB | - |
| 3 | N/A | SB | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | Packet 1 must be used if position report is based on 2 BG | N/A | SB | - |
| 4 | N/A | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded | O | JRU | Packet 1 must be used if position report is based on 2 BG | N/A | SB | - |
| 5 | N/A | SB | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27, packet 140 is received | I | RTM | - | N/A | SB | - |
| 6 | N/A | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3; NID_PACKET=15; NID_PACKET=21; NID_PACKET=27; NID_PACKET=140) is recorded | O | JRU | - | N/A | SB | - |
| 7 | N/A | SB | The Mode symbol "Full Supervision" is displayed | O | DMI | The on-board equipment immediately switches to FS mode because all conditions required for the on-board equipment to switch to FS mode are met. | N/A | FS | - |
| 8 | N/A | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | N/A | FS | - |
| 9 | N/A | FS | The Main Window is DISPLAYED | O | DMI | - | N/A | FS | - |
| 10 | N/A | FS | The driver selects “Train Running Number” | I | DMI | - | N/A | FS | - |
| 11 | N/A | FS | The Train Running Numer Window is DISPLAYED | O | DMI | The Train Running is equal to the NID_OPERATIONAL received in packet 140 | N/A | FS | - |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 140 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | UNCHANGED | - |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |
| Train Running Number | STORED | The NID_OPERATIONAL received in packet 140 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| FS mode is displayed on the DMI | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Train reports unknown position and RBC rejects the train |  | 20 |
| Applicable Mode/Level Combinations | L2/L3: SB |  |  |
| Target of Test | The target of this test is to check that within the procedure of start of mission in level 2 or 3 with session opened : On-board reports unknown position to RBC (A34) RBC is not able to confirm position (D33) RBC does not accept the train (D22) RBC reports "train rejected " (A38), with NID_LRBG set to unknown (special reserved value : 16777215). On-board Equipment terminates session (A39). On-board Equipment informs driver (A40) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-4.8.3.1.1 Train Rejected |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 Train Rejected |  |
|  |  | Subset-026-4.7.2 table - Output informationTrain is rejected |  |
|  |  | Subset-026-5.4.3.2 D32 |  |
|  |  | Subset-026-5.4.3.2 A34 |  |
|  |  | Subset-026-5.4.3.2 A39 |  |
|  |  | Subset-026-5.4.3.2 A40 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.15 Message 157: SoM Position Report |  |
|  |  | Subset-026-8.7.18 Message 40: Train Rejected |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that On-board Equipment informs driver that the train has been Rejected by the RBC. The driver has to unlock the situation to continue e.g. selection of new level. Check on JRU the messages exchanged between RBC and On-board Equipment. |
| Constraints | The train has to be in SB MODE with session open. A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | UNKNOWN | Q_STATUS = 10b (Unknown) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver revalidates the RBC-ID/phone if invalid | - | - | Use_FT5040300.39 “If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid”.” | N/A | N/A | - |
| 2 | N/A | N/A | Radio Communication session is established | - | - | Use_FT3050300.23 (Level L2/L3): “Establishing a radio communication initiated by on-board to RBC via RBC ID and RBC Phone Number” | N/A | N/A | - |
| 3 | N/A | N/A | Onboard reports unknown position to RBC | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | Message 157 RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | RBC reports TRAIN REJECTED (A38) | I | RTM | Stored message 40, with NID_LRBG set to unknown | N/A | N/A | - |
| 6 | N/A | N/A | Message 40 RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | On-board Equipment terminates the session | - | - | Use_FT3050500.7 Termination of communication session due to error | N/A | N/A | - |
| 8 | N/A | N/A | On-board equipment informs driver with the system status message “Train is rejected”. | O | DMI | - | N/A | N/A | - |

| Step 3: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | SoM position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 10 | Unknown |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | 16777215 | UNKNOWN |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 40 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 40 | Train rejected |
| L_MESSAGE | 10 | 74 | If no optional packet |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| M_ACK | 1 | 0 | No ack required |
| NID_LRBG | 10+14 | 16777215 | UNKNOWN |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | UNKNOWN | - |
| ERTMS/ETCS level | 3/4 | L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message “Train is rejected” is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB or PT mode and in ERTMS Level 2 or 3, the on-board equipment cyclically requests the RBC for a Movement Authority from the time the driver requests the “Start of Mission” until the RBC responds to the MA request. No MA request parameters were received before and the default values are used |  | 21 |
| Applicable Mode/Level Combinations | L2: SB, PT L3: SB, PT |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB or PT mode and in Level 2 or 3, the on-board equipment : Sends the radio message 132 “MA request” to the RBC after the driver has requested ”Start”, Cyclically requests the RBC for a MA until reception from the RBC of the radio message 2 or 3. Note: This test case shall be tested together with the feature FT5040300 “Initiation of Start of Mission through driver”. The “Start” is requested. Valid responses from the RBC after the cyclical MA request by the on-board equipment are : Radio message 2 “SR authorisation” – Covered in FT5040300.17 . On Sight / Shunting MA (OS / LS / SH mode) : radio message 3 with packets 21 & 27 & 80 – Covered in FT5040300.18 Full Supervision MA (FS mode) : radio message 3 with packets 21 & 27 – Covered in FT5040300.19 . |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.6 |  |
|  |  | Subset-026-3.8.2.7 |  |
|  |  | Subset-026-3.8.2.8 a) |  |
|  |  | Subset-026-A.3.1 - MA request repetition cycle, default value |  |
|  |  | Subset-026-5.4.3.2 S21 |  |
|  |  | Subset-026-4.7.2 table - Input information Start |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |
|  |  | Subset-026-A.3.1 - MA request repetition cycle, default value |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that : The radio message 132 “MA request” is sent to the RBC immediately after the driver has selected “Start” on the DMI. The radio message 132 is cyclically sent to the RBC and that the cycle time is equal to T_ CYCRQSTD seconds (until reception of the radio message 2 or 3 from the RBC). The difference of time between two consecutive MA requests is equal to T_ CYCRQSTD. The radio message 132 is NO LONGER sent to the RBC as soon as the radio message 2 or 3 is received from the RBC. In message 132, the variable Q_MARQSTREASON=xxxx1 (Start selected by driver). Check on the JRU that : The radio message 132 “MA request” sent to the RBC immediately after the driver has selected “Start” on the DMI is recorded. The radio message 132 cyclically sent to the RBC is recorded The difference of time between two consecutive MA requests is recorded and is equal to T_ CYCRQSTD. There are no more recordings of radio message 132 sent to the RBC as soon as the radio message 2 or 3 is received from the RBC. In message 132, the variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The RBC does NOT respond to the MA request from the on-board equipment during a time at least equal to N * T_ CYCRQSTD (T_ CYCRQSTD is the time between two cyclic MA requests and N ( 1 shall be chosen by the tester). If T_CYCRQST = 255, the on-board equipment will request ONLY ONCE the RBC for a new MA ( N is then equal to 0. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Radio communication session | ESTABLISHED | A radio communication session is established. |
| MA Request Parameters | UNKNOWN | Default values are used. T_CYCRQST=60s. |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 ERTMS/ETCS Level 2 or 3 has been entered or confirmed by the driver |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| Desk open | I | TIU | - |
| The “Start” button is AVAILABLE. ERTMS Level 2 or 3 and SB/PT mode are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects ”Start”. | I | DMI | Action performed within Use_FT5040300.17-19 . | N/A | N/A | - |
| 2 | N/A | N/A | T = t’ The on-board equipment sends the radio message 132 “MA request“ to the RBC. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). | O | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | The radio message 132 “MA request” sent to the RBC is recorded. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | T = t’ + T_ CYCRQSTD The on-board equipment sends the radio message 132 “MA request“ to the RBC. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). | O | RTM | Check that the difference of time between two consecutive MA requests is effectively equal to T_ CYCRQSTD = 60s (default value) | N/A | N/A | - |
| 5 | N/A | N/A | The radio message 132 “MA request” sent to the RBC is recorded. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | t’ + T_ CYCRQSTD < T < t’ + N * T_ CYCRQSTD The radio message 132 “MA request” is sent cyclically to the RBC every T_ CYCRQSTD seconds (= 60s ) (+ recorded on the JRU). The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). | O | RTM | Radio message 132 is sent to the RBC through the RTM (step i) Radio message 132 sent to the RBC is recorded on the JRU (step i+1) N ( 2 and K = 2*N + 1 (N is chosen by the tester) ( The MA request is performed at least twice. Check that the difference of time between two consecutive MA requests is effectively equal to T_ CYCRQSTD. | N/A | N/A | - |
| 7 | N/A | N/A | The radio message 132 “MA request” sent to the RBC is recorded. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | T = t’ + N * T_ CYCRQSTD The on-board equipment sends the radio message 132 “MA request“ to the RBC. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). | O | RTM | Check that the difference of time between two consecutive MA requests is effectively equal to T_ CYCRQSTD = 60s (default value) | N/A | N/A | - |
| 9 | N/A | N/A | The radio message 132 “MA request” sent to the RBC is recorded. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t’ (step 2) / t’ + T_ CYCRQSTD ( step 4) / … t’ + N * T_ CYCRQSTD (step 2*N + 2) |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| Q_MARQSTREASON | 5 | xxxx1 | Start selected by driver |
| NID_PACKET | 8 | 0 | Or NID_PACKET=1, Position report OR position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | ID of LRBG |
| NID_PRVBG | 10+14 | FINITE VALUE | ID of Previous LRBG ONLY if NID_PACKET = 1 |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 / 1 | Reverse or nominal |
| Q_DLRBG | 2 | 0 / 1 | Reverse or nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | Train integrity is confirmed |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 / 1 | Reverse or nominal |
| M_MODE | 4 | 6 / 8 | SB / PT |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 4: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t’ (step 1) / t’ + T_ CYCRQSTD ( step 3) / … t’ + N * T_ CYCRQSTD (step 2*N + 1) |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| Q_MARQSTREASON | 5 | xxxx1 | Start selected by driver |
| NID_PACKET | 8 | 0 | Or NID_PACKET=1, Position report OR position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | ID of LRBG |
| NID_PRVBG | 10+14 | FINITE VALUE | ID of Previous LRBG ONLY if NID_PACKET = 1 |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 / 1 | Reverse or nominal |
| Q_DLRBG | 2 | 0 / 1 | Reverse or nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | Train integrity is confirmed |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 / 1 | Reverse or nominal |
| M_MODE | 4 | 6 / 8 | SB / PT |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 6: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t’ (step 1) / t’ + T_ CYCRQSTD ( step 3) / … t’ + N * T_ CYCRQSTD (step 2*N + 1) |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| Q_MARQSTREASON | 5 | xxxx1 | Start selected by driver |
| NID_PACKET | 8 | 0 | Or NID_PACKET=1, Position report OR position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | ID of LRBG |
| NID_PRVBG | 10+14 | FINITE VALUE | ID of Previous LRBG ONLY if NID_PACKET = 1 |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 / 1 | Reverse or nominal |
| Q_DLRBG | 2 | 0 / 1 | Reverse or nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | Train integrity is confirmed |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 / 1 | Reverse or nominal |
| M_MODE | 4 | 6 / 8 | SB / PT |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 8: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t’ (step 1) / t’ + T_ CYCRQSTD ( step 3) / … t’ + N * T_ CYCRQSTD (step 2*N + 1) |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| Q_MARQSTREASON | 5 | xxxx1 | Start selected by driver |
| NID_PACKET | 8 | 0 | Or NID_PACKET=1, Position report OR position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | ID of LRBG |
| NID_PRVBG | 10+14 | FINITE VALUE | ID of Previous LRBG ONLY if NID_PACKET = 1 |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 / 1 | Reverse or nominal |
| Q_DLRBG | 2 | 0 / 1 | Reverse or nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | Train integrity is confirmed |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 / 1 | Reverse or nominal |
| M_MODE | 4 | 6 / 8 | SB / PT |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 UNCHANGED | SB / PT |
| Radio communication session | UNCHANGED | - |
| MA Request Parameters | UNCHANGED | - |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | On-board Equipment reports its valid position to RBC at start up |  | 22 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that, within the procedure of start of mission in SB mode, the On-board Equipment reports its valid position to RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 A31 |  |
|  |  | Subset-026-5.4.3.2 D31 |  |
|  |  | Subset-026-5.4.3.2 D32 |  |
|  |  | Subset-026-5.4.3.2 A33 |  |
|  |  | Subset-026-5.4.3.3 - Following D31: session has been successfully opened |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.6.15 Message 157: SoM position report |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM/JRU that Radio communication session is opened (Initiation of a communication session, Session established). Check on the RTM/JRU that On-board reports “valid position to RBC” |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159; NID_PACKET=2) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157, Q_STATUS=1) containing packet 0 is transmitted | O | RTM | Packet 1 must be used if position report is based on 2 BG | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=157; Q_STATUS=1; NID_PACKET=0) is recorded | O | JRU | Packet 1 must be used if position report is based on 2 BG | N/A | N/A | - |

| Step 3: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 5: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 7: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 1 | Valid |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 6 | SB |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| Train Position | VALID | Q_STATUS = 1 |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L 2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level (L2 or L3) and Mode are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | During a start of mission, the driver shall have the opportunity to remove an existing VBC. |  | 23 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that during a start of mission, the on-board equipment allows the driver to remove an existing VBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.9.2 |  |
|  |  | Subset-026-3.15.9.5 c) |  |
|  |  | Subset-026-4.7.2 table - Input information Virtual Balise Cover |  |
|  |  | Subset-026-5.4.3.2 S1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver is allowed to remove an existing VBC. Check on the JRU that : The virtual balise cover remove by the driver is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. There is, at least, one VBC stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The driver is allowed to set/remove a Virtual Balise Cover. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment offers the possibility to set/remove a Virtual Balise Cover. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver chooses to set/remove a Virtual Balise Cover | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver removes an existing VBC. | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The deletion of VBC is RECORDED. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Start of Mission procedure is engaged |  | 24 |
| Applicable Mode/Level Combinations | L0: SB L1: SB L2: SB L3: SB LNTC: SB |  |  |
| Target of Test | The target of this test case is to check that: the Start of Mission procedure is engaged. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that after opening the desk the mode SB is DISPLAYED. |
| Constraints | Mode changes to SB are not tested in the present feature. No communication session is established or is being established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| All desks closed | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Driver opens desk | I | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | The “CAB STATUS” message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The actual mode SB is DISPLAYED | O | DMI | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 UNCHANGED | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| One desk open | I | TIU | - |
| Mode is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | During a start of mission, the driver shall have the opportunity to set a new VBC. |  | 25 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that during a start of mission, the on-board equipment allows the driver to set a new VBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.9.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Virtual Balise Cover |  |
|  |  | Subset-026-5.4.3.2 S1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is allowed to set a new VBC. Check on the JRU that: The virtual balise cover set by the driver is recorded and is identical to the data entered by the driver. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The driver is allowed to set/remove a Virtual Balise Cover. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment offers the possibility to set/remove a Virtual Balise Cover. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver chooses to set/remove a Virtual Balise Cover | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver sets and validate a new VBC. | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The new VBC is RECORDED. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the procedure ends as soon as the desk is closed |  | 26 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that the Start of Mission procedure ends as soon as the desk is closed and start a SoM procedure from the beginning when desk is reopens. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2.1 |  |
|  |  | Subset-026-5.4.3.2.1 b) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver can select either SH or NL (“Non-Leading” button is enabled if the “non leading” input signal is received) or ‘Train Data Entry’ The on-board equipment requires the driver to enter a Driver ID (start a SoM procedure from the beginning) Check on the TIU that : The driver closes the desk The driver opens the desk Check on the JRU that : “CAB STATUS” closed and open are RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1 / L2 / L 3 |
| Driver ID | VALID | A valid Driver ID is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The driver can select either SH or NL or ‘Train Data Entry’. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver can select either SH or NL or ‘Train Data Entry’ | O | DMI | Starting conditions The “Non-Leading” button is enabled if the “non leading” input signal is received. | N/A | N/A | - |
| 2 | N/A | N/A | The driver closes the desk | I | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | “CAB STATUS” closed is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver opens the desk | I | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | “CAB STATUS” open is RECORDED | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The on-board equipment requires the driver to enter a Driver ID | O | DMI | The SoM procedure starts from the beginning | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The on-board equipment requires the driver to enter a Driver ID. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The driver requires “Start” within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 0. |  | 27 |
| Applicable Mode/Level Combinations | L0: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB mode and in Level 0, the on-board equipment : Allows the driver to requests ‘Start’ Displays an acknowledgement request for running in UN mode on the DMI as soon as the driver requested ‘Start’, Requests (waits for) the driver to acknowledge the acknowledgement request for running in UN mode (subset-026-5.4.4.1 E21). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S23 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-5.4.3.2 S20 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to request ‘Start’ (starting condition within the scope of the Start Of Mission procedure at any time. Once the driver has requested ‘Start’, the On-Board equipment requests the driver to acknowledge for running in UN mode. Check on the JRU that : The selection by the driver of ‘Start’ is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 0 VALID | L0 - ERTMS/ETCS Level 0 has been entered or confirmed by the driver |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The On-Board equipment requests the driver to select “Start“. | O | DMI | ERTMS Level 0 and SB mode are displayed on the DMI. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects ”Start” . | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Driver’s selection of ”Start” is recorded on the JRU. | O | JRU | Triggered by: The driver acts on the on-board MMI. | N/A | N/A | - |
| 3 | N/A | N/A | An acknowledgement request for running in Unfitted mode is displayed on the DMI | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Unfitted mode | N/A | N/A | - |
| 4 | N/A | N/A | Acknowledgement request for running in Unfitted mode is recorded on the JRU | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| Train Data | UNCHANGED | - |
| ERTMS/ETCS level | 0 UNCHANGED | L0 |
| Train Running Number | UNCHANGED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| An acknowledgement request for running in Unfitted mode is displayed on the DMI. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The train is at standstill and the “non leading” input signal is received. The driver selects NL. The system switches to NL. |  | 28 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to NL mode when the driver requests NL mode If the position is invalid, the train position data is deleted (new status: “unknown”) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 10 column 2 |  |
|  |  | Subset-026-5.4.3.2.1 |  |
|  |  | Subset-026-5.4.3.2.1 a) |  |
|  |  | Subset-026-4.6.3 Condition Id [46] |  |
|  |  | Subset-026-4.7.2 table - Input information Non Leading request |  |
|  |  | Subset-026-5.4.3.3 - Following E10, E12, E30, E31, E32, on-board deletes stored position data |  |
|  |  | Subset-026-5.4.3.2 S10 d) |  |
|  |  | Subset-026-5.4.3.2 S10 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to NL. Check on the JRU that : The new current mode NL is recorded. Check on the TIU that : The “non leading” input signal is received. |
| Constraints | Train is at standstill. The “non leading” input signal is received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | INVALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 VALID | L0 / LNTC / L1 |
| Driver ID | VALID | A valid Driver ID is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | SB | The non-leading input has the value “Non-leading permitted”. | I | TIU | - | L0 / LNTC / L1 | SB | - |
| 2 | L0 / LNTC / L1 | SB | The train is at standstill and the driver selects Non Leading | I | DMI | - | L0 / LNTC / L1 | SB | - |
| 3 | L0 / LNTC / L1 | SB | Selection of NL by the driver is RECORDED | O | JRU | - | L0 / LNTC / L1 | SB | - |
| 4 | L0 / LNTC / L1 | SB | The on-board equipment switches to NL mode. The driver is informed on the DMI of the change to NL mode. | O | DMI | - | L0 / LNTC / L1 | NL | - |
| 5 | L0 / LNTC / L1 | NL | The mode change to NL is stored. | O | JRU | This ends the SoM procedure | L0 / LNTC / L1 | NL | - |
| 6 | L0 / LNTC / L1 | NL | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded | O | JRU | On-board Equipment deletes stored position data in case they were invalid (new status: “unknown”). | L0 / LNTC / L1 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Train Position | UNKNOWN | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NL mode is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 29

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The train is at standstill and the “non leading” input signal is received. The driver selects NL. The system switches to NL. |  | 29 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that : on-board switches to NL mode when the driver requests NL mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 10 column 2 |  |
|  |  | Subset-026-5.4.3.2.1 |  |
|  |  | Subset-026-5.4.3.2.1 a) |  |
|  |  | Subset-026-4.6.3 Condition Id [46] |  |
|  |  | Subset-026-4.7.2 table - Input information Non Leading request |  |
|  |  | Subset-026-5.4.3.2 S10 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to NL. Check on the JRU that : The new current mode NL is recorded. Check on the TIU that : The “non leading” input signal is received. |
| Constraints | Train is at standstill. The “non leading” input signal is received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | UNKNOWN/VALID | - |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | The non-leading input has the value “Non-leading permitted”. | I | TIU | - | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | The train is at standstill and the driver selects Non Leading | I | DMI | - | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | Selection of NL by the driver is RECORDED | O | JRU | - | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | The on-board equipment switches to NL mode. The driver is informed on the DMI of the change to NL mode. | O | DMI | - | L2 / L3 | NL | - |
| 5 | L2 / L3 | NL | The mode change to NL is stored. | O | JRU | This ends the SoM procedure | L2 / L3 | NL | - |
| 6 | L2 / L3 | NL | Use_FT3060500.13 : Inform trackside about mode changes with existing communication session | - | - | - | L2 / L3 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | In L2/L3 only |
| Train Position | UNKNOWN/VALID | If the status of train position was invalid |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NL mode is DISPLAYED. | O | DMI | Unchanged |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 30

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | During the SoM procedure, if the stored train position and level are valid (for example, when closing the desk and making the transition SR-SB) and if level is 2 or 3, and if at least one mobile terminal is registered to a radio network (D7), the process shall go to A31. |  | 30 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that when the stored train position and level are valid within the scope of the Start of Mission procedure in SB in L2 or L3, if at least one mobile terminal is registered to a radio network (D7), the process shall go to A31. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.2 D3 |  |
|  |  | Subset-026-5.4.3.2 D7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | - |
| Constraints | The on-board equipment is in normal operation during a mission. At least one Mobile Terminal is registered to a Radio Network. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| Desk open | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN=0 | I | INT | Train is still at standstill |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Driver sets Driver ID | - | - | Use_FT5040300.7-9 | N/A | N/A | - |
| 2 | N/A | N/A | On-board contacts RBC and reports “valid position” | - | - | Use_FT5040300.22 | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN=0 | I | INT | Train is still at standstill |

## Test case 31

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | On-board equipment reports to RBC “invalid train position” and RBC confirms train position |  | 31 |
| Applicable Mode/Level Combinations | L2/L3 : SB |  |  |
| Target of Test | The target of this test is to check that within the procedure of start of mission in level 2 or 3 with session opened: On-board reports invalid position to RBC (A34), RBC is able to confirm position (D33) and reports to On-board "Valid position" (A35). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 SoM Position Report Confirmed by RBC |  |
|  |  | Subset-026-4.8.4.2 SoM Position Report Confirmed by RBC |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-5.4.3.2 A34 |  |
|  |  | Subset-026-5.4.3.2 A35 |  |
|  |  | Subset-026-5.4.3.2 D32 |  |
|  |  | Subset-026-5.4.3.3 - A35: RBC reports to on-board: position valid |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.15 Message 157: SoM Position Report |  |
|  |  | Subset-026-8.7.21 Message 43: SoM position report confirmed by RBC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI the final status of the interface with the driver (S10): the driver is offered to select among several active options. To continue the procedure of Start of mission, the driver can select : NL mode (“Non-Leading” button is enabled if the “non leading” input signal is received), SH mode and Data Entry in SB mode. Check on JRU the messages exchanged between RBC and On-board Equipment. |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | INVALID | Q_STATUS of SoM position report Message 157 is invalid (Q_STATUS=0). NID_LRBG and D_LRBG has a FINITE VALUE. Q_DIRLRBG NOMINAL: Qualifier telling on which side of the LRBG the estimated front end is. It has to be set as Q_DIRLRBG in those cases the train did not enter in SB mode from reverse mode RV. Q_DIRTRAIN NOMINAL: Direction of the train movement in relation to the LRBG orientation. It refers to the movement of the train when passed the LRBG in previous mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver revalidates the RBC-ID/phone if invalid | - | - | Use_FT5040300.39 “If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid”.” | N/A | N/A | - |
| 2 | N/A | N/A | Radio Communication session is established | - | - | Use_FT3050300.23 “Establishing a radio communication initiated by on-board to RBC via RBC ID and RBC phone number” | N/A | N/A | - |
| 3 | N/A | N/A | On-board reports invalid position to RBC (A34) | O | RTM | MESSAGE 157, SoM position report | N/A | N/A | - |
| 4 | N/A | N/A | Message recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | RBC is able to confirm position (D33), it reports valid position (A35) | I | RTM | Message 43. SoM position report confirmed by RBC | N/A | N/A | - |
| 6 | N/A | N/A | Message RECORDED | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | Message 157 SoM Position Report On-board Equipment to RBC , including Position report, are stored |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 00 | Invalid |
| NID_PACKET | 8 | 0 | Packet 0 has to be prepared by On-board Equipment for sending message 157, the variable are available and stored onboard |
| L _PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | NOMINAL | - |
| Q_DLRBG | 2 | NOMINAL | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or Q_LENGTH = 2 |
| V_TRAIN | 7 | 0 | - |
| Q_DIRTRAIN | 2 | NOMINAL | - |
| M_MODE | 4 | 6 | SB |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 5: Radio Message 43 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 43 | MESSAGE 43 SoM POSITION REPORT confirmed by RBC |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 10+14 | FINITE VALUE | The LRBG of the SoM report |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | VALID | Q_STATUS = 1 |
| ERTMS/ETCS level | 3/4 UNCHANGED | L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train data entry or NL or SH displayed | O | DMI | Driver is offered to select (Procedure S10) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 32

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | On-board reports invalid position and RBC accepts the train |  | 32 |
| Applicable Mode/Level Combinations | L2/L3: SB |  |  |
| Target of Test | The target of this test is to check that within the procedure of start of mission in level 2 or 3 with session opened: On-board reports invalid position to RBC, (A34) RBC is not able to confirm position (D33) RBC accepts the train (D22, A23, A24) RBC reports "train accepted" with NID_LRBG set to unknown (special reserved value : 16777215) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |
|  |  | Subset-026-3.6.2.2.2.1 |  |
|  |  | Subset-026-3.6.2.2.2.3 |  |
|  |  | Subset-026-4.8.3.1.1 Train Accepted |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 Train Accepted |  |
|  |  | Subset-026-5.4.3.2 A24 |  |
|  |  | Subset-026-5.4.3.2 A34 |  |
|  |  | Subset-026-5.4.3.2 D32 |  |
|  |  | Subset-026-5.4.3.3 - A24: On-board deletes stored position data |  |
|  |  | Subset-026-8.6.15 Message 157: SoM Position Report |  |
|  |  | Subset-026-8.7.19 Message 41: Train Accepted |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI the final status of the interface with the driver: the driver is offered to select among several active options. To continue the procedure of Start of mission, the driver can select : NL mode (“Non-Leading” button is enabled if the “non leading” input signal is received), SH mode, Data Entry. Check on JRU the messages exchanged between RBC and On-board Equipment and the deletion of the stored position data. |
| Constraints | The train has to be in SB MODE with session open. A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | INVALID | Q_STATUS = 00b (invalid) NID_LRBG and D_LRBG has a FINITE VALUE. Q_DIRLRBG NOMINAL: Qualifier telling on which side of the LRBG the estimated front end is. It has to be set as Q_DIRLRBG in those cases the train did not enter in SB mode from reverse mode RV. Q_DIRTRAIN NOMINAL: Direction of the train movement in relation to the LRBG orientation. It refers to the movement of the train when passed the LRBG in previous mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver revalidates the RBC-ID/phone if invalid | - | - | Use_FT5040300.39 “If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid”.” | N/A | N/A | - |
| 2 | N/A | N/A | Radio Communication session is established | - | - | Use_FT3050300.23 “Establishing a radio communication initiated by on-board to RBC via RBC ID and RBC phone number” | N/A | N/A | - |
| 3 | N/A | N/A | Onboard reports invalid position to RBC | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SoM position report RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | RBC is Not able to confirm position (D33), it accepts the train (D22), it reports train accepted (A23) | I | RTM | message 41, with NID_LRBG set to unknown | N/A | N/A | - |
| 6 | N/A | N/A | message 41, with NID_LRBG set to unknown RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded | O | JRU | On-board Equipment deletes stored position data in case they were invalid (new status: “unknown”). | N/A | N/A | - |

| Step 3: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 00 | INVALID |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 41 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 41 | MESSAGE 41 Train accepted |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 10+14 | 16777215 | UNKNOWN |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | UNKNOWN | NID_LRBG = 16777215 (unknown) |
| ERTMS/ETCS level | 3/4 | L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train data entry or NL or SH displayed | O | DMI | Driver is offered to select (Procedure S10) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 33

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Train reports invalid position and RBC rejects the train |  | 33 |
| Applicable Mode/Level Combinations | L2/L3: SB |  |  |
| Target of Test | The target of this test is to check that within the procedure of start of mission in level 2 or 3 with session opened : On-board reports invalid position to RBC (A34) RBC is not able to confirm position (D33) RBC does not accept the train (D22) RBC reports "train rejected " (A38), with NID_LRBG set to unknown (special reserved value : 16777215). On-board Equipment terminates session (A39) and deletes stored position data. On-board Equipment informs driver (A40) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-4.8.3.1.1 Train Rejected |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 Train Rejected |  |
|  |  | Subset-026-4.7.2 table - Output informationTrain is rejected |  |
|  |  | Subset-026-5.4.3.2 D32 |  |
|  |  | Subset-026-5.4.3.2 A34 |  |
|  |  | Subset-026-5.4.3.2 A39 |  |
|  |  | Subset-026-5.4.3.2 A40 |  |
|  |  | Subset-026-5.4.3.3 - A39: on-board terminates session, deletes stored position data |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.15 Message 157: SoM Position Report |  |
|  |  | Subset-026-8.7.18 Message 40: Train Rejected |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that On-board Equipment informs driver that the train has been Rejected by the RBC. The driver has to unlock the situation to continue e.g. selection of new level. Check on JRU the messages exchanged between RBC and On-board Equipment and the deletion of the stored position data. |
| Constraints | The train has to be in SB MODE with session open. A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | INVALID | Q_STATUS = 00b (invalid) NID_LRBG and D_LRBG has a FINITE VALUE. Q_DIRLRBG NOMINAL: Qualifier telling on which side of the LRBG the estimated front end is. It has to be set as Q_DIRLRBG in those cases the train did not enter in SB mode from reverse mode RV. Q_DIRTRAIN NOMINAL: Direction of the train movement in relation to the LRBG orientation. It refers to the movement of the train when passed the LRBG in previous mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver revalidates the RBC-ID/phone if invalid | - | - | Use_FT5040300.39 “If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid”.” | N/A | N/A | - |
| 2 | N/A | N/A | Radio Communication session is established | - | - | Use_FT3050300.23 “Establishing a radio communication initiated by on-board to RBC via RBC ID and RBC phone number” | N/A | N/A | - |
| 3 | N/A | N/A | Onboard reports invalid position to RBC | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | Message 157 RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | RBC reports TRAIN REJECTED (A38) | I | RTM | Stored message 40, with NID_LRBG set to unknown | N/A | N/A | - |
| 6 | N/A | N/A | Message 40 RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded | O | JRU | On-board Equipment deletes stored position data in case they were invalid (new status: “unknown”). | N/A | N/A | - |
| 8 | N/A | N/A | On-board Equipment terminates the session | - | - | Use_FT3050500.7-8 : Termination of communication session due to error | N/A | N/A | - |
| 9 | N/A | N/A | On-board equipment informs driver with the system status message “Train is rejected”. | O | DMI | - | N/A | N/A | - |

| Step 3: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | SoM position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 00 | Invalid |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 40 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 40 | Train rejected |
| L_MESSAGE | 10 | 74 | If no optional packet |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| M_ACK | 1 | 0 | No ack required |
| NID_LRBG | 10+14 | 16777215 | UNKNOWN |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | UNKNOWN | Deleted |
| ERTMS/ETCS level | 3/4 | L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message “Train is rejected” is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 34

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | During the SoM procedure, if the stored train position and level are valid (for example, when closing the desk and making the transition SR-SB) and if level is 2 or 3, and if no mobile terminal is registered to a radio network (D7), the ERTMS/ETCS on-board equipment shall wait a sufficient time is elapsed since it has sent the latest Radio Network registration order to a Mobile Terminal (S4); in such case the driver shall be informed (A29) and the process shall go to S10. |  | 34 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that - when the stored train position and level are valid within the scope of the Start of Mission procedure in SB in L2 or L3, - if no mobile terminal is registered to a radio network (D7), - the ERTMS/ETCS on-board equipment shall wait a sufficient time is elapsed since it has sent the latest Radio Network registration order to a Mobile Terminal (S4); in such case the driver shall be informed (A29) and the process shall go to S10. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-A.3.1 - Time from the latest Radio Network registration order to a Mobile Terminal after which the registration is considered as failed |  |
|  |  | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-4.7.2 table - Output information Radio Network registration failed |  |
|  |  | Subset-026-5.4.3.2 D3 |  |
|  |  | Subset-026-5.4.3.2 D7 |  |
|  |  | Subset-026-5.4.3.2 S4 |  |
|  |  | Subset-026-5.4.3.2 S4 b) |  |
|  |  | Subset-026-5.4.3.2 A29 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that after entered the driver ID: - the ERTMS/ETCS on-board equipment waits a sufficient time (refer to Appendix A3.1) is elapsed since it has sent the latest Radio Network registration order to a Mobile Terminal (E7); - the driver is informed that the radio network registration has failed (A29) and the on-board goes to S10. |
| Constraints | The on-board equipment is in normal operation during a mission. No Mobile Terminal is registered to a Radio Network. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 There is a valid ERTMS/ETCS Level data stored on-board. |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| Desk open | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Driver sets Driver ID | - | - | Use_FT5040300.7-9 | N/A | N/A | - |
| 2 | N/A | N/A | The Main window is displayed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit42=1>) is recorded | O | DMI | The Main window is presented to the driver with all buttons ‘disabled’. The symbol ST05 is displayed to show that the onboard still awaits the registration to the Radio Network. | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-REGISTRATION.Indication containing a non-empty MNID list is received | I | RTM | After the "Time from the latest Radio Network registration order to a Mobile Terminal after which the registration is considered as failed.", "SA-REGISTRATION.Indication" is not received or is received with an empty MNID list. | N/A | N/A | - |
| 5 | N/A | N/A | The system status message ‘Radio network registration failed’ is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The “DMI SYSTEM STATUS MESSAGE” message is RECORDED with the Bit 23 (Radio network registration failed) = 1 (displayed) for variable SYSTEM_STATUS_MESSAGE. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| The system status message ‘Radio network registration failed’ is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 35

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | On-board equipment offers the driver the possibility to re-enter the Radio Network ID. The driver select a new Radio Network ID from the proposed list. |  | 35 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that, within the procedure S3 of start of mission in SB mode, the on-board equipment offers the driver the possibility to re-enter the Radio Network ID. The driver select a new Radio Network ID from the proposed list. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.2 |  |
|  |  | Subset-026-3.18.4.3.6 |  |
|  |  | Subset-026-4.7.2 table - Input information Radio network-id |  |
|  |  | Subset-026-5.4.3.2 S3 |  |
|  |  | Subset-026-5.4.3.2 S3 b) |  |
|  |  | Subset-026-5.4.3.3 - S3: driver has re-entered a radio network ID |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Radio data window is displayed with the possibility to the driver to select ‘Use short number’, ‘Enter RBC data’ and ‘Radio network ID’. The driver presses the ‘Radio network ID’ button. The Radio Network ID window is displayed with a list of available and allowed networks. The driver selects a new Radio Network ID and validates it. The Radio data window is displayed again. Check on the JRU that : The “RADIO NETWORK ID ENTERED BY THE DRIVER” message is RECORDED with the variable NID_MN identical to the data selected by the driver. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| RBC ID/Phone Number | INVALID / VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level, Mode and Radio data window are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment displays the Radio data window and offers the possibility to the driver to select: - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses the ‘Radio network ID’ button | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The Radio Network ID window is displayed with a list of available and allowed networks. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver selects a new Radio Network ID and validates it. | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The Radio data window is displayed again. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | RADIO NETWORK ID ENTERED BY THE DRIVER (NID_MESSAGE_JRU=48) is recorded | O | JRU | NID_MN is identical to the data selected by the driver | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |
| RBC ID/Phone Number | INVALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level, Mode and Radio data window are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 36

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | On-board equipment offers the driver the possibility to re-enter the Radio Network ID. The list of available and allowed networks is empty. |  | 36 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that, within the procedure S3 of start of mission in SB mode, the on-board equipment offers the driver the possibility to re-enter the Radio Network ID. When the list of available and allowed networks is empty, the driver is informed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.2 |  |
|  |  | Subset-026-4.7.2 table - Output information Radio Network registration failed |  |
|  |  | Subset-026-5.4.3.2 S3 |  |
|  |  | Subset-026-5.4.3.2 S3 a) |  |
|  |  | Subset-026-5.4.3.2 A29 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Radio data window is displayed with the possibility to the driver to select ‘Use short number’, ‘Enter RBC data’ and ‘Radio network ID’. The driver presses the ‘Radio network ID’ button. The system status message ‘Radio network registration failed’ is DISPLAYED. Check on the JRU that : The “DMI SYSTEM STATUS MESSAGE” message is RECORDED with the Bit 23 (Radio network registration failed) = 1 (displayed) for variable SYSTEM_STATUS_MESSAGE. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| RBC ID/Phone Number | INVALID / UNKNOWN | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level, Mode and Radio data window are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment displays the RBC contact window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses the ‘Radio network ID’ button | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The system status message ‘Radio network registration failed’ is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit23=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |
| RBC ID/Phone Number | INVALID / UNKNOWN | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message ‘Radio network registration failed’ is DISPLAYED. | O | DMI | ERTMS/ETCS Level and Mode are DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 37

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | During the SoM procedure, if the stored train position and level are valid (for example, when closing the desk and making the transition SR-SB) and if level is 0, 1 or NTC, the process shall go to S10. |  | 37 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB |  |  |
| Target of Test | The target of this test case is to check that when the stored train position and level are valid within the scope of the Start of Mission procedure in SB in L0, L1 or LNTC, the on-board equipment shall go to S10 following the D2 and D3. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 D3 |  |
|  |  | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.2 S10 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that after entered the driver ID, the on-board goes immediately to S10. |
| Constraints | The on-board equipment is in normal operation during a mission. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 VALID | L0 / LNTC / L1 There is a valid ERTMS/ETCS Level data stored on-board. |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Driver sets Driver ID | - | - | Use_FT5040300.7-9 | N/A | N/A | - |
| 2 | N/A | N/A | The “Non-Leading”, “Shunting”, and “Train data” buttons are enabled. | O | DMI | “Non-Leading” button is enabled if the “non leading” input signal is received. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 0 / 1 / 2 UNCHANGED | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 38

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to enter the RBC-ID/phone number if its status is “unknown”. |  | 38 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that, within the procedure S3 of start of mission in SB mode, the on-board equipment offers the driver the possibility to enter the RBC-ID/phone number if its status is “unknown”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.2 |  |
|  |  | Subset-026-3.18.4.3.5 |  |
|  |  | Subset-026-A.3.11 RBC phone number |  |
|  |  | Subset-026-5.4.3.2 S3 |  |
|  |  | Subset-026-5.4.3.2 S3 e) |  |
|  |  | Subset-026-5.4.3.3 - Following S3: driver has entered RBC ID/phone number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Radio data window is displayed with the possibility to the driver to select ‘Use short number’, ‘Enter RBC data’ and ‘Radio network ID’. The driver presses the ‘Enter RBC data’ button. The RBC data window is displayed with no value for the RBC ID/Phone number (since the status of this information is UNKNOWN). The driver enters valid RBC ID/Phone number. The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). Check on the JRU that : The “RBC CONTACT INFORMATION ENTERED BY THE DRIVER” message is RECORDED with the variable Q_RBCENTRY=2 (Enter RBC data) and with the variables NID_C, NID_RBC and NID_RADIO identical to the data entered by the driver. |
| Constraints | At least one Mobile Terminal registered to a Radio Network RBC ID/Phone Number STATUS is UNKNOWN |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | UNKNOWN | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level, Mode and Radio data window are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment displays the Radio data window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses the ‘Enter RBC data’ button | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The on-board equipment displays the RBC data window with no value for the RBC ID/Phone number (since the status of this information is UNKNOWN). | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver enters valid RBC ID/Phone number. | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded | O | JRU | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level and Mode are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 39

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid” or "valid". |  | 39 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that, within the procedure S3 of start of mission in SB mode, the on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid” or "valid". |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.2 |  |
|  |  | Subset-026-4.7.2 table - Input information RBC contact information - RBC-id |  |
|  |  | Subset-026-4.7.2 table - Input information RBC contact information - RBC phone number |  |
|  |  | Subset-026-5.4.3.2 S3 |  |
|  |  | Subset-026-5.4.3.2 S3 e) |  |
|  |  | Subset-026-5.4.3.3 - Following S3: driver has re-validated/re-entered RBC ID/phone number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Radio data window is displayed with the possibility to the driver to select ‘Use short number’, ‘Enter RBC data’ and ‘Radio network ID’. The driver presses the ‘Enter RBC data’ button. The RBC data window is displayed with an initial value for the RBC ID/Phone number (since the status of this information is INVALID). The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). Check on the JRU that : The “RBC CONTACT INFORMATION ENTERED BY THE DRIVER” message is RECORDED with the variable Q_RBCENTRY=2 (Enter RBC data) and with the variables NID_C, NID_RBC and NID_RADIO identical to the data validated by the driver. |
| Constraints | At least one Mobile Terminal registered to a Radio Network |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | INVALID / VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level, Mode and Radio data window are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment displays the Radio data window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses the ‘Enter RBC data’ button | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The on-board equipment displays the RBC data window with an initial value for the RBC ID/Phone number (since the status of this information is INVALID). | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded | O | JRU | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level and Mode are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 40

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to re-enter the RBC-ID/phone number if its status is “invalid” or "valid". |  | 40 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that, within the procedure S3 of start of mission in SB mode, the on-board equipment offers the driver the possibility to re-enter the RBC-ID/phone number if its status is “invalid” or "valid". The RBC ID is limited must be spread from 0 to 16777214. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.2 |  |
|  |  | Subset-026-3.18.4.3.5 |  |
|  |  | Subset-026-A.3.11 RBC ID |  |
|  |  | Subset-026-4.7.2 table - Input information RBC contact information - RBC-id |  |
|  |  | Subset-026-4.7.2 table - Input information RBC contact information - RBC phone number |  |
|  |  | Subset-026-5.4.3.2 S3 |  |
|  |  | Subset-026-5.4.3.2 S3 e) |  |
|  |  | Subset-026-5.4.3.3 - Following S3: driver has re-validated/re-entered RBC ID/phone number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Radio data window is displayed with the possibility to the driver to select ‘Use short number’, ‘Enter RBC data’ and ‘Radio network ID’. The driver presses the ‘Enter RBC data’ button. The RBC data window is displayed with an initial value for the RBC ID/Phone number (since the status of this information is INVALID). The driver modifies the existing RBC ID/Phone number. RBC ID must be spread from 0 to 16777214. The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). Check on the JRU that : The “RBC CONTACT INFORMATION ENTERED BY THE DRIVER” message is RECORDED with the variable Q_RBCENTRY=2 (Enter RBC data) and with the variables NID_C, NID_RBC and NID_RADIO identical to the data modified by the driver. |
| Constraints | At least one Mobile Terminal registered to a Radio Network |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | INVALID / VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level, Mode and Radio data window are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment displays the Radio data window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses the ‘Enter RBC data’ button | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The on-board equipment displays the RBC data window with an initial value for the RBC ID/Phone number (since the status of this information is INVALID). | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver modifies "RBC Data" | I | DMI | The Driver tries to set a RBC ID value out of [0-16777214] | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Driver validates the new RBC ID | I | DMI | The validation of RBC ID has no effect. | N/A | N/A | - |
| 6 | N/A | N/A | The driver modifies "RBC Data" | I | DMI | The Driver set a RBC ID value in [0-16777214] | N/A | N/A | - |
| 7 | N/A | N/A | The Driver validates the new RBC ID | I | DMI | The validation of RBC ID enable the "RBC data entry complete? Yes" | N/A | N/A | - |
| 8 | N/A | N/A | The driver modifies the existing RBC Phone number. | I | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded | O | JRU | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ERTMS/ETCS Level and Mode are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 41

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Entry of the Train running number through the Driver ID window when the Train running number is unknown |  | 41 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that during the SoM procedure, at S1, it is possible to enter the Train running number through the Driver ID window when the Train running number is unknown. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S1 |  |
|  |  | Subset-026-5.4.3.3 - Following S1: driver has entered train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - It is possible to enter the Train running number through the Driver ID window when the Train running number is unknown |
| Constraints | The on-board equipment is in a SoM at S1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Train Running Number | UNKNOWN | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Driver ID window is displayed | O | DMI | The on-board is in the SoM procedure in S1 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SB | The on-board equipment offers the possibility to enter the Train running number | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SB | The driver enters the "Train Running Number" [DRIVER ID] | I | DMI | Train Running Number entered: NID_OPERATIONAL_1 | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SB | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Train Running Number | VALID | NID_OPERATIONAL_1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 42

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Entry of the Train running number through the Driver ID window when the Train running number is invalid |  | 42 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that during the SoM procedure, at S1, it is possible to enter the Train running number through the Driver ID window when the Train running number is invalid. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S1 |  |
|  |  | Subset-026-5.4.3.3 - Following S1: driver has re-validated/re-entered train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - It is possible to confirm the Train running number through the Driver ID window when the existing Train running number is invalid |
| Constraints | The on-board equipment is in a SoM at S1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Train Running Number | INVALID | NID_OPERATIONAL_1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Driver ID window is displayed | O | DMI | The on-board is in the SoM procedure in S1 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SB | The on-board equipment offers the possibility to re-validate the Train running number | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SB | The driver confirms the "Train Running Number" [DRIVER ID] | I | DMI | Train Running Number entered: NID_OPERATIONAL_1 | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SB | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Train Running Number | VALID | NID_OPERATIONAL_1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 43

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | During the SoM procedure, following S10 or S20, if the driver chooses to re-enter the level, the status of the stored ERTMS/ETCS level and RBC ID/Phone Number changes from valid to invalid. |  | 43 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that during the SoM procedure, following S10 or S20, if the driver chooses to re-enter the level, the status of the stored ERTMS/ETCS level and RBC ID/Phone Number changes from valid to invalid. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.3 - Following S10 or S20: driver chooses to re-enter the level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The status of the ERTMS/ETCS level and of the RBC ID/Phone Number is valid - The driver selects the ERTMS/ETCS Level window - After this selection, the status of the ERTMS/ETCS level and of the RBC ID/Phone Number changes to invalid |
| Constraints | The on-board equipment is in a SoM at S10 or S20. At least one Mobile Terminal is registered to a Radio Network. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| RBC ID/Phone Number | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Main window is displayed | O | DMI | The on-board is in the SoM procedure in S10 or S20 |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN=0 | I | INT | Train is still at standstill |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | The Main window is displayed. | O | DMI | - | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | The Level symbol "L2 / L3" is displayed | O | DMI | The ERTMS/ETCS level status is valid | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | The driver presses "Data view" | I | DMI | - | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | The data from "Data View" window are displayed: RBC ID/Phone Number | O | DMI | The RBC ID/Phone Number status is valid | L2 / L3 | SB | - |
| 5 | L2 / L3 | SB | The driver presses "Level" | I | DMI | - | L2 / L3 | SB | - |
| 6 | L2 / L3 | SB | No Level symbol is displayed in C8 | O | DMI | The ERTMS/ETCS level status is invalid | L2 / L3 | SB | - |
| 7 | L2 / L3 | SB | The invalid ERTMS/ETCS level stored on-board is presented to the driver | O | DMI | - | L2 / L3 | SB | - |
| 8 | L2 / L3 | SB | The driver confirms "Level" | I | DMI | - | L2 / L3 | SB | - |
| 9 | L2 / L3 | SB | The driver presses "Contact last RBC" | I | DMI | This button is available only if RBC ID/Phone Number is invalid | L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN=0 | I | INT | Train is still at standstill |

## Test case 44

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. Radio communication session is NOT established. |  | 44 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB, the on-board equipment : Captures the Train Data and requests the driver to re-validate/enter a complete set of Train Data. Requests the driver to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S12 |  |
|  |  | Subset-026-5.4.3.2 D12 |  |
|  |  | Subset-026-5.4.3.2 D10 |  |
|  |  | Subset-026-5.4.3.2 D11 |  |
|  |  | Subset-026-5.4.3.2 S10 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI: A complete set of not yet validated Train Data is available Check that the driver is allowed to validate the train data or to modify them again The on-board requests the driver to enter/re-validate the Train running number if the status of the Train running number is “unknown” or “invalid”. Check on the JRU that Driver’s validation action is recorded Each data of the presented set of train data is recorded once again after the validation action of the driver. It can be also checked if the newly recorded set of data is identical to the initial set previously recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The train data shall be acquired by the on-board system of a leading engine. The set of Train Data will not be entered by means of external devices. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Data | INVALID / UNKNOWN | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| Desk open | I | TIU | - |
| The Train Data window is displayed to the driver. | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | If the status of the Train data is "invalid"/"unknown", the driver is requested to re-validate/enter the Train data. | - | - | Use_FT5040300.47 (unknown Train data) Use_FT5040300.2 (invalid Train data validated by driver) Use_FT5040300.3 (invalid Train data modified by driver before validation) | N/A | N/A | - |
| 2 | N/A | N/A | If the status of the Train running number is "invalid"/"unknown", the driver is requested to re-validate/enter the Train running number. | - | - | Use_FT5040300.45 (invalid Train running number) OR Use_FT5040300.46 (unknown Train running number) | N/A | N/A | - |
| 3 | N/A | N/A | The "Non-Leading", "Shunting", and "Train data" buttons are enabled. | O | DMI | "Non-Leading" button is enabled if the "non leading" input signal is received. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | Unchanged |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Main window is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 45

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is “invalid”, the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  | 45 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure, after the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is “invalid”, the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.5.1 |  |
|  |  | Subset-026-5.4.3.2 S13 |  |
|  |  | Subset-026-5.4.3.3 - Following S13: driver has re-validated/re-entered train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The on-board requests the driver to re-validate the Train running number and the driver does so. Check on the JRU that : The “TRAIN RUNNING NUMBER ENTERED BY THE DRIVER” message is RECORDED with the variable NID_OPERATIONAL set to the value validated by the driver. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure in S13 after the set of Train Data has been entered/re-validated by the driver (in S12). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2 /L3 |
| Train Running Number | INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The Train running number window with the invalid train running number is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver is requested to re-validate the Train running number. | O | DMI | Displayed Train Running Number: NID_OPERATIONAL_1 | N/A | N/A | - |
| 2 | N/A | N/A | The driver confirms the "Train Running Number" [MAIN] | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 46

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is “unknown”, the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  | 46 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure, after the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is “unknown”, the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.5.1 |  |
|  |  | Subset-026-5.4.3.2 S13 |  |
|  |  | Subset-026-5.4.3.3 - Following S13: driver has entered train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The on-board requests the driver to enter the Train running number and the driver does so. Check on the JRU that : The “TRAIN RUNNING NUMBER ENTERED BY THE DRIVER” message is RECORDED with the variable NID_OPERATIONAL set to the value validated by the driver. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure in S13 after the set of Train Data has been entered/re-validated by the driver (in S12). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2 /L3 |
| Train Running Number | UNKNOWN | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The Train running number window is displayed without any value for the train running number | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver is requested to enter the Train running number. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver enters the "Train Running Number" [MAIN] | I | DMI | Entered Train Running Number: NID_OPERATIONAL_1 | N/A | N/A | - |
| 3 | N/A | N/A | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 47

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  | 47 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB, the on-board equipment : Captures the Train Data and requests the driver to enter a complete set of Train Data. Note: The data input order and the way the on-board equipment displays data that needs to be input is out of the scope of this test case (this is a matter of implementation). As soon as a complete set of train data is available on-board, the on-board equipment will authorise the driver to validate the data. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.3.2 |  |
|  |  | Subset-026-3.18.3.2 a) |  |
|  |  | Subset-026-3.18.3.2 b) |  |
|  |  | Subset-026-3.18.3.2 c) |  |
|  |  | Subset-026-3.18.3.2 d) |  |
|  |  | Subset-026-3.18.3.2 e) |  |
|  |  | Subset-026-3.18.3.2 f) |  |
|  |  | Subset-026-3.18.3.2 g) |  |
|  |  | Subset-026-3.18.3.2 h) |  |
|  |  | Subset-026-3.18.3.2 i) |  |
|  |  | Subset-026-3.18.3.2 k) |  |
|  |  | Subset-026-3.18.3.2.2 |  |
|  |  | Subset-026-3.18.3.3 |  |
|  |  | Subset-026-4.7.2 table - Input information Train Data (refer to 3.18.3.2) |  |
|  |  | Subset-026-5.4.3.2 S12 |  |
|  |  | Subset-026-5.4.3.3 - Following S12: train data have been entered |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : If train data available on-board (e.g. from another source or already stored on-board), these train data are initially displayed to the driver (starting condition within the scope of the Start Of Mission procedure). The driver is not allowed to validate the train data UNTIL the driver has entered a complete set of valid Train Data. When the driver has input a valid set of Train Data, the driver is allowed to validate the train data. Check on the JRU that : Driver‘s validation action is recorded. Each data of the set of Train Data is recorded (Data Entry Completed) and is identical to what the driver has entered. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure in S12 after the driver has selected Train Data (in S10). The train data shall be acquired by the on-board system of a leading engine. The set of Train Data will not be entered by means of external devices. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | UNKNOWN | At least one of the data of the set of Train Data is unknown on-board. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1/ L2 /L3 A valid ERTMS/ETCS Level has been entered/confirmed by the driver |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The Train Data window is displayed to the driver. | O | DMI | Train data validation is not allowed |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. | O | DMI | The driver might be requested to enter the “Train category(ies)”, “Train length”, “Traction / brake parameters”, “Maximum train speed”, “Loading gauge”, “Axle load category”, “Train fitted with airtight system”, but the driver shall never be requested to enter the “Traction system(s) accepted by the engine”, “List of National Systems available on-board” or “Axle number”. | N/A | N/A | - |
| 2 | N/A | N/A | The driver enters each Train data unknown on-board | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver is allowed to validate the train data | O | DMI | A complete set of valid Train Data is available | N/A | N/A | - |
| 4 | N/A | N/A | The driver validates the train data. | I | DMI | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Triggered by : The driver validation. | N/A | N/A | - |
| 6 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | Triggered by : Data Entry completed | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 48

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | On-board reports unknown position and RBC accepts the train |  | 48 |
| Applicable Mode/Level Combinations | L2/L3: SB |  |  |
| Target of Test | The target of this test is to check that within the procedure of start of mission in level 2 or 3 with session opened: On-board reports unknown position to RBC, (A34) RBC is not able to confirm position (D33) RBC accepts the train (D22, A23, A24) RBC reports "train accepted" with NID_LRBG set to unknown (special reserved value : 16777215) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |
|  |  | Subset-026-3.6.2.2.2.1 |  |
|  |  | Subset-026-3.6.2.2.2.3 |  |
|  |  | Subset-026-4.8.3.1.1 Train Accepted |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 Train Accepted |  |
|  |  | Subset-026-5.4.3.2 A24 |  |
|  |  | Subset-026-5.4.3.2 A34 |  |
|  |  | Subset-026-5.4.3.2 D32 |  |
|  |  | Subset-026-8.6.15 Message 157: SoM Position Report |  |
|  |  | Subset-026-8.7.19 Message 41: Train Accepted |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI the final status of the interface with the driver: the driver is offered to select among several active options. To continue the procedure of Start of mission, the driver can select : NL mode (“Non-Leading” button is enabled if the “non leading” input signal is received), SH mode, Data Entry. Check on JRU the messages exchanged between RBC and On-board Equipment . |
| Constraints | The train has to be in SB MODE with session open. A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | UNKNOWN | Q_STATUS = 10b (Unknown) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver revalidates the RBC-ID/phone if invalid | - | - | Use_FT5040300.39 “If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid”.” | N/A | N/A | - |
| 2 | N/A | N/A | Radio Communication session is established | - | - | Use_FT3050300.23-25 “Establishing a radio communication initiated by on-board to RBC via RBC ID and RBC phone number” | N/A | N/A | - |
| 3 | N/A | N/A | Onboard reports unknown position to RBC | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SoM position report RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | RBC is Not able to confirm position (D33), it accepts the train (D22), it reports train accepted (A23) | I | RTM | message 41, with NID_LRBG set to unknown | N/A | N/A | - |
| 6 | N/A | N/A | message 41, with NID_LRBG set to unknown RECORDED | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 10 | UNKNOWN |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | 16777215 | UNKNOWN |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 41 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 41 | MESSAGE 41 Train accepted |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 10+14 | 16777215 | UNKNOWN |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | UNKNOWN | NID_LRBG = 16777215 (unknown) |
| ERTMS/ETCS level | 3/4 | L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train data entry or NL or SH displayed | O | DMI | Driver is offered to select (Procedure S10) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 49

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The driver requires “Start” within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 0 and is allowed to re-enter the level |  | 49 |
| Applicable Mode/Level Combinations | L0: SB |  |  |
| Target of Test | The target of this test case is to check that within the scope of the Start of Mission procedure in SB mode and in Level 0, the on-board equipment : Allows the driver to requests ‘Start’ The driver is allowed to re-enter the level. ERTMS/ETCS Level and RBC ID/PHONE Number became INVALID. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S23 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-5.4.3.2 S20 b) |  |
|  |  | Subset-026-5.4.3.3 - Following S10 or S20: driver chooses to re-enter the level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to request ‘Start’ (starting condition within the scope of the Start Of Mission procedure at any time. The On-Board equipment allows the driver to re-enter the level. The On-Board equipment allows the driver to revalidate the ERTMS/ETCS Level and RBC ID/PHONE Number |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 0 VALID | L0 - ERTMS/ETCS Level 0 has been entered or confirmed by the driver |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| The On-Board equipment requests the driver to select “Start“. | O | DMI | ERTMS Level 0 and SB mode are displayed on the DMI. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Main window is displayed. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The Level symbol "L0" is displayed | O | DMI | The ERTMS/ETCS level status is valid | N/A | N/A | - |
| 3 | N/A | N/A | The driver presses "Level" | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | No Level symbol is displayed in C8 | O | DMI | The ERTMS/ETCS level status is invalid | N/A | N/A | - |
| 5 | N/A | N/A | The invalid ERTMS/ETCS level stored on-board is presented to the driver | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver confirms "Level" | I | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | UNCHANGED | - |
| ERTMS/ETCS level | 0 | L0 |
| Train Running Number | UNCHANGED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 50

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Following D2, if the stored position is "unknown", the status of the ERTMS/ETCS Level and of the RBC ID/Phone Number information is invalidated. |  | 50 |
| Applicable Mode/Level Combinations | L0: NP LNTC: NP L1: NP L2: NP L3: NP |  |  |
| Target of Test | The target of this test case is to check that, following D2, if the stored position is "unknown", the status of the ERTMS/ETCS Level and of the RBC ID/Phone Number information is invalidated. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.3 - Following D2: stored position is "invalid" or "unknown" |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | In NP mode, the status of the stored information is: - ERTMS/ETCS Level: Invalid - Train Position : Invalid or Unknown - RBC ID/Phone Number: Invalid or Unknown Once the ETCS on-board equipment is powered, the SoM path S0-S1-D2-S2-S3-A31-D31-D32-A34-D33-D22-A23-A24 is followed. As a result status of the stored information becomes: - ERTMS/ETCS Level: Valid - Train Position : Unknown - RBC ID/Phone Number: Valid Then the desk is closed and the communication session is terminated. Then the desk is opened again and the SoM path S0-S1-D2 is followed. Following D2, both the ERTMS/ETCS Level and the RBC ID/Phone Number information are invalidated. This is checked in this way: - ERTMS/ETCS Level: There is no DMI object shown in C8, but the invalid level data is presented to the driver * Level is invalid or unknown because of ERTMS_015560-8.2.3.2.3: If the current ERTMS/ETCS level is unknown or invalid, no DMI object shall be displayed in area C8. * Level is invalid or valid because because of ERTMS_015560-11.7.1.4: If the status is “invalid” or “valid”, the value stored on-board shall be presented to the driver when entering the corresponding data entry window. * As a consequence, the Level is invalid - RBC ID/Phone Number: The button "Contact last RBC" is enabled (see ERTMS_015560v300-Table37) |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | NP | NP |
| Train Position | INVALID / UNKNOWN | - |
| ERTMS/ETCS level | INVALID | L0 / LNTC / L1 / L2 / L3 |
| RBC ID/Phone Number | INVALID / UNKNOWN | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | NP | The ETCS on-board equipment is powered | - | - | Use_FT4060300.10 | N/A | SB | - |
| 2 | N/A | SB | The Driver opens desk | - | - | Use_FT5040300.24 | N/A | SB | - |
| 3 | N/A | SB | The driver enters a driver ID | - | - | Use_FT5040300.7 | N/A | SB | - |
| 4 | N/A | SB | The driver selects "Level 2/3" and enters "RBC Data" | I | DMI | The status of the ERTMS/ETCS Level is set to valid. The status of the RBC ID/Phone Number is set to valid. | N/A | SB | - |
| 5 | N/A | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<36¦37>) is recorded | O | JRU | The driver selects Level | L2 / L3 | SB | - |
| 6 | L2 / L3 | SB | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded | O | JRU | The driver enters "RBC Data" | L2 / L3 | SB | - |
| 7 | L2 / L3 | SB | The driver revalidates the RBC-ID/phone if invalid | - | - | Use_FT5040300.39 “If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid”.” | L2 / L3 | SB | - |
| 8 | L2 / L3 | SB | Radio Communication session is established | - | - | Use_FT3050300.23 | L2 / L3 | SB | - |
| 9 | L2 / L3 | SB | SA-DATA.Indication with Euroradio Message "Train Accepted" (NID_MESSAGE=41) is received | I | RTM | - | L2 / L3 | SB | - |
| 10 | L2 / L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=41) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 11 | L2 / L3 | SB | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded | O | JRU | If the status of the Train Position is invalid, it is set to unknown | L2 / L3 | SB | - |
| 12 | L2 / L3 | SB | The driver closes the desk | - | - | Use_FT3050500.10 | L2 / L3 | SB | - |
| 13 | L2 / L3 | SB | Cab active | I | TIU | - | L2 / L3 | SB | - |
| 14 | L2 / L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_A_STATUS=1) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 15 | L2 / L3 | SB | The driver confirms the "Driver ID" | I | DMI | S1-D2 | L2 / L3 | SB | - |
| 16 | L2 / L3 | SB | The Level symbol "Level <L2/L3>" is removed | O | DMI | The status of the ERTMS/ETCS Level is set to invalid (Following D2, if the stored position is unknown, the status of the ERTMS/ETCS Level and of the RBC ID/Phone Number is set to invalid) | L2 / L3 | SB | - |
| 17 | L2 / L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit4=0&Bit5=0>) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 18 | L2 / L3 | SB | The ERTMS/ETCS level window is displayed. | O | DMI | S2 The status of the ERTMS/ETCS Level is invalid. Thus the invalid value stored onboard is presented to the driver. | L2 / L3 | SB | - |
| 19 | L2 / L3 | SB | The driver selects "Level <2/3>" | I | DMI | The status of the ERTMS/ETCS Level is set to valid. | L2 / L3 | SB | - |
| 20 | L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<36¦37>) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 21 | L2 / L3 | SB | The Radio data window is displayed. | O | DMI | S3 The button "Contact last RBC" is enabled because the RBC ID/Phone Number is invalid. If the button "Contact last RBC" is not enabled, it must be checked in the test bench that at least one mobile terminal is registered to a radio network to ensure the error is due the not invalid RBC ID/Phone Number. | L2 / L3 | SB | - |

| Step 9: Radio Message 41 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 41 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | UNKNOWN | - |
| ERTMS/ETCS level | VALID | L2 / L3 |
| RBC ID/Phone Number | INVALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

## Test case 51

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Driver selects “Shunting”, train is at standstill (Transition [5]) and the system switches to SH mode. |  | 51 |
| Applicable Mode/Level Combinations | L0: SB L1: SB LNTC: SB |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SH mode after the driver selects “Shunting” and train is at standstill In case of selection of SH mode by the driver, the switch to shunting is always accepted by the on – board equipment The on-board equipment deletes stored position data. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.8.1.6 |  |
|  |  | Subset-026-4.6.2 row 4 column 2 |  |
|  |  | Subset-026-4.6.3 Condition Id [5] |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S10 a) |  |
|  |  | Subset-026-5.4.3.2 S10 d) |  |
|  |  | Subset-026-5.4.3.3 - Following E10, E12, E30, E31, E32, on-board deletes stored position data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SH mode symbol is DISPLAYED Check on the JRU that: The Driver selection is RECORDED. The transition to SH mode is RECORDED. The stored position data is deleted. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | INVALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The option for selection of SH is AVAILABLE. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | O | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | SB | The driver presses “Shunting” button. | I | DMI | - | L0 / LNTC / L1 | SB | - |
| 2 | L0 / LNTC / L1 | SB | Driver selection is RECORDED. | O | JRU | - | L0 / LNTC / L1 | SB | - |
| 3 | L0 / LNTC / L1 | SB | The SH symbol, MO01 is DISPLAYED. | O | DMI | - | L0 / LNTC / L1 | SH | - |
| 4 | L0 / LNTC / L1 | SH | The displayed symbol for SH mode is RECORDED | O | JRU | - | L0 / LNTC / L1 | SH | - |
| 5 | L0 / LNTC / L1 | SH | Any Message ( NID_MESSAGE_JRU=all; NID_LRBG= 16777215) is recorded. | O | JRU | - | L0 / LNTC / L1 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Train Position | UNKNOWN | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Indication of SH mode, symbol MO01 is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 52

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Driver selects “Shunting” (train is at standstill in L2/L3) and the shunting request is refused by the RBC. |  | 52 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: If the driver selects SH (E12), the process continues in the same way as the procedure “Shunting initiated by the driver”. If, in level 2 or 3, the RBC rejects the request for Shunting (E13), the process goes back to S10. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S10 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: If the driver selects SH (E12), the process continues in the same way as the procedure “Shunting initiated by the driver”. If, in level 2 or 3, the RBC rejects the request for Shunting (E13), the process goes back to S10. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The option for selection of SH is AVAILABLE. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | O | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | Use_FT5060200.4 | - | - | The driver selects shunting. The shunting request is refused by the RBC. | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | The Main window is displayed. | O | DMI | As the RBC rejects the request for Shunting, the process goes back to S10 state. | L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Main window is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 53

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is UNKNOWN. The driver selects a level. |  | 53 |
| Applicable Mode/Level Combinations | L0: SB LNTC : SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that, within the scope of the Start of Mission procedure in SB mode, when the status of the ERTMS/ETCS Level data is UNKNOWN, the on-board equipment : Requests the driver selects a level. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.2.1 |  |
|  |  | Subset-026-4.7.2 table - Input information ERTMS/ETCS level |  |
|  |  | Subset-026-5.4.3.2 S2 |  |
|  |  | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.3 - Following S2: driver has entered level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver selects a Level Check on the JRU that : Driver‘s action is recorded. The ERTMS/ETCS Level data selected by the driver is recorded. |
| Constraints | To respect the UNKNOWN status of the ERTMS/ETCS Level information requested as starting condition, the on-board supplier must provide a specific procedure to erase the non-volatile memory. Because of this specific procedure, this test case must be isolated in a single test sequence dedicated to its coverage. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | UNKNOWN / INVALID / VALID | - |
| ERTMS/ETCS level | UNKNOWN | - |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | UNKNOWN / INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | - |
| A list of all available ERTMS/ETCS Level data is displayed to the driver. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | SB | The driver selects “Level <L0/LNTC/L1/L2/L3>” | I | DMI | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SB | The Level symbol "Level < L0/LNTC/L1/L2/L3>" is displayed | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 VALID |
| Train Running Number | INVALID | Unchanged |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk open | I | TIU | Unchanged |
| The ERTMS/ETCS Level selected by the driver is displayed. | O | DMI | The On-Board equipment requests the driver either : -L2/L3: to enter the RBC ID & RBC Phone Number OR -L0/LNTC/L1: to select “Train Data Entry“. |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |

## Test case 54

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The driver closes the desk during a SoM procedure after the safe radio connection is set up. |  | 54 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: - within the procedure of start of mission in SB mode, the driver closes the desk after the safe radio connection is set up and between the emission of the message “initiation of comm. Session” and the reception of the message “system version”; - the on-board abort the process of establishing the communication session. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 a) |  |
|  |  | Subset-026-5.4.3.2 A31 |  |
|  |  | Subset-026-5.4.3.2 D31 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM/JRU that Radio communication session is opened (Establishment initiated by on-board, message sent to RBC “Initiation of a communication session”). Check on the TIU/JRU that the CAB is not active Check on RTM that the safe radio connection is released |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Cab not active | I | TIU | The driver closes the desk between the emission of the message “initiation of comm. Session” and the reception of the message “system version” | N/A | N/A | - |
| 6 | N/A | N/A | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |

| Step 3: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | Q_STATUS = 1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The desk is closed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 55

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | The driver closes the desk during a SoM procedure before the safe radio connection is set up. |  | 55 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: - within the procedure of start of mission in SB mode, the driver closes the desk before the safe radio connection is set up; - the on-board abort the process of establishing the communication session. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 a) |  |
|  |  | Subset-026-5.4.3.2 A31 |  |
|  |  | Subset-026-5.4.3.2 D31 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that the on-board requests the set-up of a safe radio connection Check on the TIU/JRU that the CAB is not active Check on RTM that the safe radio connection is released |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 |
| Driver ID | VALID | A valid Driver ID is available on-board |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | Cab not active | I | TIU | The driver closes the desk before the safe radio connection is set up | N/A | N/A | - |
| 3 | N/A | N/A | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | The on-board has aborted the process of establishing a communication session | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | Q_STATUS = 1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The desk is closed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 56

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.3: Table of requirements for “Start of Mission” procedure |  | 5040300 |
| Test Case of Feature | During the SoM procedure, if the stored train position and level are valid (for example, when closing the desk and making the transition SR-SB) and if level is 2 or 3, and if no mobile terminal is registered to a radio network (D7), the ERTMS/ETCS on-board equipment shall wait at least one Mobile Terminal is registered to a Radio Network (E6); in such case the process shall go to A31. |  | 56 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that - when the stored train position and level are valid within the scope of the Start of Mission procedure in SB in L2 or L3, - if no mobile terminal is registered to a radio network (D7), - the ERTMS/ETCS on-board equipment shall wait at least one Mobile Terminal is registered to a Radio Network (E6) before the "Time from the latest Radio Network registration order to a Mobile Terminal after which the registration is considered as failed.", in such case the process shall go to A31. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.3.2 D2 |  |
|  |  | Subset-026-5.4.3.2 D3 |  |
|  |  | Subset-026-5.4.3.2 D7 |  |
|  |  | Subset-026-5.4.3.2 S4 |  |
|  |  | Subset-026-5.4.3.2 S4 a) |  |
|  |  | Subset-026-5.4.3.2 A31 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that after entered the driver ID the ERTMS/ETCS on-board equipment waits at least one Mobile Terminal is registered to a Radio Network (E6) and the on-board goes to A31. |
| Constraints | No Mobile Terminal is registered to a Radio Network. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 VALID | L2 / L3 There is a valid ERTMS/ETCS Level data stored on-board. |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT SAFE CONNECTION SET-UP | O | RTM | - |
| Desk open | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is at standstill (V_TRAIN=0) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Driver sets Driver ID | - | - | Use_FT5040300.7-9 | N/A | N/A | - |
| 2 | N/A | N/A | The Main window is displayed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit42=1>) is recorded | O | DMI | The Main window is presented to the driver with all buttons ‘disabled’. The symbol ST05 is displayed to show that the onboard still awaits the registration to the Radio Network. | N/A | N/A | - |
| 4 | N/A | N/A | SA-REGISTRATION.Indication containing <NID_MN=654321h> is received | I | RTM | Before the "Time from the latest Radio Network registration order to a Mobile Terminal after which the registration is considered as failed." | N/A | N/A | - |
| 5 | N/A | N/A | On-board contacts RBC and reports “valid position” | - | - | Use_FT5040300.22 | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 UNCHANGED | SB |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| Desk open | I | TIU | Unchanged |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN | I | INT | Train is still at standstill |