| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5080300 (5.8.3: Once the “Override” procedure has been triggered) Total: 8 Test case(s) |  |
| REF: | Subset-076-5-2-5080300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | ALL | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, TCs renumbered | MULTITEL |
| Version 2.9.2 24.10.2013 | All | Modifications requested by Review001_Subset076-5-2-5080300_v291 | MULTITEL |
| Version 2.9.3 15.11.2013 | All | Response to Review002_Subset-076-5-2-5080300_v291 | MULTITEL |
| Version 2.9.4 26.11.2013 | TC6 | Response to issue 46 from Review004_Subset-076-5-2-5080300_v291 | MULTITEL |
| Version 2.9.5 09.01.2014 | TC1 | Modifications requested by Review001_Subset-076-5-2-5080300_v294 | MULTITEL |
| Version 2.9.6 20.01.2014 | TC1 | Modifications requested by Review001_Subset-076-5-2-5080300_v295 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.3: Once the “Override” procedure has been triggered |  | 5080300 |
| Test Case of Feature | As long as the Override function is active, new SR distance information received from EUROLOOP shall be rejected. |  | 1 |
| Applicable Mode/Level Combinations | L1: SR |  |  |
| Target of Test | The target of this test is to check that: As long as the Override function is active, new SR distance information received from EUROLOOP shall be rejected. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.8.3.7 |  |
|  |  | Subset-026-5.8.3.8 |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-7.4.2.3.1 Packet Number 13: Staff Responsible distance information from loop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The target of the Test is to check that: when the Override function is active, the reception of a new value for D_SR from EUROLOOP is REJECTED. |
| Constraints | Override function is active. It has not been activated by an STM. Nothing happens during the TC that activates braking. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | NOT STORED | No national values are received or available on-board. Thus, for each National Value, the corresponding Default Value (Subset-026-A.3.2) will be used. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Override function is active | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | SR | Override status is displayed. | O | DMI | The Override status is already activated The starting point for D_NVOVTRP is the position of the train when the Overrided was selected. | L1 | SR | - |
| 2 | L1 | SR | Packet 13:Staff Responsible distance information from loop, Including D_SR & a correct value for NID_C is RECEIVED. | I | LTM | D_SR should end before D_NVOVTRP. The starting point for D_SR is the refence balise mentioned in packet 13 (second NID_BG). | L1 | SR | - |
| 3 | L1 | SR | Packet 13:Staff Responsible distance information from loop, Including D_SR & a correct value for NID_C is RECORDED | O | JRU | - | L1 | SR | - |
| 4 | L1 | SR | No brake application is displayed | O | DMI | The train has run during a distance such as D_SR is exceeded and D_NVOVTRP is not. The new value of D_SR is REJECTED (the brakes aren’t applied). | L1 | SR | - |

| Step 2: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Euroloop |
| NID_C | 10 | FINITE VALUE | - |
| NID_LOOP | 14 | FINITE VALUE | The same as in recorded Packet 134 (EOLM) |
| NID_PACKET | 8 | 13 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Main signal balise group. |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Reference balise |
| D_SR | 15 | FINITE VALUE | D_SR should end before D_NVOVTRP |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY (k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | Reference balise. |
| D_SR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 UNCHANGED | SR |
| ERTMS/ETCS level | 2 UNCHANGED | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.3: Once the “Override” procedure has been triggered |  | 5080300 |
| Test Case of Feature | The MRSP shall include the Override function related Speed Restriction (see 3.11.10) as long as the Override function is active |  | 2 |
| Applicable Mode/Level Combinations | L0: UN, SH L1: SR, SH L2: SR, SH L3: SR, SH |  |  |
| Target of Test | The target of the test is to check that: When Override function is active, the MRSP includes the Override function related Speed Restriction (V_NVSUPOVTRP). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-4.5.2.1 table - Override related speed restriction |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-3.11.10.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The driver selects "Override EOA" - The "Override status" is DISPLAYED - Train speed is DISPLAYED and > V_NVSUPOVTRP - Service Brake Indication is DISPLAYED Check on the TIU that: - The Service Brake is APPLIED - After the Override procedure ends and when the train overpasses the speed V_NVSUPOVTRP (with V_TRAIN < MRSP), the Service Brake shall not be applied. Check on the JRU that: - The Selection and Display of "Override EOA" is RECORDED - The Service Brake intervention is RECORDED. |
| Constraints | The Most Restrictive Speed Profile is V_NVSUPOVTRP. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 3 / 4 | SR / SH / UN |
| National Values | STORED | V_NVSUPOVTRP different from Default value 6 = 30 km/h |
| Train Data | STORED | Validated train data is STORED (except in SH) |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train Speed is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Override EOA" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The train runs and overpasses the permitted speed limit when the override function is active. Current speed is DISPLAYED. | O | DMI | Train speed > V_NVSUPOVTRP | N/A | N/A | - |
| 6 | N/A | N/A | Service Brake is APPLIED. | O | TIU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Service Brake intervention is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The Service Brake intervention is RECORDED. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train slows under V_NVSUPOVTRP | O | DMI | Train speed < V_NVSUPOVTRP | N/A | N/A | - |
| 10 | N/A | N/A | Service Brake is NOT APPLIED. | O | TIU | - | N/A | N/A | - |
| 11 | N/A | N/A | The Override status finishes. | O | DMI | The override status finishes because the D_NVOVTRP is passed or T_NVOVTRP expires. | N/A | N/A | - |
| 12 | N/A | N/A | The end of OVERRIDE is RECORDED | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Train accelerates: MRSP > V_TRAIN > V_NVSUPOVTRP. | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | Service Brake is NOT APPLIED. | O | TIU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 3 / 4 | SR / SH / UN |
| National Values | STORED | - |
| Train Data | UNCHANGED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.3: Once the “Override” procedure has been triggered |  | 5080300 |
| Test Case of Feature | Train is at standstill. The driver modifies the value of given distance to run in SR and the maximum SR speed when Override function is active. The on-board equipment takes into account the new distance value. |  | 3 |
| Applicable Mode/Level Combinations | L1 : SR L2 : SR L3 : SR |  |  |
| Target of Test | The target of this test is to check that : The driver is allowed to modify the value of the SR mode speed limit (V_NVSTFF) and of the distance to run in SR mode (D_NVSTFF) at standstill The on-board accepts the new values The train can restart and continue in SR |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.11.1.5 |  |
|  |  | Subset-026-4.4.11.1.5.2 |  |
|  |  | Subset-026-4.4.11.1.6.3 |  |
|  |  | Subset-026-4.7.2 table - Input information SR mode speed limit and distance |  |
|  |  | Subset-026-4.7.2 table - Output information Target distance |  |
|  |  | Subset-026-A.3.11 Distance to run in SR mode |  |
|  |  | Subset-026-A.3.11 Maximum SR speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: the distance and speed for SR can be entered the new values are presented when requested the distance is extended Check on the JRU that: the distance and speed for SR are recorded |
| Constraints | The value for D_NVSTFF should be different from the default value (infinite) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | D_NVSTFF different from default value |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SR Indication is DISPLAYED. The "Override status" is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN=0 | I | INT | Train is at standstill |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SR | The "Override status" is DISPLAYED | O | DMI | - | L1/L2/L3 | SR | - |
| 2 | L1/L2/L3 | SR | Train is at standstill (V_TRAIN=0; TIME= 20) | I | INT | Before reaching the end of the national SR distance the train reaches standstill | L1/L2/L3 | SR | - |
| 3 | L1/L2/L3 | SR | The driver modifies "SR speed/distance" | I | DMI | The driver tries to select the entry of distance (>100000m) | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | The echo text "++++" in red is displayed | O | DMI | - | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | The driver modifies "SR speed/distance" | I | DMI | The driver enters a Distance value in the range [0m-100000m] | L1/L2/L3 | SR | - |
| 6 | L1/L2/L3 | SR | The echo text "++++" in red is removed | O | DMI | - | L1/L2/L3 | SR | - |
| 7 | L1/L2/L3 | SR | The driver modifies "SR speed/distance" | I | DMI | The driver tries to select the entry of speed for SR (> 600km/h). | L1/L2/L3 | SR | - |
| 8 | L1/L2/L3 | SR | The echo text "++++" in red is displayed | O | DMI | - | L1/L2/L3 | SR | - |
| 9 | L1/L2/L3 | SR | The driver modifies "SR speed/distance" | I | DMI | The driver enters a Maximum SR speed value in the range [0km/h-600km/h] with a resolution of 5 km/h. | L1/L2/L3 | SR | - |
| 10 | L1/L2/L3 | SR | The echo text "++++" in red is removed | O | DMI | - | L1/L2/L3 | SR | - |
| 11 | L1/L2/L3 | SR | The driver confirms "SR speed/distance" | I | DMI | - | L1/L2/L3 | SR | - |
| 12 | L1/L2/L3 | SR | SR SPEED/DISTANCE ENTERED BY THE DRIVER (NID_MESSAGE_JRU=25) is recorded | O | JRU | - | L1/L2/L3 | SR | - |
| 13 | L1/L2/L3 | SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | L1/L2/L3 | SR | - |
| 14 | L1/L2/L3 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=27) is recorded | O | JRU | - | L1/L2/L3 | SR | - |
| 15 | L1/L2/L3 | SR | The permitted speed is updated according new SR speed | O | DMI | - | L1/L2/L3 | SR | - |
| 16 | L1/L2/L3 | SR | The new target distance is AVAILABLE | O | DMI | - | L1/L2/L3 | SR | - |
| 17 | L1/L2/L3 | SR | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | - | L1/L2/L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SR Indication is DISPLAYED. New target distance is AVAILABLE. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board Equipment |  |  |
| Tested Feature | 5.8.3: Once the “Override” procedure has been triggered |  | 5080300 |
| Test Case of Feature | The train is in SR mode and the driver selects Override EOA. The SR distance and speed defined by the National values (D_NVOVTRP and V_NVSUPOVTRP) substitute the previous values given by the trackside (V_NVSTFF, D_NVSTFF). When the Override status finishes, the driver checks that the Permitted Speed (PS) and Target Distance (TD) change to be in line with the default values for V_NVSTFF and D_NVSTFF. |  | 4 |
| Applicable Mode/Level Combinations | L1/L2/L3: SR |  |  |
| Target of Test | The target of this test is to check that: The onboard uses a Permitted Speed (PS) and a Target Distance (TD) related to National Values received from trackside: V_NVSTFF and D_NVSTFF. After overriding the EOA, the onboard deletes the values of V_NVSTFF and D_NVSTFF previously entered by driver or given by trackside and uses the PS and TD related to V_NVSUPOVTRP and D_NVOVTRP. Once Override procedure is finished, the onboard uses a PS and a TD related to the default values of V_NVSTFF and D_NVSTFF. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-4.4.11.1.6.5 |  |
|  |  | Subset-026-4.4.11.1.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-5.8.3.7 |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The onboard uses a PS and a TD related to National Values received from trackside: V_NVSTFF and D_NVSTFF. The driver modifies "SR speed/distance" when the train is at standstill and, after it restart, selects Override. After overriding the EOA, the onboard uses the PS and TD related to V_NVSUPOVTRP and D_NVOVTRP. Once Override procedure is finished, the onboard uses a PS and a TD related to the default values of V_NVSTFF and D_NVSTFF. |
| Constraints | In this TC, the MRSP must not be influenced by any speed (e.g. V_TSR) other than V_NVSTFF or V_NVSUPOVTRP. There must be a difference between the values of Speed and Distance supervised by the train and related to: - V_NVSTFF and D_NVSTFF given by trackside - V_NVSUPOVTRP and D_NVOVTRP given by the trackside - the default values for V_NVSTFF and D_NVSTFF |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Train Data | STORED | Validated train data is STORED |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SR | The train receives and accepts National Values: V_NVSTFF = V_NVSTFF_1 V_NVSUPOVTRP = V_NVSUPOVTRP_1 D_NVSTFF = D_NVSTFF_1 D_NVOVTRP = D_NVOVTRP_1 | - | - | Use_FT4080426.3: On-board Equipment accepts NATIONAL VALUES transmitted by RBC OR Use_FT4080426.4: On-board Equipment accepts NATIONAL VALUES transmitted by balise group | L1/L2/L3 | SR | - |
| 2 | L1/L2/L3 | SR | The driver requests to see the PS and TD. | I | DMI | If not yet available | L1/L2/L3 | SR | - |
| 3 | L1/L2/L3 | SR | The current PS (V_NVSTFF_1) and TD (related to D_NVSTFF_1) are displayed | O | DMI | The starting point of D_NVSTFF_1 refers to the estimated front end when SR mode was entered | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | Train is at standstill (V_TRAIN=0) | I | INT | - | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | The driver modifies "SR speed/distance" | I | DMI | - | L1/L2/L3 | SR | - |
| 6 | L1/L2/L3 | SR | SR SPEED/DISTANCE ENTERED BY THE DRIVER (NID_MESSAGE_JRU=25) is recorded | O | JRU | - | L1/L2/L3 | SR | - |
| 7 | L1/L2/L3 | SR | Train is moving (V_TRAIN<V_NVALLOWOVTRP) | I | INT | - | L1/L2/L3 | SR | - |
| 8 | L1/L2/L3 | SR | The driver selects “Override Request” before reaching the end of the SR area. | I | DMI | - | L1/L2/L3 | SR | - |
| 9 | L1/L2/L3 | SR | The driver’s action is recorded. | O | JRU | - | L1/L2/L3 | SR | - |
| 10 | L1/L2/L3 | SR | The Override status is displayed. | O | DMI | - | L1/L2/L3 | SR | - |
| 11 | L1/L2/L3 | SR | The new PS (V_NVSUPOVTRP_1) and TD (related to D_NVOVTRP_1) are displayed | O | DMI | The starting point of D_NVOVTRP_1 refers to the estimated front end when Override was activated | L1/L2/L3 | SR | - |
| 12 | L1/L2/L3 | SR | The Override status finishes. | O | DMI | The override status finishes because the D_NVOVTRP is passed or T_NVOVTRP expires. | L1/L2/L3 | SR | - |
| 13 | L1/L2/L3 | SR | The PS and TD related to the default values for V_NVSTFF_1 and D_NVSTFF_1 are DISPLAYED | O | DMI | - | L1/L2/L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SR mode is DISPLAYED. Override status is NOT DISPLAYED. The PS and TD related to the default values for V_NVSTFF and D_NVSTFF are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.3: Once the “Override” procedure has been triggered |  | 5080300 |
| Test Case of Feature | In LNTC, when the Override function is active, the MRSP shall NOT include the Override function related Speed Restriction (see 3.11.10). |  | 5 |
| Applicable Mode/Level Combinations | LNTC: SH, SN |  |  |
| Target of Test | The target of the test is to check that: when the Override function is active, service brake shall NOT be applied when speed is higher than V_NVSUPOVTRP. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Service Brake Indication is NOT DISPLAYED. Check on the TIU that : When the Override procedure ends, the train overpasses the speed V_NVSUPOVTRP and the Service Brake shall not be applied. |
| Constraints | The Override function has been activated while being in level NTC and the "Override status" is DISPLAYED. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 13 | SH / SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train Speed is DISPLAYED. The "Override status" is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SH/SN | The “Override status” is DISPLAYED. | O | DMI | - | LNTC | SH/SN | - |
| 2 | LNTC | SH/SN | The train runs and overpasses the permitted speed limit. Current speed is DISPLAYED. | O | DMI | Train speed > V_NVSUPOVTRP | LNTC | SH/SN | - |
| 3 | LNTC | SH/SN | Service Brake is NOT APPLIED. | O | TIU | - | LNTC | SH/SN | - |
| 4 | LNTC | SH/SN | The Service Brake intervention is NOT DISPLAYED. | O | DMI | - | LNTC | SH/SN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 13 UNCHANGED | SH / SN |
| ERTMS/ETCS level | 1 UNCHANGED | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.3: Once the “Override” procedure has been triggered |  | 5080300 |
| Test Case of Feature | When “Override” is selected and “Override” is already active, the supervision of the “distance for train trip suppression when Override function is triggered” (national value D_NVOVTRP) is re-started. |  | 6 |
| Applicable Mode/Level Combinations | L1: SR L2: SR L3: SR |  |  |
| Target of Test | The target of this test is to check that: When “Override” is selected and Override is already active, the supervision of the distance (see 5.8.4.1 b)) for train trip suppression shall be re-started. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.8.3.7 |  |
|  |  | Subset-026-5.8.3.9 |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: - The “Override” is selected - After the train has run a distance D>0, the driver selects “Override” again - The “Override” ends after a distance = D+ D_NVOVTRP, this checks that the supervision of the distance D_NVOVTRP has been re-started in D. Check on JRU that: - “The selection of OVERRIDE” is RECORDED - “The end of OVERRIDE” is RECORDED |
| Constraints | Override function is active. It has not been activated by an STM. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | NOT STORED | No national values are received or available on-board. Thus, for each National Value, the corresponding Default Value (Subset-026-A.3.2) will be used.” |
| ERTMS/ETCS level | / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SR | Override status is displayed. | O | DMI | The Override status is already activated | L1/L2/L3 | SR | - |
| 2 | L1/L2/L3 | SR | The driver selects OVERRIDE | I | DMI | At distance D. Then, the train runs during some distance D > 0 & D < D_NVOVTRP | L1/L2/L3 | SR | - |
| 3 | L1/L2/L3 | SR | “The selection of OVERRIDE” is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | The Override status is DESACTIVATED | O | DMI | The train has run in override during D + D_NVOVTRP The Override procedure ends | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | The end of OVERRIDE is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 UNCHANGED | SR |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Override status is DESACTIVATED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.3: Once the “Override” procedure has been triggered |  | 5080300 |
| Test Case of Feature | When “Override” is selected and Override is already active, the supervision of the time (see 5.8.4.1 a)) for train trip suppression shall be re-started. |  | 7 |
| Applicable Mode/Level Combinations | L1: SR L2: SR L3: SR |  |  |
| Target of Test | The target of this test is to check that: When “Override” is selected and “Override” is already active, the supervision of the “max. time for train trip suppression when Override function is triggered” (national value T_NVOVTRP) is re-started. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.8.3.7 |  |
|  |  | Subset-026-5.8.3.9 |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: - The “Override” is selected - After the train has run during T>0, the driver selects “Override” again - The “Override” ends after running in “Override” during T+ T_NVOVTRP, this checks that the supervision of the max. time T_NVOVTRP has been re-started at T. Check on JRU that: - “The selection of OVERRIDE” is RECORDED - “The end of OVERRIDE” is RECORDED |
| Constraints | Override function is active. It has not been activated by an STM. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | NOT STORED | No national values are received or available on-board. Thus, for each National Value, the corresponding Default Value (Subset-026-A.3.2) will be used.” |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SR | Override status is displayed. | O | DMI | The Override status is already activated | L1/L2/L3 | SR | - |
| 2 | L1/L2/L3 | SR | The driver selects OVERRIDE | I | DMI | At time T. Then, the train runs during some time T > 0 & T < T_NVOVTRP | L1/L2/L3 | SR | - |
| 3 | L1/L2/L3 | SR | “The selection of OVERRIDE” is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | The Override status is DESACTIVATED | O | DMI | The train has run in override during T + T_NVOVTRP The Override procedure ends | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | The end of OVERRIDE is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 UNCHANGED | SR |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Override status is DISACTIVATED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.3: Once the “Override” procedure has been triggered |  | 5080300 |
| Test Case of Feature | The driver selects “override request”, the system does not change the mode and override status is displayed. After switching to Level LNTC, the override status display is removed. |  | 8 |
| Applicable Mode/Level Combinations | L1: SR L2: SR L3: SR |  |  |
| Target of Test | The target of this test is to check that:. The override status is indicated to the driver since the driver selects “override request”; The override status indication is removed when the train switches to Level LNTC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.7.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The override status is displayed when the driver selects “override request” The “override status” display is removed when the train switches to level LNTC Check on the JRU that: Selection of “override request“ is recorded The “override status” is displayed The train switches to level LNTC The “override status” display is removed |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 2/3/4 | L1/L2/L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SR | The driver selects “override request” | I | DMI | The “override request” is available only when the train speed is below V_NVALLOWOVTRP | L1/L2/L3 | SR | - |
| 2 | L1/L2/L3 | SR | The selection of “override request” is RECORDED | O | JRU | As soon as the override request is received | L1/L2/L3 | SR | - |
| 3 | L1/L2/L3 | SR | The “override status” is DISPLAYED. | O | DMI | - | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | USE_FT4060300.6 | - | - | ETCS level switches to Level NTC. Transition Condition Id [56]. | LNTC | SR | - |
| 6 | LNTC | SR | The “override status” indication is REMOVED | O | DMI | - | LNTC | SR | - |
| 7 | LNTC | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=0>) is recorded | O | JRU | - | LNTC | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The "override status" is not DISPLAYED. V_NVSUPOVTRP is not displayed as permitted speed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |