| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5180700 (5.18.7: Inhibition of a defined type of brake) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-5180700 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 21.08.2013 | All | Answer to Review002_Subset-076-5-2-5180700_v290 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.7: Inhibition of a defined type of brake |  | 5180700 |
| Test Case of Feature | Passing the area forbidden for Regenerative braking. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, TR, PT, NL L2 : FS, LS, OS, TR, PT, NL L3 : FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that the On-board Equipment switches off the permission for the Regenerative braking when the estimated front end is located into the area forbidden for the regenerative braking. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.1.2.1 |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Brakes control |  |
|  |  | Subset-026-5.18.7.3 |  |
|  |  | Subset-026-5.18.7.3.1 |  |
|  |  | Subset-026-5.18.7.3.1 a) |  |
|  |  | Subset-026-5.18.7.3.1 b) |  |
|  |  | Subset-026-5.18.7.3.2 |  |
|  |  | Subset-026-5.18.7.4 |  |
|  |  | Subset-026-5.18.7.4 a) |  |
|  |  | Subset-026-5.18.7.4 b) |  |
|  |  | Subset-026-5.18.7.5 |  |
|  |  | Subset-026-5.20.5.3 |  |
|  |  | Subset-026-5.20.5.4 |  |
|  |  | Subset-026-5.20.5.4 a) |  |
|  |  | Subset-026-5.20.5.4 b) |  |
|  |  | Subset-026-5.20.5.5 |  |
|  |  | Subset-026-5.20.5.6 |  |
|  |  | Subset-026-A.3.1 - Time between minimum safe rear end of the train leaving a track condition area and on-board deleting the applicable indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU, DMI, TIU the distance at which the regenerative braking forbidden area starts and ends: -on the JRU, in telegrams from balise group or in messages from RBC containing the track condition stored on JRU during the phase of preparation of the test for setting the initial conditions ; -on DMI , when the visualisation of track condition are available on operator request (according to a specific action to be found in the USER MANUAL of DMI supplier Subset-026-4.7.2.1.2 Output Information Track Condition excluding big metal masses) or because they have been sent by the on-board equipment Subset-026-3.12.1.5a) during the phase of preparation of the test and they are still available . -on TIU relevant output, during the phase of preparation of the test according to Subset-026-3.12.1.5b) In level 1 check on all available interfaces the distance travelled by the train from the reference balise group transmitting the track condition. In level 2, check on all available interfaces of On-board Equipment the distance travelled by the train since the LRBG (D_LRBG). Check when the "distance travelled" from the reference balise group / LRBG becomes > DTRACKCOND (entering in the area ) and when becomes > DTRACKCOND+L_TRACKCOND (going out from the area). Check on TIU relevant output if after the entry point of area forbidden for the regenerative braking On-board Equipment stops to permit regenerative braking Check on TIU and JRU that: The information related to track condition is provided to the ERTMS/ETCS external function |
| Constraints | The train is approaching the regenerative braking forbidden area In this test, when level 2 and level 3 are considered, the LRBGONB and LRBGRBC coincide. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/7/8/11/12 | FS/OS/TR/PT/NL/LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND FINITE VALUE L_TRACKCOND FINITE VALUE Length of track with regenerative braking forbidden M_TRACKCOND 0110 : stop regenerative braking in L_TRACKCOND D_LRBG FINITE VALUE Distance between LRBG and the estimated front end of the train. Q_DIR (of packet 68 ) (01) Nominal w.r.t. LRBG Q_DIRLRBG (1) Nominal Orientation of the train w.r.t the LRBG . Q_DIRTRAIN (1) Nominal Direction of the train movement |
| ERTMS/ETCS level | 2/3/4 | L1/ L 2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| REGENERATIVE BRAKE SWITCHED ON (PERMITTED) | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Inhibition of regenerative brake announcement” is DISPLAYED It is DISPLAYED if the related functionality is executed: Automatically manually (the driver is requested to act) | O | DMI | The max safe front end of the train reaches a location (Point C) in rear of the beginning of the regenerative brake inhibition area taking into account the time necessary for performing the required actions and the current train speed. Distance from the reference balise (DLRBG)<D_TRACKCOND | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit60=1¦Bit61=1>) is recorded | O | JRU | TC17 (automatic execution) or TC18 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the regenerative brake inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=3, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the regenerative brake inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=3, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START and D_MINSRE_TO_END are decreasing | N/A | N/A | - |
| 7 | N/A | N/A | “Inhibition of regenerative brake announcement” is REMOVED | O | DMI | The train reaches the max safe front end of the start location of “brake inhibition area” | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit61=0>) is recorded | O | JRU | TC18 If the related functionality is executed automatically, there is no removal of the symbol TC18, because it it symbol TC17 that has been displayed since the announcement | N/A | N/A | - |
| 9 | N/A | N/A | “Inhibition of regenerative brake” is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit60=1>) is recorded | O | JRU | TC17 If the related functionality is executed automatically, there is no new display of the symbol, because it has been displayed since the announcement | N/A | N/A | - |
| 11 | N/A | N/A | Regenerative braking switched off | I | TIU | The On-board Equipment enters in the area forbidden for regenerative braking | N/A | N/A | - |
| 12 | N/A | N/A | REGENERATIVE BRAKE STATUS (NID_MESSAGE_JRU=33; M_RB_STATUS=0) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the regenerative brake inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 14 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=3, D_MAXSFE_TO_START=-32768, D_MINSRE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START not relevant and D_MINSRE_TO_END decreased | N/A | N/A | - |
| 15 | N/A | N/A | “Inhibition of regenerative brake” is REMOVED | O | DMI | The train reaches with its min safe rear end the end location of brake inhibition area | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit60=0>) is recorded | O | JRU | TC17 | N/A | N/A | - |
| 17 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | Regenerative braking switched on | I | TIU | - | N/A | N/A | - |
| 19 | N/A | N/A | REGENERATIVE BRAKE STATUS (NID_MESSAGE_JRU=33; M_RB_STATUS=1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/7/8/11/12 UNCHANGED | FS/OS/TR/PT/NL/LS UNCHANGED |
| ERTMS/ETCS level | 2/3/4 | L1/ L 2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| REGENERATIVE BRAKING PERMISSION SWITCHED ON | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.7: Inhibition of a defined type of brake |  | 5180700 |
| Test Case of Feature | Passing the area forbidden for eddy currents braking for service brake. |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, TR, PT, NL L2 : FS, LS, OS, TR, PT, NL L3 : FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that the On-Board Equipment switches off the permission for the eddy currents braking for service brake when the estimated front end is into the area in which the eddy currents braking is forbidden. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.1.2.1 |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Brakes control |  |
|  |  | Subset-026-5.18.7.3 |  |
|  |  | Subset-026-5.18.7.3.1 a) |  |
|  |  | Subset-026-5.18.7.3.1 b) |  |
|  |  | Subset-026-5.18.7.3.2 |  |
|  |  | Subset-026-5.18.7.4 |  |
|  |  | Subset-026-5.18.7.5 |  |
|  |  | Subset-026-5.20.5.3 |  |
|  |  | Subset-026-5.20.5.4 |  |
|  |  | Subset-026-5.20.5.4 a) |  |
|  |  | Subset-026-5.20.5.4 b) |  |
|  |  | Subset-026-5.20.5.5 |  |
|  |  | Subset-026-5.20.5.6 |  |
|  |  | Subset-026-A.3.1 - Time between minimum safe rear end of the train leaving a track condition area and on-board deleting the applicable indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Operator requests the visualisation of the track condition on DMI (Available according to specific action to be found in the USER MANUAL of DMI supplier) . Check on the JRU and on DMI the distance at which the eddy currents braking for service brake forbidden area starts: on the JRU the telegrams or the messages containing the track condition and on DMI the visualisation of track condition on operator request. Check on all available interfaces of On-board Equipment the distance travelled by the train since the reference balise group or LRBG (D_LRBG). Check when the "distance travelled" from LRBG becomes > DTRACKCOND. Check on available interface of TIU provided by the supplier of On-board Equipment if after the entry point of area forbidden for the eddy currents braking On-board Equipment stops to permit eddy currents braking Check if after the exit from the area the On-board Equipment switches on the permission Check on TIU and JRU that: The information related to track condition is provided to the ERTMS/ETCS external function |
| Constraints | The train is approaching eddy currents braking for service brake forbidden area In this test, when level 2 and level 3 are considered, the LRBGONB and LRBGRBC coincide. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/7/8/11/12 | FS/OS/TR/PT/NL/LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND FINITE VALUE L_TRACKCOND FINITE VALUE Length of track with eddy currents braking forbidden M_TRACKCOND 0111 : stop eddy currents braking for service brake in L_TRACKCOND D_LRBG FINITE VALUE Distance between LRBG and the estimated front end of the train. Q_DIR (of packet 68 ) (01) Nominal w.r.t. LRBG Q_DIRLRBG (1) Nominal Orientation of the train w.r.t the LRBG. Q_DIRTRAIN (1) Nominal Direction of the train movement |
| ERTMS/ETCS level | 2/3/4 | L1/ L 2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EDDY CURRENTS BRAKE PERMITTED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Inhibition of eddy current brake announcement” is DISPLAYED It is DISPLAYED if the related functionality is executed: automatically manually (the driver is requested to act) | O | DMI | The max safe front end of the train reaches a location (Point C) in rear of the beginning of the eddy currents brake inhibition area taking into account the time necessary for performing the required actions and the current train speed. DLRBG<D_TRACKCOND | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit58=1¦Bit59=1>) is recorded | O | JRU | TC15 (automatic execution) or TC16 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the eddy currents brake inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=6, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the eddy currents brake inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=6, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START and D_MINSRE_TO_END are decreasing | N/A | N/A | - |
| 7 | N/A | N/A | “Inhibition of eddy current brake announcement” is REMOVED | O | DMI | The train reaches the max safe front end of the start location of “brake inhibition area” | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit59=0>) is recorded | O | JRU | TC16 If the related functionality is executed automatically, there is no removal of the symbol TC16, because it it symbol TC15 that has been displayed since the announcement | N/A | N/A | - |
| 9 | N/A | N/A | “Inhibition of eddy current brake” is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit58=1>) is recorded | O | JRU | TC15 If the related functionality is executed automatically, there is no new display of the symbol, because it has been displayed since the announcement | N/A | N/A | - |
| 11 | N/A | N/A | Eddy currents braking for service brake permission is switched off. | I | TIU | The On-board Equipment enters the area DLRBG>D_TRACKCOND | N/A | N/A | - |
| 12 | N/A | N/A | EDDY CURRENT BRAKE STATUS (NID_MESSAGE_JRU=35; M_ECB_STATUS=0) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the eddy currents brake inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 14 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=6, D_MAXSFE_TO_START=-32768, D_MINSRE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START not relevant and D_MINSRE_TO_END decreased | N/A | N/A | - |
| 15 | N/A | N/A | “Inhibition of eddy current brake” is REMOVED | O | DMI | The train reaches with its min safe rear end the end location of brake inhibition area | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit58=0>) is recorded | O | JRU | TC15 | N/A | N/A | - |
| 17 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | Eddy currents braking for service brake permission is switched on | I | TIU | DLRBG-L_TRAIN>D_TRACKCOND + L_TRACKCOND | N/A | N/A | - |
| 19 | N/A | N/A | EDDY CURRENT BRAKE STATUS (NID_MESSAGE_JRU=35; M_ECB_STATUS=1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/7/8/11/12 UNCHANGED | FS/OS/TR/PT/NL/LS |
| ERTMS/ETCS level | 2/3/4 | L1/ L 2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EDDY CURRENTS BRAKING PERMISSION SWITCHED ON | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.7: Inhibition of a defined type of brake |  | 5180700 |
| Test Case of Feature | Going into the area forbidden for magnetic shoes braking. |  | 3 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, TR, PT, NL L2 : FS, LS, OS, TR, PT, NL L3 : FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that the On-board Equipment switches off the permission for the magnetic shoes braking when the estimated front end passes the entry point of the area in which the magnetic shoe braking is forbidden. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Brakes control |  |
|  |  | Subset-026-5.18.7.3 |  |
|  |  | Subset-026-5.18.7.3.1 a) |  |
|  |  | Subset-026-5.18.7.3.1 b) |  |
|  |  | Subset-026-5.18.7.3.2 |  |
|  |  | Subset-026-5.18.7.4 |  |
|  |  | Subset-026-5.18.7.5 |  |
|  |  | Subset-026-5.20.5.3 |  |
|  |  | Subset-026-5.20.5.4 |  |
|  |  | Subset-026-5.20.5.4 a) |  |
|  |  | Subset-026-5.20.5.4 b) |  |
|  |  | Subset-026-5.20.5.5 |  |
|  |  | Subset-026-5.20.5.6 |  |
|  |  | Subset-026-A.3.1 - Time between minimum safe rear end of the train leaving a track condition area and on-board deleting the applicable indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU and on DMI the distance to which the magnetic shoes braking forbidden area starts: on the JRU the telegrams or the messages containing the track condition are recorded, and on DMI the visualisation of track condition is available on operator request. Check on all available interfaces of On-board Equipment the distance travelled by the train since the LRBG (D_LRBG). Check when the "distance travelled" from LRBG becomes > DTRACKCOND. Check on available interface provided by the supplier of On-board Equipment if , after the entry point of area forbidden for the magnetic shoes braking, On-board Equipment stops to permit magnetic shoes braking . Check when the "distance travelled" from LRBG becomes > [DTRACKCOND+L_TRACKCOND]. Check on available interface provided by the supplier of On-board Equipment if , after the exit from the area forbidden for the magnetic shoes braking, On-board Equipment permits magnetic shoes braking . Check on TIU and JRU that: The information related to track condition is provided to the ERTMS/ETCS external function |
| Constraints | The train is approaching magnetic shoes braking forbidden area In this test, when level 2 and level 3 are considered, the LRBGONB and LRBGRBC coincide. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/7/8/11/12 | FS/OS/TR/PT/NL/LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND FINITE VALUE L_TRACKCOND FINITE VALUE Length of track with magnetic shoes braking forbidden M_TRACKCOND 1000 : stop magnetic shoes braking in L_TRACKCOND D_LRBG FINITE VALUE Distance between LRBG and the estimated front end of the train. Q_DIR (of packet 68 ) (01) Nominal w.r.t. LRBG Q_DIRLRBG (1) Nominal Orientation of the train w.r.t the LRBG. Q_DIRTRAIN (1) Nominal Direction of the train movement |
| ERTMS/ETCS level | 2/3/4 | L1/ L 2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| MAGNETIC SHOES BRAKE PERMITTED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Inhibition of magnetic shoe brake announcement” is DISPLAYED It is DISPLAYED if the related functionality is executed: automatically manually (the driver is requested to act) | O | DMI | The max safe front end of the train reaches a location (Point C) in rear of the beginning of the magnetic shoes brake inhibition area taking into account the time necessary for performing the required actions and the current train speed. DLRBG<D_TRACKCOND | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit56=1¦Bit57=1>) is recorded | O | JRU | TC13 (automatic execution) or TC14 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the magnetic shoes inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=4, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the magnetic shoes inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=4, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START and D_MINSRE_TO_END are decreasing | N/A | N/A | - |
| 7 | N/A | N/A | “Inhibition of magnetic shoe brake announcement” is REMOVED | O | DMI | The train reaches the max safe front end of the start location of “brake inhibition area” | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit57=0>) is recorded | O | JRU | TC14 If the related functionality is executed automatically, there is no removal of the symbol TC14, because it it symbol TC13 that has been displayed since the announcement | N/A | N/A | - |
| 9 | N/A | N/A | “Inhibition of magnetic shoe brake” is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit56=1>) is recorded | O | JRU | TC13 If the related functionality is executed automatically, there is no new display of the symbol, because it has been displayed since the announcement | N/A | N/A | - |
| 11 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the magnetic shoes inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 12 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=4, D_MAXSFE_TO_START=-32768, D_MINSRE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START not relevant and D_MINSRE_TO_END decreased | N/A | N/A | - |
| 13 | N/A | N/A | Magnetic shoes braking permission is switched off. | I | TIU | The On-board Equipment enters in the area DLRBG>D_TRACKCOND | N/A | N/A | - |
| 14 | N/A | N/A | MAGNETIC SHOE BRAKE STATUS (NID_MESSAGE_JRU=34; M_MSB_STATUS=0) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | “Inhibition of magnetic shoe brake” is REMOVED | O | DMI | The train reaches with its min safe rear end the end location of brake inhibition area | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit56=0>) is recorded | O | JRU | TC13 | N/A | N/A | - |
| 17 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | Magnetic shoe braking permission is switched on. | I | TIU | - | N/A | N/A | - |
| 19 | N/A | N/A | MAGNETIC SHOE BRAKE STATUS (NID_MESSAGE_JRU=34; M_MSB_STATUS=1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/7/8/11/12 UNCHANGED | FS/OS/TR/PT/NL/LS |
| ERTMS/ETCS level | 2/3/4 | L1/ L 2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| MAGNETIC SHOES BRAKING PERMISSION SWITCHED ON | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.7: Inhibition of a defined type of brake |  | 5180700 |
| Test Case of Feature | Passing the area forbidden for Regenerative braking, deletion of the area is requested by trackside. |  | 4 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, TR, PT, NL L2 : FS, LS, OS, TR, PT, NL L3 : FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that the ERTMS/ETCS on-board equipment stop providing information related the area forbidden for Regenerative braking to the ERTMS/ETCS external function since the deletion of the area is requested by trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.7.3.2 c) |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Brakes control |  |
|  |  | Subset-026-5.18.7.3 |  |
|  |  | Subset-026-5.18.7.3.1 |  |
|  |  | Subset-026-5.18.7.3.1 a) |  |
|  |  | Subset-026-5.18.7.3.1 b) |  |
|  |  | Subset-026-5.20.1.2 |  |
|  |  | Subset-026-5.20.5.3 |  |
|  |  | Subset-026-5.20.5.4 |  |
|  |  | Subset-026-5.20.5.4 a) |  |
|  |  | Subset-026-5.20.5.4 b) |  |
|  |  | Subset-026-5.20.5.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU, DMI, TIU the distance at which the regenerative braking forbidden area starts and ends: -on the JRU, in telegrams from balise group or in messages from RBC containing the track condition stored on JRU during the phase of preparation of the test for setting the initial conditions ; -on DMI , when the visualisation of track condition are available on operator request (according to a specific action to be found in the USER MANUAL of DMI supplier Subset-026-4.7.2.1.2 Output Information Track Condition excluding big metal masses) or because they have been sent by the on-board equipment Subset-026-3.12.1.5a) during the phase of preparation of the test and they are still available . -on TIU relevant output, during the phase of preparation of the test according to Subset-026-3.12.1.5b) In level 1 check on all available interfaces the distance travelled by the train from the reference balise group transmitting the track condition. In level 2, check on all available interfaces of On-board Equipment the distance travelled by the train since the LRBG (D_LRBG). Check on TIU and JRU that: The information related to track condition is provided to the ERTMS/ETCS external function The track condition information is no more transmitted after the trackside request the deletion of the powerless section |
| Constraints | The train is approaching the regenerative braking forbidden area In this test, when level 2 and level 3 are considered, the LRBGONB and LRBGRBC coincide. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/7/8/11/12 | FS/OS/TR/PT/NL/LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND FINITE VALUE L_TRACKCOND FINITE VALUE Length of track with regenerative braking forbidden M_TRACKCOND 0110 : stop regenerative braking in L_TRACKCOND D_LRBG FINITE VALUE Distance between LRBG and the estimated front end of the train. Q_DIR (of packet 68 ) (01) Nominal w.r.t. LRBG Q_DIRLRBG (1) Nominal Orientation of the train w.r.t the LRBG . Q_DIRTRAIN (1) Nominal Direction of the train movement |
| ERTMS/ETCS level | 2/3/4 | L1/ L 2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| REGENERATIVE BRAKE SWITCHED ON (PERMITTED) | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Inhibition of regenerative brake announcement” is DISPLAYED It is DISPLAYED if the related functionality is executed: Automatically manually (the driver is requested to act) | O | DMI | The max safe front end of the train reaches a location (Point C) in rear of the beginning of the regenerative brake inhibition area taking into account the time necessary for performing the required actions and the current train speed. Distance from the reference balise (DLRBG)<D_TRACKCOND | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit60=1¦Bit61=1>) is recorded | O | JRU | TC17 (automatic execution) or TC18 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the regenerative brake inhibition area is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=3, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message containing packet 68 (Q_TRACKINIT = 1; D_TRACKINIT=0) is received | I | BTM | The information related to the regenerative brake inhibition area is deleted | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT Track Conditions transmitted | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1 |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/7/8/11/12 | FS/OS/TR/PT/NL/LS |
| ERTMS/ETCS level | 2/3/4 | L1/ L 2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| REGENERATIVE BRAKING PERMISSION SWITCHED ON | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |