| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3040200 (3.4.2: Balise Co-ordinate System) Total: 6 Test case(s) |  |
| REF: | Subset-076-5-2-3040200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 16.09.2013 | All | review | DLR |
| Version 2.9.2 05.12.2013 | TC 8 | This TC 8 has been created in order to test the requirement 3.4.2.4.1 | CEDEX |
| Version 2.9.3 12.12.2013 | TC 8 | review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.4.2: Balise Co-ordinate System |  | 3040200 |
| Test Case of Feature | On-board behaviour after passing Single Balise Groups |  | 1 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, TR, PT, NL, RV L3: FS, LS, OS, SR, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that : - After passing a Single BG, the on-board will send a position report based on two balise groups using as direction reference a move from the “previous LRBG” towards this single balise group |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.4.2.3.3.2 |  |
|  |  | Subset-026-7.4.3.2 Packet Number 1: Position Report based on two balise groups |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM that : • A Single BG is received. Check on the JRU that: • The Single BG is recorded Check on the RTM: • A position report based on two balise groups is sent with the directional information set to “nominal”. |
| Constraints | Radio link is established. The previous LRBG is known on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 / 14 | FS / OS / SR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 is received | I | BTM | Single Balise Group NID_BG = Bgx | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 1 is transmitted | O | RTM | The position refers to the front end of the respective engine with regards to the train orientation. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First |
| N_TOTAL | 3 | 0 | Single balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BGx |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 1 | Position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGx |
| NID_PRVLRBG | 24 | FINITE VALUE | Used as reference for all directional information in the packet: a move from PRVLRBG towards the LRBG defines the "nominal" direction. |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0/1 | Depends on the mode: for RV and PT shall be 0 (reverse) and 1 (nominal) for all other applicable modes. |
| Q_DLRBG | 2 | 1 | Nominal. Train front position according to reference direction. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 1 | Nominal. Actual running direction according to reference direction. |
| M_MODE | 4 | 0 / 1 / 2 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / TR / PT / NL / LS / RV |
| M_LEVEL | 4 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 / 14 | FS / OS / SR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.4.2: Balise Co-ordinate System |  | 3040200 |
| Test Case of Feature | On-board behavior after passing Single Balise Groups |  | 2 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, TR, PT, NL, RV L3: FS, LS, OS, SR, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that : - When the previous LRBG is unknown, after passing a Single BG, the on-board will send a position report based on two balise groups with the previous LRBG and all directional information set to Unknown. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.4.2.3.3.3 |  |
|  |  | Subset-026-7.4.3.2 Packet Number 1: Position Report based on two balise groups |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM that : • A Single BG is received. Check on the JRU that: • The Single BG is recorded Check on the RTM: • A position report based on two balise groups is sent with the “previous LRBG” and the directional information set to “Unknown”. |
| Constraints | Radio link is established. The previous LRBG is not known on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 / 14 | FS / OS / SR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| Train Position | UNKNOWN | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 is received | I | BTM | Single Balise Group NID_BG = BGx | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 1 is transmitted | O | RTM | A position report based on two balise groups is sent with the “previous LRBG” and the directional information set to “Unknown”. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First |
| N_TOTAL | 3 | 0 | Single Balise Group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGx" |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | "BGx" |
| NID_PRVLRBG | 24 | 16777215 | Unknown |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 2 | Unknown |
| Q_DLRBG | 2 | 2 | Unknown |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | Actual running direction according to reference direction. |
| M_MODE | 4 | 0 / 1 / 2 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / TR / PT / NL / LS / RV |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 / 14 | FS / OS / SR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.4.2: Balise Co-ordinate System |  | 3040200 |
| Test Case of Feature | On-board behaviour after passing Single Balise Groups |  | 3 |
| Applicable Mode/Level Combinations | L2: FS, LS, SR, OS, NL L3: FS, LS, SR, OS, NL |  |  |
| Target of Test | The target of this test case is to check that : - When a new single balise group (BG2), different from the current LRBG (BG1), becomes LRBG while the running direction of the train is opposite to the running direction when this current LRBG (BG1) was last passed, the “previous LRBG” and all directional information of the position report based on two balise groups shall be reported as “unknown”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.4.2.3.3.4 |  |
|  |  | Subset-026-7.4.3.2 Packet Number 1: Position Report based on two balise groups |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM that : • BG1 is received. • New Single BG2 is received after the train has changed its orientation. Check on the JRU that: • The BG1 and the Single BG2 are recorded Check on the RTM: • A position report based on two balise groups is sent after passing the new Single BG2, with the “previous LRBG” and the directional information set to “Unknown”. |
| Constraints | Radio link is established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS / OS / SR / LS / NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 is received | I | BTM | Single balise group a (BGa) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 6 | N/A | N/A | The train changes its orientation | - | - | Use_ FT4060300.8 (Desk is closed) AND Use_ FT3050500.5 (Communication session terminated) AND Use_ FT5120200.1 (Same engine used) AND Use_ FT5040500.14 (Start selected) AND Use_ FT4060300.48 (Change to FS mode) | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train starts moving in the direction opposite to the direction when BGa was last passed. | N/A | N/A | - |
| 8 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 is received | I | BTM | Single balise group b (BGb) | N/A | N/A | - |
| 9 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 1 is transmitted | O | RTM | A position report based on two balise groups is sent with the “previous LRBG” and the directional information set to “Unknown”. | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First balise |
| N_TOTAL | 3 | 0 | Single balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Single balise group a (BGa) |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 8: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First balise |
| N_TOTAL | 3 | 0 | Single balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Single balise group b (BGb) |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 1 | Position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGb |
| NID_PRVLRBG | 24 | 16777215 | Unknown |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 2 | Unknown |
| Q_DLRBG | 2 | 2 | Unknown |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 2 | Unknown |
| M_MODE | 4 | 0 / 1 / 2 /11 / 12 | FS/ LS/ OS/ SR/ NL |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS / OS / SR / LS / NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.4.2: Balise Co-ordinate System |  | 3040200 |
| Test Case of Feature | On-board behaviour after passing Single Balise Groups |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, NL L3: FS, LS, OS, SR, NL |  |  |
| Target of Test | The target of this test case is to check that : - When a single balise group, being the LRBG, is detected again, the LRBG and the “previous LRBG” of the position report based on two balise groups shall remain unchanged. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.4.2.3.3.5 |  |
|  |  | Subset-026-7.4.3.2 Packet Number 1: Position Report based on two balise groups |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM that : • BG1 is received. • Single BG2 is received. • Single BG2 is received again after the train has changed its orientation. Check on the JRU that: • The BG1 and the Single BG2 are recorded Check on the RTM: • When the train passes the SBG2 for the second time, a position report based on two balise groups is sent with the LRBG and the “previous LRBG” unchanged (from the values of the position report sent the first time the train passed the SBG2). |
| Constraints | Radio link is established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS / OS / SR / LS / NL |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 is received | I | BTM | Single balise group a (BGa) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 1 is transmitted | O | RTM | A position report based on two balise groups is sent with the LRBG set to “BGa”. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 is received | I | BTM | Single balise group b (BGb) | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 1 is transmitted | O | RTM | A position report based on two balise groups is sent with the “previous LRBG” set to “BGa”. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 10 | N/A | N/A | The train changes its orientation | - | - | Use_ FT4060300.8 (Desk is closed) AND Use_ FT3050500.5 (Communication session terminated) AND Use_ FT5120200.1 (Same engine used) AND Use_ FT5040500.14 (Start selected) AND Use_ FT4060300.48 (Change to FS mode) | N/A | N/A | - |
| 11 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train starts moving in the direction opposite to the direction when BGa and BGb was last passed. | N/A | N/A | - |
| 12 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 is received | I | BTM | Single balise group b (BGb) | N/A | N/A | - |
| 13 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 1 is transmitted | O | RTM | A position report based on two balise groups is sent with the “previous LRBG” set to “BGa”. The LRBG and the “previous LRBG” of the position report based on two balise groups shall remain unchanged. | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First balise |
| N_TOTAL | 3 | 0 | Single balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Single balise group a (BGa) |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 1 | Position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa |
| NID_PRVLRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 1 | Nominal |
| Q_DLRBG | 2 | 1 | Nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 1 | Nominal |
| M_MODE | 4 | 0 / 1 / 2 /11 / 12 | FS / LS / OS / SR / NL |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First balise |
| N_TOTAL | 3 | 0 | Single balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Single balise group b (BGb) |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 1 | Position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGb |
| NID_PRVLRBG | 24 | FINITE VALUE | BGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 1 | Nominal |
| Q_DLRBG | 2 | 1 | Nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 1 | Nominal |
| M_MODE | 4 | 0 / 1 / 2 /11 / 12 | FS / LS / OS / SR / NL |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 12: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First balise |
| N_TOTAL | 3 | 0 | Single balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Single balise group b (BGb) |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 14: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 1 | Position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGb |
| NID_PRVLRBG | 24 | FINITE VALUE | BGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 | Reverse |
| Q_DLRBG | 2 | 0 | Reverse |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 | Reverse |
| M_MODE | 4 | 0 / 1 / 2 /11 / 12 | FS / LS / OS / SR / NL |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS / OS / SR / LS / NL |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.4.2: Balise Co-ordinate System |  | 3040200 |
| Test Case of Feature | For balise groups consisting of a single balise, the information "direction with which the linked balise group will be passed over" received from a previous balise group shall assign a co-ordinate system to the balise. The assignment of the co-ordinate system shall be by means of linking data. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, OS, SR, UN, LS L2: FS, OS, SR, UN, LS L3: FS, OS, SR, UN, LS |  |  |
| Target of Test | The target of this test is to check that: • The linking information is used by the on-board equipment to determine in which direction the balise group is being passed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.4.2.3.2.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • The message displayed in the DMI correspond to the direction in which the balise group is being passed Check on the JRU that : • The message displayed in the DMI correspond to the one recorded with the balise group for the direction in which the balise group was passed |
| Constraints | • To determine whether the balises are passed in nominal / reverse direction, different text messages for each direction will be included. • The reference for the linking data shall be either a single balise group if a co-ordinate system has been assigned to it before, or a balise group consisting of two or more balises (with "inherent" co-ordinate system) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Linking | STORED | First SBG (Nominal Dir) Second SBG (Reverse dir) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | Train is running to a set of two consecutive balise groups | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message (N_TOTAL=0; NID_C="SBGa") containing packet 72 (Q_DIR=01) and packet 72 (Q_DIR=00) and packet 255 is received | I | BTM | First Single Balise Group is passed in its inherent nominal direction | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The "plain text message" is displayed | O | DMI | Text Message corresponding to the Text Message for nominal direction (Q_DIR=01) programmed in the first passed balise group is DISPLAYED | N/A | N/A | - |
| 5 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message (N_TOTAL=N; NID_C="BGb") containing packet 72 (Q_DIR=01) and packet 72 (Q_DIR=00) and packet 255 is received | I | BTM | Second Single Balise Group is passed in its inherent reverse direction | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The "plain text message" is displayed | O | DMI | Text Message corresponding to the Text Message for reverse direction programmed in the Second passed balise group is DISPLAYED | N/A | N/A | - |
| 9 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First position in the group |
| N_TOTAL | 3 | 0 | Single Balise Group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | SBGa | Identity of the balise group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 1 | Valid for nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 00, 01 | auxiliary or important information |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | 0 - 5 | Start event. Not 6, 7 (spare) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | 0 | No confirmation required |
| L_TEXT | 8 | FINITE VALUE | Length of X_TEXT |
| X_TEXT(L_TEXT) | 8 | "SBGa Passed in Nominal Dir" | Text String Element |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 0 | Valid for Reverse Direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | Not 11 (spare) |
| Q_TEXTCLASS | 2 | 00, 01 | auxiliary or important information |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | 0 - 5 | Start event. Not 6, 7 (spare) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | 0 | No confirmation required |
| L_TEXT | 8 | FINITE VALUE | Length of X_TEXT |
| X_TEXT(L_TEXT) | 8 | "SBGa Passed in Reverse Dir" | Text String Element |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First position in the group |
| N_TOTAL | 3 | 0 | Single balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Not 254. Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | SBGb | Identity of the balise group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 1 | Valid for nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 00, 01 | auxiliary or important information |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | 0 - 5 | Start event. Not 6, 7 (spare) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | 0 | No confirmation required |
| L_TEXT | 8 | FINITE VALUE | Length of X_TEXT |
| X_TEXT(L_TEXT) | 8 | "SBGb Passed in Nominal Dir" | Text String Element |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 0 | Valid for Reverse Direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 00, 01 | auxiliary or important information |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | 0 - 5 | Start event. Not 6, 7 (spare) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | 0 | No confirmation required |
| L_TEXT | 8 | FINITE VALUE | Length of X_TEXT |
| X_TEXT(L_TEXT) | 8 | "SBGb Passed in Reverse Dir" | Text String Element |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.4.2: Balise Co-ordinate System |  | 3040200 |
| Test Case of Feature | A group of two balises duplicating each other shall be treated as a single balise group in case where only one balise is correctly read |  | 6 |
| Applicable Mode/Level Combinations | L0: SH, UN, NL, PS, SL LNTC: NL, SN, PS, SL L1: FS, LS, OS, SR, SB, SH, TR, PT, NL, RV, PS, SL L2: FS, LS, OS, SR, SB, SH, TR, PT, NL, RV, PS, SL L3: FS, LS, OS, SR, SB, SH, TR, PT, NL, RV, PS, SL |  |  |
| Target of Test | The target of this test case is to check that : - When there is no linking information used on-board, after passing a group of two balises duplicating each other and only one balise is correctly read, then it shall be treated as a single balise group. So the train position information of the new LRBG will be recorded as Unknown. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.4.2.4.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM that : - Only one balise out of the two balises of the duplicating BG is received. Check on the JRU that: - Only one balise out of the two balises of the duplicating BG is recorded - Q_DIRLRBG=2 (Unknown) and Q_DLRBG=2 (Unknown) are recorded in the header of the message |
| Constraints | No linking information is used on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1;Q_LINK=1; M_DUP=10) containing packet 255 is received | I | BTM | Only one balise is received from the duplicated BG. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_DIRLRBG=10; Q_DLRBG=10) is recorded | O | JRU | Q_DIRLRBG=2 (Unknown) and Q_DLRBG=2 (Unknown) are recorded in the header of the message | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 01 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 10 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |