| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3130234 (3.13.2.3.4: Track conditions) Total: 6 Test case(s) |  |
| REF: | Subset-076-5-2-3130234 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.1 31.03.2013 | All | First version/review | CEDEX |
| Version 2.9.2 10.10.2013 | All | Requirements correction | CEDEX |
| Version 2.9.3 18.11.2013 | All | General review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.4: Track conditions |  | 3130234 |
| Test Case of Feature | Use of the track condition: “switch off eddy current brake for service brake” in the determination of supervision limits for target speed monitoring, when they are derived from the SBD curve, which will depend on the status ("active" or "not active") of this special brake. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, OS, LS L2/L3: FS, OS, LS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition “switch off eddy current brake for service brake” by the on-board equipment to determine the speed supervision limits for target speed monitoring. If the status of this special brake is “not active”, all locations shall be considered without the contribution of this special brake. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.13.2.3.4.1 |  |
|  |  | Subset-026-3.13.5.1 |  |
|  |  | Subset-026-3.13.5.2 |  |
|  |  | Subset-026-3.13.6.3.1.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | If the status of the special brake is “active”, the speed and distance monitoring will be checked before and after the train passes over the end of the track condition area and while the target has not changed. This is done in order to properly test the requirement 3.13.5.1. The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). The design of the Test Case requires a decrease in the permitted speed within the braking curve. |
| Constraints | It will be ensured that the supervision limits will be derived from the SBD curve (the target is an EOA). Q_NVGUIPERM=0, i.e., no permission to use guidance curves (this is the default value, see the appendix A3.2 of the SRS). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS or LS |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN=<well below V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 68 (M_TRACKCOND= 0111) is received | I | BTM | Only for L1 M_TRACKCOND= 0111, i.e., switch off eddy current brake for service brake | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (M_TRACKCOND= 0111) is received | I | RTM | Only for L2 /L3 M_TRACKCOND= 0111, i.e., switch off eddy current brake for service brake | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 /L3 | N/A | N/A | - |
| 7 | N/A | N/A | Eddy current brake for service braking inhibited | O | TIU | Only if the status of this special brake is “active” for the on-board equipment under test. The max safe front end of the train reaches the Switch off Eddy Current Brake area | N/A | N/A | - |
| 8 | N/A | N/A | Not Eddy current brake for service braking inhibited | O | TIU | Only if the status of this special brake is “not active” for the on-board equipment under test. The max safe front end of the train reaches the Switch off Eddy Current Brake area | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group is used to reset the odometry. This group has to be in a place slightly beyond the position marked by D_TRACKCOND | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 12 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has passed with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the SBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 13 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V1>) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V1> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 16 | N/A | N/A | Eddy current brake for service braking not inhibited | O | TIU | Only if the status of this special brake is “active” for the on-board equipment under test. The min safe rear end of the train has left the track condition area defined by L_ TRACKCOND. | N/A | N/A | - |
| 17 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | Only if the status of this special brake is “active” for the on-board equipment under test. This balise group is used to reset the odometry. This group has to be placed slightly beyond the position marked by D_TRACKCOND + L_ TRACKCOND + L_TRAIN | N/A | N/A | - |
| 18 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 20 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. When the train has passed with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the SBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 21 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V2>) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V2> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 22 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 23 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 111 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 111 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2/3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS and LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.4: Track conditions |  | 3130234 |
| Test Case of Feature | Use of the track condition: “switch off eddy current brake for emergency brake” in the determination of supervision limits for target speed monitoring, when they are derived from the EBD curve, which will depend on the status ("active" or "not active") of this special brake. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, OS, LS L2/L3: FS, OS, LS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition “switch off eddy current brake for emergency brake” by the on-board equipment to determine the speed supervision limits for target speed monitoring. If the status of this special brake is “not active”, all locations shall be considered without the contribution of this special brake. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.13.2.3.4.1 |  |
|  |  | Subset-026-3.13.5.1 |  |
|  |  | Subset-026-3.13.5.2 |  |
|  |  | Subset-026-3.13.6.2.1.5 |  |
|  |  | Subset-026-3.13.6.2.2.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Only if the speed dependent deceleration model(s) for the emergency brake are acquired as part of Train Data. If the status of the special brake is “active”, the speed and distance monitoring will be checked before and after the train passes over the end of the track condition area and while the target has not changed. This is done in order to properly test the requirement 3.13.5.1. The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). The design of the Test Case requires a decrease in the permitted speed within the braking curve. |
| Constraints | It will be ensured that the supervision limits will be derived from the EBD curve (the target is not an EOA). Q_NVGUIPERM=0, i.e., no permission to use guidance curves (this is the default value, see the appendix A3.2 of the SRS). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN=<below V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 68 (M_TRACKCOND= 1010) is received | I | BTM | Only for L1 M_TRACKCOND= 1010, i.e., switch off eddy current brake for emergency brake | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (M_TRACKCOND= 1010) is received | I | RTM | Only for L2 /L3 M_TRACKCOND= 1010, i.e., switch off eddy current brake for emergency brake | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 /L3 | N/A | N/A | - |
| 7 | N/A | N/A | Eddy current brake for emergency braking inhibited | O | TIU | Only if the status of this special brake is “active” for the on-board equipment under test. The max safe front end of the train reaches the Switch off Eddy Current Brake area | N/A | N/A | - |
| 8 | N/A | N/A | Not Eddy current brake for service braking inhibited | O | TIU | Only if the status of this special brake is “not active” for the on-board equipment under test. The max safe front end of the train reaches the Switch off Eddy Current Brake area | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group is used to reset the odometry. This group has to be in a place slightly beyond the position marked by D_TRACKCOND | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 12 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has passed with its maximum safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 tables 8 or 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 13 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V1>) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V1> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 16 | N/A | N/A | Eddy current brake for emergency braking not inhibited | O | TIU | Only if the status of this special brake is “active” for the on-board equipment under test. The min safe rear end of the train has left the track condition area defined by L_ TRACKCOND. | N/A | N/A | - |
| 17 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | Only if the status of this special brake is “active” for the on-board equipment under test. This balise group is used to reset the odometry. This group has to be placed slightly beyond the position marked by D_TRACKCOND + L_ TRACKCOND + L_TRAIN | N/A | N/A | - |
| 18 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 20 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. When the train has passed with its maximum safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 tables 8 or 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 21 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V2>) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V2> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 22 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 23 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 1010 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 1010 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.4: Track conditions |  | 3130234 |
| Test Case of Feature | Use of the track condition: “Switch off magnetic shoe brake” in the determination of supervision limits for target speed monitoring, which will depend on the status ("active" or "not active") of this special brake. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, OS, LS L2/L3: FS, OS, LS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition “Switch off magnetic shoe brake” by the on-board equipment to determine the speed supervision limits for target speed monitoring. If the status of this special brake is “not active”, all locations shall be considered without the contribution of this special brake. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.13.2.3.4.1 |  |
|  |  | Subset-026-3.13.5.1 |  |
|  |  | Subset-026-3.13.5.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | If the status of the special brake is “active”, the speed and distance monitoring will be checked before and after the train passes over the end of the track condition area and while the target has not changed. This is done in order to properly test the requirement 3.13.5.1. The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). The design of the Test Case requires a decrease in the permitted speed within the braking curve. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 *) |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN=<well below V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 68 (M_TRACKCOND= 1000) is received | I | BTM | Only for L1 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (M_TRACKCOND= 1000) is received | I | RTM | Only for L2 /L3 | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 /L3 | N/A | N/A | - |
| 7 | N/A | N/A | Magnetic shoe brake inhibited | O | TIU | Only if the status of this special brake is “active” for the on-board equipment under test. The train reaches the Switch off Magnetic shoe Brake area. | N/A | N/A | - |
| 8 | N/A | N/A | Not Magnetic shoe brake inhibited | O | TIU | Only if the status of this special brake is “not active” for the on-board equipment under test. The train reaches the Switch off Magnetic shoe Brake area. | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group is used to reset the odometry. This group has to be in a place slightly beyond the position marked by D_TRACKCOND | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 12 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has passed with its maximum safe front end or with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 tables 8 or 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 13 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V1>) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V1> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 16 | N/A | N/A | Magnetic shoe brake not inhibited | O | TIU | The min safe rear end of the train has left the track condition area defined by L_ TRACKCOND. | N/A | N/A | - |
| 17 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | Only if the status of this special brake is “active” for the on-board equipment under test. This balise group is used to reset the odometry. This group has to be placed slightly beyond the position marked by D_TRACKCOND + L_ TRACKCOND + L_TRAIN | N/A | N/A | - |
| 18 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 20 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. When the train has passed with its maximum safe front end or with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 tables 8 or 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 21 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V2>) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V2> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 22 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 23 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 1000 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 1000 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.4: Track conditions |  | 3130234 |
| Test Case of Feature | Use of the track condition: “Powerless section – lower pantograph” or “Powerless section – switch off the main power switch” in the determination of supervision limits for target speed monitoring, which will depend on the status ("active" or "not active") of regenerative brake not independent from the presence of voltage in the catenary. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, OS, LS L2/L3: FS, OS, LS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition “Powerless section – lower pantograph” or “Powerless section – switch off the main power switch” by the on-board equipment to determine the speed supervision limits for target speed monitoring. The deceleration of the service brake or emergency brake might be affected if the effect of a regenerative brake not independent from the presence of voltage in the catenary was included in the calculation of the deceleration value. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.13.2.3.4.1 |  |
|  |  | Subset-026-3.13.5.1 |  |
|  |  | Subset-026-3.13.5.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | If the status of the regenerative brake not independent from the presence of voltage in the catenary is “active”, the speed and distance monitoring will be checked before and after the train passes over the end of the track condition area and while the target has not changed. This is done in order to properly test the requirement 3.13.5.1. The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). The design of the Test Case requires a decrease in the permitted speed within the braking curve. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | In OS or LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN=<well below V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 68 (M_TRACKCOND= 0011 or 1001) is received | I | BTM | Only for L1 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (M_TRACKCOND= 0011 or 1001) is received | I | RTM | Only for L2 /L3 | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 /L3 | N/A | N/A | - |
| 7 | N/A | N/A | Lower pantograph | O | TIU | Only if the status of this special brake (the regenerative brake not independent from the presence of voltage in the catenary) is “active” for the on-board equipment under test. If M_TRACKCOND= 0011 is included in packet 68. The train reaches the powerless section area | N/A | N/A | - |
| 8 | N/A | N/A | Main Power Switch open | O | TIU | Only if the status of this special brake (the regenerative brake not independent from the presence of voltage in the catenary) is “active” for the on-board equipment under test. If M_TRACKCOND= 1001 is included in packet 68. The train reaches the powerless section area | N/A | N/A | - |
| 9 | N/A | N/A | Not Lower pantograph | O | TIU | Only if the status of this special brake (the regenerative brake not independent from the presence of voltage in the catenary) is “not active” for the on-board equipment under test. If M_TRACKCOND= 0011 is included in packet 68. The train reaches the powerless section area | N/A | N/A | - |
| 10 | N/A | N/A | Not Main Power Switch open | O | TIU | Only if the status of this special brake (the regenerative brake not independent from the presence of voltage in the catenary) is “not active” for the on-board equipment under test. If M_TRACKCOND= 1001 is included in packet 68. The train reaches the powerless section area | N/A | N/A | - |
| 11 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group is used to reset the odometry. This group has to be in a place slightly beyond the position marked by D_TRACKCOND | N/A | N/A | - |
| 12 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has passed with its maximum safe front end or with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 tables 8 or 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V1>) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V1> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 16 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 17 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 18 | N/A | N/A | Raise pantograph | O | TIU | Only if the status of this special brake (the regenerative brake not independent from the presence of voltage in the catenary) is “active” for the on-board equipment under test. If M_TRACKCOND= 0011 is included in packet 68. The min safe front end of the train has left the track condition area defined by L_ TRACKCOND. | N/A | N/A | - |
| 19 | N/A | N/A | Main Power Switch close | O | TIU | Only if the status of this special brake (the regenerative brake not independent from the presence of voltage in the catenary) is “active” for the on-board equipment under test. If M_TRACKCOND= 1001 is included in packet 68. The min safe front end of the train has left the track condition area defined by L_ TRACKCOND. | N/A | N/A | - |
| 20 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | Only if the status of this special brake (the regenerative brake not independent from the presence of voltage in the catenary) is “not active” for the on-board equipment under test. This balise group is used to reset the odometry. This group has to be in a place slightly beyond the position marked by D_TRACKCOND + L_ TRACKCOND | N/A | N/A | - |
| 21 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if the status of this special brake (the regenerative brake not independent from the presence of voltage in the catenary) is “not active” for the on-board equipment under test. | N/A | N/A | - |
| 22 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 23 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. When the train has passed with its maximum safe front end or with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 tables 8 or 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 24 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V2>) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V2> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 25 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 26 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 0011/1001 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 0011/1001 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 11: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 11: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 20: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 20: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.4: Track conditions |  | 3130234 |
| Test Case of Feature | Use of the track condition: “Switch off regenerative brake” in the determination of supervision limits for target speed monitoring, which will depend on the status ("active" or "not active") of this special brake. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, OS, LS L2/L3: FS, OS, LS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition “Switch off regenerative brake” by the on-board equipment to determine the speed supervision limits for target speed monitoring. If the status of this special brake is “not active”, all locations shall be considered without the contribution of this special brake. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.13.2.3.4.1 |  |
|  |  | Subset-026-3.13.5.1 |  |
|  |  | Subset-026-3.13.5.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | If the status of the special brake is “active”, the speed and distance monitoring will be checked before and after the train passes over the end of the track condition area and while the target has not changed. This is done in order to properly test the requirement 3.13.5.1. The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). The design of the Test Case requires a decrease in the permitted speed within the braking curve. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | In OS or LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN=<well below V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 68 (M_TRACKCOND= 0110) is received | I | BTM | Only for L1 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (M_TRACKCOND= 0110) is received | I | RTM | Only for L2 /L3 | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 /L3 | N/A | N/A | - |
| 7 | N/A | N/A | Regenerative brake inhibited | O | TIU | Only if the status of this special brake is “active” for the on-board equipment under test. The train reaches the Switch off regenerative Brake area | N/A | N/A | - |
| 8 | N/A | N/A | Not Regenerative brake inhibited | O | TIU | Only if the status of this special brake is “not active” for the on-board equipment under test. The train reaches the Switch off regenerative Brake area | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group is used to reset the odometry. This group has to be in a place slightly beyond the position marked by D_TRACKCOND | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 12 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has passed with its maximum safe front end or with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 tables 8 or 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 13 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V1>) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V1> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 16 | N/A | N/A | Regenerative brake not inhibited | O | TIU | The min safe rear end of the train has left the track condition area defined by L_ TRACKCOND. | N/A | N/A | - |
| 17 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | Only if the status of this special brake is “active” for the on-board equipment under test. This balise group is used to reset the odometry. This group has to be placed slightly beyond the position marked by D_TRACKCOND + L_ TRACKCOND + L_TRAIN | N/A | N/A | - |
| 18 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The DMI aspect is coherent with Indication Status when braking to a target. | N/A | N/A | - |
| 20 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. When the train has passed with its maximum safe front end or with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 tables 8 or 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 21 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V2>) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V2> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 22 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 23 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 110 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 110 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.4: Track conditions |  | 3130234 |
| Test Case of Feature | Use of the track condition: “switch off eddy current brake for service brake” in the determination of supervision limits for target speed monitoring when they are derived from the GUI curve, which will depend on the status ("active" or "not active") of this special brake. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, OS L2/L3: FS, OS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition “switch off eddy current brake for service brake” by the on-board equipment to determine the speed supervision limits for target speed monitoring. If the status of this special brake is “not active”, all locations shall be considered without the contribution of this special brake. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.13.2.3.4.1 |  |
|  |  | Subset-026-3.13.5.1 |  |
|  |  | Subset-026-3.13.5.2 |  |
|  |  | Subset-026-3.13.6.4.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | If the status of the special brake is “active”, the speed and distance monitoring will be checked before and after the train passes over the end of the track condition area and while the target has not changed. This is done in order to properly test the requirement 3.13.5.1. The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). The design of the Test Case requires a decrease in the permitted speed within the braking curve. |
| Constraints | It will be ensured that the supervision limits will be derived from the GUI curve. Q_NVGUIPERM=1, i.e., permission to use guidance curves. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | If level 2,3 |
| National Values | STORED | Q_NVGUIPERM=1, i.e., permission to use guidance curves |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN=<well below V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 68 (M_TRACKCOND= 0111) is received | I | BTM | Only for L1 M_TRACKCOND= 0111, i.e., switch off eddy current brake for service brake | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (M_TRACKCOND= 0111) is received | I | RTM | Only for L2 /L3 M_TRACKCOND= 0111, i.e., switch off eddy current brake for service brake | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 /L3 | N/A | N/A | - |
| 7 | N/A | N/A | Eddy current brake for service braking inhibited | O | TIU | Only if the status of this special brake is “active” for the on-board equipment under test. The max safe front end of the train reaches the Switch off Eddy Current Brake area | N/A | N/A | - |
| 8 | N/A | N/A | Not Eddy current brake for service braking inhibited | O | TIU | Only if the status of this special brake is “not active” for the on-board equipment under test. The max safe front end of the train reaches the Switch off Eddy Current Brake area | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group is used to reset the odometry. This group has to be in a place slightly beyond the position marked by D_TRACKCOND | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has passed with either its estimated or its max safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated either from the SBD or the EBD, then the supervision status changes to "Overspeed Status". So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V1>) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V1> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 15 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status when braking to a target. Permitted speed displayed is compared to ERA tool calculation with the new condition (Switch off Eddy current brake). The location of the Permitted speed supervision limit shall be derived from the GUI curve. | N/A | N/A | - |
| 16 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 17 | N/A | N/A | Eddy current brake for service braking not inhibited | O | TIU | Only if the status of this special brake is “active” for the on-board equipment under test. The min safe rear end of the train has left the track condition area defined by L_ TRACKCOND. | N/A | N/A | - |
| 18 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | Only if the status of this special brake is “active” for the on-board equipment under test. This balise group is used to reset the odometry. This group has to be placed slightly beyond the position marked by D_TRACKCOND + L_ TRACKCOND + L_TRAIN | N/A | N/A | - |
| 19 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. | N/A | N/A | - |
| 20 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. When the train has passed with either its estimated or its max safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated either from the SBD or the EBD, then the supervision status changes to "Overspeed Status". So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 21 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM=<V2>) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only in FS or OS mode: V_PERM=<V2> is compared with the results of a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA_Braking_curves_tool_vx.x). Only in LS mode: V_TRAIN (current train speed), instead of V_PERM, will have to be considered. | N/A | N/A | - |
| 22 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if the status of this special brake is “active” for the on-board equipment under test. The train decelerates until the DMI aspect is coherent with the Indication Status. | N/A | N/A | - |
| 23 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | Only if the status of this special brake is “active” for the on-board equipment under test. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 111 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 111 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | If level 2/3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |